3GPP TSG RAN WG1 #112bis-e		R1-2302577

e-Meeting, April 17th – April 26th, 2023

Source:	OPPO

Title:	   Discussion on UE features for multi-carrier enhancement

Agenda Item:	9.17.10

Document for:	Discussion and Decision

# Introduction

This contribution elaborates our views on UE features for Rel-18 multi-carrier enhancement.

# Discussion

UE feature for multi-cell scheduling with DCI 0_X/1_X

In the current UE feature structure, DCI 0_0/1_0/0_1/1_1 are mandatory Rel-15 feature (in FG3-1) without capability signaling while DCI 0_2/1_2 is optional Rel-16 feature (FG11-1) with capability signaling. In our view, some characteristics of DCI 0_2/1_2 feature can apply to DCI 0_X/1_X.

Proposal 1: A new FG is defined for Rel-18 support of multi-cell scheduling with DCI 0_X/1_X.

- The new FG is optional with UE capability signaling.

- The new FG does not differentiate between FR1 and FR2.

  - FFS whether/how to capture RAN1 agreement that the co-scheduled cells per single DCI 0_X/1_X shall have the same carrier type between FR1 and FR2.

- The new FG does not differentiate between FDD and FDD.

As for the new FG components, RAN1 agreed that, although the spec-allowed maximum number of co-scheduled cells per single DCI 0_X/1_X is 4, the maximum number of co-scheduled cells by a DCI format 0_X and the maximum number of co-scheduled cells by a DCI format 1_X for a UE can be the same or different, and can be smaller than or equal to 4. This should be reflected as a UE capability.

However, according to the RAN1 agreements on DCI 0_X/1_X payload size determination, the “maximum number of cells in a co-scheduled cell set” can have stronger influence to the UE implementation on PDCCH processing than the “maximum number of co-scheduled cells by a single DCI 0_X/1_X”.  The latter one may be more meaningful for UE’s capability on co-scheduled PDSCH, while the former one is more meaningful for UE’s implementation capability on PDCCH. Therefore we suggest including “maximum number of cells in a co-scheduled cell set” in the new FG components as well. Additionally, RAN1 already agreed that the number of cell sets that UE can be configured and scheduled from a same scheduling cell is reported as UE capability.

Proposal 2: The new FG in Proposal 1 has the following FG components:

- Component-1:  Supports monitoring DCI format 1_X for DL scheduling, with one DCI format 1_X scheduling PDSCH on up to NDL,max cells in a co-scheduled cell set that can have maximum Nset_size,max cells, where 2≤NDL,max ≤ Nset_size,max≤4.

- Component-2:  Supports monitoring DCI format 0_X for UL scheduling, with one DCI format 0_X scheduling PUSCH on up to NUL,max cells in a co-scheduled cell set that can have maximum Nset_size,max cells, where 2≤NUL,max ≤ Nset_size,max≤4.

- Note: NDL,max , NUL,max and Nset_size,max above are UE capability parameters

- Component-3: Maximum number of co-scheduled cell sets (Nset,max) that can be scheduled from a same scheduling cell.

- FFS whether to introduce multiple combinations of values for these capability parameters.

Note that the “maximum number of cells in a co-scheduled cell set” (i.e., Nset_size,max above) can work for both table-configuration based indication and FDRA-reuse based indication for the co-scheduled cell combination. For a capability indication with more precise capability granularity, the maximum number of table rows can be considered as an alternative capability parameter for table-configuration based cell combination indication.

Regarding to the FFS in Proposal 2, Proposal 2 introduces four capability parameters < NDL,max , NUL,max, Nset_size,max, Nset,max > and gives individual maximum values, under the assumption of no dependency among these maximum values. However, it is also possible to take these four capability parameters into a multi-dimensional capability space, with multiple combinations of maximum values, where one combination of maximum values may observe a larger value for one capability parameter and a smaller value for another capability parameter comparing to another combination of maximum values. Under this scheme, a UE may report, e.g., both <3,3,4,2> and <2,2,3,4> as capability. This scheme may allow better utilization of UE implementation resources but may lead to more sophisticated RAN1 discussions.

There was a brief discussion in RAN1 #112 regarding to whether/how to have UE capability indication for UE’s support of two mechanisms to indicate the co-scheduled cell combinations: table-based indication and FDRA-reuse-based indication. We certainly do not think the UE implementation should be mandated to support both. There was a proposal in RAN1 #112 to make the FDRA-reuse-based method mandatory and the table-based method optional. However, it was also commented that the DCI 0_X/1_X payload size derivations (3rd and 4th bullets) in the following FDRA-reuse based method are not clearly finalized and are still open to solution explore. It is premature to lock some uncertainty as mandatory at this stage.

- The UE determines the actually scheduled cell(s) based on the FDRA field of each cell of the set of cells. - For Type 0 FDRA, all 0s indicates the cell is not scheduled. - For Type 1 FDRA, all 1s indicates the cell is not scheduled. - The size of the Type 2 fields for each cell does not change according to actually co-scheduled cells. - The payload size of DCI format 0_X is derived by UE based on RRC configuration of the active BWP(s) of all cells within the set of cells. - The payload size of DCI format 1_X is derived by UE based on RRC configuration of the active BWP(s) of all cells within the set of cells.

Proposal 3: At least one method of co-scheduled cell combination indication, between table-based and FRDA-reuse-based, is capability optional in FG component.

- RAN1 clarifies details of the DCI 0_X/1_X payload size derivation for the FDRA-reuse based cell combination indication, before determining which method is selected as optional.

Given RAN1 agreement does not prevent DCI 0_X/1_X from being configured in the same search space with legacy DCI formats (if we correctly understood the RAN1 #112 agreement about “independent configuration of separate search space sets between DCI 0_X/1_X and legacy DCIs”), there should be another FG similar to FG11-1a, created for support of monitoring DCI 0_X/1_X and any of legacy unicast DCI in the same search space.

Proposal 4: With clarification of RAN1 #112 agreement on “independent configuration on separate search space”, a second new FG is introduced for DCI 0_X/1_X upon necessity to fulfill the similar purpose of FG11-1a.

Besides the above UE features that can find logic source from existing UE feature architecture and earlier RAN1 discussion in Rel-18 WI, there are some DCI 0_X/1_X related features that are logically brand-new in Rel-18 and therefore may need further discussion. One example is the new configuration, Configuration 3, of nominal RBG size for RA type 0. Within Configuration 3, RBG size of 32 is a new RBG size that never appears in legacy specification, and therefore the UE implementation should be given a chance not to implement it in support of scheduling with DCI 0_X/1_X in RA type 0. Another example is Type-2 HARQ codebook that contains two sub-codebooks.

Proposal 5: The following FG component is included in the new FG in Proposal 1.

- Support of nominal RBG size of Configuration 3.

- Support of Type-2 HARQ codebook within multi-cell scheduling by DCI 0_X/1_X.

# Conclusion

This contribution concludes with following proposals:

Proposal 1: A new FG is defined for Rel-18 support of multi-cell scheduling with DCI 0_X/1_X.

- The new FG is optional with UE capability signaling.

- The new FG does not differentiate between FR1 and FR2.

  - FFS whether/how to capture RAN1 agreement that the co-scheduled cells per single DCI 0_X/1_X shall have the same carrier type between FR1 and FR2.

- The new FG does not differentiate between FDD and FDD.

Proposal 2: The new FG in Proposal 1 has the following FG components:

- Component-1:  Supports monitoring DCI format 1_X for DL scheduling, with one DCI format 1_X scheduling PDSCH on up to NDL,max cells in a co-scheduled cell set that can have maximum Nset_size,max cells, where 2≤NDL,max ≤ Nset_size,max≤4.

- Component-2:  Supports monitoring DCI format 0_X for UL scheduling, with one DCI format 0_X scheduling PUSCH on up to NUL,max cells in a co-scheduled cell set that can have maximum Nset_size,max cells, where 2≤NUL,max ≤ Nset_size,max≤4.

- Note: NDL,max , NUL,max and Nset_size,max above are UE capability parameters

- Component-3: Maximum number of co-scheduled cell sets that can be scheduled from a same scheduling cell.

- FFS whether to introduce multiple combinations of values for these capability parameters.

Proposal 3: At least one method of co-scheduled cell combination indication, between table-based and FRDA-reuse-based, is capability optional in FG component.

- RAN1 clarifies details of the DCI 0_X/1_X payload size derivation for the FDRA-reuse based cell combination indication, before determining which method is selected as optional.

Proposal 4: With clarification of RAN1 #112 agreement on “independent configuration on separate search space”, a second new FG is introduced for DCI 0_X/1_X upon necessity to fulfill the similar purpose of FG11-1a.

Proposal 5: The following FG component is included in the new FG in Proposal 1.

- Support of nominal RBG size of Configuration 3.

- Support of Type-2 HARQ codebook within multi-cell scheduling by DCI 0_X/1_X.

The above proposals are summarized in an UE feature table in Appendix.

# Reference

- TS 38.822 v17.0.0,  NR User Equipment (UE) feature list (Release 17)

# Appendix

Table 1 Expected UE features for Rel-18 single DCI scheduling multi-cells

| Feature group | Components | Prerequisite feature groups | Need of FDD / TDD differentiation | Need of FR1 / FR2 differentiation | Note | Mandatory / Optional |
| --- | --- | --- | --- | --- | --- | --- |
| XYZ | - Supports monitoring DCI format 1_X for DL scheduling, with one DCI format 1_X scheduling multiple PDSCH on up to NDL,max cells in a co-scheduled cell set that can have maximum Nset_size,max cells. - Supports monitoring DCI format 0_X for UL scheduling, with one DCI format 0_X scheduling multiple PUSCH on up to NUL,max cells in a co-scheduled cell set that can have maximum Nset_size,max cells. - Maximum number of co-scheduled cell sets that can be scheduled from a same scheduling cell. - Support co-scheduled cell combination indication based on [table configuration vs. FDRA field reuse]. - Support of nominal RBG size of Configuration 3. - Support of Type-2 HARQ codebook. |  | No | No | In component 1, 2≤NDL,max ≤ Nset_size,max≤4; In component 2, 2≤NUL,max ≤ Nset_size,max≤4; FFS: multiple combinations of maximum values | Optional with capability signalling |
| XYZ1 | - Supports monitoring both DCI format 0_X/1_X and any of DCI format 0_1/0_2/1_1/1_2 in the same search space. [Note: this FG should be removed if RAN1 clarifies the “separate search spaces” in RAN1 #112 agreement means “different search spaces”.] | XYZ | No | No |  | Optional with capability signalling |