3GPP TSG-RAN WG1 #112bis-e	R1-2302755

e-Meeting, April 17th – April 26th, 2023

Title 	:  Remaining issues for MBS UE feature

Source 	:  ZTE

Agenda item	:  7.2

Document for	:  Discussion/Decision

Introduction

After RAN1#112 meeting, the UE features for MBS have been finalized with some minor remaining issues. In this contribution, the remaining issue is discussed.

Discussion

In RAN1#112 meeting, companies had different views on whether UE is allowed to indicate support or not of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH activation. From network perspective, the ACK/NACK for SPS group-common PDSCH activation is important to align the understanding on SPS activation between gNB and UE. Thus, we have the following proposal.

Proposal 1: Update the UE feature 33-5-1a for R17 MBS as follows.

| 33. NR_MBS | 33-5-1a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | 1. Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, [SPS group-common PDSCH activation] 2. Support of PTM retransmission for SPS multicast associated with G-CS-RNTI 3. Support of Type-1 and Type-2 HARQ-ACK CB for SPS multicast feedback only 4. Support of shared SPS-PUCCH-AN-List configuration from unicast SPS | 33-5-1 |
| --- | --- | --- | --- | --- |

Conclusion

In this contribution, the remaining issue for MBS UE features is discussed with the following proposal.

Proposal 1: Update the UE feature 33-5-1a for R17 MBS as follows.

| 33. NR_MBS | 33-5-1a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | 1. Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, [SPS group-common PDSCH activation] 2. Support of PTM retransmission for SPS multicast associated with G-CS-RNTI 3. Support of Type-1 and Type-2 HARQ-ACK CB for SPS multicast feedback only 4. Support of shared SPS-PUCCH-AN-List configuration from unicast SPS | 33-5-1 |
| --- | --- | --- | --- | --- |

Reference

- R1-2302024, Updated RAN1 UE features list for Rel-17 NR after RAN1 #112_final, RAN1#112.