3GPP TSG RAN WG1 #112bis-e			R1-2303469

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.1.2

Source:	Apple Inc.

Title:	Views on Rel-18 MIMO CSI enhancement

Document for:	Discussion/Decision

# Introduction

In the following, we summarize the NR CSI design evolution from Rel-15/16/17.

- In Rel-15, both Type I and Type II codebook were introduced for advanced PMI reporting. The codebook structure relies on W=W1*W2 in which W1 is the spatial bases selection matrix, and W2 is the combination coefficient matrix. Spatial bases are constructed based on the oversampled two-dimensional DFT matrix

  - For Type I codebook, both single panel and multiple panel codebook are supported

  - For Type II codebook, both regular Type II and port selection codebook are supported

- In Rel-16, to reduce Type II codebook CSI overhead, frequency domain compression was introduced for enhanced Type II codebook based on frequency bases. Frequency bases are constructed based on DFT matrix. The codebook structure of the enhanced Type II codebook is W=W1*W2*Wf in which W1 is the spatial bases selection matrix, W2 is the combination coefficient matrix, and Wf is frequency bases selection matrix.

- In Rel-17, Type II port selection is further enhanced by allowing more flexible port selection while restricting the frequency bases selection. In addition, CSI report enhancement was introduced to support Non-Coherent Joint Transmission (NCJT). However, the scope of CSI enhancement for NCJT is only limited to Type I codebook and only covers the Single-DCI Multi-TRP scheme 1a, i.e., SDM scheme

In Rel-18, as part of the approved WID in RP-213598, [1], the following two objectives are considered for CSI enhancement

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking - Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e., 32

In last RAN1 meeting #112 [2], the following agreement/conclusion were reached regarding Rel-18 CSI enhancement

Conclusion On the Parameter Combination of Type-II codebook refinement for CJT mTRP, there is no consensus on adding a new (not previously agreed) codebook parameter, as well as replacing the legacy parameter L with a new (not previously agreed) parameter. - Note: Since dynamic {Ln} selection was agreed, this implies that the list of supported {Ln} combinations will be discussed separately from the list of supported {pv,} combinations - FFS: Whether/how the list of supported {Ln} combinations can be linked with the list of supported {pv,} combinations without introducing a new (not previously agreed) codebook parameter, e.g. via some UE capability Agreement On the Type-II codebook refinement for CJT mTRP, only support NL ={2,4} as additional candidate values to NL=1. - FFS: Additional restriction(s) depending on the configured value for NTRP Agreement On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, support at least the following combinations of {Ln} for the higher-layer-configured value of NTRP (FFS by RAN1#112: whether the bracketed permutations are also supported): - FFS by RAN1#112: whether other combinations can be supported FFS (by RAN1#112bis-e): Whether/how the supported combinations of {n} for Rel-17-based refinement are derived from the supported combinations of {Ln} for Rel-16-based refinement FFS: Whether the total number of Ln is a UE capability NTRP; {Ln} combination. 1; {2}. {4}. {6} (analogous to legacy, only for total # ports =32, rank 1-2, R=1. 2; {2,2}. {2,4}, [{4,2}]. {4,4}. 3; {2,2,2}. {2,2,4} [and its other permutations]. {4,4,4}. 4; {2,2,2,2}. {2,2,2,4} [and its other permutations]. {2,2,4,4} [and its other permutations]. {4,4,4,4}. Agreement On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, regarding the list of supported combinations of {Ln}, only support the following additional combinations: NTRP; {Ln} combination. 2; {4,2}. 3; {2,4,2}, {4,2,2}. No other permutations are supported. FFS: For NTRP>1, in addition to the supported combinations/permutations, whether to support at least one additional combination where at least one of the Ln values (n=1, …, NTRP) is 6 Agreement On the Parameter Combination of Type-II codebook refinement for CJT mTRP, support linkage between the list of supported {Ln} combinations and list of supported {pv,} combinations via pairing each combination for {pv,} with at least one combination for {Ln}, for each NTRP value. - FFS (by RAN1#112bis-e): The exact list of supported pairs/linkage, or restriction of {Ln} when paired to each of {pv,} - FFS (by RAN1#112bis-e): Whether/How to support configuration signalling for indicating the linkage - Note: While no additional codebook parameter will be introduced, the total number of SD basis vectors across CSI-RS resources can still be used as a criterion for choosing the supported pairs/linkage Agreement On the Type-II codebook refinement for CJT mTRP, for mode-1, down select (in RAN1#112) only one from the following schemes - Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: where is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource with , and is commonly selected across N CSI-RS resources - Alt2. independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset) For all the above alternatives, the legacy FD basis selection indication scheme is applied on each selected FD basis. Note: Per previous agreements, the number of selected FD basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources Agreement On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, support at least the following combinations of {pv,} from where the value of {pv,} is gNB-configured via higher-layer (RRC) signaling: - FFS by RAN1#112: whether other combinations can be supported FFS (by RAN1#112bis-e): Whether/how the supported combinations of {M} for Rel-17-based refinement are derived from the supported combinations of {pv ,} for Rel-16-based refinement pv for layers 1-4; ; Condition(s). {1/8, 1/8, 1/16, 1/16}; ¼; --. ½; --. {1/4, 1/4, 1/8, 1/8}; ¼ (*); --. ½ (*); --. {1/4, 1/4, 1/4, 1/4}; ¾ (*); --. {1/2, 1/2, 1/2, 1/2}; ½; - Only applicable when NTRP≤3 and NL=1 - Optional. (*) Supported by legacy Rel-16 Agreement On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, down-select between the following three alternatives (by RAN1#112-bis where n denotes the n-th CSI-RS resource): - Alt1. Prio(,l,m,n)=() .N.RI.P(m)+N.RI.l(n)+N.n - Note: This implies that CSI-RS resource is designated the highest priority - Alt2. Prio(,l,m,n)=2L’.Qn).RI.N3+2L’.RI. P(m)+RI.l(n)+ - Note: This implies that CSI-RS resource is designated the lowest priority (after FD basis) - Note: L’ denotes the max value of Ln from all selected N CSI-RS resources - FFS: Q(n) maps the index n according to a rule, e.g., Q(n)=n, or Q(n)=0 if n corresponds to strongest TRP/SCI. - Alt3. Replace SD basis index l in legacy Prio calculation with , i.e., SD basis index over all resources: Prio(,l,m,n) = 2Ltot.RI.P(m)+ RI.+RI.l(n)+ FFS: FD permutation P(.) as Rel-16-analogous, or no permutation i.e. P(m)=m Agreement On the Type-II codebook refinement for CJT mTRP, regarding CBSR, at least for restricting SD basis selection, the legacy CBSR scheme is fully reused for each of the RRC-configured NTRP CSI-RS resources (resulting in CSI-RS-resource-specific SD beam group restriction) - FFS: Whether amplitude restriction is CSI-RS-resource-common or specific, and soft vs hard restriction - FFS: Whether CBSR can be configured to be off for a CSI-RS resource The same rank restriction is applied across NTRP CSI-RS resources Proposal 2.A: The Rel-18 Type-II codebook refinement for high/medium velocities comprises refinement of the following codebooks: - Refinement of the Rel-16 eType-II regular codebook, with N4>=1 - Refinement of the Rel-17 FeType-II port selection (PS) codebook, based on the common design with the Refinement of the Rel-16 eType-II regular codebook, except for the supported set of parameter combinations, with N4=1 - Time-/Doppler-domain reciprocity is not assumed Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), as well as the number of CQIs (=X) in one sub-band and one CSI reporting instance, support only the following: - Basic feature: X=1 and the CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices - Optional features: - X=1 and the CQI is associated with: - the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - the last slot of the CSI reporting window (slot l+WCSI–1) and the N4-thW2 matrix - X=2 and - The 1st CQI is associated with the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - The 2nd CQI is associated with the middle slot of the CSI reporting window (slot l+WCSI/2) and the (N4 /2)-thW2 matrix - FFS: Whether/how to include CQI overhead reduction for X=2 Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, CQI is defined per legacy CQI definition (ensuring at most 10% BLER) within the slot(s) which a CQI is associated with. Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, on the quantization scheme when N4>1, reuse the following components of the legacy per-coefficient quantization scheme: - Alphabets for amplitude and phase - Quantization of phase and quantization of differential amplitude relative to a reference, reference amplitude (with SCI determining the location of one reference amplitude), where the reference is defined for each layer and each “group” of coefficients Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, on the quantization scheme when N4>1, for each layer: - One (common) SCI (Strongest Coefficient Indicator) applies across all Q selected DD basis vectors - One group comprises one polarization across all Q selected DD basis vectors (Cgroup,phase=1, Cgroup,amp=2) - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported Agreement For the Type-II codebook refinement for high/medium velocities, - The constraint on the maximum number of non-zero coefficients (NZCs) per-layer (K0) is defined jointly across all Q DD basis vectors. - FFS: How K0 is calculated - Also support a constraint on the total number of non-zero coefficients (NZCs) summed across all Q DD basis vectors and across all layers: - Following the legacy specification, the maximum total number is 2K0 Agreement For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, down-select one from the following alternatives (no later than RAN1#112bis-e): - Alt1. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design i.e. the size of the bitmap for each selected DD basis vector is 2LMv - Alt3A: A single 2-dimensional bitmap of size to report the selected pairs of FD basis vector and DD basis vector and a single 2-dimensional bitmap of size for indicating the location of the NZCs, where each row corresponds to a selected SD basis vector and each column corresponds to one of the selected pairs of FD basis vector and DD basis vector. - Alt4. A bitmap that includes bits associated with the set of {(, ,)} with , where is the threshold that can be configured by gNB, , and denotes a reference SD basis index and a reference FD basis index and a reference DD basis index associated with SCI, respectively. Agreement For the Type-II codebook refinement for high/medium velocities, regarding the down-selection of bitmap(s) for indicating the locations of the NZCs (in RAN1#112bis-e), the following is used as a guidance for evaluation: - Following the agreed EVM, use “UPT vs. overall overhead (including CQI and PMI)” to compare across alternatives, assuming at least FTP1 traffic model and Rel-16 Parameter Combinations (L, beta, pv) - Use only the supported codebook parameter values (e.g. Q, K, m, d, delta, N4) - Companies are to state their assumptions on UE-side prediction (e.g. ideal or realistic, CSI-RS type, CSI-RS overhead calculation in relation to UPT, assumptions on WCSI and l) and the use of rank adaptation Conclusion For the Type-II codebook refinement for high/medium velocities, for N4>1, regarding the parameter Q, there is no consensus in supporting additional candidate values Conclusion For the Type-II codebook refinement for high/medium velocities, regarding the parameter N4 (length of DFT vector, unit-less), there is no consensus in supporting additional candidate values Agreement For the Type-II codebook refinement for high/medium velocities, for N4>2 and Q=2, the selection of Q out of N4 DD basis vectors is indicated by a -bit indicator in CSI part 2 - Analogous to FD basis selection, DD basis index 0 (representing DC) is always selected. Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter K (the number of AP-CSI-RS resources for the CMR), optionally support only K=12 as an additional candidate value Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter d (in slots), - for P/SP-CSI-RS, support d equal to the periodicity of the CSI-RS resource - for AP-CSI-RS, also support d =1 Agreement For the Type-II codebook refinement for high/medium velocities, regarding the parameter δ (in slots), in addition to 0 and 2, δ=1 is additionally supported Conclusion On the Parameter Combination of Type-II codebook refinement for high/medium velocities, there is no consensus on including another non-UCI Doppler codebook parameter as a variable in the list of supported Parameter Combinations. - Note: This implies that other non-UCI Doppler codebook parameters will be a part of RRC configuration (either explicit or implicit) Agreement On the Type-II codebook refinement for high/medium velocities based on Rel-16 regular eType-II codebook (if supported), for the purpose of choosing the supported Parameter Combinations - Regarding the codebook parameter pv, in addition to the supported values from the legacy specification, introduce as additional candidate values - pv =1/8 for v=1,2 (hence 1/16 for v=3,4) - pv =1/2 for v=1,2,3,4 - Regarding the codebook parameter , in addition to the supported values from the legacy specification, introduce as an additional candidate value = 1/8 - Regarding the codebook parameter L, the supported values from the legacy specification apply Agreement On the Type-II codebook refinement for high/medium velocities, regarding UCI omission, down-select between the following three alternatives (by RAN1#112bis-e where q denotes the q-th DD basis vector): - Alt1. Prio(,l,m,q)=2L. Q.RI.P(m)+Q.RI.l+Q.q - Note: This implies that DD basis is designated the highest priority - Alt2. Prio(,l,m,q)=2L.S(q).RI.N3+2L.RI. P(m)+RI.l+ - Note: This implies that DD basis is designated the lower priority (after FD basis) - FFS: S(q) maps the index q according to a rule - Alt3. Prio(,l,m,q)=2L.RI.Mv.q + 2L.RI.P(m)+ RI.l +  - Note: This implies that DD basis is designated the least priority - Alt4. Prio(,l,m,q)=2L.P(m).RI.Q+2L.RI.S(q)+RI.l+ - Note: This implies that DD basis is designated with lower priority (after SD basis) and higher priority (before FD basis) - FFS: S(q) maps the index q according to a rule FFS: FD permutation P(.) as Rel-16-analogous, or no permutation i.e. P(m)=m q=0,…,Q-1 Agreement For aiding gNB determination of codebook switching and SRS periodicity with the Rel-18 TRS -based TDCP reporting, support reporting quantized wideband normalized amplitude/phase of the time-domain correlation profile with Y≥1 delay(s) as follows: - Basic feature: Y=1 with delay≤ Dbasic symbols, only wideband quantized normalized amplitude is reported - FFS: Candidate values for delay - Optional feature: Y=1 with delay>Dbasic symbols and Y≥1, wideband quantized normalized amplitude and phase for each delay are reported - For Y>1, the phase can be configured to be absent for all the Y delays - TBD: Whether the value of Y is configurable or following the delays from the configured TRS resource - TBD: Candidate value(s) for Y>1 - FFS: Value of Dbasic Agreement For the Rel-18 TRS-based TDCP reporting, the priority of the CSI report(s) associated with TDCP reporting is down-selected from the following alternatives: - Alt1. Lower than other CSI reports - Alt2. Same as CSI report(s) not carrying L1-RSRP or L1-SINR - Alt3. Higher than other CSI reports - Other alternatives are not precluded. Agreement For the Rel-18 TRS-based TDCP reporting, regarding the value of parameter Y for Y>1, down-select from the following alternatives: - Alt1. The value of Y is gNB-configured via higher-layer (RRC) signalling. - Alt2. The value of Y follows the delays from the configured TRS resource. - Alt3. The value of Y is UE-selected and reported. The value of Y is a UE capability. Agreement For the Rel-18 TRS-based TDCP reporting, support multiplexing TDCP reporting with other UCI parameters on PUSCH following the legacy UCI multiplexing rule for AP-CSI Agreement For the Rel-18 TRS-based TDCP reporting, for TDCP measurement and calculation, by RAN1#112bis-e, decide between the following alternatives: - Alt1. Fully reuse legacy TRS - Alt2. Study enhancements on TRS (e.g. periodicities) Note. If there is no consensus on Alt2, Alt1 is the default outcome

In this contribution, we provide our views on the CSI enhancement in the following three areas.

- Type-II codebook refinement for Coherent Joint Transmission (CJT) for Multi-TRP (mTRP)

- Type-II codebook refinement for high/medium UE velocities exploiting time-domain correlation/Doppler-domain information

- UE reporting of time-domain channel properties (TDCP) measured via CSI-RS for tracking (TRS)

# Type-II codebook refinement for CJT

Figure below illustrates one example of the Coherent Joint Transmission (CJT) from 4 TRPs. Each TRP is configured with 2x2 linear antenna array in which there are 2 V-Pol and 2 H-Pol antenna elements per vertical and per horizontal direction.

Figure 1 Illustration of Coherent Joint Transmission (CJT) from 4 TRP

One remaining issue from the last RAN1#112 meeting is the detailed frequency basis selection for mode-1 among 3 alternatives quoted below,

Agreement On the Type-II codebook refinement for CJT mTRP, for mode-1, down select (in RAN1#112) only one from the following schemes - Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: where is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource with , and is commonly selected across N CSI-RS resources - Alt2. independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset) For all the above alternatives, the legacy FD basis selection indication scheme is applied on each selected FD basis. Note: Per previous agreements, the number of selected FD basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources

It is important to note that, for us, there are at least two things we need to consider ensuring coherent transmission from mTRP

- Different TRP may experience different channel group delay because distance between the UE can be different TRP may be different.

- Different TRP may experience different channel frequency selectivity, i.e., channel delay spread, due to different signal propagation condition such as reflection, etc.

In our view, Alt1 handles the first issue and Alt2 handles the second issue. We have the following proposal,

Proposal 1.1, On the Type-II codebook refinement for CJT mTRP, for mode-1

- Further discuss whether φn can be fraction, similar as oversampling

- If  φn can be fraction, we slightly prefer Alt1., i.e., The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources.

In terms of the UCI omission design, since UE selected the TRPs to be reported in CSI, once UE selects the corresponding TRP, it is desirable to transmit PMI for the selected TRP as much as possible. Therefore, we think it is beneficial to design CSI-RS resource as the highest priority.

Proposal 1.2, On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, we prefer Alt1., i.e., CSI-RS resource is designated the highest priority.

For CBSR design, we have the following proposal,

Proposal 1.3, On the Type-II codebook refinement for CJT mTRP, regarding CBSR,

- Amplitude restriction is CSI-RS resource common.

- Only support hard restriction, do not support soft restriction.

# Type-II codebook refinement exploiting time-domain correlation

We agreed that the CSI reference resource can be used to define the CSI prediction timeline. In NR, for CSI processing timeline, CSI reference resource is defined in Clause 5.2.2.5 in TS38.214 [3]. Conceptually, CSI reference resource is the latest/last slot that UE can receive CSI measurement resource to report the CSI measurement result in the corresponding CSI report.

For aperiodic CSI report with periodic or semi-persistent measurement resource, CSI reference resource is defined based on Z’ defined in Clause 5.4 in TS38.214 [3], quoted below

- where for aperiodic CSI reporting, if the UE is indicated by the DCI to report CSI in the same slot as the CSI request, nCSI_ref is such that the reference resource is in the same valid downlink slot as the corresponding CSI request, otherwise nCSI_ref is the smallest value greater than or equal to , such that slot n- nCSI_ref corresponds to a valid downlink slot, where Z' corresponds to the delay requirement as defined in clause 5.4. - when periodic or semi-persistent CSI-RS/CSI-IM or SSB is used for channel/interference measurements, the UE is not expected to measure channel/interference on the CSI-RS/CSI-IM/SSB whose last OFDM symbol is received up to Z' symbols before transmission time of the first OFDM symbol of the aperiodic CSI reporting.

To determine Z’, in Clause 5.4 in TS38.214, SCS is needed for the table lookup. In the current specification, the SCS is determined based on the SCS among aperiodic CSI triggering DCI, PUSCH carrying aperiodic CSI and aperiodic NZP-CSI-RS for CSI measurement, quoted below

- µ of table 5.4-1 and table 5.4-2 corresponds to the min (µPDCCH, µCSI-RS, µUL) where the µPDCCH corresponds to the subcarrier spacing of the PDCCH with which the DCI was transmitted and µUL corresponds to the subcarrier spacing of the PUSCH with which the CSI report is to be transmitted and µCSI-RS corresponds to the minimum subcarrier spacing of the aperiodic CSI-RS triggered by the DCI

However, aperiodic CSI can also be triggered with periodic or semi-persistent measurement resource, the SCS of which is not considered in the minimum SCS determination. To resolve the issue, we propose the following clarification

Proposal 2.1, Clarify that, for aperiodic CSI report with periodic or semi-persistent measurement resource, to determine the SCS for Z', SCS of all associated periodic or semi-persistent measurement resource shall be considered in the minimum operation.

In RAN1#112, we agreed to support X=2 CQI, in terms of the CQI encoding, we have the following proposal,

Proposal 2.2, For Type-II codebook refinement exploiting time-domain correlation, in terms of the CQI report, when X=2 CQIs are reported

- Each CQI is independently encoded for both the wideband and subband.

- Both CQIs are encoded in CSI part 1

In terms of bitmap indication of the NZC location, since N4 should be typically small, so Q  is even smaller, it is better to have independent bitmap for SD-FD or different selected DD basis.

Proposal 2.3, For Type-II codebook refinement exploiting time-domain correlation, in terms of the bitmap indication of the location of the  NZCs, we prefer Alt1. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design i.e. the size of the bitmap for each selected DD basis vector is 2LMv.

# UE reporting of TDCP measured via TRS

In RAN1#112, we agreed to support channel time domain correlation for TDCP. Regarding the priority of the TDCP CSI report, we have the following proposal,

Proposal 3.1, For the Rel-18 TRS-based TDCP reporting, the priority of the CSI report(s) associated with TDCP reporting is lower than other CSI reports, i.e., CSI reports not carrying L1-RSRP or L1-SINR.

# Conclusion

In contribution, we provide our views on the CSI enhancement in the following three areas for Rel-18 MIMO evolution

- Type-II codebook refinement for Coherent Joint Transmission (CJT) for Multi-TRP (mTRP)

- Type-II codebook refinement for high/medium UE velocities exploiting time-domain correlation/Doppler-domain information

- UE reporting of time-domain channel properties (TDCP) measured via CSI-RS for tracking (TRS)

We have the following proposals

Type-II codebook refinement for CJT

Proposal 1.1, On the Type-II codebook refinement for CJT mTRP, for mode-1

- Further discuss whether φn can be fraction, similar as oversampling

- If  φn can be fraction, we slightly prefer Alt1., i.e., The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources.

Proposal 1.2, On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, we prefer Alt1., i.e., CSI-RS resource is designated the highest priority.

Proposal 1.3, On the Type-II codebook refinement for CJT mTRP, regarding CBSR,

- Amplitude restriction is CSI-RS resource common.

- Only support hard restriction, do not support soft restriction.

Type-II codebook refinement exploiting time-domain correlation

Proposal 2.1, Clarify that, for aperiodic CSI report with periodic or semi-persistent measurement resource, to determine the SCS for Z', SCS of all associated periodic or semi-persistent measurement resource shall be considered in the minimum operation.

Proposal 2.2, For Type-II codebook refinement exploiting time-domain correlation, in terms of the CQI report, when X=2 CQIs are reported

- Each CQI is independently encoded for both the wideband and subband.

- Both CQIs are encoded in CSI part 1

Proposal 2.3, For Type-II codebook refinement exploiting time-domain correlation, in terms of the bitmap indication of the location of the  NZCs, we prefer Alt1. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design i.e. the size of the bitmap for each selected DD basis vector is 2LMv.

UE reporting of TDCP measured via TRS

Proposal 3.1, For the Rel-18 TRS-based TDCP reporting, the priority of the CSI report(s) associated with TDCP reporting is lower than other CSI reports, i.e., CSI reports not carrying L1-RSRP or L1-SINR..

# Reference

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, 3GPP TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 - 17, 2021

[2] Chairman Notes, 3GPP TSG RAN WG1 #112, Athens, Greece, February 27th – March 3rd  2023

[3] 3GPP TS 38.214,Physical layer procedures for data, V15.16.0 (2022-03)