3GPP TSG RAN WG1 #112bis-e		R1-2304389

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.1.1.1

Source: 	Moderator (MediaTek Inc.)

Title: 	Moderator summary on extension of unified TCI framework (Round 0)

Document for:	Discussion and Decision

- Introduction

In RAN#94e, the Rel-18 WID of MIMO evolution for downlink and uplink is approved. In the approved WID, extension of unified TCI framework is a part of the RAN1 objectives, and the detailed scope of this agenda item (AI 9.1.1.1) includes the following highlighted objectives:

RAN1: - Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. - Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

- Plan

Based on the contributions from companies [1]-[32], the followings are provided in this document:

- Summary of companies’ views on each of open issues raised by interested companies, where the open issues are categorized as follow:

  - Issue 1 – General issue for unified TCI extension

  - Issue 2 – TCI state update and activation

  - Issue 3 – How to inform UE which indicated TCI state(s) that UE shall apply to target channel/signal

  - Issue 4 – UL power control for UL MTRP operation

  - Issue 5 – PDSCH-CJT Tx scheme

  - Issue 6 – Beam failure recovery

- Observations and recommended proposals based on the summary of companies’ views

- Contact Person

For potential offline discussion, companies/delegates are encouraged to enter the contact information in the table below:

Table 0 Contact Information

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |
| Apple | Hong | hhe5@apple.com |
| CEWiT | Vishakha Singh | Vish@cewit.org.in |
| CMCC | Yan | liyanwx@chinamobile.com |
| Ericsson | Claes | Claes.tidestav@ericsson.com |
| FGI | Cubie | wanchen.lin@fginnov.com |
| Fraunhofer IIS/HHI | Sutharshun | sutharshun.varatharaajan@iis.fraunhofer.de |
| Fujitsu | Jian | zhangjian1288@fujitsu.com |
| Futurewei | Zhigang | zrong@futurewei.com |
| Google | Alex | alexliou@google.com |
| Huawei, HiSilicon | Keyvan | Keyvan.zarifi@huawei.com |
| Hyundai | Jeongsu Lee | Jeongsu.lee@hyundai.com |
| Intel | Avik | avik.sengupta@intel.com |
| InterDigital | Jonghyun | jonghyun.park@interdigital.com |
| Lenovo | Bingchao Liu | liubc2@lenovo.com |
| LG | Jaehoon | jhoon.chung@lge.com |
| MediaTek | Darcy | darcy.tsai@mediatek.com |
| MediaTek | Rebecca | rebecca.chen@mediatek.com |
| NEC | Peng | guan_peng@nec.cn |
| NTT DOCOMO | Yuki | yuuki.matsumura.vz@nttdocomo.com |
| NTT DOCOMO | Weiqi | sunwq@docomolabs-beijing.com.cn |
| OPPO | Jeffrey | caojianfei@oppo.com |
| Panasonic | Khalid | khalid.zeineddine@eu.panasonic.com |
| Qualcomm | Yan | yanzhou@qti.qualcomm.com |
| Samsung | Dalin | dalin.zhu@samsung.com |
| Sharp | Taka | fukui.takahisa@sharp.co.jp |
| Spreadtrum | Qiyishu Li | qiyishu.li@unisoc.com |
| vivo | Yang | songyang@vivo.com |
| Xiaomi | Mingju LI | limingju@xiaomi.com |
| ZTE | Bo | gao.bo1@ZTE.com.cn |

- Proposal to be discussed in the online session

Proposal 3.3.A (Both): On unified TCI framework extension for S-DCI based MTRP, for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured w/o the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the scheduled/activated PDSCH reception

- If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2, above applies regardless of the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception

- If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above applies when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold

Support/fine: Xiaomi, OPPO, ZTE, Futurewei, Docomo (SFN), MTK, Lenovo, Nokia, QC, Fujitsu, Intel, Huawei/HiSilicon, Fraunhofer, CMCC, ZTE, Spreadtrum

Concern: Sharp, FGI, vivo, LG, Panasonic, Ericsson

Proposal 3.3.B (RRC): On unified TCI framework extension for S-DCI based MTRP, for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured w/o the [TCI selection field], using RRC configuration to inform that the UE shall apply the first, the second, or both indicated joint/DL TCI states to the scheduled/activated PDSCH reception

- If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2, above applies regardless of the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception

- If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above applies when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold

Support/fine: OPPO, Apple, Futurewei, Huawei/HiSilicon, Sharp, NEC, MTK, Docomo (non-SFN), Ericsson, Panasonic, Intel, FGI, TransHold, ITRI

Concern: vivo, QC, Xiaomi, Nokia, LG, CMCC, Fujitsu, CATT, IDC, ZTE

Proposal 5.1.A: On unified TCI framework extension for S-DCI based MTRP, the following three alternatives are supported for PDSCH-CJT applying both indicated joint TCI states (if the UE supports two indicated joint/DL states for PDSCH-CJT):

- Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA

- Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state

- Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB

Introduce a UE capability on which alternative(s) is supported, and either one of above alternatives can be configured by RRC according to the UE capability

Support: ZTE, Huawei/HiSilicon, Docomo, Ericsson, Lenovo, Nokia, Samsung, OPPO, CMCC

Concern: QC

Proposal 3.1: On unified TCI framework extension for S-DCI based MTRP:

- If a CORESET other than a CORESET with index 0 is associated only with USS sets and/or Type3-PDCCH CSS sets, the CORESET is configured by RRC to apply the first one, the second one, or both of the indicated joint/DL TCI states to PDCCH reception on the CORESET

- If a CORESET other than a CORESET with index 0 is associated at least with CSS sets other than Type3-PDCCH CSS sets, the CORESET is configured by RRC to apply the first one, the second one, both, or none of the indicated joint/DL TCI states to PDCCH reception on the CORESET

- For a CORESET with index 0:

  - If the CORESET is associated with SS#0 for Type 0/0A/2 CSS sets, the CORESET is configured by RRC to apply the first one, the second one, or none of the indicated joint/DL TCI state to PDCCH reception on the CORESET

  - Otherwise, the CORESET is configured by RRC to apply the first one, the second one, both, or none of the indicated joint/DL TCI states to PDCCH reception on the CORESET

Note: RAN1 already agrees to use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the indicated joint/DL TCI states to a CORESET in S-DCI based MTRP.

Note: There is no consensus in RAN1 on whether to reuse the Rel-17 RRC parameter followUnifiedTCIstate as a part of above RRC configuration, and whether to reuse followUnifiedTCIstate is up to RAN2 design

Question: Whether to use the codepoint “11” of the [TCI selection field]? Yes: Spreadtrum, vivo, ZTE, Hyundai, LG, Docomo No (reserved): Panasonic, OPPO, ITRI, QC, Nokia, Futurewei, Fujitsu, Ericsson, Huawei/HiSilicon

Proposal 3.2: On unified TCI framework extension for S-DCI based MTRP, when a 2-bit [TCI selection field] is configured by RRC to be present in a DCI format 1_1/1_2 in a DL BWP:

- If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to PDSCH reception scheduled/activated by the DCI format 1_1/1_2 based on the Rel-16 rules for mapping legacy TCI states to PDSCH transmission occasions, CDM groups, or non-overlapping frequency domain resource allocations by replacing the first and the second indicated legacy TCI states with the first and the second indicated joint/DL TCI states, respectively

- The codepoint "11" of the [TCI selection field] is reserved

- If the DCI format 1_1/1_2 indicates codepoint "11" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to PDSCH reception scheduled/activated by the DCI format 1_1/1_2 based on the Rel-16 rules for mapping legacy TCI states to PDSCH transmission occasions, CDM groups, or non-overlapping frequency domain resource allocations by replacing the first and the second indicated legacy TCI states with the second and the first indicated joint/DL TCI states, respectively

Proposal 2.3: On unified TCI framework extension, the following cases for CA operation are supported:

- A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP and CC(s) operating in S-DCI based MTRP

  - FFS: How to support common TCI state ID activation/update for this case

- A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP and CC(s) operating in M-DCI based MTRP

  - FFS: How to support common TCI state ID activation/update for this case

Note: “A CC operates in STRP” for above means a CC operates in Rel-17 unified TCI framework

Note: “A CC operates in S/M-DCI based MTRP” for above means a CC operates in Rel-18 unified TCI framework extension for S/M-DCI based MTRP operation

Support: IDC, Xiaomi, OPPO, vivo, Futurewei, Nokia, Docomo, ZTE, Fujitsu, Docomo, Apple, CATT, Huawei/HiSilicon

Concern: QC (Case 1), NEC (Case 1), CMCC, Samsung, Panasonic, Ericsson, Fraunhofer

Proposal 3.5: On unified TCI framework extension for S-DCI based MTRP, when two indicated joint/UL TCI states are applied to a PUSCH transmission

- For SDM and SFN based PUSCH Tx schemes, the UE shall apply the first indicated joint/UL TCI state to the PUSCH antenna port(s) associated with the first SRS resource set, and the second indicated joint/UL TCI state to the PUSCH antenna port(s) associated with the second SRS resource set, respectively.

- Note: The association between PUSCH antenna port(s) and an SRS resource set is discussed and defined in STxMP AI

Proposal 3.6: On unified TCI framework extension for S-DCI based MTRP, when two indicated joint/UL TCI states are applied to a PUCCH resource/resource group:

- For TDM based PUCCH Tx scheme, the UE shall apply two indicated joint/UL TCI states to repetitions of the PUCCH transmission corresponding to the PUCCH resource/resource group based on the Rel-17 rules for mapping spatial settings to the repetitions by replacing the first and second spatial settings with the first and second indicated joint/UL TCI states, respectively.

- For SFN based PUCCH Tx scheme, the UE shall apply two indicated joint/UL TCI states to the PUCCH transmission corresponding to the PUCCH resource/resource group

- Discussion

- Issue 1 – General issue for unified TCI extension

Table 1-1 Summary for Issue 1

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 1.1 | Support of inter-cell S-DCI based MTRP operation | Question: In Rel-18 unified TCI framework extension, whether to support inter-cell S-DCI based MTRP? - Yes: Samsung, Apple (if time permits), Intel (open), FGI (if time permits) - No: vivo, QC, CMCC, OPPO, Xiaomi, Google, Nokia, CMCC, ZTE, Spreadtrum, Panasonic, Futurewei, Huawei/HiSilicon, Sharp, NEC, Fujitsu, CATT, Docomo, Lenovo, IDC, CEWiT FL note: Based on feedback from companies, it seems majority think it is not proper to introduce new MTRP scheme in this AI at this moment, and I will not recommend any proposal (for conclusion/agreement) in this meeting if situation is not changed. |
| 1.2 | Common beam for PDCCH/PDSCH | Question: In Rel-17 unified TCI framework, it can be guaranteed that PDCCH and respective PDSCH follow the common beam for DL reception. In Rel-18 unified TCI framework extension, it is possible that PDCCH and respective PDSCH follow different beams for DL reception if they apply different indicated joint/DL TCI states. Then, whether specification should restrict that two indicated joint/DL TCI states must be associated with different TRPs, i.e., following different beams for PDCCH and PDSCH is only allowed for MTRP operation? - Yes: Samsung, NEC, ZTE - No: Google, Futurewei, CATT, vivo, Panasonic, Panasonic, Ericsson, Lenovo, OPPO, IDC - Not critical: Docomo, Huawei/HiSilicon, QC, Apple, CMCC |

Table 1-2 Company input for Issue 1

| Company | Input to Round 0 summary |
| --- | --- |
| Mod V00 | Please share your further input to the questions in Issue 1.1 and 1.2, if any. |
| Nokia | Issue 1.1: We share the same view as FL that it is not proper to introduce new MTRP scheme at this moment. Issue 1.2: We don’t see the need for the restriction. |
| QC | For 1.1, agree with FL’s assessment. For 1.2, the issue seems not critical |
| Futurewei | Issue 1.1: Agree with FL’s assessment. Issue 1.2: Our view is that the restriction is unnecessary. |
| vivo | Same view as above companies. |
| NEC | Issue 1.1: Agree with FL’s assessment. Issue 1.2: We are supportive to have a method to guarantee common beam operation for PDCCH/PDSCH for the same TRP, we don’t think it is a restriction. |
| OPPO | Issue 1.1: Agree with FL’s assessment. Issue 1.2: Consider the same issue in Rel.17 (CORESET configured not to follow indicated DL/joint TCI state for STRP), we still think it’s not necessary to introduce such restriction in Rel.18 for MTRP. |
| Xiaomi | Issue 1.2: We are not clear about the problem. Does it mean there is a problem in S-DCI based MTRP or in STRP? In S-DCI based MTRP, the combination of STRP PDCCH and MTRP PDSCH, and MTRP PDCCH and STRP PDSCH are supported. In these combinations, PDCCH and PDSCH will follow different indicated joint/DL TCI states. While in STRP scenario, there is only one indicated joint/DL TCI state, PDCCH and PDSCH cannot follow different indicated joint/DL TCI states. |
| IDC | Issue 1.1: Agree with FL’s assessment. Issue 1.2: Such restriction is not needed for Rel-18, as the Rel-18 UTCI framework is already an extension from Rel-17. |
| ZTE | For 1.1: We agree with FL’s assessment. For 1.2: Support. |
| LG | Issue 1.1: Agree with FL’s assessment Issue 1.2: It seems that the restriction is not needed. |
| Fujitsu | Issue 1.1: Agree with FL’s assessment. Issue 1.2: Our view is that the restriction is unnecessary. We think it is a natural result of Rel-18 extension from Rel-17 which does not occur in legacy TCI framework. |
| Docomo | Issue 1.1: Agree with FL’s assessment. It’s not critical. Issue 1.2: It’s not critical. The common beam between scheduling PDCCH and scheduled PDSCH is not always true in Rel.17. If CORESET does not follow indicated TCI, and if PDSCH is scheduled with < timeDurationForQCL, there is no common beam between scheduling PDCCH and scheduled PDSCH (QCL of PDSCH is indicated TCI for intra-cell, or R15 default QCL for inter-cell). Hence, we think this discussion is not critical. |
| Apple | Issue 1.1: Although we are open for inter-cell sDCI-based mTRP if time allows, FL’s assessment on current situation is fairly true and we are ok with this. Issue 1.2: Not critical. |
| CMCC | Issue 1.1: Agree with FL’s assessment. Issue 1.2: The restriction is not needed. |
| Samsung | Issue 1.1: As we commented before, higher layer parameter TCI-State can include a PCI index, hence a TCI codepoint can comprise a TCI state associated to a PCI (either the serving cell PCI or a PCI other than the serving cell PCI). That is, if there is no further restriction, RAN1 specification can already support the association between an indicated TCI state and a PCI for SDCI meaning that this is not a new feature. Would appreciate it if companies can share their views regarding this. Issue 1.2: In RAN1#112bis-e, we have agreed to indicate, in the TCI state activation MAC CE, whether a TCI state of a TCI codepoint corresponds to the “first” or “second” TCI state so that the UE can perform the corresponding full-set or sub-set based TCI state(s) update. The TCI state signaling for SDCI, however, is still incomplete as the current RAN1 agreements did not say that the “first” and “second” TCI states of a TCI codepoint should be respectively from two different TRPs. When the “first” and “second” TCI states of a TCI codepoint are associated to the same TRP, the common beam principle of unified TCI framework no longer holds. @Xiaomi: what you have described “In S-DCI based MTRP, the combination of STRP PDCCH and MTRP PDSCH, and MTRP PDCCH and STRP PDSCH are supported. In these combinations, PDCCH and PDSCH will follow different indicated joint/DL TCI states.” can already be supported by RAN1 agreements as long as the two indicated TCI states are respectively from the two TRPs (LHS of the above figure). But as explained above, the current RAN1 agreements do not exclude that the two indicated TCI states can be from the same TRP (RHS of the above figure) – hence per TRP common beam principle is violated for such settings. @Oppo, Docomo: it is not our intention to enforce CORESETs that do not follow indicated TCI state in Rel-17 to start following indicated TCI states in Rel-18 and sharing common beams with the scheduled PDSCH. Rule(s)/configuration(s) of following or not following should be retained from Rel-17, and this issue is being discussed under 3.1 – we are actually fine with proposal 3.1. It is indeed better to clarify that the CORESETs we are considering here should be those following the indicated TCI state(s) as in Rel-17 unified TCI framework. |
| FGI | Issue 1.1: Agree that this is a low priority issue. Issue 1.2: The restriction is not needed. |
| Huawei/HiSilicon | Issue 1.1: Agree with FL. Issue 1.2: We still think it is not a critical issue. If most companies prefer to discuss it, we are open to discuss it too. |
| Fraunhofer IIS/HHI | Issue 1.1: Fine with FL assessment. Issue 1.2: This issue does not seem critical. |
| Google | Issue 1.1: Agree with FL Issue 1.2: We hold the same view |
| Ericsson | Issue 1.1: In our understanding, this is already supported: what is needed is an RRC parameter and a UE feature. Any limitation seems artificial. Issue 1.2: We do not see why this would be beneficial: it is up to NW configuration |
| CATT | Same view as Nokia, QC and some other companies. |
| Lenovo | Issue 1.1: Agree with FL. Issue 1.2: The restriction is unnecessary |
| Spreadtrum | Issue 1.1: Agree with FL Issue 1.2: Not critical. |
| CEWiT | Issue 1.1: Agree with FL’s assessment. |

- Issue 2 – TCI state update and activation

Table 2-1 Summary for Issue 2

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 2.1 | (S-DCI) How to configure/determine that a CC is operated in Rel-18 unified TCI framework extension for S-DCI based MTRP? | Question: How to configure/determine that a CC is operated in Rel-18 unified TCI framework extension for S-DCI based MTRP: - Alt1: A CC is operated in Rel-18 unified TCI framework extension for S-DCI based MTRP if an Rel-18 TCI state activation command (MAC-CE) for S-DCI based MTRP operation that activates at least one TCI codepoint mapped with more than one join TCI states, more than one DL TCI states, or more than one UL TCI states is received and applied in the CC - If all joint/DL/UL TCI states activated by the TCI state activation command are the first joint/DL/UL TCI states or the second joint/DL/UL TCI states, the UE shall fallback to Rel-17 unified TCI framework in the CC - Alt2: A CC is operated in Rel-18 unified TCI framework extension for S-DCI based MTRP if an Rel-18 TCI state activation command (MAC-CE) for S-DCI based MTRP operation (which is different from the TCI state activation command (MAC-CE) used for Rel-17 unified TCI framework) is received and applied in the CC Support Alt1: ZTE, Intel, Huawei/HiSilicon, Lenovo, Xiaomi, Futurewei, Fujitsu, Docomo, Sharp, QC, CEWiT Support Alt2: vivo, FGI, OPPO, Apple, IDC FL note: Given the majority view on this issue, the following proposal is recommended: Proposal 2.1: On unified TCI framework extension for S-DCI based MTRP: - A CC/BWP is operated in Rel-18 unified TCI framework extension for S-DCI based MTRP if the UE receives and applies an Rel-18 TCI state activation command (MAC-CE) for S-DCI based MTRP operation that activates at least one TCI codepoint mapped with more than one join TCI states, more than one DL TCI states, or more than one UL TCI states to the CC/BWP - A CC/BWP is operated in Rel-17 unified TCI framework if the UE receives and applies an Rel-18 TCI state activation command (MAC-CE) for S-DCI based MTRP operation in which all activated joint/DL/UL TCI states are the first joint/DL/UL TCI states or the second joint/DL/UL TCI states to the CC/BWP Support: Nokia, QC, Futurewei, Xiaomi, Docomo, CMCC, ZTE, Fujitsu, Docomo, Apple, Samsung, CATT, Google, Fraunhofer, Spreatrum, CEWiT Concern: vivo, NEC (2nd bullet), OPPO, IDC, Panasonic, LG (prefer RRC configured), Ericsson |
| 2.2 | (S-DCI) After a TCI activation MAC-CE is received and applied by the UE, if more than one TCI codepoints are activated, default behavior before the UE receives and applies the first beam indication DCI | Question: After an Rel-18 TCI state activation command (MAC-CE) for S-DCI based MTRP operation is received and applied by the UE, if more than one TCI codepoints are activated by the TCI state activation command, whether to introduce a default behavior (e.g., the UE shall apply the lowest TCI codepoint among the activated TCI codepoints mapping with more than one join TCI states, more than one DL TCI states, or more than one UL TCI states) before the UE receives and applies the first beam indication DCI? Yes: Intel, Xiaomi, Lenovo, QC, NEC, Nokia, Panasonic, Fraunhofer, FGI No: Apple, OPPO, Futurewei, CMCC, ZTE, Docomo, Samsung, CATT, Ericsson, Google, Huawei/HiSilicon |
| 2.3 | (CA) Common TCI state ID activation/update for a CC list comprised of a mix of STRP CC(s) and MTRP CC(s) | Question: Whether to support each of the following cases for common TCI state ID activation/update: - Case 1: A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP and CC(s) operating in S-DCI based MTRP - Case 2: A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP and CC(s) operating in M-DCI based MTRP - Case 3: A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in S-DCI based MTRP and CC(s) operating in M-DCI based MTRP - Case 4: A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP, CC(s) operating in S-DCI based MTRP, and CC(s) operating in M-DCI based MTRP Note: A CC operates in STRP means a CC operates in Rel-17 unified TCI framework Note: A CC operates in S/M-DCI based MTRP means a CC operates in Rel-18 unified TCI framework extension for S/M-DCI based MTRP Support Case 1: ZTE, Spreadtrum, CATT, Fujitsu, Xiaomi, Huawei/HiSilicon, Lenovo, Google, Apple, OPPO, IDC Support Case 2: ZTE, vivo, MTK, Spreadtrum, CATT, Xiaomi, Huawei/HiSilicon, Lenovo, Google, Apple, OPPO, IDC Support Case 3: CATT, Xiaomi, Lenovo, Google, OPPO Support Case 4: CATT, Lenovo, LG Not support all of them: Ericsson, Qualcomm, Panasonic FL note: Given that Case 1 and Case 2 are supported by many companies, the following proposal is recommended. For proponents of Case 1 and/or Case 2, it would be appreciated if you can provide the corresponding solutions for the FFS on how to support Case 1 and/or Case 2. Proposal 2.3: On unified TCI framework extension, the following cases for CA operation are supported: - A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP and CC(s) operating in S-DCI based MTRP - FFS: How to support common TCI state ID activation/update for this case - A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP and CC(s) operating in M-DCI based MTRP - FFS: How to support common TCI state ID activation/update for this case Note: “A CC operates in STRP” for above means a CC operates in Rel-17 unified TCI framework Note: “A CC operates in S/M-DCI based MTRP” for above means a CC operates in Rel-18 unified TCI framework extension for S/M-DCI based MTRP operation Support: IDC, Xiaomi, OPPO, vivo, Futurewei, Nokia, Docomo, ZTE, Fujitsu, Docomo, Apple, CATT, Huawei/HiSilicon Concern: QC (Case 1), NEC (Case 1), CMCC, Samsung, Panasonic, Ericsson, Fraunhofer |

Table 2-2 Company input for Issue 2

| Company | Input to Round 0 summary |
| --- | --- |
| Mod V00 | - Please check Proposal 2.1 and 2.3 and share your comment, if any. - Please also provide your view on the question in Issue 2.2 |
| Nokia | Proposal 2.1: Support Proposal 2.2: We support defining a default behaviour. Issue 2.3: Support Case 1 and Case 2. Proposal 2.3: Support |
| QC | For proposal 2.1, support For proposal 2.2, support For proposal 2.3, do not support. It does not work without any additional signaling to differentiate sTRP CC and sDCI mTRP CC, which cannot coexist simultaneously. Because if UE receives one MAC-CE activating at least one codepoint mapped to 2 joint TCIs, then all CCs will become sDCI mTRP CCs. In other words, all CCs must be all sDCI mTRP CCs or all sTRP CCs depending on the MAC-CE activated TCI codepoints. |
| Futurewei | Proposal 2.1: Support. Issue 2.2: It is unclear to us the need to introduce such a default behavior. Proposal 2.3: Support. |
| vivo | Proposal 2.1: Don’t support. If switching to STRP mode is dependent on all TCI codepoint is mapped to one joint/DL/UL TCI state, the indication in the above MAC CE is not needed. In this sense, the proposal contradicts the previous agreement that “TCI state activation command (MAC-CE) should indicate that each joint/DL/UL TCI state mapped to a TCI codepoint is the first or second joint/DL/UL TCI state”. Based on the previous agreement, following MAC CE should be regarded as the MAC CE to activate TCI states for S-DCI based MTRP. Codepoint 1: {joint TCI state#1 to update the 1st joint TCI state} Codepoint 2: {joint TCI state#2 to update the 1st joint TCI state} Codepoint 3: {joint TCI state#3 to update the 2nd joint TCI state} Codepoint 4: {joint TCI state#4 to update the 2nd joint TCI state} Issue 2.2: We think the behaviour can be discussed after we have an agreement on issue 2.1. Proposal 2.3: Fine. |
| NEC | Proposal 2.1: Support the first bullet, we have concerns on the second bullet since we don’t know the design of Rel-18 TCI state activation command (MAC-CE) and whether this new MAC CE could provide sufficient and correct information for Rel-17 unified TCI framework. Issue 2.2: Support to have a default behavior. The example, on the other hand, mandates that UE should always support more than one default, which we don’t support, because whether UE supports more than one default beam shall be based on UE capability. Proposal 2.3: same question as QC if the second bullet of proposal 2.1 is supported. |
| OPPO | Issue 2.1: On Alt1, we think there could be an issue introduced by partial TCI state updated which was agreed in RAN1#112bis-e. That seems similar to the issue as mentioned by vivo. Specifically, if UE already maintains/stores a codepoint with two joint/DL/UL TCI states and then all codepoints are updated by Rel-18 MAC CE to single joint/DL/UL TCI state, the codepoint (previously with two joint/DL/UL TCI states) cannot be changed back for STRP operation, since UE always maintains one additional TCI state. How should we address it, if it is an issue? Issue 2.2: No need. UE could trust NW to send MAC CE activating more than 1 codepoints of joint/DL/UL TCI state(s) followed by indicated one in DCI format 1_1/1_2. Moreover, the case as mentioned by FL would make UE to always get ready for MTRP operation, while NW may intend to STRP operation only. Proposal 2.3: Support. |
| Xiaomi | Proposal 2.1: We are generally fine with this proposal. And suggest the following update Proposal 2.1: On unified TCI framework extension for S-DCI based MTRP: - A CC/BWP is operated in Rel-18 unified TCI framework extension for S-DCI based MTRP if the UE receives and applies an Rel-18 TCI state activation command (MAC-CE) for S-DCI based MTRP operation that activates at least one TCI codepoint mapped with more than one joint TCI states, more than one DL TCI states, or more than one UL TCI states to the CC/BWP - A CC/BWP is operated in Rel-17 unified TCI framework if the UE receives and applies an Rel-18 TCI state activation command (MAC-CE) for S-DCI based MTRP operation that activates no TCI codepoint mapped with more than one joint TCI states, more than one DL TCI states, or more than one UL TCI states in which all activated joint/DL/UL TCI states are the first joint/DL/UL TCI states or the second joint/DL/UL TCI states to the CC/BWP Issue 2.2 Support to introduce a default behavior Proposal 2.3: Support this proposal. While for case 1, the S-DCI based MTRP CC will be included in two CC lists. As for the MAC CE, there are two options. Option 1 is to use one MAC CE to activate the TCI states for S-DCI based MTRP CC. With Option 1, additional RRC configuration is needed to configure whether the first joint/DL/UL TCI states or the second joint/DL/UL TCI states will be applied for the STRP CC(s) in each CC list. Option 2 is to use two MAC CE, one to activate the TCI states for STRP CC in CC list 1 and the other one to activate the TCI states for STRP CC in CC list 2. With Option 2, the activated TCI states in both two MAC CEs will be applied in S-DCI based MTRP CC without any additional signaling. While for case 2, it is similar as case 1. But the RRC configuration will configure whether the activated TCI states for CORESETPoolindex#0 or for CORESETPoolindex#1 will be applied for the STRP CC(s) in each CC list. |
| IDC | Proposal 2.1: Not support. Alt2 is sufficient and clear, whereas Alt1 seems to have a mixed-up interpretation across codepoints of TCI field activated by Rel-18 MAC-CE command, which we fail to see why we need to consider such special way of switching to Rel-17. If no agreement is made, we think Alt2 is already the current baseline, so no further agreement is also fine to us. Proposal 2.3: Support. |
| Mod V09 | Proposal 2.3: Can proponents of Case 1 (STRP+S-DCI MTRP) response to the concern from QC? QC: “Because if UE receives one MAC-CE activating at least one codepoint mapped to 2 joint TCIs, then all CCs will become sDCI mTRP CCs. In other words, all CCs must be all sDCI mTRP CCs or all sTRP CCs depending on the MAC-CE activated TCI codepoints.” |
| ZTE | Proposal 2.1: Support. We prefer to reuse the legacy rule. Proposal 2.2: No. Our views have been provided well last meeting. Proposal 2.3: Support. - Regarding QC’s comments, if our understanding is correct, we may handle this interpretation issue (i.e., which one of two TCI states are used) together. In our views, we may have “first TCI state” in S-DCI and “TCI state corresponding to lowest CORESETPoolID” is used for sTRP operation. |
| LG | Proposal 2.1: Not support. The necessity to switch between Rel-17 and Rel-18 unified TCI framework dynamically is unclear and need to be clarified since Rel-18 unified TCI framework can support STRP and MTRP operation altogether. Regarding the alternatives, the issue is mainly due to the M/N configuration by RRC per CC. While a certain CC is configured with M=N=1 (i.e. operating STRP in the CC), it can be addressed in Rel-18 unified TCI framework by associating the 1st or 2nd indicated TCI state to the CC regardless the details of MAC-CE design such as Alt1 and Alt2 above. In other words, Rel-18 operation on a CC configured with M=N=1 is functionally identical to Rel-17 unified TCI operation, thus one of the two indicated TCI states in Rel-18 can be used for supporting STRP operation in this case. Proposal 2.3: Update our view in the above table (Case 4) To mix CC(s) operating in S-DCI based MTRP and M-DCI based MTRP, it is possible to determine the number of indicated TCI states based on the largest M/N value among CCs in a CC list and to allow different M/N configuration across CCs. Then, implicit/explicit mapping between the indicated TCI state(s) and each CC can address the issue of mixing the corresponding CCs in a same CC list. For example, when the M/N configuration for CC1 is {M=2, N=2, S-DCI based}, CC2 is {M=2, N=2, M-DCI based} and CC3 is {M=N=1, STRP}, the number of indicated TCI states applying common TCI state update can be {M_common=2, N_common=2}. For CC1, each TCI state is associated to each TO for mTRP/multi-panel transmission. For CC2, each TCI state is associated to each CORESET pool. For CC3, only one of the two DL/UL TCI states is applied for sTRP operation. In this way, configuration and signaling overhead for beam indication will be significantly reduced. |
| Fujitsu | Proposal 2.1: Support Proposal 2.3: Support |
| Docomo | Proposal 2.1: Support. Regarding to the necessity of switching between R17 and R18 depends on the [TCI selection field] becomes mandatory feature of R18 unified TCI. In previous releases, DCI based switching of sTRP and mTRP was separate UE capability. In that case, if proposal 2.1 is not agreed, only option is to use RRC to switch between sTRP and mTRP. We have concern to rely on RRC for the switching, and we believe MAC CE level switching between sTRP and mTRP should be supported same as previous releases. Regarding to the comparison between Alt.1 and Alt.2, Alt.2 requires to send Rel.18 MAC CE to deactivate all TCI codepoint and send Rel.17 MAC CE to activate all TCI codepoint. On the other hand, Alt.1 requires one Rel.18 MAC CE. Hence, Alt.1 is more reasonable. Issue 2.2: We don’t see the necessity. What is a problem for UE to wait DCI indication after MAC CE activation? Proposal 2.3: Support. Re QC’s comment, we agree with ZTE. For a CC configured with sTRP will can select one TCI from the two indicated TCIs. |
| Apple | Proposal 2.1: Fine for progress to go with majority view. We do not see big difference between Alt.1 and Alt.2 honestly. Issue 2.2: We do not see the necessity to define default behaviour, assuming either Alt.1 or Alt.2 would be taken for P2.1 and UE knows the mTRP mode accordingly. Then, even with a single TCI-state, UE knows which TCI-state of the full set is to be updated by the single TCI-state. It is almost same as in Rel-17 uTCI where a single DL TCI-state is indicated by TCI field without a UL TCI-state. There is no default behaviour defined in Rel-17. Why we need here? Proposal 2.3: Support. As commented above, concern raised by QCM can be addressed by adding a signaling for sTRP CCs as anyway signaling is needed to indicate which one of <the first, the second> on the sDCI mTRP CC is used for sTRP CC. |
| CMCC | Proposal 2.1: Support. Proposal 2.2: No. In Rel-17, before application of indicated TCI state, the SSB UE identified during the initial access procedure can be used. For M-TRP, before the UE receives and applies the first beam indication DCI, the SSB UE identified during the initial access procedure can be used as legacy scheme. Proposal 2.3: Not support. If a CC list can be comprised of a mix of STRP CC(s) and MTRP CC(s), the freedom to transmit PDCCH/PDSCH from either of the TRP1 or TRP2 will be lost. For example, there are two mixed CC lists corresponding to TRP1 and TRP2: {mDCI CC1, STRP CC2} and {mDCI CC1, STRP CC3}. TCI state for CORESET pool 0 of mDCI CC1 is updated together with STRP CC2, and TCI state for CORESET pool 1 of mDCI CC1 is updated together with STRP CC3. STRP CC2 is always transmitted from TRP1, when CORESET pool 0 is associated to the CC list of {mDCI CC1, STRP CC2}, then the reference RS of TCI state for CORESET pool 0 must be from TRP1, and PDCCH and PDSCH associated to CORESET pool 0 will be ALWAYS transmitted from TRP1 and CANNOT transmitted from TRP2. Especially for the case that TRP1 has PDCCH congestion on the resources associated to CORESET pool 0, TRP2 could not be used for a backup. This will strongly restrict dynamic TRP switching. |
| Samsung | Proposal 2.1: fine Proposal 2.2: do not see a need for default behavior, can be handled by the network. Proposal 2.3: do not support. As mentioned by some companies, additional signaling to support such mixture would be needed here or there, which would dim the claimed benefits of simplification. |
| Panasonic | Issue 2.1: Support Alt 2. We do not support Proposal 2.1. We think there is no need for this extra constraint. It should be totally permissible that Rel-18 MAC-CE activates codepoints each containing a single UL/DL TCI state. Issue 2.2: We are okay with Proposal. Proposal 2.3: We do not support. As per the agreement in RAN1#112be, - A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in S-DCI based MTRP - A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in M-DCI based MTRP we believe that release 18 already supports the case of having on one list, CCs operating in single TRP transmission mode (Rel-18 activation) and CCs operating in multi-TRP transmission mode (Rel-18 activation). In our understanding, the unified TCI framework extension is meant to have one framework for both single TRP transmission and multi-TRP transmission by having new TCI state activation MAC-CE on one hand and by configuring every channel/signal to operate either in single TRP transmission or multi-TRP transmission on the other hand. |
| FGI | Proposal 2.1: For our point of view, using different MAC CE (MAC-CE for Rel-17 sTRP operation and MAC-CE for Rel-18 mTRP operation) to indicate TCI for sTRP operation and mTRP operation is flexible enough. The ambiguity raised by vivo is also needed to be addressed if Rel-18 MAC CE could indicate TCI for sTRP operation. Thus, we don’t see a sensible reason to apply Rel-18 MAC CE for sTRP operation. Issue 2.2: Agree to define the default behavior. Issue 2.3: We agree with QC that the additional signaling might be needed to support these cases, so we would like to add an FFS to study the potential impact on reference CC configuration when a CC list includes CCs operate in sTRP and mTRP as shown in the following: |
| Huawei, HISilicon | Proposal 2.1: We don’t think Alt2 can work as it seems contradictory to assume that a CC is an MTRP CC when TCIs corresponding to only one TRP are activated. We, in principle, agree with the intention behind Proposal 2.1. However, we think that even if every TCI codepoint is mapped to only one joint TCI state or only one pair of UL/DL TCI states, if, additionally, the activated joint/UL/DL TCI states belong to two different TRPs (the activated TCI states corresponding to some codepoints are the first joint/DL/UL TCI states while the activated TCI states corresponding to other codepoints are the second joint/DL/UL TCI states), CC/BWPP still operates in MTRP regime. In fact, any case that is not described by the second bullet of Proposal 2.1 can be considered a MTRP case. Therefore, we suggest the following: Proposal 2.1 (modified): On unified TCI framework extension for S-DCI based MTRP: - A CC/BWP is operated in Rel-18 unified TCI framework extension for S-DCI based MTRP if the UE receives and applies an Rel-18 TCI state activation command (MAC-CE) for S-DCI based MTRP operation that activates at least one TCI codepoint mapped with more than one join TCI states, more than one DL TCI states, or more than one UL TCI states to the CC/BWP - A CC/BWP is operated in Rel-17 unified TCI framework if the UE receives and applies an Rel-18 TCI state activation command (MAC-CE) for S-DCI based MTRP operation in which all activated joint/DL/UL TCI states are the first joint/DL/UL TCI states or the second joint/DL/UL TCI states to the CC/BWP, CC/BWP is operated in Rel-17 unified TCI framework, otherwise the CC/BWP is operated in Rel-18 unified TCI framework extension for S-DCI based MTRP. Issue 2.2: No. The motivation is unclear. A similar scenario may happen in Rel-17 unified TCI framework where the activation command has been received but the first beam indication DCI is not applied yet. However, we don’t see any such default behavior in Rel-17. The default behaviors in Rel-17 are designed for the time interval after UE receives the RRC configuration of dl-OrJointTCI-StateList and before applying the indicated TCI. We are open to discuss whether the same default behaviors can be applied to Rel-18 or some modifications may be required but we don’t think we need to add a new default behavior for the time between activation command and the first beam indication DCI. Proposal 2.3: Support. The benefit of mixed “sTRP CC/sDCI-based mTRP CC” and “sTRP CC/mDCI-based mTRP CC” list has been fully discussed in the previous meetings and include reducing the number of CC lists and the number of TCI configuration/activation/indication signaling. As for the FFS, we suggest the following two simple rules: - In a CC list containing both sTRP CC and mTRP CC, only mTRP CC can be configured as the reference CC. - When two joint/DL/UL TCI states are indicated in the reference CC, an RRC parameter configured in each non-reference CC is used to indicate that the first, the second or both joint/DL/UL TCI states are applied for the non-reference CC. |
| Fraunhofer IIS/HHI | Proposal 2.1: Support Issue 2.2: Fine with introducing a default behaviour. Issue 2.3: Do not support mixing of STRP and MTRP CCs. |
| Google | Proposal 2.1: Support. Issue 2.2 Question: No. In legacy, we had the similar issue and debate without consensus to have a solution on it. We don’t see the situation would change. Proposal 2.3: Support in general except the first note. The first note is dependent on the result of Proposal 2.1. Suggest removing it for now. |
| Ericsson | Proposal 2.1: Do not support. Why is this needed?? Does this impact the behavior in any way? Issue 2.2: Does not seem critical Proposal 2.3: Do not support any of the cases. This would require that we reinterpret “Rel.17” MAC CEs for a R18 CC, and vice versa. And the benefit is small: we could just as well use more MAC CEs. |
| CATT | For Proposal 2.1, fine. For Issue 2.2, the necessity is not quite clear to define such default behavior. For Proposal 2.3, fine. |
| Lenovo | For Proposal 2.1: Fine with FL proposal. For Issue 2.2: We are fine to introduce such a default behavior or leave it to NW implementation For Proposal 2.3: Support |
| Sharp | Proposal 2.1: Support. Issue 2.2: Yes, we think the default behavior is needed as with the legacy unified TCI framework. Proposal 2.3: Fine. |
| Spreadtrum | Proposal 2.1: Support Issues 2.3: We only support Case 1 and Case 2, our position is updated in the table. |
| CEWiT | Proposal 2.1: Support Issue 2.2: Support to introduce a default behavior |

- Issue 3 – How to inform UE which indicated TCI state(s) that UE shall apply to target channel/signal

A plan for discussion in this meeting on the TCI selection scheme for each target channel/signal and remaining issues is provided in the following table, including both S-DCI and M-DCI based MTRP operation:

Table 3-0 Summary of TCI selection scheme for each target channel/signal in S-DCI/M-DCI based MTRP operation

| S-DCI based MTRP operation |  |  |
| --- | --- | --- |
| Channel/signal | Conclusion | TCI selection scheme |
| PDCCH | Yes | RRC configuration (first/second/both/none) per CORESET (FFS: whether to reuse Rel-17 rule, discussed in Issue 3.1) |
| PDSCH scheduled/activated by DCI format 1_1/1_2 if the [TCI selection field] is present in DCI format 1_1/1_2 | Yes | [TCI selection field] in DCI format 1_1/1_2 (FFS: details including whether/how to use the codepoint “11” and the applying/mapping order if applies both, discussed in Issue 3.2) |
| PDSCH scheduled/activated by DCI format 1_1/1_2 if the [TCI selection field] is not present in DCI format 1_1/1_2 | No | Discussed in Issue 3.3 |
| PDSCH scheduled/activated by DCI format 1_0 (including DG and SPS) | No | Discussed in Issue 3.4 |
| PDSCH scheduled/activated by DCI format 1_1/1_2 before threshold if the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2 (regardless the presence of the [TCI selection field]) | No | FFS: Detail of the capability of two default beams for S-DCI based MTRP and the threshold value Discussed after there are conclusions for Issue 3.3 and Issue 3.4 |
| PUSCH scheduled/activated by DCI format 0_1/0_2 (including DG and Type2 CG) | Yes | The existing SRS resource set indicator in DCI format 0_1/0_2 (FFS: UE behaviors for SDM and SFN based PUSCH Tx schemes if applies both, discussed in Issue 3.5) |
| PUSCH scheduled/activated by DCI format 0_0 (including DG and Type2 CG) | Yes | Apply the first indicated joint/UL TCI state |
| Type1 CG-PUSCH | Yes | RRC configuration (first/second/both) per Type1 CG configuration (FFS: UE behaviors for SDM and SFN based PUSCH Tx schemes if applies both, discussed in Issue 3.5) |
| PUCCH | Yes | RRC configuration (first/second/both) per PUCCH resource/resource group (FFS: the applying/mapping order if applies both, discussed in Issue 3.6) |
| AP CSI-RS for CSI/BM | Yes | RRC configuration (first/second) per resource or per resource set (FFS: whether to have a fixed rule for NCJT CSI, AP CSI-RS triggered before threshold) |
| SRS for CB/NCB/AS and AP SRS for BM | No | Discussed in Issue 3.7 |
| M-DCI based MTRP operation |  |  |
| Channel/signal | Conclusion | TCI selection scheme |
| PDCCH | Yes | According to coresetPoolIndex value |
| PDSCH scheduled/activated by DCI format 1_0/1_1/1_2 | Yes | According to coresetPoolIndex value corresponding to scheduling PDCCH |
| PUSCH scheduled/activated by DCI format 0_0/0_1/0_2 (including DG and Type2 CG) | Yes | According to coresetPoolIndex value corresponding to scheduling PDCCH |
| PUCCH | No | RRC configuration (first/second) per PUCCH resource/resource group (FFS: whether to support Opt3 and/or Opt4, discussed in Issue 3.9) |
| Type1 CG-PUSCH | Yes | RRC configuration (first/second) per Type1 CG configuration |
| AP CSI-RS for CSI/BM | No | Discussed in Issue 3.8 |
| SRS for CB/NCB/AS and AP SRS for BM | No | Discussed in Issue 3.7 |

Table 3-2 Summary for Issue 3

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 3.1 | (S-DCI) PDCCH, whether Rel-17 rule/parameter is reused when provide the RRC configuration for TCI selection | FL note: In the last meeting, RAN1 concluded no consensus on whether to reuse the Rel-17 RRC parameter followUnifiedTCIstate as a part of the RRC configuration that informs the UE shall apply the first one, the second one, both, or none of the indicated joint/DL TCI states to a CORESET. However, this group still has consensus to reuse Rel-17 rules for providing the RRC configuration to a CORESET. Thus, Proposal 3.1 is recommended: Proposal 3.1: On unified TCI framework extension for S-DCI based MTRP: - If a CORESET other than a CORESET with index 0 is associated only with USS sets and/or Type3-PDCCH CSS sets, the CORESET is configured by RRC to apply the first one, the second one, or both of the indicated joint/DL TCI states to PDCCH reception on the CORESET - If a CORESET other than a CORESET with index 0 is associated at least with CSS sets other than Type3-PDCCH CSS sets, the CORESET is configured by RRC to apply the first one, the second one, both, or none of the indicated joint/DL TCI states to PDCCH reception on the CORESET - For a CORESET with index 0: - If the CORESET is associated with SS#0 for Type 0/0A/2 CSS sets, the CORESET is configured by RRC to apply the first one, the second one, or none of the indicated joint/DL TCI state to PDCCH reception on the CORESET - Otherwise, the CORESET is configured by RRC to apply the first one, the second one, both, or none of the indicated joint/DL TCI states to PDCCH reception on the CORESET Note: RAN1 already agrees to use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the indicated joint/DL TCI states to a CORESET in S-DCI based MTRP. Note: There is no consensus in RAN1 on whether to reuse the Rel-17 RRC parameter followUnifiedTCIstate as a part of above RRC configuration, and whether to reuse followUnifiedTCIstate is up to RAN2 design |
| 3.2 | (S-DCI) PDSCH scheduled/activated by DCI format 1_1/1_2, details of the [TCI selection field] in DCI format 1_1/1_2 | Question: Whether to use the codepoint “11” of the [TCI selection field]? Yes: Spreadtrum, vivo, ZTE, Hyundai, LG, Docomo No (reserved): Panasonic, OPPO, ITRI, QC, Nokia, Futurewei, Fujitsu, Ericsson, Huawei/HiSilicon FL note: Based on contributions in this meeting, the proponents of using the codepoint “11” of the [TCI selection field] are slightly more than the opponents, thus the following proposal is recommended for this issue: Proposal 3.2: On unified TCI framework extension for S-DCI based MTRP, when a 2-bit [TCI selection field] is configured by RRC to be present in a DCI format 1_1/1_2 in a DL BWP: - If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to PDSCH reception scheduled/activated by the DCI format 1_1/1_2 based on the Rel-16 rules for mapping legacy TCI states to PDSCH transmission occasions, CDM groups, or non-overlapping frequency domain resource allocations by replacing the first and the second indicated legacy TCI states with the first and the second indicated joint/DL TCI states, respectively - The codepoint "11" of the [TCI selection field] is reserved |
| 3.3 | (S-DCI) PDSCH scheduled/activated by DCI format 1_1/1_2 if the [TCI selection field] is not present in DCI format 1_1/1_2 | FL note: Let’s make down-selection from the following two proposals in this meeting: Proposal 3.3.A (Both): On unified TCI framework extension for S-DCI based MTRP, for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured w/o the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the scheduled/activated PDSCH reception - If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2, above applies regardless of the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception - If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above applies when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold Support/fine: Xiaomi, OPPO, ZTE, Futurewei, Docomo (SFN), MTK, Lenovo, Nokia, QC, Fujitsu, Intel, Huawei/HiSilicon, Fraunhofer, CMCC, ZTE, Spreadtrum Concern: Sharp, FGI, vivo, LG, Panasonic, Ericsson Proposal 3.3.B (RRC): On unified TCI framework extension for S-DCI based MTRP, for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured w/o the [TCI selection field], using RRC configuration to inform that the UE shall apply the first, the second, or both indicated joint/DL TCI states to the scheduled/activated PDSCH reception - If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2, above applies regardless of the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception - If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above applies when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold Support/fine: OPPO, Apple, Futurewei, Huawei/HiSilicon, Sharp, NEC, MTK, Docomo (non-SFN), Ericsson, Panasonic, Intel, FGI, TransHold, ITRI Concern: vivo, QC, Xiaomi, Nokia, LG, CMCC, Fujitsu, CATT, IDC, ZTE |
| 3.4 | (S-DCI) PDSCH scheduled/activated by DCI format 1_0 (including DG and SPS), at least for the case if the scheduling offset >= a threshold | Question: Which alternative in the followings is adopted for this case: Alt1: If the UE is configured with PDSCH-SFN or PDSCH-CJT, the UE shall apply both first and second indicated joint/DL TCI states to PDSCH reception scheduled/activated by DCI format 1_0. Otherwise, the UE shall apply the first indicated joint/DL TCI state to PDSCH reception scheduled/activated by DCI format 1_0. Support: Panasonic, vivo (SFN only), Xiaomi, CMCC, Docomo, Ericsson Alt2: The UE shall apply the first indicated joint/DL TCI state to PDSCH reception scheduled/activated by DCI format 1_0 Support: CATT, Fujitsu, Intel, Huawei/HiSilicon, Fraunhofer, Qualcomm, MTK, Nokia, vivo, OPPO, IDC, Futurewei, LG, Apple, CATT, Ericsson, FGI, Spreadtrum Alt3: Using RRC configuration to inform that the TCI selection for PDSCH reception scheduled/activated by DCI format 1_0 Support: Ericsson Alt4: The UE shall apply the same joint/DL TCI state(s) that is applied to the PDCCH reception with the scheduling/activation DCI to the scheduled/activated PDSCH reception Support: ZTE, Google, Spreadtrum Proposal 3.4: On unified TCI framework extension for S-DCI based MTRP, for PDSCH reception scheduled/activated by DCI format 1_0, the UE shall the first indicated joint/DL TCI state to the scheduled/activated PDSCH reception - If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2, above applies regardless of the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception - If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above applies when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold |
| 3.5 | (S-DCI) The applying /mapping rule if applies two indicated joint/UL TCI states to PUSCH transmission with SDM and SFN based Tx schemes | Question: Whether to support the following proposal? Proposal 3.5: On unified TCI framework extension for S-DCI based MTRP, when two indicated joint/UL TCI states are applied to a PUSCH transmission - For SDM and SFN based PUSCH Tx schemes, the UE shall apply the first indicated joint/UL TCI state to the PUSCH antenna port(s) associated with the first SRS resource set, and the second indicated joint/UL TCI state to the PUSCH antenna port(s) associated with the second SRS resource set, respectively. - Note: The association between PUSCH antenna port(s) and an SRS resource set is discussed and defined in STxMP AI |
| 3.6 | (S-DCI) The applying /mapping rule if applies two indicated joint/UL TCI states to PUCCH transmission | Question: Whether to support the following proposal? Proposal 3.6: On unified TCI framework extension for S-DCI based MTRP, when two indicated joint/UL TCI states are applied to a PUCCH resource/resource group: - For TDM based PUCCH Tx scheme, the UE shall apply two indicated joint/UL TCI states to repetitions of the PUCCH transmission corresponding to the PUCCH resource/resource group based on the Rel-17 rules for mapping spatial settings to the repetitions by replacing the first and second spatial settings with the first and second indicated joint/UL TCI states, respectively. - For SFN based PUCCH Tx scheme, the UE shall apply two indicated joint/UL TCI states to the PUCCH transmission corresponding to the PUCCH resource/resource group |
| 3.7 | (S-DCI & M-DCI) SRS for CB/NCB/AS and AP SRS for BM | Question: Which alternative in the followings is adopted for this case: Alt1: For a P/SP/AP SRS resource set for CB/NCB/AS or an AP SRS resource set for BM, if the SRS resource set is configured to follow unified TCI state, an RRC configuration can be provided to the SRS resource set to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the SRS resource set - For M-DCI based MTRP operation, the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. Support: Spreadtrum, CATT, Fujitsu, Ericsson, Apple, Qualcomm, Samsung, Nokia, NEC, Docomo, ZTE, LG, Fujitsu, CATT, Google, FGI, CEWiT Alt2: When two SRS resource sets for CB/NCB are configured, the UE shall apply the first indicated joint/UL TCI state to the first SRS resource set for CB/NCB (the one with lower resource set ID) if the first SRS resource set for CB/NCB is configured to follow unified TCI state, and the second indicated joint/UL TCI state to second SRS resource set for CB/NCB if the second SRS resource set for CB/NCB is configured to follow unified TCI state. Otherwise, for a P/SP/AP SRS resource set for CB/NCB/AS or an AP SRS resource set for BM, if the SRS resource set is configured to follow unified TCI state, an RRC configuration can be provided to the SRS resource set to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the SRS resource set. - For M-DCI based MTRP operation, the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. Support: Panasonic, Xiaomi, Huawei/HiSilicon, OPPO, Futurewei, vivo, Docomo |
| 3.8 | (M-DCI) AP CSI-RS for CSI/BM | Question: Which alternative in the followings is adopted for this case: Alt1 (the same RRC configuration used in S-DCI based MTRP): An RRC configuration can be provided to an aperiodic CSI-RS resource set or a CSI-RS resource in an aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS resource set or to the CSI-RS resource in the aperiodic CSI-RS resource set, if the aperiodic CSI-RS resource set for CSI/BM is configured to follow unified TCI state - The first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. Support: Panasonic, Ericsson, Nokia, NEC, LG, Samsung, Panasonic, Huawei/HiSilicon Alt2 (the same scheme used for PDSCH/PUSCH in M-DCI based MTRP): For an aperiodic CSI-RS resource set triggered by PDCCH on a CORESET associated with a coresetPoolIndex value, if the aperiodic CSI-RS resource set for CSI/BM is configured to follow unified TCI state, the UE shall apply the indicated joint/DL TCI state specific to the coresetPoolIndex value to the aperiodic CSI-RS resource set Support: ZTE, Spreadtrum, Xiaomi, OPPO, QC, Futurewei, Docomo, CMCC, Fujitsu, Apple, CATT, Google Alt3: Combining Alt1 and Alt2 Support: vivo |
| 3.9 | (M-DCI) PUCCH | Question: Whether to support Opt3 and/or Opt4? Opt3: For a PUCCH transmission triggered by PDCCH on a CORESET when the UCI in the PUCCH transmission carries HARQ-ACK information only, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is determined from the one associated with the CORESET. - FFS: Whether Opt3 applies only when the UE is not provided with ackNackFeedbackMode = joint Opt4: For a PUCCH transmission with an LRR trigged for either the first BFD-RS set (q0,0) or the second BFD-RS set (q0,1) when the UE is provided only one or two schedulingRequestID-BFR configuration, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is 1 when the LRR is trigged for the first BFD-RS set (q0,0) and the coresetPoolIndex value is 0 when the LRR is trigged for the second BFD-RS set (q0,1). Support Opt3: Futurewei, vivo, Fraunhofer, Apple, OPPO, Sharp, CATT Support Opt4: Futurewei, vivo, Huawei/HiSilicon, CATT Support neither: Panasonic, Spreadtrum, CATT, Ericsson, Google, Nokia, QC, Docomo, ZTE, LG, Samsung |
| 3.10 | How to handle the error case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission | FL note: Most companies think if we strictly follow current RAN1 agreements, the UE should follow the indicated joint/UL TCI state(s) applied to the PUSCH transmission, regardless of whether the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) is the same or different from the spatial Tx filter(s) used for the corresponding SRS transmission(s). Majority also prefer no handling in specification for the error case that the spatial Tx filters are not aligned between PUSCH and SRS, i.e., it should/could be avoided by NW implementation. Based on above observations, I think it is beneficial to have the following conclusion to clarify the UE behavior. Conclusion 3.10: On unified TCI framework extension, for a PUSCH transmission, the UE shall apply the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applying to the PUSCH transmission regardless of whether the determined spatial Tx filter(s) is the same or different from the spatial Tx filter(s) used for the SRS transmission(s) corresponding to the SRS resource(s) indicated to the PUSCH transmission - Note: This conclusion does not introduce any specification impact |

Table 3-3 Company input for Issue 3

| Company | Input to Round 0 summary |
| --- | --- |
| Nokia | Issue 3.1: Support Proposal 3.1 Issue 3.2: Not to support Proposal 3.2. We don’t see the need for use of TCI selection field codepoint “11” in downlink. Issue 3.3: Support Proposal 3.3.A Issue 3.5: We don’t see need for Proposal 3.5. for CB/NCB PUSCH as the existing SRI based indication works also with unified TCI framework. [Mod] This proposal is provided for mapping the two indicated joint/UL TCI states for STxMP case. In previous meetings, we had agreements for TDM case (how to map them to PUSCH Tx occasions), but STxMP case was FFS. Issue 3.6: Support Proposal 3.6. Issue 3.7: Support Alt1 Issue 3.8: Support Alt1 - On 3.9, we think that neither option should be supported for the following reasons (as also explained in our Tdoc): - Option 3 follows similar logic as dynamically scheduled PUSCH, that is to say, PUCCH with a corresponding PDCCH follows the TCI state of the CORESETPoolIndex to which the CORESET used for PDCCH transmission belongs. However, the gNB still has the flexibility of indicating a PUCCH resource (through PRI indication in DCI), meaning that the gNB could control which TCI state to use for the PUCCH through that indication. Thus, Option 3 doesn’t seem necessary on top of Option 2. - Regarding Option 4 which is targeting BFR cases, when there are two PUCCH-SR configured (or SR configurations) each of which associated to a BFD-RS set, Option 2 would be sufficient as each PUCCH-SR could be configured to follow suitable (indicated) TCI state under this option. If, on the other hand, we assume that there is a single PUCCH-SR (or SR configuration) which can be associated to both BFD-RS sets: - In legacy, if for BFR the PUCCH-SR is indicated with two spatial relation info, it’s up the UE to select one of the two spatial relation info for a PUCCH transmission using this resource. - In Rel-18, for multi-DCI case, there is no case defined yet where an UL resource would be associated with two (indicated) TCI states at a time. In our view, we should not make an exception now for PUCCH-SR for BFR, especially that the gNB could (i) still configure two PUCCH-SR, or (ii) could live with configuring a single PUCCH-SR which is configured to follow one of the indicated TCI states if it decides so. |
| QC | For Proposal 3.1, support For Proposal 3.2, prefer “11” to be reserved as in legacy For Proposal 3.3.A, support For issue 3.4, support Alt2 at least for the case when PDSCH-SFN and PDSCH-CJT are not configured, which can be discussed separately For Proposal 3.5, support For Proposal 3.6, support For Proposal 3.7, support Alt1. Alt2 is unnecessarily complicated. For issue 3.8, support Alt2, resource level indication may not be needed for mDCI mTRP For issue 3.9, prefer not to support both option 3 and 4 For conclusion 3.10, support conclusion 3.10 |
| Futurewei | Proposal 3.1: Support. Proposal 3.2: It seems our position was misinterpreted. Our preference is to reserve the codepoint “11”. Issue 3.3: We are fine with both Proposal 3.3.A and Proposal 3.3.B, with a slight preference for Proposal 3.3.B. Issue 3.4: Support Alt2. Proposal 3.6: Support Issue 3.7: Prefer Alt2. Issue 3.8: Prefer Alt2. Issue 3.9: Support both Opt3 and Opt4. Regarding Opt4, for Opt2 to work for BFR cases, as pointed out by Nokia, it requires that the UE is provided two configurations for PUCCH transmission of LRR, each associated with a BFD-RS set (e.g., one TRP). However, supporting two configurations for PUCCH transmission of LRR is a UE capability as indicated by the yellow highlighted text below (from TS 38.213, Section 6). For those UE not supporting “twoLRRcapability”, the gNB can only provide one configuration for PUCCH transmission of LRR to the UE, and this PUCCH transmission needs to be able to associate with either of the two BFD-RS sets and be able to target at either of the TRPs depending on which one is working. However, in Opt2, only one (the first or the second) indicated TCI state is RRC configured for the PUCCH transmission. Therefore, the PUCCH transmission can only be targeted at one of the TRPs, which could be the non-working TRP, thus resulting in BFR failure. So Opt2 does not work in this case. Note that in Rel-17, the spatial relation for this PUCCH transmission is up to UE implementation, so the UE still has the flexibility to dynamically change the targeting TRP based on its own implementation. However, with Opt2 only, the spatial relation/TCI state of this PUCCH transmission is limited by the RRC configuration, and the UE can only target the PUCCH transmission at one of the TRPs, regardless of whether it is the working TRP or not, making it even worse than Rel-17, which should be avoided. With Opt4, the target TRP of the PUCCH transmission can be dynamically changed depending on which one is the working TRP, thus solving this problem. So Opt4 should be supported in addition to Opt2. Text from TS 38.213, Section 6: “A UE can be provided, by schedulingRequestID-BFR-SCell, a configuration for PUCCH transmission with a link recovery request (LRR) as described in clause 9.2.4 for the UE to transmit PUCCH [11, TS 38.321]. If the PCell or the PSCell is associated with sets q0,0 and q1,0, and with sets q0,1 and q1,1, the UE can be provided by schedulingRequestID-BFR a first configuration for PUCCH transmission with a LRR and, if the UE provides twoLRRcapability, the UE can be provided by schedulingRequestID-BFR2 a second configuration for PUCCH transmission with a LRR. If the UE is provided only the first configuration, the UE transmits a PUCCH with LRR for either set q0,0 or q0,1. If the UE is provided both the first and second configurations, the UE uses the first configuration to transmt a PUCCH with LRR associated with set q0,0 and the second configuration to transmit a PUCCH with LRR associated with set q0,1 [11, TS 38.321]. “ |
| vivo | Proposal 3.1: Support. Proposal 3.2: Support. TCI state swapping is beneficial to URLLC services if transmit the repetitions from the TRP with good channel condition firstly. Proposal 3.3.A and Proposal 3.3.B: Don’t support either one. We prefer to applying the first of two indicated TCI states, which can be a common solution for all cases involving default TCI state. Issue 3.4: Alt1: Fine for PDSCH-SFN, but for PDSCH-CJT, whether supporting 2 TCI states is a UE capability. Alt2: Support. Proposal 3.5: Support. Proposal 3.6: Support. Issue 3.7: We prefer Alt2. Besides, at least for M-DCI based MTRP, for an AP SRS resource set for CB/NCB/AS/BM configured to follow unified TCI, if the RRC configuration is absent, the AP SRS can apply the joint/UL TCI state corresponding to the coresetPoolIndex of the triggering PDCCH. This provides flexibility to allow either TRP to trigger an AP SRS. Revised Alt2: Alt2: When two SRS resource sets for CB/NCB are configured, the UE shall apply the first indicated joint/UL TCI state to the first SRS resource set for CB/NCB (the one with lower resource set ID) if the first SRS resource set for CB/NCB is configured to follow unified TCI state, and the second indicated joint/UL TCI state to second SRS resource set for CB/NCB if the second SRS resource set for CB/NCB is configured to follow unified TCI state. Otherwise, for a P/SP/AP SRS resource set for CB/NCB/AS or an AP SRS resource set for BM, if the SRS resource set is configured to follow unified TCI state, an RRC configuration can be provided to the SRS resource set to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the SRS resource set. For M-DCI based MTRP operation, if the RRC configuration is not provided, the UE shall apply the indicated joint/DL TCI state specific to the coresetPoolIndex value to the AP SRS. - For M-DCI based MTRP operation, the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. Issue 3.8: Similar to issue 3.7, two Alts can be combined. That is, if the RRC configuration is not provided in Alt1, Alt2 is applied. Issues 3.9: Support both Opt3 and Opt4. For the FFS in Opt3, we don’t think joint HARQ-ACK feedback is treated differently, because it is clear the PUCCH carrying joint HARQ-ACK information is determined by last DCI which is associated to a specific coresetPoolIndex. Allowing such a flexibility by sharing the PUCCH resources/groups between two TRPs and applying the joint/UL TCI states corresponding to different TRPs based on the scheduling DCI, the benefit of Opt3 is reduced workload of PUCCH configuration and saved PUCCH resources. Conclusion 3.10: Support. |
| NEC | Proposal 3.2: OK. Issue 3.3: support Proposal 3.3.B. And if this RRC configuration is absent, we are OK to have the solution from Proposal 3.3.A. Issue 3.4: to have a common solution as for issue 3.3., we are OK to Alt3. Proposal 3.6: OK Proposal 3.6: OK Issue 3.7: Prefer Alt1. Issue 3.8: Prefer Alt1. Conclusion 3.10: Support, and we also prefer it to be an agreement other than a conclusion. |
| OPPO | Proposal 3.1: Support. Proposal 3.2: Not support. As many others (not captured in table 3.2 under Issue 3.2), we still hold the thought that dynamically switching indicated joint/DL TCI states between TRPs would not bring too much benefits, but change legacy behavior of UEs. Issue 3.4: Preference over Alt.2 added in above table. Proposal 3.5: Support. Proposal 3.6: Support in principle. But the wording “based on the Rel-17 rules for mapping spatial settings to the repetitions by replacing the first and second spatial settings with the first and second indicated joint/UL TCI states” somehow implies to replace spatial relation information in Rel.17 MAC CE with joint/ULTCI states. If that’s the case, it seems a new MAC CE to us. Hope that’s not the intention of this proposal. [Mod] Right, this is not the intension of this proposal. Issue 3.10: Thanks to FL for bringing this issue up again. Intuitively we share the same understanding that the error case in FL’s assessment can be avoided by NW implementation. But as in last meeting, the reason why we hesitated to support it lies in the concern that the conclusion may change UE behavior in current spec on PUSCH port(s) following SRS port(s). If majority still believe this conclusion is clarifying, could we suggest to add a note as Conclusion 3.10: On unified TCI framework extension, for a PUSCH transmission, the UE shall apply the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applying to the PUSCH transmission regardless of whether the determined spatial Tx filter(s) is the same or different from the spatial Tx filter(s) used for the SRS transmission(s) corresponding to the SRS resource(s) indicated to the PUSCH transmission Note: above conclusion does not introduce any specification impact |
| Xiaomi | Proposal 3.1 Support Proposal 3.2 We don’t see the stronger motivation to support it. Proposal 3.3.A and 3.3.B Prefer 3.3.A Issue 3.4 For Alt 1, we support it with the following update. Since if only one TRP is selected or configured for PDSCH-CJT, there is no reason to apply both indicated joint/DL TCI states to PDSCH. Alt1: If the UE is configured with both PDSCH-SFN, or and if the UE is configured with PDSCH-CJT with more than one selected TRP or more than one configured TRP with restriction on TRP selection, the UE shall apply both first and second indicated joint/DL TCI states to PDSCH reception scheduled/activated by DCI format 1_0. Otherwise, the UE shall apply the first indicated joint/DL TCI state to PDSCH reception scheduled/activated by DCI format 1_0. Proposal 3.5/3.6 Support Issue 3.7 Prefer Alt 2. Issue 3.8 Prefer Alt 2. Issue 3.9 No additional option is needed in addition to Opt2. Conclusion 3.10 ok |
| IDC | Proposal 3.1: Support Proposal 3.2: Fine Issue 3.3: We prefer to applying the 1st one of two indicated TCI states for consistency with other cases for the default unified TCI determination. Issue 3.4: Our view is updated above. We support Alt2 (the 1st UTCI) for consistency with other cases for the default unified TCI determination. Proposal 3.5: OK Proposal 3.6: OK |
| ZTE | Issue 3.1: Support. Issue 3.2: Support. @Nokia, Futurewei and QC, regarding the motivation, we have the following clarification. Please review that: - In technical, quite different from legacy procedure in Rel-16 mTRP, in Rel-18 unified TCI, the TCI indication field is used for indicating first and second TCI state(s) (not only for PDSCH transmission). It is quite inconvenient for gNB to switch the order of TCI state by indicating another different TCI state pair (just changing the order), because, if that is doing, it also changes the procedure of other channel reception or transmission, like PDCCH, PUSCH and PUCCH (for instance, a CORESET/PUCCH is associated with 'first' TCI state). Issue 3.3: We support Proposal 3.3.A. It seems that ZTE is not captured in the supporting list. Issue 3.4: In our views, the following behavior should be based on legacy as much as possible, considering co-existence between Rel-18 and before. Otherwise, from NW perspective, we may experience that the PDSCH scheduled by DCI format 0_0 may not be served by one given TRP. - Alt4: The UE shall apply the same joint/DL TCI state(s) that is applied to the PDCCH reception with the scheduling/activation DCI to the scheduled/activated PDSCH reception Proposal 3.5: Not support for SFN. In SFN, we should simplify that the both TCI states are applied to PUSCH (i.e., to each of PUSCH port) as SFN-PUCCH case. Proposal 3.6: Support Issue 3.7: Alt1 Issue 3.8: Support Alt2, but the following clarification is needed as in S-DCI CSI-RS solution. …. - Above applies at least if the offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in the aperiodic CSI-RS resource set is equal to or larger than a threshold (if the threshold is needed) Issue 3.9: Support neither Conclusion 3.10: We are fine. A conclusion is needed. |
| LG | Issue 3.1: Fine with the proposal. Issue 3.2: Support the proposal. Issue 3.3: Don’t support both proposals and prefer to apply indicated TCI state(s) based on the TCI field in DCI. This is due to the fact that both proposals restrict PDSCH for a certain transmission mode permanently, i.e. STRP or MTRP mode only. The method with the TCI field in DCI supports STRP/MTRP dynamic switching without increasing DCI payload but with slightly reduced flexibility of beam update due to the coupling of unified TCI update with STRP/MTRP switching. Issue 3.4: Support Alt2 Issue 3.7: Support Alt1 Issue 3.8: Fine with Alt1 Issue 3.9: Prefer not to support both option 3 and 4 |
| Fujitsu | Proposal 3.1: Support Proposal 3.2: Not support. To keep “11” reserved as in legacy is preferred. Issue 3.3: Proposal 3.3.A is preferred. Issue 3.4: Support Alt2. Proposal 3.5: Support Proposal 3.6: Support Issue 3.7: Support Alt1. Issue 3.8: Support Alt2. |
| Docomo | Issue 3.1: Fine with the proposal, but we think we can leave it to spec. editor based on the agreements. Proposal 3.2: Support. TCI state swapping is beneficial, and we have concern to preclude behavior which was possible in the existing spec. Proposal 3.3: We suggest to discuss separately for SFN and Non-SFN, because it is aligned with existing spec. Another possible way forward may be to support both Proposal 3.3A and Proposal 3.3B and to be switched by RRC. In this case, gNB can switch depending on whether SFN operation or not. Issue 3.4: Support Alt1. Alt.2 is not aligned with the current HST-SFN spec. Proposal 3.5: OK. Proposal 3.6: OK. Issue 3.7: Either is OK. If Alt.1 is supported, we would configure the same as Alt.2. Issue 3.8: We prefer Alt.2. Issue 3.9: - Opt.3: Not needed. In Rel.16 mDCI, such option was discussed, and finally it becomes up to gNB implementation (i.e. PRI field can indicate different PUCCH with different spatial relation) - Opt.4: We dont think the proposal is needed. Conclusion 3.10: Support. |
| Apple | Issue 3.1: Support. This is best we can do in RAN1 and leave the details to RAN2. We do not see the need to keep Rel-17 parameter for this indication. Introducing a Rel-18 parameter is a clean solution without messing up with the Rel-17 IE. Proposal 3.2: Ok to support. We think ZTE provides a valid use case to justify the need of new mechanism for PDSCH repetition in Rel-18 mTRP, which avoids the unnecessary impact on other channels. Proposal 3.3: Fine with either P3.3A and P3.3B. Issue 3.4: Alt.2 Proposal 3.5: Support. Issue 3.7: Alt.1. Issue 3.8: Alt.2. Issue 3.9: Opt.3. |
| CMCC | Proposal 3.1: Support. Proposal 3.2: Support. It is beneficial to improve robustness of PDSCH repetition transmission. Proposal 3.3: Prefer Proposal 3.3.A. Proposal 3.4: Prefer Alt1, which considers the performance of PDSCH-SFN and PDSCH-CJT. Proposal 3.5: We share similar view with ZTE that both TCI states are applied to SFN based PUSCH. Proposal 3.6: Support. Proposal 3.7: For P/SP/AP SRS resource set for CB/NCB/AS and S-DCI based AP SRS for BM, we are fine with Alt2. For M-DCI based AP SRS for BM, the UE may apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the SRS set triggered by PDCCH on a CORESET that is associated with the same coresetPoolIndex value. Proposal 3.8: Support Alt2. Proposal 3.10: Support. |
| Samsung | Proposal 3.1: support Proposal 3.2: seems not critical Proposal 3.3: prefer RRC based method Proposal 3.5/3.6: support in principle Proposal 3.7: prefer Alt.1, which is a simple extension Proposal 3.8: prefer Alt.1, it is simple. Proposal 3.9: do not support additional options Conclusion 3.10: support. |
| Panasonic | Proposal 3.1: Support Proposal 3.2: Do not support, as we think this is an enhancement to the Rel 16 and not necessary for extending the unified TCI framework. Proposal 3.3A: We do not support, as we think signaling should enable switching between single TRP and multi-TRP transmission. The network might choose to operate without the TCI selection field so this is not a corner/exception case. Proposal 3.3B: Support. Proposal 3.4: Why can’t we just add an FFS to Alt 2 Alt2: The UE shall apply the first indicated joint/DL TCI state to PDSCH reception scheduled/activated by DCI format 1_0 FFS: If the UE is configured with either PDSCH-SFN or PDSCH-CJT [Mod] The intension of Alt2 is no “special case”. Proposal 3.5: Support Proposal 3.6: Support Proposal 3.7: Support the first part of Alt2 (still confused about the “Otherwise” part) Proposal 3.8: Support Alt 1, because this is what we did for the S-DCI case. Proposal 3.9: We have a complete solution already, why make it more complicated? |
| FGI | Proposal 3.1: Support Proposal 3.2: Support Proposal 3.3.B: Support Issue 3.4: Support Alt.2 Proposal 3.5: Support Proposal 3.6: Support Proposal 3.7: Support Alt.1 Conclusion 3.10: Support |
| Huawei, HiSilicon | Proposal 3.1: Support Proposal 3.2: Not support. We prefer to keep “11” reserved. We don’t see the necessity of such TRP swapping for PDSCH reception. Proposal 3.3A/3.3B: We slightly prefer 3.3A to align the solution with the case where the scheduling offset is smaller than a threshold and the UE supports two default beams in FR2. However, we are also OK with 3.3B. Issue 3.4: First, we think, Alt1 was meant to be written as follows as UE is not supposed to be configured with both SFN and CJT at the same time: Alt1: If the UE is configured with both PDSCH-SFN and or PDSCH-CJT, the UE shall apply both first and second indicated joint/DL TCI states to PDSCH reception scheduled/activated by DCI format 1_0. Otherwise, the UE shall apply the first indicated joint/DL TCI state to PDSCH reception scheduled/activated by DCI format 1_0. Second, we suggest to discuss SFN and CJT scheduled/activated by DCI format 1_0 separately. For other cases, the first indicated joint/DL TCI state could be used. Maybe the following proposal would be a good starting point: Proposal: At least for the case that UE is not configured with PDSCH-SFN or PDSCH-CJT, the UE shall apply the first indicated joint/DL TCI state to PDSCH reception scheduled/activated by DCI format 1_0. - FFS: The applied joint/DL TCI state(s) when UE is configured with PDSCH-SFN or PDSCH-CJT. Proposal 3.5: We don’t recall having an agreement that SDM or SFN can be configured using Type1 CG or Type2 CG. We can support the following: Proposal 3.5: On unified TCI framework extension for S-DCI based MTRP, when two indicated joint/UL TCI states are applied to a PUSCH transmission (including DG, Type1 CG, and Type2 CG) - For SDM and SFN based PUSCH Tx schemes, the UE shall apply the first indicated joint/UL TCI state to the PUSCH antenna port(s) associated with the first SRS resource set, and the second indicated joint/UL TCI state to the PUSCH antenna port(s) associated with the second SRS resource set, respectively. Note: The association between PUSCH antenna port(s) and an SRS resource set is discussed and defined in STxMP AI Proposal 3.6: Support Issue 3.7: We support Alt2. Based on the agreement in RAN1#112, when SRS resource set indicator = “00”, the first indicated joint/UL TCI state applies to PUSCH and when SRS resource set indicator = “01”, the second indicated joint/UL TCI state applies to PUSCH. However, according to Table 7.3.1.1.2-36 of 38.212, SRS resource set indicator = “00” is associated with the first SRS resource set and SRS resource set indicator = “01” is associated with the second SRS resource set. Further, based on the same agreement, when SRS resource set indicator = “10” or “11”, the UE applies the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set. Above discussion shows that, regardless of the value of the SRS resource set indicator, the first indicated joint/UL TCI state applies to the PUSCH or PUSCH transmission occasions that are associated with the first SRS resource set and the second indicated joint/UL TCI state applies to the PUSCH or PUSCH transmission occasions that are associated with the second SRS resource set. Therefore, when followUnifiedTCI-StateSRS is configured for CB/NCB SRS, to avoid a mismatch between the SRS beam and the corresponding PUSCH or PUSCH transmission occasions beam, the first indicated joint/UL TCI state should be applied to the first SRS resource set and the second indicated joint/UL TCI state should be applied to the second SRS resource set. Issue 3.8: We prefer Alt1 to have a unified solution for sDCI and mDCI cases. Important note: We think the agreement for aperiodic CSI-RS of sDCI based MTRP is incomplete. In the last meeting, we had the following agreement: Agreement On unified TCI framework extension for S-DCI based MTRP, an RRC configuration can be provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTrigger State for each CSI-RS resource set or for each CSI-RS resource in each aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the CSI-RS resource if the aperiodic CSI-RS resource set for CSI/BM is configured to follow unified TCI state - Above applies at least if the offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in the aperiodic CSI-RS resource set is equal to or larger than a threshold (if the threshold is needed) - FFS: If the UE is configured for CSI-RS resource set, for an aperiodic CSI-RS resource set configured with two Resource Groups for NCJT CSI and configured to follow unified TCI state, if above RRC configuration is not provided to the aperiodic CSI-RS resource set, the UE shall apply the first indicated joint/DL TCI state to the CSI-RS resource(s) in Group 1 and the second indicated joint/DL TCI state to the CSI-RS resource(s) in Group 2. - ‘per CSI-RS resource set’ or ‘per CSI-RS resource’ is up to UE capability. Since ‘per CSI-RS resource set’ or ‘per CSI-RS resource’ indicated TCI selection configuration is up to UE capability, if UE only supports ‘per CSI-RS resource set’ configuration and is further configured with aperiodic CSI-RS resource set with two Resource Groups for NCJT CSI, based on the current agreement, UE would have to apply the same indicated TCI to the two resource groups. This would be a misbehaviour from UE since the two resource groups are transmitted from two different TRPs. Therefore, we do believe that, at least for the case that UE does not support ‘per CSI-RS resource’ configuration, the rule in the FFS of the above agreement should apply: For an aperiodic CSI-RS resource set configured with two Resource Groups for NCJT CSI and configured to follow unified TCI state, if the configuration of the indicated TCI selection is not provided per CSI-RS resource, the UE shall apply the first indicated joint/DL TCI state to the CSI-RS resource(s) in Group 1 and the second indicated joint/DL TCI state to the CSI-RS resource(s) in Group 2. Issue 3.9: We think it is necessary to support Opt4. In mTRP BFR procedure, the PUCCH-SR should be transmitted to the non-failed TRP. This cannot be accommodated by Option 2 since gNB cannot know in advance which TRP will be the non-failed TRP and, therefore, cannot configure the RRC parameter correctly. Issue 3.10: We don’t think this conclusion is required. The spec is already very clear that PUSCH should follow uTCI. |
| Fraunhofer IIS/HHI | Proposal 3.1: Support Issue 3.3: Support proposal 3.3A. Proposal 3.6: OK Issue 3.9: Similar view as vivo on Opt3. Support Opt3. Conclusion 3.10: In our view, the alignment between beams of PUSCH and the associated SRS is not just beneficial but should be normal operating procedure. If there is no consensus among companies on specifying such a restriction, it's fine to leave it to implementation and take the conclusion. |
| Google | Proposal 3.1: OK Proposal 3.2: OK Proposal 3.5: Support Proposal 3.6: Support Proposal 3.7: Support Alt1. Alt2 can be achieved by Alt1. Proposal 3.8: Support Alt2. Conclusion 3.10: Support. |
| Ericsson | Proposal 3.1: But why is this needed? It’s the same rule for all the CORESET types. Proposal 3.2: Don’t support. Not needed. Proposal 3.3. We prefer 3.3B: with a fixed rule, we cannot support all cases where there are two indicated TCI states but only one is applicable to PDSCH. Issue 3.4: Alt1 is fine as well. Even Alt2 could be ok. Issue 3.8: With Alt2, cross-TRP triggering of CSI-RS becomes impossible. Shouldn’t that be supported? Conclusion 3.10: OK |
| CATT | For Proposal 3.1, support For Proposal 3.2, fine. For Proposal 3.3A and Proposal 3.3B, we prefer to use the first indicated TCI state. For progress, we are also fine with Proposal 3.3A. For issue 3.4, support Alt2 For Proposal 3.5, support For Proposal 3.6, support For Proposal 3.7, support Alt1. For issue 3.8, Alt2 is preferred. For issue 3.9, both option 3 and 4 are not supported. For conclusion 3.10, support. |
| Lenovo | For Proposal 3.1: Fine For Proposal 3.2: Fine. For Issue 3.3: We support Proposal 3.3A. For Issue 3.4: Support Alt2. For Proposal 3.5: Support For Proposal 3.6: Support For Proposal 3.7: Support Alt2. For Issue 3.8: Prefer Alt2. For Issue 3.9: Neither option 3 nor 4 are supported. For Conclusion 3.10: Support. |
| Sharp | Proposal 3.1: Support. Proposal 3.3.A: If the UE receives one joint/DL TCI state from the TCI field and keeps only one joint/DL TCI state (e.g., when the UE have one joint/DL TCI state after receiving the first beam indication DCI, or when the lowest TCI codepoint has only one joint/DL TCI state that is related to Issue 2.2), how does the UE apply both indicated joint/DL? In these cases, we think the one joint/DL TCI state is applied to a PDSCH. Proposal 3.3.B: Support. Issue 3.4: Support Alt 1. Proposal 3.5: Support. Proposal 3.6: Support. Issue 3.7: Support Alt 1. Proposal 3.9: Support Opt3. For Opt3, for a given time-frequency resource, the number of PUCCH resources can be saved because a PUCCH resource can be associated with both CORESET pool indexes by scheduling PDCCH. Conclusion 3.10: Support. |
| Mod V27 | - Add Proposal 3.4 based on majority view - Proposal 3.2 is modified based on companies’ feedback |
| Spreadtrum | Proposal 3.1: Support Issue 3.3: Support Proposal 3.3.A Issue 3.4: Our position is added, we are ok with Alt 2 and Alt 4 Proposal 3.5, 3.6: Support. |
| CEWiT | Proposal 3.1. Support Proposal 3.2. Fine Proposal 3.5. Support Proposal 3.6. Support Proposal 3.7. Support Alt1. |

- Issue 4 – UL power control for UL MTRP operation

Table 4-1 Summary for Issue 4

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 4.1 | UL PC for S-DCI based STxMP (including SDM/SFN based PUSCH Tx and SFN based PUCCH Tx) | Agreement On unified TCI framework extension, if an indicated joint/UL TCI state(s) applies to a PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s), the UE shall determine UL Tx power for the PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) based on the UL PC parameter setting for PUSCH/PUCCH/SRS, if any, and the PL-RS included in the indicated joint/UL TCI state - FFS: For STxMP, the maximum Tx power when the UE determines UL Tx power for the PUSCH/PUCCH transmission occasion(s) or antenna port(s) (discussed after receiving RAN4 reply on UE power limitation for STxMP in FR2) - FFS: Default UL PC parameter setting(s) if one or both of indicated joint/UL TCI states applied to PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) does/do not include the UL PC parameter setting(s) for PUCCH/PUSCH/SRS FL note: Based on above RAN1 agreement, for S-DCI based PUSCH/PUCCH STxMP, the UE needs to determine two UL Tx power according to the first and second indicated joint/UL TCI states for a same PUSCH/PUCCH Tx occasion, where each determined UL Tx power is applied to the corresponding antenna port(s). However, in current UL PC procedure of TS 38.213, only one UL Tx power is determined per PUSCH/PUCCH Tx occasion. Thus, to support S-DCI based PUSCH/PUCCH STxMP, some companies propose to introduce a new parameter (t) in current UL PC determination for PUSCH/PUSCH as follows: PPUSCH,b,f,c,ti,j,qd,l= minPCMAX,f,c,tiPO_PUSCH,b,f,c,tj+10log102μ∙MRB,b,f,cPUSCHi+αb,f,c,tj∙PLb,f,cqd,t+∆TF,b,f,ci+fb,f,ci,lt [dBm], where t=0,1 corresponds to the first and second indicated joint/UL TCI states, respectively. PPUCCH,b,f,c,ti,qu,qd,l= minPCMAX,f,c,tiPO_PUCCH,b,f,c,t(qu)+10log102μ⋅MRB,b,f,cPUCCH(i)+PLb,f,c(qd,t)+∆F_PUCCH(F)+∆TF,b,f,ci+gb,f,c(i,lt) [dBm], where t=0,1 corresponds to the first and second indicated joint/UL TCI states, respectively. Question: Whether to support above modifications to current UL PC procedure of TS 38.213 for S-DCI based PUSCH/PUCCH STxMP? Yes: QC, vivo, NEC, OPPO, Xiaomi, LG, Docomo, CMCC, CATT, Ericsson, FGI No: Samsung, Huawei/HiSilicon FL note: Based on the comments from companies, the following two proposals are recommended, and the detail/definition of PCMAX,f,c,ti is left to RAN4. Proposal 3.10 (PUSCH): On unified TCI framework extension for S-DCI based MTRP, support the following Tx power determination for S-DCI based PUSCH STxMP (including SDM and SFN based Tx schemes) in TS 38.213: PPUSCH,b,f,c,ti,j,qd,l= minPCMAX,f,c,tiPO_PUSCH,b,f,c,tj+10log102μ∙MRB,b,f,cPUSCHi+αb,f,c,tj∙PLb,f,cqd,t+∆TF,b,f,ci+fb,f,ci,lt [dBm], where t=0,1 corresponds to the first and second indicated joint/UL TCI states, respectively. Note: Detail/definition of PCMAX,f,c,ti is up to RAN4, including whether to reuse the definition of PCMAX,f,ci for PCMAX,f,c,ti Proposal 3.10 (PUCCH): On unified TCI framework extension for S-DCI based MTRP, support the following Tx power determination for S-DCI based PUCCH STxMP (including SFN based Tx scheme) in TS 38.213: PPUCCH,b,f,c,ti,qu,qd,l= minPCMAX,f,c,tiPO_PUCCH,b,f,c,t(qu)+10log102μ⋅MRB,b,f,cPUCCH(i)+PLb,f,c(qd,t)+∆F_PUCCH(F)+∆TF,b,f,ci+gb,f,c(i,lt) [dBm], where t=0,1 corresponds to the first and second indicated joint/UL TCI states, respectively. Note: Detail/definition of PCMAX,f,c,ti is up to RAN4, including whether to reuse the definition of PCMAX,f,ci for PCMAX,f,c,ti |
| 4.2 | Tx power reduction for STxMP (including both S-DCI and M-DCI based STxMP) | Question: Whether prioritization procedure for Tx power reductions specified in TS38.213 Session 7.5 should also consider STxMP so that the total UE Tx power for transmissions on serving cells in the frequency range wouldn’t exceed a total power limitation PCMAX(i) used in TS 38.213 (clause 7.5) and defined in TS 38.101? Yes: QC, vivo (panel-specific prioritization procedure), Nokia, OPPO, Xiaomi, LG, Docomo, CMCC, Panasonic, FGI No: ZTE, Ericsson, Huawei/HiSilicon |

Table 4-2 Company input for Issue 4

| Company | Input to Round 0 summary |
| --- | --- |
| Mod V00 | Please share your answers and comments to the two questions for Issue 4.1 and 4.2 |
| Nokia | - Fine to discuss 4.1, but this is not essential to discuss now. - On 4.2, we don’t think there is a decision to make between Yes and NO here. The existing power reduction prioritization rule in TS 38.213 shall be discussed/improved to now account for STxMP. - There are other important power control related aspects and proposals that needs to be discussed, as also discussed in our Tdoc: - For Rel-18 multi-TRP PUSCH/PUCCH operations under unified TCI framework, reuse the Rel-17 multi-TRP power control enhancement as follows: - a second TPC field can be configured in DCI formats 1_1/1_2 and 0_1/0_2 to support per-TRP closed-loop power control. Specifically, first and second indicated TPC commands would apply to UL transmissions/repetitions mapped to first and second indicated TCI states, respectively; - if the second TPC field is not configured, indicated TPC command applies to UL transmissions/repetitions mapped to both first and second indicated TCI states. - Discuss PH (power headroom) aspects considering the Rel-18 schemes, including simultaneous UL transmissions, such as (i) number of PHRs to report at a given time and which UL scheme to use for the calculation of the PHR(s), and (ii) the cases where the UE is not configured with two-PHR mode. - Which assumption(s) to consider regarding power limitation for STxMP (e.g., per panel). - Decide whether MPE is an issue for simultaneous UL transmissions. |
| QC | For issue 4.1, the answer is Yes. Because the existing PC equation does not consider the STxMP scenario. Without any change, it may imply the total PUSCH power should be less than the existing Pcmax,f,c. This may not fully exploit the separate PA per UE panel. So the proposal is to clarify the per-TCI PUSCH power should be less thatn the per-TCI Pcmax,f,c For issue 4.2, we think it is necessary to consider power prioritization in case of STxMP |
| vivo | Issue 4.1: Yes Issue 4.2: Yes, and we think the prioritization procedure of Tx power reductions should be panel-specific. |
| NEC | Issue 4.1: Support to have modifications. OK to have subscript ‘t’ for PCmax, maybe more discussion for other items in the equations. Issue 4.2: open to have discussions. |
| OPPO | Issue 4.1: Yes. Issue 4.2: Yes. |
| Xiaomi | First of all, before any discussion on the power control for STxMP, RAN1 needs to decide whether a total power limitation over all panels and power limitation per panel should be introduced. Q1: Yes We support the modifications only when per-panel power limitation is agreed to be introduced. And PCMAX,f,c,ti in the formulas above is the power limitation for the panel corresponding to the new parameter t. Q2: Yes We are fine with the power reduction. But the total UE transmit power should be also smaller than or equal a total power limitation over all panels, if introduced, not just current per UE power limitation PCMAX(i). |
| ZTE | For issues 4.1, we are general fine with the modifications to facilitate the independent power control of different panels. However, for the power limitation P_c,max, the current modifications seem imply that two UE-configured maximum output power values are mandatory enabled for respective panels. Since whether to have one or two UE-configured maximum output power value(s) is still undetermined, we prefer to add a FFS to clarify whether PCMAX,f,c,ti in the formula is a per-panel power limitation or a per-UE power limitation. For issue 4.2, Not support. In our views, power re-allocation mechanisms (equally scaling once total power across multiple panel is beyond UE power limit) should be considered as a starting point for in case of prioritization rules, especially for PUCCH/PUSCH STxMP in S-DCI based MTRP operation. Then, for M-DCI based operation, considering that we do not need to update the UL power control formula, and then we may further study whether power prioritization is needed. |
| LG | Update our view in the above table. Support to modify UL PC determination for PUSCH/PUCCH taking per-panel/TCI index power control into account. Based on this, prioritization procedure to meet the Tx power limitation on UE should be considered accordingly. |
| Docomo | Issue 4.1: Yes. The modification is needed to support per panel power control. Issue 4.2: Yes. We support to consider prioritization procedure with considering STxMP. Meanwhile, we feel existing priority order can be reused as much as possible. |
| CMCC | Issue 4.1: Yes. Issue 4.2: Yes. |
| Samsung | For open-loop power control parameters: We do not support panel specific configuration of OL-PC parameters Current spec and agreement already support beam or TCI specific open-loop power control and we think this is sufficient to support panel specific OL-PC. For panel specific closed-loop PC process index: We do not support. Beam specific CL-PC is also supported by current spec. And panel specific CL-PC can be supported via beam specific CL-PC. [Mod] They are “TCI-specific” parameters if you check the definition of “t”. If we go with current equation specified in 213, only one Tx power can be determined for one PUCCH/PUSCH Tx occasion. It’s unclear that how to determine two Tx power for S-DCI based STxMP based on current equation in spec even we have agreed the UE shall use two set of PC parameters to determine the two Tx power. For panel specific Pc,max No support the proposed modification.. We basically agree that Pc,max can be different per panel depending on UE’s implementation. But this is also true for Rel-17 MPUE. As the biggest difference comparing Rel-18 MPUE operation and Rel-17 MPUE panel selection, panel specific peak power of Rel-18 MPUE can be changed depending scheduled/configured UL transmission. For example, let assume UE has two panels each has peak power of 23dBm while UE peak power is 23dBm. When single panel Tx is scheduled, UE may apply up to 23dBm for that single panel, but UE should apply less than 23dBm, e.g. 20dBm of per panel peak power when STxMP UL is scheduled. We think this variation of per panel peak power is the only difference compared with Rel-17 MPUE in a perspective of power control, and RAN1 should focus on this issue. |
| Panasonic | Issue 4.1: Our only concern is that PCMAX,f,c,ti is not be well defined at the moment (RAN4), so we prefer to use a generic variable name or perhaps Pmax,f,c,t(i), at least to distinguish it from Pcmax. Issue 4.2: Support. |
| FGI | Issue 4.1: Yes Issue 4.2: Yes |
| Huawei, HiSilicon | Issue 4.1: Not before some more fundamental issues are resolved. We think the two PC formulas is a jump to a final solution without discussing and agreeing on some intermediate steps. There are a few issues/ambiguities regarding the PC formulas in Issue 4.1 that we would like them to be clarified before deciding whether or not these formulas should be supported: - The two formulas assume that “per panel” UE configured maximum output power PCMAX,f,c,ti is already agreed. This is not the case. - The current UE configured maximum output power PCMAX,f,c, has a clear definition in RAN4 spec (38.101-2 Clause 6.2.4). Further, in RAN4 spec, there are restrictions as follow on the value of PCMAX,f,c PPowerclass + PIBE – MAX(MAX(MPRf,c, A- MPRf,c,) + ΔMBP,n, P-MPRf,c) – MAX{T(MAX(MPRf,c, A- MPRf,c,)), T(P-MPRf,c)} ≤ PUMAX,f,c ≤ EIRPmax In particular, the right hand side inequality guarantees that, PUMAX,f,c, the maximum configured value for PCMAX,f,c, is less than EIRP_max to satisfy the regulatory requirements. In contrast, per panel configured maximum output power PCMAX,f,c,t is not defined in RAN4 and, hence, there is no restriction on its value. If RAN1 agrees to use PCMAX,f,c,t without agreeing on its upper or lower bounds, there is no guarantee that the regulatory requirements are satisfied. Note that, according to RAN4 LS reply, “the per-UE power limitation would be applicable at all the time. ‘Limitation’ here applies to regulatory compliance rather than a configured power requirement.”. As such, using PCMAX,f,c,t without at least defining its upper and lower bound doesn’t seem to be a technically sound solution. - Finally, we think that, if PCMAX,f,c,t is agreed to be defined, to align the used terminology in other STxMP agreements, it should be defined per SRS resource set and not per uTCI. Issue 4.2: The intention of such discussion is not clear. Prioritization rules in Clause 7.5 of 38.213 are defined for a UE that is configured with UL CA. In that Clause, different simultaneous channels/signals on different CCs are prioritized so that the UE total transmit power on all CCs does not exceed PCMAX(i) that is the linear value of PCMAX(i) . In turn, according to 38.101-2 Clause 6.2A.4.1, PCMAX is the total configured maximum output power over all UL carriers. We don’t see any direct relation between the UL CA prioritization/power reduction and STxMP prioritization/power reduction as STxMP may operate in a single CC. We think we should first address power reduction/dropping rule of the two panel in ONE CELL, when the sum of the max power of the two panels derived from the formulas (if agreed) in 4.1 is larger than EIRPmax Also, the question states “Whether prioritization procedure for Tx power reductions specified in TS38.213 Session 7.5 should also consider STxMP so that the total UE Tx power for transmissions on serving cells in the frequency range wouldn’t exceed a total power limitation PCMAX(i) used in TS 38.213 (clause 7.5) and defined in TS 38.101?”. Does the above question mean the following: Whether to incorporate PUxCH1 (from the first panel) and PUxCH2 (from the second panel) into the current prioritization rules so that, for the UE that is configured with UL CA and, in one or multiple of its active CCs is further configured with STxMP, different simultaneous channels/signals (including, in part, the PUxCHs transmitted using STxMP) on all UL CCs are prioritized so that the UE total transmit power on all CCs does not exceed PCMAX(i). [Mod] Yes, current spec in Clause 7.5 of 38.213 is only specified for UL CA, and question in Issue 4.2 is whether to add “single cell + STxMP” and “CA + STxMP” in the power allocation procedure. |
| Ericsson | Issue 4.1. Should be solved. Would prefer that we talk about Tx power per ports/layers rather than TCI state. It could be that the TCI states are the same. Issue 4.2: No. Power split is up to the UE. |
| CATT | Issue 4.1: Yes. As mentioned earlier, the existing PC equation in TS 38.213 section 7.1 does not consider the STxMP scenario where two UE panels each has independent/different power amplifier. It has limitation and only adapts to the case where two panels share one power amplifier or two panels each has its independent power amplifier but the maximum value limitation of the two power amplifiers are same. So the proposal clarifies that per-panel(TCI state) PUSCH power is less than the per-panel(TCI state) Pcmax,f,c,t. Issue 4.2: Basically yes, we think it necessary to consider power prioritization for STxMP to ensure that the sum of two UL Tx power values for PUSCH/PUCCH should not exceed a UE-configured maximum output power value PCMAX. PCMAX(i) is proposed to be used for prioritization for Tx power reduction over PUSCH/PUCCH concurrently transmitted over different cells in an UL CA scheme. Whether the PCMAX(i) can be used to determine the two UL Tx power values for PUSCH/PUCCH STxMP transmission on a single CC needs to be further considered. |
| Lenovo | Issue 4.1: The answer should be yes and we support both Proposal 3.10s for PUSCH and PUCCH. Issue 4.2: Yes. |
| Sharp | Issue 4.1: We think new parameter t is needed to determine Tx power for each set of antenna port(s). Issue 4.2: Yes. |
| Spreadtrum | Issue 4.1: Yes, the TCI/panel-specific power control could be supported. Issue 4.2: OK |

- Issue 5 – PDSCH-CJT Tx scheme

Table 5-1 Summary for Issue 5

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 5.2 | QCL type(s)/assumption(s) if two indicated joint TCI states are applied to PDSCH-CJT | Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA - Support: Huawei, Spreadtrum, OPPO, Ericsson, Xiaomi, CATT, Qualcomm, Nokia, Docomo, CMCC, Lenovo, NEC, LG, Intel, Samsung, Sharp - Concern: ZTE Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state - Support: Huawei, ZTE, Ericsson, Xiaomi, Qualcomm, Docomo, CMCC, Samsung - Concern: Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB - Support: ZTE, Ericsson - Concern: QC, Samsung FL note: Let’s make down-selection from the following two proposals in this meeting: Proposal 5.2.A: On unified TCI framework extension for S-DCI based MTRP, the following three alternatives are supported for PDSCH-CJT applying both indicated joint TCI states (if the UE supports two indicated joint/DL states for PDSCH-CJT): - Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA - Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state - Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB Introduce a UE capability on which alternative(s) is supported, and either one of above alternatives can be configured by RRC according to the UE capability Support: ZTE, Huawei/HiSilicon, Docomo, Ericsson, Lenovo, Nokia, Samsung, OPPO, CMCC Concern: QC Proposal 5.2.B: On unified TCI framework extension for S-DCI based MTRP, the following two alternatives are supported for PDSCH-CJT applying both indicated joint TCI states (if the UE supports two indicated joint/DL states for PDSCH-CJT): - Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA - Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state Introduce a UE capability on which alternative(s) is supported, and either one of above alternatives can be configured by RRC according to the UE capability Support: Huawei/HiSilicon, Ericsson, Xiaomi, Qualcomm, Docomo, CMCC, Samsung Concern: ZTE |

Table 5-2 Company input for Issue 5

| Company | Input to Round 0 summary |
| --- | --- |
| Mod V00 | Let’s make down-selection from Proposal 5.2.A and Proposal 5.2.B in this meeting |
| QC | We are fine for Proposal 5.2.B. It is also fine for no agreement on this, which implies Alt1 only. In fact, we have doubts on the real performance of PDSCH CJT. We are open to study all CJT related enhancements in R19, but not in R18, which has no agenda to study PDSCH CJT itself. For Alt3, our concerns are re-copied below - Study of CJT PDSCH enhancement is not in TCI WID - Companies should have agreed sim scenarios and type to carry out evaluations like R17 SFN, R18 STxMP, not like CJT PDSCH, which has no agenda for its EVM assumptions. - Allowing to discuss CJT in TCI is already a big compromise, no further enhancement is preferred without agreed EVM - However, we are open to study CJT enhancement in R19 - We are not clear on the modeling/scenario of Alt3 evaluation, and hence cannot justify its effectiveness especially for large TRP delay difference - E.g. how to model the delay compensation at gNB, by applying phase shifts across SBs, or by adjusting TRP Tx timing? Zero discussion in TCI so far - Also, what is the max delay difference among TRPs allowed for CJT? X times larger than CP? - CJT TRPs with small delay difference should be the baseline, which may not need Alt3 - Only TRPs with DL Rx timing within CP is considered in R16/17 mTRP. So there should be plenty deployment scenarios without the need of Alt3 - For CJT TRPs with large delay difference, there can be simple alternative to Alt3 - For example, if both Doppler and delay need to be compensated, NW can indicate a single TCI, whose Doppler/Delay properties are used for both TRPs after compensation - Finally, we think LLS is the rigorous tool to model the channel estimation - Thanks ZTE for the SLS results, but we are unclear how SLS can accurately model the channel estimation |
| vivo | Fine with Proposal 5.2.B. |
| ZTE | To make our position clear: We support Alt2 and Alt3, but we have concerns about Alt1 where there is a serious performance loss. Evaluation results can be found in our contribution. After that, for progress, we can be flexible for having Proposal 5.2.A. Removing Alt3 (with significant performance gains and deployment usages mentioned by several gNB vendors, e.g., E///, ZTE, etc.) but having Alt1 is unacceptable for us. After that, thanks for QC’s comments on Alt3. Please find our following reply, although some comments may not be for us. - Study of CJT PDSCH enhancement is not in TCI WID - Companies should have agreed sim scenarios and type to carry out evaluations like R17 SFN, R18 STxMP, not like CJT PDSCH, which has no agenda for its EVM assumptions. [ZTE]: Our evaluation is based on CJT evaluation assumption in 9.1.2 - Allowing to discuss CJT in TCI is already a big compromise, no further enhancement is preferred without agreed EVM - However, we are open to study CJT enhancement in R19 - We are not clear on the modeling/scenario of Alt3 evaluation, and hence cannot justify its effectiveness especially for large TRP delay difference - E.g. how to model the delay compensation at gNB, by applying phase shifts across SBs, or by adjusting TRP Tx timing? Zero discussion in TCI so far [ZTE]: In our current evaluation, the phase shifting is provided across RB for delay pre-compensation. Except for considering that case larger than CP, to be honest, we do not think that it is not discussed. On the contrary, we have CJT-CSI mode-1 (inter-site) and mode-2 (intra-site), and also the following agreement (φn represents the delay/phase-shift) is achieved for pre-compensating the loss. In short, we do not agree that there is no discussion. Agreement On the Type-II codebook refinement for CJT mTRP, for mode-1, support the use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources, i.e. (example formulation) Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where: - Wf is commonly selected across N CSI-RS resources - φn is the layer-common FD basis selection offset for CSI-RS resource n relative to a layer-common reference CSI-RS resource n with φn=0 - Therefore, (N – 1) FD basis selection offset values φnn≠n are reported - Basic feature: φn∈0,1,2,…,N3-1 - Optional feature: φn∈0,14,12…,N3-14 - FFS: UCI design details, details on n. - Also, what is the max delay difference among TRPs allowed for CJT? X times larger than CP? [ZTE]: In our views, it may be relevant to Rel-19 discussion. - CJT TRPs with small delay difference should be the baseline, which may not need Alt3 - Only TRPs with DL Rx timing within CP is considered in R16/17 mTRP. So there should be plenty deployment scenarios without the need of Alt [ZTE]: Our evaluation is based on the agreed EVM/typical scenario. - For CJT TRPs with large delay difference, there can be simple alternative to Alt3 - For example, if both Doppler and delay need to be compensated, NW can indicate a single TCI, whose Doppler/Delay properties are used for both TRPs after compensation - Finally, we think LLS is the rigorous tool to model the channel estimation - Thanks ZTE for the SLS results, but we are unclear how SLS can accurately model the channel estimation [ZTE]: The question is unclear for us. In SLS, there is channel estimation error mode for emulating the real-field channel estimation. BTW, TCI state is just to provide a large-scale channel property. One more question from us to better understand the position of QC: - We fail to understand the logic of supporting Alt1 (2TCI state with full QCL) but being against Alt3 (1 TCI with full QCL, but second TCI with Doppler-only) from UE vendor. - From product perspective, it seems beneficial for UE’s implementation, due to the fact that UE can cancel additional efforts on Delay-domain sync for second TCI state (which is handled by gNB pre-compensation, and, if performance loss occurs, gNB will take the responsibility) |
| Docomo | Proposal 5.2.A/B: Either is ok for us. |
| Apple | Fine with Proposal 5.2.B. |
| Samsung | We prefer proposal 5.2.A more. As editorial, we suggest to change alt to option. |
| Huawei, HiSilicon | We support 5.2.A. We don’t see why Alt3 should be excluded. |
| Google | Proposal 5.2.A: We can support it. |
| CATT | Support Proposal 5.2B. |
| Lenovo | Support Proposal 5.2B. |

- Issue 6 – Beam failure recovery

Table 6-1 Summary for Issue 6

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 6.1 | Implicit BFD-RS determination for S-DCI based MTRP | Proposal 6.1: On unified TCI framework extension for S-DCI based MTRP, if the UE is provided the first candidate beam RS list (q1,0) and the second candidate beam RS set (q1,1) but not explicitly provided the first BFD-RS set (q0,0) and the second BFD-RS set (q0,1) for TRP-specific BFR and if both first and second indicated joint/DL TCI states are configured by RRC to be applied to CORESETs for PDCCH reception except PDCCH-SFN, the UE determines the BFD-RS for the first and second BFD-RS sets from the first and second indicated joint/DL TCI states, respectively. - FFS: The case if any CORESET is configured to apply both first and second indicated joint/DL TCI states for PDCCH-SFN - FFS: Whether and how to handle the case if one or both of the first and second indicated joint/DL TCI states is/are NOT configured by RRC to be applied to CORESET(s) for PDCCH reception Support/fine: QC, Xiaomi, Samsung, ZTE, LG, Apple, Futurewei, Spreadtrum, MTK, Huawei/HiSilicon, CATT, vivo, Docomo, Ericsson, Intel, FGI, Lenovo, TCL, CMCC, Panasonic Concern: Google, OPPO |
| 6.2 | Enhancement to beam update after NW response to TRP-specific BFR request | Question: Do we need similar enhancement for S-DCI based MTRP as well? Yes: Docomo, ZTE, Intel, FGI, Xiaomi, vivo, Apple, QC, Nokia, Huawei/ HiSilicon, Samsung, OPPO, CMCC No: FL note: Based majority view, the following proposal is recommended for S-DCI based MTRP. For proponents of this proposal, it would be appreciated if you can provide the corresponding solution for the FFS. Proposal 6.2: On unified TCI framework extension for S-DCI based MTRP, after NW response to TRP-specific BFR request to a BFD-RS set: - If the BFD-RS set is the first BFD-RS set (q0,0), QCL assumption/spatial Tx filter/PL-RS for channel(s)/signal(s) that applies the first indicated joint/DL/UL TCI state are updated according to the new beam (qnew) corresponding to the BFD-RS set. - If the BFD-RS set is the second BFD-RS set (q0,1), QCL assumption/spatial Tx filter/PL-RS for channel(s)/signal(s) that applies the second indicated joint/DL/UL TCI state are updated according to the new beam (qnew) corresponding to the BFD-RS set. - FFS: The case if channel(s) applies both first and second indicated joint/DL/UL TCI states |

Table 6-2 Company input for Issue 6

| Company | Input to Round 0 summary |
| --- | --- |
| Mod V00 | Please provide your comment to Proposal 6.1 and Proposal 6.2, if any. |
| QC | For proposal 6.1, support For proposal 6.2, support |
| Futurewei | Proposal 6.1: Support. |
| vivo | Proposal 6.1: We recognize that TRP-specific BFR is not supported in MTRP PDCCH repetition by far. Thus, there should be another exception that no linkage is configured between search spaces associated with two different CORESETs applying two indicated joint/DL TCI states respectively. Proposal 6.2: Support. |
| OPPO | Proposal 6.1: Not support. (updated in table 6-1 accordingly) For S-DCI MTRP BFR in Rel.17, if we remember correctly, only explicit BFD RS setting was supported. But proposal 6.1 assume implicit BFD RS determination. This seems not extending Rel.17 MTRP with unified TCI states, but also enhancement on it. Proposal 6.2: Fine to extend beam reset for S-DCI MTRP BFR. |
| Xiaomi | Proposal 6.1 Support Proposal 6.2 Support and suggest to remove the sub-bullet of FFS since if channel(s) applies both first and second indicated joint/DL/UL TCI states, the 1st and the 2nd sub-bullet can also be applied. It means if it is the first BFD-RS set, the first TCI state will be updated to the new beam corresponding to the first BFD-RS set and keep the second TCI state if the second BFD-RS set is not failed. |
| ZTE | For proposal 6.1, support For proposal 6.2, support |
| LG | Proposal 6.1: Support. |
| Docomo | Proposal 6.1: Support. Proposal 6.2: Support. P6.2 is very essential. If UE does not update QCL assumption after BFR completion, gNB can only send PDCCH to update TCI state in old/failed beam, which does not work. Note that any of RRC/MAC CE/DCI level TCI update requires PDCCH transmission and PUCCH transmission, and if UE cannot receive PDCCH or transmit PUCCH in the new beam, there is no way for gNB to update TCI. |
| Apple | Proposal 6.1: Support Proposal 6.2: Support. |
| CMCC | Proposal 6.1: Support Proposal 6.2: Support. |
| Samsung | Support both proposals. |
| Panasonic | Proposal 6.1: Support |
| Huawei, HiSilicon | Proposal 6.1: Support Proposal 6.2: Support |
| Google | Proposal 6.1: Not support. This is not an essential feature to make per-TRP BFR for S-DCI MTRP workable. We also have doubt whether we can resolve these FFS in two meetings. Proposal 6.2: Support |
| Ericsson | Proposal 6.1: BFD monitors PDCCH quality. Why should it trigger for something that is not related to PDCCH quality? Proposal 6.2: Support |
| CATT | Support proposal 6.1 and 6.2. |
| Lenovo | Proposal 6.1: Support Proposal 6.2: Support. |

- Other issue

If there is any important issue not captured in the discussion of previous meetings, company can input to Table 7-1.

Table 7-1 Company inputs for other issue

| Company | Input |
| --- | --- |
| FGI | We’ve already had an agreement on the behavior when the TCI field is present, and the TCI selection field is absent. However, the case that the TCI field is absent, but the TCI selection field is present still needs to be clarified. In such case, it is unclear that the codepoint of the TCI selection field corresponds to which set of indicated TCI states. If the UE is not expected to be configured with such situation, we might need to specify it in the meeting. |
| Huawei, HiSilicon | We think the agreement for aperiodic CSI-RS of sDCI based MTRP is incomplete. In the last meeting, we had the following agreement: Agreement On unified TCI framework extension for S-DCI based MTRP, an RRC configuration can be provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTrigger State for each CSI-RS resource set or for each CSI-RS resource in each aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the CSI-RS resource if the aperiodic CSI-RS resource set for CSI/BM is configured to follow unified TCI state - Above applies at least if the offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in the aperiodic CSI-RS resource set is equal to or larger than a threshold (if the threshold is needed) - FFS: If the UE is configured for CSI-RS resource set, for an aperiodic CSI-RS resource set configured with two Resource Groups for NCJT CSI and configured to follow unified TCI state, if above RRC configuration is not provided to the aperiodic CSI-RS resource set, the UE shall apply the first indicated joint/DL TCI state to the CSI-RS resource(s) in Group 1 and the second indicated joint/DL TCI state to the CSI-RS resource(s) in Group 2. - ‘per CSI-RS resource set’ or ‘per CSI-RS resource’ is up to UE capability. Since ‘per CSI-RS resource set’ or ‘per CSI-RS resource’ indicated TCI selection configuration is up to UE capability, if UE only supports ‘per CSI-RS resource set’ configuration and is further configured with aperiodic CSI-RS resource set with two Resource Groups for NCJT CSI, based on the current agreement, UE would have to apply the same indicated TCI to the two resource groups. This would be a misbehaviour from UE since the two resource groups are transmitted from two different TRPs. Therefore, we do believe that, at least for the case that UE does not support ‘per CSI-RS resource’ configuration, the rule in the FFS of the above agreement should apply: For an aperiodic CSI-RS resource set configured with two Resource Groups for NCJT CSI and configured to follow unified TCI state, if the configuration of the indicated TCI selection is not provided per CSI-RS resource, the UE shall apply the first indicated joint/DL TCI state to the CSI-RS resource(s) in Group 1 and the second indicated joint/DL TCI state to the CSI-RS resource(s) in Group 2. |
| Fraunhofer IIS/HHI | The default TCI states in the following scenarios, if applicable, should be discussed: - PDSCH configured for STRP while PDCCH configured with MTRP repetition or SFN. - PUSCH configured for STRP while PUCCH configured for MTRP. - Other hybrid STRP-MTRP configurations across various channels. |

- Appendix: Agreements/conclusions before/in RAN1#113

RAN1#113

Hope we can have similar progress as we had in the previous meeting

RAN1#112b-e

Conclusion On unified TCI framework extension for S-DCI based MTRP operation, there is no consensus to support dynamic switching between single-TRP operation and multi-TRP operation for channels/signals based on the number of TCI states mapped to the received TCI codepoint in DCI format 1_1/1_2 - FFS: How to switch between Rel-17 sTRP operation and Rel-18 mTRP operation Agreement On unified TCI framework extension, the Rel-17 timeline for updating the indicated joint/DL/UL TCI state(s) is retained, i.e., the indicated joint/DL/UL TCI state(s) applied to the DL reception or UL transmission in each slot is updated based on the Rel-17 beam application time Agreement On unified TCI framework extension for S-DCI based MTRP, the UE shall apply the first indicated joint/UL TCI state to PUSCH transmission(s) scheduled/activated by DCI format 0_0 (including DG and Type2 CG) Agreement On unified TCI framework extension for S-DCI based MTRP, an RRC configuration is provided to a Type1 CG configuration to inform that the UE shall apply the first, the second, or both indicated joint/UL TCI states to the corresponding CG-PUSCH transmission - If the first or the second indicated joint/UL TCI state is applied, the UE shall apply the first or the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If both indicated joint/UL TCI states are applied: - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB - FFS: SDM and SFN based PUSCH Tx schemes Agreement On unified TCI framework extension for S-DCI based MTRP, PDSCH-CJT Tx scheme is RRC-configured, and dynamic switching between PDSCH-CJT and other S-DCI based PDSCH Tx schemes is not supported Agreement If the UE is configured with SSB-MTC-AdditionalPCI and receives TCI state activation command (MAC-CE) that activates a set of joint/DL /UL TCI state(s) specific to each coresetPoolIndex value for M-DCI based MTRP in unified TCI framework extension, the activated joint/DL /UL TCI state(s) specific to one coresetPoolIndex value is associated with the serving cell PCI and the activated joint/DL /UL TCI state(s) specific to another coresetPoolIndex value can be associated with a PCI other than the serving cell PCI . - Note: How to implement above in specification is up to spec editor Agreement On unified TCI framework extension for M-DCI based MTRP , after NW response to TRP-specific BFR request to a BFD-RS set associated with a coresetPoolIndex value, QCL assumption/spatial Tx filter/PL-RS for channel(s)/signal(s) that applies the indicated joint/DL /UL TCI state specific to the coresetPoolIndex value are updated according to the new beam (q new ) corresponding to the BFD-RS set. Agreement On unified TCI framework extension for S-DCI based MTRP, the presence of the [TCI selection field] can be RRC-configured per DL BWP - FFS: Whether the presence of the [TCI selection field] can be configured individually for DCI format 1_1 and DCI format 1_2 in the same DL BWP Agreement On unified TCI framework extension for S-DCI based MTRP operation, support the followings: - For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {first joint TCI state, second joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - TCI state activation command (MAC-CE) should indicate that each joint/DL/UL TCI state mapped to a TCI codepoint is the first or second joint/DL/UL TCI state (detail on how to indicate above is up to RAN2 design) - The first/second indicated joint/DL/UL TCI state(s) is updated according to the corresponding first/second joint/DL/UL TCI state(s) mapped to the TCI codepoint received by the UE - If the UE receives a TCI codepoint mapped with a sub-set of {first joint TCI state, second joint TCI state} or {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state}, the UE shall update the first/second indicated joint/DL/UL TCI state(s) according to the first/second joint/DL/UL TCI state(s) in the subset and keep other indicated first/second joint/DL/UL TCI state(s) that is not updated by the received TCI codepoint Agreement On unified TCI framework extension for M-DCI based MTRP, support at least Opt2 for PUCCH transmission, and Opt1 is not supported - Note: Opt3 and Opt4 are not precluded Conclusion On unified TCI framework extension for S-DCI based MTRP, there is no consensus in RAN1 on whether to reuse the Rel-17 RRC parameter followUnifiedTCIstate as a part of the RRC configuration that informs the UE shall apply the first one, the second one, both, or none of the indicated joint/DL TCI states to a CORESET - Above does not impact how RAN2 writes their specifications Agreement On unified TCI framework extension for S-DCI based MTRP, an RRC configuration can be provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTrigger State for each CSI-RS resource set or for each CSI-RS resource in each aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the CSI-RS resource if the aperiodic CSI-RS resource set for CSI/BM is configured to follow unified TCI state - Above applies at least if the offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in the aperiodic CSI-RS resource set is equal to or larger than a threshold (if the threshold is needed) - FFS: If the UE is configured for CSI-RS resource set, for an aperiodic CSI-RS resource set configured with two Resource Groups for NCJT CSI and configured to follow unified TCI state, if above RRC configuration is not provided to the aperiodic CSI-RS resource set, the UE shall apply the first indicated joint/DL TCI state to the CSI-RS resource(s) in Group 1 and the second indicated joint/DL TCI state to the CSI-RS resource(s) in Group 2. - ‘per CSI-RS resource set’ or ‘per CSI-RS resource’ is up to UE capability Agreement On unified TCI framework extension, support the following cases for CA operation: - A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in S-DCI based MTRP - A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in M-DCI based MTRP - FFS: A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP and CC(s) operating in S-DCI based MTRP - FFS: How to support common TCI state ID activation/update for this case - FFS: A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP and CC(s) operating in M-DCI based MTRP - FFS: How to support common TCI state ID activation/update for this case - FFS: A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in S-DCI based MTRP and CC(s) operating in M-DCI based MTRP - FFS: How to support common TCI state ID activation/update for this case - FFS: A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP, CC(s) operating in S-DCI based MTRP, and CC(s) operating in M-DCI based MTRP - FFS: How to support common TCI state ID activation/update for this case Agreement On unified TCI framework extension for M-DCI based MTRP, an RRC configuration is provided to a Type1 CG configuration to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding CG-PUSCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively.

RAN1#112

Agreement On unified TCI framework extension for S-DCI based MTRP, a 2-bit [TCI selection field] can be configured by RRC to be present in a DCI format 1_1/1_2 that schedules/activates PDSCH reception (including dynamic PDSCH and SPS PDSCH) according to the followings: - If the DCI format 1_1/1_2 indicates codepoint "00" for the [TCI selection field], the UE shall apply the first one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "01" for the [TCI selection field], the UE shall apply the second one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the PDSCH reception scheduled/activated by the DCI format 1_1/1_2 - FFS: Whether and how to use the codepoint "11" of the [TCI selection field] If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2 regardless of threshold, above apply to PDSCH reception(s) scheduled/activated by the DCI format 1_1/1_2. - Note: If the UE supports the capability of two default beams for S-DCI based MTRP in FR2, UE uses both indicated joint/DL TCI states to buffer the received signal before a threshold. If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above apply to the scheduled/activated PDSCH reception when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold - FFS: How to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2 FFS: Detail of the capability of two default beams for S-DCI based MTRP FFS: The threshold value Agreement On unified TCI framework extension for S-DCI based MTRP, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG): - If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "10" or “11” for the existing SRS resource set indicator: - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB (note: the association between an SRS resource set for CB/NCB and PUSCH transmission occasions(s) is defined according to TS 38.214) - FFS: SDM and SFN based PUSCH Tx schemes FFS: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission Agreement On unified TCI framework extension, if an indicated joint/UL TCI state(s) applies to a PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s), the UE shall determine UL Tx power for the PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) based on the UL PC parameter setting for PUSCH/PUCCH/SRS, if any, and the PL-RS included in the indicated joint/UL TCI state - FFS: For STxMP, the maximum Tx power when the UE determines UL Tx power for the PUSCH/PUCCH transmission occasion(s) or antenna port(s) (discussed after receiving RAN4 reply on UE power limitation for STxMP in FR2) - FFS: Default UL PC parameter setting(s) if one or both of indicated joint/UL TCI states applied to PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) does/do not include the UL PC parameter setting(s) for PUCCH/PUSCH/SRS Agreement On unified TCI framework extension for M-DCI based MTRP, down-select from the following options for PUCCH transmission: - Opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission - Opt2: An RRC configuration can be provided per PUCCH resource/resource group to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding PUCCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. - Opt3: For a PUCCH transmission triggered by PDCCH on a CORESET when the UCI in the PUCCH transmission carries HARQ-ACK information only, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is determined from the one associated with the CORESET. Otherwise, either Opt1 or Opt2 is adopted. - FFS: Whether Opt3 applies only when the UE is not provided with ackNackFeedbackMode = joint - Opt4: For a PUCCH transmission with an LRR trigged for either the first BFD-RS set (q0,0) or the second BFD-RS set (q0,1) when the UE is provided only one or two schedulingRequestID-BFR configuration, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is 1 when the LRR is trigged for the first BFD-RS set (q0,0) and the coresetPoolIndex value is 0 when the LRR is trigged for the second BFD-RS set (q0,1). Otherwise, either Opt1 or Opt2 is adopted. Note: Either Opt1 or Opt2 must be supported Agreement On unified TCI framework extension for S-DCI based MTRP, down-select at least one of the followings for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured w/o the [TCI selection field]: - Alt1: Using RRC configuration to inform that the UE shall apply the first one, the second one, or both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt2: The UE shall apply the first one of two indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception - Alt3: The UE shall apply both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt3A: The UE shall apply the same joint/DL TCI state(s) that is applied to the PDCCH reception with the scheduling/activation DCI to the scheduled/activated PDSCH reception - Alt4: Which indicated joint/DL TCI state(s) is/are applied to the scheduled/activated PDSCH reception is determined according to the existing TCI field of the most recently applied beam indication DCI Above applies at least if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold (if the threshold is needed)

RAN1#111

Agreement On unified TCI framework extension for S-DCI based MTRP, in one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL/UL TCI state(s) for one or both of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - FFS: Increase on the size of the TCI field - Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS Agreement On unified TCI framework extension for S-DCI based MTRP, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - The presence of the DCI field is configurable by RRC; when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: Details on the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: The DCI field is a new indicator field or an existing field (e.g., the existing TCI field) - FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0. Above applies for the case where PDSCHs scheduled by the same DCI. Agreement On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group - Note: Detail of the RRC configuration is left to RAN2 design Agreement On unified TCI framework extension, PDSCH-CJT is supported as a S-DCI based MTRP scheme Note: Above does not preclude discussions specific to PDSCH-CJT design in the unified TCI framework Agreement On unified TCI framework extension for S-DCI based MTRP, use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 Agreement On unified TCI framework extension, down-select at least one of the following alternatives for PDSCH-CJT applying both indicated joint TCI states (if the UE supports two indicated joint/DL states for PDSCH-CJT): - Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA - Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state - Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB Agreement On unified TCI framework extension for M-DCI based MTRP, the same configuration/rule used in Rel-17 unified TCI framework (for determining whether the UE shall apply the indicated joint/DL TCI state to PDCCH on a CORESET and respective PDSCH) is reused to determine whether the UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET associated with the same coresetPoolIndex value and PDSCH scheduled/activated by the PDCCH. Agreement On unified TCI framework extension for M-DCI based MTRP, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to PUSCH transmission scheduled/activated by PDCCH (including DG-PUSCH and Type2 CG-PUSCH) on a CORESET that is associated with the same coresetPoolIndex value Agreement On unified TCI framework extension for S-DCI based MTRP, a new indicator field is supported as the DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - FFS: Detail design of the new indicator field

RAN1#110b-e

Conclusion On unified TCI framework extension in Rel-18, there is no consensus to support simultaneous configuration of both joint and separate DL/UL TCI modes in a serving cell Conclusion On unified TCI framework extension in Rel-18, there is no consensus to support separate RRC-configured TCI state list(s) for each of TRPs Agreement On unified TCI framework extension for M-DCI based MTRP: - The existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) associated with one coresetPoolIndex value can indicate the joint/DL/UL TCI state(s) specific to the same coresetPoolIndex value - FFS: The UE shall apply the indicated joint/DL/UL TCI state(s) specific to a coresetPoolIndex value to channel(s)/signal(s) that have explicit or implicit association with the same coresetPoolIndex value - A coresetPoolIndex value field is included in TCI state activation command (MAC-CE) to indicate that the mapping between the activated TCI state(s) and the TCI codepoint(s) is specific to which coresetPoolIndex value Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, support the following: - Use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET or a group of CORESETs (if CORESET group configuration is supported) Agreement On unified TCI framework extension for M-DCI based MTRP: - For a serving cell configured with joint DL/UL TCI mode, one joint TCI state can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) - For a serving cell configured with separate DL/UL TCI mode, a DL TCI state, an UL TCI state, or a pair of DL and UL TCI states can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) Agreement On unified TCI framework extension for S-DCI based MTRP, down-select one alternative from the followings in RAN1#111 for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2: - Alt1: Use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 - Alt2: PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 follows the spatial domain transmission filter(s) used for the SRS resource(s) indicated by the DCI format 0_1/0_2 - FFS: PL-RS(s), and UL PC parameter setting(s) (including P0, alpha, and closed loop index) for the PUSCH Agreement On unified TCI framework extension for S-DCI based MTRP, down-select one alternative from the followings in RAN1#111 for PUCCH transmission: - Alt1: Use RRC configuration to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/ group - Alt2: Use RRC configuration to inform the association between a CORESET group and a PUCCH resource/group, and the indicated joint/UL TCI state(s) associated with the CORESET group applies to the PUCCH resource/group associated with the same CORESET group - Alt3: Use MAC-CE to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group - Note: the association indicates whether the UE shall apply the first one, the second one, or both of the joint/UL TCI states indicated by DCI/MAC-CE to a PUCCH resource/group Agreement On unified TCI framework extension, up to 2 joint TCI states can be indicated by MAC-CE/DCI and applied to CJT-based PDSCH reception (PDSCH-CJT) in a BWP/CC configured with joint DL/UL TCI mode - Support of 1 or 2 indicated joint TCI states for PDSCH-CJT is up to UE capability - FFS: QCL type(s)/assumption(s) of the indicated joint TCI state(s) applied to PDSCH-CJT - Note: On how to inform UE to apply which indicated joint TCI state(s) to target channel(s)/signal(s) in the BWP/CC, it is discussed individually in AI 9.1.1.1 Agreement On unified TCI framework extension for M-DCI based MTRP: - The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET that is associated with the same coresetPoolIndex value - The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDSCH scheduled/activated by PDCCH on a CORESET that is associated with the same coresetPoolIndex value - FFS: Other channel(s)/signal(s) that has explicit or implicit association with a coresetPoolIndex value - FFS: Other channel(s)/signal(s) that doesn’t have association with a coresetPoolIndex value Above are applicable to the CORESET(s) that is configured/allowed to follow the indicated joint/DL TCI state FFS: The configuration/rule to configure/allow CORESET(s) to follow the indicated joint/DL TCI state, including the option to reuse the same configuration/rule as in Rel-17 unified TCI framework Agreement On unified TCI framework extension, study the following enhancements for TRP-specific BFR: - Implicit BFD-RS determination based on the indicated joint/DL TCI states for S-DCI based MTRP - Enhancement to beam update after NW response to TRP-specific BFR request Agreement On unified TCI framework extension for S-DCI based MTRP, down-select one alternative from the followings in RAN1#111: - Alt1: In one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL /UL TCI state(s) for one of the two TRPs or both TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - Alt2: In one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL /UL TCI state(s) only specific to one of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - Note: According to the agreement in RAN1#109-e, support of one additional TCI field or a field associating the TCI field to the TRP(s) is not precluded Note: It has been agreed to use the existing TCI field for TCI state indication for S-DCI based MTRP in RAN1#109e Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS FFS: The behavior if the UE receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP

RAN1#110

Agreement On unified TCI framework extension, for the target use cases agreed in RAN1#109-e in AI 9.1.1.1, up to 4 TCI states can be indicated in a CC/BWP or a set of CCs/BWPs in a CC list to DL receptions and/or UL transmissions, where these TCI states are indicated/updated by MAC-CE/DCI with the necessary MAC-CE based TCI state activation - FFS: The possible combination(s) of joint/DL/UL TCI states that can be indicated to DL receptions and/or UL transmissions in a BWP/CC/TRP - Note: This agreement does not imply that there will be more than 2 DL or UL or joint TCI states indicated in a CC/BWP for the target use cases agreed in RAN1#109-e in AI 9.1.1.1 - Note: The maximum number of TCI states that can be indicated to each of the target use cases agreed in RAN1#109-e in AI 9.1.1.1 is remained the same as in Rel-16/17 Note: The maximum number of TCI states that can be indicated simultaneously to CJT-based PDSCH reception and the required type(s) of TCI states (i.e., DL /UL/joint) are independently discussed in this AI Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, down-selection at least one alternative from the followings: - Alt1-1: Use RRC parameter(s) in a CORESET configuration to inform the UE whether and/or which indicated joint/DL TCI state(s) shall be applied to the corresponding PDCCH receptions on the CORESET - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE - Alt1-2: Use an RRC parameter in a CORESET configuration to inform that the CORESET belongs to which CORESET group(s), and the indicated joint/DL TCI state(s) is associated with each CORESET group - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the CORESET group(s) - FFS: How to associate the indicated joint/DL TCI state(s) with each CORESET group - FFS: The UE applies the indicated joint/DL TCI state(s) to a CORESET according to the CORESET group(s) the CORESET belongs to, or the UE applies the indicated joint/DL TCI state(s) associated with the CORESET group(s) in which the beam indication DCI is received to all PDCCH receptions - Alt2: The association between a CORESET and the indicated joint/DL TCI state(s) is determined based on a fixed rule, and the UE shall apply the indicated joint/DL TCI state(s) to the corresponding PDCCH receptions on the CORESET - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE - Alt3: Use MAC-CE to inform the UE whether and/or which indicated joint/DL TCI state(s) shall be applied to the corresponding PDCCH receptions on a CORESET - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE Switching between multi-TRP and single TRP operation is not precluded Agreement On unified TCI framework extension for S-DCI based MTRP, for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2, down-selection one alternative from the followings: - Alt1: Use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in a DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 - Alt2: PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 follows the spatial domain transmission filter(s) used for the SRS resource(s) indicated by the DCI format 0_1/0_2 - Alt3: Use an RRC parameter in a CORESET configuration to inform that the CORESET belongs to which CORESET group(s), and the indicated joint/UL TCI state(s) is associated with each CORESET group. When a scheduling/activation DCI format 0_1/0_2 is received in a CORESET group, the indicated joint/UL TCI state(s) associated with the CORESET group is applied to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 - FFS: Details of CORESET group(s) FFS: PUSCH transmission scheduled/activated by a DCI format 0_0 and Type-1 CG-PUSCH Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with joint/UL TCI state(s) indicated by DCI/MAC-CE for PUCCH transmission, down-selection at least one alternative from the followings: - Alt1: Use RRC configuration to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/ group - Alt2: Use RRC configuration to inform the association between a CORESET group and a PUCCH resource/group, and the indicated joint/UL TCI state(s) associated with the CORESET group applies to the PUCCH resource/group - Alt3: Use MAC-CE to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group - Alt4: Use DCI to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group

RAN1#109e

Agreement On unified TCI framework extension, consider all the intra and inter-cell MTRP schemes specified in Rel-16 and Rel-17 - Consider, if STxMP is supported, Rel-18 MTRP scheme(s) with STxMP Agreement On unified TCI framework extension at least for single-DCI based MTRP, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate multiple joint/DL/UL TCI states in a CC/BWP or a set of CCs/BWPs in a CC list - FFS: Detail of mapping joint/DL/UL TCI state ID(s) to a TCI codepoint, e.g., possible combinations of joint, DL, and/or UL TCI state IDs that can be mapped to a TCI codepoint - FFS: Whether to increase the max number of MAC CE activated TCI codepoints, i.e., more than 8 codepoints - FFS: Whether to increase the max number of TCI field bits, i.e., more than 3 bits - Note: This doesn't imply that support of one additional TCI field or a field associating the TCI field to the TRP(s) is precluded Note: The term TRP is used only for the purposes of discussions in RAN1 and whether/how to capture this is FFS Agreement On unified TCI framework extension for M-DCI based MTRP, consider the following alternatives for TCI state update: - Alt1: Reuse the same TCI state update scheme for S-DCI based MTRP - Atl2: Use the existing TCI field in the DCI format 1_1/1_2 (with or without DL assignment) associated with one of CORESETPoolIndex values to indicate the joint/DL/UL TCI state(s) corresponding to the same CORESETPoolIndex value - Alt3: Use the existing TCI field in any DCI format 1_1/1_2 (with or without DL assignment) to indicate all joint/DL/UL TCI states corresponding to both CORESETPoolIndex values - Study the association between the indicated joint/DL/UL TCI state(s) and a CORESETPoolIndex value - Alt4: Use the existing TCI field in the DCI format 1_1/1_2 (with or without DL assignment) associated with one of CORESETPoolIndex values to indicate joint/DL/UL TCI state(s) corresponding to the same or different CORESETPoolIndex value. - Study whether the indicated joint/DL/UL TCI state(s) applies to the channels/signals associated with the same CORESETPoolIndex value or different CORESETPoolIndex value is indicated by DCI Agreement On unified TCI framework extension for S-DCI based MTRP, consider at least the following alternatives to map/associate a joint/DL TCI state to PDCCH reception(s) - Atl1: Use RRC configuration to inform the mapping/association between a configured or indicated joint/DL TCI state and a CORESET or a CORESET group - Alt2: Use RRC configuration to inform the mapping/association between a configured or indicated joint/DL TCI state and a search space set - Alt3: Use MAC-CE to inform the mapping/association between an activated or indicated joint/DL TCI state and a CORESET or a CORESET group - Alt4: Use DCI to inform the mapping/association between an indicated joint/DL TCI state and a CORESET or a CORESET group - Alt5: Based on a fixed mapping/association rule, e.g., the first indicated joint/DL TCI state always applies to PDCCH receptions Consider above alternatives for PDCCH repetition, PDCCH-SFN, PDCCH w/o repetition/SFN, and potential support of dynamic switching between S-TRP and M-TRP for PDCCH. It is not precluded to adopt one single alternative or multiple alternatives to support these cases. Agreement On unified TCI framework extension, if an indicated joint or UL TCI state applies to a PUSCH/PUCCH transmission occasion at least for S-DCI based PUSCH/PUCCH repetition with TDM and the indicated joint or UL TCI state is associated with an UL PC parameter setting for PUSCH/PUCCH (including P0, alpha for PUSCH, and closed loop index) and a PL-RS, the UE should apply the UL PC parameter setting and the PL-RS for the PUSCH /PUCCH transmission occasion. - FFS: How to extend to other Rel-18 MTRP scheme(s) with STxMP, if supported - FFS: UL PC enhancement for CB and non-CB SRS in above case FFS: The applied UL PC parameter setting if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH Agreement On UE power limitation for STxMP for FR2, send LS to RAN4 to check the followings: - Whether it is feasible to assume power limitation per panel for STxMP (Assumption 1) - Whether it is feasible to assume a total power limitation per UE over all UE panels used for STxMP (Assumption 2) - In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class? - If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)? FFS: Detail of exact LS if agreed Note: Scenarios of above include at least single carrier scenario for FR2 Note: Above power limitation includes both total radiated power and EIRP LS to RAN4 is endorsed in R1-2205639.

- References

| 1 | R1-2305495 | Views on unified TCI extension focusing on m-TRP | Samsung |
| --- | --- | --- | --- |
| 2 | R1-2304963 | Discussion on unified TCI framework extension for multi-TRP | Hyundai Motor Company |
| 3 | R1-2304950 | Discussion of unified TCI framework for multi-TRP | Lenovo |
| 4 | R1-2305583 | Discussion on unified TCI framework extension for multi-TRP | NTT DOCOMO, INC. |
| 5 | R1-2305077 | Discussion on unified TCI framework extension for multi-TRP | CMCC |
| 6 | R1-2305060 | Multi-TRP enhancements for the unified TCI framework | Fraunhofer IIS, Fraunhofer HHI |
| 7 | R1-2304988 | Discussion on unified TCI framework extension for multi-TRP | NEC |
| 8 | R1-2305008 | Discussion on unified TCI framework extension for multi-TRP | Google |
| 9 | R1-2305401 | Unified TCI framework extension for multi-TRP | OPPO |
| 10 | R1-2305288 | Unified TCI framework extension for multi-TRP/panel | LG Electronics |
| 11 | R1-2305318 | Extension of unified TCI framework for mTRP | Qualcomm Incorporated |
| 12 | R1-2305226 | Unified TCI framework extension for multi-TRP | Apple |
| 13 | R1-2304817 | On Unified TCI Framework for multi-TRP | Intel Corporation |
| 14 | R1-2304873 | Unified TCI framework extension for multi-TRP | xiaomi |
| 15 | R1-2304783 | Unified TCI framework extension for multi-TRP | Ericsson |
| 16 | R1-2304760 | Discussion on unified TCI framework extension for multi-TRP | Fujitsu |
| 17 | R1-2304705 | Discussion on unified TCI framework extension for multi-TRP operation | CATT |
| 18 | R1-2304636 | Discussion on unified TCI framework extension for multi-TRP | Huawei, HiSilicon |
| 19 | R1-2304542 | Discussion on unified TCI framework extension for multi-TRP | Spreadtrum Communications |
| 20 | R1-2304421 | Enhanced Unified TCI for mTRP | InterDigital, Inc. |
| 21 | R1-2304375 | Unified TCI framework extension for multi-TRP | Panasonic |
| 22 | R1-2304392 | Enhancements on unified TCI framework extension for multi-TRP | ZTE |
| 23 | R1-2304463 | Further discussion on unified TCI framework extension for multi-TRP | vivo |
| 24 | R1-2304348 | Unified TCI framework extension for multi-TRP | FUTUREWEI |
| 25 | R1-2305892 | Discussion on Unified TCI framework extension for multi-TRP | CEWiT |
| 26 | R1-2305771 | Discussion on unified TCI framework extension for multi-TRP | ITRI |
| 27 | R1-2305776 | Discussion on unified TCI framework extension for multi-TRP | FGI |
| 28 | R1-2305748 | Unified TCI framework extension for multi-TRP | Nokia, Nokia Shanghai Bell |
| 29 | R1-2305733 | Unified TCI framework extension for multi-TRP | Sharp |
| 30 | R1-2305704 | Discussion on unified TCI framework extension for multi-TRP | Transsion Holdings |
| 31 | R1-2305642 | Unified TCI framework extension for multi-TRP | MediaTek Inc. |