3GPP TSG RAN WG1 #113	 R1-2304404

Incheon, Korea, May 22nd – May 26th, 2023

Title:	Draft reply LS to RAN4 on RS supported for group-based reporting

Response to:	R4-2306394

Release:	Release 17

Work Item:	NR_FR2_multiRX_DL-Core

Source:	ZTE

To:	RAN WG4

Cc:	-

Contact Person:

#### Name:

E-mail Address:

1. Overall Description:

RAN1 would like to thank RAN4 for the questions in LS in R4-2306394 on RS supported for group-based reporting and would like to provide the following response.

Question 1: RAN4 understands that two resource sets consisting of SSB + SSB or CSI-RS + CSI-RS can be configured for Rel-17 group-based L1-RSRP measurement and report. However, RAN4 is not clear whether the two resource sets configured for Rel-17 group-based L1-RSRP measurement and report can contain a mix of SSB and CSI-RS. Can RAN1 clarify whether a mix of SSB and CSI-RS can be configured for Rel-17 group-based L1-RSRP measurement and report?

Answer to Question 1: RAN1 confirms that the mix of SSB and CSI-RS can be configured for Rel-17 group based L1-RSRP measurement and report.

Question 2: When CSI-RS resources are configured as resource set for Rel-17 group-based L1-RSRP measurement and report, can the CSI-RS resource with repetition ON be configured for Rel-17 group-based L1-RSRP report?

Answer to Question 2: RAN1 confirms that the CSI-RS resource with repetition ON can be configured for Rel-17 group-based L1-RSRP report. In such case, CRI for the corresponding set in the Rel-17 group-based L1-RSRP report should not be reported.

Question 3: Does RAN1 specification define UE reporting behaviour for the case when NW configures UE with Rel-17 group-based L1-RSRP measurement and report, and UE did not find any pair of beams for report (e.g., the measured L1-RSRP of at least one of the beams of any beam pair is lower than a certain value, e.g., -140dBm)?

Answer to Question 3: From RAN1 perspective, the UE should report the given number of pairs of beams in the report, based on the report configuration, regardless of measured L1-RSRP value. Then, there is no RAN1 specification related to the UE reporting behaviour on not finding any pair of beams for report.

2. Actions:

To RAN2 group.

ACTION: 	RAN1 requests RAN4 to kindly take the above information into account.

3. Date of Next TSG-RAN WG1 Meeting:

TSG-RAN WG1 Meeting #114	August 21st – August 25th, 2023		Toulouse, FR

TSG-RAN WG1 Meeting #114-bis 	October 9th – October 13th, 2023		Xiamen, CN