3GPP TSG-WG1 Meeting #113			R1-2304415

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.11.2

Title:	Low Power WUS receiver architectures

Source:	Nokia, Nokia Shanghai Bell

Document for:	Discussion and Decision

1. Introduction

At the RAN#94-e meeting, it was agreed to study Wake Up Signal and Receivers designs.  These designs are to be primarily targeted at delay and power-sensitive, small form-factor devices, such as industrial sensors, controllers and wearables. Unlike previous power saving study items, the objectives for this study encompasses new signals and new receiver architectures.

In this contribution, we discuss:

- Various LP-WUR architectures for OOK, FSK and OFDMA based LP-WUS.

- The LP-WUR hardware aspects to consider for the:

  - maintenance of timing and frequency synchronisation

  - accommodating fading when mobility is considered

2. Discussion

In the following sub-sections, key observations, proposals and metrics for various waveform–architecture combinations are shared.  Note these discussions assume the following requirements:

Flexible Multi-band support

A device made for worldwide operation supporting multiple bands, can be expected to need to support LP-WUS functionality in different bands. This will require more advanced and more power-hungry LP-WUR architectures.

WUS waveforms that require minimal guard bands

To minimise guard bands and maximise network spectral efficiency, architectures will be required to support and maintain accurate frequency synchronisation.  This will impose minimum capabilities on the LP-WUR filters and frequency tracking capabilities.

2.1   Comparison of the various WUS modulation/waveform and architecture combinations

To expediate the comparison of different WUS waveform and receiver architecture combinations, the feature lead has shared the template below, for companies to capture key metrics (e.g. power consumption, noise figure).

| Company-Index # | Company-OOK01 |
| --- | --- |
| Reference tdoc | R1-23044 |
| WUS waveform |  |
| LP WUR achitecture type |  |
| Relative power consumption (ON) |  |
| Relative power consumption (OFF) |  |
| Noise figure |  |
| WUS bandwidth (including guard band) |  |
| Guard band for WUS |  |
| Frequency location within a carrier |  |
| Presence of RF LNA, and the corresponding gain, if any |  |
| Local oscillator, PLL/FLL, and the assumed time/frequency impairments |  |
| Presence of IF/BB AMP, and the corresponding gain, if any |  |
| ADC sampling rate |  |
| ADC bit-width |  |
| RF/IF/BB filter characteristics (e.g. type of filter, order, cut-off frequency/frequencies) |  |
| Baseband processing |  |
| Frequency band(s) |  |
| Support of band/carrier tuning |  |
| Adjacent channel interference rejection capability |  |
| Adjacent subcarrier interference rejection capability |  |
| Handling of inter-cell interference |  |
| Duty cycle handling of WUS and other signals (if any) |  |
| Whether there is any mobility support function, e.g. measurement capability |  |
| Other (e.g. AGC, …) |  |

Table 1:   Template for capturing key architecture related metrics

For the power consumption metric, we attempt to provide ranges in terms of the MR deep sleep state, as agreed in the following RAN1#112 agreement.

Agreement

For the study on LP WUR architecture, power consumption relative to the deep sleep state of the MR is provided.

- Deep sleep state of non-RedCap UE should be assumed

2.1.1   MC-OOK

In this sub-section, three receiver architectures (listed below) for MC-OOK are evaluated.

- RF Envelope detector

- IF Envelope Detector

- Zero-IF receiver

The RF Envelope detector receiver architecture is illustrated in Figure 1. The RF envelope detection receiver architecture is mainly aimed for single band usage, i.e., bands within the bandwidth of the RF envelope detector. The application of a tuneable RF filter in the RF path will affect the NR Main Radio performance (insertion loss, higher NF etc) when the antenna and RF frontend are reused. If the RF envelope detector LP-WUR is preferred, it would be beneficial to have full separation (no reuse) of the NR Main radio and the LP-WUR.

Figure 1 Illustration of a MC-OOK receiver architecture with RF envelope detector.

The IF envelope detection Heterodyne receiver architecture is aimed for multi band usage. This architecture allows for an easy re-use of the main radio elements, such as the NR frontend, antennas, LNA and potentially also the mixer. The IF envelope detection Heterodyne receiver architecture is illustrated in Figure 2.

Figure 2 Illustration of the Heterodyne receiver architecture with IF Envelope detection

A Zero-IF receiver can be relatively easily modified to reuse many of the existing NR OFDM based main radio components, including the frontend, antennas, LNA, mixer ADC and parts of the uP/statemachine as illustrated in Figure 3. One benefit of reusing the NR main radio parts, is that it would readily support flexible bandwidth and placement of the LP-WUS within different bands.

Figure 3 Illustration of OFDM/ Zero-IF Receiver architecture

For the analysis of the three receiver architectures with respect to MC-OOK modulation following assumption for the evaluation results in Table 1 is following:

- MC-OOK Bandwidth 1.44MHz

  - 6 sub-carriers with a sub-carrier spacing of 240 kHz

- 5MHz total bandwidth for the WUS including guard band

- Assumed 4 bits (8 Manchester encoded) MC-OOK data for each NR OFDM symbol

  - OOK-1 implementation, with 8 bits per symbol, which are Manchester Encoded, enabling support for 4 information bits.

Table 2: Analysis of MC-OOK with 1.44MHz signal bandwidth

| Company-Index # | Nokia-OOK-1 | Nokia-OOK-1 | Nokia-OOK-1 |
| --- | --- | --- | --- |
| Reference Tdoc | R1-2304415 | R1-2304415 | R1-2304415 |
| WUS waveform | OOK-1 | OOK-1 | OOK-1 |
| LP WUR architecture type | RF Envelope detector | IF envelope detector | Zero IF |
| Relative power consumption (ON) | 4 (Note 1) | 12 (Note 2) | 12 (Note 3) |
| Relative power consumption (OFF) | 0.0015 (Note 4) | 0.0015 (Note 4) | 0.0015 (Note 4) |
| Noise figure | 31 dB | 10 dB | 9 dB |
| WUS bandwidth (including guard band) | 1.44MHz (5MHz) | 1.44MHz (5MHz) | 1.44MHz (5MHz) |
| Guard band for WUS | 2x 1.780MHz | 2x 1.780MHz | 2x 1.780MHz |
| Frequency location within a LP-WUS carrier | Centre of 5MHz BWP | Centre of 5MHz BWP | Centre of 5MHz BWP |
| Presence of RF LNA, and the corresponding gain, if any | RF LNA 15dB | RF LNA 15dB | RF LNA 15dB |
| Local oscillator, PLL/FLL, and the assumed time/frequency impairments | RTC would be needed for time track during DRx cycle. Time tuning / update is needed. | RTC. Frequency Accuracy +/-50 ppm + tuning option Drift: 0.1ppm/s | RTC. Frequency Accuracy +/-50 ppm + tuning option Drift: 0.1ppm/s |
| Presence of IF/BB AMP, and the corresponding gain, if any | BB Amp 40dB | IF AMP 20dB | BB amp 40dB |
| ADC sampling rate | 480KHz | 480KHz | 1440KHz |
| ADC bit-width | Bit-width: 4.16 us | Bit-width: 4.16 us | Bit-width: 4.16 us |
| RF/IF/BB filter characteristics (e.g. type of filter, order, cut-off frequency/frequencies) | Tuneable BPF filter 10-15 dB sup. @ 2.5 MHz | SAW BPF IF filter 20-25dB sup.@ 2.5 MHz | 5th order LPF 20-30 dB sup. @ 2.5 MHz |
| Baseband processing | State machine including functionality for: ISI, AGC, time correction, level estimation and decoding. | Statemachine including functionality for: ISI, AGC, time and frequency correction, level estimation and decoding. | Statemachine including functionality for: ISI, AGC, time and frequency correction, level estimation and decoding. |
| Frequency band(s) | Assumed this type of receiver can support 1-2 bands. | Same as main receiver | Same as main receiver |
| Support of band/carrier tuning | Assumed this type of receiver can support 1-2 bands. Can only support fixed number of carriers | Multiband support and carrier tuning | Multiband support and carrier tuning |
| Adjacent channel interference rejection capability | As Main radio. Same BPF is used. | As Main radio. Same BPF is used. | As Main radio. Same BPF is used. |
| Adjacent subcarrier interference rejection capability | ~10dB rejection. | ~20dB rejection. | ~30dB rejection. |
| Handling of inter-cell interference | NA | NA | NA |
| Duty cycle handling of WUS and other signals (if any) | DRx included | DRx included | DRx included |
| Whether there is any mobility support function, e.g. measurement capability | Not possible as only few bands are supported | Yes | Yes |
| Other (e.g. AGC, …) | AGC needed for mobility | AGC needed for mobility | AGC needed for mobility |
| Additional info | Can be Full standalone WUR receiver. Optional reuse of frontend band pass filter. | Frontend, LNA, Mixer and optional IF amplifier can be reused from main radio. | High degree of reuse from MR components is possible. |

Note 1: The relative power consumption for the RF envelope receiver is quite low with respect to the RF HW section. The baseband processing is same across all the receiver types. As the baseband is required to do the decoding, maintain time tracking, handle the AGC for mobility and potentially also perform RRM measurements, the power consumption will not be negligible.

Note 2: The baseband is identical to the baseband section for the RF envelope detector but extended with a frequency offset detection algorithm and functionality to programme the FLL/PLL.

Note 3: The baseband section has same functionality as the baseband section for the IF envelope detector with increased ADC sample rate.

Note 4: Relative power consumption (OFF) is based on the Deep Sleep state agreed in RAN1#112. The content of the OFF state should be evaluated further, e.g., if RTC and wakeup timer functionality for DRx is considered and if some memory is retained for the LP-WUR.

In Table 2 is the same analysis done as in Table 1 but with different data bandwidth:

- MC-OOK Bandwidth 4.32MHz

  - 18 sub-carriers with a sub-carrier spacing of 240 kHz

- 5MHz total bandwidth for the WUS including guard band

- Assumed 4 bits (8 Manchester encoded) MC-OOK data for each NR OFDM symbol

  - OOK-1 implementation, with 8 bits per symbol, which are Manchester Encoded, enabling support for 4 information bits.

Table 3: Analysis of MC-OOK with 4.32MHz signal bandwidth

| Company-Index # | Nokia-OOK-1 | Nokia-OOK-1 | Nokia-OOK-1 |
| --- | --- | --- | --- |
| Reference tdoc | R1-2304415 | R1-2304415 | R1-2304415 |
| WUS waveform | OOK-1 | OOK-1 | OOK-1 |
| LP WUR architecture type | RF Envelope detector | IF envelope detector | Zero IF |
| Relative power consumption (ON) | 4 | 12 | 12 |
| Relative power consumption (OFF) | 0.0015 | 0.0015 | 0.0015 |
| Noise figure | 31 dB | 10 dB | 9 dB |
| WUS bandwidth (including guard band) | 4.324MHz (5MHz) alternative 10MHz Note 1 | 4.32MHz (5MHz) alternative 10MHz Note 1 | 4.32MHz (5MHz) alternative 10MHz Note 1 |
| Guard band for WUS | 2x 340KHz Alternative 2x3840KHz | 2x 340KHz Alternative 2x3840KHz | 2x 340KMHz Alternative 2x3840KHz |
| Frequency location within a LP-WUS carrier | Centre of 5MHz/ 10MHz BWP | Centre of 5MHz/ 10MHz BWP | Centre of 5MHz/ 10MHz BWP |
| Presence of RF LNA, and the corresponding gain, if any | RF LNA 15dB | RF LNA 15dB | RF LNA 15dB |
| Local oscillator, PLL/FLL, and the assumed time/frequency impairments | RTC would be needed for time track during DRx cycle. Time tuning / update is needed. | RTC. Frequency Accuracy +/-50 ppm + tuning option Drift: 0.1ppm/s | RTC. Frequency Accuracy +/-50 ppm + tuning option Drift: 0.1ppm/s |
| Presence of IF/BB AMP, and the corresponding gain, if any | BB Amp 40dB | IF AMP 20dB | BB amp 40dB |
| ADC sampling rate | 480KHz | 480KHz | 4320KHz |
| ADC bit-width | Bit-width: 4.16 us | Bit-width: 4.16 us | Bit-width: 4.16 us |
| RF/IF/BB filter characteristics (e.g. type of filter, order, cut-off frequency/frequencies) | Tuneable BPF filter [<6dB] sup. @ 2.5 MHz 10 dB sup @+/-5MHz | SAW BPF IF filter 6dB sup.@+/-2.5 MHz 20 dB sup @+/-5MHz | 5th order LPF 6 dB sup. @ +/-2.5 MHz >30 dB sup @+/-5MHz |
| Baseband processing | State machine including functionality for:ISI, AGC, time correction, level estimation and decoding. | State machine including functionality for:ISI, AGC, time and frequency correction, level estimation and decoding. | State machine including functionality for:ISI, AGC, time and frequency correction, level estimation and decoding. |
| Frequency band(s) | Assumed this type of receiver can support 1-2 bands. | Same as main receiver | Same as main receiver |
| Support of band/carrier tuning | Assumed this type of receiver can support 1-2 bands. Can only support fixed number of carriers | Multiband support and carrier tuning | Multiband support and carrier tuning |
| Adjacent channel interference rejection capability | As Main radio. Same BPF is used. | As Main radio. Same BPF is used. | As Main radio. Same BPF is used. |
| Adjacent subcarrier interference rejection capability | <6dB rejection. Alternative 10 dB | App 6dB rejection. Alternative 20 dB | App 6dB rejection. Alternative >30 dB |
| Handling of inter-cell interference | NA | NA | NA |
| Duty cycle handling of WUS and other signals (if any) | DRx included | DRx included | DRx included |
| Whether there is any mobility support function, e.g. measurement capability | Not possible aas only few bands are supported | Yes | Yes |
| Other (e.g. AGC, …) | AGC needed for mobility | AGC needed for mobility | AGC needed for mobility |
| Additional info | Can be Full standalone WUR receiver. Optional reuse of frontend band pass filter. | Frontend, LNA, Mixer and optional IF amplifier can be reused from main radio. | High degree of reuse from MR components is possible. |

Note 1: Based on the low Adjacent subcarrier interference rejection capability, the bandwidth including guard band should be 10MHz if the WUS signal bandwidth is 4.32MHz.

Observation 1:   	Tuneable filters capable of providing a RF envelope detector architecture with the flexibility to support multiband bands along with sufficient adjacent subcarrier suppression for fading in low SNR conditions, are not readily available.

Observation 2:	The RF Envelope detector receiver architecture does not require a reference oscillator or PLL/FLL if the baseband processing section is not considered as a part of the LP-WUR architecture. If a baseband processing/state machine is considered as part of the architecture, then there is a need for a reference clock to ensure reliable processing and sampling of the ADC. The RTC should also be used for DRx cycling.

Observation 3:       	For the Zero-IF receiver and the Heterodyne receiver architecture with IF Envelope detection, the uncertainty of the reference oscillator will affect the ability of the receiver to maintain the ideal centre frequency. Any offset from the ideal centre frequency offset will degrade the LP-WUR performance, due to NR data being included in the receiver pass band.

Observation 4:	The size of the guard bands needed are linked to how effectively the receiver architecture can minimise frequency offset.

Observation 5:        A frequency control loop can be added to the Zero-IF receiver and the Heterodyne receiver architectures to increase the maximum amount of frequency drift that can be tolerated.

Observation 6:  	The RF Envelope detector receiver architecture filter with multi-carrier capability, is dependent on the support of a tuneable filter. The accuracy of the tuneable filter required to support reasonable guard bands will require unrealistically high component tolerances for most applications.

Proposal 1:	RAN1 defines a common use case to allow fairer comparison of the power consumption estimates for the various waveform-architecture combinations shared by different companies.of the power consumption

Proposal 2:	A maximum level of frequency drift that an architecture can tolerate, is defined, and accounted for in the dimensioning of the signal and associated guard bands. For architectures with reference clock devices with larger frequency tolerances frequency compensation/ correction must be applied.

Proposal 3: 	For the homodyne/zero-IF architecture with baseband envelope detection for OOK, the characteristics for the receiver architecture is listed in Table 2 and in Table 3.

Proposal 4:	For the heterodyne/ IF envelope detector architecture with baseband envelope detection for OOK, the characteristics for the receiver architecture is listed in Table 2 and in Table 3.

Proposal 5:	For the RF envelope detector architecture with baseband envelope detection for OOK, the characteristics for the receiver architecture is listed in Table 2 and in Table 3.

Proposal  6:	Study of the RF Envelope detector receiver architecture is deprioritised.

2.1.2   FSK

In this subsection we compare three FSK architectures. All these architectures are compared assuming BFSK as the reference FSK modulation scheme.

Figure 4 Illustration of a NR band with BFSK modulation located in the middle of the band.

The parallel homodyne receiver architecture is based on 2 zero-IF LP-WUS receivers working in parallel to detect the BFSK “mark” and “space” bits. The parallel homodyne receiver architecture is illustrated in Figure 5.

Figure 5 Parallel homodyne BFSK receiver

The parallel heterodyne receiver architecture is based on 2 heterodyne architectures with IF envelope detection receivers working in parallel to detect the BFSK “mark” and “space” bits. The parallel heterodyne receiver architecture is illustrated in Figure 6.

Figure 6 Parallel heterodyne BFSK receiver

The BFSK Quadrature demodulator is based on two Homodyne receiver with frequency to amplitude conversion. The carrier frequency in located in the middle of the FSK signal as illustrated in Figure 4 fo_alternate .

Figure 7 BFSK Quadrature demodulator

For the evaluation of these three receiver architectures presented in Table 3, the following assumptions were made:

- MC-FSK Bandwidth of 1.44MHz and 4.32MHz is assumed, where the guard band is equally distributed beside and between the FSK signals. For 1.44MHz is the total bandwidth (WUS bandwidth including guard band) set to 5MHz. For WUS bandwidth of 4.32MHz is the total bandwidth (WUS bandwidth including guard band) set to 10MHz

  - With equal distributed guard band and band separation between the two FSK bands and a signal bandwidth of 1.44MHz will have (5MHz-1.44MHz)/3 = 1187KHz guard-band and band separation..

With equal distributed guard band and band separation between the two FSK bands and a signal bandwidth of 4.32MHz will have (10MHz-4.32MHz)/3 = 1890KHz guard-band and band separation..

- 4 bits MC-FSK data for each NR symbol

Table 4 Analysis of MC-FSK receiver architectures

| Company-Index # | Nokia-FSK | Nokia-FSK | Nokia-FSK |
| --- | --- | --- | --- |
| Reference tdoc | R1-2304415 | R1-2304415 | R1-2304415 |
| WUS waveform | FSK-1 | FSK-1 | FSK-1 |
| LP WUR architecture type | Parallel homodyne | Parallel heterodyne | Homodyne receiver with frequency to amplitude conversion |
| Relative power consumption (ON) | 12 | 14 | 14 |
| Relative power consumption (OFF) | 0.0015 | 0.0015 | 0.0015 |
| Noise figure | 9 dB | 10 dB | 9 dB |
| WUS bandwidth (including guard band) | 1.44MHz (5MHz) 4.324MHz (10MHz) | 1.44MHz (5MHz) 4.324MHz (10MHz) | 1.44MHz (5MHz) 4.324MHz (10MHz) |
| Guard band for WUS | (5MHz) 3x 1187KHz ((10MHz) 3x1890KHz | 5MHz) 3x 1187KHz ((10MHz) 3x1890KHz | 5MHz) 3x 1187KHz ((10MHz) 3x1890KHz |
| Frequency location within a LP-WUS carrier | Centre of 5MHz/ 10MHz BWP | Centre of 5MHz/ 10MHz BWP | Centre of 5MHz/ 10MHz BWP |
| Presence of RF LNA, and the corresponding gain, if any | RF LNA 15dB | RF LNA 15dB | RF LNA 15dB |
| Local oscillator, PLL/FLL, and the assumed time/frequency impairments | RTC would be needed for time track during DRx cycle. Time tuning / update is needed. | RTC. Frequency Accuracy +/-50 ppm + tuning option Drift: 0.1ppm/s | RTC. Frequency Accuracy +/-50 ppm + tuning option Drift: 0.1ppm/s |
| Presence of IF/BB AMP, and the corresponding gain, if any | BB Amp 40dB | IF AMP 20dB | BB amp 40dB |
| ADC sampling rate | 480KHz | 480KHz | 1440KHz/ 4320KHz |
| ADC bit-width | Bit-width: 4.16 us | Bit-width: 4.16 us | Bit-width: 4.16 us |
| RF/IF/BB filter characteristics (e.g. type of filter, order, cut-off frequency/frequencies) | 5th order LPF 20/30 dB sup. | 5th order LPF 20/30 dB sup. | 5th order LPF 20/30 dB sup. |
| Baseband processing | State machine including functionality for: ISI, AGC, time and frequency correction, level estimation and decoding. | State machine including functionality for: ISI, AGC, time and frequency correction, level estimation and decoding. | State machine including functionality for: ISI, AGC, time and frequency correction, level estimation and decoding. |
| Frequency band(s) | Same as main receiver | Same as main receiver | Same as main receiver |
| Support of band/carrier tuning | Multiband support and carrier tuning | Multiband support and carrier tuning | Multiband support and carrier tuning |
| Adjacent channel interference rejection capability | As Main radio. Same BPF is used. | As Main radio. Same BPF is used. | As Main radio. Same BPF is used. |
| Adjacent subcarrier interference rejection capability | 20/30 dB rejection. | 20/30 dB rejection. | 20/30 dB rejection. |
| Handling of inter-cell interference | NA | NA | NA |
| Duty cycle handling of WUS and other signals (if any) | DRx included | DRx included | DRx included |
| Whether there is any mobility support function, e.g. measurement capability | Yes | Yes | Yes |
| Other (e.g. AGC, …) | AGC needed for mobility | AGC needed for mobility | AGC needed for mobility |
| Additional info | Frontend, LNA, Mixer and optional IF amplifier can be reused from main radio. | Frontend, LNA, Mixer and optional IF amplifier can be reused from main radio. | High degree of reuse from MR components is possible. |

Observation 7:     	To keep same total bandwidth, the guard-band bandwidth is decreased relative to the comparable MC-OOK modulation approach.

Observation 8:	The 1.44 MHz modulation bandwidth for MC-FSK within a 5 MHz bandwidth, enables wider guard bands to be applied, thereby simplifying the complexity of the filters required.

Proposal 7:	Evaluate the main functionality in the receiver baseband section. The power consumption and “ON” time depends on the functionality.

2.1.3   OFDMA baseband receiver evaluation

In following sub-section, the OFDMA receiver is analysed to highlight potential simplifications that could be made for the LP-WUR to reduce power consumption.

For this evaluation, the proposed signal bandwidth is 1.44MHz and the subcarrier spacing is 30KHz and the comparison is made relative to SSB processing.

Figure 8 illustrates the main sub-components of the OFDMA receiver being analysed.

Figure 8 Illustration of the OFDMA receiver

Table 5 comparison between an SSB and LP-WUS receiver with OFDM modulation

| Component | SSB | LP-WUS | Improvements/ Comments |
| --- | --- | --- | --- |
| RF | BW 7.2 MHz (20RB SCS=30KHz | BW 1.44MHz (4RB SCS=30KHz) | No major improvements in the RF section. Minor improvements might be done in the PLL due to less required tolerances. |
| ADC | Sample rate 14.4MHz | Sample Rate 2.88MHz | Assuming linear relation between sample rate and current consumption the saving is 6x |
| Remove CP | NA | NA | The saving will be in the range of 3-4x (Highly depending on the HW implementation) |
| S/P | NA | NA | The saving will be in the range of 3-4x (Highly depending on the HW implementation) |
| FFT | FFT Size 512 | FFT Size 64 | From simulation this will save a factor of 16x |
| Subcarrier De-mapping | NA | NA | The saving will be in the range of 3-4x (Highly depending on the HW implementation) |
| IDFT | NA | NA | The saving is expected to be in the range of 10x |
| P/S | NA | NA | The saving will be in the range of 3-4x (Highly depending on the HW implementation) |
| Demodulation | NA | NA | The saving will be in the range of 3-4x (Highly depending on the HW implementation) |
| Ref clock | 0.1ppm | 10ppm | By keeping the total BW=5MHz even higher tolerances can be accepted. |

Given the ranges expressed above, the table below, provides our estimate of the power consumption of a DFT based OFDMA receiver.

Table 6: Overview of the OFDM LP-WUR

| Company-Index # | Nokia-OFDM |
| --- | --- |
| Reference tdoc | R1-2304415 |
| WUS waveform | OFDM |
| LP WUR architecture type | Same as main radio |
| Relative power consumption (ON) | 12 Note 1 (Redcap) |
| Relative power consumption (OFF) | 0.8 (Same as RedCap device)Note 2 |
| Noise figure | Same as main radio |
| WUS bandwidth (including guard band) | 1.44MHz (5MHz) |
| Guard band for WUS | 2x 1780KHz |
| Frequency location within a LP- WUS carrier | Centre of 5MHz |
| Presence of RF LNA, and the corresponding gain, if any | Same as main radio |
| Local oscillator, PLL/FLL, and the assumed time/frequency impairments | RTC would be needed for time track during DRx cycle. Time tuning / update is needed. |
| Presence of IF/BB AMP, and the corresponding gain, if any | Same as main radio |
| ADC sampling rate | 2880KHz |
| ADC bit-width | Same as main radio |
| RF/IF/BB filter characteristics (e.g. type of filter, order, cut-off frequency/frequencies) | 5th order LPF 20/30 dB sup. |
| Baseband processing | Same as main radio with reduced functionality as described in Table 4 |
| Frequency band(s) | Same as main receiver |
| Support of band/carrier tuning | Multiband support and carrier tuning |
| Adjacent channel interference rejection capability | As Main radio. Same BPF is used. |
| Adjacent subcarrier interference rejection capability | 20/30 dB rejection. |
| Handling of inter-cell interference | NA |
| Duty cycle handling of WUS and other signals (if any) | DRx included |
| Whether there is any mobility support function, e.g. measurement capability | Yes |
| Other (e.g. AGC, …) | AGC needed for mobility |
| Additional info | Same as main radio |

Note 1: The estimate is based on further optimization of the RedCap SSB power consumption estimate using the techniques listed Table 5.

Note : Relative power consumption (OFF) is derived from RedCap specification. OFF is DEEP SLEEP and defined to be 0.8.

Observation 9:  	Table 4 illustrates how power consumption savings that could be made for a LP-WUS OFDM receiver compared an OFDM receiver used to detect SSB. The major savings come from the reduced bandwidth, the reduced sampling rate and the reduction in the FFT size.

Observation 10:	By using an OFDM based WUS, no additional HW would be needed for the device. This should make it simpler to use WUS in dedicated mode without any HW modifications.

Proposal 8:	For the OFDM signal based detector architecture, the expected relative power consumption is given by Table 6.

2.2   LP-WUR considerations for mobility, Time, and Frequency synchronisation

As we have discussed in previous contributions [2], the LR ability to accurately maintain timing and frequency, has significant impacts on the guard bands required and the ability to support mobility.  In this sub-section we provide our thoughts on:

- Hardware aspects to consider for the maintenance LR timing and frequency synchronisation

- Mobility considerations - how to handle gain settings and fading

2.2.1  Hardware aspects to consider for the maintenance LR timing and frequency synchronisation

In the previous sections it is assumed the LP-WUR is only active when the WUS is expected to be received. This means that timing control for enabling the LP-WUR would be needed. To ensure correct timing even with a low precision clock would require correction and/or synchronisation. Figure 9 below, illustrates an example of a sync sequence. In the figure, the sync sequence is illustrated as an alternating 1-0- pattern. The sync sequence pattern could be any unique sequence, the only requirement is that it is known by the receiver and is simple to detect. The sync sequence (i.e a known sequence) could also be repeated at the end of the WUS to provide a more accurate timing estimate. The decoded information can also be used to calibrate an equalizer receiver type to perform ISI/match filter compensation.

Figure 9 illustration of a sync sequence

In Figure 10, a full DRx cycle is illustrated. If the receiver determines the time offset estimate based on the time reference points for two or more DRx periods – the estimated time offset can also be used to estimate the frequency offset. This can be done by estimating the time offset in ppm and applying the same offset in ppm to the frequency correction circuit.

Figure 10 Illustration of a DRx=1.28s sequence

Observation 11:  	For accurate wakeup time for a LP-WUR, the reference time needs to be continuously adjusted/corrected

Proposal 9: 	Introduce a time control loop in the evaluation of the LP-WUR. The introduction of a sync sequence as a part of the WUS and/or Beacon should be considered.

3. Conclusion

In this contribution, we have discussed:

- Various LP-WUR architectures for OOK, FSK and OFDMA based LP-WUS.

- The LP-WUR hardware aspects to consider for the:

  - maintenance of timing and frequency synchronisation

  - accommodating fading when mobility is considered

From those discussions, we have the following observations and proposals.

Observation 1:   	Tuneable filters capable of providing a RF envelope detector architecture with the flexibility to support multiband bands along with sufficient adjacent subcarrier suppression for fading in low SNR conditions, are not readily available.

Observation 2:	The RF Envelope detector receiver architecture does not require a reference oscillator or PLL/FLL if the baseband processing section is not considered as a part of the LP-WUR architecture. If a baseband processing/state machine is considered as part of the architecture, then there is a need for a reference clock to ensure reliable processing and sampling of the ADC. The RTC should also be used for DRx cycling.

Observation 3:       	For the Zero-IF receiver and the Heterodyne receiver architecture with IF Envelope detection, the uncertainty of the reference oscillator will affect the ability of the receiver to maintain the ideal centre frequency. Any offset from the ideal centre frequency offset will degrade the LP-WUR performance, due to NR data being included in the receiver pass band.

Observation 4:	The size of the guard bands needed are linked to how effectively the receiver architecture can minimise frequency offset.

Observation 5:        A frequency control loop can be added to the Zero-IF receiver and the Heterodyne receiver architectures to increase the maximum amount of frequency drift that can be tolerated.

Observation 6:  	The RF Envelope detector receiver architecture filter with multi-carrier capability, is dependent on the support of a tuneable filter. The accuracy of the tuneable filter required to support reasonable guard bands will require unrealistically high component tolerances for most applications.

Observation 7:     	To keep same total bandwidth, the guard-band bandwidth is decreased relative to the comparable MC-OOK modulation approach.

Observation 8:	The 1.44 MHz modulation bandwidth for MC-FSK within a 5 MHz bandwidth, enables wider guard bands to be applied, thereby simplifying the complexity of the filters required.

Observation 9:  	Table 5, illustrates the power consumption savings that could be made for a LP-WUS OFDM receiver compared an OFDM receiver used to detect SSB. The major savings come from the reduced bandwidth, the reduced sampling rate and the reduction in the FFT size.

Observation 10:	By using an OFDM based WUS, no additional HW would be needed for the device. This should make it simpler to use WUS in dedicated mode without any HW modifications.

Proposal 1:	RAN1 defines a common use case to allow fairer comparison of the power consumption estimates for the various waveform-architecture combinations shared by different companies.of the power consumption

Proposal 2:	A maximum level of frequency drift that an architecture can tolerate, is defined, and accounted for in the dimensioning of the signal and associated guard bands. For architectures with reference clock devices with larger frequency tolerances frequency compensation/ correction must be applied.

Proposal 3: 	For the homodyne/zero-IF architecture with baseband envelope detection for OOK, the characteristics for the receiver architecture is listed in Table 2 and in Table 3.

Proposal 4:	For the heterodyne/ IF envelope detector architecture with baseband envelope detection for OOK, the characteristics for the receiver architecture is listed in Table 2 and in Table 3.

Proposal 5:	For the RF envelope detector architecture with baseband envelope detection for OOK, the characteristics for the receiver architecture is listed in Table 2 and in Table 3.

Proposal 6:	Study of the RF Envelope detector receiver architecture is deprioritised.

Proposal 7:	Evaluate the main functionality in the receiver baseband section. The power consumption and “ON” time depends on the functionality.

Proposal 8:	For the OFDM signal based detector architecture the expected relative power consumption is given by Table 6.

Proposal 9: 	Introduce a time control loop in the evaluation of the LP-WUR. The introduction of a sync sequence as a part of the WUS and/or Beacon should be considered.

4. References

[1]  RP-222644    Revised SID: Study on low-power Wake-up Signal and Receiver for NR       Vivo

[2]  R1-2304417  L1 signal design and procedures for low power WUS     Nokia, Nokia Shanghai Bell