3GPP TSG RAN WG1 #113	R1-2304435

Incheon, Korea, 22nd – 26th May, 2023

Agenda item:		9.9.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Aspects related to network verified UE location for NR over NTN

Document for:		Discussion and Decision

# Introduction

In RAN#98 meeting, the following WID objective was defined for network-verified UE location [1]:

Based on RAN1 conclusions of the study phase, RAN to prioritize the specification of necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view [RAN1, 2, 3, 4]. DL-TDoA methods for verification may be considered as lower priority and if time permits and condition in Note is satisfied. Note 1: Enhancements assume reuse of the RAT dependent positioning framework. Note 2: The specification of DL-TDOA enhancements will be subject to the study of the impact of realistic UE clock drift onto DL-TDOA performance. Note 3: The target accuracy for position verification purposes is as documented in clause « recommendations » of the 3GPP TR 38.882 (i.e. 10 km granularity). Note 4 : Multiple satellite in view by the UE may be considered if time allows. Note 5 : The enhancements may be subject to relevant SA WGs (e.g. SA3/SA3-LI) feedbacks on the reliability of UE reports involved. Note 6 : The enhancements should take into account the mirror-image ambiguity. Note 7 : Network verified UE location is an optional UE feature.

Based on the RAN plenary outcome, this contribution focuses on an analysis of the refined objectives and presents observations and proposals for development of the target features.

# Discussion

## Reference point for the gNB Rx-Tx time difference

For terrestrial deployments, the specification TS 38.215 in [2] defines TgNB-Rx and TgNB-Tx which are the reference points to measure the time difference at the gNB. The time difference definition says as follows:

The gNB Rx – Tx time difference is defined as TgNB-RX – TgNB-TX Where: TgNB-RX is the Transmission and Reception Point (TRP) [18] received timing of uplink subframe #i containing SRS associated with UE, defined by the first detected path in time. TgNB-TX is the TRP transmit timing of downlink subframe #j that is closest in time to the subframe #i received from the UE. Multiple SRS resources can be used to determine the start of one subframe containing SRS. The reference point for TgNB-RX shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. The reference point for TgNB-TX shall be: - for type 1-C base station TS 38.104 [9]: the Tx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Tx antenna (i.e. the centre location of the radiating region of the Tx antenna), - for type 1-H base station TS 38.104 [9]: the Tx Transceiver Array Boundary connector.

For NTN, the multi-RTT positioning method requires enhancements to accomodate large propagation delays ,which are greater than one subframe duration. Furthermore, these enhancements require considering the role of the satellite access node in a transparent architecture; that is where the physical time measurements are taken and what is the timing point used by the LMF to estimate the round-trip time (RTT).

Extensive discussions during the last RAN1 meetings (see [3]) led to the following proposed working assumption:

Proposed working assumption In NTN, for the definition of the gNB Rx – Tx time difference reported to the LMF, further discuss the following two options for the reference point of the definition: Option 1: The uplink time synchronization reference point Option 2: The reference point is on the gNB Note: if option 1 and option 2 are both not feasible, RAN1 can consider defining the reference point on the satellite.

The proposed working assumption considers two options for the reference point definition. The first definition places the reference point for the gNB Rx-Tx time difference at an arbitrary point between the satellite and the gNB, while the second option considers the reference point at the gNB.

In our view, there are some aspects that require clarification before reaching a final agreement.

Role of the satellite access node

Since Rel-17, the specification work has focused on the transparent payload. That means the satellite access node acts as a “mirror” node, where signals coming from the UE are amplified and repeated towards the gNB on ground, and viceversa. Therefore, transparent satellites do not transport on-board any 3GPP functionality. Some companies have argued that the transmission and reception point (TRP) should be onboard the satellite. In our view, this is not possible for transparent payloads because it requires LPP functionalities on board. If physical time measurements (i.e., TgNB-Rx and TgNB-Tx) are conducted at the satellite, then the satellite’s payload is no longer a transparent node.

Observation 1: Satellites with transparent payloads cannot conduct 3GPP protocol functionalities.

TgNB-Rx and TgNB-Tx are physical time measurements

The definition of TgNB-Rx and TgNB-Tx in TS 38.215 [2] refer to physical time measurements taken at the TRP associated to the gNB. In TN, it is straightforward where are the measurement points; signals only travel through one interface and between two nodes (i.e., UE and gNB). However, in NTN the scenario is more complex with the addition of the satellite access node between the UE and the gNB. Hence, there should be a differentiation between the measurement point (i.e., where the physical time measurements are taken) and the timing point (i.e., the virtual reference point that the LMF uses to estimate the RTT).

As shown in Figure 1, time measurements TgNB-Rx and TgNB-Tx shall be taken in a 3GPP entity with LPP functionalities. For that reason, the only possible place is at the gNB.

Observation 2: Defining the measurement point for TgnB-Rx and TgNB-Tx at the SAN is not feasible for transparent architectures which has been the reference architecture for discussions since Release 17.

Figure 1 Multi-RTT scenario showing where the timing point and the measurement point are.

Timing point used by the LMF

As argued before, the measurement point should not be the same as the timing point. The latter is merely a virtual point used by the LMF to derive the RTT. Using time measurements taken at the gNB and additional assistance information, the LMF should be able to accuretaly estimate the RTT between the UE and the satellite access node.

Observation 3: Based on gNB Rx-Tx time difference measurements taken at the measurement point, the LMF can infer the Rx-Tx time difference at the timing point (i.e. satellite).

Proposal 1: RAN1 to decide that measurement point is the point where time measurements are conducted and that it is placed at the gNB.

## UE and gNB receive-transmit time difference measurements

In RAN1#112-bis meeting, the following agreement was taken [3]:

Agreement For RTT determination in NTN, discuss further the accuracy, and reporting details of combinations of the following UE and gNB receive-transmit time difference measurements: - Alt-1: UE Rx-Tx time difference based on Option 3 and gNB Rx-Tx time difference as defined in TS 38.215. - Note 1: The signaling method of UE Rx-Tx time difference definition option 1 is not precluded if Alt1 is adopted - Alt-2: UE Rx-Tx time difference based on Option 2 and gNB Rx-Tx time difference as defined in TS 38.215. - Note 2: The LMF will use the time stamp of the PRS and the time stamp of SRS to calculate the time difference between the transmission of PRS and the reception of SRS - Alt-3: UE Rx-Tx time difference based on Option 2 and gNB Rx-Tx time difference based on Option 4 FFS: One or multiple SRS can be used in determining the arrival time FFS: Additional enhancement including additional information to be reported, if justified Note 3: The impact of UE autonomous adjustment of TA (when applied) should be taken into account Note 4: The gNB Rx-Tx time difference option in the above alternatives may need updates accordingly based on the outcome of discussion on reference point for the gNB Rx – Tx time difference

Definition of Rx-Tx time difference measurements: when time measurements are conducted

During the last RAN1 meeting, the pros and cons of different options proposed for UE and gNB Rx-Tx time difference were discussed without reaching a final agreement. As shown above, three alternatives were proposed.

Alternative-1 adopts the legacy definitions of UE and gNB Rx-Tx time difference as defined in TS 38.215 [2]. Further, it considers the reporting of an offset to shift the UE Rx-Tx time difference measurements. In our view, this option holds a good compromise between accuracy, signalling and specification effort. Alternative-3 considers specification changes that require new definitions for both UE and gNB Rx-Tx time difference. This would have a high specification impact, which can be difficult to conduct considering the specification timeline of RAN1. Last, Alternative-2 only introduces changes in the UE Rx-Tx time difference definition. However, this option still requires higher specification impact and considers the reporting of time stamps of the PRS and SRS to the LMF. The wording does not specify if the time stamps are reported only at gNB or at both UE and gNB ends. In any case, it will largely increase the amount of information conveyed from the UE/gNB to the LMF.

Observation 4: Considering the lack of common agreement, Alternative-1 shows the best compromise between accuracy, signalling and specification effort, while Alternative-3 could be considered for future releases.

Proposal 2: For RTT determination in NTN, RAN1 to adopt legacy definitions of UE and gNB Rx-Tx time difference as in TS 38.215 plus the reporting of an offset. The offset can be reported by the gNB to mitigate the signalling overhead on the Uu interface.

Accuracy of the UE Rx-Tx time difference reporting range

An aspect to consider is the reporting range of UE Rx-Tx time difference. As indicated in TS 38.133 [4], the resolution of the multi-RTT report is given by 2k×Tc - where Tc is the basic time unit – and the report range is from -985024Tc to 985024Tc. This means that the report range supported in current specifications only covers for [-0.5, 0.5] ms. However, it is insufficient to resolve the multiple possibilities of UL slot offset in NTN, especially for the geostationary case. If the reporting range is increased to accommodate large propagation delays but the number of bits is maintained, the resolution will decrease. This issue will worsen the accuracy of the UE location estimation. On the other hand, if the number of bits is increased to accommodate larger reporting range, the signalling overhead will be increased.

Observation 5: The accuracy of the UE location estimation depends on the time difference reporting range.

Observation 6: In NTN, the current UE Rx-Tx time difference reporting range is insufficient to resolve multiple possibilities of UL slot offset.

Proposal 3: RAN1 to discuss how to maintain the accuracy of the UE location estimation without significantly increasing the reporting ranges.

Another aspect to bear in mind is the TA adjustment during the RTT determination. In multi-RTT, the gNB Rx-Tx is added to UE Rx-Tx to account for the transmission timing error. If during the Rx-Tx time difference measurements the UE-specific TA is not adjusted, gNB Rx-Tx is enough to mitigate timing error. However, if TA adjustment is applied, gNB Rx-Tx is not enough and the UE may need, when transmitting the SRS, to indicate to the gNB/LMF any adjustment on TA due to UE autonomous TA control.

Observation 7: The network does not know when the UE autonomously adjusts the TA.

Observation 8: During multi-RTT measurements, any TA adjustment may lead to lower accuracy.

Proposal 4: RAN1 to discuss means for gNB/LMF to know whether the UE has adjusted the TA value.

UE and gNB reports: Which information is sent to the LMF

As mentioned before, the best compromised solution is Alternative-1 because it considers legacy framework but includes the reporting of an offset by the UE. The use of scheduling offsets (e.g., Koffset) can be used not only to estimate the appropriate UE Rx-Tx time difference but also to achieve greater RTT estimation accuracy by deducting the value of Koffset from the UE Rx-Tx time difference. This provides a smaller range to be reported over the air interface and will thereby increase the accuracy without substantial additional overhead. Any remaining slot difference in frame timing would potentially be reported as a separate value.

Proposal 5: The UE Rx-Tx difference is compensated for the Koffset value before reporting it to the LMF.

Proposal 6: The slot offset between DL and UL slot is reduced by Koffset prior to being reported to the LMF to reduce the additional overhead of reporting.

## Mirror-point ambiguity

Using only a single satellite, time measurements will be sampled on a single line which is described by satelite’s path during the fly-over. This reduction of the “space” when limiting to a single satellite will reduce the general accuracy. On top of this, the multi-RTT method is solely based on rough estimations of the RTT between the UE and a reference point. These RTT measurements are later mapped into distances that are used to estimate the UE location using multilateration. Only using distance-based positioning methods within the network will cause the problem of “mirror images”, where two geographical points or areas will show the same physical characteristics when being observed from the satellite’s viewpoint. These two points or areas will be seen as symmetrical around the orbital plane.

In the RAN1#112 meeting, the following was agreed to resolve the mirror-point ambiguity:

Agreement Study the following options to resolve the mirror positions ambiguity for multi-RTT positioning: - Option 1: gNB or LMF implementation to solve the mirror error issue. - FFS: whether there is spec impact - Option 2: Reuse existing ECID method (e.g. combine UE neighbor measurements to solve the ambiguity between mirror positions), with potential enhancements - Option 3: NR NTN UE should report the Doppler calculated on the service link - Option 4: a VSAT UE should report its beam pointing in respect to satellite beam line of sight - Option 5: Reporting of cell coverage information (e.g. cell footprint and reference point, or antenna pattern) to the LMF - Option 6: Support and potentially enhance the optional Rel-17 UL-AoA measurements defined for multi-RTT positioning. Other solutions are not precluded.

One aspect to note is that at RAN2#120, RAN2 agreed that the mirror point ambiguity can be resolved through neighbour cell measurements.

Agreement

- RAN2 assumes that in general the mirror point issue can be resolved by properly configuring neighbor cell measurement to UE, for example, measurement of two neighbor cells in the opposite side of a satellite beam. FFS if there are any cases that require anything in the specs

Furthermore, the following was proposed by the moderator during the last RAN1#112-bis meeting:

Proposal 6-2 NR E-CID method (e.g. combine UE neighbor measurements to solve the ambiguity between mirror positions) is with higher priority for further discussion on the mirror positions ambiguity for multi-RTT positioning in NTN - FFS: Whether enhancements are needed

Observation 9: Neighbor cells measurements can solve the mirror-point ambiguity.

In the example below we show how UE measurements can be used to differentiate between the two mirror points for two different cases. The orbital plane is right in the middle of the 7 cells in the vertical direction. The cells have a 50 km diameter and a LEO satellite at 600 km is used. The two cases are as follows:

Case a: UE is 16.7 km away from the orbital plane

Case b: UE is 5.5km away from the orbital plane

- b)

Figure 2 Illustration of the two setups (a and b) with one serving cell, a UE (red triangle) and movement due to satellite movement (red dashed line with arrow) for Earth moving cells.

Figure 2 shows the ideal RSRP traces (no fading, no measurement errors) over time of the different cells for setup a and b. Looking at setup a) it can be seen that the RSRP levels for the cells on the right side (i.e. SB6, SB2, and SB10) are stronger than the corresponding RSRP levels for the cells on the left side (i.e. SB7, SB3, and SB11), whereas the difference is smaller for case b, as the UE is closer to the orbital plane.

- b)

Figure 3 RSRP traces for setup a and b versus time.

In general the following cases may be considered:

- UE on or very close to the orbital line. The two mirror points are very close together (the distance between the points is less that 10 km apart, i.e. the UE position less that 5 km away from the orbital line), so no differentiation is needed, as the required accuracy is already reached.

- UE is very far away from the orbital line, as in the situation where is associated to a cell which is not having coverage area intersecting with the orbital line. In that case, the serving cell association may be used to assist the differentiation for verification purposes.

- UE is in between case 1 and case 2. This is the case where additional extra information is needed for validation purposes, so further studies should focus on this situation.

In particular the worst point to consider when neighboring cells are used is when the UE is 5.5 km away from the orbital line, as the neighboring cells are far away while the two mirrorpoint are separated by more than 10 km. This is like setup b) above. It should be noted that any inaccuracy in the specific positioning method should also be accounted when considering the minimum needed separation from the orbital line. That is, if e.g. multi-RTT based method will determine the UE position with an accuracy relative to the orbital plane of +/- 1 km, the associated mirror point/area algorithm need to take this into account and further increase the requirements of the separation algorithm (by reducing from 5.5 km to 4.5 km in this example).

As earlier stated the RAN2 agreements indicates a direction for the resolving the mirror point ambiguity could be resolved through neighbour cell measurements, but it is unclear which entity in the network would be responsible of configuring such measurements.

Proposal 7: RAN1 to ask RAN3/RAN2 for guidance on which entity configures the measurements needed for resolving the mirror point ambiguity.

One counter argument for using the RSRP measurements as proposed above is that this creates extra overhead for the reporting. However the signalling can be compressed as what matters is the relationship between cells on the line pendicular to the orbital line.

Proposal 8: RAN1 to study how to reduce the signalling overhead for the reporting of neigbor signal level relationships.

## Multiple satellites

The WID objectives in [1] captures the following note:

Note 4: Multiple satellite in view by the UE may be considered if time allows.

While the primary target is that the network can verify the UE position with a single satellite, multiple satellites are not excluded. One of the benefits of using multiple satellites is that the required time to determine the UE location decreases significantly, as the method then does not depend on the movement of a single satellite, when there are at least 3 satellites in view. Even with 2 satellites the required time is lowered significantly. At the same time the number of satellites - especially in low orbits - is increasing significantly and satellite systems grow in number of satellites, like for instance Starlink and OneWeb show.

As pointed out in [2], a long period of verification is undesired due to UE power saving concerns and the UE accessing a PLMN that may not be allowed. Similarly as when other positioning methods are used, using a larger number of uncorrelated space samples will benefit the verification method to estimate the UE location significantly reducing duration of the procedure.

Observation 10: The benefit of using multiple satellites is that the required time for determining the UE position decreases significantly.

In case that multiple satellites are in view of the UE, it was concluded earlier that it is beneficial to use them in order to determine the UE position faster. However, as the UE needs to listen to the downlink signals from different TRPs (which may be shifted due to different Doppler shifts from different satellites) and transmit in the uplink, some additional assistance information would be useful for NTN:

- Satellite ephemeris of the relevant satellites.

- Gaps to listen to/transmit reference signals to the relevant cells. These gaps need to be known by the UE and the serving cells, in order to avoid the serving cells scheduling the UE at those gaps, while the gaps depend on the timing of these signals in the relevant neighboring satellites.

Proposal 9: RAN1 to discuss the need and way to specify NTN specific assistance information for UE location determination with multiple satellites.

## RSRP dependency

Positioning methods such as multi-RTT take into account the propagation delays that reference signals experience from one end to the other of the radio link. These delays are used to estimate the corresponding physical communication distance and approximate the UE location. In this context, a clear line-of-sight (LOS) path between UE and TRP is preferable because the positioning accuracy is subject to the similarity between the actual geometric distance and the propagation delay based distance. Furthermore, in NTN a poor link budget is expected due to the long distances between UE and TRPs, especially in the uplink where the transmission power is limited. This may hinder proper transmission/reception, when the UE is shadowed by a surrounding element such as buildings, terrain elevations and canopy.

Observation 11: The estimation accuracy and the execution time of the UE location verification procedure may depend on the radio channel conditions.

For multi-RTT case, the method requires measurements from, at least, one satellite over time to create satellite location samples. However, as indicated above, the combined duration of the collection of the measurement samples for the evaluation is an important factor because a long period of verification would allow a UE in a not-allowed location to obtain service from that PLMN. For that reason, the UE location verification process should be initiated as soon as possible, even at low elevation angles when the likelihood of NLOS is larger and the link budget poorer.

Observation 12: The UE location verification process should be initiated as soon as possible even at low elevation angles when radio propagation conditions are not favorable.

Proposal 10: RAN1 to discuss mitigation techniques for poor radio propagation conditions.

# Conclusion

The following observations and proposals are made in this document:

Observation 1: Satellites with transparent payloads cannot conduct 3GPP protocol functionalities.

Observation 2: Defining the measurement point for TgnB-Rx and TgNB-Tx at the SAN is not feasible for transparent architectures which has been the reference architecture for discussions since Release 17.

Observation 3: Based on gNB Rx-Tx time difference measurements taken at the measurement point, the LMF can infer the Rx-Tx time difference at the timing point (i.e. satellite).

Observation 4: Considering the lack of common agreement, Alternative-1 shows the best compromise between accuracy, signalling and specification effort, while Alternative-3 could be considered for future releases.

Observation 5: The accuracy of the UE location estimation depends on the time difference reporting range.

Observation 6: In NTN, the current UE Rx-Tx time difference reporting range is insufficient to resolve multiple possibilities of UL slot offset.

Observation 7: The network does not know when the UE autonomously adjusts the TA.

Observation 8: During multi-RTT measurements, any TA adjustment may lead to lower accuracy.

Observation 9: Neighbor cells measurements can solve the mirror-point ambiguity.

Observation 10: The benefit of using multiple satellites is that the required time for determining the UE position decreases significantly.

Observation 11: The estimation accuracy and the execution time of the UE location verification procedure may depend on the radio channel conditions.

Observation 12: The UE location verification process should be initiated as soon as possible even at low elevation angles when radio propagation conditions are not favorable.

Proposal 1: RAN1 to decide that measurement point is the point where time measurements are conducted and that it is placed at the gNB.

Proposal 2: For RTT determination in NTN, RAN1 to adopt legacy definitions of UE and gNB Rx-Tx time difference as in TS 38.215 plus the reporting of an offset. The offset can be reported by the gNB to mitigate the signalling overhead on the Uu interface.

Proposal 3: RAN1 to discuss how to maintain the accuracy of the UE location estimation without significantly increasing the reporting ranges.

Proposal 4: RAN1 to discuss means for gNB/LMF to know whether the UE has adjusted the TA value.

Proposal 5: The UE Rx-Tx difference is compensated for the Koffset value before reporting it to the LMF.

Proposal 6: The slot offset between DL and UL slot is reduced by Koffset prior to being reported to the LMF to reduce the additional overhead of reporting.

Proposal 7: RAN1 to ask RAN3/RAN2 for guidance on which entity configures the measurements needed for resolving the mirror point ambiguity.

Proposal 8: RAN1 to study how to reduce the signalling overhead for the reporting of neigbor signal level relationships.

Proposal 9: RAN1 to discuss the need and way to specify NTN specific assistance information for UE location determination with multiple satellites.

Proposal 10: RAN1 to discuss mitigation techniques for poor radio propagation conditions.

# References

- RP-223534, “Revised WID: NR NTN (Non-Terrestrial Networks) enhancements”, Thales, 3GPP RAN#98.

- ”TSG RAN; NR; Physical layer measurements (Release 17)”, 3GPP, TSG RAN, TS 38.215 v17.2.0 (2022-09).

- R1-2302404, ”FL Summary #3: Network verified UE location for NR NTN”, Thales, 3GPP RAN1#112bis-e, April 17th-26th, 2023.

- ”TSG RAN; NR; Requirements for support of radio resource management (Release 18)” 3GPP, TSG RAN, TS 38.133 v18.0.0 (2022-12).