3GPP TSG RAN WG1 #113			R1-2304451

Incheon, Korea, May 22nd - May 26th, 2023

Title:	Draft reply LS on monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs

Response to:	R1-2304331 (R2-2304562)

Release:	Rel-17

Work Item:	NR_SmallData_INACTIVE-Core, NR_redcap

Source:	vivo [To be RAN1]

To:	RAN2, RAN4

Cc:

Contact Person:

Name:

E-mail Address:	< >

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN1 thanks RAN2 for the LS. RAN1 has discussed the question from RAN2 and would like to provide the following RAN1’s understandings to RAN2 and RAN4.

In current RAN1 specification of TS 38.213, clause 17.2 defines that a HD-UE does not expect to receive both a Type-0/0A/1/2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and dedicated higher layer parameters configuring transmission in the set of symbols. It is RAN1’s understanding that the case that a paging occasion conflicts with a CG-SDT occasion will not happen for a HD-FDD RedCap UE. There is no impact on RAN2 specification if the UE is only required to monitor paging for SI change indication in any paging occasion at least once per modification period during SDT if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB. Therefore, there is no misalignment between RAN1 and RAN2 specifications.

TS 38.213 V17.5.0 clause 17.2 [Omit irrelevant text] A HD-UE does not expect to receive both dedicated higher layer parameters configuring transmission in a set of symbols and dedicated higher layer parameters configuring reception in the set of symbols. A HD-UE does not expect to receive both a Type-0/0A/1/2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and dedicated higher layer parameters configuring transmission in the set of symbols.

However, in current RAN4 specification of TS 38.133, clause 5.1B.2.6 allows the case that a paging occasion conflicts with a CG-SDT occasion to happen, which is not aligned with RAN1 specification. In addition, in case such collision happens, in RAN4 specification of TS 38.133, clause 5.1B.2.6 defines that the HD-FDD UE is allowed to drop the CG-SDT transmission, which is not aligned with RAN2 specification which states that it is unnecessary to drop all the CG-SDT occasions that overlap with any paging occasion as UE is only required to monitor the paging in any paging occasion at least once per modification period. In other words, the HD-FDD UE can select other conflict-free paging occasions within the modification period to monitor the paging for SI change. Or a HD-FDD UE only drops a CG-SDT occasion that conflicts with the selected paging occasion for monitoring paging within the modification period for SI change indication but the UE is still allowed to transmit on other CG-SDT occasions even if they overlap with the paging occasion(s).

TS 38.133, V18.1.0 clause 5.1B.2.6 #### 5.1B.2.6 Maximum interruption in paging reception The requirements in clause 4.2B.2.6 shall apply for RedCap UEs. For RedCap UE in HD-FDD mode, if a paging occasion overlaps with CG-SDT transmission then the UE shall monitor the paging during the paging occasion. In this case the UE is allowed to drop the CG-SDT transmission.

Therefore, RAN1 would like to ask RAN2 and RAN4 to take above understandings into account and kindly ask RAN4 to discuss possible amendment to resolve the misalignment between RAN4 specification and RAN1/RAN2 specifications.

2. Actions: RAN1 kindly requests RAN4 to discuss possible amendment to resolve on misalignment between RAN4 specification and RAN1/RAN2 specifications.

3. Dates of Next RAN WG1 Meetings:

TSG RAN WG1 Meeting #114	21st – 25th August 2023		Toulouse, France

TSG RAN WG1 Meeting #114bis	09th – 18th October 2023		Xiamen, China