3GPP TSG-RAN WG1 Meeting #113                    	          R1-2304459

Incheon, Korea, May 22nd – May 26th, 2023

Title:	Draft reply LS on multicast reception in RRC_INACTIVE

Response to:	R1-2304325 (R2-2304330)

Release:	Rel-18

Work Item:	NR_MBS_enh-Core

Source:	RAN1

To:	RAN2

Contact Person:

Name:	Na Li

E-mail Address:	lina5g@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	No

1. Overall Description:

RAN1 would like to thank RAN2 for sending their agreements and questions in LS R1-2304325(R2-2304330) on multicast reception in RRC_INACTIVE.

Please find RAN1 answers on the questions below.

- Question 1: Is the following RAN2 assumption feasible? If feasible, whether both DCI format 4-1 and DCI format 4-2 are needed? - For MTCH, RAN2 assumes to reuse the same DCI formats of R17 multicast (i.e. DCI format 4-1/4-2) for dynamic scheduling of multicast in RRC INACTIVE. RAN2 assumes for multicast MCCH scheduling, DCI format 4-0 is used.

- Answer1: From RAN1 perspective, the above RAN2 assumption is feasible if corresponding higher layer parameters to determine each field of DCI format 4_2 is provided in RRCRelease or SIB.  RAN1 does not see clear use case for DCI format 4-2.

  - For Example, the following higher layer parameters to determine the field of DCI format 4_2 needs to be provided in RRCRelease or SIB

    - locationAndBandwidthMulticast

    - resourceAllocation in pdsch-ConfigMulticast

    - pdsch-TimeDomainAllocationList

    - vrb-ToPRB-Interleaver in pdsch-ConfigMulticast

    - prb-BundlingType in pdsch-ConfigMulticast

    - rateMatchPatternGroup1 and rateMatchPatternGroup2 in pdsch-ConfigMulticast

    - aperiodic ZP CSI-RS resource sets configured in pdsch-ConfigMulticast

    - maxNrofCodeWordsScheduledByDCI configured in pdsch-ConfigMulticast

    - dmrs-DownlinkForPDSCH-MappingTypeA and/or dmrs-DownlinkForPDSCH-MappingTypeB,

    - tci-PresentInDCI in pdcch-ConfigMulticast

    - sizeDCI-4-2

- Question 2: Is the following RAN2 understanding correct? - RAN2 understanding is that PDSCH aggregation is supported for multicast MTCH in RRC_INACTIVE (as that is supported in Rel-17 for multicast MTCH in RRC_CONNECTED as well as for broadcast MTCH).

Answer2: The above RAN2 understanding is correct.

  - For multicast MTCH in RRC_INACTIVE, supporting PDSCH aggregation same as that in Rel-17 for multicast MTCH in RRC_CONNECTED as well as for broadcast MTCH means the following is supported for multicast MTCH in RRC_INACTIVE:

  - (Config A) UE can be configured with pdsch-AggregationFactor per G-RNTI, applied to DCI format with the G-RNTI for multicast MTCH in RRC_INACTIVE state.

  - (Config B) UE can be configured with TDRA table with repetitionNumber as part of the TDRA table in PDSCH-Config for multicast MTCH in RRC_INACTIVE state.

  - If UE is configured with Config B, UE does not expect to be configured with Config A for the same GC-PDSCH.

  - To support Config A for multicast MTCH in RRC_INACTIVE, pdsch-AggregationFactor per G-RNTI should be provided in RRCRelease or SIB

  - To support Config B for multicast MTCH in RRC_INACTIVE, TDRA table with repetitionNumber as part of the TDRA table in PDSCH-Config for multicast MTCH should be provided in RRCRelease or SIB

- Question 3: Is it feasible to reuse the following Rel-17 CSS design for multicast MTCH and multicast MCCH? - 3.1) Reusing the same CSS or the same CSS type for multicast MTCH in RRC_INACTIVE (same as multicast MTCH in RRC_CONNECTED). - 3.2) Separate CSS(es) for multicast MCCH and multicast MTCH in RRC_INACTIVE.

Answer3: It is feasible to reuse the following Rel-17 CSS design for multicast MTCH and multicast MCCH.

  - For 3.1), Network should provide SearchSpace in pdcch-ConfigMulticast in RRCRelease message or SIB to configure the same or same type CSS if the same CSS or the same CSS type as multicast MTCH in RRC_CONNECTED is reused for multicast MTCH in RRC_INACTIVE.

  - For 3.2), it is feasible to use separate CSS(es) for multicast MCCH and multicast MTCH in RRC_INACTIVE. Since no CSS defined in Rel-17 for multicast MCCH, network should provide associated signaling to configure CSS for multicast MCCH in RRC_INACTIVE via RRCRelease message or SIB if separate CSS(es) for multicast MCCH and multicast MTCH in RRC_INACTIVE is used.

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully ask RAN2 to take above information into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #114		Aug 21th – 25th, 2023   		Toulouse, FR

TSG RAN WG1 Meeting #114bis		Oct.	9th – 13th, 2023	      	Xiamen, CN