3GPP TSG RAN WG1 #113 	R1-2304468

Incheon, Korea, May 22nd – May 26th, 2023

Source: 	vivo

Title:	Further discussion on UL precoding indication for multi-panel transmission

Agenda Item:	9.1.4.1

Document for:	Discussion and Decision

Introduction

Significant progress has been made in this item, in this contribution, we discuss remaining issues on simultaneous multi-panel transmission in mTRP operation.

STxMP in SDCI-based mTRP operation

In this section, we discuss signaling design for SDCI-based STxMP PUSCH with SDM and SFN schemes and PUCCH with SFN scheme.

Design on SDM scheme

SRS configuration and indication

To support two panels simultaneous transmission, two separate sets of TxRUs can be implemented associating with two panels, as shown in Figure 1. In such implementation, it is straight forward to configure two SRS resource sets and port mapping could be fixed which doesn’t change for sTRP or SDM transmission. That means PUSCH ports mapping is per panel. PUSCH ports for SDM transmission is sum of PUSCH ports which are used for sTRP transmission.

Independent digital ports architecture

Another architecture for simultaneous multi panel transmission can be such that one set of TxRUs is shared between two panels. When s-panel (sTRP) transmission is indicated, all TxRUs are mapped to one panel. When two panels simultaneous (SDM) transmission is indicated, the TxRUs are split into two groups and one group is mapped one panel. Motivation of such design/implantation is that max rank for s-panel transmission can be achieved as in legacy system and for SDM transmission since it is limited to max rank of 2 per panel, total layers for sTRP and SDM transmission can be same without doubling the hardware. Figure 2 shows an example of two panels with 4 antennas each and 4 TxRUs in total are shared between two panels, which means maximum number of TxRUs for sTRP transmission and SDM transmission should be same.

Support both shared digital ports and independent digital ports architecture for simultaneous two panels transmission.

To support such implementation, and to support max rank of 4 for sTRP transmission and max rank of 2 per panel for SDM transmission, SRS configuration and association should be further discussed. For sTRP transmission, 4 TxRUs are switched between panel 0 and panel 1, which means the UE shall be configured with 4 ports SRS resource(s) in each set which are transmitted in TDM manner. gNB can indicate which panel to transmit using the SRS set indicator bit field in DCI. For codebook based PUSCH transmission, 4Tx codebook is used and corresponding TPMI is indicated. Now, for SDM (two panel simultaneous) transmission, there are different approaches with different degree of spec impacts.

Option 1: configure separate codebook subsets for sTRP transmission and SDM transmission. Codebook subset for sTRP transmission includes precoders for rank=1~4 (whether full, partial or non-coherent subset depends on per panel capability) and codebook subset for SDM transmission includes partial and non-coherent rank =1 precoders and non-coherent rank=2 precoders only. This option has small spec impact.

Option 2:  configure multiple SRS resources in a set with different number ports. In the example as shown in Figure 2, each SRS resource sets is configured with at least 2 SRS resources, one 4 port SRS resource and one 2 port SRS resource. When sTRP transmission is indicated by SRS set indicator field in DCI, it implies that 4 port SRS resource is applied and the corresponding codebook. When SDM transmission is indicated by SRS set indicator field in DCI, which implies that 2 port SRS resource is applied and the corresponding codebook. This option has small spec impact, configuration of multiple SRS resources with different number of ports is already supported in Rel-16 for UL full power mode 2, and similar mechanism can be reused. The main drawback of this option is additional SRS overhead. Besides, by configuring SRS ports accordingly, independent digital ports architecture and shared digital ports architecture can work properly.

Option 3: use 4Tx codebook for sTRP transmission and 2Tx codebook for SDM transmission. In the example as shown in Figure 2, for SDM transmission using 2Tx codebook for each panel, first (or last) 2 ports of 4 port SRS resource can be used. This is equivalent to introducing new precoders for 4Tx codebook, for example indicating following 2Tx rank=2 precoder 12111-1  based on first 2 ports of 4 port SRS is equivalent to a following new 4Tx rank=2 precoder  12101010-10, which is against the guidance from WID. And, performance wise it is sub-optimal compared to option 1 due loss in antenna selection gain.

Shared digital ports architecture

To keep a common design for these two architectures of digital ports implementation, support option 2 as first preference and option 1 as second preference, and for Option 2:

Support configuring SRS resources with different number of ports in one SRS resource set to support sTRP transmission and SDM transmission for the shared digital ports architecture.

Support configuring SRS resources with same number of ports in one SRS resource set to support sTRP transmission and SDM transmission for the independent digital ports architecture.

Precoding indication

Agreement The codepoints of “SRS resource set indicator” in DCI for dynamic switching between STxMP SDM and sTRP transmission are interpreted and the SRI/TPMI fields are designed as follows: - The codepoints 00 and 01 indicate sTRP transmission. 00 indicates the first SRS resource set and 01 indicates the second SRS resource set. For SRI/TPMI field design, down-select one from the following Alts: - Alt1: The DCI has two SRI fields and two TPMI fields. The first SRI field and first TPMI field are associated the first SRS resource set if codepoint = 00 or the second SRS resource set if codepoint = 01. The second SRI field and second TPMI fields are reserved. - Alt2: the DCI has only one SRI field and one TPMI field. The SRI and TPMI field are associated with the first SRS resource set if codepoint=00 or the second SRS resource set if codepoint = 01. - Alt3: The DCI has two SRI fields and two TPMI fields. The first SRI field and second SRI field are concatenated into one SRI field. The first TPMI field and second TPMI field are concatenated into one TPMI field. The concatenated SRI field and the concatenated TPMI field are associated with first SRS resource set if codepoint = 00 or the second SRS resource set if codepoint = 01. - FFS: If the concatenated bits are not sufficient, additional bits are appended to concatenated bits in order to support this feature - Alt4: the DCI has two SRI fields and two TPMI fields. - When the codepoint is 00, the first SRI field and first TPMI field are associated with the first SRS resource set. The second SRI field and second TPMI field are reserved. - When the codepoint is 01, the second SRI field and second TPMI field are associated with the second SRS resource set. The first SRI field and first TPMI field are reserved. - The codepoints 10 indicate SDM transmission with the first and second SRS resource set. - The first SRI field and first TPMI field are associated with the first SRS resource set and they indicate the precoder(s)/rank/SRI for the first SRS resource set. - The second SRI field and second TPMI field are associated with the first SRS resource set and they indicate the precoder(s)/rank/SRI for the second SRS resource set. - FFS: The codepoint 11 is reserved.

It has been agreed that for SDM scheme of single-DCI based STxMP PUSCH, for codebook-based PUSCH, DCI indicates two TPMI fields, and each TPMI field separately indicates the precoding information and the number of layers conveyed over the SRS ports of the indicated SRS resource in each SRS resource set. However, only one TPMI/SRI field is needed for precoder indication for sTRP transmission. And, dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and sTRP transmission has been agreed. How to design overall precoder indication to support two-transmission schemes should be further discussed.

Table 1 lists TPMI field sizes for s-panel transmission for different number of SRS ports and codebook subsets, and total number of bits for two TPMI fields for SDM transmission under different configuration. SRS ports configuration for the third and fourth rows are based on the shared architecture. SRS ports for s-panel are configured for s-panel transmission while only two SRS ports for each panel is enabled for SDM. maxRank configured for s-panel can be 1/2/4 while max rank configured for each SRS resource set for sTxMP can be 1/2. More specifically, as shown in the third row, max rank for s-panel transmission is configured as 4 and max rank for SDM is configured as 2 for each SRS resource set, sum of TPMI fields is less than one TPMI field if codebooksubset is configured as partial-and-non-coherent. Also, on the fourth row, sum of TPMI fields is less than one TPMI field if codebooksubset is configured as partial-and-non-coherent.

Besides, Bit length of one TPMI/SRI field for s-panel transmission is equal or larger than that required for sTxMP transmission, for maxRank and SRS ports for each TPMI field is less than that of s-panel transmission.

TPMI field overhead for independent and shared architectures

| SRS ports for SDM | SRS ports for s-Panel | MaxRank for s-panel | maxRank for sTxMP | TPMI field size required for s-panel Fullpartialnon/partialnon/non | TPMI fields size required for sTxMP Fullpartialnon/partialnon/non |
| --- | --- | --- | --- | --- | --- |
| 1+1 | 1 | 1 | 1 | 0 | 0 |
| 2+2 | 2 | 2 | 1+1 | 4/2 | 3+3/1+1 |
| 2+2 | 4 | 2 | 2+2 | 6/5/4 | 4+4/N/2+2 |
| 2+2 | 4 | 4 | 1+1 | 6/5/4 | 3+3/N/1+1 |
| 4+4 | 4 | 4 | 1+1 | 6/5/4 | 5+5/4+4/2+2 |

Total number of bits of two TPMI fields for indication two precoders for SDM transmission can be less than a required TPMI field bits for indication one precoder for s-Panel transmission.

Number of bits of one TPMI/SRI field for s-panel transmission is equal or larger than that required for SDM transmission.

To dynamically indicate precoder for s-panel transmission, following three alternatives can be considered.

Alt1: For CB based PUSCH, the 1st TPMI field indicates the precoding/rank and the 1st SRI field indicates the SRS resource. For NCB based PUSCH, the 1st SRI field indicates the precoding/rank. 2nd TPMI/SRI field is reserved/unused or absent.

Alt2: For CB based PUSCH, concatenating the 1st and 2nd TPMI fields for indicating the precoding/rank and the 1st SRI field indicates the SRS resource. For NCB based PUSCH, concatenating the 1st and 2nd SRI fields for indicating the precoding/rank.

Alt3:

      - When codepoint of SRS resource set indicator field is 00: for CB based PUSCH, the 1st TPMI field indicates the precoding/rank and the 1st SRI field indicates the SRS resource. For NCB based PUSCH, the 1st SRI field indicates the precoding/rank.

      - When codepoint of SRS resource set indicator field is 01: for CB based PUSCH, the 2nd TPMI field indicates the precoding/rank and the 2nd SRI field indicates the SRS resource. For NCB based PUSCH, the 2nd SRI field indicates the precoding/rank.

      - The other TPMI/SRI fields are unused.

Considering that sum of bits of two TPMI fields for indicating two precoders for sTxMP transmission can be less than bit size of TPMI field for indicating one precoder for s-Panel transmission, which means two concatenated TPMI fields may not be sufficient for precoding/rank for s-Panel transmission. So, Alt2 is not a promising solution, which could complicate DCI design in some special cases. As discussed above, bit size of one TPMI/SRI field for s-panel transmission is equal to or larger than that total number of bits for SDM transmission, using the first SRI/TPMI field to indicate precoders for the first and the second panel transmission can streamline the size of second TPMI field can be designed based on sTxMP transmission which require less bit length. So, Alt1 is superior to Alt3. Besides, inheriting the same design in Rel-17 mTRP, Alt1 has small spec impact.

Support Alt1: for CB based PUSCH, the 1st TPMI field indicates the precoding/rank and the 1st SRI field indicates the SRS resource. For NCB based PUSCH, the 1st SRI field indicates the precoding/rank. 2nd TPMI/SRI field is reserved/unused or absent.

Further clarification on SDM scheme

DFT-s-OFDM waveform is typically enabled for cell-edge user, so only single layer transmission is supported to keep low PAPR property. However, for two panels simultaneous transmission, independent PA architecture for two panels makes it possible to support 2-layers (1 layer on each panel) SDM transmission. Besides, for mDCI based STXMP, simultaneous transmission of rank 1 with DFT-s-OFDM waveform from each panel is supported by default.

Clarify that SDM with at least 2 layers should be supported for DFT-s-OFDM waveform.

Design on SFN

Agreement The codepoints of “SRS resource set indicator” in DCI for dynamic switching between STxMP SFN and sTRP transmission are interpreted and the design of SRI/TPMI fields are as follows: - The codepoints 00 and 01 indicate sTRP transmission. 00 indicates the first SRS resource set and 01 indicates the second SRS resource set. - For the design of SRI/TPMI fields, re-use the design that is decided for dynamic switching between STxMP SDM and sTRP transmission. - The codepoint 10 indicates STxMP SFN transmission with the first and second SRS resource set. - The first SRI field and first TPMI field are associated with the first SRS resource set and they indicate the precoder(s)/rank/SRI for the first SRS resource set. - The second SRI field and second TPMI field are associated with the first second SRS resource set and they indicate the precoder(s)/SRI for the second SRS resource set (the rank is indicated by the first SRI field for NCB or the first TPMI field for CB) - FFS: The codepoint 11 is reserved.

For SFN scheme, same number of layers are indicated by two TPMI fields, so indication of TPMI with same RI indicated by the first SRI/TPMI field is sufficient for the second SRI/TPMI similar as the design in Rel-17 TDM scheme.

Reuse precoder indication designed in Rel-17 for SFN precoder indication:

S-panel: For CB based PUSCH, the 1st TPMI field indicates the precoding/rank and the 1st SRI field indicates the SRS resource. For NCB based PUSCH, the 1st SRI field indicates the precoding/rank. 2nd TPMI/SRI field are reserved or absent.

SFN: The 1st TPMI/SRI field indicates the rank and precoding/SRI for the first SRS resource set and the 2nd TPMI/SRI field indicates the precoding/SRI corresponding to the rank indicated by the 1st TPMI/SRI field for the second SRS resource set.

Transmission scheme indication for PUCCH resource

In Rel-17, when two spatial relation infos are associated to one PUCCH resource, TDM mTRP repetition scheme is enabled. In Rel-18, SFN is supported for PUCCH transmission. RRC configuration per PUCCH resource to indicate SFN transmission scheme for differentiating from TDM transmission scheme when two TCI states are indicated.

Transmission scheme should be configured per PUCCH resource to differentiate between STxMP SFN scheme and Rel-17 mTRP time-domain repetition scheme when two TCI states are indicated.

STxMP in MDCI-based MTRP operation

SRI/TPMI indication for mDCI-based STxMP

Agreement For STxMP PUSCH+PUSCH transmission in multi-DCI based system: - The maximal number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain can be 1 or 2 subjects to UE capability.

To support simultaneous transmission from panels with different capabilities, different number of SRS ports and SRS resources can be configured for two SRS resource sets. Besides, maxRank can also be configured separately for each SRS resource set or corresponding value. Per-panel configuration can potentially reduce DCI overhead. However, the DCI size should be kept same for a typical DCI format even though it will be decoded from coresets associate with different coresetpoolindex to avoid increasing UE capability.

Separate maxRank can be configured for two SRS resource sets.

keep same DCI payload for the same DCI format carried by coreset associates to different Coresetpoolindex.

Overlapped PUSCHs

Agreement Multi-DCI based STxMP PUSCH+PUSCH transmission at least supports the following PUSCH combinations: - DG-PUSCH + DG-PUSCH - CG-PUSCH + DG-PUSCH Agreement The multi-DCI based STxMP PUSCH+PUSCH transmission supports fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain. - FFS whether/how to handle the PUSCH power adjustment when two PUSCHs are fully/partially overlapped in time domain (Depending on RAN4’s input on Pcmax requirements). - Note: No symbol-level power adjustment within a PUSCH transmission occassion in the case of fully/partially overlapping in time domain Agreement (Ran111) Support CG PUSCH + CG PUSCH in multi-DCI based STxMP PUSCH+PUSCH transmission.

Simultaneous transmission for CG+CG is supported. UE is able to select two CG associated with different coresetpoolindex to simultaneously transmit when data arrives. For these selected CGs, HARQ-ID should not overlap to keep one HARQ process with one TB within a HARQ process. HARQ process ID is calculated by the time instance index including slot index and starting symbol index. For the overlapped CGs, such calculation may cause overlapped HARQ ID. The HARQ process ID offset configured by RRC in current spec can achieve different HARQ-ID for the two overlapped CGs.

Different HARQ-ID is required for the selected CGs for STxMP.

UCI multiplexing

Agreement For multi-DCI based STxMP PUSCH+PUSCH, study enhancements of the UCI multiplexing rule to address the case that one PUCCH overlaps with two overlapped PUSCHs of STxMP PUSCH+PUSCH. Agreement For case that one PUCCH overlaps with two overlapped PUSCHs in multi-DCI based STxMP PUSCH+PUSCH, down-select one for the UCI multiplexing: - Option 1: the UCI is multiplexed into the PUSCH associated with the same TRP. And among the PUSCHs associated with the same TRP, the legacy PUSCH priority order for UCI multiplexing is applied. FFS: determining the PUSCH associated with the same TRP. - Option 2: the legacy PUSCH priority order for UCI multiplexing is first applied and if at last, there are two PUSCHs with the same start time in one same CC, the UCI is multiplexed in (FFS: one or two) of these two PUSCHs, and FFS which one PUSCH. - Option 3: - When joint HARQ-ACK feedback is configured, the legacy PUSCH priority order for UCI multiplexing is first applied and if at last, there are two PUSCHs with the same start time in one same CC, the UCI is multiplexed in (FFS: one or two) of these two PUSCHs, and FFS which one PUSCH. - When separate HARQ-ACK feedback is configured, at least when the UCI includes HARQ-ACK, the UCI is multiplexed into the PUSCH associated with the same TRP. And among the PUSCHs associated with the same TRP, the legacy PUSCH priority order for UCI multiplexing is applied. FFS: determining the PUSCH associated with the same TRP. - FFS: When the UCI does not include HARQ-ACK (CSI and/or SR), whether to follow the same behavior as above, or to follow the behavior of the case that joint HARQ-ACK feedback is configured. - Note: Here using joint HARQ-ACK feedback and separate HARQ-ACK feedback is mainly for discussion purpose. FFS: whether to introduce a new RRC parameter to indicate that. - FFS the impact of the following legacy restriction on the above options: when separate HARQ feedback is configured, a PUCCH transmission triggered by DCI associated with one coresetPoolIndex cannot overlap in time with a PUSCH transmission triggered by DCI associated with another coresetPoolIndex. - Note: each of the above options is applied to the system when the system is configured with multi-DCI based STxMP PUSCH+PUSCH.

In Rel-15 and Rel-16, the multiplexing rule based on the priority of UCI or physical channel is defined. In Rel-17, this rule is reused for UL SDCI-based and MDCI-based mTRP without any enhancement. However, for STxMP transmission, two overlapped PUSCHs or two overlapped PUCCHs can be simultaneously transmitted towards two TRPs. How to multiplex the UCI on the overlapped PUSCHs or PUCCHs as shown in Figure 3 should be further discussed. One simple solution is to reuse legacy rules for each TRP. Since simultaneous transmission of a PUCCH and a PUSCH is not part of the WID, how to handle the overlapped PUCCH and PUSCH for TRP-specific UCI multiplexing should be further studied.

PUSCH scheduling cases

For the case that one PUCCH overlaps with two overlapped PUSCHs in multi-DCI based STxMP PUSCH+PUSCH, the following three options are available for down selection:

- Option 1: the UCI is multiplexed into the PUSCH associated with the same TRP. And among the PUSCHs associated with the same TRP, the legacy PUSCH priority order for UCI multiplexing is applied. FFS: determining the PUSCH associated with the same TRP.

- Option 2: the legacy PUSCH priority order for UCI multiplexing is first applied and if at last, there are two PUSCHs with the same start time in one same CC, the UCI is multiplexed in (FFS: one or two) of these two PUSCHs, and FFS which one PUSCH.

- Option 3:

  - When joint HARQ-ACK feedback is configured, the legacy PUSCH priority order for UCI multiplexing is first applied and if at last, there are two PUSCHs with the same start time in one same CC, the UCI is multiplexed in (FFS: one or two) of these two PUSCHs, and FFS which one PUSCH.

  - When separate HARQ-ACK feedback is configured, at least when the UCI includes HARQ-ACK, the UCI is multiplexed into the PUSCH associated with the same TRP. And among the PUSCHs associated with the same TRP, the legacy PUSCH priority order for UCI multiplexing is applied. FFS: determining the PUSCH associated with the same TRP.

    - FFS: When the UCI does not include HARQ-ACK (CSI and/or SR), whether to follow the same behavior as above, or to follow the behavior of the case that joint HARQ-ACK feedback is configured.

  - Note: Here using joint HARQ-ACK feedback and separate HARQ-ACK feedback is mainly for discussion purpose. FFS: whether to introduce a new RRC parameter to indicate that.

- Option 1 is the most straightforward way for sTxMP. Per-panel/TRP multiplexing is beneficial for TRP decoding in both ideal-backhaul and non-ideal backhaul case. To achieve this, PUSCH and PUCCH transmission can be associated to Coresetpoolindex. UCI carried on PUCCH can be multiplexed on PUSCH associated with the same Coresetpoolindex. The association between PUCCH/PUSCH transmission and coresetpoolindex has been defined in AI 9.1.1.1.

Option 2 cannot work well when separate HARQ-ACK feedback is configured, for cross TRP multiplexing may occur. Option 3 is a complicated solution, and the benefit is unclear.

Support Option1, and the PUSCH associated with the same TRP can be achieved by the association between PUSCH and Coresetpoolindex as defined in AI 9.1.1.1.

As PUCCH+PUCCH and PUCCH+PUSCH are not supported for mDCI-based STxMP, rules to handle overlapped PUCCH or overlapped PUCCH/PUSCH after per-panel UCI multiplexing should be further discussed. One default way is to define PUCCH/PUSCH associates with one panel has higher priority.

Define default rule to handle the overlapped PUCCH+PUCCH, PUCCH+PUSCH in mDCI based STxMP.

Beam report

In Rel-15 if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'enabled', the UE shall report in a single reporting instance two different CRI or SSBRI for each report setting, where CSI-RS and/or SSB resources can be received simultaneously by the UE either with a single spatial domain Rx filter, or with multiple spatial domain Rx filters. In Rel-17 MTRP, if the UE is configured with the higher layer parameter groupBasedBeamReporting-r17, the UE shall report in a single reporting instance nrofReportedRSgroup, if configured, group(s) of two CRIs or SSBRIs selecting one CSI-RS or SSB from each of the two CSI Resource Sets for the report setting, where CSI-RS and/or SSB resources of each group can be received simultaneously by the UE. Rel-18 enhanced group-based beam report for simultaneous transmission from multiple panels with three alternatives were discussed in RAN1#112bis-e.

Agreement Enhance the Rel-17 group-based beam L1-RSRP reporting to support STxMP-based transmission and down-select one in RAN1#113 meeting: - Alt1: In each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously, and the reported pair of CRIs or SSBRIs can be received simultaneously. - Alt2: In each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously. - Alt3: In each reported pair of CRIs or SSBRIs, UE indicates if the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously, and/or if the reported pair of CRIs or SSBRIs can be received simultaneously. - FFS: Introduce an indicator to support the above, and the number of bits and interpretation of each codepoint of the indicator

The enhanced group-based beam reporting can be evolved from the Rel-15/Rel-17 group-based beam reporting where a pair of CSI-RSs or SSB resources indicated by CRI or SSBRI can be received simultaneously by the UE. Meanwhile, there are possibilities that the spatial Rx filter(s) determined by the reported pair of CRIs or SSBRIs that can be received simultaneously by the UE does not always mean the UL spatial Tx filters determined by the reported pair of CRIs or SSBRIs can be applied simultaneously. For example, there can be a single spatial Rx filter to receive signals from two Tx beams; or even a UE has multiple panels with simultaneous transmission capability and is undergoing an MPE issue in one panel. Therefore, Alt3 is preferred to provide the flexibility of the group-based beam reporting, in which an indicator for each reported pair of CRIs or SSBRIs can be introduced to indicate whether the reported beam pair can be simultaneously transmitted or not, and can be simultaneously received or not.

Support Alt3 to enhance the Rel-17 group-based beam L1-RSRP reporting to support STxMP-based transmission, i.e., in each reported pair of CRIs or SSBRIs, UE indicates if the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously, and/or if the reported pair of CRIs or SSBRIs can be received simultaneously.

An indicator for each reported pair of CRIs or SSBRIs is used to indicate whether the reported beam pair can be simultaneously transmitted or not, and can be simultaneously received or not.

PHR report for SDCI-based and MDCI-based STxMP

Panel-specific PHR should be reported to assist gNB for controlling transmission power from each panel in SDCI-based mTRP operation. To achieve this, the design in Rel-17 TRP-specific PHR reporting can be considered as a baseline. In Rel-17 SDCI-based mTRP, when PHR MAC-CE is reported in slot n, for a CC that is configured with mTRP PUSCH repetition, the second PHR value is determined as follows:

If the first PHR value is actual PHR (based on Rel. 15/16) corresponding to a repetition among mTRP PUSCH repetitions associated with a given TRP, the second PHR value

is actual only when a repetition associated with the other TRP is transmitted in slot n. Otherwise, it is virtual.

If the first PHR value is actual PHR (based on Rel. 15/16) but not corresponding to a repetition among mTRP PUSCH repetitions (corresponds to sTRP PUSCH), a second PHR value is reported as virtual PHR.

If the first PHR value is virtual, a second PHR value is reported as virtual PHR.

For panel-specific PHR reporting for STxMP, where transmission from panels occur in the same slot, two PHRs can be determined as follows:

If the first PHR value is the actual PHR corresponding to a panel for STxMP, the second PHR is actual PHR.

If the first PHR value is actual PHR corresponding to a panel for single panel transmission, a second PHR value is reported as virtual PHR.

If the first PHR value is virtual, a second PHR value is reported as virtual PHR.

TRP-specific PHR reporting designed in Rel-17 SDCI-based mTRP should be reused for panel-specific PHR reporting as much as possible.

The PHR trigger condition can be configured per panel for trigger events are essentially determined per-panel. Per-panel TCI state indication makes it possible for UE to track PL-RS panel-specifically. Besides, panel-specific PHR trigger is more efficient for PHR reporting, for across panel judgement is not needed. So, we propose to support Per-panel PHR trigger for both sDCI-based and mDCI-based STxMP.

Support Per-panel PHR triggering for both sDCI-based and mDCI-based STxMP.

Considering that transmission power of one panel will be determined by the other panel for simultaneous transmission, reporting power headrooms for both panels will better assist NW to control transmission power of two panels. In addition, reporting two PHs together is helpful even for sTRP scheduling for NW will select a panel with large power to improve better performance. So, we propose to report PHs for both panels together when PHR triggered.

Support to report PHs for both panels together when PHR triggered.

For MDCI-based mTRP/STxMP, as shown in Figure 4, when PUSCHs scheduled by two DCIs corresponding to two different CoresetPoolIndex are non-overlapping with each other, the first PUSCH will be the transmission occasion for calculating and reporting PHR based on the description in current specification. However, for the overlapped PUSCHs simultaneously transmit from two panels, how UE reports and calculates PHR is unclear. Panel-specific PHR reporting for both non-overlapped and overlapped PUSCHs simplifies power control for each panel. For instance, gNB can make a better decision on whether to schedule two PUSCHs for STxMP based on the reported two PHRs.

PHR reporting in mTRP operation.

In ideal-backhaul deployment scenario, PHR can be reported on PUSCH targets for any TRP. However, for the typical mDCI mTRP scenario non-ideal backhaul deployment, it is better to report PHRs for both TRP. Joint PHRs report on PUSCH towards any TRP is efficient while separate PHRs report on PUSCHs towards both TRPs is helpful in non-ideal backhaul case. To capture both modes adaptive to two scenarios, we support RRC-configured separate PHR report mode or joint PHR report mode for mDCI-based STxMP.

Support RRC-configuration of separate PHR report mode or joint PHR report mode for mDCI-based STxMP.

Conclusions

In this contribution, we have the following proposals:

- Total number of bits of two TPMI fields for indication two precoders for SDM transmission   can be less than a required TPMI field bits for indication one precoder for s-Panel transmission.

Number of bits of one TPMI/SRI field for s-panel transmission is equal or larger than that required for SDM transmission.

- Support both shared digital ports and independent digital ports architecture for simultaneous two panels transmission.

To keep a common design for these two architectures of digital ports implementation, support option 2 as first preference and option 1 as second preference, and for Option 2:

Support configuring SRS resources with different number of ports in one SRS resource set to support sTRP transmission and SDM transmission for the shared digital ports architecture.

Support configuring SRS resources with same number of ports in one SRS resource set to support sTRP transmission and SDM transmission for the independent digital ports architecture.

Support Alt1: for CB based PUSCH, the 1st TPMI field indicates the precoding/rank and the 1st SRI field indicates the SRS resource. For NCB based PUSCH, the 1st SRI field indicates the precoding/rank. 2nd TPMI/SRI field is reserved/unused or absent.

Clarify that SDM with at least 2 layers should be supported for DFT-s-OFDM waveform.

Reuse precoder indication designed in Rel-17 for SFN precoder indication:

S-panel: For CB based PUSCH, the 1st TPMI field indicates the precoding/rank and the 1st SRI field indicates the SRS resource. For NCB based PUSCH, the 1st SRI field indicates the precoding/rank. 2nd TPMI/SRI field are reserved or absent.

SFN: The 1st TPMI/SRI field indicates the rank and precoding/SRI for the first SRS resource set and the 2nd TPMI/SRI field indicates the precoding/SRI corresponding to the rank indicated by the 1st TPMI/SRI field for the second SRS resource set.

Transmission scheme should be configured per PUCCH resource to differentiate between STxMP SFN scheme and Rel-17 mTRP time-domain repetition scheme when two TCI states are indicated.

Separate maxRank can be configured for two SRS resource sets.

keep same DCI payload for the same DCI format carried by coreset associates to different coresetpoolindex.

Different HARQ-ID is required for the selected CGs for STxMP.

Support Option1, and the PUSCH associated with the same TRP can be achieved by the association between PUSCH and Coresetpoolindex as defined in AI 9.1.1.1.

Define default rule to handle the overlapped PUCCH+PUCCH, PUCCH+PUSCH in mDCI based STxMP.

Support Alt3 to enhance the Rel-17 group-based beam L1-RSRP reporting to support STxMP-based transmission, i.e., in each reported pair of CRIs or SSBRIs, UE indicates if the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously, and/or if the reported pair of CRIs or SSBRIs can be received simultaneously.

An indicator for each reported pair of CRIs or SSBRIs is used to indicate whether the reported beam pair can be simultaneously transmitted or not, and can be simultaneously received or not.

TRP-specific PHR reporting designed in Rel-17 SDCI-based mTRP should be reused for panel-specific PHR reporting as much as possible.

Support Per-panel PHR triggering for both sDCI-based and mDCI-based STxMP.

Support to report PHs for both panels together when PHR triggered.

Support RRC-configuration of separate PHR report mode or joint PHR report mode for mDCI-based STxMP.

References

RAN1 Chair’s Notes, RAN1#112-bis-e.