3GPP TSG RAN WG1 #113		                                                      R1-2304512

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.16.8

Source: 	vivo

Title: 	Discussion on UE features for R18 eRedCap

Document for:	Discussion and Decision

- Introduction

Following are the objectives for WI on Enhanced support of reduced capability NR devices [1].

The objective is to specify support for the following enhancements: Power saving/energy efficiency enhancements - Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4] - Note that this objective requires SA2, CT1 and CT4 involvement Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2] - UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap. - Both 15 kHz SCS and 30 kHz SCS are supported. - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction. - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified. Notes: - The work defined as part of this WI is not to overlap with LPWA use cases. - Coexistence with non-RedCap UEs and Rel-17 RedCap UEs should be ensured. - This WI considers all applicable duplex modes unless otherwise specified. Check in RAN#99 regarding: - Whether UE peak data rate reduction for UE is limited only with UE BB bandwidth reduction or standalone

In RAN#99 meeting, following was endorsed for UE peak data reduction for the Rel-18 eRedCap devices [2]:

Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps Note 1: Peak data rate of "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is same including unicast and broadcast respectively. Note 2: PRB processing capability of "Rel-18 eRedCap: UE capable of 20MHz + PR1" is not limited to "25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS" and it corresponds to PRB size corresponding to 20 MHz. Note 3: The only difference between "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is Note 2 and vLayers·Qm·f in order to have the same peak rate. Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following: - Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1

This contribution presents our view on RAN1 UE features for Rel-18 eRedCap.

- Discussion

The WID aims to define at most one Rel-18 RedCap UE type for further UE complexity reduction [1]. In RAN#99 meeting, during the discussion on the support of standalone PR1, i.e., “20MHz + PR1”, in order to meet the design target of “one Rel-18 RedCap UE type” to minimize market fragmentation and simplify network support, there seems common preference that the same peak data rate, i.e., 10Mbps is the criterion/label of the Rel-18 RedCap UE type. There are two ways: the Rel-18 RedCap UE is either capable of “20MHz+ PR1” or capable of “BW3/PR3 + PR1” to achieve the same ~10Mbps peak data rate. Therefore, following two FGs, FG48-1 and FG48-2 are proposed to be the Rel-18 RedCap UE basic features and Rel-18 RedCap UE must indicate one between the FG48-1 and FG48-2 is supported.

Proposal 1: Following FG48-1 and FG48-2 should be adopted as the Rel-18 RedCap UE basic FGs and a Rel-18 RedCap UE must indicate one between the FG48-1 and FG48-2 is supported.

- FG48-1: Rel-18 RedCap UEs capable of peak data reduction alone (i.e., “20MHz+ PR1”)

  - Componnets:

    - Maximum FR1 RedCap UE bandwidth is 20 MHz.

    - Support both 15kHz SCS and 30KHz SCS

    - The constraint of vLayers·Qm·f [=] 4 is relaxed to vLayers·Qm·f [=] 0.8.

    - Rel-17 early indication of Msg1, MsgA and Msg3 for random access

    - Rel-17 configuration of separate initial UL BWP for RedCap UEs

      - It includes the configuration(s) needed for RedCap UE to perform random access

      - Enabling/disabling of frequency hopping for common PUCCH resources

    - Rel-17 configuration of separate initial DL BWP for RedCap UEs

      - It includes CSS/CORESET for random access

      - For separate initial DL BWP used for paging, CD-SSB is included

      - For separate initial DL BWP only used for RACH, SSB may or may not be included

      - For separate initial DL BWP used in connected mode as BWP#0 configuration option 1, CD-SSB is included

    - 1 UE-specific RRC configured DL BWP per carrier

    - 1 UE-specific RRC configured UL BWP per carrier

    - RRC reconfiguration of any parameters related to BWP

    - UE-specific RRC configured DL BWP with CD-SSB or NCD-SSB

    - NCD-SSB based measurements in RRC-configured DL BWP

  - Prerequisite feature groups: NA

  - Type: Per UE

  - Need of FDD/TDD differentiation: No

  - Need of FR1/FR2 differentiation: Yes, FR1 only

  - Note: Similar as for Rel-17 RedCap, following can be added in the Rel-18 RedCap Note Column.

    - Rel-18 RedCap UEs do not support carrier aggregation or dual connectivity.

    - It is up to RAN2 whether/how to capture the capabilities for Rel-18 early indication of Rel-18 RedCap UE in Msg 3 and Msg A

    - A UE supporting this FG is not required to support FG 6-1

  - Mandatory/Optional:

    - Optional with capability signalling, Rel-18 RedCap UE must indicate either FG48-1 or FG48-2 is supported

- FG48-2: Rel-18 RedCap UEs capable of BB bandwidth reduction and peak data reduction (i.e., “BW3/PR3+ PR1”)

  - Componnets:

    - Maximum FR1 RedCap UE RF bandwidth is 20 MHz.

    - Support both 15kHz SCS and 30KHz SCS

    - UE baseband bandwidth reduction

      - For PUSCH, the maximum number of PRBs that the UE can transmit per slot or per hop is 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

      - For PDSCH, the maximum number of PRBs that the UE can process per slot is 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS.

    - The constraint of vLayers·Qm·f [=] 4 is relaxed to vLayers·Qm·f [=] 3.2.

    - Rel-17 early indication of Msg1, MsgA and Msg3 for random access

    - Rel-17 configuration of separate initial UL BWP for RedCap UEs

      - It includes the configuration(s) needed for RedCap UE to perform random access

      - Enabling/disabling of frequency hopping for common PUCCH resources

    - Rel-17 configuration of separate initial DL BWP for RedCap UEs

      - It includes CSS/CORESET for random access

      - For separate initial DL BWP used for paging, CD-SSB is included

      - For separate initial DL BWP only used for RACH, SSB may or may not be included

      - For separate initial DL BWP used in connected mode as BWP#0 configuration option 1, CD-SSB is included

    - 1 UE-specific RRC configured DL BWP per carrier

    - 1 UE-specific RRC configured UL BWP per carrier

    - RRC reconfiguration of any parameters related to BWP

    - UE-specific RRC configured DL BWP with CD-SSB or NCD-SSB

    - NCD-SSB based measurements in RRC-configured DL BWP

  - Prerequisite feature groups: NA

  - Type: Per UE

  - Need of FDD/TDD differentiation: No

  - Need of FR1/FR2 differentiation: Yes, FR1 only

  - Note: Similar as for Rel-17 RedCap, following can be added in the Rel-18 RedCap Note Column.

    - Rel-18 RedCap UEs do not support carrier aggregation or dual connectivity.

    - It is up to RAN2 whether/how to capture the capabilities for Rel-18 early indication of Rel-18 RedCap UE in Msg 3 and Msg A

    - A UE supporting this FG is not required to support FG 6-1

  - Mandatory/Optional:

    - Optional with capability signalling, Rel-18 RedCap UE must indicate either FG48-1 or FG48-2 is supported

- Half-duplex FDD operation type A for RedCap UE

Rel-17 RedCap UE can optionally support Half-duplex FDD operation type A, i.e., FG28-3, it makes sense that Rel-18 RedCap UE can also optionally support the Half-duplex FDD operation type A. Considering the Prerequisite feature group for indicating FG28-3 is FG28-1 that is the support of Rel-17 RedCap UEs which should not be supported for Rel-18 RedCap UEs for cost reduction, following a separate FG48-3 is introduced.

Proposal 2: FG48-3 Half-duplex FDD operation type A for RedCap UE is introduced for Rel-18 RedCap UEs.

  - Component: Half-duplex FDD operation (instead of full-duplex FDD operation) type A for RedCap UE with/without BB bandwidth reduction

  - Prerequisite feature groups: NA

  - Type: Per band

  - Need of FDD/TDD differentiation: No

  - Need of FR1/FR2 differentiation: FR1 only

  - Mandatory/Optional: Optional with capability signaling

- Supported maximum modulation order and maximum number of MIMO layers

There was a big discussion in the last meeting on the interpretation of the RAN#99 agreement on peak data rate i.e., whether the 10Mbps is the minimum peak data rate or fixed peak data rate for following:

Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps.

From Table 1, Table 2 and Table 3 in our companion contribution [3] for the peak data calculation for Rel-18 RedCap UEs with full duplex FDD, it is observed that the 10-Mbps peak rate target can be achieved for vLayers·Qm·f = 3.2 correspoding to (vLayers, Qm, f) combination of (1, 4, 0.8), (2, 2, 0.8) and (2, 4, 0.4) for Rel-18 eRedCap UE with UE BB bandwidth reduction and can be achieved for vLayers·Qm·f = 0.8 correspoding to (vLayers, Qm, f) combination of (1, 1, 0.8), (1, 2, 0.4) and (2, 1, 0.4) for Rel-18 eRedCap UE without UE BB bandwidth reduction. Therefore, in our view, for Rel-18 RedCap UE with and without BB bandwidth reduction, 256QAM is not supported, but 2Rx and 2layers can be optionally supported. The final decision on related UE features can be made after the clarification on the peak data reduction.

Proposal 3: If 256QAM is not supported for Rel-18 eRedCap UE with and without BB bandwidth reduction, following text can be added to the existing UE capabilities of “pdsch-256QAM-FR1” and “pusch-256QAM” in TS 38.306.

- pdsch-256QAM-FR1

pdsch-256QAM-FR1 Indicates whether the UE supports 256QAM modulation scheme for PDSCH for FR1 as defined in 7.3.1.2 of TS 38.211 [6]. It is mandatory with capability signalling for non-RedCap UEs and optional for RedCap UEs indicating supportOfRedCap-r17. It is not supported for RedCap UEs indicating support of [FG48-1] or [FG48-2]

- pusch-256QAM

pusch-256QAM Indicates whether the UE supports 256QAM modulation scheme for PUSCH as defined in 6.3.1.2 of TS 38.211 [6]. It is not supported for RedCap UEs indicating support of [FG48-1] or [FG48-2].

Proposal 4: If 2 layers can be optionally supported for Rel-18 eRedCap UE with and without BB bandwidth reduction, similar as Rel-17 RedCap UE, exisiting maxNumberMIMO-LayersPDSCH can be reused with following notes.

- For FR1, 1 DL MIMO layer if 1 Rx branch is supported, and 2 DL MIMO layers if 2 Rx branches are supported; UE features and corresponding capabilities related to more than 2 UE Rx branches or more than 2 DL MIMO layers, as well as UE features and capabilities related to more than 1 UE Tx branch or more than 1 UL MIMO layer are not supported by RedCap UEs indicating support of [FG48-1] or [FG48-2].

- Conclusion

This contribution provides our views for UE features required for Rel-18 eRedCap. Following is the proposal summary followed by the proposaed Feature Table “48. NR_redcap_enh”.

Proposal 1: Following FG48-1 and FG48-2 should be adopted as the Rel-18 RedCap UE basic FGs and a Rel-18 RedCap UE must indicate one between the FG48-1 and FG48-2 is supported.

- FG48-1: Rel-18 RedCap UEs capable of peak data reduction alone (i.e., “20MHz+ PR1”)

  - Componnets:

    - Maximum FR1 RedCap UE bandwidth is 20 MHz.

    - Support both 15kHz SCS and 30KHz SCS

    - The constraint of vLayers·Qm·f [=] 4 is relaxed to vLayers·Qm·f [=] 0.8.

    - Rel-17 early indication of Msg1, MsgA and Msg3 for random access

    - Rel-17 configuration of separate initial UL BWP for RedCap UEs

      - It includes the configuration(s) needed for RedCap UE to perform random access

      - Enabling/disabling of frequency hopping for common PUCCH resources

    - Rel-17 configuration of separate initial DL BWP for RedCap UEs

      - It includes CSS/CORESET for random access

      - For separate initial DL BWP used for paging, CD-SSB is included

      - For separate initial DL BWP only used for RACH, SSB may or may not be included

      - For separate initial DL BWP used in connected mode as BWP#0 configuration option 1, CD-SSB is included

    - 1 UE-specific RRC configured DL BWP per carrier

    - 1 UE-specific RRC configured UL BWP per carrier

    - RRC reconfiguration of any parameters related to BWP

    - UE-specific RRC configured DL BWP with CD-SSB or NCD-SSB

    - NCD-SSB based measurements in RRC-configured DL BWP

  - Prerequisite feature groups: NA

  - Type: Per UE

  - Need of FDD/TDD differentiation: No

  - Need of FR1/FR2 differentiation: Yes, FR1 only

  - Note: Similar as for Rel-17 RedCap, following can be added in the Rel-18 RedCap Note Column.

    - Rel-18 RedCap UEs do not support carrier aggregation or dual connectivity.

    - It is up to RAN2 whether/how to capture the capabilities for Rel-18 early indication of Rel-18 RedCap UE in Msg 3 and Msg A

    - A UE supporting this FG is not required to support FG 6-1

  - Mandatory/Optional:

    - Optional with capability signalling, Rel-18 RedCap UE must indicate either FG48-1 or FG48-2 is supported

- FG48-2: Rel-18 RedCap UEs capable of BB bandwidth reduction and peak data reduction (i.e., “BW3/PR3+ PR1”)

  - Componnets:

    - Maximum FR1 RedCap UE RF bandwidth is 20 MHz.

    - Support both 15kHz SCS and 30KHz SCS

    - UE baseband bandwidth reduction

      - For PUSCH, the maximum number of PRBs that the UE can transmit per slot or per hop is 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

      - For PDSCH, the maximum number of PRBs that the UE can process per slot is 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS.

    - The constraint of vLayers·Qm·f [=] 4 is relaxed to vLayers·Qm·f [=] 3.2.

    - Rel-17 early indication of Msg1, MsgA and Msg3 for random access

    - Rel-17 configuration of separate initial UL BWP for RedCap UEs

      - It includes the configuration(s) needed for RedCap UE to perform random access

      - Enabling/disabling of frequency hopping for common PUCCH resources

    - Rel-17 configuration of separate initial DL BWP for RedCap UEs

      - It includes CSS/CORESET for random access

      - For separate initial DL BWP used for paging, CD-SSB is included

      - For separate initial DL BWP only used for RACH, SSB may or may not be included

      - For separate initial DL BWP used in connected mode as BWP#0 configuration option 1, CD-SSB is included

    - 1 UE-specific RRC configured DL BWP per carrier

    - 1 UE-specific RRC configured UL BWP per carrier

    - RRC reconfiguration of any parameters related to BWP

    - UE-specific RRC configured DL BWP with CD-SSB or NCD-SSB

    - NCD-SSB based measurements in RRC-configured DL BWP

  - Prerequisite feature groups: NA

  - Type: Per UE

  - Need of FDD/TDD differentiation: No

  - Need of FR1/FR2 differentiation: Yes, FR1 only

  - Note: Similar as for Rel-17 RedCap, following can be added in the Rel-18 RedCap Note Column.

    - Rel-18 RedCap UEs do not support carrier aggregation or dual connectivity.

    - It is up to RAN2 whether/how to capture the capabilities for Rel-18 early indication of Rel-18 RedCap UE in Msg 3 and Msg A

    - A UE supporting this FG is not required to support FG 6-1

  - Mandatory/Optional:

    - Optional with capability signalling, Rel-18 RedCap UE must indicate either FG48-1 or FG48-2 is supported

Proposal 2: FG48-3 Half-duplex FDD operation type A for RedCap UE is introduced for Rel-18 RedCap UEs.

  - Component: Half-duplex FDD operation (instead of full-duplex FDD operation) type A for RedCap UE with/without BB bandwidth reduction

  - Prerequisite feature groups: NA

  - Type: Per band

  - Need of FDD/TDD differentiation: No

  - Need of FR1/FR2 differentiation: FR1 only

  - Mandatory/Optional: Optional with capability signaling

Proposal 3: If 256QAM is not supported for Rel-18 eRedCap UE with and without BB bandwidth reduction, following text can be added to the existing UE capabilities of “pdsch-256QAM-FR1” and “pusch-256QAM” in TS 38.306.

Proposal 4: If 2 layers can be optionally supported for Rel-18 eRedCap UE with and without BB bandwidth reduction, similar as Rel-17 RedCap UE, exisiting maxNumberMIMO-LayersPDSCH can be reused with following notes.

- For FR1, 1 DL MIMO layer if 1 Rx branch is supported, and 2 DL MIMO layers if 2 Rx branches are supported; UE features and corresponding capabilities related to more than 2 UE Rx branches or more than 2 DL MIMO layers, as well as UE features and capabilities related to more than 1 UE Tx branch or more than 1 UL MIMO layer are not supported by RedCap UEs indicating support of [FG48-1] or [FG48-2].

- NR_redcap_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 48. NR_redcap_enh | 48-1 | Rel-18 RedCap UEs capable of peak data reduction alone | - Maximum FR1 RedCap UE bandwidth is 20 MHz. - Support both 15kHz SCS and 30KHz SCS - The constraint of vLayers·Qm·f [=] 4 is relaxed to vLayers·Qm·f [=] 0.8. - Rel-17 early indication of Msg1, MsgA and Msg3 for random access - Rel-17 configuration of separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources - Rel-17 configuration of separate initial DL BWP for RedCap UEs - It includes CSS/CORESET for random access - For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included - For separate initial DL BWP used in connected mode as BWP#0 configuration option 1, CD-SSB is included - 1 UE-specific RRC configured DL BWP per carrier - 1 UE-specific RRC configured UL BWP per carrier - RRC reconfiguration of any parameters related to BWP - UE-specific RRC configured DL BWP with CD-SSB or NCD-SSB - NCD-SSB based measurements in RRC-configured DL BWP |  | Yes |  | Network assumes the UE is not a Rel-18 RedCap UE capable of 20MHz+PR1 | Per UE | No | FR1 only |  | - Rel-18 RedCap UEs do not support carrier aggregation or dual connectivity. - It is up to RAN2 whether/how to capture the capabilities for Rel-18 early indication of Rel-18 RedCap UE in Msg 3 and Msg A - A UE supporting this FG is not required to support FG 6-1 | Optional with capability signalling, Rel-18 RedCap UE must indicate either FG48-1 or FG48-2 is supported |
| 48. NR_redcap_enh | 48-2 | Rel-18 RedCap UEs capable of BB bandwidth reduction and peak data reduction | - Maximum FR1 RedCap UE RF bandwidth is 20 MHz. - Support both 15kHz SCS and 30KHz SCS - UE baseband bandwidth reduction - For PUSCH, the maximum number of PRBs that the UE can transmit per slot or per hop is 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS. - For PDSCH, the maximum number of PRBs that the UE can process per slot is 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS. - The constraint of vLayers·Qm·f [=] 4 is relaxed to vLayers·Qm·f [=] 3.2. - Rel-17 early indication of Msg1, MsgA and Msg3 for random access - Rel-17 configuration of separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources - Rel-17 configuration of separate initial DL BWP for RedCap UEs - It includes CSS/CORESET for random access - For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included - For separate initial DL BWP used in connected mode as BWP#0 configuration option 1, CD-SSB is included - 1 UE-specific RRC configured DL BWP per carrier - 1 UE-specific RRC configured UL BWP per carrier - RRC reconfiguration of any parameters related to BWP - UE-specific RRC configured DL BWP with CD-SSB or NCD-SSB - NCD-SSB based measurements in RRC-configured DL BWP |  | Yes |  | Network assumes the UE is not a Rel-18 RedCap UE capable of BW3/PR3+PR1 | Per UE | No | FR1 only |  | - Rel-18 RedCap UEs do not support carrier aggregation or dual connectivity. - It is up to RAN2 whether/how to capture the capabilities for Rel-18 early indication of Rel-18 RedCap UE in Msg 3 and Msg A - A UE supporting this FG is not required to support FG 6-1 | Optional with capability signalling, Rel-18 RedCap UE must indicate either FG48-1 or FG48-2 is supported |
| 48. NR_redcap_enh | 48-3 | Half-duplex FDD operation type A for RedCap UE | 1. Half-duplex FDD operation (instead of full-duplex FDD operation) type A for RedCap UE with/without BB bandwidth reduction | 48-1 or 48-2 | Yes |  | UE is assumed to support FD-FDD in that band | Per band | FDD only | FR1 only |  |  | Optional with capability signaling |

- Reference

- RP-223544, Revised WID on Enhanced support of reduced capability NR devices, Ericsson

- RP-230778, Proposal for PR1 in eRedCap, Moderator (CMCC)

- R1-2304491, Discussion on further UE complexity reduction, vivo

- Appnedix on Rel-17 RedCap UE features

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28. NR_redcap | 28-1 | RedCap UE | 1. Maximum FR1 RedCap UE bandwidth is 20 MHz. 2. Maximum FR2 RedCap UE bandwidth is 100 MHz. 3. Early indication of RedCap UE in Msg.1 for 4-step RACH 4. Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources 5. Separate initial DL BWP for RedCap UEs - It includes CSS/CORESET for random access - For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included - For separate initial DL BWP used in connected mode as BWP#0 configuration option 1, CD-SSB is included 6. 1 UE-specific RRC configured DL BWP per carrier 7. 1 UE-specific RRC configured UL BWP per carrier 8. RRC reconfiguration of any parameters related to BWP 9. UE-specific RRC configured DL BWP with CD-SSB or NCD-SSB 10. NCD-SSB based measurements in RRC-configured DL BWP |  | Yes |  | Network assumes the UE is not a RedCap UE | Per UE | No | No |  | RedCap UEs do not support carrier aggregation or dual connectivity. It is up to RAN2 whether/how to capture the capabilities for early indication of RedCap UE in Msg 3 and Msg A A UE supporting this FG is not required to support FG 6-1 | Optional with capability signaling RedCap UE must indicate this FG is supported |
| 28. NR_redcap | 28-1a | RRC-configured DL BWP without CD-SSB or NCD-SSB | RRC-configured DL BWP without CD-SSB or NCD-SSB | 28-1 | Yes |  |  | Per band | N/A | N/A |  |  | Optional with capability signaling |
| 28. NR_redcap | 28-3 | Half-duplex FDD operation type A for RedCap UE | 1. Half-duplex FDD operation (instead of full-duplex FDD operation) type A for RedCap UE | 28-1 | Yes |  | UE is assumed to support FD-FDD in that band | Per band | FDD only | FR1 only |  |  | Optional with capability signaling |