3GPP TSG RAN WG1 #113	R1-2304534

# Incheon, Korea, May 22nd – May 26th, 2023

Source:	ZTE

Title:	Evaluation on AI CSI feedback enhancement

Agenda Item:	9.2.2.1

Document for:	Discussion and Decision

- Introduction

According to discussion in 3GPP RAN1#112bis-e meeting [1], some progress has been made on evaluation methodology for CSI feedback enhancement. In this contribution, we provide our views on further details for evaluation methodology and share some evaluation results. In our companion contribution [2], potential specification impacts are discussed accordingly.

- Evaluation methodology on CSI compression in spatial and frequency domain

  - CSI training dataset and inference dataset separation

- Questions in RAN1#110 Question: For the evaluation of the AI/ML based CSI compression sub use cases, do you think there is a need to align the method to avoid correlation of samples between the training dataset and inference dataset?

In RAN1#110 meeting [4], the issue of CSI training dataset and inference dataset separation was raised for discussion, since inappropriate dataset separation for AI training/inference may cause an unconvincing conclusion. For example, if the training data and test data show a strong correlation, the model may show over-estimated performances.

Considering dataset construction for training and inference, there are at least two methods can be applied.

- Method 1: Training samples are generated for only one TTI from multiple drops, which brings abundant channel diversity across drops.

- Method 2: Training samples are generated for multiple TTIs from the same drop (or a few drops), which may introduce a strong time correlation across different TTIs of the same drop.

Two different dataset construction types would have different performance gains, wherein AI model has realistic performance gains with Method 1, while over-estimated performance gains are obtained using Method 2. From our perspective, Method 1 can be adopted for better dataset separation, i.e. dataset should be generated in diverse drops rather than in diverse TTIs.

For CSI dataset construction, dataset should be generated in diverse drops rather than in diverse TTIs to ensure dataset diversity, e.g., training samples are generated from various drops and only one TTI is used in a single drop.

- Remaining issues on the template for CSI compression

  - CSI feedback overhead for eventual KPI

- In RAN1#112bis-e meeting Working Assumption For the template of Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, the CSI feedback overhead for the metric of eventual KPI (e.g., mean/5% UPT) is re-determined as: - CSI feedback overhead A: <=β* 80 bits. - CSI feedback overhead B: β* (100bits – 140 bits). - CSI feedback overhead C: >=β* 230 bits. - Note: β=1 for max rank = 1, and β=1.5 for max rank = 2/3/4. - FFS for rank 2/3/4, whether to add an additional CSI feedback overhead D: >=γ* 230 bits, γ= [1.9], and limit the range of CSI feedback overhead C as:β* 230 bits-γ* 230 bits. - Note: companies additionally report the exact CSI feedback overhead they considered

In RAN1#112b-e meeting [1], a working assumption was drawn, where the metric of CSI feedback overhead for eventual KPI is classified into A, B, C, and the FFS is whether to add an additional CSI feedback overhead D for rank2/3/4. The concern is that CSI feedback overhead C (>=β* 230 bits) with the β value of 1.5 has a wide range, where multiple feedback overhead values fall in this range which may be hard to align the similar overhead  assumptions among companies. For example, eType II PC5 (>400 bits)-PC8(>700 bits) are included in the CSI feedback range C, and the rank distribution differentiates among companies to a large extent. So for rank 2/3/4, it’s necessary to add an additional CSI feedback overhead D: >=γ* 230 bits, γ= 1.9, and limit the range of CSI feedback overhead C as:β* 230 bits-γ* 230 bits.

For rank 2/3/4, it is necessary to add an additional CSI feedback overhead D: >=γ* 230 bits, γ= 1.9, and limit the range of CSI feedback overhead C as:β* 230 bits-γ* 230 bits.

- Evaluation of the high-resolution quantization of the ground-truth CSI

In RAN1#110bis-e meeting [7], several high-resolution quantization methods were mentioned, e.g., Float32, Float16, R16 Type II-like method with new parameter combinations. Several companies showed that R16 Type II-like method with new parameter combinations can achieve attractive performances with low overhead [8]-[11]. This section shares our evaluation for R16 Type II-like method with new parameter combinations, wherein two parameter combinations mentioned in [8] are used, and float32 format is used as an ideal performance baseline. For convenience, the two parameter combinations are denoted by eType-II PC9 and eType-II PC10 respectively, and the details are as following:

- eType-II PC9: L = 8, pv=0.75, β=0.31, Reference amplitude = 8 bits, Differential amplitude = 4 bits, Phase = 6 bits

- eType-II PC10: L = 10, pv=0.9, β=0.31, Reference amplitude = 8 bits, Differential amplitude = 4 bits, Phase = 6 bits

To achieve a higher resolution, the two parameter combinations of eType-II PC9 and eType-II PC10 have larger L values and larger pv values than legacy eType-II PCs, to achieve a higher resolution. Table 4-1 shows the maximum overhead and SGCS (first layer and second layer) for the ground-truth CSI quantized by eType-II PC8, eType-II PC9, eType-II PC10 and float32 for rank 2. The overheads of the eType-II PCs are compressed to only 1.2% to 3.8% compared to float32. Both two layer’s SGCS of eType II PC8 are below 0.9, both two layer’s SGCS of eType II PC10 above 0.9, and the first layer’s SGCS of eType II PC9 is above 0.9 with its second layer’s SGCS below 0.9.

Table 4-1: Maximum overhead and SGCS for the ground-truth CSI quantization methods for Rank = 2

|  | eType II PC8 | eType II PC9 | eType II PC10 | float 32 |
| --- | --- | --- | --- | --- |
| Maximum overhead (bits) | 635(1.2%) | 1358(2.5%) | 2011(3.8%) | 53248(100%) |
| SGCS | 0.8792/0.8037 | 0.9334 /0.8852 | 0.9561 /0.9249 | 1.0/1.0 |

Table 4-2 shows SGCS performance for models trained on data in different quantization methods, where the test data applies float32 format. Compared to float 32 format, the model trained on eType II PC8 shows a large performance loss in terms of SGCS.

Tabel 4-2: SGCS for output of model trained on quantized dataset

| Feedback Overhead(bit) | model trained on eType II PC8 dataset | model trained on eType II PC9 dataset | model trained on eType II PC10 dataset | model trained on float32 dataset |
| --- | --- | --- | --- | --- |
| 120 bits | 0.6923/0.5589 (-3.82%/-5.05%) | 0.7025/0.5842 (-2.40%/-0.75%) | 0.7157/0.5873 (-0.57%/-0.22%) | 0.7198/0.5886 (0%/0%) |
| 644 bits | 0.8711/0.7951 (-5.50%/-8.19%) | 0.9083/0.8458 (-1.46%/-2.34%) | 0.9182/0.8603 (-0.39%/-0.67%) | 0.9218/0.8661 (0%/0%) |

The R16 Type II method with larger L values and larger pv values than legacy eType-II PCs has the possibility to achieve high resolution quantization with low overhead.

To evaluate various high-resolution quantization methods of the ground-truth CSI for model training, companies should use float32 dataset as test data for easy comparison.

New parameter combinations for enhanced R16 Type II method should be supported/evaluated to achieve high resolution CSI with acceptable overhead for ground-truth CSI collection.

- Evaluation of CQI determination

Some agreements was achieved in RAN1#112 meeting [3] and RAN1#112bis-e meeting [1], some CQI determination methods were mentioned as following.

- In RAN1#112 meeting Agreement In CSI compression using two-sided model use case, further study the following options for CQI determination in CSI report, if CQI in CSI report is configured. - Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 1a: CQI is calculated based on target CSI with realistic channel measurement - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment - Option 1c: CQI is calculated based on legacy codebook - Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW. - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder. - Other options are not precluded - Note1: feasibility of different options should be evaluated - Note2: Gap analyses between the UE side CQI calculation results and the NW side results, as well as the impact on the scheduling performance should be evaluated - Note3: Complexity of CQI calculation needs to be evaluated, including the computing complexity and potential RS/signaling overhead - In RAN1#112bis-e meeting Agreement For the evaluation of CSI compression, companies are allowed to report (by introducing an additional field in the template to describe) the specific CQI determination method(s) for AI/ML, e.g., - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment - Option 2a-1: The CSI reconstruction part for CQI calculation at the UE same as the actual CSI reconstruction part at the NW - Option 2a-2: The CSI reconstruction part for CQI calculation at the UE is a proxy model, which is different from the actual CSI reconstruction part at the NW - Option 2b: CQI is calculated using two stage approach, UE derives CQI using precoded CSI-RS transmitted with a reconstructed precoder - Option 1a: CQI is calculated based on the target CSI from the realistic channel estimation - Option 1b: CQI is calculated based on the target CSI from the realistic channel estimation and potential adjustment - Option 1c: CQI is calculated based on traditional codebook - Other options if adopted, to be described by companies

We have provided our analysis of pros and cons for different options in our companion contribution [2]. In this contribution, the performance of Option 1a, Option 2a-1 and Option 2a-2 for CQI determination are evaluated in SLS. For Option 2a-2, more details can be found in our companion contribution [2]. In Figure 1, the performance of the different CQI determination options compared to eType-II scheme and Ideal CSI scheme are shown. With regard to average sector throughput gain, Option 2a-1 and Option 2a-2 have similar performance. The average sector throughput gains of Option 2a-1 and Option 2a-2 are obviously higher than Option 1a and eType-II scheme. Option 1a has worse performance than eType-II scheme in low CSI feedback overhead, and better performance than eType-II scheme in high CSI feedback overhead, which is due to that the mismatch on PMI and CQI is hard to be adjusted by gNB side unless high SINR is observed.

Figure 1 Average sector throughput gain

However, for Option 2a-1, it is an issue that CSI reconstruction model is not available at the UE, which can be an upper-bound to compare different options. For Option 2a-2, the working mode is similar as model monitoring as we discussed in [2] and section 6. Therefore, for further study on CQI determination, the feasibility of practical deployment should be considered for different options.

Option 2a-1 is not practical in reality, but it can provide a performance upper-bound to compare different options.

With regard to average sector throughput gain, Option 2a-1 and Option 2a-2 have similar performance. The average sector throughput gains of Option 2a-1 and Option 2a-2 are obviously higher than Option 1a and eType-II scheme.

Option 1a has worse performance than eType-II scheme in low CSI feedback overhead, and better performance than eType-II scheme in high CSI feedback overhead, which is due to that the mismatch on PMI and CQI is hard to be adjusted by gNB side.

- Evaluation of model performance monitoring

In RAN1#112b-e meeting [1], several agreements were archived on intermediate KPI based monitoring as following.

- In RAN1#112bis-e meeting Agreement #1 To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, the model monitoring methodology is considered as: - Step1: Generate test dataset including K test samples - FFS how to obtain the K test samples - Step2: For each of K test samples, a bias factor of monitored intermediate KPI () is calculated as a function of , where is the actual intermediate KPI, and is the genie-aided intermediate KPI. - Step3: Calculate the statistical result of the over K test samples which represents the monitoring accuracy performance. - Note: is introduced for the evaluation and comparison purpose; it may not be available in the real network. - Note: the complexity, overhead and latency of the monitoring scheme are reported by companies. FFS how to evaluate latency. Agreement #2 To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, for Step2 of the model monitoring methodology, the per sample is considered for - Case 1: NW side monitoring of intermediate KPI, where the monitoring accuracy is evaluated for a given ground-truth CSI format (e.g., quantized ground-truth CSI with 8 bits scalar, R16 eType II-like method, etc.) or SRS measurements, where - is calculated with the output CSI at the NW side and the given ground-truth CSI format or SRS measurements. - is calculated with output CSI (as for ) and the ground-truth CSI of Float32. - Note: if Float32 is used for , the monitoring accuracy is 100% if and are based on the same CSI sample. - Case 2: UE side monitoring of intermediate KPI with a proxy model, where the monitoring accuracy is evaluated for the output of the proxy model at UE: - Case 2-1: the proxy model is a proxy CSI reconstruction part, and is calculated based on the inference output of the proxy CSI reconstruction part at UE and the ground-truth CSI. - Note: if the proxy CSI reconstruction model is the same as the actual CSI reconstruction model at the NW, the monitoring accuracy is 100% - Case 2-2: the proxy model directly outputs intermediate KPI () - is calculated with the output CSI at the NW side and the same ground-truth CSI. - FFS how to train the proxy model and the resulting monitoring performance, to be reported by companies. - FFS whether/how to evaluate the generalization performance of the proxy model. - Case 3: others are not precluded Agreement #3 To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, is in forms of - Option 1: Gap between and , i.e. ; - Monitoring accuracy is the percentage of the samples for which , where is a threshold of the intermediate KPI gap. - Option 2: Binary state where and have different relationships to their threshold(s), i.e., , where can be same or different from - Monitoring accuracy is the percentage of the samples for which . - FFS other metrics: Misdetection, False alarm, etc. - FFS the values of , , . - FFS whether/how to evaluate the monitoring metrics for Rank>1

Two options were agreed for calculating the KPIDiff in RAN1#112bis-e meeting [1]. Option 1 uses the KPI gap between KPIActual and KPIGenie to evaluate the monitoring accuracy, which cannot reflect whether the model performance is applicable or not. For example, we assume actual SGCS is 0.5 and genie-aided SGCS is 0.55, hence the SGCS gap is 0.05, which can satisfy the predefined monitoring threshold. However, it is meaningless for evaluating this monitoring performance because AI/ML model performance is so bad that it is not applicable for current situation. Option 2 clearly shows the relationship between the monitoring threshold and the results of KPIActual and KPIGenie, where KPIDiff can reflect whether the model performance is applicable if the monitoring threshold is reasonably set.

KPIDiff in Option 1 cannot reflect whether the model maintains its high performance, while KPIDiff in Option 2 can actually reflect the model performance.

Option 2 should be adopted for evaluating monitoring metrics for intermediate KPI based monitoring.

In the following section, the monitoring accuracy for Case 1 and Case 2-1 is evaluated based on the method of Option 2, where SGCS is used as intermediate KPI.

Case 1: NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side

Intermediate KPIs are calculated by NW based on traditional CSI and CSI reconstruction model output. As shown in Figure 2, due to the fact that network cannot directly obtain the ground-truth label to calculate the monitoring metrics, UE should report ground-truth CSI to for network to calculate the monitoring metrics. In order to improve the performance of network-based model monitoring, a higher resolution ground-truth label needs to be reported by UE.

Figure 2 Intermediate KPIs calculated by NW based on traditional CSI and CSI reconstruction model output

AI model performance is considered invalid when the SGCS between the reconstructed CSI and the target CSI is less than a threshold, otherwise valid. With the model monitoring method, UE reports a reference CSI, and gNB calculates the SGCS between the reconstructed CSI and the reference CSI. gNB determines AI model performance is invalid when the SGCS between the reconstructed CSI and the reference CSI is less than the threshold, otherwise valid. Sometimes wrong determination will occur.

In this evaluation, the feedback overhead of monitored CSI is equal to that of eType-II PC8, the mean SGCS of monitored CSI is 0.9218, and the CSI of eType-II PC8 and the CSI of eType-II PC10 are used as the ground-truth CSI respectively. In this simulation, the same threshold value is applied to KPItℎ 2 and KPItℎ 3 and the monitoring accuracy is the percentage of the samples for which KPIDiff=0. Thresholds of 0.7 and 0.8 are used respectively. For convenience, the details of eType-II PC10 [6] are followed:

- eType-II PC10: L = 10, pv=0.9, β=0.31, Reference amplitude = 8 bits, Differential amplitude = 4 bits, Phase = 6 bits

Table 6-1 shows the evaluation result for Rank 1 in terms of monitoring accuracy. As be shown from Table 6-1, under a high feedback overhead of AI model, for the SGCS threshold of 0.8, with the reference CSI of eType-II PC8, the monitoring accuracy is lower than 90%, while for others the monitoring accuracy is higher than 95%.

Table 6-1: Performance of model performance monitoring methods with different types of ground-truth CSI

| Reference CSI: eType-II PC8 |  |
| --- | --- |
| Threshold | Monitoring Accuracy |
| 0.7 | 96.63% |
| 0.8 | 88.31% |
| eType-II PC10 |  |
| Threshold | Monitoring Accuracy |
| 0.7 | 99.32% |
| 0.8 | 97.61% |

For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side, the monitoring accuracy is lower than 90% based on the reference CSI of eType-II PC8 when the SGCS threshold is 0.8, while for SGCS threshold smaller than 0.8, the monitoring accuracy is higher than 95%.

For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side, the monitoring accuracy is higher than 97% based on the reference CSI of eType-II PC10 when the SGCS threshold is 0.8, and for SGCS threshold smaller than 0.8, the monitoring accuracy is higher than 99%.

For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side, eType II CSI with new parameter combination can be used as ground-truth CSI at NW to achieve a good monitoring accuracy.

It’s up to companies to report the threshold to determine the monitoring accuracy.

Case 2-1: UE-side monitoring based on the output of the proxy CSI reconstruction model at the UE side

Intermediate KPIs are calculated by UE based on the output of the proxy CSI reconstruction model at the UE side. As shown in Figure 3, UE has its own proxy reconstruction part in CSI generation model which is not the same as the actual CSI reconstruction model used at the NW side. Hence, CSI generation model has two output modules. The first one is feedback part, which is used for model input of CSI reconstruction model. The second output (i.e., proxy output) is for model monitoring. As shown in the Figure 3, the proxy output data is trained to imitate reconstruction model output as much as possible via knowledge distillation technology [5]. By doing this, UE can monitor the KPIActual to check the KPIGenie. If the monitoring metrics between input and proxy output cannot meet a target requirement, so as the monitoring metrics between input and actual output.

Figure 3 Intermediate KPIs calculated by UE based on the output of the proxy CSI reconstruction model at UE

In this method, UE calculates the SGCS between the monitored CSI and the target CSI. AI model performance is invalid when the SGCS between the proxy output CSI and the target CSI is less than the threshold, otherwise valid. Sometimes false detections are observed since UE only relies on the proxy output CSI to do model monitoring. In this simulation, the same threshold value is applied to KPItℎ 2 and KPItℎ 3, and the monitoring accuracy is the percentage of the samples for which KPIDiff=0. As is shown from Table 6-2, for the SGCS thresholds of 0.7 and 0.8, the monitoring accuracy is higher than 96%.

Table 6-2: Performance of model performance monitoring at UE side

| Threshold | Monitoring Accuracy |
| --- | --- |
| 0.7 | 98.22% |
| 0.8 | 96.11% |

For model performance monitoring at UE side, output CSI of proxy model at UE can be used as CSI reconstruction part output to achieve a good monitoring accuracy.

Discussion on False alarm and Misdetection for model monitoring performance

False alarm and Misdetection for model monitoring performance is still FFS. The monitoring accuracy shows the monitoring performance in a coarse manner, and False alarm and Misdetection show the monitoring performance in a fine-grained manner. False alarm is the case that model performance is judged to be invalid but it is actually valid.  Misdetection is the case that model performance is judged to be valid but it is actually invalid. False alarm rate (FAR) can be used for the statistic of False alarm, and miss alarm rate (MAR) can be used for the statistic of misdetection. The definition of FAR and MAR are as following:

- FAR:

- samples for (KPIActual<KPItℎ 2,KPIGenie>KPItℎ 3)samples for (KPIGenie>KPItℎ 3)×100%

- MAR:

- samples for (KPIActual>KPItℎ 2,KPIGenie<KPItℎ 3)samples for (KPIGenie<KPItℎ 3)×100%

Table 6-3 shows the evaluation results of Rank 1 for Case 1 in terms of FAR and MAR, where KPItℎ 2 and KPItℎ 3 use the same threshold value. For the threshold of 0.8, based on the groud-truth CSI of eType-II PC8, the FAR is 11.75%, and the MAR is 10.99%; based on the groud-truth CSI of eType-II PC10, the FAR is 1.86%, and the MAR is 8.38%. For the threshold of 0.7, based on the groud-truth CSI of eType-II PC8, the FAR is 3.05%, and the MAR is 21.91%; based on the groud-truth CSI of eType-II PC10, the FAR is 0.42%, and the MAR is 15.6%.

Table 6-3: FAR and MAR of model performance monitoring methods with different types of ground-truth CSI

| Reference CSI: eType-II PC8 |  |  |  |
| --- | --- | --- | --- |
| Threshold | FAR | MAR | Monitoring Accuracy |
| 0.7 | 3.05% | 21.91% | 96.63% |
| 0.8 | 11.75% | 10.99% | 88.31% |
| Reference CSI: eType-II PC10 |  |  |  |
| Threshold | FAR | MAR | Monitoring Accuracy |
| 0.7 | 0.42% | 15.60% | 99.32% |
| 0.8 | 1.86% | 8.38% | 97.61% |

For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side, the FAR (False Alarm Rate) is higher than 11% based on the ground-truth CSI of eType-II PC8 when the SGCS threshold is 0.8, while for other cases, the FAR is lower than 4%.

For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side, eType II CSI with new parameter combination achieve a good performance for monitoring accuracy and FAR/MAR.

For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side, eType II CSI with new parameter combination can be used as ground-truth CSI at NW to achieve a good FAR and MAR.

FAR (false alarm rate) and MAR (miss alarm rate) can be used in addition to monitoring accuracy to analyze monitoring performance. The definition of FAR and MAR are as following:

- FAR:

- samples for (KPIActual<KPItℎ 2,KPIGenie>KPItℎ 3)samples for (KPIGenie>KPItℎ 3)×100%

- MAR:

- samples for (KPIActual>KPItℎ 2,KPIGenie<KPItℎ 3)samples for (KPIGenie<KPItℎ 3)×100%

- Evaluation Results of CSI compression

We perform a preliminary simulation on spatial-frequency domain CSI compression sub-use case to evaluate the performance of AI models, and the Rel-16 eTypeII is also simulated as baseline. In SLS, we further compare the throughput between AI-based feedback and ideal CSI feedback to see how much performance margin that AI-based approach has in comparison with an upper bound, where ideal eigenvector feedback is used as ideal CSI feedback. For Rel-16 eTypeII, 8 parameter combinations are used, and wherein paramCombination 7 and paramCombination 8 are directly extended for Rank=3 and Rank=4. In order to fairly compare the SLS performance, AI-based approach has the same overhead as that of Rel-16 eTypeII, i.e., corresponding to the 8 parameter combinations. For SLS evaluation metric, two types (Option 1 and Option 2b as mentioned in Section 3.3) of overhead calculation are adopted, wherein Option 1 marked as Maximum overhead, and Option 2b is Weighted Avg. overhead. The detailed simulation assumptions are shown in Table 8-1. In addition, detailed AI model training parameters are listed in Table 8-2. The system-level channel data is generated from 3000 simulation drops and 210 UEs per simulation drop, which results in 630K samples in total. Then, the datasets are randomly divided into three parts which are training, validation, and testing datasets with 600K, 10K, and 20K samples respectively. In our evaluation, ideal sub-band eigenvectors are used as the input of AI models for training, validation and testing. The number of parameters for a model ranges from about 9M-11M, where the CSI generation part is about 4M-5M and the CSI reconstruction part is 5M-6M. In addition, the FLOPs of CSI generation part and CSI reconstruction part are approximately 25M-26M and 27M-29M, respectively.

Based on the table templates, our evaluation results for CSI compression are collected in 4 tables in the attachment as following:

- Table 1. 1-on-1 joint training

- Table 2. Evaluation results for CSI compression with model generalization

- Table 3. Evaluation results for CSI compression with model scalability

- Table 5. separate training

  - Rank 1

In this section, we evaluate the intermediate KPIs, e.g. SGCS, between our AI approach and the Rel-16 eType II. In order to explore the eventual UPT performance of our AI model, the SLS is performed for the case of up to 4 layers MU scheduling under FTP3 traffic model with 70% RU. These results are shown in Table 1. 1-on-1 joint training (1st sheet: FTP, Max rank = 1). According to the Table 1, we have following observations:

For Rank 1, AI-based CSI reconstruction completely outperforms the Rel-16 eType II in term of SGCS. With the low feedback overhead, AI-based CSI recovery can obtain up to 6.43% SGCS gains; with the medium feedback overhead, AI-based CSI recovery can obtain up to 6.44% SGCS gains; with the high feedback overhead, AI-based CSI recovery can obtain up to 7% SGCS gains.

For Rank 1, AI-based CSI reconstruction achieves minor average UPT gain, where less than 1% average UPT gain is obtained over Rel-16 eTypeII with the same feedback overhead.

For Rank 1, for 5% tail UPT, AI-based CSI reconstruction can achieve up to 20.43% performance gain with the CSI feedback overhead A, up to 10.13% performance gain with the CSI feedback overhead B, and up to 0.85% performance gain with the CSI feedback overhead C.

For Rank 1, the margin between AI-based CSI recovery and ideal CSI feedback is quite small for average UPT, as AI-based CSI reconstruction and Rel-16 eTypeII can already work well in single layer scheduling.

For Rank 1, the margin between AI-based CSI recovery and ideal CSI feedback is 5.6% for 5% tail UPT.

  - Up to Rank 2

In this section, we evaluate the intermediate KPIs, e.g. SGCS, between our AI approach and the Rel-16 eType II for Rank 2. In order to explore the eventual UPT performance of our AI model, the SLS is performed for the case of up to 8 layers MU scheduling under FTP3 traffic model with 50% and 70% RU. These results are shown in Table 1. 1-on-1 joint training (2nd sheet: FTP, Max rank = 2). According to the evaluation results, we have following observations:

For Rank 2, AI-based CSI reconstruction completely outperforms the Rel-16 eType II in term of SGCS. With the low feedback overhead, AI-based CSI recovery can obtain up to 4.67% SGCS gains in Layer 1 and 5.92% SGCS gains in Layer 2; with the medium feedback overhead, AI-based CSI recovery can obtain up to 4.04% SGCS gains in Layer 1 and 5.90% SGCS gains in Layer 2; with the high feedback overhead, AI-based CSI recovery can obtain up to 6.44% SGCS gains in Layer 1 and 9.95% SGCS gains in Layer 2.

For up to Rank 2, under the case of 50% RU, AI based CSI reconstruction has about up to 4.09% average UPT gain over the Rel-16 eType II with the CSI feedback overhead A, up to 4.08% average UPT gain over the Rel-16 eType II with the CSI feedback overhead B, up to 4.45% average UPT gain over the Rel-16 eType II with the CSI feedback overhead C, and up to 4.55% average UPT gain over the Rel-16 eType II with the CSI feedback overhead D.

For up to Rank 2, under the case of 50% RU, for 5% tail UPT, AI based CSI reconstruction has about up to 9.72% performance gain over the Rel-16 eType II with the CSI feedback overhead A, up to 20.00% performance gain over the Rel-16 eType II with the CSI feedback overhead B, up to 25.62% performance gain over the Rel-16 eType II with the CSI feedback overhead C, and up to 14.37% performance gain over the Rel-16 eType II with the CSI feedback overhead D.

With regard to up to Rank 2, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 5.44% under the case of 50% RU.

With regard to up to Rank 2, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 24.04% under the case of 50% RU.

For up to Rank 2, under the case of 50% RU, AI-based CSI reconstruction can achieve up to 48.60% overhead reduction with the CSI feedback overhead A,up to 39.80% overhead reduction with the CSI feedback overhead B, up to 38.66% overhead reduction with the CSI feedback overhead C, and up to 40.22% overhead reduction with the CSI feedback overhead D.

For up to Rank 2, under the case of 70% RU, AI based CSI reconstruction has about up to 5.30% average UPT gain over the Rel-16 eType II with the CSI feedback overhead A, up to 4.92% average UPT gain over the Rel-16 eType II with the CSI feedback overhead B, up to 5.23% average UPT gain over the Rel-16 eType II with the CSI feedback overhead C, and up to 6.94% average UPT gain over the Rel-16 eType II with the CSI feedback overhead D.

For up to Rank 2, under the case of 70% RU, for 5% tail UPT, AI based CSI reconstruction has about up to 10.26% performance gain over the Rel-16 eType II with the CSI feedback overhead A, up to 15.02% performance gain over the Rel-16 eType II with the CSI feedback overhead B, up to 13.67% performance gain over the Rel-16 eType II with the CSI feedback overhead C, and up to 16.26% performance gain over the Rel-16 eType II with the CSI feedback overhead D.

With regard to up to Rank 2, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 9.50% under the case of 70% RU.

With regard to up to Rank 2, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 28.75% under the case of 70% RU.

For up to Rank 2, under the case of 70% RU, AI-based CSI reconstruction can achieve up to 44.44% overhead reduction with the CSI feedback overhead A,up to 38.12% overhead reduction with the CSI feedback overhead B, up to 36.47% overhead reduction with the CSI feedback overhead C and up to 40.41% overhead reduction with the CSI feedback overhead D.

The SGCS performance for AI/ML-based CSI compression presents positive correlation with the throughput performance for up to Rank 2.

AI/ML approaches show better average throughput performance with heavy traffic load for up to Rank 2.

  - Up to Rank 4

In this section, we evaluate the intermediate KPIs, e.g. SGCS, between our AI approach and the Rel-16 eType II for Rank 4. In order to explore the eventual UPT performance of our AI model, the SLS is performed for the case of up to 8 layers MU scheduling under FTP3 traffic model with 50% and 70% RU, and under full buffer traffic model. These results are shown in Table 1. 1-on-1 joint training (4th sheet: FTP, Max rank = 4 and 8th sheet: Full buffer, Max rank = 4). According to the evaluation results, we have following observations:

For Rank 4, AI-based CSI reconstruction completely outperforms the Rel-16 eType II in term of SGCS. With the low feedback overhead, AI-based CSI recovery can obtain up to 3.96% SGCS gains in Layer 1,  6.77% SGCS gains in Layer 2, 32.12% SGCS gains in Layer 3, and 91.85% SGCS gains in Layer 4; with the medium feedback overhead, AI-based CSI recovery can obtain up to 3.11% SGCS gains in Layer 1, 3.72% SGCS gains in Layer 2, 15.02% SGCS gains in Layer 3, and 37.16% SGCS gains in Layer 4; with the high feedback overhead, AI-based CSI recovery can obtain up to 5.64% SGCS gains in Layer 1, 4.79% SGCS gains in Layer 2, 14.92% SGCS gains in Layer 3, and 31.94% SGCS gains in Layer 4.

For up to Rank 4, under the case of 50% RU, AI based CSI reconstruction has about up to 7.95% average UPT gain over the Rel-16 eType II with the CSI feedback overhead A, up to 3.11% average UPT gain over the Rel-16 eType II with the CSI feedback overhead B, up to 7.64% average UPT gain over the Rel-16 eType II with the CSI feedback overhead C, and up to 6.88% average UPT gain over the Rel-16 eType II with the CSI feedback overhead D.

For up to Rank 4, under the case of 50% RU, for 5% tail UPT, AI based CSI reconstruction has about up to 9.25% performance gain over the Rel-16 eType II with the CSI feedback overhead A, up to 6.17% performance gain over the Rel-16 eType II with the CSI feedback overhead B, up to 9.47% performance gain over the Rel-16 eType II with the CSI feedback overhead C, and up to 8.11% performance gain over the Rel-16 eType II with the CSI feedback overhead D.

With regard to up to Rank 4, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 25.06% under the case of 50% RU.

With regard to up to Rank 4, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 37.72% under the case of 50% RU.

For up to Rank 4, under the case of 50% RU, AI-based CSI reconstruction can achieve up to 56.01% overhead reduction with the CSI feedback overhead A, up to 36.10% overhead reduction with the CSI feedback overhead B, up to 38.14% overhead reduction with the CSI feedback overhead C, and up to 25.40% overhead reduction with the CSI feedback overhead D.

For up to Rank 4, under the case of 70% RU, AI based CSI reconstruction has about up to 12.35% average UPT gain over the Rel-16 eType II with the CSI feedback overhead A, up to 5.00% average UPT gain over the Rel-16 eType II with the CSI feedback overhead B, up to 8.40% average UPT gain over the Rel-16 eType II with the CSI feedback overhead C, and up to 6.88% average UPT gain over the Rel-16 eType II with the CSI feedback overhead D.

For up to Rank 4, under the case of 70% RU, for 5% tail UPT, AI based CSI reconstruction has about up to 23.27% performance gain over the Rel-16 eType II with the CSI feedback overhead A, up to 10.20% performance gain over the Rel-16 eType II with the CSI feedback overhead B, up to 6.83% performance gain over the Rel-16 eType II with the CSI feedback overhead C, and up to 14.20% performance gain over the Rel-16 eType II with the CSI feedback overhead D.

With regard to up to Rank 4, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 25.09% under the case of 70% RU.

With regard to up to Rank 4, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 35.58% under the case of 70% RU.

For up to Rank 4, under the case of 70% RU, AI-based CSI reconstruction can achieve up to 60.29% overhead reduction with the CSI feedback overhead A,up to 36.41% overhead reduction with the CSI feedback overhead B, up to 37.93% overhead reduction with the CSI feedback overhead C, and up to 24.81% overhead reduction with the CSI feedback overhead D.

For up to Rank 4, under full buffer traffic, AI based CSI reconstruction has about up to 8.51% average UPT gain over the Rel-16 eType II with the CSI feedback overhead A, up to 7.44% average UPT gain over the Rel-16 eType II with the CSI feedback overhead B, up to 9.95% average UPT gain over the Rel-16 eType II with the CSI feedback overhead C, and up to 9.98% average UPT gain over the Rel-16 eType II with the CSI feedback overhead D.

For up to Rank 4, under full buffer traffic, for 5% tail UPT, AI based CSI reconstruction has about up to 5.36% performance gain over the Rel-16 eType II with the CSI feedback overhead A, up to 4.72% performance gain over the Rel-16 eType II with the CSI feedback overhead B, up to 3.59% performance gain over the Rel-16 eType II with the CSI feedback overhead C, and up to 6.15% performance gain over the Rel-16 eType II with the CSI feedback overhead D.

With regard to up to Rank 4, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 19.75% under full buffer traffic.

With regard to up to Rank 4, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 45.21% under full buffer traffic.

For up to Rank 4, under full buffer traffic, AI-based CSI reconstruction can achieve up to 70.53% overhead reduction with the CSI feedback overhead A,up to 47.74% overhead reduction with the CSI feedback overhead B, up to 42.59% overhead reduction with the CSI feedback overhead C and up to 39.40% overhead reduction with the CSI feedback overhead D.

The SGCS performance for AI/ML-based CSI compression presents positive correlation with the throughput performance for up to Rank 4.

AI/ML approaches show better average throughput performance with heavy traffic load for up to Rank 4.

The case of rank>1 should be prioritized in future study.

  - Model generalization

In this section, we provide some results for the evaluation of model generalization capability.

Various deployment scenarios:

These results are shown in Table 2. Evaluation results for CSI compression with model generalization (2nd sheet: Deployment scenarios, rank=2,4). According to the evaluation results, we have following observations:

Case 3(AI model trained in mixed deployment scenarios) shows good generalization performance for various scenarios included in the mixed deployment scenarios.

The AI model trained in UMa/UMi scenario shows good generalization performance to InH dataset, while the AI model trained in InH scenario shows generalization degradation to UMa/UMi scenario.

The AI model trained in the scenario of InH and fine-tuned with the dataset from a UMa/UMi scenario shows 1%-5% SGCS performance gain over the AI model trained only in InH scenario and tested in UMa/UMi scenario.

With the increased samples of fine-tuning dataset from testing scenario, the AI model shows better performance in testing scenario.

Various configurations on bandwidth:

We simulate AI generalization for different datasets with bandwidths of 10MHz (52RBs) and 20MHz (104RBs). According to agreements made in previous meetings, 4RBs per sub-band for 10MHz and 8RBs per sub-band for 20MHz are adopted, so the AI input sizes keep the same for both datasets. These results are shown in Table 3. Evaluation results for CSI compression with model scalability (2nd sheet: Bandwidths, rank=2 or 4). According to the evaluation results, we have following observations:

AI/ML approaches can achieve good generalization performance for the case that the training dataset and testing dataset are generated with different bandwidth configurations but with the same model input size.

Various configurations on antenna port number:

We also simulate AI generalization for various antenna port numbers, where three cases of generalization are considered. These results are shown in Table 3. Evaluation results for CSI compression with model scalability (4th sheet: Tx ports, rank=2 or 4). According to the evaluation results, we have following observations:

Case 3(AI model trained in mixed configurations of antenna port numbers) shows good generalization performance for various antenna port numbers.

AI model trained in the configuration of 32 antenna ports would suffer from performance degradation in16 antenna ports when input data is zero padding from 16 antenna ports to 32 antenna ports.

Model generalization across layers:

Model generalization across layers is shown in Table 3. Evaluation results for CSI compression with model scalability (8th sheet: Rank-layer Num, Max rank = 4). According to the evaluation results, we have following observations:

The AI/ML model shows a good generalization capability across layers in the following cases:

Trained with data from all layers and tested for different layers

Trained with data from the first layer and tested for different layers

Trained with data from the first two layers and tested for different layers

  - Performance evaluation for Type 3 sequential training

Performance for Type 3 sequential training is evaluated in this section, where both NW-first training and UE-first training are considered. These results are shown in Table 5. separate training. According to the evaluation results, we have following observations:

For both Type 3 training with NW-first and Type 3 training with UE-first, two cases have similar performance. Type 3 training with NW-first has nearly the same performance as Type 1 training, while Type 3 training with UE-first has a slightly lower performance than Type 1 training.

  - Evaluation of quantization type (VQ vs SQ)

Evaluation of quantization is shown in Table 1. 1-on-1 joint training (7th sheet: IntermediateKPI). One is vector quantization with training awareness Case 2-2, where the quantization method/parameters are updated in together with the AI/ML models during the training. When training is finished, the final quantization codebook is applied for the inference phase. The other is scalar quantization with training awareness Case 2-1, where fixed/pre-configured quantization method/parameters are applied during the training phase. The same quantization codebook is applied for the inference phase, and 2-bit uniform quantization is used for the scalar quantization. According to the evaluation results, the we have following observations:

2-bit scalar quantization with training awareness Case 2-1 has similar SGCS gain with vector quantization with training awareness Case 2-2, where the scalar quantization has a slightly higher SGCS gain over vector quantization under X/Y overhead, and has a slightly lower SGCS gain over vector quantization under Z overhead.

- Evaluation Results of CSI Prediction

Based on the templates, our evaluation results for CSI compression are collected in in 2 tables in the attachment as following:

- Table 6. CSI prediction without generalization

- Table 7. CSI prediction with generalization

  - CSI prediction without generalization

We perform a preliminary simulation on the sub-use case of CSI prediction to evaluate the performance of AI models. In addition, the nearest historical CSI and a non-AI approach (i.e., Wiener filtering) are adopted for performance comparison. These results are shown in Table 6. CSI prediction without generalization. According to the evaluation results, we have following observations and proposals:

Both AI-based CSI prediction and Wiener filtering-based CSI prediction can completely outperform the nearest historical CSI. Moreover, Wiener filtering-based non-AI CSI prediction shows even better performance than the AI-based approach when enough historical CSIs are applied.

The prediction accuracy of AI-based approach and Wiener filtering-based approach drops seriously when the predicted time becomes longer due to the channel aging. However, AI-based CSI prediction can maintain the performance for a longer time than Wiener filtering-based CSI prediction.

After eTypeII quantization on the predicated CSI via post-processing, AI/ML based CSI prediction and Wiener filtering-based non-AI CSI prediction have similar SGCS performances, where both the methods completely outperform the CSI predication based on the nearest historical CSI.

The prediction accuracy of both AI-based approach and Wiener filtering-based approach improve with the increased number of historical CSIs as input and presents certain positive correlation with the number of input historical CSIs.

AI-based CSI prediction shows less or even no performance gain over Wiener filtering-based algorithm with the increased number of historical CSIs in model input.

Various lengths of observation window and prediction window should be evaluated to have a fair comparison between AI-based CSI prediction and non-AI based CSI prediction

The prediction accuracy of AI-based approach outperforms non-AI approaches when sub-band eigenvectors are adopted as model input.

The performance of eigenvector prediction for AI-based approach drops dramatically compared with the input of raw channel, and AI-based approach shows marginal performance gain over non-AI algorithms.

  - CSI prediction with generalization

We perform a preliminary simulation on the sub-use case of CSI prediction to evaluate the performance of AI models. In addition, the nearest historical CSI and a non-AI approach (i.e., Wiener filtering) are adopted for performance comparison. These results are shown in Table 7. CSI prediction with generalization (3rd sheet: UE speeds). According to the evaluation results, we have following observations and proposals:

The model trained with a high UE speed can achieve a good generalization performance to a low UE speed. However, on the contrary, the model trained with a low UE speed can achieve a poor generalization performance to a high UE speed.

Case 3(AI model trained with mixed UE speeds) shows good generalization performance for various UE speeds.

- Conclusion

In this contribution, the evaluations on AI/ML for CSI feedback enhancement are discussed, and simulation results are provided. We have the following observations and proposals.

Proposal 1: For CSI dataset construction, dataset should be generated in diverse drops rather than in diverse TTIs to ensure dataset diversity, e.g., training samples are generated from various drops and only one TTI is used in a single drop.

Proposal 2: For rank 2/3/4, it is necessary to add an additional CSI feedback overhead D: >=γ* 230 bits, γ= 1.9, and limit the range of CSI feedback overhead C as:β* 230 bits-γ* 230 bits.

Proposal 3: To evaluate various high-resolution quantization methods of the ground-truth CSI for model training, companies should use float32 dataset as test data for easy comparison.

Proposal 4: New parameter combinations for enhanced R16 Type II method should be supported/evaluated to achieve high resolution CSI with acceptable overhead for ground-truth CSI collection.

Proposal 5: Option 2 should be adopted for evaluating monitoring metrics for intermediate KPI based monitoring.

Proposal 6: For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side, eType II CSI with new parameter combination can be used as ground-truth CSI at NW to achieve a good monitoring accuracy.

Proposal 7: It’s up to companies to report the threshold to determine the monitoring accuracy.

Proposal 8: For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side, eType II CSI with new parameter combination can be used as ground-truth CSI at NW to achieve a good FAR and MAR.

Proposal 9: FAR and MAR can be used in addition to monitoring accuracy to analyze monitoring performance. The definition of FAR and MAR are as following:

- FAR:

- samples for (KPIActual<KPItℎ 2,KPIGenie>KPItℎ 3)samples for (KPIGenie>KPItℎ 3)×100%

- MAR:

- samples for (KPIActual>KPItℎ 2,KPIGenie<KPItℎ 3)samples for (KPIGenie<KPItℎ 3)×100%

Proposal 10: The case of rank>1 should be prioritized in future study.

Proposal 11: Various lengths of observation window and prediction window should be evaluated to have a fair comparison between AI-based CSI prediction and non-AI based CSI prediction.

Observation 1: The R16 Type II method with larger L values and larger pv values than legacy eType-II PCs has the possibility to achieve high resolution quantization with low overhead.

Observation 2: Option 2a-1 is not practical in reality, but it can provide a performance upper-bound to compare different options.

Observation 3: With regard to average sector throughput gain, Option 2a-1 and Option 2a-2 have similar performance. The average sector throughput gains of Option 2a-1 and Option 2a-2 are obviously higher than Option 1a and eType-II scheme.

Observation 4: Option 1a has worse performance than eType-II scheme in low CSI feedback overhead, and better performance than eType-II scheme in high CSI feedback overhead, which is due to that the mismatch on PMI and CQI is hard to be adjusted by gNB side.

Observation 5: KPIDiff in Option 1 cannot reflect whether the model maintain its high performance, while KPIDiff in Option 2 can actually reflect the model performance.

Observation 6: For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side, the monitoring accuracy is lower than 90% based on the reference CSI of eType-II PC8 when the SGCS threshold is 0.8, while for SGCS threshold smaller than 0.8, the monitoring accuracy is higher than 95%.

Observation 7: For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side, the monitoring accuracy is higher than 97% based on the reference CSI of eType-II PC10 when the SGCS threshold is 0.8, and for SGCS threshold smaller than 0.8, the monitoring accuracy is higher than 99%.

Observation 8: For model performance monitoring at UE side, output CSI of proxy model at UE can be used as CSI reconstruction part output to achieve a good monitoring accuracy.

Observation 9: For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side, the FAR (False Alarm Rate) is higher than 11% based on the ground-truth CSI of eType-II PC8 when the SGCS threshold is 0.8, while for other cases, the FAR is lower than 4%.

Observation 10: For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side, eType II CSI with new parameter combination achieve a good performance for monitoring accuracy and FAR/MAR.

Observation 11: For Rank 1, AI-based CSI reconstruction completely outperforms the Rel-16 eType II in term of SGCS. With the low feedback overhead, AI-based CSI recovery can obtain up to 6.43% SGCS gains; with the medium feedback overhead, AI-based CSI recovery can obtain up to 6.44% SGCS gains; with the high feedback overhead, AI-based CSI recovery can obtain up to 7% SGCS gains.

Observation 12: For Rank 1, AI-based CSI reconstruction achieves minor average UPT gain, where less than 1% average UPT gain is obtained over Rel-16 eTypeII with the same feedback overhead.

Observation 13: For Rank 1, for 5% tail UPT, AI-based CSI reconstruction can achieve up to 20.43% performance gain with the CSI feedback overhead A, up to 10.13% performance gain with the CSI feedback overhead B, and up to 0.85% performance gain with the CSI feedback overhead C.

Observation 14: For Rank 1, the margin between AI-based CSI recovery and ideal CSI feedback is quite small for average UPT, as AI-based CSI reconstruction and Rel-16 eTypeII can already work well in single layer scheduling.

Observation 15: For Rank 1, the margin between AI-based CSI recovery and ideal CSI feedback is 5.6% for 5% tail UPT.

Observation 16: For Rank 2, AI-based CSI reconstruction completely outperforms the Rel-16 eType II in term of SGCS. With the low feedback overhead, AI-based CSI recovery can obtain up to 4.67% SGCS gains in Layer 1 and 5.92% SGCS gains in Layer 2; with the medium feedback overhead, AI-based CSI recovery can obtain up to 4.04% SGCS gains in Layer 1 and 5.90% SGCS gains in Layer 2; with the high feedback overhead, AI-based CSI recovery can obtain up to 6.44% SGCS gains in Layer 1 and 9.95% SGCS gains in Layer 2.

Observation 17: For up to Rank 2, under the case of 50% RU, AI based CSI reconstruction has about up to 4.09% average UPT gain over the Rel-16 eType II with the CSI feedback overhead A, up to 4.08% average UPT gain over the Rel-16 eType II with the CSI feedback overhead B, up to 4.45% average UPT gain over the Rel-16 eType II with the CSI feedback overhead C, and up to 4.55% average UPT gain over the Rel-16 eType II with the CSI feedback overhead D.

Observation 18: For up to Rank 2, under the case of 50% RU, for 5% tail UPT, AI based CSI reconstruction has about up to 9.72% performance gain over the Rel-16 eType II with the CSI feedback overhead A, up to 20.00% performance gain over the Rel-16 eType II with the CSI feedback overhead B, up to 25.62% performance gain over the Rel-16 eType II with the CSI feedback overhead C, and up to 14.37% performance gain over the Rel-16 eType II with the CSI feedback overhead D.

Observation 19: With regard to up to Rank 2, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 5.44% under the case of 50% RU.

Observation 20: With regard to up to Rank 2, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 24.04% under the case of 50% RU.

Observation 21: For up to Rank 2, under the case of 50% RU, AI-based CSI reconstruction can achieve up to 48.60% overhead reduction with the CSI feedback overhead A,up to 39.80% overhead reduction with the CSI feedback overhead B, up to 38.66% overhead reduction with the CSI feedback overhead C, and up to 40.22% overhead reduction with the CSI feedback overhead D.

Observation 22: For up to Rank 2, under the case of 70% RU, AI based CSI reconstruction has about up to 5.30% average UPT gain over the Rel-16 eType II with the CSI feedback overhead A, up to 4.92% average UPT gain over the Rel-16 eType II with the CSI feedback overhead B, up to 5.23% average UPT gain over the Rel-16 eType II with the CSI feedback overhead C, and up to 6.94% average UPT gain over the Rel-16 eType II with the CSI feedback overhead D.

Observation 23: For up to Rank 2, under the case of 70% RU, for 5% tail UPT, AI based CSI reconstruction has about up to 10.26% performance gain over the Rel-16 eType II with the CSI feedback overhead A, up to 15.02% performance gain over the Rel-16 eType II with the CSI feedback overhead B, up to 13.67% performance gain over the Rel-16 eType II with the CSI feedback overhead C, and up to 16.26% performance gain over the Rel-16 eType II with the CSI feedback overhead D.

Observation 24: With regard to up to Rank 2, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 9.50% under the case of 70% RU.

Observation 25: With regard to up to Rank 2, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 28.75% under the case of 70% RU.

Observation 26: For up to Rank 2, under the case of 70% RU, AI-based CSI reconstruction can achieve up to 44.44% overhead reduction with the CSI feedback overhead A,up to 38.12% overhead reduction with the CSI feedback overhead B, up to 36.47% overhead reduction with the CSI feedback overhead C and up to 40.41% overhead reduction with the CSI feedback overhead D.

Observation 27: The SGCS performance for AI/ML-based CSI compression presents positive correlation with the throughput performance for up to Rank 2.

Observation 28: AI/ML approaches show better average throughput performance with heavy traffic load for up to Rank 2.

Observation 29: For Rank 4, AI-based CSI reconstruction completely outperforms the Rel-16 eType II in term of SGCS. With the low feedback overhead, AI-based CSI recovery can obtain up to 3.96% SGCS gains in Layer 1,  6.77% SGCS gains in Layer 2, 32.12% SGCS gains in Layer 3, and 91.85% SGCS gains in Layer 4; with the medium feedback overhead, AI-based CSI recovery can obtain up to 3.11% SGCS gains in Layer 1, 3.72% SGCS gains in Layer 2, 15.02% SGCS gains in Layer 3, and 37.16% SGCS gains in Layer 4; with the high feedback overhead, AI-based CSI recovery can obtain up to 5.64% SGCS gains in Layer 1, 4.79% SGCS gains in Layer 2, 14.92% SGCS gains in Layer 3, and 31.94% SGCS gains in Layer 4.

Observation 30: For up to Rank 4, under the case of 50% RU, AI based CSI reconstruction has about up to 7.95% average UPT gain over the Rel-16 eType II with the CSI feedback overhead A, up to 3.11% average UPT gain over the Rel-16 eType II with the CSI feedback overhead B, up to 7.64% average UPT gain over the Rel-16 eType II with the CSI feedback overhead C, and up to 6.88% average UPT gain over the Rel-16 eType II with the CSI feedback overhead D.

Observation 31: For up to Rank 4, under the case of 50% RU, for 5% tail UPT, AI based CSI reconstruction has about up to 9.25% performance gain over the Rel-16 eType II with the CSI feedback overhead A, up to 6.17% performance gain over the Rel-16 eType II with the CSI feedback overhead B, up to 9.47% performance gain over the Rel-16 eType II with the CSI feedback overhead C, and up to 8.11% performance gain over the Rel-16 eType II with the CSI feedback overhead D.

Observation 32: With regard to up to Rank 4, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 25.06% under the case of 50% RU.

Observation 33: With regard to up to Rank 4, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 37.72% under the case of 50% RU.

Observation 34: For up to Rank 4, under the case of 50% RU, AI-based CSI reconstruction can achieve up to 56.01% overhead reduction with the CSI feedback overhead A, up to 36.10% overhead reduction with the CSI feedback overhead B, up to 38.14% overhead reduction with the CSI feedback overhead C, and up to 25.40% overhead reduction with the CSI feedback overhead D.

Observation 35: For up to Rank 4, under the case of 70% RU, AI based CSI reconstruction has about up to 12.35% average UPT gain over the Rel-16 eType II with the CSI feedback overhead A, up to 5.00% average UPT gain over the Rel-16 eType II with the CSI feedback overhead B, up to 8.40% average UPT gain over the Rel-16 eType II with the CSI feedback overhead C, and up to 6.88% average UPT gain over the Rel-16 eType II with the CSI feedback overhead D.

Observation 36: For up to Rank 4, under the case of 70% RU, for 5% tail UPT, AI based CSI reconstruction has about up to 23.27% performance gain over the Rel-16 eType II with the CSI feedback overhead A, up to 10.20% performance gain over the Rel-16 eType II with the CSI feedback overhead B, up to 6.83% performance gain over the Rel-16 eType II with the CSI feedback overhead C, and up to 14.20% performance gain over the Rel-16 eType II with the CSI feedback overhead D.

Observation 37: With regard to up to Rank 4, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 25.09% under the case of 70% RU.

Observation 38: With regard to up to Rank 4, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 35.58% under the case of 70% RU.

Observation 39: For up to Rank 4, under the case of 70% RU, AI-based CSI reconstruction can achieve up to 60.29% overhead reduction with the CSI feedback overhead A,up to 36.41% overhead reduction with the CSI feedback overhead B, up to 37.93% overhead reduction with the CSI feedback overhead C, and up to 24.81% overhead reduction with the CSI feedback overhead D.

Observation 40: For up to Rank 4, under full buffer traffic, AI based CSI reconstruction has about up to 8.51% average UPT gain over the Rel-16 eType II with the CSI feedback overhead A, up to 7.44% average UPT gain over the Rel-16 eType II with the CSI feedback overhead B, up to 9.95% average UPT gain over the Rel-16 eType II with the CSI feedback overhead C, and up to 9.98% average UPT gain over the Rel-16 eType II with the CSI feedback overhead D.

Observation 41: For up to Rank 4, under full buffer traffic, for 5% tail UPT, AI based CSI reconstruction has about up to 5.36% performance gain over the Rel-16 eType II with the CSI feedback overhead A, up to 4.72% performance gain over the Rel-16 eType II with the CSI feedback overhead B, up to 3.59% performance gain over the Rel-16 eType II with the CSI feedback overhead C, and up to 6.15% performance gain over the Rel-16 eType II with the CSI feedback overhead D.

Observation 42: With regard to up to Rank 4, for average UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 19.75% under full buffer traffic.

Observation 43: With regard to up to Rank 4, for 5% tail UPT, the margin between AI-based CSI recovery and ideal CSI feedback is 45.21% under full buffer traffic.

Observation 44: For up to Rank 4, under full buffer traffic, AI-based CSI reconstruction can achieve up to 70.53% overhead reduction with the CSI feedback overhead A,up to 47.74% overhead reduction with the CSI feedback overhead B, up to 42.59% overhead reduction with the CSI feedback overhead C and up to 39.40% overhead reduction with the CSI feedback overhead D.

Observation 45: The SGCS performance for AI/ML-based CSI compression presents positive correlation with the throughput performance for up to Rank 4.

Observation 46: AI/ML approaches show better average throughput performance with heavy traffic load for up to Rank 4.

Observation 47: Case 3(AI model trained in mixed deployment scenarios) shows good generalization performance for various scenarios included in the mixed deployment scenarios.

Observation 48: The AI model trained in UMa/UMi scenario shows good generalization performance to InH dataset, while the AI model trained in InH scenario shows generalization degradation to UMa/UMi scenario.

Observation 49: The AI model trained in the scenario of InH and fine-tuned with the dataset from a UMa/UMi scenario shows 1%-5% SGCS performance gain over the AI model trained only in InH scenario and tested in UMa/UMi scenario.

Observation 50: With the increased samples of fine-tuning dataset from testing scenario, the AI model shows better performance in testing scenario.

Observation 51: AI/ML approaches can achieve good generalization performance for the case that the training dataset and testing dataset are generated with different bandwidth configurations but with the same model input size.

Observation 52: Case 3(AI model trained in mixed configurations of antenna port numbers) shows good generalization performance for various antenna port numbers.

Observation 53: AI model trained in the configuration of 32 antenna ports would suffer from performance degradation in16 antenna ports when input data is zero padding from 16 antenna ports to 32 antenna ports.

Observation 54: The AI/ML model shows a good generalization capability across layers in the following cases:

 Trained with data from all layers and tested for different layers

 Trained with data from the first layer and tested for different layers

 Trained with data from the first two layers and tested for different layers

Observation 55: For both Type 3 training with NW-first and Type 3 training with UE-first, two cases have similar performance. Type 3 training with NW-first has nearly the same performance as Type 1 training, while Type 3 training with UE-first has a slightly lower performance than Type 1 training.

Observation 56: 2-bit scalar quantization with training awareness Case 2-1 has similar SGCS gain with vector quantization with training awareness Case 2-2, where the scalar quantization has a slightly higher SGCS gain over vector quantization under X/Y overhead, and has a slightly lower SGCS gain over vector quantization under Z overhead.

Observation 57: Both AI-based CSI prediction and Wiener filtering-based CSI prediction can completely outperform the nearest historical CSI. Moreover, Wiener filtering-based non-AI CSI prediction shows even better performance than the AI-based approach when enough historical CSIs are applied.

Observation 58: The prediction accuracy of AI-based approach and Wiener filtering-based approach drops seriously when the predicted time becomes longer due to the channel aging. However, AI-based CSI prediction can maintain the performance for a longer time than Wiener filtering-based CSI prediction.

Observation 59: After eTypeII quantization on the predicated CSI via post-processing, AI/ML based CSI prediction and Wiener filtering-based non-AI CSI prediction have similar SGCS performances, where both the methods completely outperform the CSI predication based on the nearest historical CSI.

Observation 60: The prediction accuracy of both AI-based approach and Wiener filtering-based approach improve with the increased number of historical CSIs as input and presents certain positive correlation with the number of input historical CSIs.

Observation 61: AI-based CSI prediction shows less or even no performance gain over Wiener filtering-based algorithm with the increased number of historical CSIs in model input.

Observation 62: The prediction accuracy of AI-based approach outperforms non-AI approaches when sub-band eigenvectors are adopted as model input.

Observation 63: The performance of eigenvector prediction for AI-based approach drops dramatically compared with the input of raw channel, and AI-based approach shows marginal performance gain over non-AI algorithms.

Observation 64: The model trained with a high UE speed can achieve a good generalization performance to a low UE speed. However, on the contrary, the model trained with a low UE speed can achieve a poor generalization performance to a high UE speed.

Observation 65: Case 3(AI model trained with mixed UE speeds) shows good generalization performance for various UE speeds.

- References

- Chairman's notes, RAN1 #112bis-e.

- R1-2304535, Discussion on other aspects for AI CSI feedback enhancement, ZTE.

- Chairman's notes, RAN1 #112.

- R1-2207840 Summary#3_9.2.2.1_CSI_Eva_v002, RAN1 #110, Moderator.

- Knowledge distillation, https://en.wikipedia.org/wiki/Knowledge_distillation.

- R1-2300171, Evaluation on AI CSI feedback enhancement, ZTE Corporation.

- R1-2210753 Summary#7 of [110bis-e-R18-AIML-02]_CSI_Eva, Moderator.

- R1-2210885, Evaluation on AI/ML for CSI feedback enhancement, Huawei, HiSilicon.

- R1-2210954, Evaluations on AI-CSI, Ericsson.

- R1-2210998, Evaluation on AI/ML for CSI feedback enhancement, vivo.

- R1-2211073, Evaluation on AI/ML for CSI feedback enhancement, Fujitsu.

- Appendix A

Table 11-1 SLS assumptions for AI/ML based CSI feedback

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, 2GHz |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 41 dBm for 10MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz |
| Simulation bandwidth |  | 10MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation. |
| MIMO layers |  | Maximum MU 4 layers for max rank=1 Maximum MU 8 layers for max rank=2 |
| CSI feedback |  | Feedback assumption at least for baseline scheme CSI feedback periodicity (full CSI feedback) : 5 ms, Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | 2 OFDM symbols for PDCCH，type 2 for DMRS(24 REs/PRB/slot) CSI-RS overhead(32 REs/PRB/5 slot) |
| Traffic model |  | FTP 3 |
| Traffic load (Resource utilization) |  | RU 50% and 70% |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI feedback overhead |
| Baseline for performance evaluation |  | Rel-16 TypeII Codebook |

Table 11-2  Training parameters of AI/ML model for CSI compression

| Parameter | Value |
| --- | --- |
| Backbone | Transformer |
| Parameter type | Real value |
| Input CSI type | Eigenvectors of the ideal channel matrix estimated by UE |
| Output CSI type | Recovered eigenvectors by AI/ML model in gNB |
| Data-processing | Normalization |
| Quantization | Vector quantization |
| CSI generation part: Number of parameters/M | 4-5 |
| CSI generation part: FLOPs/M | 25-26 |
| CSI reconstruction part: Number of parameters/M | 5-6 |
| CSI reconstruction part: FLOPs/M | 27-29 |
| Training dataset | 600K |
| Validation dataset | 10K |
| Testing dataset | 20K |
| Batch size | 400 |
| Optimizer | Adam |
| Loss function | MSE |

Table 11-3  Training parameters of AI/ML model for CSI prediction based on raw channels

| Parameter | Value |
| --- | --- |
| Backbone | ResNet |
| Parameter type | Real value |
| Input CSI type | Historical channel matrices measured by UE |
| Output CSI type | Predicted channel matrix by AI/ML model in UE |
| Model input size | () |
| Model output size | () |
| Training dataset | 80K |
| Validation dataset | 2K |
| Testing dataset | 2K |
| Batch size | 200 |
| Optimizer | Adam |
| Loss function | MSE |

Table 11-4  Training parameters of AI/ML model for CSI prediction based on eigenvectors

| Parameter | Value |
| --- | --- |
| Backbone | ResNet |
| Parameter type | Real value |
| Input CSI type | Sub-band eigenvectors |
| Output CSI type | Predicted eigenvectors by AI/ML model in UE |
| Model input size | () |
| Model output size | () |
| Training dataset | 80K |
| Validation dataset | 2K |
| Testing dataset | 2K |
| Batch size | 200 |
| Optimizer | Adam |
| Loss function | MSE |