3GPP TSG RAN WG1 Meeting #113	R1-2304535

Incheon, Korea, May 22nd – May 26th, 2023

Source:	ZTE

Title:	Discussion on other aspects for AI CSI feedback enhancement

Agenda Item:	9.2.2.2

Document for:	Discussion and Decision

- Introduction

According to discussions in 3GPP RAN1#112bis-e meeting [1], much progress has been made on other aspects for AI CSI feedback enhancement and some agreements have been reached.

In this contribution, we provide our views on the potential specification impacts for CSI feedback enhancement. In our companion contribution [2], some related evaluation results on AI/ML for CSI feedback are discussed and assessed.

- Spatial-frequency domain CSI compression using two-sided model

  - Training collaboration

This sub use case involves two-sided model inference operations performed at both UE and gNB. Specifically, UE deploys (or be configured with) CSI generation part and gNB deploys corresponding CSI reconstruction part, where the former one is for CSI compression and the latter one is to recover more accurate CSI for better MU operation in massive MIMO scenario. However, how to train and collaborate the two-sided AI model is a key problem, which will have impacts on the existing specifications. The following progress about training collaborations has been reached in RAN1#110 meeting [5], RAN1#111 meeting [4] and the following features to compare the training collaboration types are concluded in RAN1#112 meeting [3].

Agreement: In CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied: - Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided. - Type 2: Joint training of the two-sided model at network side and UE side, respectively. - Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively. - Note: Joint training means the generation model and reconstruction model should be trained in the same loop for forward propagation and backward propagation. Joint training could be done both at single node or across multiple nodes (e.g., through gradient exchange between nodes). - Note: Separate training includes sequential training starting with UE side training, or sequential training starting with NW side training [, or parallel training] at UE and NW - Other collaboration types are not excluded. Conclusion - In CSI compression using two-sided model use case, training collaboration type 2 over the air interface for model training (not including model update) is deprioritized in R18 SI. Conclusion: In CSI compression using two-sided model use case, further discuss the pros/cons of different offline training collaboration types including at least the following aspects: - Whether model can be kept proprietary - Requirements on privacy-sensitive dataset sharing - Flexibility to support cell/site/scenario/configuration specific model - gNB/device specific optimization – i.e., whether hardware-specific optimization of the model is possible, e.g. compilation for the specific hardware - Model update flexibility after deployment - feasibility of allowing UE side and NW side to develop/update models separately - Model performance based on evaluation in 9.2.2.1 - Whether gNB can maintain/store a single/unified model - Whether UE device can maintain/store a single/unified model - Extendability: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use - Whether training data distribution can be matched to the device that will use the model for inference - Whether device capability can be considered for model development - Other aspects are not precluded - Note: training data collection and dataset/model delivery will be discussed separately

After several rounds of e-mail discussion on the pros/cons of different training types in RAN1#112bis-e meeting [1], some consensuses have been reached among companies, and the initial table is shown as follows.

- Proposed observation 2-1-1: In CSI compression using two-sided model use case, the following table capture the pros/cons of different offline training collaboration types: Training types Characteristics; Type 1; Type 2; Type 3. NW-sided; UE-sided; NW first; UE first. Whether model can be kept proprietary; No; No; Yes; Yes (Note 3); Yes (Note 3). Whether require privacy-sensitive dataset sharing; No (Note 1); No; No (Note 1); No (Note 1); No (Note 1). Flexibility to support cell/site/scenario/configuration specific model; Yes; Yes. With assisted information signaling. Less flexible than Type 1-NW side.; Difficult; Semi-flexible; Semi-flexible. with assisted information signaling. Whether gNB/device specific optimization is allowed; Restricted; Restricted; Yes; Yes; Yes. Model update flexibility after deployment; Flexible (note 4); Conditional, flexible with assisted information (note 4); Not flexible (note 4); Semi-flexible; Conditional semi-flexible, with assisted information (note 4). Feasibility of allowing UE side and NW side to develop/update models separately; Limited (Note 2); Limited (Note 2); Infeasible; Feasible; Feasible. Whether gNB can maintain/store a single/unified model for a CSI report configuration; Yes; No; Pending evaluation in 9.2.2.1; Pending evaluation in 9.2.2.1; Pending evaluation in 9.2.2.1. Whether UE device can maintain/store a single/unified model for a CSI report configuration; No; Yes; Pending evaluation in 9.2.2.1; Pending evaluation in 9.2.2.1; Pending evaluation in 9.2.2.1. Extendibility: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use; Limited (Note 2); Limited (Note 2); Limited; Support; Support. Whether training data distribution can match the inference device; Conditional, with assisted information from UE for device specific model.; Yes; Restricted; Conditional, with assisted information from UE; Yes. Software/hardware compatibility (Whether device capability can be considered for model development); Yes for device specific model. No for device-agnostic model.; Yes; Compatible; Compatible; Compatible. Model performance based on evaluation in 9.2.2.1; Pending evaluation in 9.2.2.1; Pending evaluation in 9.2.2.1; Pending evaluation in 9.2.2.1; Pending evaluation in 9.2.2.1; Pending evaluation in 9.2.2.1. Note 1: Assume high accuracy PMI is not privacy sensitive data. FFS: other information such as channel matrix and assisted information. Note 2: For example, after deploying model 1 on the UE side, a new UE model can be obtained by using model 1 as the teacher model and using knowledge distillation method. Model 1 can also refer to a nominal model while the real deployed model can be developed based on the nominal model. Note 3: Assume information on model structure is not required to be disclosed in training collaboration type 3. Note 4: Flexibility after deployment is evaluated by the amount of offline cross-vendor co-engineering effort. Flexible indicates minimum additional co-engineering between vendors, semi-flexible indicates additional co-engineering effort between vendors. Note 5: Yellow highlighted rows are for further discussion.

However, some characteristics still remains controversial (i.e., the underlined characteristics), and the correspondig issues are analyzed as below.

Model update flexibility after deployment

- Type 1 training:

  - NW-sided training: Network can be flexible to retrain/update the models to adapt to channel variations.

  - UE-sided training: Model updating at UE side is less flexible than NW-sided training, since UE is hard to train the model at the UE device due to the limitation of the UE capability and the model would be retrained/updated at UE server.

- Type 2 training:

  - Joint/Sequential gradient-exchange training: Due to the fact that offline engineering to align the gradient-exchange across multiple vendors are required, model update after deployment is clearly not flexible.

- Type 3 training:

  - NW-first training: Since the required dataset size of model updating/fine-tuning may be much less than that of model training, dataset sharing for model updating can be also performed via air interface with acceptable overhead. Compared with Type 1 joint training at NW side, there may be semi-flexible for model updating, since for Type 1, the delivered model can be directly used, or implemented after compiling; while for Type 3, UE still needs to wait for the model training/update in private server after the dateset sharing.

  - UE-first training: Model update is semi-flexible after deployment for the UE-first training, since NW still needs to wait for the model training/update after the dateset sharing. However, it is also less flexible than NW-first training because the training entity of the UE side would be the non-3GPP entity rather than the UE device due to the limitation of the UE capability. While for NW-first training for comparison, the gNB can perform the update of the Network model more flexibly.

Whether gNB can maintain/store a single/unified model for a CSI report configuration

- Type 1 training:

  - NW-sided training: Network can train a unified CSI reconstruction model to adapt to multiple CSI generation models from different UE vendors. It avoids the problem of multi-vendor model pairing that network needs to maintain many pairs of UE-network specific models.

  - UE-sided training: It is obviously that gNB cannot maintain/store a single/unified model since the models from other UE vendors are unseen.

- Type 2/3 training:

  - This issue is still pending since no sufficient observations have been concluded about the feasibility in agenda item 9.2.2.1. Therefore, it’s better to wait for the conclusion in agenda item 9.2.2.1.

Whether UE can maintain/store a single/unified model for a CSI report configuration

- Type 1 training:

  - NW-sided training: It is obviously that UE cannot maintain/store a single/unified model since the models from other gNB vendors are unseen.

  - UE-sided training: UE can train a unified CSI generation model to adapt to multiple CSI reconstruction models from different NW vendors. It avoids the problem of multi-vendor model pairing that UE needs to maintain many pairs of UE-network specific models.

- Type 2/3 training:

  - This issue is also pending since no sufficient evaluation results prove the feasibility and it needs further evaluation in agenda item 9.2.2.1.

Feasibility of allowing UE side and NW side to develop/update models separately

- Type 1 training:

  - NW-sided training: Although joint training is performed at NW side, UE side can imitate the CSI generation model delivered from NW side to develop/update the UE-sided models, e.g., using knowledge distillation method. However, the dataset for model development/update collected by UE would be different from the dataset for model training at NW side in data distribution, where training dataset may be a mixed dataset over multiple UE vendors. Therefore, it may incur the mismatch between the pairing of UE-part model and NW-part model, and the feasibility and performance of this method should be evaluated in agenda item 9.2.2.1.

  - UE-sided training: Similar to NW-sided training, NW side can also imitate the CSI reconstruction model delivered from UE side to develop/update the NW-sided models using knowledge distillation method. Whether this method is feasible needs further evaluation in agenda item 9.2.2.1.

- Type 2 training:

  - Joint/Sequential gradient-exchange training: Due to the fact that offline engineering to align multiple vendors are required, it is apparently infeasible to allow UE side and NW side to develop/update models separately.

- Type 3 training:

  - NW-first/UE-first training: The individual engineering of developing/updating can be better ensured since the Network part model and the UE part model are developed and trained based on the corresponding training dataset, respectively.

Whether training data distribution can match the inference device

- Type 1 training:

  - NW-sided training: Considering the training dataset may be collected from multiple UE vendors, the training data may follow different data distributions due to the hardware variations and different proprietary data processing operations of UE vendors. The training data distribution would mismatch the actual inference data. However, only if NW trains the model with UE specific dataset via some UE assisted information, training data distribution can match the inference device.

  - UE-sided training: Since the training dataset is collected from the specific UE vendor and the data processing operation is known by UE vendor, hence training data distribution can match the actual inference data.

- Type 2 training:

  - Joint/Sequential gradient-exchange training: The training dataset should keep aligned between UE side and NW side during Type 2 training process, thus UE should provide some assisted information (e.g., data processing method) for NW-sided model training. Therefore, training data distribution can match the inference device with some restrictions.

- Type 3 training:

  - NW-first: The data distribution can be the same as the device if the ground-truth CSI is shared from the target UE device via some assisted information from UE. Therefore, the training data distribution can be matched to the model for inference with some conditions.

  - UE-first: Training data distribution can be matched to the device that will use the model for inference, since the data processing method can be aligned, which is up to UE implementation.

Software/hardware compatibility (Whether device capability can be considered for model development)

- Type 1 training:

  - NW-sided training: When the supported UE-model structure is known by network side, network can train the UE-specific model which is compatible with the UE capability.

  - UE-sided training: It certainly satisfies the software/hardware compatibility during model development, since it is up to UE vendor implementation.

- Type 2/3 training:

  - It can avoid the model incompatibility issue and model format misalignment issue due to UE-part model training is performed at UE side and operate the specific optimization method according to its own software/hardware capability.

In addition, some controversial training types still remains pending as following, which needs further discussions to have a consensus for drawing final conclusion.

- Conclusion In CSI compression using two-sided model use case, gradient-exchange based sequential training over the air interface is deprioritized in R18 SI. - Proposal 2-1-1 In CSI compression using two-sided model use case, to capture observation of different training collaboration types, separate UE side training collaboration type 3 into device-specific and device agnostic. - Proposal 2-1-3 In CSI compression using two-sided model use case, to capture observation related to gradient-exchange based sequential training, separate training collaboration type 3 into joint training and sequential training.

Regarding discussion on the pros and cons of NW-sided Type 1 training , there is no need to split it into two types: device agnostic and device specific. In the end of Rel-18 SI, it is enough to only focus on the NW-sided Type 1 training from a high-level perspective.. It is unnecessary to make the table so complicated that the discussion cannot be converged and no conclusion would be drawn in the end. Therefore, we propose to deprioritize the discussion of the pros and cons of NW-sided Type 1 training from two types of device agnostic and device specific in R18 SI. Instead, as shown in the initial table, there are already some entries to reflect this issue. For example, regarding the extendability, only when the model structure is known at network side, network can train new NW-side model compatible with UE-side model in use.

- Proposal 1: Deprioritize the discussion of the pros and cons of NW-sided Type 1 training from two types of device agnostic and device specific. If needed, some conditions can be added to reflect this issue.

In RAN1#112bis-e meeting, gradient-exchange based sequential training was discussed heatedly, yet there is no consensus on the categorization of this training type. To our understanding, it is more likely an evolved Type 2 training method and some thinking is shown as follows.

- Type 2 training needs UE and NW to jointly train the CSI generation part and CSI construction part in the same loop, while the gradient-exchange based sequential training freezes the NW-sided parameter update during the training process. However, whether NW updates parameters or not is the NW implementation manner, which is unseen by the UE side - NW can also update the parameters if needed.

- NW-first Type 3 training needs NW to deliver the dataset to the UE side (i.e., an one-way sharing), while the gradient-exchange based sequential training needs FP and BP information exchange between NW and UE, which is not an one-way sharing mode like Type 3 training.

- In addition, the gradient-exchange based sequential training requires real-time FP/BP information exchange under multiple epochs between NW and UE, which is similar to Type 2 training with FP/BP interactions. A dedicated API needs to be supported across vendors to enable the training procedure.

- Therefore, the gradient-exchange based sequential training should be categorized into Type 2 training, and then Type 2 training can be classified as ‘Joint/Sequential gradient-exchange training’ in the table.

- Proposal 2: The gradient-exchange based sequential training should be categorized into Type 2 training, and then Type 2 training can be classified as ‘Joint/Sequential gradient-exchange training’ in the table.

- During the discussion, some companies suggested to add an column ‘Specification transparent way’ other than ‘Type 1’ and ‘Type 3’, to cover Type 2 joint gradient-exchange training and sequential gradient-exchange training. However, the specification transparent way can be applicable to all training collaboration types rather than only Type 2 training. In addition, there is little difference on spec impact between joint gradient-exchange training and sequential gradient-exchange training. Whether NW updates parameters or not is the NW implementation manner, which is unseen by the UE side. Therefore, joint gradient-exchange training and sequential gradient-exchange training can be discussed together in the table. Based on the above analysis, we propose to further discuss the pros and cons of different offline training types according to the following table.

- Table 1. The pros/cons of different offline training collaboration types

| Training type Characteristics | Type 1 |  | Type 2 | Type 3 |  |
| --- | --- | --- | --- | --- | --- |
|  | NW-sided | UE-sided | Joint/Sequential gradient exchange | NW first | UE first |

- Proposal 3: Further discuss the pros and cons of different offline training types according to the following table.

| Training type Characteristics | Type 1 |  | Type 2 | Type 3 |  |
| --- | --- | --- | --- | --- | --- |
|  | NW-sided | UE-sided | Joint/Sequential gradient exchange | NW first | UE first |

Based on the above analysis, for Type 1 joint training, we propose to prioritize NW side training for further study. In addition, model transfer/delivery in training Type 1 would bring great specification impact, and this issue can be further discussed in Agenda item 9.2.1. Besides, from procedure and signaling point of view, the two cases should share as much commonality as possible. Therefore, we should further study whether study outcomes of Type 1 joint training at NW side can be also applicable to Type 1 joint training at UE side.

Proposal 3: Prioritize Type 1 joint training at NW side for further study and model transfer/delivery can be further discussed in agenda item 9.2.1.

Besides, we also propose to prioritize Type 3 NW-first training for further study. To our understanding, dataset used for the model training at the other side/entity may have some specification impact, which needs further discussion in this agenda. In addition, from the view of procedure and signaling, the two cases should share as many commonalities as possible. Therefore, we should further study whether outcomes of NW-first training can be also applicable to UE-first training.

Proposal 4: For training Type 3, NW-first training should be prioritized over UE-first training.

Proposal 5: For training Type 3, further study potential specification impact on the dataset used for the model training at the other side/entity.

In summary, the pros and cons of different offline training types are listed in the following Table 1.

- Table 1. The pros/cons of different offline training collaboration types

| Training types Characteristics | Type 1 |  | Type 2 | Type 3 |  |
| --- | --- | --- | --- | --- | --- |
|  | NW-sided | UE-sided | Joint/Sequential gradient exchange | NW first | UE first |
| Whether model can be kept proprietary | No | No | Yes | Yes | Yes |
| Whether require privacy-sensitive dataset sharing | No | No | No | No | No |
| Flexibility to support cell/site/scenario/configuration specific model | Yes | Yes. With assisted information signaling. Less flexible than Type 1-NW side. | Difficult | Semi-flexible | Semi-flexible. With assisted information signaling |
| Whether gNB/device specific optimization is allowed | Restricted | Restricted | Yes | Yes | Yes |
| Model update flexibility after deployment | Flexible | Flexible. Less flexible than Type 1-NW side. | Not flexible | Semi-flexible | Semi-flexible. Less flexible than Type 3-NW side. |
| Feasibility of allowing UE side and NW side to develop/update models separately | Limited (Pending evaluation in 9.2.2.1) | Limited (Pending evaluation in 9.2.2.1) | Infeasible | Feasible | Feasible |
| Whether gNB can maintain/store a single/unified model for a CSI report configuration | Yes | No | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Whether UE device can maintain/store a single/unified model for a CSI report configuration | No | Yes | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Extendibility: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use | Limited | Limited | Limited | Support | Support |
| Whether training data distribution can match the inference device | Conditional, with assisted information from UE | Yes | Restricted | Conditional, with assisted information from UE | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | Conditional, with assisted information from UE | Yes | Compatible | Compatible | Compatible |
| Model performance based on evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |

  - Data collection

As is known to all, the performance of AI/ML model is correlated to the training data and inference data, so data collection is a fundamental process in AI model’s LCM. For data collection, an agreement was reached in Agenda item 9.2.2.2 in RAN1#112 meeting [3].

Agreement: In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact of UE side data collection enhancement including at least - Enhancement of CSI-RS configuration to enable higher accuracy measurement. - Assistance information for UE data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc. - The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side. - Signaling for triggering the data collection In CSI compression using two-sided model use case, further discuss the necessity, feasibility, and potential specification impact for NW side data collection including at least: - Enhancement of SRS and/or CSI-RS measurement and/or CSI reporting to enable higher accuracy measurement. - Contents of the ground-truth CSI including: - Data sample type, e.g., precoding matrix, channel matrix etc. - Data sample format: scaler quantization and/or codebook-based quantization (e.g., e-type II like). - Assistance information (e.g., time stamps, and/or cell ID, Assistance information for Network data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator) - Latency requirement for data collection - Signaling for triggering the data collection

For network side data collection, UE is required to conduct measurements based on configured reference signals. Then, the measurements may need to be reported to network side. When model training or monitoring is performed at network side, UE needs to measure the configured CSI-RS and then report the ground-truth CSI to the network. Besides, the overhead of the ground-truth label transmitted over the air-interface is a huge concern if the ground-truth CSI is an ideal CSI (e.g., raw channels, eigenvectors). Therefore, one solution is to enhance legacy CSI to increase its reliability, e.g., higher resolution CSI based on legacy Rel-16 eType II codebook design. In addition, as evaluated in our companion contribution [2], the overhead of enhanced Type II CB (i.e., PC10) for one training sample increases by 50% compared with the maximal payload of Rel-16 TypeII CB (i.e., PC8) but keeps similar model performance as ideal CSI, which can be acceptable to be carried on UCI. In this regard, if the ground-truth CSI is reported per sample, the ground-truth CSI can be reported through PHY signaling, e.g., UCI on PUSCH. On the other hand, if the ground-truth CSI is reported per batch, higher layer signaling may be more appropriate, e.g., RRC signaling.

Observation 1: When model training or monitoring is performed at network side, the overhead of the ground-truth label transmitted over the air-interface from UE to network is a huge concern if the ground-truth CSI is an ideal CSI (e.g., raw channels, eigenvectors).

Observation 2: The overhead of enhanced Type II CB (i.e., PC10) for one training sample increases by 50% compared with the maximal payload of Rel-16 TypeII CB (i.e., PC8) but keeps similar model performance as ideal CSI, which can be acceptable to be carried on UCI.

Proposal 6: For network side data collection, support to further study

- Enhanced Rel-16 TypeII codebook to get high-resolution CSI;

- PHY signaling or RRC signaling to report the high-resolution CSI.

For data collection at NW side, the quality of data reported from different UEs may vary greatly. For example, the data collected by UEs at the edge of a cell may suffer from low signal strength and interference from neighbor cell, resulting in a low quality of collected data. In this case, if the low-quality data is used for model training, model performance degradation maybe incurred. Therefore, there are two potential solutions to tackle this problem. On one hand, UE can report associated information with collected data, e.g., SINR, CQI, positioning information, and NW judges the data quality and determines whether/how to apply the collected data. On the other hand, NW can configure a threshold of data quality to UE. Then UE judges whether the quality of collected data meets the requirement and only reports the qualified data. Therefore, we propose to further study the potential solutions and specification impacts regarding the data quality during data collection at least for:

- UE reports associated information to NW, e.g., SINR, CQI, positioning information

- NW configures a threshold of data quality to UE and UE only reports the qualified data to NW

Proposal 7: To enable high-quality data collection, at least support

- UE reports associated information to NW, e.g., SINR, CQI, positioning information

- NW configures a threshold of data quality to UE and UE only reports the qualified data to NW

In addition, offline field data collected by one side can also be shared with the other side for model training, model monitoring, and model update. Type 3 training method for two-sided model needs dataset delivery between UE side and network side. For example, gNB-first training requires network to provide a dataset, including labels and intermediate results, to UE side. Then, the UE side will train a CSI generation part based on the delivered dataset. After that, the UE may inform description of the available CSI generation part to network via model/functionality identification process. During model/functionality identification, the UE should disclose which dataset has been used for training the CSI generation part to facilitate network to choose corresponding CSI reconstruction part.

Observation 3: For Type 3 training collaboration of a two-sided model, common understanding on the dataset used for model training is necessary, which can facilitate the pairing of CSI generation part and CSI reconstruction part.

In addition, for a specific model/functionality, aligning the same understanding on the dataset used for model training can be useful for testing/monitoring the model/functionality performance (i.e., the dataset implicitly defines the test vector for the model/functionality). For network, it can provide a dataset (with ground-truth label) to verify the performance of a model/functionality. Moreover, companies have strong concerns to disclose proprietary information. Therefore, during the data collection, the dataset ID can avoid sharing the proprietary information explicitly across vendors (e.g., codebook implementation, antenna patterns and Rx beam information). In this way, the applicable conditions of the model/functionality can be implicitly indicated by the dataset used for model training. As we discussed in our companion contribution [8], a new terminology for dataset identification may need to be defined to have a common understanding between the NW and the UE on a dataset.

Observation 4: Dataset alignment between UE and network can be used for testing/monitoring the model/functionality performance.

Observation 5: Dataset ID can avoid sharing the proprietary information explicitly across vendors during data collection.

Proposal 8: Support to use dataset ID to identify the delivered dataset of Type 3 training collaboration.

  - Model inference operation

- Agreement - In CSI compression using two-sided model use case, further study the feasibility and methods to support the legacy CSI reporting principles including at least: - The priority rule regarding CSI collision handling and CSI omission - Codebook subset restriction - CSI processing Unit - Agreement - The study of AI/ML based CSI compression should be based on the legacy CSI feedback signaling framework. Further study potential specification enhancement on - CSI-RS configurations (No discussion on CSI-RS pattern design enhancements) - CSI reporting configurations - CSI report UCI mapping/priority/omission - CSI processing procedures. - Other aspects are not precluded.

After model training, model delivery, and model deployment, model inference operation is an important part in LCM. In RAN1#112bis-e meeting [1], some model inference procedures have some consensus while the potential specification impacts have not been discussed. For example, when uplink transmission resources are not enough to feedback all CSI information, the mapping priority and the omission rules should be defined like legacy Type II codebook. For example, UE can use dynamic quantization resolution for AI/ML model output to reduce payload and to fit in the allocated uplink resource. Then, the quantization type/level/pattern needs to be reported to NW. Besides, AI/ML CSI can be divided into multiple groups based on, e.g. layer, subband, port, different priorities. Then, UE can omit the CSI groups with low priority. In addition, one CSI report can be separated into multiple sub-reports, which are reported in different time slots due to the limited transmission resources in a single physical channel. Therefore, the latter sub-CSI reports need to establish the association with the previous part of sub-CSI report to keep integrity.

- Dynamic quantization resolution to reduce payload

- Divide the CSI into multiple groups with different priority and omit the CSI groups with low priority, e.g., according to layer, subband and port

- CSI reporting is separated into multiple sub-reports to report in different time slots, e.g., to establish the association among the multiple sub-reports

Proposal 9: In CSI compression using two-sided model use case, further study the methods and potential specification impact on mapping priority and omission rule for AI/ML CSI report,

- Dynamic quantization resolution to reduce payload

- Divide the CSI into multiple groups with different priority and omit the CSI groups with low priority, e.g., according to layer, subband and port

- CSI reporting is separated into multiple reports, e.g., to establish the association among the multiple reports

- 2.3.1 CQI determination

Agreement: In CSI compression using two-sided model use case, further study the following options for CQI determination in CSI report, if CQI in CSI report is configured. - Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 1a: CQI is calculated based on target CSI with realistic channel measurement - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment - Option 1c: CQI is calculated based on legacy codebook - Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW. - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder. - Other options are not precluded - Note1: feasibility of different options should be evaluated - Note2: Gap analyses between the UE side CQI calculation results and the NW side results, as well as the impact on the scheduling performance should be evaluated - Note3: Complexity of CQI calculation needs to be evaluated, including the computing complexity and potential RS/signaling overhead

In RAN1#112 meeting [3], CQI determination options were discussed and finally agreed. According to the above cases for CQI calculation, the pros and cons of each option are analyzed as below:

- For Option 1a, since UE is not aware of the output CSI recovered by the CSI reconstruction model at Network, a straightforward way is that UE adopts the target CSI with realistic channel measurement for CQI calculation. One issue needs to be considered that AI/ML model may not be able to reconstruct a lossless CSI. Therefore, if CQI is calculated based on the target CSI with realistic channel measurement, UE may over-estimate the channel condition and reconstructed PMI and CQI are not matched. In this way, network may always need to make some adjustment on UE reported CQI according to outer loop control. Therefore, our simulation results [2] show that the system performance loss is obvious if no advanced CQI adjustment algorithm is used.

- Considering the situation that Option 1a would over-estimate the CQI, Option 1b is proposed to introduce some adjustment to calculate a more accurate CQI. In the contribution [6], UE can calculate CQI adjustment value based on the input CSI and the previous output of CSI reconstruction model provided from NW. In this case, the method needs to send back the output of CSI reconstruction part from NW side to UE, which will lead to additional latency. However, the channel condition may already change a lot (e.g., interference) so that PMI and CQI mismatch is unavoidable. In addition, the recovered CSI should be quantized (e.g., by eType II codebook), which will lead to additional quantization loss. Moreover, sending the recovered CSI needs enhanced specification to support it. Therefore, it is not appropriate for this option to determine CQI calculation. To our understanding, a simple way is to allow gNB to provide CQI adjustment strategy to UE. For example, NW can construct a CQI adjustment table according to some channel characteristics based on some priori information at gNB side. Then, UE can calculate the similarity-related metrics between measured channel and the channel characteristics to do corresponding CQI adjustment. Therefore, Option 1b may be a feasible way for CQI determination, however, how to design/calculate the potential CQI adjustment needs to be further studied in 9.2.2.1. Therefore, we propose to further categorize the Option 1b:

  - Option 1b-1: CQI is calculated based on target CSI with realistic channel measurement and adjusted by previous CSI reconstruction output provided by NW.

  - Option 1b-2: CQI is calculated based on target CSI with realistic channel measurement and adjusted by CQI adjustment table provided by NW.

- For Option 1c, UE may not support traditional codebook and AI/ML codebook simultaneously, which will largely increase the UE complexity. Meanwhile, PMI and CQI mismatching is also unavoidable. If traditional codebook can already get accurate PMI, it is not necessary to implement AI/ML models.

- For Option 2a, there are two ways of calculating CQI based on the output of CSI reconstruction part. On one hand, UE can calculate CQI based on the output of CSI reconstruction model if actual CSI reconstruction model is available at the UE and UE can perform reconstruction model inference. In this case, UE may also be not expected to have CSI reconstruction model as it increases UE computation/storage/power consumption burden to a large extent. In addition, the CSI reconstruction model is generally a proprietary design by network side. On the other hand, UE can calculate CQI based on the output of proxy CSI reconstruction part at the UE, which is different from the actual CSI reconstruction part used at the NW. For example, if UE has its own reference model/monitoring model and makes the output of reference model/monitoring model similar to the CSI reconstruction model output. Hence, the proxy reconstructed PMI at UE can be used for CQI calculation, which is shown in Figure 1. In our companion contribution [2], the CQI calculation based on the proxy output CSI at UE can be applicable for CSI compression using two-sided model sub use case and shows that the average system UPT can be achieved almost the same as the case of CQI calculation based on the output of actual CSI reconstruction model (i.e., the performance upper-bound for all options). In addition to the feasibility of this method, specification impact should also be taken into consideration. For example, compared with traditional NW-first Type 3 training, an additional dataset needs to be transferred from NW to UE for model training, since UE side has to train a proxy model output to imitate the output of actual CSI generation part at NW side. Therefore, we propose to further study potential specification impact on the CQI determination based on the output of proxy CSI reconstruction part at the UE. Besides, we propose to further categorize the Option 2a:

  - Option 2a-1: CQI is calculated based on CSI reconstruction output, where CSI reconstruction part at the UE is the same as the actual CSI reconstruction part used at the NW.

  - Option 2a-2: CQI is calculated based on proxy CSI reconstruction output at UE side, where CSI reconstruction part at the UE is different from the actual CSI reconstruction part used at the NW.

Figure 1 CQI calculation based on the output of proxy CSI reconstruction part at UE side

- Option 2b needs two-step procedure to finish CQI determination. UE has to receive a CSI-RS and report the PMI compressed by AI/ML model in the first step. Then, UE is to receive a precoded CSI-RS based on the reconstructed PMI at network and report the CQI determined by precoded CSI-RS in the second step. In current specification, this mode is already supported (i.e., when the report quantity is cri-RI-CQI) and less specification impact is foreseen. Besides, the two-step procedure increases the time span of the CQI determination process, which may face the channel variation/aging so that the current CQI cannot match the previous CSI. Therefore, we don’t think additional specification is needed.

Based on the above analysis, we propose to evaluate the performance of different CQI determination options in agenda item 9.2.2.1. In principal, Option 2a can be considered as the performance upper-bound for all options. From our perspective, we propose to conclude the feasibility and necessity for different options according to the evaluation results in 9.2.2.1 and specification impacts.

Observation 6: For CQI calculation based on target CSI with realistic channel measurement, UE may over-estimate the channel condition and reconstructed PMI and CQI are not matched. Our simulation results show that the system performance loss is obvious if no advanced CQI adjustment algorithm is used.

Observation 7: For CQI calculation based on target CSI with realistic channel measurement and adjusted by previous CSI reconstruction output provided by NW:

- The output of CSI reconstruction part needs to be provided to UE from NW side, which will lead to additional latency and specification impacts.

- The channel condition may already change a lot (e.g., interference) so that PMI and CQI mismatch is unavoidable.

- The recovered CSI should be quantized (e.g., by eType II codebook), which will lead to additional quantization loss.

Observation 8: For CQI calculation based on target CSI with realistic channel measurement and adjusted by CQI adjustment table provided by NW, NW can construct a CQI adjustment table according to some channel characteristics based on some priori information at gNB side. Then, UE can calculate the similarity-related metrics between measured channel and the channel characteristics to do corresponding CQI adjustment.

Observation 9: For CQI calculation based on legacy codebook, UE may not support traditional codebook and AI/ML codebook simultaneously, which will largely increase the UE complexity. Meanwhile, PMI and CQI mismatching is also unavoidable. If traditional codebook can already get accurate PMI, it is not necessary to implement AI/ML models.

Observation 10: For CQI calculation based on CSI reconstruction output, where CSI reconstruction part at the UE is the same as the actual CSI reconstruction part used at the NW. UE may also be not expected to have CSI reconstruction model as it increases UE computation/storage/power consumption burden to a large extent. In addition, the CSI reconstruction model is generally a proprietary design by network side.

Observation 11: For CQI calculation based on the output of proxy CSI reconstruction model at UE, this method can be applicable for CSI compression using two-sided model sub use case and shows that the average system UPT can be achieved almost the same as the case of CQI calculation based on the output of actual CSI reconstruction model (i.e., performance upper-bound for all options).

Observation 12: For CQI calculation using two stage approach, it is already supported (i.e., when the report quantity is cri-RI-CQI) and less specification impact is foreseen. Besides, the two-step procedure increases the time span of the CQI determination process, which may face the channel variation/aging so that the current CQI cannot match the previous CSI.

Proposal 10: The performance of different CQI determination options should be evaluated in agenda item 9.2.2.1. The pros and cons of all the options should be concluded in 9.2.2.2.

Proposal 11: Further categorize the Option 1b as following:

- Option 1b-1: CQI is calculated based on target CSI with realistic channel measurement and adjusted by previous CSI reconstruction output provided by gNB

- Option 1b-2: CQI is calculated based on target CSI with realistic channel measurement and adjusted by CQI adjustment table provided by gNB.

Proposal 12: According to initial evaluations on performance and specification impacts, the following down-selections are proposed:

- Further study the specification impacts (including the feasibility and necessity) on Option 1a, Option 1b-2 and Option 2a-2.

- No further discussion on specification impacts for Option1b-1, Option 1c, Option 2a-1 and Option 2b.

Finally, according to the following descriptions in TS 38.214, LI (Layer Indicator) should also be determined according to PMI and CQI. In current specification, LI is applied to indicate the strongest layer in the precoding matrix. For AI/ML-based feedback, whether UE needs to indicate the LI is related to the UE-sided model design feasibility. For example, rank specific model would output the multi-layer AI precoders at the same time, and then Layer indicator should be reported to indicate which is the strongest layer. Therefore, we propose to further study LI determination as well as CQI determination.

5.2.1.4 Reporting configurations The UE shall calculate CSI parameters (if reported) assuming the following dependencies between CSI parameters (if reported) - LI shall be calculated conditioned on the reported CQI, PMI, RI and CRI - CQI shall be calculated conditioned on the reported PMI, RI and CRI - PMI shall be calculated conditioned on the reported RI and CRI - RI shall be calculated conditioned on the reported CRI.

Proposal 13: In CSI compression using two-sided model use case, LI determination should be studied along with CQI determination.

### 2.3.2 RI determination

In addition, RI determination options need further discussion as below:

- Proposal 3-3-3: In CSI compression using two-sided model use case, if RI is configured to be reported, legacy RI determination can be reused as a starting point. - Further enhancements are not precluded

As cited above in TS 38.214, RI determination is nothing to do with PMI. For RI determination, UE can reuse the legacy approach to calculate RI.

Proposal 14: In CSI compression using two-sided model use case, if RI is configured to be reported, legacy RI determination can be reused as a starting point.

In addition, as we stated in another contribution [2], precoding can be refined by using approaches such as SLNR and zero-forcing to reduce inter UE interference in the scenario of MU-MIMO. In such case, MCS estimation based on precoding is quite challenging at gNB side. As discussed in MIMO agenda (section 3.7), more channel information (e.g. wideband Rxx including receiver side information, full rank report including eigenvectors and eigenvalues) can increase system performance significantly. Due to sufficient channel information of UEs at gNB side, precoding for MU-MIMO is more accurate and interference between UEs is controlled effectively. To better analyze the problem, we propose to further study potential specification impact on full rank report based on the AI/ML model.

Proposal 15: Further study potential specification impact on more channel information reported for MU-MIMO scheduling, e.g., full rank report based on the AI/ML model.

### 2.3.3 CSI configuration and report

In RAN1#112 meeting [3], CSI configuration and report was discussed and reached an agreement, which is shown as below:

- Agreement In CSI compression using two-sided model use case, further study the following aspects for CSI configuration and report: - NW configuration to determine CSI payload size, e.g., possible CSI payload size, possible rank restriction and/or other related configuration. - How UE determines/reports the actual CSI payload size and/or other CSI related information within constraints configured by the network

In legacy CSI feedback framework, the maximum rank number, the codebook type and the codebook parameter combinations (including the number of frequency/spatial domain basis and the maximum number of non-zero coefficients), can be configured so that the gNB has the flexibility on the CSI feedback payload size. In addition, UE can autonomously determine the RI and the number of non-zero coefficients which are fed back to the gNB so that UE also has the flexibility of determining the actual CSI feedback payload. For AI/ML-based CSI feedback framework, it is a novel solution to generate the CSI feedback payload, which is different from the legacy codebook-based method. Therefore, how to design the AI/ML-based CSI feedback scheme is quite crucial, which should also ensure the flexibility of configuring/determining the CSI payload size by both gNB side and UE side.

For Network configuration to determine CSI payload size, there are at least two options can be further studied.

- Option 1: gNB can configure the UE with an identifier to enable the UE to choose a CSI generation model compatible with the CSI reconstruction model used by the gNB. The model configured by gNB may have one or more candidate CSI payload sizes for each layer separately, where the candidate CSI payload size(s) depend(s) on whether the AI/ML model has scalability. Then UE can determine the RI based on its own measurement algorithm, and select the actual CSI payload size for each layer from the configured CSI payload size(s) subject to the selected RI. After that, the UE needs to report the selected RI and the actual CSI payload size for each layer subject to the reported RI.

- Option 2: gNB can configure the UE with a list of identifiers indicating CSI payload sizes for each layer, and the potential CSI generation models UE can choose that are compatible with the CSI reconstruction models used by the gNB. Then, UE can choose one of the identifier configured by gNB to determine an AI/ML model to apply. Then, actual CSI payload size can be determined as Option 1. In addition, UE can choose one of the identifier configured by gNB to determine a CSI payload size for each layer. Then, UE needs to report the selected RI and the selected identifier of actual CSI payload size for each layer subject to the reported RI.

Proposal 16: In CSI compression using two-sided model use case, further study at least the following options:

- Option 1:

  - For network configuration to determine CSI payload size, gNB can configure the UE with an identifier

  - For UE determination/reporting of the actual CSI payload size, UE reports the selected RI.

- Option 2:

  - For network configuration to determine CSI payload size, gNB can configure a list of identifiers

  - For UE determination/reporting of the actual CSI payload size, UE reports the selected RI and the identifier

Meanwhile, how to report the actual CSI payload size for AI/ML scheme should also be studied. A straightforward way is to report the identifier of the actual CSI payload size for each layer among the configured CSI payload size(s).

The total CSI payload size is dependent on the RI, and the actual CSI payload size per layer. How to map the RI, CQI, identifier(s), actual CSI payload size per layer into a CSI report should be further studied, which should be compatible with the current CSI part 1 and part 2 framework.

Proposal 17: In CSI compression using two-sided model use case, further study how to map RI, CQI, identifier(s) of actual CSI payload size per layer under the current CSI part 1 and part 2 framework.

  - Model monitoring

In the RAN1#112 meeting [3], the following agreement related to monitoring based on intermediate KPIs had been achieved.

Agreement: In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact for intermediate KPIs based monitoring including at least: - NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side. - UE-side monitoring based on the output of the CSI reconstruction model, subject to the aligned format, associated to the CSI report, indicated by the NW or obtained from the network side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - UE-side monitoring based on the output of the CSI reconstruction model at the UE-side - Note: CSI reconstruction model at the UE-side can be the same or different comparing to the actual CSI reconstruction model used at the NW-side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - FFS: Other solutions, e.g., UE-side uses a model that directly outputs intermediate KPI. Network-side monitoring based on target CSI measured via SRS from the UE. Note: Monitoring approaches not based on intermediate KPI are not precluded Note: the study of intermediate KPIs based monitoring should take into account the monitoring reliability (accuracy), overhead, complexity, and latency.

For training Type 1, though CSI generation model and CSI reconstruction model are deployed at UE side and NW side, respectively, they essentially belong to a unified model. So, if model performance loss happens, it should contribute to the whole two-sided model. Thus, AI/ML model monitoring can be performed at either UE side or NW side to monitor the current performance of two-sided model.

Similar to CQI determination, CSI reconstruction model is not available at UE side, hence it’s hard for UE to monitor the model performance based on the actual CSI reconstruction model output. In addition, UE can monitor the performance based on the output of the actual CSI reconstruction model provided from NW to UE. In this case, the method needs to send back the output of CSI reconstruction part from NW side to UE, which will lead to additional latency. However, the channel condition may already change a lot (e.g., interference) so that PMI and CQI mismatch is unavoidable. In addition, the recovered CSI should be quantized (e.g., by eType II codebook), which will lead to additional quantization loss. Moreover, sending the recovered CSI needs enhanced specification to support it. It is also not appropriate for this method to perform UE-side monitoring. Therefore, we propose two methods to monitor the model performance of a two-sided model at UE side and NW side, respectively:

- UE-side monitoring based on the output of the proxy CSI reconstruction model at the UE side: Intermediate KPIs are calculated by UE based on the output of the proxy CSI reconstruction model at the UE side. As shown in Figure 2, UE has its own reference reconstruction part in CSI generation model which is not the same as the actual CSI reconstruction model used at the NW side. Hence, CSI generation model has two output modules. The first one is feedback part, which is used for model input of CSI reconstruction model. The second output (i.e., proxy output) is for model monitoring. As shown in the Figure 2, the proxy model output data is trained to imitate reconstruction model output as much as possible via knowledge distillation technology [7]. By doing this, UE can monitor the loss2 to check the situation of loss1. If the monitoring metrics between input and proxy model output cannot meet a target requirement, so as the monitoring metrics between input and actual output. In our companion contribution [2], this method shows good performance monitoring accuracy, which can be adopted as a candidate method for monitoring at UE side.

Figure 2 Intermediate KPIs calculated by UE based on the output of the CSI reconstruction model at the UE side

The potential specification impact for the monitoring method based on the proxy model should be further studied and discussed. Regarding it is an UE-sided monitoring method, the monitoring metrics, e.g., SGCS, NMSE, etc., should be reported to NW for subsequent decision on AI/ML models. In addition, how to train or enable the proxy CSI reconstruction model at UE may also need to be specified. For example,

- For Type 1 training at NW side, the proxy model is trained at NW side, and NW can configure the appropriate proxy model for performance monitoring and deliver it along with the corresponding CSI generation model from NW to UE.

- For NW-first Type 3 training, the dataset for proxy model training should be at UE side, since UE side has to train a proxy model output to imitate the output of actual CSI generation part at NW side.

Therefore, we propose to further study the potential specification impact to enable performance monitoring using the proxy CSI reconstruction model at UE, at least for

- Report monitoring metrics by UE;

- Model transfer of the proxy model for Type 1 network side training (if applicable);

- The dataset required for training the proxy model at UE side for Type 3 NW-first training.

Proposal 18: In CSI compression using two-sided model use case, for UE-side monitoring, further study the potential specification impact to enable performance monitoring using the proxy model at UE side, at least for

- Report monitoring metrics by UE

- Model transfer of the proxy model for Type 1 network side training (if applicable);

- The dataset required for training the proxy model at UE side for Type 3 NW-first training.

NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE side: Intermediate KPIs are calculated by NW based on traditional CSI and CSI reconstruction model output. As shown in Figure 3, due to the fact that network cannot directly obtain the ground-truth label to calculate the monitoring metrics, UE should report ground-truth CSI to for network to calculate the monitoring metrics. In order to improve the performance of network-based model monitoring, a higher resolution ground-truth label needs to be reported by UE, where our companion contribution shows the monitoring results based on legacy CSI with higher resolution [2]. Similar to data collection, overhead to transmit ground-truth CSI is a big concern. Therefore, an enhanced Type II codebook with acceptable overhead is a promising solution. In our companion contribution [2], this solution is evaluated, which provides good performance monitoring accuracy with acceptable overhead increase of ground-truth CSI. This solution can be considered as a candidate solution for monitoring at NW side.

Figure 3 Intermediate KPIs calculated by NW based on traditional CSI and CSI reconstruction model output

Based on the above analysis, we propose to evaluate the performance of different monitoring cases based on intermediate KPIs and discuss the related evaluation KPIs for companies to calibrate the monitoring performance in agenda item 9.2.2.1. Besides, we propose to further study at least the following two cases for model performance monitoring based on intermediate KPIs in prior,

- Case 1: UE-side monitoring based on the output of the proxy CSI reconstruction model at the UE-side

- Case 2: NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side.

Proposal 19: Prioritize to study the specification impacts on at least the following two cases for model performance monitoring,

- Case 1: UE-side monitoring based on the output of the proxy CSI reconstruction model at the UE-side

- Case 2: NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side.

Proposal 20: For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, further study a high-resolution CSI based on traditional codebook as ground-truth label.

For training type 3, CSI generation model and CSI reconstruction model are actually two separate models. Therefore, if the performance of output CSI is degraded, it cannot be decided whether it’s due to the performance loss of CSI generation model or CSI reconstruction model. Therefore, we should study the mechanisms to monitor model performance of the two models separately (i.e., monitoring the performance of CSI generation part and CSI reconstruction part, respectively). To our understanding, monitoring the performance of CSI reconstruction part can be left up to network implementation, and some assistance information from network is necessary to assist the monitoring of the model trained by UE side. For example, network can send some reference dataset to UE. Then, UE has to feedback the model performance based on the reference dataset. The procedures of dataset sharing can be the same as the procedures of dataset exchange for training type 3. The difference is that the data number for model monitoring can be much smaller than the data number for model training.

Observation 13: For training type 3, CSI generation model and CSI reconstruction model are actually two separate models. Therefore, if the performance of output CSI is degraded, it cannot be decided whether it’s due to the performance loss of CSI generation model or CSI reconstruction model.

Proposal 21: Further study the potential mechanisms and specification impacts on monitoring model performance of the CSI generation model and CSI reconstruction model separately.

Apart from the performance monitoring methods based on intermediate KPIs, eventual KPI-based and input/output-based monitoring methods as below are also agreed in the RAN1#110bis-e meeting [9].

- Agreement In CSI compression using two-sided model use case, further study at least the following options for performance monitoring metrics/methods: - Intermediate KPIs as monitoring metrics (e.g., SGCS) - Eventual KPIs (e.g., Throughput, hypothetical BLER, BLER, NACK/ACK). - Legacy CSI based monitoring: schemes using additional legacy CSI reporting - Other monitoring solutions, at least including the following option: - Input or Output data based monitoring: such as data drift between training dataset and observed dataset and out-of-distribution detection

For performance monitoring based on eventual KPIs, it is hard to identify whether the eventual performance degrades due to the impacts of other factors other than model performance, e.g., varying channel status, varying scheduling/pairing mechanism, etc. In addition, it is not clear about how to remove the impacts of other factors other than the model performance for the Network side monitoring. In [6], an UE-sided monitoring method based on the eventual KPI is proposed that gNB schedules two PDSCHs under the same MU pairing/resource assignment/MCS mechanism but one applied with the precoder from AI/ML-based CSI compression while the other with the precoder from non-AI/ML-based CSI compression (e.g., Type II CB) for performance comparison. To enable UE to make a pair-wise comparison between AI/ML and non-AI/ML in terms of eventual KPI, gNB can indicate which precoding type/CSI feedback mechanism is applied to the PDSCH transmission in the monitoring time window, e.g., PDSCH transmission precoded using the AI/ML-based CSI feedback or using the non-AI/ML-based CSI feedback. However, whether/how gNB will use the CSI provided by gNB is up to gNB implementation. Even with AI/ML-based CSI, gNB may need to make some adjustments on the AI/ML-based CSI in order to perform MU scheduling. From network perspective, the overall system performance is more important than the performance of a single UE. The precoding on PDSCH is transparent to UE side by nature and the gNB behavior should not be mandated. Therefore, this method is not feasible for gNB vendor to support by monitoring the status of data channel to differentiate the performance variations between the legacy scheme and AI/ML scheme. We propose to deprioritize the model performance monitoring based on eventual KPIs and prioritize to study the potential specification impact on AI/ML performance monitoring based on intermediate KPI.

Observation 14: Whether/how gNB will use the CSI provided by gNB is up to gNB implementation. Even with AI/ML-based CSI, gNB may need to make some adjustment on the AI/ML-based CSI in order to perform MU scheduling. From network perspective, the overall system performance is more important than the performance of a single UE. The precoding on PDSCH is transparent to UE side by nature and the gNB behavior should not be mandated.

Observation 15: It is not feasible to support model monitoring based on eventual KPI, e.g., by monitoring the status of data channel to differentiate the performance variations between the legacy scheme and AI/ML scheme.

Proposal 22: Deprioritize the model performance monitoring based on eventual KPIs.

Proposal 23: Prioritize to study the potential specification impact on AI/ML performance monitoring based on intermediate KPI.

For input-based monitoring, it is a method to detect the input data distribution drift between training dataset and measured data and then determine the AI/ML monitoring performance. In principal, input-based monitoring solutions are applied for judging the applicable scope of AI/ML models. For example, the AI/ML model is trained based on the dataset for indoor scenarios and the AI/ML model parameters can apply to the distribution features of indoor data, while the outdoor data follows another data distribution, which is out of the applicable scope of AI/ML models and performance degradation occurs. To our understanding, we should firstly identify what features of data distribution can be applied and what metrics can be adopted for evaluating the feature of monitored data. Hence, the way to calculate data distribution drift can be considered. For example, NW can summarize a table of AI/ML recovery performance with multiple levels based on different datasets in advance, e.g., corresponding to different areas or channel scenarios. Then, UE can calculate the similarity-related metrics between input CSI and different dataset, e.g., SGCS, NMSE, etc., to determine the input CSI belongs to which areas/scenarios and then judge the current AI/ML model performance level for model monitoring.

For output-based monitoring, it is a method to detect the output data distribution drift between reference dataset and observed data and then determine the AI/ML monitoring performance. In principal, output-based monitoring solutions are applied for judging whether the current AI/ML model is available for performance monitoring. If large data distribution drift occurs between the output data and the reference dataset, the AI/ML model is not available to the current channel scenarios and the model switching/updating may be needed. Therefore, we propose to further study the feasibility of input/output-based monitoring methods in Agenda item 9.2.2.1.

Proposal 24: Further study the feasibility of input/output-based monitoring methods in Agenda item 9.2.2.1.

For performance monitoring, the legacy CSI feedback mechanism can be used as a baseline to compare/judge whether the current AI/ML model performs well. If the AI/ML performance is inferior to the legacy, then the decision for model deactivation/switching/updating or fallback to legacy mode can be triggered based on the monitoring results, which is a two-step procedure. Therefore, the study of performance monitoring should be decoupled with the subsequent decision based on the monitoring results.

Proposal 25: The study of performance monitoring should be decoupled with the subsequent decision based on the monitoring results.

  - UE capability

Agreement: In CSI compression using two-sided model use case, further study potential specification impact related to potential co-existence and fallback mechanisms between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode.

AI/ML-based CSI feedback enhancement is a novel topic for NR, and diverse UEs have different capabilities of AI/ML processing. Therefore, UE capability for supporting AI/ML-based CSI feedback enhancement in LCM should be studied. For example, different UE has different capacities of model training/updating/monitoring/inference latency, which results in different CSI report timelines for AI/ML-based CSI feedback. In addition, how UE deals with the conflict problem between AI/ML-based CSI feedback and legacy CSI feedback needs further study. In CSI compression using two-sided model use case, the following issues related to UE capability need to be considered.

- How to quantize the UE capability for a CSI report based on the CSI generation model, e.g., CPU occupancy rule and CSI computation time .

- UE capability related to LCM procedures

Proposal 26: In CSI compression using two-sided model use case, further study potential specification impact of the following UE capability options:

- How to quantize the UE capability for a CSI report based on the CSI generation model, e.g., CPU occupancy rule and CSI computation time .

- UE capability related to LCM procedures

- Conclusion

In this contribution, we discuss the sub use case for AI/ML-based CSI feedback enhancement and identify some specification impacts. We have the following observations and proposals:

- Proposal 1: Deprioritize the discussion of the pros and cons of NW-sided Type 1 training from two types of device agnostic and device specific. If needed, some conditions can be added to reflect this issue.

- Proposal 2: The gradient-exchange based sequential training should be categorized into Type 2 training, and then Type 2 training can be classified as ‘Joint/Sequential gradient-exchange training’ in the table.

- Proposal 3: Prioritize Type 1 joint training at NW side for further study and model transfer/delivery can be further discussed in agenda item 9.2.1.

- Proposal 4: For training Type 3, NW-first training should be prioritized over UE-first training.

- Proposal 5: For training Type 3, further study potential specification impact on the dataset used for the model training at the other side/entity.

- Observation 1: When model training or monitoring is performed at network side, the overhead of the ground-truth label transmitted over the air-interface from UE to network is a huge concern if the ground-truth CSI is an ideal CSI (e.g., raw channels, eigenvectors).

- Observation 2: The overhead of enhanced Type II CB (i.e., PC10) for one training sample increases by 50% compared with the maximal payload of Rel-16 TypeII CB (i.e., PC8) but keeps similar model performance as ideal CSI, which can be acceptable to be carried on UCI.

- Proposal 6: For network side data collection, support to further study

- Enhanced Rel-16 TypeII codebook to get high-resolution CSI;

- PHY signaling or RRC signaling to report the high-resolution CSI.

- Proposal 7: To enable high-quality data collection, at least support

- UE reports associated information to NW, e.g., SINR, CQI, positioning information

- NW configures a threshold of data quality to UE and UE only reports the qualified data to NW

Observation 3: For Type 3 training collaboration of a two-sided model, common understanding on the dataset used for model training is necessary, which can facilitate the pairing of CSI generation part and CSI reconstruction part.

Observation 4: Dataset alignment between UE and network can be used for testing/monitoring the model/functionality performance.

Observation 5: Dataset ID can avoid sharing the proprietary information explicitly across vendors during data collection.

Proposal 8: Support to use dataset ID to identify the delivered dataset of Type 3 training collaboration.

Proposal 9: In CSI compression using two-sided model use case, further study the methods and potential specification impact on mapping priority and omission rule for AI/ML CSI report,

- Dynamic quantization resolution to reduce payload

- Divide the CSI into multiple groups with different priority and omit the CSI groups with low priority, e.g., according to layer, subband and port

- CSI reporting is separated into multiple reports, e.g., to establish the association among the multiple reports

Observation 6: For CQI calculation based on target CSI with realistic channel measurement, UE may over-estimate the channel condition and reconstructed PMI and CQI are not matched. Our simulation results show that the system performance loss is obvious if no advanced CQI adjustment algorithm is used.

Observation 7: For CQI calculation based on target CSI with realistic channel measurement and adjusted by previous CSI reconstruction output provided by NW:

- The output of CSI reconstruction part needs to be provided to UE from NW side, which will lead to additional latency and specification impacts.

- The channel condition may already change a lot (e.g., interference) so that PMI and CQI mismatch is unavoidable.

- The recovered CSI should be quantized (e.g., by eType II codebook), which will lead to additional quantization loss.

Observation 8: For CQI calculation based on target CSI with realistic channel measurement and adjusted by CQI adjustment table provided by NW, NW can construct a CQI adjustment table according to some channel characteristics based on some priori information at gNB side. Then, UE can calculate the similarity-related metrics between measured channel and the channel characteristics to do corresponding CQI adjustment.

Observation 9: For CQI calculation based on legacy codebook, UE may not support traditional codebook and AI/ML codebook simultaneously, which will largely increase the UE complexity. Meanwhile, PMI and CQI mismatching is also unavoidable. If traditional codebook can already get accurate PMI, it is not necessary to implement AI/ML models.

Observation 10: For CQI calculation based on CSI reconstruction output, where CSI reconstruction part at the UE is the same as the actual CSI reconstruction part used at the NW. UE may also be not expected to have CSI reconstruction model as it increases UE computation/storage/power consumption burden to a large extent. In addition, the CSI reconstruction model is generally a proprietary design by network side.

Observation 11: For CQI calculation based on the output of proxy CSI reconstruction model at UE, this method can be applicable for CSI compression using two-sided model sub use case and shows that the average system UPT can be achieved almost the same as the case of CQI calculation based on the output of actual CSI reconstruction model (i.e., performance upper-bound for all options).

Observation 12: For CQI calculation using two stage approach, it is already supported (i.e., when the report quantity is cri-RI-CQI) and less specification impact is foreseen. Besides, the two-step procedure increases the time span of the CQI determination process, which may face the channel variation/aging so that the current CQI cannot match the previous CSI.

Proposal 10: The performance of different CQI determination options should be evaluated in agenda item 9.2.2.1. The pros and cons of all the options should be concluded in 9.2.2.2.

Proposal 11: Further categorize the Option 1b as following:

- Option 1b-1: CQI is calculated based on target CSI with realistic channel measurement and adjusted by previous CSI reconstruction output provided by gNB

- Option 1b-2: CQI is calculated based on target CSI with realistic channel measurement and adjusted by CQI adjustment table provided by gNB.

Proposal 12: According to initial evaluations on performance and specification impacts, the following down-selections are proposed:

- Further study the specification impacts (including the feasibility and necessity) on Option 1a, Option 1b-2 and Option 2a-2.

- No further discussion on specification impacts for Option1b-1, Option 1c, Option 2a-1 and Option 2b.

Proposal 13: In CSI compression using two-sided model use case, LI determination should be studied along with CQI determination.

Proposal 14: In CSI compression using two-sided model use case, if RI is configured to be reported, legacy RI determination can be reused as a starting point.

Proposal 15: Further study potential specification impact on more channel information reported for MU-MIMO scheduling, e.g., full rank report based on the AI/ML model.

Proposal 16: In CSI compression using two-sided model use case, further study at least the following options:

- Option 1:

  - For network configuration to determine CSI payload size, gNB can configure the UE with an identifier

  - For UE determination/reporting of the actual CSI payload size, UE reports the selected RI.

- Option 2:

  - For network configuration to determine CSI payload size, gNB can configure a list of identifiers

  - For UE determination/reporting of the actual CSI payload size, UE reports the selected RI and the identifier

Proposal 17: In CSI compression using two-sided model use case, further study how to map RI, CQI, identifier(s) of actual CSI payload size per layer under the current CSI part 1 and part 2 framework.

Proposal 18: In CSI compression using two-sided model use case, for UE-side monitoring, further study the potential specification impact to enable performance monitoring using the proxy model at UE side, at least for

- Report monitoring metrics by UE

- Model transfer of the proxy model for Type 1 network side training (if applicable);

- The dataset required for training the proxy model at UE side for Type 3 NW-first training.

Proposal 19: Prioritize to study the specification impacts on at least the following two cases for model performance monitoring,

- Case 1: UE-side monitoring based on the output of the proxy CSI reconstruction model at the UE-side

- Case 2: NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side.

Proposal 20: For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, further study a high-resolution CSI based on traditional codebook as ground-truth label.

Observation 13: For training type 3, CSI generation model and CSI reconstruction model are actually two separate models. Therefore, if the performance of output CSI is degraded, it cannot be decided whether it’s due to the performance loss of CSI generation model or CSI reconstruction model.

Proposal 21: Further study the potential mechanisms and specification impacts on monitoring model performance of the CSI generation model and CSI reconstruction model separately.

Observation 14: Whether/how gNB will use the CSI provided by gNB is up to gNB implementation. Even with AI/ML-based CSI, gNB may need to make some adjustment on the AI/ML-based CSI in order to perform MU scheduling. From network perspective, the overall system performance is more important than the performance of a single UE. The precoding on PDSCH is transparent to UE side by nature and the gNB behavior should not be mandated.

Observation 15: It is not feasible to support model monitoring based on eventual KPI, e.g., by monitoring the status of data channel to differentiate the performance variations between the reference scheme and AI/ML scheme.

Proposal 22: Deprioritize the model performance monitoring based on eventual KPIs.

Proposal 23: Prioritize to study the potential specification impact on AI/ML performance monitoring based on intermediate KPI.

Proposal 24: Further study the feasibility of input/output-based monitoring methods in Agenda item 9.2.2.1.

Proposal 25: The study of performance monitoring should be decoupled with the subsequent decision based on the monitoring results.

Proposal 26: In CSI compression using two-sided model use case, further study potential specification impact of the following UE capability options:

- How to quantize the UE capability for a CSI report based on the CSI generation model, e.g., CPU occupancy rule and CSI computation time .

- UE capability related to LCM procedures

- References

- Chairman’s notes, RAN1#112bis-e.

- R1-2304534, Evaluation on AI CSI feedback enhancement, ZTE Corporation.

- Chairman’s notes, RAN1#112.

- Chairman’s notes, RAN1#111.

- Chairman’s notes, RAN1#110.

- R1-2302359, Discussion on AI/ML for CSI feedback enhancement, Huawei, HiSilicon.

- https://en.wikipedia.org/wiki/Knowledge_distillation

- R1-2302436, Discussion on general aspects of common AI PHY framework, ZTE Corporation.

- Chairman’s notes, RAN1#110bis-e.