3GPP TSG RAN WG1 #113	                                       	 R1-2304547

# Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:     9.1.4.1

Source:	Spreadtrum Communications

Title:	             Discussion on UL precoding indication for multi-panel transmission

Document for:	Discussion and decision

Introduction

The 3GPP Rel-18 NR MIMO evolution WID [1] includes the objectives as follows, which is different from the UL panel selection in Rel-17 unified TCI framework and the TDM-based UL transmission in UL multi-TRP enhancement in Rel-17. In this contribution, we will share our views on the issues for simultaneous multi-panel UL transmission (STxMP) including single-DCI based and multi-DCI based multi-TRP operation, assuming up to two panels at UE side and up to two TRPs for PUSCH/PUCCH transmission.

6. Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

Discussion on STxMP for single-DCI based mTRP

Switching mechanism for STxMP PUSCH

Agreement - Support DCI-based dynamic switching between SFN scheme of single-DCI based STxMP PUSCH and sTRP transmission - The DCI field “SRS resource set indicator” is used to indicate the switching between SFN scheme and sTRP transmission. - (Conclusion) There is no consensus to support DCI-based dynamic switching between SFN scheme and SDM scheme Agreement For dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and sTRP transmission: - Use the 2-bit “SRS resource set indicator” DCI field to dynamically indicate the sTRP or SDM transmission. - FFS: how to interpret each codepoint of “SRS resource set indicator”. - For the maximal number of layers for sTRP transmission, down-select: - Option1: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) in current spec - Option2: Configuring one additional maximal numbers of layers for sTRP transmission, in addition to maxRank in current spec. - Down-select one from the following for maximal number of layers of SDM transmission: - Alt1: Configure one single maximal number of layers (separate from the maximal number(s) of layers for sTRP), that is applied to the first SRS resource set and the second SRS resource set, separately. - Alt1a: The maxRank (or Lmax) in current spec is also applied to the first SRS resource set and the second SRS resource set, separately. - Alt2: Configure separate maximal numbers of layers for the first SRS resource set and the second SRS resource set - Alt3: no dedicated configuration for SDM. The maximal number(s) of layers of sTRP and the UE capability reporting for SDM are used to determine the maximal number of layers of SDM transmission. - Alt4: The maximal number(s) of layers in above Option1/2 also applied to the first SRS resource set and the second SRS resource set, separately. - FFS: To ensure the same size of DCI for sTRP and SDM cases, how to use/interpret the TPMI/SRI field(s) and whether to do reserved bit or zero padding. Working Assumption For dynamic switching between STxMP SDM scheme and sTRP transmission, support the following: - For sTRP transmission: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) as in current spec (i.e., Option 1) - For SDM scheme: configure one single maximal number of layers (separate from maxRank (or Lmax) for sTRP) that is applied to the first SRS resource set and the second SRS resource set, separately (i.e., Alt1) - FFS: Whether/How to enable that the total number of used PUSCH antenna ports for the SDM and sTRP is the same. - Note: This corresponds to the case that digital ports are shared between the panels - Note: RAN1 supports both implementations that digital ports are shared or separate among panels Agreement On dynamic switching between STxMP SFN scheme and sTRP transmission: - The legacy maxRank/Lmax is applied to sTRP transmission - For configuration of SFN, - Alt2: Configure a separate parameter for the maximal number of layers for STxMP SFN Agreement The codepoints of “SRS resource set indicator” in DCI for dynamic switching between STxMP SDM and sTRP transmission are interpreted and the SRI/TPMI fields are designed as follows: - The codepoints 00 and 01 indicate sTRP transmission. 00 indicates the first SRS resource set and 01 indicates the second SRS resource set. For SRI/TPMI field design, down-select one from the following Alts: - Alt1: The DCI has two SRI fields and two TPMI fields. The first SRI field and first TPMI field are associated the first SRS resource set if codepoint = 00 or the second SRS resource set if codepoint = 01. The second SRI field and second TPMI fields are reserved. - Alt2: the DCI has only one SRI field and one TPMI field. The SRI and TPMI field are associated with the first SRS resource set if codepoint=00 or the second SRS resource set if codepoint = 01. - Alt3: The DCI has two SRI fields and two TPMI fields. The first SRI field and second SRI field are concatenated into one SRI field. The first TPMI field and second TPMI field are concatenated into one TPMI field. The concatenated SRI field and the concatenated TPMI field are associated with first SRS resource set if codepoint = 00 or the second SRS resource set if codepoint = 01. - FFS: If the concatenated bits are not sufficient, additional bits are appended to concatenated bits in order to support this feature - Alt4: the DCI has two SRI fields and two TPMI fields. - When the codepoint is 00, the first SRI field and first TPMI field are associated with the first SRS resource set. The second SRI field and second TPMI field are reserved. - When the codepoint is 01, the second SRI field and second TPMI field are associated with the second SRS resource set. The first SRI field and first TPMI field are reserved. - The codepoints 10 indicate SDM transmission with the first and second SRS resource set. - The first SRI field and first TPMI field are associated with the first SRS resource set and they indicate the precoder(s)/rank/SRI for the first SRS resource set. - The second SRI field and second TPMI field are associated with the first SRS resource set and they indicate the precoder(s)/rank/SRI for the second SRS resource set. - FFS: The codepoint 11 is reserved. Agreement The codepoints of “SRS resource set indicator” in DCI for dynamic switching between STxMP SFN and sTRP transmission are interpreted and the design of SRI/TPMI fields are as follows: - The codepoints 00 and 01 indicate sTRP transmission. 00 indicates the first SRS resource set and 01 indicates the second SRS resource set. - For the design of SRI/TPMI fields, re-use the design that is decided for dynamic switching between STxMP SDM and sTRP transmission. - The codepoint 10 indicates STxMP SFN transmission with the first and second SRS resource set. - The first SRI field and first TPMI field are associated with the first SRS resource set and they indicate the precoder(s)/rank/SRI for the first SRS resource set. - The second SRI field and second TPMI field are associated with the second SRS resource set and they indicate the precoder(s)/SRI for the second SRS resource set (the rank is indicated by the first SRI field for NCB or the first TPMI field for CB) - FFS: The codepoint 11 is reserved.

In RAN1#110bis meeting, in addition to SDM scheme for UL throughput and reliability enhancement, SFN-based transmission scheme for STxMP PUSCH is also supported, which can be a supplement to the SDM scheme for transmit diversity and high reliability of STxMP PUSCH with single layer or multiple layers. For switching between SDM scheme of single-DCI based STxMP PUSCH and sTRP transmission, and for switching between SFN scheme of single-DCI based STxMP PUSCH and sTRP transmission, the DCI-based dynamic switching scheme is supported. The SRS resource set indicator filed is used to indicate the sTRP or SDM/SFN transmission.

In RAN1#112bis meeting, it was agreed that the codepoints 00  and 01 indicate sTRP transmission and the codepoints 10 indicate SDM/SFN transmission. For sTRP transmission, 00 indicates the first SRS resource set and 01 indicates the second SRS resource set. For SDM/SFN scheme, the first SRI field and first TPMI field are associated with the first SRS resource set and they indicate the precoder(s)/rank/SRI for the first SRS resource set, the second SRI field and second TPMI field are associated with the second SRS resource set and they indicate the precoder(s)/rank/SRI for the second SRS resource set. For codepoint 11, there is no motivation to introduce additional design and it can be reserved.

For TPMI/SRI field design when sTRP transmission is indicated, the solution with only one TPMI/SRI field associated the first or second SRS resource set is not aligned with the field design for STxMP, where two TPMI/SRI fields are used. The solution with the concatenated TPMI/SRI fields associated the first or second SRS resource set is almost identical to the previous solution except for the DCI field determination and zero padding. For this solution with the concatenated TPMI/SRI fields, the benefit for DCI overhead reduction is not significant but the field design is very complex.

For the overhead reduction of signaling design and detection complexity at UE, the DCI field size varying should be avoided for SDM scheme and sTRP transmission. When sTRP transmission is indicated, the first TPMI/SRI field can be used to indicate the precoding, rank and SRS resource for sTRP. For the purpose of same DCI field size, the size of first TPMI/SRI field can be determined by the larger value between maxRank for sTRP and new maxRank for SDM scheme, while for sTRP transmission the second TPMI/SRI field can be ignored. This is a direct solution with small spec impact for the UE with symmetric panels.

Similarly, for dynamic switching between SFN scheme and sTRP transmission, the first TPMI/SRI field is used to indicate the precoding and rank for first SRS resource set for SFN scheme and also for sTRP transmission.

Proposal 1:

- For TPMI/SRI field design when sTRP transmission is indicated, support Alt1, i.e. the DCI has two SRI fields and two TPMI fields. The first SRI field and first TPMI field are associated the first SRS resource set if codepoint = 00 or the second SRS resource set if codepoint = 01. The second SRI field and second TPMI fields are reserved.

  - To maintain the same field size, the size of first TPMI/SRI field is determined by the larger value between the maximal number of layers for sTRP and the maximal number of layers corresponding to the first TPMI/SRI field for SDM scheme.

Proposal 2:

- For dynamic switching between SDM/SFN scheme of single-DCI based STxMP PUSCH and sTRP transmission,

  - Codepoint ‘11’ in SRS resource set indicator field is reserved.

When SDM scheme and sTRP transmission are dynamically switched by DCI indication, it should be clarified how to determine the maxRank of SDM scheme and the maxRank of sTRP transmission. For the UE with symmetric panels, the legacy maxRank can be reused for sTRP transmission, and one new maxRank can be configured for SDM scheme that is applied to the first SRS resource set and the second SRS resource set, separately. For example, legacy maxRank is configured to be 4 for sTRP and the new maxRank for SDM is set to 2. In case that digital ports are shared between the panels, for sTRP transmission 4 layers are transmitted from first panel or second panel, but for SDM scheme each panel can be used to transmit 2 layers. In case that digital ports are not shared between two panels, even if 4 SRS ports are supported actually for each panel, for SDM scheme the SRS port subset, e.g. 2 ports from each panel, can be selected by UE implementation to transmit PUSCH with 2+2 layers.

Proposal 3:

- Confirm the working assumption in RAN1#112 as follows:

For dynamic switching between STxMP SDM scheme and sTRP transmission, support the following: - For sTRP transmission: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) as in current spec (i.e., Option 1) - For SDM scheme: configure one single maximal number of layers (separate from maxRank (or Lmax) for sTRP) that is applied to the first SRS resource set and the second SRS resource set, separately (i.e., Alt1)

Signaling for STxMP PUSCH

Agreement For SDM scheme of single-DCI based STxMP PUSCH - Configure two SRS resource sets for CB or NCB. - FFS: These two SRS resource sets can have different number of SRS resources for codebook -based or non-codebook based. - For codebook -based PUSCH, DCI indicates two TPMI fields, and each TPMI field separately indicates the precoding information and the number of layers conveyed over the SRS ports of the indicated SRS resource in each SRS resource set. - For non-codebook based PUSCH and codebook -based PUSCH, DCI indicates two SRI fields and each field indicates SRS resource(s) for each SRS resource set separately. - FFS : For codebook -based PUSCH , the two SRS resources indicated by the two SRI fields can have different number of SRS ports Agreement For the SFN scheme of single-DCI based STxMP PUSCH: - Configure two SRS resource sets for CB or NCB. - FFS: Number of SRS resources of SRS resource set, and number of SRS ports of SRS resource - The DCI indicates two SRI fields and TPMI fields for SFN transmission, - On the indication of number of layers for CB and NCB PUSCH: - Alt1: Similar to rel-17 mTRP TDM scheme, the number of layers is indicated by the first SRI field (for NCB PUSCH) or the first TPMI field (for CB PUSCH) Conclusion - RAN1 has no consensus to support the following in Rel-18: - Configure different number of SRS resources in the two SRS resource sets for CB (if fullpowermode 2 is not configured) or NCB for single-DCI based STxMP transmission. - For CB PUSCH, the two SRS resources indicated by two SRI fields for single-DCI based STxMP transmission can have different number of SRS ports. - For the two SRS resource sets configured for multi-DCI based STxMP PUSCH+PUSCH in Rel-18, - Legacy Rel-17 specification is reused with respect to the maximal number of SRS resources/ports in each set FFS: whether/how to support two different configurations with regards to full power mode and antenna port coherency type among SRS resource sets. Conclusion There is no consensus to support layer combinations {1+3} and {3+1} in SDM scheme of single-DCI based STxMP PUSCH. Conclusion There is no consensus to support 2 CWs in single-DCI based STxMP SDM scheme Agreement - Reuse the DCI field ‘Antenna Ports’ in DCI format 0_1 and 0_2 to indicate DMRS ports for SDM scheme of single-DCI based STxMP PUSCH: - The total numbers of layers, L, indicated by two TPMI fields of CB PUSCH or two SRI fields of NCB PUSCH is used to determine the DMRS port indication table. - L1 of the indicated DMRS ports are associated with the L1 PUSCH layers which are indicated by the first TPMI field for CB PUSCH or the first SRI field for NCB PUSCH and the rest L- L1 of the indicated DMRS ports are associated with the L-L1 PUSCH layers which are indicated by the second TPMI field for CB PUSCH or the second SRI field for the NCB PUSCH. - FFS: how to partition the indicated DMRS ports among the PUSCH layers. - Down-select one from the following two Alts for SDM scheme in RAN1#111: - Alt1: the DMRS ports associated with two TPMI/SRI fields must be in different CDM groups. - Alt2: the DMRS ports associated with two TPMI/SRI fields can be in same or different CDM groups. Agreement For the DMRS port indication for SDM scheme single-DCI based STxMP transmission, support Alt2: - Alt2: the DMRS ports associated with two TPMI/SRI fields can be in same or different CDM groups. Agreement To support indicating DMRS ports in different CDM groups for layer combination {1+2} in SDM - Add new entry {0, 2, 3} to the DMRS table for the layer combination {1+2}; - This is optional UE capability for UE that supports sDCI based STxMP SDM Agreement When max 2 PTRS ports are configured for SDM scheme of single-DCI based STxMP PUSCH: - Actual number of PTRS ports in SDM is 2 and 2-bit “PTRS-DMRS association” DCI field is used to indicate the PTRS-DMRS association for the DMRS ports associated with two TMPI/SRI fields. - The MSB indicates the association between PTRS port 0 and the DMRS port(s) associated with the first TPMI/SRI field. - The LSB indicates the association between PTRS port 1 and the DMRS port(s) associated with the second TPMI/SRI field. - Regarding the “ptrs-PortIndex” configured to SRS resource for NCB PUSCH of SDM scheme, the UE ignores the configuration of “ptrs-PortIndex” per SRS resource. FFS: Whether additional RRC configuration is needed for the max number of PTRS ports for SDM transmission Agreement For single-DCI based STxMP SFN scheme, - Alt2: When maxNrofPorts = 2 is configured for PTRS in SFN scheme, the actual number of PTRS port(s) in SFN is determined by the 1st TPMI field for CB or 1st SRI field for NCB - Each PTRS port is transmitted in SFN manner

In order to enable TRP-specific UL precoding indication, similar to mTRP TDM-based PUSCH repetition in Rel-17, two SRS resource sets are supported to be configured for CB or NCB, and the SRS resource set indicator field and two TPMI/SRI fields in DCI can be reused. In Rel-18, the number of SRS resources in these two SRS resource sets are same. Besides, for codebook-based PUSCH, the two SRS resources indicated by two SRI fields have the same number of SRS ports.

In Rel-18, the UE with symmetric panels is supported. It is not necessary to support different configurations with regards to full power mode and antenna port coherency type among SRS resource sets, which will increase the complexity of UE development and spec design, and the motivation and benefits are also unclear.

Proposal 4:

- Don’t support two different configurations with regards to full power mode and antenna port coherency type among SRS resource sets.

In RAN1 #112 meeting, for SDM scheme of single-DCI based STxMP PUSCH transmission, 2 PTRS ports can be configured to track phase noise for non-coherent antenna ports for two UE panels respectively. The PTRS-DMRS association field is reused to indicate the PTRS-DMRS association for DMRS ports associated with two TPMI/SRI fields. The MSB indicates the association between PTRS port 0 and the DMRS port associated with the first TPMI/SRI field. The LSB indicates the association between PTRS port 1 and the DMRS port associated with the second TPMI/SRI field. If the number of layers indicated by TPMI/SRI field for a UE panel is 1, the corresponding bit in PTRS-DMRS association can be ignored. Based on PTRS-DMRS association field in DCI, the association between PTRS port and DMRS port can be determined, and in other words the association of PTRS port and SRS resource set is determined.

For SFN scheme of single-DCI based STxMP PUSCH transmission, the actual number of PTRS ports is determined by the first TPMI field for CB or first SRI field for NCB, where the PTRS port(s) is associated with both SRS resource sets and transmitted in SFN manner. We note that the agreement only defines how to determine the actual number of PTRS ports for SFN scheme and the SFN manner for each PTRS port. However, there is no relevant conclusion for PTRS association. There is a remaining issue that needs to be discussed, i.e. the association between PTRS port and DMRS port associated with two TPMI/SRI fields for SFN scheme, especially when the actual number of PTRS ports is 2.

For example, for SFN scheme two layers are transmitted from two panels, e.g. the first TPMI is indicated as 10010000 and the second TPMI is indicated as 10000100. The number of PTRS ports is determined by the first TPMI as 2. If the association between PTRS port and DMRS port for panel 1 can be applied for both panels, the PTRS port 0 is associated with the first antenna port and the PTRS port 1 is associated with the second antenna port. But for panel2, two layers are transmitted from the first and third antenna ports which can be associated with PTRS port 0. It is not clear how to determine the PTRS port 1 association for panel2 and whether the legacy PTRS port sharing for antenna ports still apply. Thus, it is necessary to clarify the rule for PTRS port association for SFN scheme.

The another issue is when the strongest layers are different for two panels, how to ensure the performance of phase noise tracking. Similar to the principle of the association between PTRS port and DMRS port for SDM scheme, when the actual number of PTRS ports is 2, the association between two PTRS ports and DMRS ports associated with two TPMI/SRI fields can be indicated separately and the restriction on the PTRS port sharing can be removed to ensure that each PTRS port can be associated with the strongest layer on each panel.

For example, if the maximal number of layers is 4, PTRS-DMRS association field is used to indicate the association between PTRS port 0 and the DMRS port associated with the first TPMI/SRI field, Second PTRS-DMRS association field is used to indicate the association between PTRS port 1 and the DMRS port associated with the second TPMI/SRI field. While if the maximal number of layers is 2, the MSB in PTRS-DMRS association field is used to indicate the association between PTRS port 0 and the DMRS port associated with the first TPMI/SRI field, the LSB in PTRS-DMRS association field is used to indicate the association between PTRS port 1 and the DMRS port associated with  the second TPMI/SRI field. It is worth noting that two PTRS ports may be associated with the same strongest DMRS port on two panels. For the accuracy of phase noise tracking and interference reduction, different RE offsets can be used for PTRS port 0 and PTRS port 1.

Proposal 5:

- For SFN scheme of single-DCI based STxMP PUSCH transmission, when the actual number of PTRS ports is 2, the association between PTRS port 0 and DMRS port associated with the first TPMI/SRI field, and the association between PTRS port 1 and DMRS port associated with the second TPMI/SRI field can be indicated separately.

STxMP PUCCH

Agreement Support the SFN scheme for single-DCI based STxMP PUCCH transmission

In RAN1 #111 meeting, the SFN scheme for single-DCI based STxMP PUCCH transmission was supported. Similar to STxMP PUSCH, the switch between SFN scheme and the Rel-17 mTRP TDM scheme, and the switch between SFN scheme and sTRP transmission can be implemented by RRC configuration for each PUCCH resource. When two joint or UL TCI states are applied to a PUCCH resource, the UL transmission scheme, i.e. SFN or TDM, can be determined based on RRC configuration. For sTRP transmission for a PUCCH resource, the PUCCH resource applying one joint or UL TCI state can be indicated by DCI. Thus, the transmission scheme can be switched through the different PUCCH resources configured/indicated by gNB.

Proposal 6:

- For transmission scheme switching between SFN scheme and TDM scheme, RRC configuration per PUCCH resource is used.

- For transmission scheme switching between SFN scheme and sTRP transmission, the PUCCH resource indication by gNB is used.

Discussion on STxMP for multi-DCI based mTRP

STxMP PUSCH+PUSCH

Agreement Support STxMP PUSCH+PUSCH transmission in multi-DCI based system in Rel-18. - Two independent PUSCHs associated with different TRPs can be transmitted by a UE simultaneously in same active BWP. - The total number of layers of these two PUSCHs is up to 4. - FFS: whether the number of layers of each of these two PUSCHs is up to 2. Agreement For STxMP PUSCH+PUSCH transmission in multi-DCI based system: - The maximal number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain can be 1 or 2 subject to UE capability Agreement Multi-DCI based STxMP PUSCH+PUSCH transmission at least supports the following PUSCH combinations: - DG-PUSCH + DG-PUSCH - CG-PUSCH + DG-PUSCH Agreement The multi-DCI based STxMP PUSCH+PUSCH transmission supports fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain. - FFS whether/how to handle the PUSCH power adjustment when two PUSCHs are fully/partially overlapped in time domain (Depending on RAN4’s input on Pcmax requirements). - Note: No symbol-level power adjustment within a PUSCH transmission occassion in the case of fully/partially overlapping in time domain Agreement Regarding the TPMI/SRI indication for multi-DCI based STxMP PUSCH+PUSCH: - Configure two SRS resource sets for CB or NCB. - FFS: Whether/how to associate coresetPoolIndex with SRS resource set implicitly or explicitly. - FFS: the maximal number of configured/indicated SRS resources in each set for NCB/CB - FFS: the maximal number of SRS ports in each set for CB. - FFS: Separate codebooks and separate maxRanks are configured for different SRS resource sets. - For type 1 CG-PUSCH (if supported), FFS how to associate the PUSCH with one TRP - e.g., configure a coresetPoolIndex value in a type 1 CG-PUSCH - e.g., use a single CG to configure two type 1 CG PUSCHs for STxMP PUSCH+PUSCH Agreement Support CG PUSCH + CG PUSCH in multi-DCI based STxMP PUSCH+PUSCH transmission. Agreement - For multi-DCI based STxMP, to schedule a PUSCH for STxMP PUSCH+PUSCH transmission, - Alt1: The first SRS resource set is associated with coresetPoolIndex value 0 and the other SRS resource set is associated with coresetPoolIndex value 1 - The PUSCH is associated with SRS resource set with the same value of coresetPoolIndex - FFS: Which is the first SRS resource set, e.g., the set with lower set ID. - Regarding how to interpret the SRI/TPMI field in DCI: - For DG-PUSCH, the indicated SRI/TPMI field corresponds to the SRS resource set associated with same coresetPoolIndex value of the CORESET where scheduling DCI format 0_1 or 0_2 is received - For Type 2 CG-PUSCH, the indicated SRI/TPMI field corresponds to the SRS resource set associated with same coresetPoolIndex value of the CORESET where activation DCI is received. - For Type 1 CG-PUSCH, one SRS_resource_set_index value is configured in RRC in ConfiguredGrantConfig and the srs-ResourceIndicator/precodingAndNumberOfLayers correspond to the SRS resource set Agreement Among the two SRS resource sets configured for multi-DCI based STxMP PUSCH+PUSCH, the SRS resource set with lower set ID is the first SRS resource set. Agreement For multi-DCI based STxMP PUSCH+PUSCH, study enhancements of the UCI multiplexing rule to address the case that one PUCCH overlaps with two overlapped PUSCHs of STxMP PUSCH+PUSCH Conclusion - RAN1 has no consensus to support the following in Rel-18: - Configure different number of SRS resources in the two SRS resource sets for CB (if fullpowermode 2 is not configured) or NCB for single-DCI based STxMP transmission. - For CB PUSCH, the two SRS resources indicated by two SRI fields for single-DCI based STxMP transmission can have different number of SRS ports. - For the two SRS resource sets configured for multi-DCI based STxMP PUSCH+PUSCH in Rel-18, - Legacy Rel-17 specification is reused with respect to the maximal number of SRS resources/ports in each set FFS: whether/how to support two different configurations with regards to full power mode and antenna port coherency type among SRS resource sets. Agreement For case that one PUCCH overlaps with two overlapped PUSCHs in multi-DCI based STxMP PUSCH+PUSCH, down-select one for the UCI multiplexing: - Option 1: the UCI is multiplexed into the PUSCH associated with the same TRP. And among the PUSCHs associated with the same TRP, the legacy PUSCH priority order for UCI multiplexing is applied. FFS: determining the PUSCH associated with the same TRP. - Option 2: the legacy PUSCH priority order for UCI multiplexing is first applied and if at last, there are two PUSCHs with the same start time in one same CC, the UCI is multiplexed in (FFS: one or two) of these two PUSCHs, and FFS which one PUSCH. - Option 3: - When joint HARQ-ACK feedback is configured, the legacy PUSCH priority order for UCI multiplexing is first applied and if at last, there are two PUSCHs with the same start time in one same CC, the UCI is multiplexed in (FFS: one or two) of these two PUSCHs, and FFS which one PUSCH. - When separate HARQ-ACK feedback is configured, at least when the UCI includes HARQ-ACK, the UCI is multiplexed into the PUSCH associated with the same TRP. And among the PUSCHs associated with the same TRP, the legacy PUSCH priority order for UCI multiplexing is applied. FFS: determining the PUSCH associated with the same TRP. - FFS: When the UCI does not include HARQ-ACK (CSI and/or SR), whether to follow the same behavior as above, or to follow the behavior of the case that joint HARQ-ACK feedback is configured. - Note: Here using joint HARQ-ACK feedback and separate HARQ-ACK feedback is mainly for discussion purpose. FFS: whether to introduce a new RRC parameter to indicate that. - FFS the impact of the following legacy restriction on the above options: when separate HARQ feedback is configured, a PUCCH transmission triggered by DCI associated with one coresetPoolIndex cannot overlap in time with a PUSCH transmission triggered by DCI associated with another coresetPoolIndex. - Note: each of the above options is applied to the system when the system is configured with multi-DCI based STxMP PUSCH+PUSCH.

For multi-DCI based STxMP PUSCH+PUSCH, there is a remaining issue that one PUCCH overlaps with two overlapped PUSCHs. The legacy rule in Rel-15 is multiplexing UCI in the PUSCH resource based on a priority order when a UE transmits multiple PUSCHs on respective serving cells in a slot, and the case that is not included is two PUSCHs in the same CC with the same start time and these two PUSCHs are transmitted from different UE panels.

Under the non-ideal backhaul assumption for multi-DCI based STxMP, legacy PUSCH priority rule cannot work, because one TRP does not know the scheduling from the other TRP. Thus, a most direct solution is per TRP/panel UCI multiplexing, i.e. the legacy UCI multiplexing rule is applied per TRP/panel, which is still applied for UCI other than HARQ-ACK. For determining the PUSCH associated with the same panel, the same coresetPoolIndex value can be used. For the case that when joint HARQ-ACK feedback is configured, the typical assumption is idea backhaul but it will increase spec design complexity and the UCI performance loss is not clear. As an unified design, we prefer the per TRP/panel UCI multiplexing solution. For the case that one PUCCH overlaps with one PUSCH with different coresetPoolIndex value, a dropping rule can be considered in Rel-18.

Proposal 7:

- For case that one PUCCH overlaps with two overlapped PUSCHs in multi-DCI based STxMP PUSCH+PUSCH, support Option1, i.e.

  - The UCI is multiplexed into the PUSCH associated with the same TRP. Among the PUSCHs associated with the same TRP, the legacy PUSCH priority order for UCI multiplexing is applied.

  - For determining the PUCCH and the PUSCH associated with the same panel, the same coresetPoolIndex value can be used.

Mechanism for overlapping channels

For multi-DCI based mTRP, STxMP PUSCH+PUSCH transmission across multiple UE panels is supported in Rel-18. Since the backhaul between multiple TRPs may not be ideal, the cases that different types of channels/signals overlapping in time domain could take place, e.g. PUCCH+PUSCH. One illustration is shown below in Figure 1, where PUCCH, PUSCH and SRS for different TRPs are overlapped in time domain.

Figure 1 One illustration on overlapped channels/signals for different TRPs

Thus, it is necessary to consider how to select one channel/signal to transmit and drop another for overlapping cases. For example, learning from Rel-15/Rel-16/Rel-17, in our opinion the following principles can be considered:

  - Priority index of channel/signal: The larger the priority index is, the higher the priority is. For example, for case (a) in Figure 1, assuming the priority index of PUCCH is 1 and the priority index of PUSCH is 0, then PUSCH can be dropped and PUCCH would be transmitted.

  - The type of channel/signal. For example, for case (b) in Figure 1, assuming the priority of PUSCH is higher than periodic SRS, periodic SRS overlapped with PUSCH can be dropped.

  - The information carried by the channel. For example, for case (c) in Figure 1, assuming priority for overlapping channels/signals is the same, aperiodic SRS can be dropped when PUCCH carries HARQ-ACK information, link recovery request and/or SR; aperiodic SRS can be transmitted when PUCCH carries semi-persistent/periodic CSI report or semi-persistent/periodic L1-RSRP/L1-SINR report only.

With what we have said, other principles are also not precluded. The solutions are use case specific.

Proposal 8:

- When different types of channels/signals for different TRPs/panels are overlapping in time domain, the dropping mechanisms should be considered.

Discussion on group-based beam report enhancement for STxMP

Agreement Enhance the Rel-17 group-based beam L1-RSRP reporting to support STxMP-based transmission and down-select one in RAN1#113 meeting: - Alt1: In each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously, and the reported pair of CRIs or SSBRIs can be received simultaneously. - Alt2: In each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously. - Alt3: In each reported pair of CRIs or SSBRIs, UE indicates if the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously, and/or if the reported pair of CRIs or SSBRIs can be received simultaneously. - FFS: Introduce an indicator to support the above, and the number of bits and interpretation of each codepoint of the indicator

In Rel-17, the non-group based beam report is enhanced for MPUE, where the higher layer parameter reportQuantity set to ' cri-RSRP- Index' or 'ssb-Index-RSRP- Index' is used to enable a beam report including the index of UE capability value set along with the pair of SSBRI/CRI and L1-RSRP to facilitate UE-initiated panel selection.

In Rel-18, to indicate UL beam pair, a new group based beam report can be considered to support STxMP-based transmission. In legacy group based beam report, the beam pair(s) can be indicated for DL simultaneous reception. For enhanced group based beam report, on possible solution is to report at least a pair of CRIs or SSBRIs to determine the UL beam pair for STxMP and the simultaneous DL beam pair.  However, it is not necessary that every beam pair in enhanced group based beam report must be always received simultaneously without clear motivation and benefit. The DL beam report for mTRP and UL beam report for STxMP should be designed separately and there is no need to introduce a mixed beam report, similar to the Rel-15 non-group based beam report and Rel-17 enhanced non-group based beam report. It should be allowed a UE to report a beam pair just for STxMP. Thus, a new value of the RRC parameter reportQuantity can be used to indicate UL only beam pair(s) for STxMP.

The UE capability value set index in Rel-17 enhanced beam report is used to indicate different panels. In Rel-18 the UE with asymmetric panels is not supported for STxMP, which means the beam pair in the enhanced group based beam report is from two panels with same capability. So it is not necessary to report UE capability value set index for each pair of CRIs/SSBRIs.

Proposal 9:

- For the group based beam report enhancement to support STxMP-based transmission, support Alt2, i.e. in each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously.

  - A new value of the RRC parameter reportQuantity can be used to indicate this new report format for STxMP.

  - The UE capability value set index is not reported.

Discussion on power control for STxMP

For Rel-17 unified TCI framework, UL power control parameter setting (including PLRS, P0, alpha, closed loop index) can be configured in the indicated TCI state. In Rel-18 STxMP, the Rel-17 power control mechanism can be reused for panel-specific power control. When multiple unified TCI states are indicated for multi-TRP respectively, the power control parameters corresponding to each SRS resource set can be determined by the indicated TCI state corresponding to the same TRP as the SRS resource set. Besides, closed-loop power control in Rel-17 can be reused for STxMP, e.g. two TPC fields for Rel-17 UL mTRP.

Proposal 10:

- For power control of STxMP, support panel-specific power control mechanism including closed-loop power control and open-loop power control determined by indicated unified TCI states.

In Rel-15/Rel-16/Rel-17, the transmission power is calculated according to the same power control formulations in clause 7 of [2], which would not exceed the maximum allowed transmission power. However, as shown in Figure 2, when STxMP PUSCHs from different UE panels are transmitted, the transmission power of each PUSCH can be achieved independently as the legacy scheme. However, there is possibility that the sum power of the two PUSCHs may exceed the maximum allowed transmission power for one UE. Thus, enhancement on power control for STxMP PUSCH should be considered.

Figure 2 One illustration of two PUSCHs for different TRPs

Proposal 11:

- For STxMP scheme, when the total transmission power of PUSCHs from different UE panels exceeds the maximum allowed transmission power per UE, power control enhancement should be considered.

In Rel-17 for single-DCI based mTRP transmission, PHR reporting scheme is designed for TDM scheme. The similar principle can be reused for single-DCI based STxMP transmission, where two panels are used for simultaneous transmission of PUSCH and correspond to different TRPs. When the triggering condition is satisfied, two PH values are calculated and reported. Whether the PH value is actual or virtual depends on the transmission scheme, i.e. sTRP transmission or STxMP transmission. For multi-DCI based STxMP, it is reasonable to consider the separate PHR reporting per panel, especially for non-ideal backhaul case, where PHR reporting for each panel can be triggered, calculated and transmitted respectively.

Proposal 12:

- For STxMP for single-DCI based mTRP, when the joint triggering condition is satisfied, two PH values are calculated and reported.

- For STxMP for multi-DCI based mTRP, support separate PHR triggering and reporting.

In Rel-17, MPE report is enhanced to report N pairs of P-MPR and SSBRI/CRI from one candidate RS resource pool when the triggering condition is met. In Rel-18 for STxMP, the MPE report mechanism also needs to be studied, e.g. new triggering condition, candidate RS resource pool configuration, MPE report format and transmission, etc.

Proposal 13

- Study the enhanced MPE report mechanism to support STxMP.

Conclusion

In this contribution, we provide our proposals on UL precoding indication for multi-panel transmission:

Proposal 1:

- For TPMI/SRI field design when sTRP transmission is indicated, support Alt1, i.e. the DCI has two SRI fields and two TPMI fields. The first SRI field and first TPMI field are associated the first SRS resource set if codepoint = 00 or the second SRS resource set if codepoint = 01. The second SRI field and second TPMI fields are reserved.

  - To maintain the same field size, the size of first TPMI/SRI field is determined by the larger value between the maximal number of layers for sTRP and the maximal number of layers corresponding to the first TPMI/SRI field for SDM scheme.

Proposal 2:

- For dynamic switching between SDM/SFN scheme of single-DCI based STxMP PUSCH and sTRP transmission,

  - Codepoint ‘11’ in SRS resource set indicator field is reserved.

Proposal 3:

- Confirm the working assumption in RAN1#112 as follows:

For dynamic switching between STxMP SDM scheme and sTRP transmission, support the following: - For sTRP transmission: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) as in current spec (i.e., Option 1) - For SDM scheme: configure one single maximal number of layers (separate from maxRank (or Lmax) for sTRP) that is applied to the first SRS resource set and the second SRS resource set, separately (i.e., Alt1)

Proposal 4:

- Don’t support two different configurations with regards to full power mode and antenna port coherency type among SRS resource sets.

Proposal 5:

- For SFN scheme of single-DCI based STxMP PUSCH transmission, when the actual number of PTRS ports is 2, the association between PTRS port 0 and DMRS port associated with the first TPMI/SRI field, and the association between PTRS port 1 and DMRS port associated with the second TPMI/SRI field can be indicated separately.

Proposal 6:

- For transmission scheme switching between SFN scheme and TDM scheme, RRC configuration per PUCCH resource is used.

- For transmission scheme switching between SFN scheme and sTRP transmission, the PUCCH resource indication by gNB is used.

Proposal 7:

- For case that one PUCCH overlaps with two overlapped PUSCHs in multi-DCI based STxMP PUSCH+PUSCH, support Option1, i.e.

  - The UCI is multiplexed into the PUSCH associated with the same TRP. And among the PUSCHs associated with the same TRP, the legacy PUSCH priority order for UCI multiplexing is applied.

  - For determining the PUCCH and the PUSCH associated with the same panel, the same coresetPoolIndex value can be used.

Proposal 8:

- When different types of channels/signals for different TRPs/panels are overlapping in time domain, the dropping mechanisms should be considered.

Proposal 9:

- For the group based beam report enhancement to support STxMP-based transmission, support Alt2, i.e. in each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously.

  - A new value of the RRC parameter reportQuantity can be used to indicate this new report format for STxMP.

  - The UE capability value set index is not reported.

Proposal 10:

- For power control of STxMP, support panel-specific power control mechanism including closed-loop power control and open-loop power control determined by indicated unified TCI states.

Proposal 11:

- For STxMP scheme, when the total transmission power of PUSCHs from different UE panels exceeds the maximum allowed transmission power per UE, power control enhancement should be considered.

Proposal 12:

- For STxMP for single-DCI based mTRP, when the joint triggering condition is satisfied, two PH values are calculated and reported.

- For STxMP for multi-DCI based mTRP, support separate PHR triggering and reporting.

Proposal 13

- Study the enhanced MPE report mechanism to support STxMP.

- Reference

- RP-213598, MIMO Evolution for Downlink and Uplink

- 3GPP TS 38.213: "NR; Physical Layer Procedures for control".