3GPP TSG RAN WG1 #113                 		         R1-2304550

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:     9.2.2.1

Source:	Spreadtrum Communications, BUPT

Title:	             Discussion on evaluation on AIML for CSI feedback enhancement

Document for:	Discussion and decision

Introduction

In RAN#94e meeting, one SID on AI/ML for Air interface is approved [1]. Three use cases are identified as initial set of use case shown below.

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

In this paper, we would focus on the sub use case: CSI feedback enhancement including CSI compress and recovery and CSI prediction. We would also present initial evaluation results about this sub use case.

Discussion

Evaluations for CSI compression and recovery

AI model for CSI compression and recovery

In the previous meeting, it has been agreed that for the evaluation of the AI/ML based CSI compression sub use cases, a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part and an AI/ML-based CSI recovery part. So in the simulation, we use the Transformer model, where the encoder as the CSI generation part is located at the UE side and the decoder as the CSI recovery part is located at the gNB side, as shown in figure 1. The encoder mainly includes the embedding layer, multi-head attention layer and quantization layer. The decoder mainly includes the dequantization layer, embedding layer and multi-head attention layer. After the decoder, the CSI can be recovered through a fully connected layer. The AI model parameters are shown in table 1.

Table 1. AI model parameter for CSI compression and recovery

| Transformer Parameters | Values |
| --- | --- |
| Embedding dimension | 512 |
| Model Layer | 10 |
| Head of attention layer | 16 |
| Quantification method | Vector quantization |

Figure 1. AI model for CSI compression and recovery

Evaluation results for CSI compression and recovery

In this section, we provide the data set generation, training parameter and evaluation results. For the data generation, we consider a Dense Urban (Macro only) scenario operating on 2GHz FDD spectrum, where 1140 UEs (57 cells, and 20 users per cell) are generated. The number of frequency domain sub-bands is 12. The detailed evaluation assumptions can refer to the Appendix 1. We collect 570000 samples, 90% of which are used as training set and 10% as validation set. The input of AI model is eigenvector of the channel matrix. The main training parameters are shown as table 2.

Table 2. Training parameter for CSI compression and recovery

| Training parameters | Value |
| --- | --- |
| Batch size | 512 |
| Epoch | 200 |
| Optimizer | Adam |
| Learning rate | 0.0001 |

In the previous meeting, it has been agreed that SGCS (Squared Generalized Cosine Similarity) can be used as the intermediate KPIs and throughput can be used as the eventual KPI. Therefore, based on the trained AI model, we provide the evaluation results on SGCS in figure 2 and throughput gain in figure 3, and the Rel-16 eType II codebook is used as the baseline for performance evaluation. In table 3, the UPT gain is calculated between AI based CSI compression and recovery and Rel-16 eType II codebook.

Figure 2. Comparison of SGCS for AI based CSI compression and recovery

Table 3. Comparison of UPT for AI based CSI compression and recovery

| Gain of UPT Feedback bits | Mean UPT (RU 40%-69%) | Mean UPT (RU >=70%) | 5% UPT (RU 40%-69%) | 5% UPT (RU >=70%) |
| --- | --- | --- | --- | --- |
| 49 | 12.40% | 11.59% | 22.11% | 36.92% |
| 72 | 12.77% | 7.47% | 21.04% | 34.83% |
| 88 | 13.55% | 3.17% | 19.51% | 28.12% |
| 133 | 15.18% | 11.23% | 18.61% | 27.23% |
| 164 | 16.76% | 10.97% | 19.51% | 28.61% |
| 248 | 21.21% | 11.44% | 15.87% | 20.20% |

From the evaluation results in figure 2, AI based CSI compression and recovery can achieve about 14% SGCS gain than Rel-16 eType II codebook for the same feedback bits. Accordingly, AI based CSI compression and recovery can save more than 50% of feedback bits than Rel-16 eType II codebook for the same SGCS. From table 3, AI based CSI compression and recovery can achieve about 13% gain for mean UPT, and 24% gain for 5% UPT. Therefore, it can be concluded that the AI based CSI compression and recovery can achieve better SGCS and UPT performance and lower feedback bits than Rel-16 eType II codebook.

Observation 1: The AI based CSI compression and recovery can achieve better SGCS and UPT performance than Rel-16 eType II codebook.

In practical deployment, if different models are separately trained for different scenarios, several AI models need to be deployed at UE side or delivered to UE. The cost is too large. Therefore, the generalization of AI model is very important, and the following agreement about generalization was achieved in the RAN1 #110-e meeting.

Agreement For CSI enhancement evaluations, to verify the generalization performance of an AI/ML model over various scenarios, the set of scenarios are considered focusing on one or more of the following aspects as a starting point: - Various deployment scenarios (e.g., UMa, UMi, InH) - Various outdoor/indoor UE distributions for UMa/UMi (e.g., 10:0, 8:2, 5:5, 2:8, 0:10) - Various carrier frequencies (e.g., 2GHz, 3.5GHz) - Other aspects of scenarios are not precluded, e.g., various antenna spacing, various antenna virtualization (TxRU mapping), various ISDs, various UE speeds, etc. Companies to report the selected scenarios for generalization verification

To verify the generalization performance of various deployment scenarios, we use the model trained in the dense urban (Macro only) scenario to compare the inference performance in UMa (Urban Macro) and UMi (Urban Micro) with Rel-16 eType II codebook. The performance comparison results of SGCS are shown in Figure 3.

Figure 3. The verification of generalization with various deployment scenarios

According to figure 3, in both UMa and UMi scenarios, the performance of AI based CSI compression and recovery trained under the dense urban (Macro only) scenario is also better than that of Rel-16 eType II codebook. The performance of AI based CSI compression and recovery under UMa scenario is better than that under UMi scenario, since UMa scenario is similar to dense urban (Macro only) scenario.

Observation 2: The AI based CSI compression and recovery trained under different scenarios can also achieve better SGCS performance than Rel-16 eType II codebook.

To verify the performance of various outdoor/indoor UE distributions, we use the models trained in the UMa scenario with  different outdoor/indoor UE distributions to compare the inference performance in the UMa scenario with 8:2 outdoor/indoor UE distributions. The performance comparison results of SGCS are shown in table 4.

Table 4. The verification of generalization performance with various outdoor/indoor UE distributions

| SGCS Feedback bits | Training UMa_2:8 Inference UMa_8:2 | Training UMa_8:2 Inference UMa_8:2 |
| --- | --- | --- |
| 48 | 0.795495 | 0.800250 |
| 72 | 0.805596 | 0.820570 |
| 88 | 0.815343 | 0.833801 |
| 132 | 0.858298 | 0.862100 |

According to Table 4, for the inference performance in the UMa scenario with 8:2 outdoor/indoor UE distributions ratio, with the increase of feedback bits, AI/ML model trained by both 8:2 indoor/outdoor UE distribution can achieve 2% gain than 2:8 indoor/outdoor UE distribution.

Observation 3: For the inference performance in the UMa scenario with 8:2 outdoor/indoor UE distributions ratio, AI/ML model trained by 8:2 and 2:8 indoor/outdoor UE distribution ratio shows similar performance.

Evaluations for CSI prediction

AI model for CSI prediction

In the previous meeting, it has been agreed that the AI based CSI prediction is as another use case. So in the section we provide the simulation about AI based CSI prediction. As in fig.4, we use the historical channel matrix during the observation window as the input to predict the channel matrix in the future slot(s). In our simulation, we use ConvLSTM, the observation window is 4/5ms, and the prediction window is 1/5ms. The CSI feedback periodicity is 5ms. The input of AI model is the channel matrix.

Figure 4. The structure of AI based CSI prediction

Evaluation results for CSI prediction

In this section, we provide the evaluation results for CSI prediction. For the data generation, we consider a Urban Micro scenario operating on 2GHz FDD spectrum, where 1140 UEs (57 cells, and 20 users per cell) are generated. The detailed evaluation assumptions can refer to the Appendix 2. We collect 114000 samples, 90% of which are used as training sets and 10% as validation sets. The nearest historical CSI w/o prediction is used as the benchmark. The comparison of SGCS for AI based CSI prediction is shown in table 5. In the benchmark, SGCS is calculated with the nearest historical CSI w/o prediction and the corresponding ground-truth channel matrix of the target future slot.

According to Table 5, AI based CSI prediction can achieve about 21.88% SGCS gain than The nearest historical CSI w/o prediction.

Table 5. The comparison of SGCS for AI based CSI prediction

|  | AI based CSI prediction | The nearest historical CSI w/o prediction |
| --- | --- | --- |
| SGCS | 0.9182 | 0.6994 |

Observation 4: The AI based CSI prediction can achieve better SGCS performance than the nearest historical CSI w/o prediction.

In the previous meeting, we have agreed the CSI prediction-specific generalization scenario of various UE speeds is supported for performing the generalization verification.

Agreement The CSI prediction-specific generalization scenario of various UE speeds (e.g., 10km/h, 30km/h, 60km/h, 120km/h, etc.) is added to the list of scenarios for performing the generalization verification. - FFS various frequency PRBs (e.g., trained based on one set of PRBs, inference on the same/different set of PRBs)

In table 6, we use the models trained with UE speed 10km/h, 30km/h and 60km/h separately to compare the inference performance with different UE speeds. We also use the model trained with the data of mixed UE speeds to compare the inference performance with different UE speeds.

Table 6. The verification of generalization performance with various UE speeds

|  | Inference: 10km/h | Inference: 30km/h | Inference: 60km/h |
| --- | --- | --- | --- |
| Training: 10km/h | 0.9919 (case1) | 0.7465(case2) | 0.5426(case2) |
| Training: 30km/h | 0.9804(case2) | 0.9182(case1) | 0.6484(case2) |
| Training: 60km/h | 0.9707(case2) | 0.8625(case2) | 0.8302(case1) |
| Training: 10, 30, 60km/h | 0.9816(case3) | 0.8979(case3) | 0.8137(case3) |

From table 6, for the case 3 of generalization verification, AI based CSI prediction can achieve good SGCS performance, which only have about 0.02 SGCS loss compared with the case 1 of generalization verification. For the case 2 of generalization verification, the inference performance of high speed is relatively poor using the AI model trained with lower speed data set, but the inference performance of low speed is acceptable using the AI model trained with higher speed data set.

Observation 5: For the verification of generalization performance with various UE speeds, AI based CSI prediction can achieve good SGCS performance under the case 3 generalization verification.

Conclusion

In this contribution, we have the following proposal and observation:

Observation 1: The AI based CSI compression and recovery can achieve better SGCS and throughput  performance than Rel-16 eType II codebook.

Observation 2: The AI based CSI compression and recovery trained under different scenarios can also achieve better SGCS performance than Rel-16 eType II codebook.

Observation 3: For the inference performance in the UMa scenario with 8:2 outdoor/indoor UE distributions ratio, AI/ML model trained by 8:2 and 2:8 indoor/outdoor UE distribution ratio shows similar performance.

Observation 4: The AI based CSI prediction can achieve better SGCS performance than the nearest historical CSI w/o prediction.

Observation 5: For the verification of generalization performance with various UE speeds, AI based CSI prediction can achieve good SGCS performance under the case 3 generalization verification.

Reference

- RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- Appendix 1: Evaluation assumption for AI/ML based CSI feedback enhancement

Table . SLS assumptions for AI/ML based CSI feedback enhancement

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. |
| Frequency Range |  | 2GHz. |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS |
| Simulation bandwidth |  | 10 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | MU-MIMO |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Ideal channel estimation |
| Evaluation Metric |  | SGCS |
| Baseline for performance evaluation |  | Rel-16 eType II Codebook is the baseline. |

- Appendix 2: Evaluation assumption for AI/ML based CSI prediction

Table . SLS assumptions for AI/ML based CSI prediction

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Umi is a baseline. |
| Frequency Range |  | 2GHz. |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 100 for 15 kHz SCS |
| Simulation bandwidth |  | 20 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | MU-MIMO |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| UE distribution |  | - 100% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Ideal channel estimation |
| Evaluation Metric |  | SGCS |
| Baseline for performance evaluation |  | The nearest historical CSI w/o prediction |