3GPP TSG RAN WG1 #113                                                           R1-2304578

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.11.1

Source:	Spreadtrum Communications

Title:	Discussion on evaluation on low power WUS

Document for:	Discussion and decision

# Introduction

The SID of the low power WUS (LP-WUS) can be found in [1].

The study item includes the following objectives: - Identify evaluation methodology (including the use cases) & KPIs [RAN1] - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables - Other use cases are not precluded - Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4] - Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4] - Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1] - Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1] - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

The contribution focuses on the evaluation methodology and some initial evaluation results.

# Use cases

In RAN#112 [2], it was agreed to define three use cases as follows.

Agreement The following characteristics for target use cases are considered in the study item: - IoT cases including e.g., industrial wireless sensors, controllers, actuators and etc, including the following characteristics, - FFS: latency - primary for small form devices - power-sensitive - static, nomadic or limited mobility - Wearable cases including e.g., smart watches, rings, eHealth related devices, and medical monitoring devices etc., - FFS: latency - primary for small form devices, - power-sensitive - low/medium speed, FFS: high speed - eMBB cases including e.g., XR/smart glasses, smart phones and etc., - FFS: latency - devices form is various and not restricted - power-sensitive - low/medium speed, FFS: high speed Note: other use cases/characteristics are not precluded if any.

# Evaluation methodology

## Power model for the main radio

In RAN1#110bis-e [4], the power model for the main radio was widely discussed and had the good progress.

Agreement Take the following power model for main radio for evaluation in LP-WUS/WUR SI, - For IoT and wearable cases, reuse TR38.875 power model as baseline. - For eMBB and other cases, reuse TR38.840 power model as baseline. - Introduce ‘Ultra-deep sleep’ power state for main radio of UEs with LP-WUS receiver and reusing power model option 1 value of ‘Ultra-deep sleep’for LPHAP evaluation, i.e., - FFS: The details of ‘Ultra-deep sleep’ power state Agreement - The following power models are used ‘Ultra-deep sleep’ power state for main radio for evaluation Power State; Relative Power (unit); Ramp-up and down transition energy (Note1): (unit multiplied by ms); Ramp-up time; Time for sync/re-sync. Ultra-deep sleep; [0.015]; [2000 ~ 40000] - Study to converge on candidate numbers to use for evaluation - FFS: other values and reported by companies. - FFS: down-selection of the values, - companies are encouraged to provide details for down-selection; [400ms], FFS: 100ms; X. Note1: - Ramp-up time may consist of the procedure for [main radio hardware tune on e.g., boot, memory load and etc.] - Time for sync/re-sync consists of the procedure for [main radio to re-synchronization with the serving gNB etc.] - FFS: X and whether/how to have different values depending on other factors, e.g., signal-to-noise ratio. - Companies can report the assumption of X in the initial evaluation. - Ramp up and down energy includes power for ramp-up and ramp-down. Energy consumption for sync/re-sync is separately calculated. - The total time for main radio transition from ultra-deep sleep to active/micro sleep state is the sum of ramp-up time and time for sync/re-sync. - FFS whether/how to define ramp-down time, whether to separately describe the ramp-down energy consumption. Note 2: the power state transitions in this table refer to transitions between ultra deep sleep state and active / micro sleep state. Note 3: The values inside of ‘[ ]’ are to be used as starting point of future study on LP-WUS.

### Transition time/energy

It seems that the total transition energy includes the rump up/down energy and sync/re-sync energy, and the total transition time includes the rump up/down time and sync/re-sync time. For ease of understanding, we consider cell search as a part of sync/re-sync.

Ramp-up/down

In RAN1#112 [2], it was agreed there are two alternatives for transition energy and ramp-up time.

Agreement For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows, - Alt 1: (15000, 400ms) - Alt 2: ([40000], [800ms]) Company to report which alternative they use for which use cases.

In RAN1#112bis-e [3], Alt 1 is regarded as baseline, and Alt 2 is confirmed as additional alternative.

Agreement Confirm Alt 2 in the following agreement and update as follows Agreement For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows, - Alt 1: (15000, 400ms) as baseline - Alt 2: ([40000], [800ms]) Company to report which alternative they use for which use cases.

In our view, the transition energy usually means the energy gap between ultra-deep sleep and micro deep sleep, and the ramp-up time usually means the time gap between ultra-deep sleep state and micro sleep state. We think 15000 power unit and 400ms is feasible for transition energy and ramp-up time for main radio.

Cell frequency search

In general, sync/re-sync includes cell frequency search (SS-raster search), timing acquisition and frequency error correction. However, it is not clear whether cell frequency search belongs to sync/re-sync during the main radio wake-up. In our view, companies did not consider cell frequency search for discussion of sync/re-sync. We try to discuss cell frequency search initially.

Specifically, if the LP-WUR supports mobility (continuous coverage), i.e. the LP-WUR can re-select a cell in time/frequency domain during UE movement, and if the LP-WUS is deployed in the same frequency as SSB in the cell, the main radio can know the frequency of the cell to be selected via the LP-WUR. In this case, the cell frequency search is not necessary. The restriction is that the LP-WUR should support mobility and the LP-WUS should be deployed in the same frequency as SSB in the cell.

Observation 1: If the LP-WUR supports mobility (continuous coverage) and if the LP-WUS is deployed in the same frequency as SSB in the cell, the cell frequency search is not necessary at the LP-WUR after wake-up.

As counterpart, if the LP-WUR does not support mobility, i.e. the LP-WUR cannot re-select a cell during UE movement and the main radio should be switched on to re-select a cell, the main radio can know the frequency of cell to be selected. In this case, the cell frequency search is not necessary.

Observation 2: If the LP-WUR does not support mobility, the cell frequency search is not necessary at the LP-WUR after wake-up, which has been done in cell re-selection at the LP-WUR.

For simplicity, we can assume cell frequency search is not included in sync/re-sync.

Timing acquisition and frequency error correction

The main radio can only rely on periodic reference signal for timing acquisition and frequency error correction. For simple case, the periodic reference signal can be SSB. Nevertheless, the periodic reference signal can include both SSB and additional TRS (e.g. R17 defined additional TRS) for optimization purpose. In RAN1#111 [5], the number of SSBs for timing acquisition and frequency error correction is widely discussed, and only the upper bound was agreed.

Agreement For MR, at least for FR1 evaluation, - Number of SSBs for sync/re-sync for MR is up to 10 - Companies to report timeline and energy consumption - Companies to provide feasibility analysis for transition time and transition energy with aim to converge to one or two set of values in RAN1#112

Since we aim to converge to one or two set of values for the total transition time/energy in RAN1#112, we try to provide the candidate values of the number of SSBs.

In [5], additional 3 SSBs w.r.t. deep sleep is assumed for both timing acquisition and frequency error correction. In our view, the additional x SSBs can be used to acquire timing by decoding PBCH content and detecting time-index in PBCH-DMRS, and another additional y SSBs can be used for frequency error correction (from 10 ppm to 2 ppm). The value of x and y depends on different channel condition. It should be noted that 1/2/3 SSBs for wake-up from deep sleep (frequency error is from 2 ppm to 0.5ppm) are assumed in R17 PEI discussion for different channel condition respectively. Therefore, we have the following number of SSBs for different channel condition respectively and calculate sync/re-sync time/energy.

Table 1: Sync/re-sync time/energy

|  | The number of SSBs | Sync/re-sync time | Sync/re-sync energy |
| --- | --- | --- | --- |
| Deep sleep of the main radio | {1, 2, 3} | {2, 22, 42}ms Assume 2ms for duration of a SSB burst and 20ms for periodicity of SSB bursts | {60*2, 60*2 + 20*18 + 100 + 60*2, (60*2 + 20*18 + 100)*2 + 60*2} = {120, 700, 1280} |
| Ultra deep sleep of the main radio in worse channel condition | 9 3 SSBs for timing acquisition, 3 SSBs for frequency error correction, 3 SSBs for fine sync for PO reception | 162ms | (60*2 + 20*18 + 100)*8 + 60*2 = 4760 |
| Ultra deep sleep of the main radio in general channel condition | 6 2 SSBs for timing acquisition, 2 SSBs for frequency error correction, 2 SSBs for fine sync for PO reception | 102 | (60*2 + 20*18 + 100)*5 + 60*2 = 3020 |
| Ultra deep sleep of the main radio in good channel condition | 3 1 SSB for timing acquisition, 1 SSB for frequency error correction, 1 SSB for fine sync for PO reception | 42ms | (60*2 + 20*18 + 100)*2 + 60*2 = 1280 |

Proposal 1: 3/6/9 SSBs for sync/re-sync for ultra-deep sleep can be assumed for different channel condition respectively.

## Power model for LP-WUR

In RAN1#110bis-e [3], the power model for LP-WUR (including power values for LP-WUR on/off state and transition time/energy) was also widely discussed. The following agreement was achieved.

Agreement The following power model for LP-WUR/WUS evaluation is considered, - Relative power unit for LP-WUR ‘off’ state, i.e., the LP-WUR does not perform monitoring: - [0.001] - Relative power unit for LP-WUR ‘on’ state, i.e., the LP-WUR performs monitoring: - [0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4] - Other values are not precluded to be evaluated - FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation - No additional transition energy and transition time between ‘on’ and ‘off’ state as start point, FFS any transition energy and transition time if needed. Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver. Note2: the values provided is for the purpose of studying power saving gain, and the values can be further revisit and categorization depending on the receiver architecture discussion. Note3: For LP-WUR ‘on’ state, more than one values within the above range may be used for evaluation (e.g. for a single LP-WUR architecture) FFS: LP-WUR power consumption values for FR2.

In RAN1#111 [5], the power model for LP-WUR was further discussed. The following agreement was achieved.

Agreement The following power model for LP-WUR is used for evaluation for FR1, Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off; 0.001; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On; 0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4 FFS: If other values are needed. FFS: whether further categorization/sub-categorization is needed and how.

In RAN1#112bis-e [3], equation of transition energy is updated as additional transition energy based on triangle principle.

Agreement Update the additional transition energy from [TLR, ramp-up *(PON+POFF)/2] to [TLR, ramp-up *(PON-POFF)/2] for LP-WUR power model. - Note: this assumes the power consumption during the transition time is sum of additional transition energy and LP-WUR OFF energy, e.g., similar definition as the additional transition energy in TR38.840

### Power values for LP-WUR on/off state

In RAN1#112bis-e, the power values of LP-WUR on/off state was revisited, since some high-complexity receiver type/components were identified.

#8: [H] Proposal 1C-2-v12: - For LP-WUR power evaluation, - Relative Power (unit) for LP-WUR OFF state, - 0.001/ Y1/ [Y2], [FFS value(s) of Y1, Y2 e.g., 0.1, 0.01, 0.005] - Relative Power (unit) for LP-WUR ON state, - 0.01/0.05/0.1/0.2/0.5/1/2/4/10/20/30 - 10/20/30 for LP-WUR ON power state are not used for receiver types based on envelope detection for MC-ASK and MC-FSK, - For other values, the mapping between power value and receiver type are FFS - Note: Up to companies to report whether same or different values are assumed for WUS monitoring and time/frequency synchronization.

Power values for LP-WUR on state

Some companies thought the large power values for LP-WUR on state, e.g. 10/20/30, is dedicated for receiver of OFDMA-based signal/channel, but some other companies thought it is not related to receiver type instead it is related to components of receiver.

Observation 3: There is no common understanding whether large power values for LP-WUR on state, e.g. 10/20/30, are related to receiver type or components.

In our view, we defined the power values, e.g. 0.01/0.05/0.1/1/2/4, but we did not relate them to receiver type or components.

Observation 4: There is no common understanding how to relate the defined power values, e.g. 0.01/0.05/0.1/1/2/4, to receiver type/components.

It was common understanding that companies can reports which receiver type or components they use. Following this logic, we may not relate the power values to receiver type or components explicitly.

Proposal 2: Add the power values 10/20/30 for LP-WUR on state without explicit relationship between them and receiver type or components.

Proposal 3: Add clarification that companies are encouraged to report assumptions on receiver type/components for the selected power values for LP-WUR on state.

Furthermore, it is common understanding that oscillator (option 1/2/3/4) is used for LP-WUR on state, so the difference of power values may come from other components than RTC/oscillator, e.g. PLL/FLL. As mentioned by some companies, usage of PLL or FLL is also factor for carrier frequency error. It means 200 ppm for oscillator may cause too large carrier frequency error, e.g. 500 ppm. Even for OOK waveform, carrier frequency error will affect detection performance due to down-convertor in IF/zero-IF architectures.

Observation 5: Beside usage of oscillator, usage of PLL or FLL will affect the power value for LP-WUR on state and detection performance.

Power values for LP-WUR off state

Some companies thought the power value for LP-WUR off state for receiver for OFDMA-based signal/channel should be large than 0.001. For example, receiver for OFDMA-based signal/channel may use high-complexity oscillator. To be more general, it was common understanding to relate the power value for LP-WUR off state to usage of oscillator [3].

Working Assumption The following for usage of the clock is assumed for LP-WUR OFF/ON Assumption on LP-WUR OFF power; Assumptions on the clock usage. 0.001; When LP-WUR is OFF - Time offset cumulated in the off period cannot be calculated based on the parameters of the oscillator option 1/2/3/4. RTC should be used(Only RTC is running during sleep.) When LP-WUR is ON, frequency offset and time offset calculation can follow the parameters of the oscillator option 1/2/3/4 [Note2] (cumulating based on the frequency drift and not exceed maximum frequency error) - The initial frequency offset when LP-WUR switches on can be set to the [FFS: maximum frequency error or a random value within the maximum frequency error] following the parameters of the oscillator option 1/2/3/4[Note2]. - When LP-WUR is synced with LP-SS/SSB or MR is used to assist to calibrate LP-WUR to correct the time/frequency error, residual frequency error Fr is assumed at the time when the synchronization/calibration is done.. TBD: value(s); For both LP-WUR OFF and ON - Time offset cumulated in the off period can be calculated based on the parameter of the oscillator option 1/2 or option 3/4[Note2]. RTC can be used too. - Frequency offset calculation can follow the parameter of the oscillator option 1/2 or option 3/4[Note2] (cumulating based on the second value in the value pair and not exceed maximum frequency error). When at the time point after LP-WUR is synced with LP-SS/SSB or if MR can assist to calibrate LP-WUR to correct the frequency error - Frequency offset is the Fr, which is residual frequency error from previous synchronization/calibration. [Note1: Any additional LO/FLL/PLL could start running during LP-WUR On duration. The power consumption of any of those LO/FLL/PLL is captured in LP-WUR On power] FFS: Note2: option 3/4 can only be assumed when LP-WUR ON power value and LP-WUR OFF power value>=TBD2, option 1/2 can only be assumed when LP-WUR ON power value and LP-WUR OFF power value>=TBD1 Note3: The clock error (of both RTC and LO) could be improved to be less than max ppm error of option 1,2,3,4 with clock calibation based on sync signal such as LP-SS or preamble.

Our understanding frequency/time error due to drift according to above working assumption is listed as follows.

  - Case 1: The power value is 0.001 for LP-WUR off state

    - For LP-WUR off state, it is assumed that only RTC is used, and frequency/time error do not follow the oscillator option 1/2/3/4. For LP-WUR on state, it is assumed that oscillator is used, and frequency/time error follow the oscillator option 1/2/3/4.

      - Initial frequency/time error when LP-WUR switches on is maximum frequency/time error or a random value not greater than maximum frequency/time error, since only RTC is used in off sate

        - Initial frequency/time error when LP-WUR switches on is usually large

      - After correction by current sync signal, residual frequency/time error is a new value after correction

  - Case 2: The power value is much higher than 0.001 for LP-WUR off state

    - For LP-WUR on/off state, it is assumed that oscillator is used, and frequency/time error follow the oscillator option 1/2/3/4.

      - Initial frequency error when LP-WUR switches on is Fd (ppm)=ΔF (ppm) +Fr(ppm), where ΔF is oscillator clock frequency drifting during time between previous sync signal and current sync signal, and Fr is residual frequency error from previous correction

        - Initial frequency error when LP-WUR switches on is usually small

      - Initial time error when LP-WUR switches on is Te= ΔT+ Tr, where ΔT is oscillator clock time drift during time between previous sync signal and current sync signal, and Tr is residual time error from previous correction

        - Initial time error when LP-WUR switches on is usually small

      - After correction by sync signal, residual frequency/time error is a new value after correction

It can be observed that for Case 1 (i.e. the power value is 0.001 for LP-WUR off state), initial frequency/time error when LP-WUR switches on is usually large, and residual frequency/time error is usually large. Therefore, the corresponding monitoring window for duty-cycle based LP-WUS monitoring should be wide enough.

Observation 6: For Case 1 (i.e. the power value is 0.001 for LP-WUR off state), initial frequency/time error when LP-WUR switches on is usually large, and residual frequency/time error is usually large.

Summary

According to above discussion, we prefer to have 3 categories for power values for LP-WUR on/off state. The first category means Case 1. The second category means low-complexity architecture (e.g. with FLL) in Case 2. The third category means high-complexity architecture (e.g. with PLL) in Case 2.

Table 2: The relative power values for the LP-WUR

|  | Relative power (unit) | Note |
| --- | --- | --- |
| Category 1 (Case 1) | Off state: 0.001 | RTC |
|  | On state: 0.01/0.05/0.1/0.2/0.5 | Oscillator, FLL Residual frequency/time error is large Example: envelope detection architecture |
| Category 2 (low-complexity architecture in Case 2) | Off state: 0.01 | Oscillator |
|  | On state: 1/2/4 | Oscillator, FLL Residual frequency/time error is medium Example: Parallel envelope detection architecture |
| Category 3 (high-complexity architecture in Case 2) | Off state: 0.1 | Oscillator |
|  | On state: 10/20/30 | Oscillator, PLL Residual frequency/time error is medium Example: OFDMA-bases signal/channel base (I/Q two branches) architecture |

Proposal 4: The power values for LP-WUR on/off state can be modelled in 3 categories.

### Transition time/energy

From above table, we assume ramp-up time as follows.

  - Category 1: 1ms

  - Category 2: 5ms

  - Category 3: 20ms

Some companies also have argument that compared to main radio (taking RedCap as example) the ramp-up time 20ms for OFDMA-based signal/channel based architecture seems too small. In our view, OFDMA-based signal/channel based LP-WUR does not need to process PBCH/PDCCH/PDSCH, so the ADC and baseband processing is simpler. Furthermore, as mentioned by some companies, low-complexity oscillator/PLL are used which also needs small ramp-up time.

Observation 7: The ramp-up time for OFDMA signal/channel based LP-WUR has lower power consumption compared to main radio (e.g. RedCap) due to low complexity of oscillator, PLL, ADC and baseband processor.

Proposal 5: The ramp-up time for LP-WUR can be modelled in 3 categories, e.g. 1ms, 5ms and 20ms.

## Performance metrics

In RAN1#110bis-e [4], the potential performance metrics were listed as follows.

Agreement For system impact analysis, the following performance metrics are considered to be provided, Performance Metric; Note. System overhead; expressed as percentage of used part of all REs for LP-WUS (including guard band or time or others resource used for LP-WUR if any) among all resources Other assumptions related to the system overhead analysis can be reported, e.g., the LP-WUR raw data rate evaluated in the coverage evaluations.. FFS: Capacity impact; [Evaluate the system capacity impact due to introducing of LP-WUS]. FFS: NW power consumption / Energy Efficiency; [Impact of LP-WUS/WUR operation on gNB energy consumption as performance metric in system impact analysis.]. For power and latency evaluation of the LP-WUS, the following performance metrics are considered to be provided. Performance Metric; Note. Power consumption; Relative power consumption in units. The power consumption includes main radio and LP-WUR. For comparison, the relative power consumption and evaluation period for baseline schemes should also be provided, as well as the power saving gain (i.e., percentage of power consumption reduction of the proposed power saving scheme from the baseline scheme).. Latency; For IDLE/INACTIVE state, the latency is the time interval between the data arrival time at the gNB and the time of the first PO UE can [monitor/detect] the paging message - FFS: if UE is not required to monitor a PO after wake-up, e.g., latency is the time interval between the data arrival time at the gNB and the time UE transmits the PRACH after LP-WUS detection. - sync/re-sync for main radio is included For CONNECTED state, TBD. FFS: UPT; FFS Note: it is for connected mode purpose.. Companies to report baseline scheme, e.g., PO monitoring with i-DRX, e-DRX, with or without PEI Companies to report the power consumption / power saving gain considering the FAR impact , latency considering MDR impact Other performance metrics (e.g., mobility) can be reported by companies (if any)

In RAN1#111 [4], the potential performance metrics were updated as follows.

Update the agreement in RAN1#110bis-E as follows, For system impact analysis, the following performance metrics are considered to be provided, Performance Metric; Note. System overhead; expressed as percentage of used part of all REs for LP-WUS (including guard band or time or others resource used for LP-WUR if any) among all resources Other assumptions related to the system overhead analysis can be reported, e.g., the LP-WUR raw data rate evaluated in the coverage evaluations.. Capacity impact; [Evaluate the system capacity impact due to introducing of LP-WUS] Note: it is for UEs which are in connected mode. Definition is the same as in XR TR.. FFS: NW power consumption / Energy Efficiency; [Impact of LP-WUS/WUR operation on gNB energy consumption as performance metric in system impact analysis.]. For power and latency evaluation of the LP-WUS, the following performance metrics definitions provided for future study Performance Metric; Note. Power consumption; Relative power consumption in units. The power consumption includes main radio and LP-WUR. For comparison, the relative power consumption and evaluation period for baseline schemes should also be provided, as well as the power saving gain (i.e., percentage of power consumption reduction of the proposed power saving scheme from the baseline scheme).. Latency; For IDLE/INACTIVE state, - the latency is the time interval between the data arrival time at the gNB and the time of the first PO UE can [monitor/detect] the paging message - alternatively, if UE is not required to monitor a PO after wake-up, company to report detailed procedure and definition of the latency . In RAN1#111, there are no definitions being precluded - sync/re-sync for main radio is included. FFS: UPT; FFS The definition is the same as in [TR38.840] Note: it is for connected mode purpose.. Companies to report baseline scheme, e.g., PO monitoring with i-DRX, e-DRX, with or without PEI Companies to report the power consumption / power saving gain considering the FAR impact, latency considering MDR impact Other performance metrics (e.g., mobility) can be reported by companies (if any)

We will focus on RRC IDLE/INACTIVE state and discuss the system overhead, power consumption and latency.

## Traffic model

In RAN1#111 [5], the traffic model option 1 was updated as follows.

Agreement Update the IDLE/INACTIVE state traffic model option 1 as follows and remove traffic model option 2, - The traffic arrival is modeled as a Poisson Arrival Process where inter-arrival times are exponentially distributed, the mean arrival time is P = YREF / RE, REF, where - RE, REF= 1%, 0.1%, 0.01% or 0.001% and YREF = 1.28s - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group - FFS: Value of N - For LP-WUS - Both per group and UE paging can be assumed. Note： - For i-DRX with i-DRX cycle duration Y second, - Per UE paging probability RE = 1 – (1 – RE, REF )Y/YREF - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group - For e-DRX with K i-DRX cycles duration, L PTW duration of L i-DRX cycles, and an i-DRX cycle duration Y second - Per UE paging probability is - RE = 1 – (1 – RE, REF )(K-L)Y/YREF for the first i-DRX cycle within the PTW - RE = 1 – (1 – RE, REF )LY/YREF for each of the remaining L-1 i-DRX cycles within the PTW - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group - L=4 (as agreed in RAN1#110bis)

For RRC IDLE/INACTIVE state, if the LP-WUS contains the whole UE ID, the LP-WUS can wake up a UE, and per group paging probability RG is equal to per UE paging probability RE. If the LP-WUS contains a part of UE ID, the LP-WUS can wake up a group of UEs, and per group paging probability RG is much larger than per UE paging probability RE, since value of N is much larger than 1.

However, how to relate the bits number of UE ID in the LP-WUS to value of N is not simple, and value of N is also related to UE distribution in a cell. We may only use a typical value of N. For example, we can choose 10 as a value of N like discussion of R17 PEI. We should keep in mind that a small bits number of UE ID in the LP-WUS may imply a large value of N in realistic.

Observation 8: The bits number of UE ID in the LP-WUS is related to value of N, and a small bits number of UE ID in the LP-WUS may imply a large value of N in realistic.

Therefore, companies can report the value of N in evaluation.

Proposal 6: Companies can report the value of N for per group paging probability in evaluation.

# System overhead

## Comparing R17 PEI and the LP-WUS

According to that of R17 PEI (Appendix A.4), parameters for R17 PEI and the LP-WUS are listed as follows.

Table 3: Parameters for R17 PEI and the LP-WUS

|  | R17 PEI | The LP-WUS |
| --- | --- | --- |
| General parameters | Behavior A + channel based |  |
| The required MDR | 0.1% To meet the joint MDR of PEI and paging PDCCH, which is 1%. In this case, MDR of paging PDCCH approaches 1% | 1% |
| The required FAR | 1% It is mainly used for sequence-based PEI. For channel-based PEI, the actual FAR is much lower due to long CRC | 0.1% or 1% For channel-based preamble and data payload, it can be satisfied by using more than10 bits CRC |
| The required SINR | SINR when MDR of paging PDCCH is 1% (assuming the same coverage of R17 PEI and the LP-WUS) |  |
| Paging PDCCH reference | AL 8, 41-bit payload |  |
| Paging PDSCH reference | MCS0, TB scaling 1.0 It may imply the required CFO for PO reception | N/A |

It can be observed that the LP-WUS can use R17 PEI as reference in terms of the required MDR/FAR and SINR.  Therefore, we can also estimate the system overhead of the LP-WUS roughly by comparing to R17 PEI.

We provide our estimated values for the loss and gain of the LP-WUS compared to R17 PEI as follows like those reported by [5]. For simplicity, we focus on MC-ASK, i.e. OOK in short.

Table 4: The loss and gain of the LP-WUS (MC-ASK/FSK based) compared to R17 PEI

|  | Value of loss or gain | Note |
| --- | --- | --- |
| Rx number | ~3dB loss | Assume 2 Rx for R17 PEI and 1 Rx for the LP-WUS |
| Noise figure | ~8dB loss | Due to lower complexity of the LP-WUR architecture [5] |
| Over sampling (compared to modulation symbol rate) | ~5dB gain | Assuming 4 samples per modulation symbol (i.e. OOK symbol) for the LP-WUS and 1.7 samples per modulation symbol for R17 PEI* |
| Bit-width of ADC | ~3dB loss | Assuming 4 bits width of ADC for the LP-WUR, compared to 8 bits width of ADC for R17 PEI |
| Channel coding scheme | ~4dB loss | Polar code has gain compared to Manchester code |
| Waveform | ~3dB gain | OOK is more robust than QPSK+OFDM, since QPSK+OFDM is sensitive to frequency error and phase noise, in which ICI will cause degradation |
| Coverage shrinkage | ~x dB gain | For discontinuous coverage, link budget of the LP-WUS is x dB shrinkage than that of R17 PEI. In this case, the main radio is always switched on at the cell edge |

*Note 1: For OFDM waveform, sampling rate is usually subcarrier spacing (OFDM symbol rate) times the FFT size. Roughly, sampling rate is about 1.7x compared to rate of modulation symbol (a subcarrier), since FFT size is 2048 but the number of useful subcarriers is 1200 in typical case. On the other hand, how many REs in a modulation symbol for the LP-WUS (e.g. 6 REs in an OOK symbol) is not included in this loss/gain comparison. The number of REs in a modulation symbol for the LP-WUS can be regarded as a part of waveform design or coverage enhancement for the LP-WUS.

It can be observed that the LP-WUS may have about (10-x) dB performance loss compared to R17 PEI, where x is value of coverage shrinkage compared to R17 PEI.

Observation 9: The LP-WUS may have about (10-x) dB performance loss compared to R17 PEI, where x is value of coverage shrinkage compared to R17 PEI.

From above two tables, the resource overhead for the LP-WUS can be estimated according to that of R17 PEI (Appendix A.4).

Table 5: Resource overhead for R17 PEI and the LP-WUS

|  | R17 PEI |  | The LP-WUS |  |
| --- | --- | --- | --- | --- |
| Information bits | 12 bits | 41 bits | 12 bits: a small part of 48-bit for ng-5G-S-TMSI. The main radio should monitor PO after wake-up | 41 bits: the main part of 48-bit for ng-5G-S-TMSI). The main radio may not monitor PO after wake-up, if the remaining bits for ng-5G-S-TMSI is carried by location of the LP-WUS occasion, like PO location which carries some bits of UE ID |
| Occupying REs | 288 (576 may be also feasible since R17 PEI has lower MDR than paging PDCCH) | 576 | 288*y (10-x) dB loss => y times of REs compared to R17 PEI | 576*y (10-x) dB loss => y times of REs compared to R17 PEI |

It can be observed that the resource overhead of the LP-WUS is much larger than that of R17 PEI. For example, when y=5 (i.e. 5dB coverage shrinkage compared to R17 PEI), the LP-WUS may need 288*4 or 576*4 REs for 12 or 41 bits respectively.

Observation 10: System overhead of the LP-WUS is much larger than that of R17 PEI.

It seems common understanding, since in RAN1#112bis-e [3], it was agreement to further study coverage enhancement for the LP-WUS.

Agreement - Study techniques/mechanisms to enhance coverage performance of LP-WUS - Study potential gains available as well as drawback(s) of the technique(s)/mechanisms(s), e.g. system overhead, increased complexity network energy consumption etc… - Study potential issues and corresponding solutions for the case when LP-WUS coverage is insufficient - At least study fallback mechanisms where the Main Radio switches to legacy operation in case the channel condition of LP-WUS is not sufficient, e.g. below threshold.

# Power saving gain

According to power models and evaluation assumptions, Power Saving Gain (PSG) can be achieved by numerical analysis or system evaluation.

## The preferred evaluation assumptions

For evaluation assumptions with multiple options, we select the preferred options.

- For the LP-WUS

  - Power model

    - Power value for LP-WUR off state is 0.001

    - Power value for LP-WUR on state is 0.1

    - Transition time is 1ms

  - Duration of the LP-WUS burst is 2ms:  The LP-WUS is like SSB supporting beam sweeping

  - The LP-WUS is periodically detected by the LP-WUR, and the periodicity is paging cycle for I-DRX

- For the main radio

  - Power model is for eMBB and other cases, i.e. reusing TR38.840 power model as baseline

  - The number of SSBs to process is under assumption of the worse channel condition is worse

- For traffic model

  - Per UE paging probability is 1% and N is 10, and thus per group paging probability is 10%

## Baseline scheme

R17 PEI can be baseline scheme, which is more power efficient than R15 PO monitoring.

Figure 1: Illustration of baseline scheme, i.e. R17 PEI

### RRM measurement

To be fair, the neighboring-cell measurement relaxation is assumed for R17 PEI, i.e. there is no intra-frequency or inter-frequency measurement during the evaluation time.

It seems fair to consider the serving-cell measurement relaxation for R17 PEI, but the serving-cell measurement is included in SSB processing for sync before PEI in each paging cycle, and thus there seems no additional power saving for the serving-cell measurement relaxation. On the contrary, we think the serving-cell measurement can be supported by the LP-WUS, so there could be fair comparison.

Observation 11: For R17 PEI as baseline scheme, the serving-cell measurement relaxation is not considered.

### Power consumption

For DRX cycle 1.28s, UE processing timeline can be found in Appendix A.6. Total power consumption is about 2502.

## LP-WUS scheme

There could be two schemes for processing timeline for the LP-WUS.

For Scheme-1, the main radio does not need to monitor PO after wake-up. The latency can be small, but the resource overhead may be large since the LP-WUS may contain one or more entire UE ID, which could be 48 bits for ng-5G-S-TMSI.

For Scheme-2, the main radio still needs to monitors PO after wake-up, it is a little like R17 PEI, and balance between the latency and the resource overhead can be balanced.

In fact, Scheme-1 and Scheme-2 can be combined if we can set PO as 0 (i.e. no paging being monitored) for Scheme-2. Moreover, power consumption after being paged (i.e. PRACH and the following procedures) is not counted in R17 PEI evaluation. Hence, we can start from Scheme-2 for evaluation.

Figure 2: Illustration of Scheme-2

### RRM measurement

It should be noted that baseline scheme still includes serving-cell measurement. Fortunately, whether the LP-WUR supports the serving cell measurement is under discussion. With function of serving-cell measurement, the power consumption can be compared between baseline scheme and LP-WUS scheme. In this sense, both R17 PEI with (neighboring-cell measurement relaxation) and the LP-WUS can be used in the low-mobility or stationary scenarios at least.

### Power consumption

For DRX cycle 1.28s, UE processing timeline can be found in Appendix A.7. Total power consumption is about 1990. The power saving gain is about 20%.

# Latency

For discontinuously (periodically) monitoring the LP-WUS, the latency depends on the periodicity of monitoring. For continuous monitoring the LP-WUS, the latency could be reduced with cost of power consumption. Discontinuously monitoring and continuous monitoring can be used in different use cases for different latency requirement. In fact, we do not expect both power consumption and latency can be improved obviously in a single use case for WAN. For LAN, especially small house, it can be true, which is proved by commercial use, e.g. Bluetooth or power-saving WiFi. Coverage and mobility are always difficulties for WAN.

Proposal 7: The LP-WUS can be applied for different case of latency to handle tradeoff between power saving gain and latency.

# Conclusion

We have the following proposals.

Proposal 1: 3/6/9 SSBs for sync/re-sync for ultra-deep sleep can be assumed for different channel condition respectively.

Proposal 2: Add the power values 10/20/30 for LP-WUR on state without explicit relationship between them and receiver type or components.

Proposal 3: Add clarification that companies are encouraged to report assumptions on receiver type/components for the selected power values for LP-WUR on state.

Proposal 4: The power values for LP-WUR on/off state can be modelled in 3 categories.

Proposal 5: The ramp-up time for LP-WUR can be modelled in 3 categories, e.g. 1ms, 5ms and 20ms.

Proposal 6: Companies can report the value of N for per group paging probability in evaluation.

Proposal 7: The LP-WUS can be applied for different case of latency to handle tradeoff between power saving gain and latency.

# Reference

- RP-222644, “Revised SID: Study on low-power Wake-up Signal and Receiver for NR”, vivo, RAN#97e, Sep.  12th - 16th, 2022.

- 3GPP RAN1#112 Chairman’s Notes, Feb. 27th – Mar. 3rd, 2022.

- 3GPP RAN1#112bis-e Chairman’s Notes, Apr. 17th - 26th, 2022.

- 3GPP RAN1#110bis-e Chairman’s Notes, Oct. 10th - 19th, 2022.

- 3GPP RAN1#111 Chairman’s Notes, Nov. 14th - 18th, 2022.

- R1-2209503, “Low power WUS receiver architectures”, MediaTek, RAN1#110bis-e, Oct. 10th - 19th, 2022.

# Appendix

## A.1  Agreement for baseline assumptions of system level simulation

In RAN1#110bis-e [4], baseline assumptions of system level simulation was agreed as follows.

Agreement The following is assumed for RRC IDLE/INACTIVE evaluation, Parameters; Value. i-DRX cycle length; 1.28s and other values not precluded and reported by companies, consider both with PEI/ without PEI. e-DRX cycle length; 20.48s, 61.44s and other values not precluded, company to report which value(s) are used. Note: ‘ultra-deep sleep’ state can be assumed for eDRX whenever necessary for baseline UE. Number of POs in Paging Frame; 1. Number of DRXs per PTW; 4. Number of SSB before PO / PEI; 1, 2 or 3, (used for e.g., AGC adjustment, T/F tracking, serving cell and intra-F measurement) company to report which value(s) are used Note: the assumptions is for MR wakes from ‘Deep sleep’. Sync/re-sync after ultra-deep sleep; companies to report the timeline of sync/re-sync and X value, X is the time for sync/re-sync. RRM Measurement; Company to report whether and how the RRM measurement is assumed, e.g., whether RRM performed by main radio or LP-WUR, whether RRM is relaxed or not.. LP-WUS monitoring; Option 1: continuously monitoring Option 2: discontinuously monitoring, with [T] ms as the period for complete an on-and-off cycle, and [D] ms as the active time for monitoring LP-WUS every cycle.. Traffic; Option 1 (baseline): Per UE paging rate (R_E)= ([1%]) or ([0.1%]) or ([0.01%]) or ([0.001%]) within duration Y, [FFS Y is an i-DRX cycle length or an absolute time duration length] - R_G denotes as the group paging rate and R_E denotes as UE paging rate, and 1-R_G=(1-R_E)^N, where N is the number of UEs in the group, and N is [TBD] - FFS: how (R_G, R_E) for e-DRX derived from FFS: Option 2 (optional): Reusing TR 38.875 heart beat traffic model Model; FTP3. Packet size; 100 Bytes. Mean inter-arrival time; 60s (per UE paging rate≈2%). Model RRC connection phase power consumption as follows, RRC connection duration; [30ms]. Relative energy consumption of RRC connection block (Relative power x ms); [=3000]. Other options are not precluded can be reported by companies.. Others; Reported by companies.

## A.2  Agreement for baseline assumptions of link level simulation

In RAN1#112 [2], baseline assumptions of link level simulation was agreed as follows.

Agreement For link-level simulation of LP-WUS, the following table is used as starting point, - FFS for other assumptions if any - Note: The assumptions are not intended to limit the scope of the study or the design. Table XX. Simulation assumptions for LP-WUS Attributes; Assumptions. Carrier Frequency; 2.6GHz/4GHz/700MHz. Waveform; OOK , FSK , OFDM Company to report which option for OOK /FSK /OFDM is used. Channel structure; - Option 1: Sync signal /sequence+ payload + CRC, - Option 2: Sequence only, - Option 3: Payload+CRC, - Other options are not precluded - Company to report the sequence length, payload size, CRC length (may or may not be presence).. SCS of OFDM generator for NR signal; 30kHz/15KHz. Configuration for LP-WUS signal; For OOK/FSK waveform, - Option 1a: M=1 and SCSs = 15kHz (same as NR signal) - Option 1b: M=1 and SCSs = 30kHz (same as NR signal) - Option 2a: M =2/4/8 for SCS = 15KHz (same as NR signal) - Option 2b: M =2/4/8 for SCS = 30 kHz (same as NR signal) - Option 3: M=1 and SCSs = 60kHz/120kHz/240kHz - Note: M is referred to the definition of “M” in the agreements for OOK-1/2/3/4 and FSK-1/2 For OFDM: FFS, e.g., ZC sequence Other options are up to companies to report. WUS duration; Number of OFDM symbols: e.g., 1,2,4, 8, 16,24 symbols. MDR/FAR assumption; - The miss-detection rate (MDR) of LP-WUS 1%, - The false-alarm rate (FAR) of LP-WUS - [0.1%, 1%] - Other values are not precluded for studying, reported by companies - Further discuss on the following alternatives for FAR target - Alt 1: FAR target is determined per single WUS attempt/trial, - Alt 2: FAR target is determined across a reference time duration of one or multiple WUS attempts/trials - FFS: possible values for reference time durations - Companies to report details, e.g., receiver behaviour, how to compute MDR, detection threshold - Companies to report the selected reference time duration values and the associated number of WUS attempts/trials. Code scheme; Companies to report, if any, the coding scheme (e.g., Manchester code or any other schemes) and the code rate (e.g., 1/2, 1/4, ….). gNB Channel BW; 20MHz, FFS other values. LP-WUS BW; Option 1: - 5MHz including subcarriers for guard band - 4.32MHz (i.e.,12 RBs) for LP-WUS transmission for 30kHz SCS Option 2: - {2.16, 4.32} MHz including subcarriers for guard band - 1.44MHz, 2.88MHz (i.e.{4, 8} RBs) for LP-WUS transmission for 30kHz SCS FFS: other options are up to companies to report GB is symmetrically placed on each side of LP-WUS. Filter; X-th Order filter (e.g. Butterworth, Chebyshev, …) with Y MHz bandwidth, - X = {3, 5} - Companies to report Y Companies to report any other assumptions if needed. Adjacent subcarrier interference; - PDSCH mapped on resources other than that for WUS and guard band; EPRE of LP-WUS / EPRE of PDSCH =ρ, where ρ=0 dB as baseline, ρ= {3, 6} dB as optional. Sampling Rate; - Companies to report.. ADC bit width; 1-bit, 4-bit, 8-bit, ideal and other options are not precluded. Channel Model; See link coverage assumption table (will copy and paste here). Impairment modelling; - FFS: Frequency and time error model - Phase noise up to company report, e.g. the modelling used for 802.11ba - Other cell interference is up to company to report.

## A.3  Agreement for baseline assumptions of coverage evaluation

In RAN1#110bis-e [4], coverage in terms of link budget was discussed and some assumptions were achieved.

Agreement For evaluation of the coverage of LP-WUS, the methodology and assumptions in R17 CovEnh SI (described in TR38.830) is reused as baseline. - MIL is used as the metric for LP-WUS coverage evaluation - urban (2.6GHz/4GHz), rural(700MHz) scenario for FR1 are considered to be evaluated, others (e.g., FR2) are not precluded. Note: For IoT/wearables devices, refer to R17 Redcap SI TR38.875 if the assumptions differ from TR38.830. Companies report any other assumptions which differ from the TR38.875/ TR38.830, e.g., Tx and Rx loss Companies are encouraged to compare LP-WUS with at least PDCCH for paging, PUSCH, others are not precluded. FFS: Target coverage of LP-WUS

In RAN1#112 [2], coverage evaluation assumptions were agreed.

Agreement For coverage evaluation, the following is used, Number of RX chains at the UE’s MR antenna elements for UE; Case 1: 1 Rx for Redcap Case 2: 2 Rx Case 3: 4 Rx Company to report which case is being used. Further decision on antenna assumption for coverage is FFS.. Number of RX chains antenna elements for LP-WUR; 1 Rx Note: agreed in RAN1#110bis. Scenario and frequency; Urban: 4GHz (TDD), 2.6GHz (TDD) Rural: 4GHz (TDD), 2.6GHz (TDD), 2GHz (FDD), 700MHz (FDD) Rural with long distance: 700MHz (FDD), 4GHz (TDD). Reference data rates for MR eMBB; Urban: PDSCH 10Mbps, PUSCH 1Mbps Rural: PDSCH 1Mbps, PUSCH 100kbps Rural with long distance: DL 1Mbps, UL 100kbps, 30kbps (optional). Reference PDCCH configuration; SCS; 30kHz for TDD, 15kHz for FDD.. Aggregation level; 8, 16 Company to report which case is being used. Further decision on aggregation level for coverage is FFS.. Payload; 40 bits. CORESET size; 2 symbols, 48 PRBs. Tx Diversity; Reported by companies. BLER; 1% BLER,.. Pathloss model (select from LoS or NLoS); Urban: NloS Rural: NloS and LoS. Bandwidth; 100MHz for 4GHz and 2.6GHz. 20MHz for 2GHz (FDD) 20MHz (optional for 10MHz) for 700MHz. (FDD). Channel model for link-level simulation; TDL-C for NLOS, TDL-D for LOS.. Delay spread; Urban: 300ns, optional: 1000ns and companies to provide descriptions for such scenarios Rural: 300ns Rural with long distance: 30ns. UE velocity; Urban: 3km/h Rural: 3km/h, FFS: 120km/h (optional 30km/h) for outdoor. Number of antenna elements for BS; - Urban: 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) - Rural: 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) - Rural: 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1). Number of TxRUs for BS; gNB architectures to study: - 2 or 4 TXRUs for 2GHz, 700 MHz - 64TxRUs for 2.6 and 4 GHz. - Optional: 32 TXRUs at 2 GHz gNB modeling in LLS for TDL: - Option 1: 2 or 4 gNB RF chains in LLS. - Option 2 (Optional): Number of gNB RF chains = number of TXRUs in LLS. - Companies can report if and how correlation is modelled.. Note: The descriptions above does not change the agreements for coverage in the RAN1#110-bis.

## A.5  Agreement for PEI overhead in RAN1#104b-e (only Behavior A listed)

Agreement:

Observation 1a:

For the evaluation and comparison of PEI candidate designs, the following observations for coexistence with legacy PDSCH are identified:

- For coexistence with legacy PDSCH, semi-static resouce sharing by configuring RB-symbol-level or RE-level rate-matching patterns covering PEI REs is supported for all PEI candidate designs.

- For coexistence with legacy PDSCH, dynamic resource sharing can be realized for all PEI candidates if PDSCH is scheduled by DCI format 1_1

  - For PDCCH based PEI, CORESET-level rate matching can be realized for the PDSCH as per mandatory capability

  - For SSS-based PEI, CORESET-level rate matching may be realized for the PDSCH as per mandatory capability, depending on the design of SSS-based PEI and UE capability regarding number of supported CORESETs

  - For TRS/CSI-RS based PEI, RE-level rate matching can be realized for the PDSCH as per mandatory capability

  - When PDSCH is not scheduled by DCI format 1_1, it is up to gNB implementation whether and how PEI is transmitted in PDSCH resource

Observation 2a:

For the evaluation and comparison of PEI candidate designs, the following summarize the identified configurations of PEI candidate designs, including pairs of the minimum required resource and maximum UE (sub)group indication capacity per PEI, that can comply with the mandatory performance metrics agreed in RAN1 #104-e:

- If Behv-A is assumed,

| Paging Setting | PEI candidate design | Physical-layer configuration and resource | UE (sub)group indication capcity | Number of companies providing performance results |
| --- | --- | --- | --- | --- |
| PDSCH: MCS0, TB scaling 1.0 PDCCH: AL8, 41-bit payload | PDCCH-based PEI | AL4 PDCCH with 12-bit payload, occupying 288 REs | 12 bits | 5 (HW/HiSi, OPPO, ZTE, CATT, MTK) |
|  |  | AL8 PDCCH with 12-bit payload, occupying 576 REs | 12 bits | 7 (Xiaomi, Intel, QC, Samsung, IDCC, Ericsson, vivo) |
|  |  | AL8 PDCCH with 41-bit payload, occupies 576 REs | 41 bits | 1 (CATT) |
|  | SSS-based PEI | 1-symbol SSS, occupying 132 REs (11 RB x 1 symbol) | 3 bits | 1 (IDCC) |
|  |  | 2-symbol SSS, occupying 264 REs (11 RB x 2 symbols) | 1 bit | 6 (HW/HiSi, vivo, ZTE, CATT, QC, Samsung) |
|  |  |  | 3 bits | 1 (IDCC) |
|  |  | 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols) | 4 bits | 1 (MTK) |
|  | TRS/CSI-RS-based PEI | 1-slot 24-RB TRS, occupying 144 REs (24 RB x 3 REs per RB x 2 symbols) | ≥ 8 bits | 1 (Intel) |
|  |  | 1-slot 28-RB TRS, occupying 168 REs (28 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (HW/HiSi) |
|  |  | 1-slot 36-RB TRS, occupying 216 REs (36 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (Samsung) |
|  |  | 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols) | 1 bit | 3 (vivo, ZTE, Ericsson) |
|  |  |  | 6 bits | 1 (CATT) |
|  |  | 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols) | 1 bit | 2 (OPPO, QC) |
|  |  |  | 4 bits | 1 (MTK) |
| PDSCH: MCS0, TB scaling 0.5; PDCCH: AL16, 41-bit payload | PDCCH-based PEI | AL8 PDCCH with 12-bit payload, occupying 576 REs | 12 bits | 4 (OPPO, ZTE, MTK, Intel) |
|  | SSS-based PEI | 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols) | 4 bits | 1 (MTK) |
|  | TRS/CSI-RS-based PEI | 1-slot 24-RB TRS, occupying 144 REs (24 RB x 3 REs per RB x 2 symbols) | 3 bits | 1 (Intel) |
|  |  | 1-slot 36-RB TRS, occupying 216 REs (36 RB x 3 REs per RB x 2 symbols) | 8 bits | 1 (Intel) |
|  |  | 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (OPPO) |
|  |  |  | 4 bits | 1 (MTK) |

Observation 3a:

For the evaluation and comparison of PEI candidate designs, the following summarize average resource overheads per PO for PEI candidate designs, considering the configurations identified from performance observation.

- The average overhead results are based on PO settings without impact from UE sub-grouping indication within the PO.

- Note: For comparison purpose, single-beam transmission for PEI is assumed, and results with multi-beam transmission for PEI is scaled. This doesn’t preclude any beam-forming related design for PEI.

- If Behv-A is assumed:

| Paging Setting | PEI candidate design |  | Physical-layer configuration and resource | UE (sub)group indication capacity | Number of companies providing performance results |  | Average resource overhead per PO (REs) |  |  |  | PO and PEI related assumptions | Resource sharing assumption |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDSCH: MCS0, TB scaling 1.0 PDCCH: AL8, 41-bit payload | PDCCH-based PEI |  | AL4 PDCCH with 12-bit payload, occupying 288 REs | 12 bits | 5 (HW/HiSi, OPPO, ZTE, CATT, MTK) |  | 17.2 |  | OPPO |  | 1 PEI for up to 12 PO's | PEI is transmitted as a Rel-15 PDCCH in a CORESET when a UE group is paged |
|  |  |  |  |  |  |  | 17.2 |  | ZTE |  | 1 PEI for up to 12 PO's |  |
|  |  |  |  |  |  |  | 17.6 |  | HW/HiSi |  | 1 PEI for up to 12 PO's |  |
|  |  |  |  |  |  |  | 21.8 |  | MTK |  | 1 PEI for up to 12 PO's; averaged all PO settings for 1.28-sec cycle |  |
|  |  |  |  |  |  |  | 28.8 |  | CATT |  | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 288.0 |  | CATT |  | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  |  |  | AL8 PDCCH with 12-bit payload, occupying 576 REs | 12 bits | 7 (Xiaomi, Intel, QC, Samsung, IDCC, Ericsson, vivo) |  | 49.5 |  | vivo |  | 1 PEI for 4 PO | PEI is transmitted as a Rel-15 PDCCH in a CORESET when a UE group is paged |
|  |  |  |  |  |  |  | 57.6 |  | vivo |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 57.6 |  | QC |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 57.6 |  | Samsung |  | 1 PEI for 1 PO; PEI RE# scaled w.r.t. 1-beam |  |
|  |  |  | AL8 PDCCH with 41-bit payload, occupies 576 REs | 41 bits | 1 (CATT) |  | 57.6 |  | CATT |  | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 576.0 |  | CATT |  | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  | SSS-based PEI |  | 1-symbol SSS, occupying 132 REs (11 RB x 2 symbols) | 3 bits | 1 (IDCC) |  |  |  |  |  |  |  |
|  |  |  | 2-symbol SSS, occupying 264 REs (11 RB x 2 symbols) | 1 bit | 6 (HW/HiSi, vivo, ZTE, CATT, QC, Samsung) |  | 25.4 |  | Samsung |  | 1 PEI for 1 PO; PEI RE# scaled w.r.t. 1-beam | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 25.4 |  | vivo |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 26.4 |  | ZTE |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 28.8 |  | CATT |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 28.8 |  | QC |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 254.0 |  | HW/HiSi |  | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  |  |  |  |  |  |  | 264.0 |  | ZTE |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 288.0 |  | QC |  | 1 PEI for 1 PO |  |
|  |  |  |  | 3 bits | 1 (IDCC) |  |  |  |  |  |  |  |
|  |  |  | 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols) | 4 bits | 1 (MTK) |  | 34.0 |  | MTK |  | 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 437.0 |  | MTK |  | 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle; RB-symbol rate-matching pattern period up to 40 ms | Semi-static rate-matching in PDSCH |
|  | TRS/CSI-RS-based PEI |  | 1-slot 24-RB TRS, occupying 144 REs (24 RB x 3 REs per RB x 2 symbols) | ≥ 8 bits | 1 (Intel) |  | 14.4 |  | Intel |  | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  | 1-slot 28-RB TRS, occupying 168 REs (28 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (HW/HiSi) |  | 123.4 |  | HW/HiSi |  | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 168.0 |  | HW/HiSi |  | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  |  |  | 1-slot 36-RB TRS, occupying 216 REs (36 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (Samsung) |  | 21.6 |  | Samsung |  | 1 PEI for 1 PO; PEI RE# scaled w.r.t. 1-beam | Dynamic rate-matching in PDSCH |
|  |  |  | 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols) | 1 bit | 3 (vivo, ZTE, Ericsson) |  | 28.8 |  | vivo |  | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 28.8 |  | ZTE |  | 1 PEI for 1 PO |  |
|  |  |  | 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols) | 6 bits | 1 (CATT) |  | 28.8 |  | CATT |  | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 288.0 |  | CATT |  | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  |  |  | 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols) | 1 bit | 2 (OPPO, QC) |  | 30.0 |  | OPPO |  | 1 PEI for 1 PO | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 30.0 |  | QC |  | 1 PEI for 1 PO |  |
|  |  |  |  |  |  |  | 300.0 |  | QC |  | 1 PEI for 1 PO | Semi-static rate-matching in PDSCH |
|  |  |  |  | 4 bits | 1 (MTK) |  | 26.0 |  | MTK |  | 1 PEI for up to 4 PO's | Dynamic rate-matching in PDSCH |
| PDSCH: MCS0, TB scaling 0.5; PDCCH: AL16, 41-bit payload | PDCCH-based PEI | AL8 PDCCH with 12-bit payload, occupying 576 REs |  | 12 bits |  | 4 (OPPO, ZTE, MTK, Intel) | 34.4 | OPPO |  | 1 PEI for up to 12 PO's |  | PEI is transmitted as a Rel-15 PDCCH in a CORESET when a UE group is paged |
|  |  |  |  |  |  |  | 43.6 | MTK |  | 1 PEI for up to 12 PO's; averaged all PO settings for 1.28-sec cycle |  |  |
|  |  |  |  |  |  |  | 57.6 | Intel |  | 1 PEI for 1 PO |  | PEI is transmitted as a Rel-15 PDCCH in a CORESET when a UE group is paged |
|  | SSS-based PEI | 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols) |  | 4 bits |  | 1 (MTK) | 34.0 | MTK |  | 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle |  | Dynamic rate-matching in PDSCH |
|  |  |  |  |  |  |  | 437.0 | MTK |  | 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle; RB-symbol rate-matching pattern period up to 40 ms |  | Semi-static rate-matching in PDSCH |
|  | TRS/CSI-RS-based PEI | 1-slot 24-RB TRS, occupying 144 REs (24 RB x 3 REs per RB x 2 symbols) |  | 3 bits |  | 1 (Intel) | 14.4 | Intel |  | 1 PEI for 1 PO |  | Dynamic rate-matching in PDSCH |
|  |  | 1-slot 36-RB TRS, occupying 216 REs (36 RB x 3 REs per RB x 2 symbols) |  | 8 bits |  | 1 (Intel) | 21.6 | Intel |  | 1 PEI for 1 PO |  | Dynamic rate-matching in PDSCH |
|  |  | 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols) |  | 1 bit |  | 1 (OPPO) | 30.0 | OPPO |  | 1 PEI for 1 PO |  | Dynamic rate-matching in PDSCH |
|  |  |  |  | 4 bits |  | 1 (MTK) | 26.0 | MTK |  | 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle |  | Dynamic rate-matching in PDSCH |

## A.6  UE processing timeline for baseline scheme

The following table shows UE processing timeline and power consumption, when PEI indicates not to monitor PO.

Table 6: UE processing timeline and power consumption, when PEI indicates not to monitor PO

| Operation in sequence | Purpose | Time duration (ms) | Energy contribution |
| --- | --- | --- | --- |
|  |  |  | (power * time + energy overhead) |
| SSB proc. | AGC, coarse synchronization, serving-cell RRM measurement | 2 | 60 * 2 |
| Light sleep | Power saving | 16 | 20 * 16 + 100 |
| PEI | Early indicate not to monitor PO | 2 | 50 * 2 = 100 |
| Deep sleep | Power saving | x | 1 * x + 450 |
| Total |  | 1280 | 2350 |

The following table shows UE processing timeline and power consumption, when PEI indicates paging message. We assume time gap between PEI and the associated PO is well configured which can accommodate two SSB bursts.

Table 7: UE processing timeline and power consumption, when PEI indicates to monitor PO

| Operation in sequence | Purpose | Time duration (ms) | Energy contribution |
| --- | --- | --- | --- |
|  |  |  | (power * time + energy overhead) |
| SSB proc. | AGC, coarse synchronization, serving-cell RRM measurement | 2 | 60 * 2 |
| Light sleep | Power saving | 16 | 20 * 16 + 100 |
| PEI | Early indicate to monitor PO | 2 | 50 * 2 |
| SSB proc. | Coarse/fine synchronization, serving-cell RRM measurement | 2 | 60 * 2 |
| Light sleep | Power saving | 18 | 20 * 18 + 100 |
| SSB proc. | Fine synchronization, serving-cell RRM measurement | 2 | 60 * 2 |
| Light sleep | Power saving | 14 | 20 * 14 + 100 |
| PDCCH+PDSCH | Paging control proc. and data proc. | 4 | 120 * 4 |
| Deep sleep | Power saving | x | 1 * x+450 |
| Total |  | 1280 | 3870 |

## A.7  UE processing timeline for LP-WUS scheme

We assume the location of the LP-WUS is the same as that of PEI.

The following table shows UE processing timeline and power consumption, when the LP-WUS indicates not to monitor PO (if the whole UE-ID is contained in the LP-WUS, it means “UE is not paged”).

Table 8: UE processing timeline and power consumption, when the LP-WUS indicates not to monitor PO

| Operation in sequence | Purpose | Time duration (ms) | Energy contribution |
| --- | --- | --- | --- |
|  |  |  | (power * time + energy overhead) |
| LP-WUR off | Power saving | 10 + 5 | 0.001 * 15 + (0.1-0.001)*5 |
| LP-WUR on | Indicate MR not to monitor PO | 2 | 0.1 * 2 |
| LP-WUR off | Power saving |  | 0.001 * 1263 |
| Total for LP-WUR |  | 1280 | ~1.5 |
| Ultra deep sleep | Power saving | 1280 | 0.015 * 1280 |
| Total for main radio |  | 1280 | 19.2 |

The following table shows UE processing timeline and power consumption, when the LP-WUS indicates to monitor PO (if the whole UE-ID is contained in the LP-WUS, it means “UE is paged”).

Table 9: UE processing timeline and power consumption, when the LP-WUS indicates to monitor PO

| Operation in sequence | Purpose | Time duration (ms) | Energy contribution |
| --- | --- | --- | --- |
|  |  |  | (power * time + energy overhead) |
| LP-WUR off | Power saving | 10 + 5 | 0.001 * 15 + (0.1-0.001)*5 |
| LP-WUR on | Indicate NR to monitor PO | 2 | 0.1 * 2 |
| LP-WUR off | Power saving | 1263 | 0.001 * 1263 |
| Total for LP-WUR |  | 1280 | ~1.5 |
| Ultra deep sleep | Power saving | 10 + 5 | 0.015 * (10 + 5) |
| Ramp-up/down | The main radio is ramp-up/down | 400 | 15000 |
| SSB proc. within 8 SSB periodicities | The main radio is sync/re-sync | 2 * 8 | 50 * 2 * 8 |
| Light sleep within 8 SSB periodicities | Power saving | 18 * 8 | (20 * 18 + 100) * 8 |
| SSB proc. within 1 SSB periodicity | The main radio is sync/re-sync | 2 | 50 * 2 |
| Light sleep within 1 SSB periodicity | Power saving | 8 | 20 * 8 + 100 |
| PDCCH+PDSCH | Paging control proc. and data proc. | 4 | 120 * 4 = 480 |
| Light sleep | Power saving | 6 | 20 * 6 + 100 |
| Ultra deep sleep | Power saving | x | 0.015*x |
| Total for MR |  | 1280 | 19730.5 |