3GPP TSG-RAN WG1 Meeting #113	R1-2304666

Incheon, Korea, May 22 – 26, 2023

Agenda Item:	9.16.2

Source:	Huawei, HiSilicon

Title:	UE features for expanded and improved NR positioning

Document for:	Discussion and Decision

# Introduction

- In this paper, we provide our views on the UE feature for Rel-18 positioning enhancement including SL positioning, CPP, LPHAP, BW aggregation, and RedCap UE positioning.

# SL positioning

## General principles

### RRC/SLPP/LPP

Since SL positioning involve both RRC procedures (e.g. resource pool level parameters, resource allocation, etc.) and SLPP procedures (assistance data, measurement request/report), the UE capabilities should be considered for both the ones reported to gNB and the ones reported to a UEs or a LMF.

- Proposal 1: Define UE capabilities from RRC and SLPP/LPP perspective.

- With regards to how to capture the corresponding capability reporting options, Rel-16/Rel-17 labeling (“need for gNB to know”, “need for the location server to know”) can be reused. In addition, we think that another labeling of “need for a UE to know” should also be introduced.

- Proposal 2: Reuse the current labelling of “need for gNB to know” and “need for the location server to know”, and add a new labeling of “need for a UE to know”.

### Basic feature groups

For the first release of SL positioning, there may be some discussions on whether or not a basic feature group that correspond to the minimum common set of features to allow SL positioning to operate should be defined.

According to our understanding, it may be challenging to converge on the need for such a basic FG or any components pertaining to the “basic FG” without seeing the overall UE feature structure, and thus we prefer to hold on such a discussion.

Proposal 3: The discussion on the basic FG for SL positioning should be on-hold.

## Proposed UE feature structure

Below is our initial preference on the structure of UE feature group.

| Description | Components | Type | Need to know |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | gNB | LMF | UE |
| SL-PRS transmission in shared RP | 1. Support SL-PRS transmission in shared resource pool 2. Support SL-PRS open loop power control 3. Support SCI formats indicating SL-PRS | Per band | X | X | X |
| SL-PRS transmission in dedicated RP | 1. Support SL-PRS transmission in shared resource pool and dedicated resource pool 2. Support SL-PRS open loop power control 3. Support periodic reservation and non-periodic reservation for SL-PRS transmission 4. Support SCI formats indicating SL-PRS | Per band | X | X | X |
| SL-PRS reception in shared RP | 1. Support SL-PRS reception in shared resource pool 2. Support RSRP reporting for open loop power control 3. Support receiving SCI formats indicating SL-PRS | Per band | X | X | X |
| SL-PRS reception in dedicated RP | 1. Support SL-PRS reception in dedicated resource pool 2. Support RSRP reporting for open loop power control 3. Support receiving SCI formats indicating SL-PRS | Per band | X | X | X |
| SL-PRS resource allocation scheme 1 | 1. Support dynamic grant, configured grant type 1, and configured type 2 for SL-PRS transmission | Per band | X |  |  |
| SL-PRS resource allocation scheme 2 | 1. Support sensing-based resource allocation and random resource selection 2. Support congestion control for SL-PRS | Per band | X |  |  |
| SL-PRS transmission request | 1. Support transmitting SL-PRS transmission request via lower layers 2. Support receiving SL-PRS transmission request via lower layers | Per band |  |  | X |
| Inter-UE coordinate for SL-PRS in scheme 2 | 1. Support to recommend SL-PRS resource for the purpose of resource selection of another UE in scheme 2 resource allocation 2. Support to receive the recommendation from another UE for the purpose of resource selection in scheme 2 resource allocation | Per band |  |  | X |
| SL RSTD measurement | Support SL RSTD measurement reporting | Per band |  | X | X |
| SL RTOA measurement | Support SL RTOA measurement reporting | Per band |  | X | X |
| UE Rx – Tx time difference measurement | Support UE Rx – Tx time difference measurement reporting | Per band |  | X | X |
| SL PRS-RSRP measurement | Support SL PRS-RSRP measurement reporting | Per band |  | X | X |
| SL PRS-RSRPP measurement | Support SL PRS-RSRPP measurement reporting | Per band |  | X | X |
| SL AoA measurement | Supported SL AoA measurement reporting: {LCS without translation, LCS with or without translation and GCS} | Per band |  | X | X |

# CPP

With regards to CPP, we think the UE feature structure could follow the first path RSRPP. We can consider two FGs, one for DL-TDOA, and the other for Multi-RTT.

| Description | Components | Type | Need to know |  |
| --- | --- | --- | --- | --- |
|  |  |  | gNB | LMF |
| First path RSCPD measurement for UE-assisted DL-TDOA | Support first path RSCPD measurement reporting for UE-assisted DL-TDOA | Per band |  | X |
| First path RSCP measurement for Multi-RTT | Support first path RSCP measurement reporting for Multi-RTT | Per band |  | X |
| UE-based CPP | Support assistance data for UE-based CPP | Per UE |  | X |
| CPP measurement window | Support the CPP measurement window | Per UE |  | X |

# LPHAP

For LPHAP, we think that RAN1 could only consider the feature related to multi-cell SRS configuration and RRC_IDLE state positioning. The following structure can be considered.

For RRC_IDLE state positioning, we think that the existing RRC_INACTIVE state positioning capabilities should be reused, therefore, we think that UE supporting RRC_IDLE state positioning should support RRC_INACTIVE state positioning

| Description | Components | Type | Need to know |  |
| --- | --- | --- | --- | --- |
|  |  |  | gNB | LMF |
| SRS positioning validity area | 1. Support SRS configuration valid across multiple cells. 2. Support SRS pathloss based on the SSB from the camping cell 3. Support using SRS Tx timing based on the current camping cell DL timing and the TA command from the last serving cell | Per band | X | X |
| PRS measurement in RRC_IDLE | 1. Support PRS measurement in RRC_IDLE Note: UE shall also support PRS measurement in RRC_INACTIVE for the band. The processing capabilities are the same as that for the RRC_INACTIVE state | Per band | X | X |

# BW aggregation

For BW aggregation, we consider the following FGs.

| Description | Components | Type | Need to know |  |
| --- | --- | --- | --- | --- |
|  |  |  | gNB | LMF |
| PRS BW aggregated measurement within MG in RRC_CONNECTED state | 1. Supported bandwidth class for PRS BW aggregation 2. Supported (N, T) value 3. Number of aggregated PRS resources that UE can process within a slot | Per band |  | X |
| PRS BW aggregated measurement in RRC_INACTIVE state | 1. Supported bandwidth class for PRS BW aggregation 2. Supported (N, T) value 3. Number of aggregated PRS resources that UE can process within a slot | Per band |  | X |
| SRS BW aggregated transmission in RRC_CONNECTED state | 1. Supported bandwidth class for SRS BW aggregation 2. Number of aggregated SRS resources within a slot 3. Support the same SRS power reduction across aggregated carriers | Per band | X | X |
| SRS BW aggregated transmission in RRC_INACTIVE state | 1. Supported bandwidth class for SRS BW aggregation 2. Number of aggregated SRS resources within a slot 3. Support the same SRS power reduction across aggregated carriers | Per band | X | X |

# RedCap UE positioning

For RedCap UE positioning, RAN4 agreed the switching time in their LS R1-2304316, we assume that those features should be captured in RAN4’s UE feature list.

- Observation 1: It is expected that RAN4 will capture the switching time capability in their UE feature list.

From RAN1 perspective, we consider the following FGs of SRS frequency hopping transmission.

| Description | Components | Type | Need to know |  |
| --- | --- | --- | --- | --- |
|  |  |  | gNB | LMF |
| SRS frequency hopping transmission for RRC_CONNECTED state | 1. Support of SRS transmission outside the active UL BWP. 2. Support the configuration of positioning SRS resource with frequency hopping. | Per FS | X | X |
| SRS frequency hopping transmission for RRC_INACTIVE state | 1. Support of SRS transmission outside the initial UL BWP. 2. Support the configuration of positioning SRS resource with frequency hopping. | Per band | X | X |

# Conclusion

In this contribution, we provided our views on the UE feature for Rel-18 positioning enhancements. Based on the discussion, we have the following observations and proposals.

- SL positioning

- Proposal 1: Define UE capabilities from RRC and SLPP/LPP perspective.

- Proposal 2: Reuse the current labelling of “need for gNB to know” and “need for the location server to know”, and add a new labeling of “need for a UE to know”.

Proposal 3: The discussion on the basic FG for SL positioning should be on-hold.

- Redcap positioning

- Observation 1: It is expected that RAN4 will capture the switching time capability in their UE feature list.

- Overall feature groups

| SL positioning | Components | Type | Need to know |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | gNB | LMF |  | UE |
| SL-PRS transmission in shared RP | 1. Support SL-PRS transmission in shared resource pool 2. Support SL-PRS open loop power control 3. Support SCI formats indicating SL-PRS | Per band | X | X |  | X |
| SL-PRS transmission in dedicated RP | 1. Support SL-PRS transmission in shared resource pool and dedicated resource pool 2. Support SL-PRS open loop power control 3. Support periodic reservation and non-periodic reservation for SL-PRS transmission 4. Support SCI formats indicating SL-PRS | Per band | X | X |  | X |
| SL-PRS reception in shared RP | 1. Support SL-PRS reception in shared resource pool 2. Support RSRP reporting for open loop power control 3. Support receiving SCI formats indicating SL-PRS | Per band | X | X |  | X |
| SL-PRS reception in dedicated RP | 1. Support SL-PRS reception in dedicated resource pool 2. Support RSRP reporting for open loop power control 3. Support receiving SCI formats indicating SL-PRS | Per band | X | X |  | X |
| SL-PRS resource allocation scheme 1 | 1. Support dynamic grant, configured grant type 1, and configured type 2 for SL-PRS transmission | Per band | X |  |  |  |
| SL-PRS resource allocation scheme 2 | 1. Support sensing-based resource allocation and random resource selection 2. Support congestion control for SL-PRS | Per band | X |  |  |  |
| SL-PRS transmission request | 1. Support transmitting SL-PRS transmission request via lower layers 2. Support receiving SL-PRS transmission request via lower layers | Per band |  |  |  | X |
| Inter-UE coordinate for SL-PRS in scheme 2 | 1. Support to recommend SL-PRS resource for the purpose of resource selection of another UE in scheme 2 resource allocation 2. Support to receive the recommendation from another UE for the purpose of resource selection in scheme 2 resource allocation | Per band |  |  |  | X |
| SL RSTD measurement | Support SL RSTD measurement reporting | Per band |  | X |  | X |
| SL RTOA measurement | Support SL RTOA measurement reporting | Per band |  | X |  | X |
| UE Rx – Tx time difference measurement | Support UE Rx – Tx time difference measurement reporting | Per band |  | X |  | X |
| UE Rx – Tx time difference measurement | Supported UE Rx – Tx time difference measurement reporting: {type 1, type 2, both types} | Per band |  | X |  | X |
| SL PRS-RSRP measurement | Support SL PRS-RSRP measurement reporting | Per band |  | X |  | X |
| SL PRS-RSRPP measurement | Support SL PRS-RSRPP measurement reporting | Per band |  | X |  | X |
| SL AoA measurement | Supported SL AoA measurement reporting: {LCS without translation, LCS with or without translation and GCS} | Per band |  | X |  | X |
| CPP | Components | Type | Need to know |  |  |  |
|  |  |  | gNB |  | LMF |  |
| First path RSCPD measurement for UE-assisted DL-TDOA | Support first path RSCPD measurement reporting for UE-assisted DL-TDOA | Per band |  |  | X |  |
| First path RSCP measurement for Multi-RTT | Support first path RSCP measurement reporting for Multi-RTT | Per band |  |  | X |  |
| UE-based CPP | Support assistance data for UE-based CPP | Per UE |  |  | X |  |
| CPP measurement window | Support the CPP measurement window | Per UE |  |  | X |  |
| LPHAP | Components | Type | Need to know |  |  |  |
|  |  |  | gNB |  | LMF |  |
| SRS positioning validity area | 1. Support SRS configuration valid across multiple cells. 2. Support SRS pathloss based on the SSB from the camping cell 3. Support using SRS Tx timing based on the current camping cell DL timing and the TA command from the last serving cell | Per band | X |  | X |  |
| PRS measurement in RRC_IDLE | 1. Support PRS measurement in RRC_IDLE Note: UE shall also support PRS measurement in RRC_INACTIVE for the band. The processing capabilities are the same as that for the RRC_INACTIVE state | Per band | X |  | X |  |
| BW aggregation | Components | Type | Need to know |  |  |  |
|  |  |  | gNB |  | LMF |  |
| PRS BW aggregated measurement within MG in RRC_CONNECTED state | 1. Supported bandwidth class for PRS BW aggregation 2. Supported (N, T) value 3. Number of aggregated PRS resources that UE can process within a slot | Per band |  |  | X |  |
| PRS BW aggregated measurement in RRC_INACTIVE state | 1. Supported bandwidth class for PRS BW aggregation 2. Supported (N, T) value 3. Number of aggregated PRS resources that UE can process within a slot | Per band |  |  | X |  |
| SRS BW aggregated transmission in RRC_CONNECTED state | 1. Supported bandwidth class for SRS BW aggregation 2. Number of aggregated SRS resources within a slot 3. Support the same SRS power reduction across aggregated carriers | Per band | X |  | X |  |
| SRS BW aggregated transmission in RRC_INACTIVE state | 1. Supported bandwidth class for SRS BW aggregation 2. Number of aggregated SRS resources within a slot 3. Support the same SRS power reduction across aggregated carriers | Per band | X |  | X |  |
| RedCap UE positioning | Components | Type | Need to know |  |  |  |
|  |  |  | gNB |  | LMF |  |
| SRS frequency hopping transmission for RRC_CONNECTED state | 1. Support of SRS transmission outside the active UL BWP. 2. Support the configuration of positioning SRS resource with frequency hopping. | Per FS | X |  | X |  |
| SRS frequency hopping transmission for RRC_INACTIVE state | 1. Support of SRS transmission outside the initial UL BWP. 2. Support the configuration of positioning SRS resource with frequency hopping. | Per band | X |  | X |  |