3GPP TSG RAN WG1 #113 Meeting	R1-2304685

Incheon, Korea, 22 – 26 May, 2023

Agenda item:		9.2.4.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Evaluation on ML for positioning accuracy enhancement

Document for:		Discussion and Decision

# Introduction

Rel-18 study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, agreed in [1], includes a use case of positioning accuracy enhancement. For this use case, the target of agenda item 9.2.4.1 is to evaluate performance benefits of AI/ML algorithms. Here, the methodologies should be based on statistical models already defined in TR 38.857.

In this contribution, we prioritize the evaluation that were agreed in the 3GPP RAN1#112bi-e meeting. However, we also discuss proposals not agreed upon and topics for further study indicated as part of the agreements made in the previous meetings since RAN1-110e. A collection of agreements of the last 3GPP RAN1 meetings are available in Appendix A. In this contribution, the agreements used as reference are presented within a frame-block to be differentiated from the content.

The relevant aspects of evaluations assumptions and datasets used in the evaluations are described in Section 2. In Section 3 we provide the evaluation results of relevant KPIs on AI/ML positioning enhancement. In Section 4, we provide the evaluation related to performance monitoring. In Section 5 and Section 6, we include a diversity of scenarios related to model generalization based on different settings and imperfections, respectively. In Section 7, we include other complementary evaluations that are not included in the previous sections. Finally, we provide a conclusion with a summary of observations and proposals in Section 8.

The main outcomes of this contribution are related to the following:

- Evaluation of model performance when there are imperfections in the ground truth labels for AI/ML assisted positioning,

- Evaluation of the model performance and model complexity when the number of TRPs is selected and the CIR/PDP measurement is used as input parameter. This scenario considers two approaches: fixed input model size and dynamic input model size for TRP selection.

# Evaluation Assumptions and Datasets

In the 3GPP RAN1#119-e meeting the following agreements were done:

Agreement The IIoT indoor factory (InF) scenario is a prioritized scenario for evaluation of AI/ML based positioning. Agreement For evaluation of AI/ML based positioning, at least the InF-DH sub-scenario is prioritized in the InF deployment scenario for FR1 and FR2. Agreement For InF-DH channel, the prioritized clutter parameters {density, height, size} are: - {60%, 6m, 2m}; - {40%, 2m, 2m}. - Note: an individual company may treat {40%, 2m, 2m} as optional in their evaluation considering their specific AI/ML design. Agreement For evaluation of AI/ML based positioning, reuse the common scenario parameters defined in Table 6-1 of TR 38.857.

These agreements set the basic simulation parameters used in the generation of the datasets used for training, testing, updating or fine tuning, and performance monitoring in the remaining sections.

Table 1 - Parameters used in the system level simulation to obtain the target datasets for AI/ML positioning enhancement evaluation, extracted from Table 6.1-1 in [2].

|  |  | FR1 Specific Values |  | FR2 Specific Values |
| --- | --- | --- | --- | --- |
| Channel model |  | InF-DH |  | InF-DH |
| Layout | Hall size | InF-DH: (baseline) 120x60 m (optional) 300x150 m |  |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |  |
|  | Room height | 10m |  |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |  |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 Note: Other gNB antenna configurations are not precluded for evaluation | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |  |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |  |
| Penetration loss |  | 0dB |  |  |
| Number of floors |  | 1 |  |  |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be selected from - the convex hull of the horizontal BS deployment. - the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. FFS: which of the above should be baseline. FFS: if an optional evaluation area is needed |  |  |
| UE antenna height |  | Baseline: 1.5m (Optional): uniformly distributed within [0.5, X2]m, where X2 = 2m for scenario 1(InF-SH) and X2= for scenario 2 (InF-DH) FFS: if the optional UE antenna height is needed |  |  |
| UE mobility |  | 3km/h |  |  |
| Min gNB-UE distance (2D), m |  | 0m |  |  |
| gNB antenna height |  | Baseline: 8m (Optional): two fixed heights, either {4, 8} m, or {max(4,), 8}. FFS: if the optional gNB antenna height is needed |  |  |
| Clutter parameters: {density , height ,size } |  | High clutter density: - {40%, 2m, 2m} - {60%, 6m, 2m} - Note: an individual company may treat {40%, 2m, 2m} as optional in their evaluation considering their specific AI/ML design. |  |  |
| Note 1: According to Table A.2.1-7 in TR 38.802 |  |  |  |  |

Dataset generation: Using the parameters defined in Table 1 extensive system level simulation is used to generate the following datasets:

- Dataset 01: InF-DH scenario with factory clutter density of 40%, factory clutter height of 2 meters, factory clutter size of 2 meters, and factory ceiling height of 10 meters. This dataset has 21K samples and is generated with 100 different drops.

- Dataset 02: InF-DH scenario with factory clutter density of 60%, factory clutter height of 6 meters, factory clutter size of 2 meters, and factory ceiling height of 10 meters. This dataset has 12K samples and is generated with 100 different drops.

- Dataset 03: InF-DH scenario with factory clutter density of 50%, factory clutter height of 3 meters, factory clutter size of 2 meters, and factory ceiling height of 10 meters. This dataset has 20K samples and is generated with 100 different drops.

- Dataset 04: InF-DH scenario with factory clutter density of 40%, factory clutter height of 2 meters, factory clutter size of 2 meters, and factory ceiling height of 10 meters. This dataset has 38K samples and is generated for only 1 drop.

# Relevant KPIs on AI/ML Positioning Evaluation

This section aims to discuss, define, and propose relevant KPIs. The first KPI is relative to data collection using the inter-point distance metric (IPD) metric to be used as a quality indicator for data collection. The other proposed KPI is the F1-score, which provides a single metric that weights two ratios: precision and recall. Both KPIs, the IPD and the F1-score are detailed in the following subsections.

## KPI for Data Collection

The availability of required data density for model training and testing/validation is one of the critical challenges in machine learning, especially in the context of positioning, where obtaining the ground truth labels in terms of UE location, LOS / NLOS condition, etc., is challenging. These challenges are visible through the evaluation of model performance using real-world data. These datasets could also be emulated in a simulated environment by assuming the availability of a limited dataset, which could be a subset of the grid-based or UE’s uniform distribution. As discussed in Sec. 3.1.3 of [4], various techniques, such as data augmentation, could be applied in such scenarios to ensure sufficient model performance.

The final technical report should highlight the usability of data augmentation to enhance the model performance used in AI/ML positioning.

It is important to highlight that in terms of training, the dataset size could be an indicator of user area density. However, this assumption is valid only when users are dropped following a uniform distribution within the simulation.

One key challenge in AI/ML-based positioning is the availability of good quality data with sufficient diversity of positioning ground truth labels and samples with accurate information for fine tuning, updating, and monitoring.

For data collection, the required sample density (e.g., #samples/m2) as indicator to achieve a given positioning accuracy target is limited to simulated scenarios using UE’s uniform distribution. Thus, it is important to determine a complementary indicator for quality data collection in addition to sample density.

- In the 3GPP RAN1#109-e meeting, the following agreement was done between companies:

Agreement For evaluation of AI/ML based positioning, details of the training dataset generation are to be reported by proponent company. The report may include (in addition to other selected settings, if applicable): - The size of training dataset, for example, the total number of UEs in the evaluation area for generating training dataset; - The distribution of UE location for generating the training dataset may be one of the following: - Option 1: grid distribution, i.e., one training data is collected at the center of one small square grid, where, for example, the width of the square grid can be 0.25/0.5/1.0 m. - Option 2: uniform distribution, i.e., the UE location is randomly and uniformly distributed in the evaluation area.

- In the current reporting evaluation tables for direct AI/ML and AI/ML assisted positioning methods, the data collection report was limited to the dataset size. Data collection details as total number of UEs in the evaluation area was not reported.

For data collection purposes, details of the training dataset generation (e.g., total number of UEs) should be included and reported in tables containing evaluation results.

Inter-point distance metric (IPD): The IPD is a metrics that complements the dataset density measurement for data collection in AI/ML positioning. The main advantage of using IPD is on scenarios with UE deployment not based on uniform distribution. As non-uniform UEs distribution represents better realistic data collection, IPD is a useful metric that enhance the indication of the dataset quality for AI/ML positioning.

To avoid misunderstanding in the technical report, RAN1 should emphasize for specific observations and conclusions related to data collection the UEs uniform distribution used on evaluations (e.g., uniform distribution).

The collected training dataset can be characterized with IPD metric. It relates to the distance between neighboring measured locations and is dependent on the density and the overall spreading of the training dataset within the considered region of interest. Typically, for a training dataset with high density, the measured locations are very close, which corresponds to a low IPD value. On the contrary, for sparse training datasets with low density, the IPD value is high, as illustrated in Figure 1. This information may be correlated with environmental characteristics. For instance, for the same IPD value and different LOS/NLOS conditions, the reached positioning accuracy is different.

Figure 1 - Example of training datasets with low density corresponding to high IPD value (left side) and high density corresponding to low IPD value (right side).

One illustrative example of the correlation between the environment and the IPD metric is shared in Table 2. Here, the required training dataset density expressed through the IPD metric is derived from: (1) targeted positioning accuracy and (2) the NLOS probability (%). Note that this requirement table could be maintained at LMF and corresponds to the minimum IPD threshold value as a function of the targeted accuracy and NLOS probability, which can be eventually enriched with further parameters.

Table 2 - Example of the correlation between the IPD metric and the NLOS probability (environment metric).

| Targeted accuracy (meters) | NLOS probability (%) | IPD value (meters) |
| --- | --- | --- |
| 1 | 90 | 0.5 |
| 1 | 20 | 1 |
| 0.5 | 10 | 0.6 |

Figure 2 - Data collection enhanced with IPD to support specific performance target.

The IPD metric can be used to optimize data collection, as described in Figure 2. Given the collected training dataset, the first step is to estimate the related IPD metric. The IPD value is then compared to the threshold value (as shown in Table 2) according to the predefined requirements. If the condition is verified, then the model can be trained using the collected training dataset. However, if this condition is not verified, then there is a high probability that the trained ML model will not reach the target performance/accuracy. Thus, it is necessary to collect further data to reach the desired density and perform training with it. To sum up, the proposed IPD-based training procedure performs training with it. The IPD-based approach allows us to avoid going through useless data collection, training, and verification operations while ensuring the required positioning accuracy.

The following agreement was achieved for AI/ML positioning accuracy and the correlation with user density/size.:

Agreement (RAN1-111) Study how AI/ML positioning accuracy is affected by: user density/size of the training dataset. Note: details of user density/size of training dataset to be reported in the evaluation.

In the following, we present some evaluations related to the impact of the lack of diverse data availability and the application of data augmentation as a potential solution to this problem. First, we describe high-level parameters used in the evaluation, and in other sub-item, we share the performance evaluation.

Deployment scenario and simulation assumptions: Using the dataset 1, defined by a clutter density of 40%, with a clutter height of 2 meters, clutter ceiling height of 10 meters, and clutter size of 2 meters. Further details of the deployment scenario and related assumptions are shown in Table 1.

The generated dataset contains features, such as the downlink time of arrival (ToA) for each link between each BS and the specific UE (18 ToAs measurement per UE) as well the ground truth of UE horizontal-2D location.

Details of the AI/ML model trained with supervised learning are indicated in Table 3.

Performance evaluation: we consider a neural network-based positioning method (num_hidden_layers = 2, num_hidden_nodes = 500) which considers as input ToA from a variable number of TRPs (18, 9, or 6) and the 2D horizontal UE position as output.

The performance of the ML model is assessed following a split of the total collected data into two separate sets for training and testing following different partitions. The model is first trained on the selected training set, and then the positioning accuracy is estimated on the test dataset.

The dataset can be described through the distance between the geographical locations associated IPD. The IPD metric is computed using Ripley’s G function (available within pointpats, which is an open-source python library for statistical analysis of planar point patterns [7]). The G function depicts the distribution of nearest neighbor distances. IPD metric is then defined as:

IPD=max(G).

As an example, Figure 3 shows the G function for the training set corresponding to 70% of the total data and for which IPD is estimated to be 1.5 m. As expected, the higher the proportion of the training set is the closer the data points (in terms of 2d horizontal distance), which corresponds to a lower IPD value. Figure 4 shows the IPD estimated for the training set ranging from 5% to 70% of dataset 1 (21K samples). Here, for 5% of the dataset the IPD is 5m, whereas IPD=1.5 m for 70% proportion.

Table 3 - Dataset description, model parameters, and complexity used in the evaluation of user's density scenarios.

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| ToA (from 18 TRPs) | 2d horizontal position (x, y) | 2d horizontal position (x, y) | Inf DH (40%) | Inf DH (40%) | 70% | 30% | 0,261 M | 0.521 M | 2.65 |

IPD

Figure 3 - Example IPD metric estimation with Ripley’s G function for test data 70% of total data.

Figure 4 - IPD metric estimation with Ripley’s G function for different percentage of Dataset 1 (21K samples). The dataset considers UE’s uniform distribution.

RAN1 to study and evaluate for AI/M-based positioning additional quality indicators for data collection (e.g., IPD) in addition to sample/density (# samples/m2) for non-uniform UEs distribution.

### UE distribution and IPD metric

We highlight in this section the impact of UE distribution and the IPD metric. To this end, the labeled data set (radio measurements and corresponding 2D positioning) is split into training and test sets. We then consider two cases (see Figure 5):

- UE distribution case 1: The training set is located following a uniform distribution on the considered region of interest (RoI). This corresponds to the conventional case used in simulation as agreed by companies.

- UE distribution case 2: The same training set size as case 1 is considered. However, the spatial distribution of the training data is modified to have more data localized in one specific region (x≤40), whereas only sparse points are left on the rest of the RoI.

Computing the IPD metric (using the G function as shown in Figure 6), we can clearly see the gap between two sets even with the same dataset size. The spatial distribution impacts the IPD value, for UE distribution case 1 the IPD is 2.5 m, and for UE distribution case 2, the IPD is 6 m.

The same ML model configuration is trained using dataset case 1 (model 1) and dataset case 2 (model 2). Thereafter, both models are tested using the same test data set. The obtained results are depicted in Table 4 for 90% CDF horizontal positioning accuracy. For UE distribution case 1, the obtained accuracy is 2.73 m, whereas it is 4.24 m for UE distribution case 2. Thus, even with the same sample/density(#samples/m2) but with different spatial distributions, the positioning accuracy degradation can reach 55%. This clearly shows the importance of the IPD metric to assess the training set quality and proves that the sample/density(#samples/m2) is not a sufficient parameter to characterize the data collection.

Figure 5 – Training data set for UE distribution case 1: uniform distribution, and UE distribution case 2: Same sample/density as case 1 but different UEs distribution.

Figure 6 - G function for UE distribution case 1 (left), and UE distribution case 2 based on a non-uniform distribution (right).

Table 4 - Horizontal positioning accuracy at CDF=90% for the evaluation related to dataset density based on IPD.

| Case | Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Train | Test | Train | test | AI/ML |
| 1 | ToA | 2d horizontal position (x, y) | 2d horizontal position (x, y) | Inf DH (40%) | Inf DH (40%) | 20% Randomly located | 80% | 2.73 |
| 2 | ToA | 2d horizontal position (x, y) | 2d horizontal position (x, y) | Inf DH (40%) | Inf DH (40%) | 20% Same size as case 1 but re-arranged (Mostly located on the left side) | 80% | 4.24 |

For data collection, RAN1 to consider additional metrics (e.g., IPD) to sample density (e.g., #samples/m2) to identify potential non-uniform UE distribution. Consequently, to trigger methodologies (e.g., data augmentation) to enhance the data collection and model performance.

## KPI for AI/ML Assisted Positioning

### F1-score for LOS/NLOS classification

In the case of LOS/NLOS classification, which is binary classification, using metrics to measure the sensitivity of the prediction is an important factor to evaluate the performance of the AI/ML model. The popular and trivial metric is accuracy. However, the accuracy metric provides a unfair information in unbalanced LOS/NLOS scenarios (e.g., Inf-DH with clutter density of 60%). For example, if one scenario has 98% of NLOS samples and only 2% of LOS samples, the AI/ML model used for classification will indicate an accuracy of 98% which are probably related only to the NLOS samples. However, the chances to get an acceptable accuracy for LOS indications is very low. Different than accuracy, the F1-score shows sensitivity on this type of scenarios.

In the case of binary classification, such LOS/NLOS prediction uses the harmonic mean of LOS/NLOS rate prediction. To evaluate this harmonic mean, the F1-score provides a single metric that weights two ratios: precision and recall using the following formula:

F1-score=2* Precision*RecallPrecision+Recall

Where:

Precision=N. of correctly predicted LOSN. of total LOS predictions=true positivetrue positive+false positive

Recall= N. of correctly predicted  LOSN. of LOS links in the labeling dataset=true positivetrue positive+false negative

Here, the precision measures the quality of links predicted as LOS, and the recall measures the accuracy of links predicted as LOS [6].

In the evaluation section, the accuracy and F1-score are considered to evaluate the performance of the model used for LOS/NLOS indication.

Evaluation: An industrial scenario is simulated. From this setup, a snapshot of 18 links containing PDP in the downlink is used as the input parameter of an AI/ML assisted positioning. The intermediate metric that is estimated is the LOS/NLOS indication. The PDP is truncated in the first 128 samples. The summary of this evaluation is shared in Table 5. Here, it is possible to identify the usability of F1-score compared to accuracy for this binary classification problem.

Table 5 - LOS/NLOS performance evaluation in both scenarios. Scenario 01 with clutter density of 40% (dataset 1), and Scenario 2 with clutter density of 60% (dataset 2).

|  | Classification accuracy | F1score |
| --- | --- | --- |
| Scenario 01 (hard selection) Inf-DH with clutter density 40% | 98% | 0.98 |
| Scenario 02 (hard Selection) Inf-DH with clutter density 60% | 99.4% | 0.01 |

The accuracy metric provides a false indication of quality classification on extreme unbalanced LOS/NLOS samples (e.g., InF-DH with 60% of clutter density).

RAN1 to consider at least F1-score as KPIs to measure the quality/sensitivity of LOS indication for AI/ML assisted positioning.

# Performance Monitoring Evaluation

In the 3GPP RAN1#112 meeting, companies agreed the following:

Agreement For direct AI/ML positioning, study the performance of model monitoring methods, including: - Label based methods, where ground truth label (or its approximation) is provided for monitoring the accuracy of model output. - Label-free methods, where model monitoring does not require ground truth label (or its approximation).

Our evaluation focuses on methods that does not depend on the ground truth labeling for both, input and output monitoring, they are described in the following subsections.

## Input parameter monitoring

For input parameter monitoring, we propose two evaluations. The first methodology is based on the definition of a null hypothesis that two distributions are identical, and the second is based on a machine learning model classifier.

### monitoring based on the similarity of distributions

Here, we use the Kolmogorov Smirnov Test (Kstest) for two-sample test comparison. It is based on the definition of a null hypothesis that two distributions are identical, Z(x) distribution is identical to H(x) for all x samples. If the hypothesis is not true, they are not identical. The hypothesis is evaluated using the p-value. For instance, if the null hypothesis is that a sample is distributed according to Z(x), we define a confidence level of 95%; which means that we will reject the null hypothesis in favor of the alternative (Z(x) distribution is different than H(x) distribution) if the p-value is less than 0.05.

In Table 9, Table 10, and Table 11 we share the outcomes of using the Kstest on different hypothetical scenarios. In all cases we consider the null hypothesis that a dataset used for training, fine-tuning, or monitoring (Z(x)) is identical to a new dataset representing a new scenario (H(x)). In all cases we choose a confidence level of 99%; that is, we will reject the null hypothesis if the p-value is less than 0.01.

The Kstest is done on 3 different representative features. In Table 6 we evaluate the CIR measurements, in Table 10 we evaluate the PDP measurements, and in Table 11 we consider the RSRP measurements. In the case of CIR and PDP the p-value provides ambiguous indications about the comparison between datasets. For instance, in Table 6 the first three rows are expected to get p-values less than 0.01, however in the last three rows we are forcing to do a comparison of different subsets of the same dataset, which means that we expect to get a p-value greater than 0.01 which is not correlated with the obtained results. In Table 7, the row 6 is getting a p-value of 0.00987 (less than 0.01) which indicates that the two subsets with the same distribution are not identical, which is not true.

In contrast of the ambiguous results obtained for CIR and PDP, in the case of RSRP the results are more consistent and accurate. For instance, in Table 8, the hypothesis null is confirmed or reject accordingly to our expectation.

Table 6 - Kolmogorov Smirnov Test when the analysed feature is the CIR measurement on different scenarios and conditions.

| Dataset used in Kolmogorov Smirnov Test (Kstest) |  | Null hypothesis Z(x) is identical to H(x) | Probability of similarity (p-value) Note: A small p-value suggests that it is unlikely that the dataset used for training came from new dataset from new scenario. |
| --- | --- | --- | --- |
| Dataset used for Training Z(x) | New Dataset from new scenario H(x) |  |  |
| Dataset 1 | Dataset 2 | Dataset 1 is identical to Dataset 2 | 0.0 |
| Dataset 1 | Dataset 3 | Dataset 1 is identical to Dataset 3 | 0.0 |
| Dataset 2 | Dataset 3 | Dataset 2 is identical to Dataset 3 | 1.4876e-58 |
| Dataset 1 (subset 1) | Dataset 1 (subset 2) | Dataset 1 is identical to Dataset 1 | 0.005833 |
| Dataset 2 (subset 1) | Dataset 2 (subset 2) | Dataset 2 is identical to Dataset 2 | 6.44706e-8 |
| Dataset 3 (subset 1) | Dataset 3 (subset 2) | Dataset 3 is identical to Dataset 3 | 4.21623e-20 |

Table 7 - Kolmogorov Smirnov Test when the analysed feature is the PDP measurement on different scenarios and conditions.

| Dataset used in Kolmogorov Smirnov Test (Kstest) |  | Null hypothesis Z(x) is identical to H(x) | Probability of similarity (p-value) Note: A small p-value suggests that it is unlikely that the dataset used for training came from new dataset from new scenario. |
| --- | --- | --- | --- |
| Dataset used for Training | New Dataset from new scenario |  |  |
| Dataset 1 | Dataset 2 | Dataset 1 is identical to Dataset 2 | 0.0 |
| Dataset 1 | Dataset 3 | Dataset 1 is identical to Dataset 3 | 0.0 |
| Dataset 2 | Dataset 3 | Dataset 2 is identical to Dataset 3 | 3.95661e-62 |
| Dataset 1 (subset 1) | Dataset 1 (subset 2) | Dataset 1 is identical to Dataset 1 | 0.0611 |
| Dataset 2 (subset 1) | Dataset 2 (subset 2) | Dataset 2 is identical to Dataset 2 | 0.06115 |
| Dataset 3 (subset 1) | Dataset 3 (subset 2) | Dataset 3 is identical to Dataset 3 | 0.00987 |

Table 8 - Kolmogorov Smirnov Test when the analysed feature is the RSRP measurement on different scenarios and conditions.

| Dataset used in Kolmogorov Smirnov Test (Kstest) |  | Null hypothesis Z(x) is identical to H(x) | Probability of similarity (p-value) Note: A small p-value suggests that it is unlikely that the dataset used for training came from new dataset from new scenario. |
| --- | --- | --- | --- |
| Dataset used for Training | New Dataset from new scenario |  |  |
| Dataset 1 | Dataset 2 | Dataset 1 is identical to Dataset 2 | 0.0 |
| Dataset 1 | Dataset 3 | Dataset 1 is identical to Dataset 3 | 8.83912e-168 |
| Dataset 2 | Dataset 3 | Dataset 2 is identical to Dataset 3 | 2.8052e-16 |
| Dataset 1 (subset 1) | Dataset 1 (subset 2) | Dataset 1 is identical to Dataset 1 | 0.9476 |
| Dataset 2 (subset 1) | Dataset 2 (subset 2) | Dataset 2 is identical to Dataset 2 | 0.5007 |
| Dataset 3 (subset 1) | Dataset 3 (subset 2) | Dataset 3 is identical to Dataset 3 | 0.51094 |

The performance monitoring based on the distribution comparison of different datasets is impacted by the feature that is selected to monitor. For instance, in evaluating the similarity between two distributions with Kolmogorov-Smirnov Test (Kstest) method, the RSRP measurement provides an accurate distribution comparison compared to CIR and PDP.

RAN1 further evaluate the impact of at least RSRP measurement as dominant feature for performance monitoring.

### monitoring based on machine learning classifier

Another potential method to identify similarities between two datasets is a customized ML classifier. Here, we define the hypothesis that by using a well-trained ML classifier between two datasets sharing the same statistical distribution, the classification performance will be similar to a random classifier, representing a reference of an inaccurate classification. In contrast, if two datasets have different statistical distributions, the classification performance should be represented by a curve in the direction of the upper left corner, as illustrated in Figure 7. To evaluate and represent graphically the binary classification, we consider the receiver operating characteristic (ROC) curve and the area under the curve (AUC) to indicate numerically the classification quality. An AUC score of 0.5 indicates an awful binary classification quality, and a value greater than 0.6 indicates a good classifier.

Figure 7 - Illustration of ROC curve. The fundamental information is indicated to differentiate the quality of binary classifiers.

The strategy is based on the arbitrary labeling of two datasets of RSRP measurements representing two scenarios (one represents the InF-DH with a clutter density of 40% and the other InF-DH with a clutter density of 60%). For instance, dataset 1 is labeled with the flag ‘1’, and dataset 2 is labeled with the flag ‘2’. If the distribution of dataset 1 and dataset 2 are identical, it is expected to get a wildly inaccurate classifier. In contrast, if the distributions are not similar, the classifier performance could be more than acceptable. 

In Figure 8 and Figure 9, we use a ROC curve to graph the ML classifier’s performance in two different scenarios. In both figures, we include a blue line to indicate the reference of inaccurate classification (AUC score of 0.5). Figure 8 considers two datasets with identical distributions showing an awful performance of the ML classifier (AUC score of 0.5069). In contrast, Figure 9 indicates that the classifier is better able to classify, which suggests that both datasets do not share the same distribution (AUC score of 0.743).

Figure 8 - ROC curve of Binary classification performance of two datasets with similar statistical distribution of the RSRP with AUC score of 0.5069.

Figure 9 - ROC curve of Binary classification performance of two datasets with different statistical distribution of the RSRP with AUC score of 0.743.

For Performance monitoring purposes, using ML models as binary classification provides an easily discriminatory criteria between two different dataset distributions. However, in real scenarios it is expected to get a diversity set of distributions. Thus, an extension of a binary classification to a multi-class classification could be a potential alternative, in addition an extra expense of model and computational complexity could be expected.

To RAN1 further study and evaluate the monitoring performance based on AI/ML models considering the model and computational complexity of the AI/ML model used for monitoring.

# Performance Evaluation on Different Settings

## Evaluation for direct AI/ML positioning

We consider dataset 1, defined by a clutter density of 40%, with a clutter height of 2 meters, clutter ceiling height of 10 meters, and clutter size of 2 meters. Further details of the deployment scenario and related assumptions are shown in Section 2.

Compared to our previous evaluations, here we use as input parameter the downlink time of arrival (ToA) for each link between each BS and the specific UE (18 ToAs measurement per UE) as well the ground truth of UE horizontal-2D location.

### Impact of the number of TRPs measurement and IPD metric data size

Positioning accuracy is evaluated for different cases, using 90% CDF value of the calculated position error:

- ToA is measured from a variable number of 18, 9, and 6 TRPs

- Different IPD metric values derived from different training/test partitions: [95%, 80%, 60%, 50%, 30%]

Figure 10 - Positioning accuracy vs. IPD for 18, 9, and 6 TRPs cases.

Figure 10 presents the relation between the IPD metric (as shown in x-axis) and horizontal positioning accuracy on the test dataset for different TRPs cases: 18, 9, and 6.

The main conclusions derived from the obtained results are:

- The positioning accuracy is improved with a higher number of TRPs measurements: from 5m accuracy with 6TRPs measurements to less than 3m with 18 TRPs measurements.

- The positioning accuracy is enhanced with a lower IPD of the training data set with around 13% improvement. This enhancement is more important in the case of a lower number of TRPs.

### Impact of clutter density percentage

Positioning accuracy is evaluated for different clutter cases 40%, 50%, and 60% considering the ML model (Figure 11) with 18 TRPs measurements and a split of training/test data (70% & 30%). Figure 11 shows the obtained horizontal positioning accuracy at 50- and 90-percentile value respectively. As expected, the positioning accuracy is degraded with increasing clutter percentage, which corresponds to higher NLOS.

Figure 11 -Positioning accuracy vs. clutter case.

The positioning accuracy improves in correlation with a higher number of TRPs measurements.

The positioning accuracy decreases with the increase in cluster density, that is 2D positioning accuracy is inversely proportional to the InF-DH clutter density.

## Generalization for AI/ML Assisted positioning

### Different Clutter Parameters

#### LOS/NLOS classification

In the 3GPP RAN1-111 meeting, the following agreement was approved by participant companies:

Agreement For both direct and AI/ML assisted positioning methods, investigate at least the impact of the amount of fine-tuning data on the positioning accuracy of the fine-tuned model. - The fine-tuning data is the training dataset from the target deployment scenario.

Agreement For AI/ML assisted approach, for a given AI/ML model design (e.g., input, output, single-TRP vs multi-TRP), identify the generalization aspects where model fine-tuning/mixed training dataset/model switching is necessary.

The AI/ML assisted positioning methods aims to enhance specific parameters that could be used in a second instance for a legacy methodology. The metric considered in this Tdoc is the LOS/NLOS indicator. The summary of the evaluation is shared in Table 9. Here, the column related to the amount of data used in the fine-tuning is highlighted in grey shading.

In Table 9, the model is trained with dataset 2 and tuned with dataset 1. Here the performance is consistently over 90% in all cases. However, the F1-score is very low for all scenarios, which indicates that the generalization of a model trained initially in an unbalanced dataset (e.g., NLOS rate 99% and LOS rate 1%) introduces a low-quality prediction for LOS. However, the accuracy metric alone indicates a false information (accuracy 94% and F1-score 0.056).

Table 9 - Training and tuning of an AI/ML assisted positioning based on LOS/NLOS classification. The model is trained in dataset 2 and tuned with a small amount of information from dataset 1 (UE distribution area = 120x60 m).

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  |  | Dataset size |  |  | AI/ML complexity |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Fine-tune | Test | Train | Fine-tune | test | Model complexity | Computation complexity | Accuracy | F1-score |
| PDP (Real) | LOS/ NLOS | LOS/NLOS indication | Dataset 2 | - | Dataset 2 | 80% | - | 20% | 130K | 0.0132G | 99.8% | 0.66 |
| PDP (Real) | LOS/ NLOS | LOS/NLOS indication | Dataset 2 | Dataset 1 | Dataset 1 | 80% | 10% | 90% | 130K | 0.0132G | 92.8% | 0.76 |
| PDP (Real) | LOS/ NLOS | LOS/NLOS indication | Dataset 2 | Dataset 1 | Dataset 2 | 80% | 10% | 20% | 130K | 0.0132G | 94.5% | 0.056 |

The F1-score (quality LOS indication) is sensitive to fine-tuning generalization scenarios when compared to accuracy metric, especially on scenarios considering extreme unbalanced LOS/NLOS samples.

# Performance Evaluation on Imperfections

## Direct AI/ML positioning

### Labeling errors

In the 3GPP RAN1#112 meeting the following evaluation was agreed between companies:

Agreement For direct AI/ML positioning, study the impact of labelling error to positioning accuracy - The ground truth label error in each dimension of x-axis and y-axis can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of L meters, with truncation of the distribution to the [-2*L, 2*L] range. - Value L is up to sources. - Other models are not precluded - [Whether/how to study the impact of labelling error to label-based model monitoring methods] - [Whether/how to study the impact of labelling error for AI/ML assisted positioning.]

For this evaluation, the ground truth label error was generated with the premises defined in the agreement. In Table 10 is shown the performance of the horizontal positioning. We use the error CDF at 90% as reference for each noise standard deviation. The input parameter is PDP truncated on 128 samples. The model was trained using dataset 4, further details of this dataset are shared in Section 2.

In the analysis we used the same AI/ML model structure for all scenarios. The model complexity is 460K parameters and the computational complexity is 3.19G flops.

Table 10 - Evaluation of ground truth labelling error with different noise standard deviation and their impact on the horizontal 2D error at CDF 90% for N_TRP=18. The error is indicated using the mean and the variance between 20 different realizations of the same experiment (UE distribution area = 120x60 m).

| Standard deviation of L meters. | 0.0 | 0.5 | 1.0 | 1.5 | 2.0 | 2.5 | 3.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Horizontal 2D error at CDF 90% (meters) | 0.7 | 0.8617 | 1.0268 | 1.4153 | 1.7279 | 1.8259 | 1.9751 |

In the evaluation, Direct AI/ML positioning is robust to label noise with Gaussian distribution. However, modeling this error type in real-world scenarios could be a challenge.

## AI/ML assisted positioning

### Labeling errors for ToA

In 3GPP RAN1#112bis-e meeting the following agreement was done between companies:

Agreement For AI/ML assisted positioning with TOA as model output, study the impact of labelling error to TOA accuracy and/or positioning accuracy. - The ground truth label error of TOA is calculated based on location error. The location error in each dimension of x-axis and y-axis can be modelled as a truncated Gaussian distribution with zero mean and standard deviation of L meters, with truncation of the distribution to the [-2*L, 2*L] range. - Value L is up to sources. - Other models of labelling error are not precluded - Other timing information, e.g., RSTD, as model output is not precluded.

The evaluation is done using the dataset 4 (check details in Section 2). The intermediate feature estimation is evaluated using the mean-square error (MSE). In this evaluation we considered the construction based one model to evaluate to evaluate N TRPs simultaneously. For the case of N TRP = 10, the performance of the intermediate feature estimation is shown in Table 11. For this scenario the model complexity is of 1413 K parameters and the computational complexity is of 1.82 G flops. Details are shared in Table 11.

Table 11 - Evaluation of ground truth labelling error with different noise standard deviation and their impact on the estimation of the TOA (intermediate feature) for N_TRP=10. The error is indicated using the mean and the variance between 20 different realizations of the same experiment (UE distribution area = 120x60 m).

| Standard deviation L (m). | 0.0 | 1.0 | 2.0 | 3.0 | 4.0 | 5.0 | 10.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Intermediate Feature error estimation (mean-square error) | 0.45 | 1.19 | 1.36 | 1.61 | 2.43 | 3.256 | 9.508 |

An extensive simulation was done to generate the result shared in Figure 12. Here, in addition to the standard deviation L, a different number of TRPs are considered. The more critical scenarios are characterized by L=3, 4, 5, and 10. The extreme failure impact on the TOA accuracy prediction is for L=10. In addition, the simulation result indicates that the impact of labeling error is attenuated when a large amount of TRPs is used (check performance of line related to L=10). Details of complexity are shared in Table 12.

Table 12 - Model complexity and computational complexity for scenarios simulated with different number of TRPs (N TRPs) when the intermediate feature is ToA estimation.

| N TRPs | Model Complexity (parameters) | Computational complexity (G flops) |
| --- | --- | --- |
| 1 | 1,410,257 | 1.06 |
| 2 | 1,410,578 | 1.15 |
| 3 | 1,410,899 | 1.23 |
| 4 | 1,411,220 | 1.31 |
| 5 | 1,411,541 | 1.4 |
| 6 | 1,411,862 | 1.48 |
| 7 | 1,412,183 | 1.57 |
| 8 | 1,412,504 | 1.65 |
| 9 | 1,412,825 | 1.73 |
| 10 | 1,413,146 | 1.82 |
| 11 | 1,413,467 | 1.9 |
| 12 | 1,413,788 | 1.98 |
| 13 | 1,414,109 | 2.07 |
| 14 | 1,414,430 | 2.15 |
| 15 | 1,414,751 | 2.23 |
| 16 | 1,415,072 | 2.32 |
| 17 | 1,415,393 | 2.4 |
| 18 | 1,415,714 | 2.49 |

Figure 12 - ToA estimation performance evaluation using mean-square error (MSE) for different L and number of TRPs values.

The impact of labelling error on TOA accuracy (in MSE) is less than 1.5 for low/moderate standard deviation L=1, 2, and 3.

Using many TRPs do not enhance the performance of the TOA indication significantly in the presence of labelling error on TOA accuracy (in MSE), except for large L values (e.g., L=10).

### Labeling errors for LOS/NLOS indication

In 3GPP RAN1#112bis-e meeting the following evaluation was agreed between companies:

Agreement For AI/ML assisted positioning with LOS/NLOS indicator as model output, study the impact of labelling error to LOS/NLOS indicator accuracy and/or positioning accuracy. - The ground truth label error of LOS/NLOS indicator can be modelled as m% LOS label error and n% NLOS label error. - Value m and n are up to sources. - Companies consider at least hard-value LOS/NLOS indicator as model output.

Following the agreement, this evaluation considers the dataset 4 (check details in Section 2). The labeling error is modeled as m% LOS label error and n% NLOS label error. The combination of both is selected arbitrarily to evaluate the impact of the impairment on the accuracy of LOS prediction. The metrics used in this evaluation are the accuracy and the F1-score. Table 13, Table 14, and Table 15 indicate the performance of the LOS indication for N TRPs=1, 13, and 18 respectively. Details of complexity are shared in Table 16. For a variety of combinations of m% and n%. In general, the performance is degraded only after 10% of label error for NLOS and LOS. For degradation less than 30%, the accuracy/F1-score remains greater than 90%.

Table 13 – [NTRP=1] Evaluation of ground truth labelling error with different m% LOS label error and n% NLOS label error. The scenario is InF-DH with clutter density of 40% (dataset 4). The UE distribution area is 120x60 m.

| m (%) | 0 | 5 | 10 | 20 | 30 | 50 | 0 | 50 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n (%) | 0 | 10 | 20 | 30 | 20 | 0 | 50 | 50 | 100 |
| Accuracy | 99.53% | 99.5% | 98.12% | 93.02% | 96.4% | 86.5% | 79.2% | 71.45% | 19.2% |
| F1-score | 0.9825 | 0.9828 | 0.9345 | 0.7845 | 0.8685 | 0.319 | 0.5639 | 0.3701 | 0.0901 |

Table 14 – [NTRP=13] Evaluation of ground truth labelling error with different m% LOS label error and n% NLOS label error. The scenario is InF-DH with clutter density of 40% (dataset 4) for NTRP=13. The UE distribution area is 120x60 m.

| m (%) | 0 | 5 | 10 | 20 | 30 | 50 | 0 | 50 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n (%) | 0 | 10 | 20 | 30 | 20 | 0 | 50 | 50 | 100 |
| Accuracy | 98.75% | 99.3% | 97.29% | 91.35% | 93.03% | 89.6% | 83.6% | 71.7% | 25.1% |
| F1-score | 0.9835 | 0.9909 | 0.9645 | 0.8943 | 0.9061 | 0.843 | 0.8235 | 0.652 | 0.202 |

Table 15 – [NTRP=18] Evaluation of ground truth labelling error with different m% LOS label error and n% NLOS label error. The scenario is InF-DH with clutter density of 40% (dataset 4). The UE distribution area is 120x60 m.

| m (%) | 0 | 5 | 10 | 20 | 30 | 50 | 0 | 50 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n (%) | 0 | 10 | 20 | 30 | 20 | 0 | 50 | 50 | 100 |
| Accuracy | 98.1% | 99.32% | 97.18% | 90.75% | 92.36% | 89.12% | 79.2% | 70.35% | 24.98% |
| F1-score | 0.975 | 0.9914 | 0.9651 | 0.8902 | 0.9014 | 0.8405 | 0.5639 | 0.6539 | 0.2132 |

When the number of TRPs is more than one, adding a labelling error of less than 10% for LOS/NLOS indication the performance is not impacted, in some cases the performance is enhanced when compared to the benchmark (m%=0, and n%=0).

For AI/ML assisted positioning with LOS/NLOS indicator as model output, an acceptable LOS/NLOS indication is obtained until 20% of error for both, n% and m% error.

Table 16 - Model complexity and computational complexity for scenarios simulated with different number of TRPs (N TRPs) when the intermediate feature is LOS/NLOS estimation.

| N TRPs | Model Complexity (parameters) | Computational complexity (G flops) |
| --- | --- | --- |
| 1 | 509,325 | 0.102 |
| 5 | 1,558,417 | 0.313 |
| 10 | 2,869,782 | 0.576 |
| 13 | 3,656,601 | 0.735 |
| 14 | 3,918,874 | 0.787 |
| 18 | 4,967,966 | 0.998 |

To compare the impact of the number of TRPs for 9 different label error levels, a simulation campaign considered N TRPs equal to 1, 5, 10, 14, 18 are evaluated. The label errors are defined based on the following map:

- Low label error 1: m%=5, n%=10,

- Low label error 2: m%=10, n%=20,

- Medium label error 1: m%=20, n%=30,

- Medium label error 2: m%=30, n%=20,

- Medium label error 3: m%=50, n%=0,

- Strong label error 1: m%=0, n%=50,

- Strong label error 2: m%=50, n%=50,

- Strong label error 2: m%=100, n%=100.

The performance is shared in Figure 13. In general, the performance is independent of the number of TRPs are used in the model construction (one model for N TRPs simultaneously).

Figure 13 - LOS/NLOS accuracy prediction versus N TRPs (number of TRPs). One model construction for N TRPs simultaneously. Evaluation done using Dataset 4 (InF-DH 40% of clutter density and 1 drop).

The impact of labeling error for LOS/NLOS indication is independent to the number of TRPs used in the model construction.

# Other Complementary Evaluations

## Trade-off between complexity and performance evaluation

In the 3GPP RAN1#112 meeting, the following proposal was agreed between companies.

Agreement For both the direct AI/ML positioning and AI/ML assisted positioning, study the model input, considering the tradeoff among model performance, model complexity and computational complexity. - The type of information to use as model input. The candidates include at least: time-domain CIR, PDP. - The dimension of model input in terms of NTRP, Nt, and Nt’. - Note: For the direct AI/ML positioning, model input size has impact to signaling overhead for model inference.

In the 3GPP RAN1#112bis-e meeting, the following proposal was agreed

Agreement For the evaluation of AI/ML based positioning, the study of model input due to different number of TRPs include the following approaches. Proponent of each approach provide analysis for model performance, signaling overhead (including training data collection and model inference), model complexity and computational complexity. Approach 1: Model input size stays constant as NTRP=18. The number of TRPs (N’TRP) that provide measurements to model input varies. When N’TRP < NTRP, the remaining (NTRP - N’TRP) TRPs do not provide measurements to model input, i.e., measurement value is set to 0. - Approach 1-A. The set of TRPs (N’TRP) that provide measurements is fixed. - Approach 1-B. The set of TRPs (N’TRP) that provide measurements can change dynamically. - Note: for Approach 1, one model is provided to cover the entire evaluation area. Approach 2: The TRP dimension of model input is equal to the number of TRPs (N’TRP) that provide measurements as model input. When N’TRP < NTRP, the remaining (NTRP - N’TRP) TRPs are ignored by the given model. For a given AI/ML model, the set of TRPs (N’TRP) that provide measurements is fixed. - For Approach 2: one model can be provided to cover the entire evaluation area, which is equivalent to deploying N’TRP TRPs in the evaluation area for positioning if ignoring the potential inference from the remaining (18 - N’TRP) TRPs. - For Approach 2, if Nmodel (Nmodel >1) models are provided to cover the entire evaluation area, the total complexity (model complexity is the summation of the Nmodel models.

In this section we considered as model input PDP measurements truncated on N’t=128 samples, the model output is the horizontal 2d positioning. Approach 1 and approach 2 are detailed in the following.

### Approach 1

For this approach, we fixed the model input parameter size to N TRPs = 18 for all cases. When a subset of 18 TRPs is considered, the remaining of non-selected TRPs are set to zero. In Table 17 and Table 18, it is shown that the model and computational complexity remains almost the same for all cases in the same table, because the input model parameter is fixed. Here, we considered the dataset 1 (check details of the dataset in Section 2). The input parameter was CIR truncated on 128 samples.

Table 17 – For approach 1 - Evaluation for different number of TRPs (N_TRPs) considering an arbitrarily TRPs selection and compared with their respective model complexity and computational complexity. The input parameter is PDP, the N’t =128, and UE distribution area = 120x60 m.

| N_TRPs (input parameter fixed = 18) | 4 TRPs with real values and 14 TRPs set zero | 8 TRPs with real values and 10 TRPs set zero | 12 TRPs with real values and 6 TRPs set zero | 16 TRPs with real values and 2 TRPs set zero | 18 TRPs with real values and 0 TRPs set Zero |
| --- | --- | --- | --- | --- | --- |
| Model complexity (parameters) | 466.9K | 466.9K | 466.9K | 466.9K | 466.9K |
| computational complexity (flops) | 1280M | 1280M | 1280M | 1280M | 1280M |
| Horizontal 2D error at CDF 90% (meters) | 57.7 | 32.84 | 21.06 | 2.755 | 1.39 |

Increase in the model complexity will enhance the positioning accuracy performance of all scenarios. However, it is significant that the best impact is when a large number of TRPs is considered for approach 1, for scenarios with low number of TRPs, it seems that the model overfits providing a worst performance. These results are shared in Table 18.

Table 18 – For approach 1 - Evaluation for different number of TRPs (N_TRPs) considering an arbitrarily TRPs selection and compared with their respective model complexity and computational complexity. The input parameter is PDP, the N’t =128, UE distribution area = 120x60 m and using a highest computational complexity than Table 17.

| N_TRPs (input parameter fixed = 18) | 4 TRPs with real values and 14 TRPs set zero | 8 TRPs with real values and 10 TRPs set zero | 12 TRPs with real values and 6 TRPs set zero | 16 TRPs with real values and 2 TRPs set zero | 18 TRPs with real values and 0 TRPs set Zero |
| --- | --- | --- | --- | --- | --- |
| Model complexity (parameters) | 466.9K | 466.9K | 466.9K | 466.9K | 466.9K |
| computational complexity (flops) | 6390M | 6390M | 6390M | 6390M | 6390M |
| Horizontal 2D error at CDF 90% (meters) | 53.17 | 26.94 | 17.3 | 1.39 | 0.732 |

### Approach 2

The trade-off among model performance and complexity is presented in Table 19 and Table 20. Here, we can observe that there is an impact on the positioning accuracy when the number of N_TRPs is iterated from 4 to 18, and when the computational complexity is extended. As bigger the number of TRPs, the horizontal positioning accuracy is enhanced. Likewise, the model complexity and the computational complexity values increase. Specifically, the computational complexity is affected wen the number of TRPs is increased, for instance the complexity using 4 TRPs is 498M flops and using 18 TRPs, the complexity is more than the double (1280M flops). The model was trained using dataset 1, further details of the dataset are shared in Section 2.

Table 19 – For approach 2 - Evaluation for different number of TRPs (N_TRPs) considering an arbitrarily TRPs selection and compared with their respective model complexity and computational complexity. The input parameter is PDP, the N’t =128, and UE distribution area = 120x60 m.

| N_TRPs | 4 | 8 | 12 | 16 | 18 |
| --- | --- | --- | --- | --- | --- |
| Model complexity (parameters) | 462.8K | 464K | 465.2K | 466.3K | 466.9K |
| computational complexity (flops) | 498M | 720M | 943M | 1170M | 1280M |
| Horizontal 2D error at CDF 90% (meters) | 2.3317 | 1.5684 | 1.3404 | 1.173 | 0.9719 |

Table 20 – For approach 2 - Evaluation for different number of TRPs (N_TRPs) considering an arbitrarily TRPs selection and compared with their respective model complexity and computational complexity. The input parameter is PDP, the N’t =128, UE distribution area = 120x60 m and using a highest computational complexity than Table 19.

| N_TRPs | 4 | 8 | 12 | 16 | 18 |
| --- | --- | --- | --- | --- | --- |
| Model complexity (parameters) | 462.8K | 464K | 465.2K | 466.3K | 466.9K |
| computational complexity (flops) | 2490M | 3600M | 4720M | 5830M | 6390M |
| Horizontal 2D error at CDF 90% (meters) | 1.3106 | 1.13219 | 1.0132 | 0.9266 | 0.6838 |

The horizontal performance is enhanced when the number of TRPs is increased at the expense of the computational complexity.

## Semi-supervised learning (SSL)

In the 3GPP RAN1#110 meeting, the following proposal was agreed between companies.

Agreement For evaluation of AI/ML based positioning, study the performance impact from availability of the ground truth labels (i.e., some training data may not have ground truth labels). The learning algorithm (e.g., supervised learning, semi-supervised learning, unsupervised learning) is reported by participating companies.

### SSL for AI/ML assisted positioning

The evaluations of semi-supervised learning for LOS/NLOS classification aims to verify the performance gains in scenarios using a dataset with a small number of labeled samples compared to the unlabeled samples. The evaluation considers the dataset 4 (check details in Section 2). The model input is the CIR signal, and the model construction considers a unique model for a single. The results are shared in Table 21, here we include performance results of supervised learning to be compared with the semi supervised learning on similar conditions. The performance of supervised learning using a short labeled dataset achieves an accuracy of 82.4% of accuracy for LOS/NLOS indication. However, when the semi-supervised learning is applied with 100K unlabeled samples, the overall performance of LOS/NLOS indication is improved by 4 percentage on the accuracy.

Table 21 - LOS/NLOS indication performance for assisted AI/ML positioning when a semi-supervised learning is applied on scenarios with low labeled samples.

| Training dataset |  | Testing dataset | Model complexity | Computational complexity | LOS/NLOS classification accuracy |
| --- | --- | --- | --- | --- | --- |
| Labeled | Unlabeled |  |  |  |  |
| 1k | 0 | 3k | 129k | 141k | 82.40% |
| 2k | 0 | 3k | 129k | 141k | 84.37% |
| 3k | 0 | 3k | 129k | 141k | 85.93% |
| 1k | 100k | 3k | 129k | 141k | 83.83% |
| 2k | 100k | 3k | 129k | 141k | 85.07% |
| 3k | 100k | 3k | 129k | 141k | 86.37% |

The semi-supervised learning enhances the performance of LOS/NLOS classification accuracy as compared to supervised learning when there is a small number of labeled samples are available.

Further study the benefits of semi-supervised learning for AI/ML positioning in terms of performance gains, amount of labeled data, model complexity, and computational complexity.

# Conclusion

In this contribution, we made the following observations and proposals:

## Observations

Observation 1: The final technical report should highlight the usability of data augmentation to enhance the model performance used in AI/ML positioning.

Observation 2: One key challenge in AI/ML-based positioning is the availability of good quality data with sufficient diversity of positioning ground truth labels and samples with accurate information for fine tuning, updating, and monitoring.

Observation 3: For data collection, the required sample density (e.g., #samples/m2) as indicator to achieve a given positioning accuracy target is limited to simulated scenarios using UE’s uniform distribution. Thus, it is important to determine a complementary indicator for quality data collection in addition to sample density.

Observation 4: To avoid misunderstanding in the technical report, RAN1 should emphasize for specific observations and conclusions related to data collection the UEs uniform distribution used on evaluations (e.g., uniform distribution).

Observation 5: For data collection, RAN1 to consider additional metrics (e.g., IPD) to sample density (e.g., #samples/m2) to identify potential non-uniform UE distribution. Consequently, to trigger methodologies (e.g., data augmentation) to enhance the data collection and model performance.

Observation 6: The accuracy metric provides a false indication of quality classification on extreme unbalanced LOS/NLOS samples (e.g., InF-DH with 60% of clutter density).

Observation 7: The performance monitoring based on the distribution comparison of different datasets is impacted by the feature that is selected to monitor. For instance, in evaluating the similarity between two distributions with Kolmogorov-Smirnov Test (Kstest) method, the RSRP measurement provides an accurate distribution comparison compared to CIR and PDP.

Observation 8: For Performance monitoring purposes, using ML models as binary classification provides an easily discriminatory criteria between two different dataset distributions. However, in real scenarios it is expected to get a diversity set of distributions. Thus, an extension of a binary classification to a multi-class classification could be a potential alternative, in addition an extra expense of model and computational complexity could be expected.

Observation 9: The positioning accuracy improves in correlation with a higher number of TRPs measurements.

Observation 10: The positioning accuracy decreases with the increase in cluster density, that is 2D positioning accuracy is inversely proportional to the InF-DH clutter density.

Observation 11: The F1-score (quality LOS indication) is sensitive to fine-tuning generalization scenarios when compared to accuracy metric, especially on scenarios considering extreme unbalanced LOS/NLOS samples.

Observation 12: In the evaluation, Direct AI/ML positioning is robust to label noise with Gaussian distribution. However, modeling this error type in real-world scenarios could be a challenge.

Observation 13: The impact of labelling error on TOA accuracy (in MSE) is less than 1.5 for low/moderate standard deviation L=1, 2, and 3.

Observation 14: Using many TRPs do not enhance the performance of the TOA indication significantly in the presence of labelling error on TOA accuracy (in MSE), except for large L values (e.g., L=10).

Observation 15: When the number of TRPs is more than one, adding a labelling error of less than 10% for LOS/NLOS indication the performance is not impacted, in some cases the performance is enhanced when compared to the benchmark (m%=0, and n%=0).

Observation 16: For AI/ML assisted positioning with LOS/NLOS indicator as model output, an acceptable LOS/NLOS indication is obtained until 20% of error for both, n% and m% error.

Observation 17: The impact of labeling error for LOS/NLOS indication is independent to the number of TRPs used in the model construction.

Observation 18: The horizontal performance is enhanced when the number of TRPs is increased at the expense of the computational complexity.

Observation 19: The semi-supervised learning enhances the performance of LOS/NLOS classification accuracy as compared to supervised learning when there is a small amount of labeled samples are available.

Observation 20: Further study the benefits of semi-supervised learning for AI/ML positioning in terms of performance gains, amount of labeled data, model complexity, and computational complexity.

## Proposals

Proposal 1: For data collection purposes, details of the training dataset generation (e.g., total number of UEs) should be included and reported in tables containing evaluation results.

Proposal 2: RAN1 to study and evaluate for AI/M-based positioning additional quality indicators for data collection (e.g., IPD) in addition to sample/density (# samples/m2) for non-uniform UEs distribution.

Proposal 3: RAN1 to consider at least F1-score as KPIs to measure the quality/sensitivity of LOS indication for AI/ML assisted positioning.

Proposal 4: RAN1 further evaluate the impact of at least RSRP measurement as dominant feature for performance monitoring.

Proposal 5: To RAN1 further study and evaluate the monitoring performance based on AI/ML models considering the model and computational complexity of the AI/ML model used for monitoring.

# References

- 3GPP RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” 3GPP RAN-94-e.

- 3GPP TR 38.857, “Study on NR positioning enhancements,” v17.0.0, March 2021.

- 3GPP R1-2206967, “Further discussion on the general aspects of ML in air interface,” Nokia, Nokia Shanghai Bell, 3GPP RAN1-110.

- 3GPP R1-2212332, “Other aspects on AI/ML for positioning accuracy enhancement,” Nokia, Nokia Shanghai Bell, 3GPP RAN1-111.

- 3GPP R1-2206972, “Evaluation on AI/ML for positioning accuracy enhancement,” Nokia, Nokia Shanghai Bell, 3GPP RAN1-110.

- Sokolova, M., Japkowicz, N., Szpakowicz, S. (2006). Beyond Accuracy, F-Score and ROC: A Family of Discriminant Measures for Performance Evaluation. In: Sattar, A., Kang, Bh. (eds) AI 2006: Advances in Artificial Intelligence. AI 2006. Lecture Notes in Computer Science, vol 4304. Springer, Berlin, Heidelberg. https://doi.org/10.1007/11941439_114.

- Point Pattern Analysis (pointpats) – pointpats v2.1.1 Manual, URL: https://pointpats.readthedocs.io/.

# Appendix A

The agreements reached on the agenda item 9.2.4.1 related to evaluation of use cases for AI/ML based positioning accuracy enhancement are indicated in each 3GPP RAN1 meeting.

Agreements reached in 3GPP RAN1#109e meeting

Agreement The IIoT indoor factory (InF) scenario is a prioritized scenario for evaluation of AI/ML based positioning. Agreement For evaluation of AI/ML based positioning, at least the InF-DH sub-scenario is prioritized in the InF deployment scenario for FR1 and FR2. Agreement For InF-DH channel, the prioritized clutter parameters {density, height, size} are: - {60%, 6m, 2m}; - {40%, 2m, 2m}. - Note: an individual company may treat {40%, 2m, 2m} as optional in their evaluation considering their specific AI/ML design. Agreement For evaluation of AI/ML based positioning, reuse the common scenario parameters defined in Table 6-1 of TR 38.857. Agreement For evaluation of InF-DH scenario, the parameters are modified from TR 38.857 Table 6.1-1 as shown in the table below. - The parameters in the table are applicable to InF-DH at least. If another InF sub-scenario is prioritized in addition to InF-DH, some parameters in the table below may be updated. Parameters common to InF scenario (Modified from TR 38.857 Table 6.1-1) FR1 Specific Values; FR2 Specific Values. Channel model; InF-DH; InF-DH. Layout; Hall size; InF-DH: (baseline) 120x60 m (optional) 300x150 m. BS locations; 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m. Room height; 10m. Total gNB TX power, dBm; 24dBm; 24dBm EIRP should not exceed 58 dBm. gNB antenna configuration; (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 Note: Other gNB antenna configurations are not precluded for evaluation; (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed. gNB antenna radiation pattern; Single sector – Note 1; 3-sector antenna configuration – Note 1. Penetration loss; 0dB. Number of floors; 1. UE horizontal drop procedure; Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be selected from - the convex hull of the horizontal BS deployment. - the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. FFS: which of the above should be baseline. FFS: if an optional evaluation area is needed. UE antenna height; Baseline: 1.5m (Optional): uniformly distributed within [0.5, X2]m, where X2 = 2m for scenario 1(InF-SH) and X2= for scenario 2 (InF-DH) FFS: if the optional UE antenna height is needed. UE mobility; 3km/h. Min gNB-UE distance (2D), m; 0m. gNB antenna height; Baseline: 8m (Optional): two fixed heights, either {4, 8} m, or {max(4,), 8}. FFS: if the optional gNB antenna height is needed. Clutter parameters: {density , height ,size }; High clutter density: - {40%, 2m, 2m} - {60%, 6m, 2m} - Note: an individual company may treat {40%, 2m, 2m} as optional in their evaluation considering their specific AI/ML design.. Note 1: According to Table A.2.1-7 in TR 38.802. Agreement For AI/ML-based positioning evaluation, the baseline performance to compare against is that of existing Rel-16/Rel-17 positioning methods. - As a starting point, each participating company report the specific existing positioning method (e.g., DL-TDOA, Multi-RTT) used as comparison. Agreement For all scenarios and use cases, the main KPI is the CDF percentiles of horizonal accuracy. - Companies can optionally report vertical accuracy. Agreement The CDF percentiles to analyse are: {50%, 67%, 80%, 90%}. - 90% is the baseline. {50%, 67% 80%} are optional. Agreement Target positioning requirements for horizonal accuracy and vertical accuracy are not defined for AI/ML-based positioning evaluation. Agreement For evaluation of AI/ML based positioning, the KPI include the model complexity and computational complexity. - FFS: the details of model complexity and computational complexity Agreement Synthetic dataset generated according to the statistical channel models in TR38.901 is used for model training, validation, and testing. Agreement The dataset is generated by a system level simulator based on 3GPP simulation methodology. Agreement As a starting point, the training, validation and testing dataset are from the same large-scale and small-scale propagation parameters setting. Subsequent evaluation can study the performance when the training dataset and testing dataset are from different settings. Agreement For AI/ML-based positioning evaluation, RAN1 does not attempt to define any common AI/ML model as a baseline. Agreement The entry “UE horizontal drop procedure” in the simulation parameter table for InF is updated to the following. UE horizontal drop procedure; Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be selected from - (baseline) the whole hall area, and the CDF values for positioning accuracy is obtained from whole hall area. - (optional) the convex hull of the horizontal BS deployment, and the CDF values for positioning accuracy is obtained from the convex hull.. Agreement The entries “UE antenna height” and “gNB antenna height” in the simulation parameter table for InF is updated to the following. UE antenna height; Baseline: 1.5m (Optional): uniformly distributed within [0.5, X2]m, where X2 = 2m for scenario 1(InF-SH) and X2= for scenario 2 (InF-DH). …; …. gNB antenna height; Baseline: 8m (Optional): two fixed heights, either {4, 8} m, or {max(4,), 8}.. Agreement If spatial consistency is enabled for the evaluation, companies model at least one of: large scale parameters, small scale parameters and absolute time of arrival, where - the large scale parameters are according to Section 7.5 of TR 38.901 and correlation distance = for InF (Section 7.6.3.1 of TR 38.901) - the small scale parameters are according to Section 7.6.3.1 of TR 38.901 - the absolute time of arrival is according to Section 7.6.9 of TR 38.901 Agreement If spatial consistency is enabled for the evaluation of AI/ML based positioning, the baseline evaluation does not incorporate spatially consistent UT/BS mobility modelling (Section 7.6.3.2 of TR 38.901). - It is optional to implement spatially consistent UT/BS mobility modelling (Section 7.6.3.2 of TR 38.901). Agreement For evaluation of AI/ML based positioning, companies are encouraged to evaluate the model generalization. - FFS: the metrics for evaluating the model generalization (e.g., model performance based on agreed KPIs under different settings) Agreement Companies are encouraged to provide evaluation results for: - Direct AI/ML positioning - Companies are encouraged to describe at least the following implementation details for the evaluation - details of the channel observation used as the input of the AI/ML model inference (e.g., type and size of model input), model input acquisition and pre-processing - AI/ML assisted positioning - Companies are encouraged to describe at least the following implementation details for the evaluation - details of the channel observation used as the input of the AI/ML model inference (e.g., type and size of model input), model input acquisition and pre-processing - details of the output of the AI/ML model inference, how the AI/ML model output is used to obtain the UE’s location Agreement When reporting evaluation results with direct AI/ML positioning and/or AI/ML assisted positioning, proponent company is expected to describe if a one-sided model or a two-sided model is used. - If one-sided model (i.e., UE-side model or network-side model), the proponent company report which side the model inference is performed (e.g. UE, network), and any details specific to the side that performs the AI/ML model inference. - If two-sided model, the proponent company report which side (e.g., UE, network) performs the first part of interference, and which side (e.g., network, UE) performs the remaining part of the inference. Agreement For evaluation of AI/ML based positioning, the computational complexity can be reported via the metric of floating point operations (FLOPs). - Note: For AI/ML assisted methods, computational complexity for the AI/ML model is only one component of the overall complexity for estimating the UE’s location. - Note: Other metrics to measure the computational complexity are not precluded. Agreement For evaluation of AI/ML based positioning, details of the training dataset generation are to be reported by proponent company. The report may include (in addition to other selected settings, if applicable): - The size of training dataset, for example, the total number of UEs in the evaluation area for generating training dataset; - The distribution of UE location for generating the training dataset may be one of the following: - Option 1: grid distribution, i.e., one training data is collected at the center of one small square grid, where, for example, the width of the square grid can be 0.25/0.5/1.0 m. - Option 2: uniform distribution, i.e., the UE location is randomly and uniformly distributed in the evaluation area.

Agreements reached in 3GPP RAN1#110 meeting

Agreement For AI/ML-based positioning, both approaches below are studied and evaluated by RAN1: - Direct AI/ML positioning - AI/ML assisted positioning Agreement For AI/ML-based positioning, study impact from implementation imperfections. Agreement For evaluation of AI/ML based positioning, the model complexity is reported via the metric of “number of model parameters”. Agreement To investigate the model generalization capability, at least the following aspect(s) are considered for the evaluation for AI/ML based positioning: - Different drops - Training dataset from drops {A0, A1,…, AN-1}, test dataset from unseen drop(s) (i.e., different drop(s) than any in {A0, A1,…, AN-1}). Here N>=1. - Clutter parameters, e.g., training dataset from one clutter parameter (e.g., {40%, 2m, 2m}), test dataset from a different clutter parameter (e.g., {60%, 6m, 2m}); - Network synchronization error, e.g., training dataset without network synchronization error, test dataset with network synchronization error; - Other aspects are not excluded. Note: It’s up to participating companies to decide whether to evaluate one aspect at a time, or evaluate multiple aspects at the same time. Agreement When providing evaluation results for AI/ML based positioning, participating companies are expected to describe data labelling details, including: - Meaning of the label (e.g., UE coordinates; binary identifier of LOS/NLOS; ToA) - Percentage of training data without label, if incomplete labeling is considered in the evaluation - Imperfection of the ground truth labels, if any Agreement For evaluation of AI/ML based positioning, study the performance impact from availability of the ground truth labels (i.e., some training data may not have ground truth labels). The learning algorithm (e.g., supervised learning, semi-supervised learning, unsupervised learning) is reported by participating companies. Agreement For AI/ML-based positioning, for evaluation of the potential performance benefits of model finetuning, report at least the following: - training dataset setting (e.g., training dataset size necessary for performing model finetuning) - horizontal positioning accuracy (in meters) before and after model finetuning. Agreement For both direct AI/ML positioning and AI/ML assisted positioning, the following table is adopted for reporting the evaluation results. Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [with or without] model generalization, [short model description] Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Model complexity; Computational complexity; AI/ML. To report the following in table caption: - Which side the model is deployed - Model generalization investigation, if applied - Short model description: e.g., CNN Further info for the columns: - Model input: input type and size - Model output: output type and size - Label: meaning of ground truth label; percentage of training data set without label if data labeling issue is investigated (default = 0%) - Clutter parameter: e.g., {60%, 6m, 2m} - Dataset size, both the size of training/validation dataset and the size of test dataset - AI/ML complexity: both model complexity in terms of “number of model parameters”, and computational complexity in terms of FLOPs - Horizontal positioning accuracy: the accuracy (in meters) of the AI/ML based method Note: To report other simulation assumptions, if any. Agreement For evaluation of AI/ML assisted positioning, an intermediate performance metric of model output is reported. - FFS: Detailed definition of the intermediate performance metric of the model output Agreement To investigate the model generalization capability, the following aspect is also considered for the evaluation of AI/ML based positioning: (d) UE/gNB RX and TX timing error. - The baseline non-AI/ML method may enable the Rel-17 enhancement features (e.g., UE Rx TEG, UE RxTx TEG).

Agreements reached in 3GPP RAN1#110-bis-e meeting

Agreement To investigate the model generalization capability, the following aspect is also considered for the evaluation of AI/ML based positioning: (e) InF scenarios, e.g., training dataset from one InF scenario (e.g., InF-DH), test dataset from a different InF scenario (e.g., InF-HH) Agreement For both direct AI/ML positioning and AI/ML assisted positioning, if fine-tuning is not evaluated, the template agreed in RAN1#110 is updated to the following for reporting the evaluation results. Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [short model description] Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. Agreement For both direct AI/ML positioning and AI/ML assisted positioning, if fine-tuning is evaluated, the template agreed in RAN1#110 is updated to the following for reporting the evaluation results. Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [short model description] Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; test; Model complexity; Computation complexity; AI/ML. Agreement For AI/ML-assisted positioning, companies report which construction is applied in their evaluation: - Single-TRP construction: the input of the ML model is the channel measurement between the target UE and a single TRP, and the output of the ML model is for the same pair of UE and TRP. - Multi-TRP construction: the input of the ML model contains N sets of channel measurements between the target UE and N (N>1) TRPs, and the output of the ML model contains N sets of values, one for each of the N TRPs. Note: For a measurement (e.g., RSTD) which is a relative value between a given TRP and a reference TRP, the TRP in “single-TRP” and “multi-TRP” refers to the given TRP only. Note: For single-TRP construction, companies report whether they consider same model for all TRPs or N different models for TRPs Conclusion For evaluation of AI/ML based positioning, suspend the discussion on intra-site (or zone-specific) variations until concepts and channel model construction not in TR38.901 (e.g., “intra-site” or “zone”) are clarified under AI 9.2.1. - Note: An individual company can still submit evaluation results for intra-site variation. Conclusion For evaluation of AI/ML based positioning, the sampling period is selected by proponent companies. Each company report the sampling period used in their evaluation. Agreement For evaluation of AI/ML assisted positioning, the following intermediate performance metrics are used: - LOS classification accuracy, if the model output includes LOS/NLOS indicator of hard values, where the LOS/NLOS indicator is generated for a link between UE and TRP; - Timing estimation accuracy (expressed in meters), if the model output includes timing estimation (e.g., ToA, RSTD). - Angle estimation accuracy (in degrees), if the model output includes angle estimation (e.g., AoA, AoD). - Companies provide info on how LOS classification accuracy and timing/angle estimation accuracy are estimated, if the ML output is a soft value that represents a probability distribution (e.g., probability of LOS, probability of timing, probability of angle, mean and variance of timing/angle, etc.) Conclusion For evaluation of AI/ML based positioning, it’s up to each company to take into account the channel estimation error in their evaluation. Companies describe the details of their simulation assumption, e.g., realistic or ideal channel estimation, error models, receiver algorithms. Agreement For AI/ML assisted positioning, when single-TRP construction is used for the AI/ML model, companies report at least the AI/ML complexity (Model complexity, Computation complexity) for N TRPs, which are used to determine the position of a target UE. Table. Model complexity and computation complexity to support N TRPs for a target UE Model complexity to support N TRPs; Computation complexity to process N TRPs. Single-TRP, same model for N TRPs; PS When the model is at UE-side, where PS is the model complexity for the same model. FFS: if the model is at network-side; N×CS Where CS is the computation complexity of the same model for one TRP.. Single-TRP, N models for N TRPs; When the model is at UE-side, i=1,…NPS,i Where PS,i is the model complexity for the i-th AI/ML model. FFS: if the model is at network-side; i=1,…NCS,i Where CS,i is the computation complexity for the i-th AI/ML model.. Multi-TRP (i.e., one model for N TRPs); PM Where PM is the model complexity for the one model.; CM Where CM is the computation complexity for the one model.. Agreement For AI/ML based positioning, if an InF scenario different from InF-DH is evaluated for the model generalization capability, the selected parameters (e.g., clutter parameters) are compliant with TR 38.901 Table 7.2-4 (Evaluation parameters for InF). - Note: In TR 38.857 Table 6.1-1 (Parameters common to InF scenarios), InF-SH scenario uses the clutter parameter {20%, 2m, 10m} which is compliant with TR 38.901. Agreement For the model input used in evalutions of AI/ML based positioning, if time-domain channel impulse response (CIR) or power delay profile (PDP) is used as model input in the evaluation, companies report the input dimension NTRP * Nport * Nt, where NTRP is the number of TRPs, Nport is the number of transmit/receive antenna port pairs, Nt is the number of time domain samples. - Note: CIR and PDP may have different dimensions. - Note: Companies provide details on their assumption on how PDP is constructed and how (if applicable) it is mapped to Nt samples.

Agreements reached in 3GPP RAN1#111 meeting

Agreement Study how AI/ML positioning accuracy is affected by: user density/size of the training dataset. Note: details of user density/size of training dataset to be reported in the evaluation. Agreement For reporting the model input dimension NTRP * Nport * Nt of CIR and PDP, Nt refers to the first Nt consecutive time domain samples. - If N’t (N’t < Nt) samples with the strongest power are selected as model input, with remaining (Nt ‒ N’t) time domain samples set to zero, then companies report value N’t in addition to Nt. It is also assumed that timing info for the N’t samples need to be provided as model input. Agreement For reporting the model input dimension NTRP * Nport * Nt: - If the model input is CIR, then each input value of CIR is a complex number, i.e. it contains two real values, either {real, imaginary} or {magnitude, phase}. - If the model input is PDP, then each input value of PDP is a real value. Agreement At least for model inference of AI/ML assisted positioning, evaluate and report the AI/ML model output, including (a) the type of information (e.g., ToA, RSTD, AoD, AoA, LOS/NLOS indicator) to use as model output, (b) soft information vs hard information, (c) whether the model output can reuse existing measurement report (e.g., NRPPa, LPP). Agreement For AI/ML assisted positioning, evaluate the three constructions: - Single-TRP, same model for N TRPs - Single-TRP, N models for N TRPs - Multi-TRP (i.e., one model for N TRPs) Note: Individual company may evaluate one or more of the three constructions. Agreement For AI/ML assisted approach, study the performance of model monitoring metrics at least where the metrics are obtained from inference accuracy of model output. Agreement For both direct and AI/ML assisted positioning methods, investigate at least the impact of the amount of fine-tuning data on the positioning accuracy of the fine-tuned model. - The fine-tuning data is the training dataset from the target deployment scenario. Agreement For the RAN1#110bis agreement on the calculation of model complexity, the FFS are resolved with the following update: Model complexity to support N TRPs. Single-TRP, same model for N TRPs; PS where PS is the model complexity for one TRP and the same model is used for N TRPs.. Single-TRP, N models for N TRPs; i=1,…NPS,i Where PS,i is the model complexity for the i-th AI/ML model.. Note: The reported model complexity above is intended for inference and may not be directly applicable to complexity of other LCM aspects. Observation Direct AI/ML positioning can significantly improve the positioning accuracy compared to existing RAT-dependent positioning methods when the generalization aspects are not considered. - For InF-DH with clutter parameter setting {60%, 6m, 2m}, evaluation results submitted to RAN1#111 indicate that the direct AI/ML positioning can achieve horizontal positioning accuracy of <1m at CDF=90%, as compared to >15m for conventional positioning method. Agreement For AI/ML based positioning, company optionally evaluate the impact of at least the following issues related to measurements on the positioning accuracy of the AI/ML model. The simulation assumptions reflecting these issues are up to companies. - SNR mismatch (i.e., SNR when training data are collected is different from SNR when model inference is performed). - Time varying changes (e.g., mobility of clutter objects in the environment) - Channel estimation error Conclusion Companies describe how their computational complexity values are obtained. - It is out of 3GPP scope to consider computational complexity values that have platform-dependency and/or use implementation (hardware and software) optimization solutions. Observation AI/ML assisted positioning can significantly improve the positioning accuracy compared to existing RAT-dependent positioning methods when the generalization aspects are not considered. - For InF-DH with clutter parameter setting {40%, 2m, 2m}, evaluation results submitted to RAN1#111 indicate that the AI/ML assisted positioning can achieve horizontal positioning accuracy of <0.4m at CDF=90%, as compared to >9m for conventional positioning method. - For InF-DH with clutter parameter setting {60%, 6m, 2m}, evaluation results submitted to RAN1#111 indicate that the AI/ML assisted positioning can achieve horizontal positioning accuracy of <1m at CDF=90%, as compared to >15m for conventional positioning method. Note: how to capture the observation(s) into TR is separate discussion. Agreement - For AI/ML assisted approach, for a given AI/ML model design (e.g., input, output, single-TRP vs multi-TRP), identify the generalization aspects where model fine-tuning/mixed training dataset/model switching is necessary.

Agreements reached in 3GPP RAN1#112 meeting

Agreement For both direct AI/ML positioning and AI/ML assisted positioning, companies include the evaluation area in their reporting template, assuming the same evaluation area is used for training dataset and test dataset. Note: - Baseline evaluation area for InF-DH = 120x60 m. - if different evaluation areas are used for training dataset and test dataset, they are marked out separately under “Train” and “Test” instead. Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [with or without] model generalization, [short model description], UE distribution area = [e.g., 120x60 m, 100x40 m] Model input; Model output; Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Train; Test; Model complexity; Computation complexity; AI/ML. Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [short model description], UE distribution area = [e.g., 120x60 m, 100x40 m] Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; Test; Model complexity; Computation complexity; AI/ML. Agreement The agreement made in RAN1#110 AI 9.2.4.1 is updated by adding additional note: Note: if complex value is used in modelling process, the number of the model parameters is doubled, which is also applicable for other AIs of AI/ML Agreement For both the direct AI/ML positioning and AI/ML assisted positioning, study the model input, considering the tradeoff among model performance, model complexity and computational complexity. - The type of information to use as model input. The candidates include at least: time-domain CIR, PDP. - The dimension of model input in terms of NTRP, Nt, and Nt’. - Note: For the direct AI/ML positioning, model input size has impact to signaling overhead for model inference. Agreement For direct AI/ML positioning, study the performance of model monitoring methods, including: - Label based methods, where ground truth label (or its approximation) is provided for monitoring the accuracy of model output. - Label-free methods, where model monitoring does not require ground truth label (or its approximation). Agreement For AI/ML assisted approach, study the performance of label-free model monitoring methods, which do not require ground truth label (or its approximation) for model monitoring. Conclusion - No dedicated evaluation is needed for the positioning accuracy performance of model switching - It does not preclude future discussion on model switching related performance Agreement For direct AI/ML positioning, study the impact of labelling error to positioning accuracy - The ground truth label error in each dimension of x-axis and y-axis can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of L meters, with truncation of the distribution to the [-2*L, 2*L] range. - Value L is up to sources. - Other models are not precluded - [Whether/how to study the impact of labelling error to label-based model monitoring methods] - [Whether/how to study the impact of labelling error for AI/ML assisted positioning.] Observation Evaluation of the following generalization aspects show that the positioning accuracy of direct AI/ML positioning deteriorates when the AI/ML model is trained with dataset of one deployment scenario, while tested with dataset of a different deployment scenario. - The generalization aspects include: - Different drops - Different clutter parameters - Different InF scenarios - Network synchronization error - Companies have provided evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning/re-training. - Better training dataset construction: The training dataset is composed of data from multiple deployment scenarios, which include data from the same deployment scenario as the test dataset. - Model fine-tuning/re-training: the model is re-trained/fine-tuned with a dataset from the same deployment scenario as the test dataset. Note: ideal model training and switching may provide the upper bound of achievable performance when the AI/ML model needs to handle different deployment scenarios.

Agreements reached in 3GPP RAN1#112bis-e meeting

Agreement For evaluation of both the direct AI/ML positioning and AI/ML assisted positioning, company optionally adopt delay profile (DP) as a type of information for model input. - DP is a degenerated version of PDP, where the path power is not provided. Agreement For the evaluation of AI/ML based positioning, the study of model input due to different number of TRPs include the following approaches. Proponent of each approach provide analysis for model performance, signaling overhead (including training data collection and model inference), model complexity and computational complexity. - Approach 1: Model input size stays constant as NTRP=18. The number of TRPs (N’TRP) that provide measurements to model input varies. When N’TRP < NTRP, the remaining (NTRP - N’TRP) TRPs do not provide measurements to model input, i.e., measurement value is set to 0. - Approach 1-A. The set of TRPs (N’TRP) that provide measurements is fixed. - Approach 1-B. The set of TRPs (N’TRP) that provide measurements can change dynamically. - Note: for Approach 1, one model is provided to cover the entire evaluation area. - Approach 2: The TRP dimension of model input is equal to the number of TRPs (N’TRP) that provide measurements as model input. When N’TRP < NTRP, the remaining (NTRP - N’TRP) TRPs are ignored by the given model. For a given AI/ML model, the set of TRPs (N’TRP) that provide measurements is fixed. - For Approach 2: one model can be provided to cover the entire evaluation area, which is equivalent to deploying N’TRP TRPs in the evaluation area for positioning if ignoring the potential inference from the remaining (18 - N’TRP) TRPs. - For Approach 2, if Nmodel (Nmodel >1) models are provided to cover the entire evaluation area, the total complexity (model complexity is the summation of the Nmodel models. Agreement In the evaluation of AI/ML based positioning, if N’TRP<18, the set of N’TRP TRPs that provide measurements to model input of an AI/ML model are reported using the TRP indices shown below. Agreement For AI/ML assisted positioning with TOA as model output, study the impact of labelling error to TOA accuracy and/or positioning accuracy. - The ground truth label error of TOA is calculated based on location error. The location error in each dimension of x-axis and y-axis can be modelled as a truncated Gaussian distribution with zero mean and standard deviation of L meters, with truncation of the distribution to the [-2*L, 2*L] range. - Value L is up to sources. - Other models of labelling error are not precluded - Other timing information, e.g., RSTD, as model output is not precluded. Agreement For AI/ML assisted positioning with LOS/NLOS indicator as model output, study the impact of labelling error to LOS/NLOS indicator accuracy and/or positioning accuracy. - The ground truth label error of LOS/NLOS indicator can be modelled as m% LOS label error and n% NLOS label error. - Value m and n are up to sources. - Companies consider at least hard-value LOS/NLOS indicator as model output. Agreement For the evaluation of AI/ML based positioning method, the measurement size and signalling overhead for the model input is reported. Observation For AI/ML based positioning method, companies have submitted evaluation results to show that for their evaluated cases, for a given company’s model design, a lower complexity (model complexity and computational complexity) model can still achieve acceptable positioning accuracy (e.g., <1m), albeit degraded, when compared to a higher complexity model. Note: For easy reference, sources include CMCC (R1-2303228), InterDigital (R1-2303450), Ericsson (R1-2302335), Huawei/HiSilicon (R1-2302362), CATT (R1-2302699), Nokia (R1-2302632). Observation For direct AI/ML positioning, for L in the range of 0.25m to 5m, the positioning error increases approximately in proportion to L, where L (in meters) is the standard deviation of truncated Gaussian Distribution of the ground truth label error. Observation For AI/ML assisted positioning, evaluation results have been provided by sources for label-based model monitoring methods. With TOA and/or LOS/NLOS indicator as model output, the estimated ground truth label (i.e., TOA and/or LOS/NLOS indicator) is provided by the location estimation from the associated conventional positioning method. The associated conventional positioning method refers to the method which utilizes the AI/ML model output to determine target UE location. Note: Sources include vivo (R1-2302481), MediaTek (R1-2303340), Ericsson (R1-2302335) Observation For both direct AI/ML and AI/ML assisted positioning, evaluation results have been provided by sources to demonstrate the feasibility of label-free model monitoring methods. Note: Sources include vivo (R1-2302481), CATT (R1-2302699), MediaTek (R1-2303340), Ericsson (R1-2302335), Nokia (R1-2302632).