3GPP TSG RAN WG1 #113		                            R1-2304698

Incheon, Korea, May 22nd – May 26th, 2023

Source:	CATT

Title:	Discussion on switching time for DL PRS or UL SRS frequency hopping for RedCap UEs

Agenda Item:	5

Document for:	Discussion and Decision

Introduction

In the LS [1], RAN4 has following one question for RAN1:

RAN4 also thinks additional switch time may be needed for SRS transmission between the initial/active BWP to the first hop and last hop to initial/active BWP, and therefore, has below question: - Is the additional switch time for SRS transmission between the initial/active BWP to first hop and switch time between last hop to the initial/active BWP relevant for RedCap frequency hopping and should it be discussed in RAN4?

In this contribution, we discuss the issue and share our view on the response to the RAN4 LS.

Discussion

Additional switch time for SRS transmission

In our view, the potential benefit of enhanced switch time for SRS transmission between the initial/active BWP to first hop and enhanced switch time between last hop to the initial/active BWP is to reduce positioning latency of RedCap UEs. It has no impact on RedCap UEs positioning accuracy. However, currently there is no latency requirement for RedCap UEs positioning in Rel-18. From RAN1’s perspective, there’s no need to take into account an enhanced switch time in Rel-18. If there are latency requirements for RedCap UEs positioning in future release, it may be further considered to support additional switch time for SRS transmission between the initial/active BWP to first hop and switch time between last hop to the initial/active BWP.

Proposal 1: From RAN1’s perspective, there’s no need to support an enhanced switch time for SRS transmission between the initial/active BWP to first hop and switch time between last hop to the initial/active BWP relevant for RedCap frequency hopping in Rel-18.

Conclusion

In this contribution, we discussed the response to RAN4’s LS on switching time for DL PRS or UL SRS frequency hopping for RedCap UEs. Based on the discussion, we have the following proposal:

Proposal 1: From RAN1’s perspective, there’s no need to support an enhanced switch time for SRS transmission between the initial/active BWP to first hop and switch time between last hop to the initial/active BWP relevant for RedCap frequency hopping in Rel-18.

References

- R1-2304316 (R4-2306659), LS reply on switching time for DL PRS or UL SRS frequency hopping for RedCap UEs, RAN4, Ericsson.