3GPP TSG RAN WG1 #113			R1-2304701

Incheon, Korea, 22nd – 26th May, 2023

# Source:             CATT

Title:    	Discussion on RAN4 Reply LS on LP-WUR architecture

Agenda Item:    5

Document for:  Discussion and Decision

# Introduction

RAN4 had sent an LS in R1-2304312 (R4-2306618) on the channel BW assumed in LP-WUR architecture [1] . RAN4 had agreed on the framework of RF evaluation of LP-WUS.  RAN4 asked RAN1 several questions for clarification on the assumptions of channel BW of LP-WUS in the evaluation of LP-WUR.   In this paper, we discuss RAN1’s assumptions on the channel BW of low-power wakeup receiver and the proposed answers to RAN4’s questions.

# Discussion of RAN1 Assumptions on the BW of LP-WUS and the proposed replies to RAN4’s questions

RAN4 evaluates the RF impacts of LP-WUR architecture.  RAN4 reach further agreements on the framework and selected scenarios for next-step RF evaluation.  RAN4 has the question to RAN1 on the WUS channel BW as follows,

- Whether the case when the WUS/WUR is same as NR channel bandwidth, e.g. 5MHz WUS within 5MHz NR CBW (Max 25 RBs/15kHz SCS), is considered for LP-WUS/WUR evaluation.

RAN1 had agreed to assume the BW of the LP-WUS no greater than 5 or 20 MHz in RAN1#111 without any specific BW as follows,

Agreement

For the purpose of study, the BW of one LP-WUS is not greater than X (FFS X is 5 or 20) MHz for FR1, study further

- whether BW of LP-WUS is configurable (implicitly or explicitly)

- size of guard band [FFS: within or outside of BW X], if any

- whether there is different X for Idle, Connected, Inactive modes

In RAN1#112bis-e, the LP-WUS BW was discussion in response to RAN4 LS R1-2302287(R4-2303712) on LP-WUR architecture.   The discussion focuses on RAN1 agreements with the conclusion of using the agreed evaluation assumptions as the reply to RAN4 [2]  as follows,

Agreement

The number of PRBs within the LP-WUS BW would depend on the SCS used for the LP-WUS signals.   There is no agreements yet on the SCS and the number of PRBs used for LP-WUS.   Thus, the number of PRBs and the SCS are not yet agreed to be evaluated.

| LP-WUS BW | Option 1: - 5MHz including subcarriers for guard band - 4.32MHz (i.e.,12 RBs) for LP-WUS transmission for 30kHz SCS Option 2: - {2.16, 4.32} MHz including subcarriers for guard band - 1.44MHz, 2.88MHz (i.e.{4, 8} RBs) for LP-WUS transmission for 30kHz SCS FFS: other options are up to companies to report GB is symmetrically placed on each side of LP-WUS |
| --- | --- |
| Filter | X-th Order filter (e.g. Butterworth, Chebyshev, …) with Y MHz bandwidth, - X = {3, 5} - Companies to report Y Companies to report any other assumptions if needed |

From RAN1 discussion and reply LS to RAN4 in [2] , the RF channel BW was part of the LP-WUS BW in the evaluation assumption.  There is no further discussion and LS reply to RAN4 on the RF BW and number of PRBs used for LP-WUS.

Proposal 1:  There is no need in further discussion and LS reply to RAN4 on the RF BW and number of PRBs used for LP-WUS.

# Conclusion

The RAN4 LS to RAN1 on the RF BW of LP-WUS/LP-WUR was discussed in RAN1#112bis-e with reply in [2] .  The question of whether the BW of LP-WUS/WUR is same as that of NR channel BW has been included in the reply LS [2].  The proposal of RAN4 question in [1] is as follows,

- Proposal 1:  There is no need in further discussion and LS reply to RAN4 on the RF BW and number of PRBs used for LP-WUS/LP-WUR.

# Reference

- R1-2304312, “LS to RAN1 on low-power wake-up receiver architectures”, RAN4, vivo.

- R1-2304251, “Reply LS to RAN4 on LP WUR architectures”, RAN1, Apple