3GPP TSG RAN WG1 #113                                                                                   R1- 2304716

Incheon, Korea, May 22nd – May 26th, 2023

Source:              CATT

Title:                   Physical layer signals and procedures for Low-Power WUR

Agenda Item:     9.11.3

Document for:   Discussion and Decision

Introduction

In RAN1#112bis-e, the follows was agreed for studying the low-power wake-up signal and procedures [1]:

Agreement - Capture in TR: From RAN1 perspective, LP-WUS and signals/channels used by MR can be within the same FR1 band. - At least LP-WUS and signals/channels by MR can be on the same carrier in the band - Study further - Whether LP-WUS and signals/channels used by MR can be different carriers in the band - Details on the LP-WUS location within a carrier - Band can be different than band of signals/channels used by MR - LP-WUS association with BWP - LP-WUS can be configurable within guard-band of a band (like NB-IoT) Agreement Update the RAN1#112 agreement as the following: - [time/frequency resources (including any guard bands), if applicable] - [total energy of LP-WUS across the time/frequency resources] Working assumption: In place of the above deleted bullets: - Alt 1: - average EPRE within the [time]/frequency resources used for LP-WUS (including any guard bands) - time/frequency resources used for LP-WUS (including any guard bands) Alt 2: - average EPRE within the [time]/frequency resources used for LP-WUS (including any guard bands)SNR is calculated as average EPRE divided by power of noise [and interference]. - Companies to report whether and how power pooling across and within MR OFDMA symbols is used. - FFS: PAPR applicable to LP-WUS Agreement Replace in RAN1#112 agreement Companies to report - power consumption of the MR if false alarm probability/rate not fixed across MC-ASK, MC-FSK, and CP-OFDMA waveforms with - receiver architecture type and its relative power consumption Agreement - For IDLE/INACTIVE mode study at least following candidates for content of LP-WUS - information on which user(s) is/are targeted by the LP-WUS - e.g. UE-group, -subgroup or -ID - FFS: cell information - FFS: SI change and ETWS/CMAS information, tracking area information, and RAN area information - For CONNECTED mode, study at least following candidates for content of LP-WUS - information on which user(s) is/are targeted by the LP-WUS - e.g UE-group, -subgroup or -ID - indication to wake-up to PDCCH monitoring. - Other information candidates are not precluded - Study pros and cons of including above information to LP-WUS. - Note: the information may be explicitly or implicitly indicated. Agreement - For RRC connected mode, the following is assumed for LP-WUS study in RAN1 - RLM/BFD/CSI are performed by UE Main Radio (MR) - RRM measurements are performed by UE Main Radio (MR) - Ultra-deep sleep state is not allowed for MR. - Study additional support of RRM measurement by LP-WUR for RRC connected mode - Study RRC connected mode LP-WUS functionality/purpose/procedures - Study RRC connected mode LP-WUS activation/deactivation procedures. - Study RRC connected mode LP-WUS BW, whether same as IDLE/Inactive mode or different - In RRC connected, study the relationship between LP-WUS and legacy UE power saving techniques. Agreement - Study further following alternatives to carry the LP-WUS information using: - Alt 1: by sequence(s) detection/selection - FFS sequence type - Alt 2: by encoded bits - FFS: what type of encoding scheme - FFS: with or without other bits (e.g. CRC/FCS) - Other alternatives are not precluded - Study whether LP-WUS information needs to be preceded by known one or more sequence(s). Agreement At least for IDLE/Inactive mode, at least one BW-size <=5MHz is recommended to be supported for FR1 - Other BW sizes are not precluded - if additional BW-size(s) are recommended to be supported, BW-size can be up to 20MHz - LP-WUS bandwidth size (including guard-bands) is assumed to be an integer number of PRBs Agreement Study further methods to modulate input signal of the DFT/Least-Square block for OOK-4, and methods to modulate input signal of N SCs for other MC-ASK/FSK schemes - study methods with respect to - improving frequency diversity by flattening the spectrum, frequency repetition and frequency hopping - impact to dynamic range of RE power in frequency domain - FFS: impact to PAPR of generated time domain modulated MC-ASK/FSK symbol - improving robustness to timing error necessary spectrum adjustment for compatibility with CP-OFDM generation Agreement - Study techniques/mechanisms to enhance coverage performance of LP-WUS - Study potential gains available as well as drawback(s) of the technique(s)/mechanisms(s), e.g. system overhead, increased complexity network energy consumption etc… - Study potential issues and corresponding solutions for the case when LP-WUS coverage is insufficient - At least study fallback mechanisms where the Main Radio switches to legacy operation in case the channel condition of LP-WUS is not sufficient, e.g. below threshold.

This paper discussed the aspects of physical channel/signals for the low-power wakeup receiver (LP-WUR), the system design associated with low-power wake up signal (LP-WUS) and waveform, and the related procedure of using LP-WUR to achieve UE power saving.

Signal waveform for LP-WUR and system design

Signal waveform and modulation design for LP-WUR

The design of the signals and waveform of LP-WUS is critical in the power consumption of the LP-WUR. For MC-ASK waveform and MC-FSK waveform generation, several options were discussed in RAN1#111 and RAN1#112. In this section, few remaining issues are discussed as follows.

LP-WUS waveform and modulation design for LP-WUR

In RAN1#112, four options (i.e. OOK-1/2/3/4) for MC-ASK waveform generation in time and frequency domain were discussed. How to modulate the SCs for MC-ASK waveform generation should be further discussed. In current NR system, the PSK or QAM modulated with OFDM are used for current NR transmissions. To minimize the system impact and coexist with legacy UEs, the waveform of LP-WUS should be designed to be well compatible with the current OFDM waveform in DL. The OFDM modulated waveform to generate narrowband signals in the form of subcarriers should be considered for the waveform of the LP-WUS in order to multiplex the LP-WUS with other NR channels in the transmissions of the same carrier.

- MC-FSK waveform

Two options of FSK waveform generation, i.e., FSK-1/2 were discussed in RAN1#112 for MC-FSK waveform generation. FSK waveform by modulating multiple information bits directly into the one M-level FSK waveform or each bit in one sub-bands or M total sub-bands within the LP-WUS BW. M bits transmission per OFDM symbol of LP-WUS could support 14*M kbps data rate for 15 kHz SCS, e.g. 8 bits per OFDM symbol for 112 kbps rate for 15 kHz SCS.

- OFDMA-based signals/channels

The waveforms of OFDMA-based signals/channels are the baseline waveform for NR. The OFDMA or SC-FDMA waveform for LP-WUS would be another NR signal. In AI-9.11.2, The OFDMA-based receiver architecture was discussed and agreed as one of the LP-WUR architectures. The OFDM-based architecture is notarized with its high complexity and high power consumption comparing to that of OOK/FSK based receivers due to the requirement of precise RF processing and high complexity digital baseband processing. The power model of the OFDM-based receiver should have the baseline assumptions of reusing the power model of CSI-RS or PDCCH processing in Rel-16 UE power saving in TR38.840v2.0.0. The new power model of OFDM-based receiver needs to be addressed comparing to that of NR receiver before the OFDM-based receiver is captured in TR as an example if the OFDM-based LP-WUR could simplify the receiver architecture with reducing power consumption. More details can be found in our companion paper [2].

Proposal 1: The power model of the OFDM-based receiver needs to be addressed comparing to that of NR receiver before the OFDM-based receiver is captured in TR as an example.

Link performance of OOK and FSK waveform

In this subsection, the link level performance of OOK and FSK waveform are investigated. The OOK-1 and FSK-1 generation for OOK and FSK waveform are evaluated, respectively. For demodulation, the envelop detection is involved i.e. the LP-WUR demodulates LP-WUS by comparing the received energy to a detection threshold. More details of simulation assumption are summarized in Table 1 and Table 2 in Appendix.

Figure 1: The link performance of OOK and FSK under various SNR

From Figure 1, it can be seen that the OOK-1 with Manchester coding shows better link performance than that of FSK-1 since it is more accurate when demodulate the modulated LP-WUS than OOK-1 without Manchester coding. However, the coding rate decreases at 50%.

Observation 1:  OOK-1 with Manchester coding shows better link performance than that of FSK-1.

System design associated with LP-WUS signals and waveform

About system design for LP-WUS, it is necessary to support the LP-WUS coexistence with other NR legacy channels. The considerations of the system design associated with LP-WUS signals and waveform are summarized as follows,

  - Inter-channel interference (ICI) from other channel

  - Pulse shape filter

  - UE Periodic or continuous transmission of LP-WUS

  - Reference timing from local oscillator or slave-drive by NR oscillator

  - SCS of LP-WUS

The details of above considerations are discussed as following subsections.

Inter-channel interference from other channel

The LP-WUS design should take into account of ICI from/to the neighboring NR channels, e.g., PDCCH, PDSCH, CSI-RS and SSB, when LP-WUR is operating at the same NR carrier. When LP-WUS multiplexed with NR at the NR operation band, the ICI from/to existing NR channel/signal will decrease LP-WUS detection performance. Therefore, LP-WUS design target should be considered to reduce the ICI between LP-WUS and NR signals when they are multiplexed at the NR carrier. Guard band is one interference mitigation technique to avoid interference from/to adjacent neighboring channels.

Proposal 2: It is expected to support TDM and FDM multiplexing between LP-WUS and NR legacy channel/signal. The LP-WUS design should take into account inter-channel interference mitigation techniques when LP-WUS and NR signals are multiplexed in the same NR carrier.

Pulse shaping filter

The pulse shape filter is the filter at the transmitter to limit the transmission signals rolling off to the adjacent channels.  One example of the pulse shaping filter is the raised-cosine filter as shown in Figure 2. The raised-cosine filter is a filter frequently used for pulse-shaping in digital modulation due to its ability to minimize the ICI. The raise cosine function from the fact that the non-zero portion of the frequency spectrum of its simplest form (β =1) is a cosine function, 'raised' up to sit above the horizontal function. The pulse shaping filter would minimize the transmitted signal to the adjacent channels at the transmitter. Thus, the LP-WUS should take the pulse shape filter into consideration as the requirement to limit the ICI to the adjacent channel.

Figure 2: An example of Raise Cosine Filter as the pulse sharping filter for LP-WUS with different roll-off factor

Proposal 3: The LP-WUS should take the pulse shape filter into consideration as the requirement to limit the ICI to the adjacent channel.

UE periodic and continuous monitoring of LP-WUS

In RAN1#112, the LP-WUS monitoring procedure includes contiguous monitoring and periodic monitoring occasions. The following Figure 3 and Figure 4 show the different UE monitoring types of LP-WUS.

Figure 3: UE Periodic monitoring of LP-WUS

Figure 4: UE Continuous monitoring of LP-WUS

- Option1: The duty-cycle based LP-WUS associated with the DRX and PO would allow the LP WUR turning ON and OFF. The unlocked free running oscillator used for LP-WUR expects to be inexpensive and has loose frequency stability in the range of ±50- 80 ppm, which is far away from UE frequency stability requirement of ±0.1 ppm.  If the duty cycle based LP-WUR would need to turn ON and OFF to align with the periodicity of DRX or PO, the frequency stability of LP-WUR for the detection of wakeup signal needs to be calibrated and adjusted to reduce the effect caused by frequency drifting for better detection performance. However, the power consumption of wakeup receiver would increase due to the increase of power consumption of external components if the oscillator is used for LP-WUR to in lock with the NR clock for accurate detection of the wakeup signals.

- Option2: The low-power device need perform constant monitoring of wakeup signal at the configured LP-WUS transmission occasions in preparation of UE wakeup without the LP-WUR turning ON and OFF. The power consumption of front-end wakeup device for on-demand network access would need to be very low.

Proposal 4: The behavior of  LP-WUS monitoring can be rely on the UE configuration of monitoring the LP-WUS transmission occasions with duty-cycle to align the duty-cycle with the periodicity of DRX for RRC_CONNECTED mode UEs or PO for RRC_IDLE/INACTIVE mode UEs.

Reference timing from local oscillator or slave-drive by NR oscillator

The time drift/frequency drift wander of reference clock for reference timing from local oscillator or slave-drive by NR would result in timing error/frequency error for LP-WUS monitoring. The error would impact the LP-WUS monitoring window length and even introduce unnecessary wake-up or miss the WUS detection for a UE. It is shown in following Figure 5.

Figure 5: Illustration of timing error

From Figure 5, duty-cycle based LP-WUS associated with the DRX and PO would allow the turning ON and OFF of the LP-WUR for the power saving. The unlocked oscillator used for LP-WUR is inexpensive and has loose frequency stability in the range of ±50- 80 ppm, which far away from UE frequency stability requirement of ±0.1 ppm. If the duty cycle based LP-WUR would need to turn ON and OFF to align with the periodicity of DRX or PO, the frequency stability of LP-WUR for the detection of wakeup signal needs to be calibrated and adjusted to reduce the effect caused by frequency drifting for better detection performance. However, the power consumption of wakeup receiver would increase due to the increase of power consumption of external components if the oscillator used for LP-WUR would need to in lock with the NR clock for accurate detection of the wakeup signals.

Therefore, the periodic synchronization to reference timing is needed to address for reducing the timing error/frequency error. Considering the system design, the enough time resource for LP-WUS monitoring window needs to be reserved.

Proposal 5: The timing error/frequency error caused by drift and wander of reference clock would impact the system design for LP-WUS monitoring and detection. The enhancement to reduce the timing error/frequency error for synchronization and LP-WUS monitoring window should to be studied.

SCS of LP-WUS

In RAN1#112, the SCS of a CP-OFDM symbol used for LP-WUS generation were discussed for MC-ASK or MC-FSK waveform generation. The SCS used for LP-WUS and the SCS used for other NR transmissions can be same or different. The transmission waveform of LP-WUS does not need to be modulated with OFDM for the narrowband signals.  The non-OFDM modulated waveform is a wideband signals without the given SCS.  For OOK/FSK waveform, the allocated frequency resource for LP-WUS can be divided in to different granularity for each waveform, such as granularity in one or more PRBs or subcarriers. In this way, the IFFT can be omitted in transmitter for LP-WUS. For example, when the SCS of other NR transmissions are 15KHz and 30KHz, the PRB of LP-WUS can be 180KHz and 360KHz, respectively.

Proposal 6: The SCS used for LP-WUS generation and the SCS used for other NR channels/signals in the same carrier can be different.

When the SCS of the LP-WUS is different to SCS used by other NR channels/signals transmitted on the same CP-OFDM symbol, there is no additional complexity at gNB in waveform generation and multiplexing of different numerology, which is supported since Rel-15. The only consideration is the inter-symbol interference between LP-WUS and NR signals/channels when they multiplexed on the same OFDM symbol with different waveforms, which is different to the handling of multiplexing BWPs with different numerologies. The SCS is only significant to OFDM-based waveform and irrelevant to OOK/FSK modulation. The LP-WUR receiver should be implementation choice of reusing existing hardware or standalone hardware to reduce the UE’s power consumptions.

Observation 2: There is negligible impact to the gNB and the UE when the SCS of LP-WUS is different to SCS used for other NR channel/signals.

Physical signals/channel design

Network support of LP-WUR/LP-WUS

The LP-WUR architecture considers several alternatives, which include the RF-based envelop detection, IF-based envelop detection and BB-based envelop detection to trigger the activation/deactivation of the NR receiver at the UE. The considerations of the network operation for achieving UE power saving by LP-WUR with limited receiver sensitivity or restricted maximum coupling loss are discussed as follows.

System support of LP-WUR needs to inform UEs through broadcast channels

The power consumption of the LP-WUR depends on the receiver architecture and its achievable receiver sensitivity. The receiver sensitivity in dBm would determine the maximum range of the coupling loss/path loss in the radio propagation from the gNB with a given Tx power in dBm, which could derive the maximum coverage area. Since the LP-WUR intends to prioritize the UE power saving, the coverage area of 164 dB coupling loss with the support of data rate at least 160 bps for DL/UL defined in NR system [3], could not be reused. The LP-WUR is hard to achieve the receiver sensitivity over -100 dBm with data rate at 160 bps with the power consumption target in the range of µW or nW. Thus, only selected network deployment scenarios could support the limited coverage aspect of LP-WUR, such as indoor hotspot or dense urban coverage scenarios with inter-site distance less than 200 meters. The network support of UE wakeup mechanism by LP-WUR needs to inform all UEs by broadcasting the configuration of wakeup signals through SIB-1 or SIB-x at a given cell.

Proposal 7: The network support of UE wakeup mechanism by LP-WUR needs to inform all UEs by broadcast the configuration of wakeup signals through SIB-1 or SIB-x at a given cell.

UE capability of LP-WUR support

On the other hand, the UE capability should have the capability of the implemented LP-WUR and the associated receiver sensitivity information for the coverage area of supported UE wakeup mechanism. UE will report its supported category of LP-WUR in the UE capability. The network would configure the UE wakeup mechanism by LP-WUR if network supports the category of LP-WUR in the deployment.

Proposal 8:  UE capability includes the receiver sensitivity of the LP -WUR.  UE will report it’s supported of LP-WUR in the UE capability.

Specification of waveform and configuration of LP-WUS

The signals and waveform are critical in the power consumption of LP-WUR. The MC-ASK (including OOK) waveform, MC-FSK waveforms, and OFDM-based signals/channels are discussed in RAN1#111. In general, the waveform of the wakeup signal for the LP-WUR should be designed with the target of minimizing power consumption and not restricted to the NR OFDM-based waveform. Thus, it is expected that network should support the potential LP-WUS waveform and configure the LP-WUS based on supporting waveform.

Proposal 9:  Network should support the LP-WUS waveform configuration.

Configuration of LP-WUS

The NR network should configure and transmit the DL signals designed for the LP-WUR in order to support the UE wakeup mechanism by the LP-WUR in achieving the UE power saving. The network configuration of the wakeup signal transmission for the LP-WUR is discussed as follows.

Configuration of LP-WUS resource

For LP-WUS resource, time information and frequency information for LP-WUS should be configured. Furthermore, cell-specific or UE-specific configuration of LP-WUS resource could be supported. For cell-specific configuration of LP-WUS resource, all UEs associated to the cell would detect LP-WUS which results in unnecessary MR wake-up. For UE-specific configuration of LP-WUS resource, the UE would detect LP-WUS based on the specific resource and decrease the false alarm rate. However, the network overhead would be increased with the increasing of the number of UE. Therefore, the tradeoff between LP-WUS resource overhead and LP-WUS detection performance needs to be considered.

Proposal 10:  Cell-specific or UE-specific configuration of LP-WUS resource could be supported.

Content of LP-WUS

For RRC_CONNECTED mode UEs, each UE has its specific traffic and data arrival time. The unique UE-ID could be configured in a cell. However, the unique UE-ID would increase LP-WUS information length and network overhead depending on the range of uniqueness of the UE-ID (e.g., uniqueness in global, region, or local area). Therefore, it could be further considered on the unique UE-ID within a group of cells (e.g., registration area). For RRC_IDLE/INACTIVE mode UEs, amount of UEs should be supported for paging. Base on the test and analysis in Rel-17 UE power saving enhancement, paging rate could be low to 1% in current network [4]. So the UE in RRC_IDLE/INACTIVE mode could be divided in multiple groups, and each group can be configured with a wake-up signal. Other information (e.g. cell information, SI change and ETWS/CMAS information, tracking area information, and RAN area information) should not be carried in LP-WUS.  The cell information will be needed for RRM procedure, and SI change and ETWS/CMAS information, tracking area information. The RAN PLMN area information should be considered in LP-WUS if LP-WUS replace the function of paging. However, LP-WUS is used as the reference signal for RRM measurements would require additional capability in coverage and interference rejection for multi-cell identification as discussed in section 4.2. The LP-WUS needs to be designed to carry more bits in order to provide the all information carried by paging message, which will increase the UE power consumption.

Proposal 11:  For RRC_CONNECTED mode UEs, the LP-WUS should be designed to support the wakeup indication with unique UE-ID.

Proposal 12: For RRC_IDLE/INACTIVE mode UEs, the LP-WUS should be designed to support the early indication of paging of a paging group/sub-group without including any paging message, such as SI change indication, ETWS/CMAS information, tracking area, and RAN PLMN area information.

Bandwidth of LP-WUS

In RAN1#112bis-e, it was agreed that at least one BW-size <=5MHz is recommended to be supported for RRC_IDLE/INACTIVE mode. While, for RRC_CONNECTED mode, the initial BWP could be configured 20MHz. The LP-WUS BW needs to be specified as default or broadcast in the system information for RRC_IDLE/INACTIVE mode UE. Therefore, the LP-WUS bandwidth could be configured differently between RRC_CONNECTED and RRC_IDLE/INACTIVE modes. Furthermore, the LP-WUS BW should include the guard-band between LP-WUS and other NR signals/channels or other LP-WUS.

Proposal 13: The LP-WUS bandwidth could be configured differently between RRC_CONNECTED and RRC_IDLE/INACTIVE modes.

Proposal 14: Same or different BW between RRC_CONNECTED and RRC_IDLE/INACTIVE mode should be supported.

Placement of LP-WUS

In RAN1#112 and RAN1#112bis-e, the placement of LP-WUS was discussed. The frequency location of LP-WUS should be flexible as that of SSB to enable the gNB configuration in placing the LP-WUS at the lower edge or upper edge of the carrier BWP in minimizing the inter-channel interference to neighboring NR channels/signals. Therefore, the frequency location of LP-WUS is flexibly configured by the gNB to improve resource utilize efficiency.

Proposal 15: The frequency location of LP-WUS can be flexibly configured by the gNB.

Physical functionalities and procedures

Synchronization

After MR turn into ultra-deep sleep state, the local oscillator for MR might not provide the reference timing information to the LP-WUR. The LP-WUR would use the reference time of its own local oscillator with frequency stability of ±80 ppm during the long ultra-deep sleep state of MR receiver. The drift and wander of the free running clock of LP-WUR would accumulate large time and frequency errors in the timing reference generated for the LP-WUR. The RF based LP-WUR architecture can perform RF envelop detection of LP-WUS continuous with the tolerance of timing error caused the drifting of the timing reference from the local oscillator. Thus, there is no need of synchronizing with the network for the RF envelope detector in the receiving additional synchronisation signals. However, baseband-based LP-WUR architecture and OFDM-based LP-WUR architecture need to acquire synchronisation signal for calibrating the drift of local clock and oscillator. In RAN1#112, three options for synchronisation signal used by LP-WUR are provided as following:

- Option 1: A preamble signal can be transmitted as part of LP-WUS. When data arrived at gNB, the preamble would be arrived along with LP-WUS to the LP-WUR. Since the preamble is a known reference signals, the LP-WUR would use the detected preamble to derive the timing and frequency offset for the compensation to the LP-WUS before detection. The length of preamble should be sufficiently long to improve the synchronization accuracy.

- Option 2: The synchronization signals are transmitted periodically and separately from LP-WUS to allow LP-WUR synchronizing with the gNB. The periodicity of the synchronization signals does not need to be aligned with the periodicity of the LP-WUS. In particular, the LP-WUS could accumulate multiple measurements of synchronization signals to improve the synchronization accuracy. The LP-WUR can use the synchronization signals to synchronize with the gNB, and to detect the LP-WUS for triggering wakeup of the MR receiver. In this case, the synchronization signals needs to be transmitted periodically.

- Option 3: The LP-WUR would use the periodic synchronization signals for coarse synchronisation. LP-WUR can further perform fine synchronization from the preamble signal before the LP-WUS. Thus, the synchronization signals could have sparse transmission occasion with less resource overhead.

In conclusion, Option 3 provides the flexibility in the network configuration of synchronization signals and preambles to achieve more accurate synchronisation performance with less resource overhead.

Proposal 16: The RF envelop detector in the LP-WUR architecture does not require accurate timing information for the detection of LP-WUS. Option 3 provides the flexibility in the network configuration of synchronization signals and preambles to achieve obtain more accurate synchronisation performance with less resource overhead.

RRM measurement and procedures

RRM measurements are used for UE mobility management in both RRC_CONNECTED and RRC_IDLE/INACTIVE mode.  As discussed in RAN1#112, the RRM measurements can be performed on SSB by MR or LP-WUS by LP WUR, respectively.

- Alt.1: RRM measurement on SSB

The LP-WUS is used as the wakeup indication of the NR MR only and not used for RRM measurements. The RRM measurement metric, such as SS-RSRP and SS-RSRQ, would be triggered by LP-WUS to be measured by the NR MR on the SSB of each cell. In this case, the LP-WUS signal does not need to include the Cell ID information. The LP-WUR does not need to perform multiple hypothesis testing of cell detection and RRM measurements. An example is shown in Figure 6 on RRM measurements at NR MR triggered by LP-WUS. The RRM measurement includes serving cell measurement and neighboring cells (i.e. intra-frequency cells and inter-frequency cells) measurements. To reduce the power consumption, the serving cell and neighboring cells measurements can be relaxed, e.g. confining the RRM measurement within a measurement window, increasing the periodicity of the measurement window and reducing the number of sample with in a measurement period.

Figure 6: RRM measurement on SSB

- Alt.2: RRM measurements on LP-WUS

Another alternative is that the LP-WUS is used as a reference signal for RRM measurements. The LP WUR offloads the RRM measurements from MR at least during NR MR being OFF, as shown in Figure 7. Few issues should be solved in Alt.2 and the standard effort in RAN1, RAN2 and RAN4 would be large. Firstly, in order to support RRM measurements by LP-WUS, the LP-WUS should contain the Cell ID (e.g. 10 bits) information, which increases the payload of the LP-WUS. Secondly, if the LP WUR also performs the neighboring cells measurements, LP-WUS needs to have the co-channel interference rejection capability among LP-WUS signals from serving cell and neighboring cells. The LP-WUR needs to detect the LP-WUS from one cell but also from neighboring cell with multiple hypothesis. Moreover, UE would not be able to measure neighboring cells when the LP-WUS coverage is smaller than that of SSB. Considering the coverage hole between LP-WUS and legacy NR signal, the RRM measurement used for mobility management becomes insignificant when UE is out of the LP -WUS coverage. A UE behavior (e.g. wake-up the NR MR and fallback to legacy mobility procedures) needs to be defined when UE ends-up in a coverage hole of LP-WUS.  Last but not least, the signal power and signal quality of LP-WUS e.g. LP-RSRP, LP-RSRQ and the new criterions for cell selection and reselection should be further discussed.

Figure 7: RRM measurements on LP-WUS

Proposal 17: For a UE support LP-WUR in IDLE/INACTIVE mode, RRM measurements on LP-WUS should not be supported.

The reference metric of RRM measurements, such as LP-RSSI, LP-RSRP, LP-RSRQ and LP-SINR as discussed in RAN1#112bis-e, needs to be decided after the waveform and the contents of LP-WUS is decided. The reference metric of RRM measurements is used to support the UE mobility for network access among cells, such as handover, cell selection/re-selection and inter-cell interference management. The reference metric of RRM measurements requires the specification of LP-WUS waveform and the associated receiver types. Thus, the granularity of the resource and the reference metric used for the RRM measurements depend on the LP-WUS waveform and LP-WUR receiver types.

Proposal 18: The granularity of the resource and the reference metric used for RRM measurements depend on the LP-WUS waveform and LP-WUR receiver types.

Conclusion

In this contribution, we have discussed the UE wakeup mechanism and the design of the physical layer structure and procedure in support of the LP-WUR in achieving UE power saving. We have the following observation and proposals:

Observation 1:  OOK-1 with Manchester coding shows better link performance than that of FSK-1.

Observation 2: There is negligible impact to the gNB and the UE when the SCS of LP-WUS is different to SCS used for other NR channel/signals.

Proposal 1: The power model of the OFDM-based receiver needs to be addressed comparing to that of NR receiver before the OFDM-based receiver is captured in TR as an example.

Proposal 2: It is expected to support TDM and FDM multiplexing between LP-WUS and NR legacy channel/signal. The LP-WUS design should take into account inter-channel interference mitigation techniques when LP-WUS and NR signals are multiplexed in the same NR carrier.

Proposal 3: The LP-WUS should take the pulse shape filter into consideration as the requirement to limit the ICI to the adjacent channel.

Proposal 4: The behavior of  LP-WUS monitoring can be rely on the UE configuration of monitoring the LP-WUS transmission occasions with duty-cycle to align the duty-cycle with the periodicity of DRX for RRC_CONNECTED mode UEs or PO for RRC_IDLE/INACTIVE mode UEs.

Proposal 5: The timing error/frequency error caused by drift and wander of reference clock would impact the system design for LP-WUS monitoring and detection. The enhancement to reduce the timing error/frequency error for synchronization and LP-WUS monitoring window should to be studied.

Proposal 6: The SCS used for LP-WUS generation and the SCS used for other NR channels/signals in the same carrier can be different.

Proposal 7: The network support of UE wakeup mechanism by LP-WUR needs to inform all UEs by broadcast the configuration of wakeup signals through SIB-1 or SIB-x at a given cell.

Proposal 8:  UE capability includes the receiver sensitivity of the low-power wakeup receiver.  UE will report it’s supported of low-power wakeup receiver in the UE capability.

Proposal 9:  Network should support the LP-WUS waveform configuration.

Proposal 10:  Cell-specific or UE-specific configuration of LP-WUS resource could be supported.

Proposal 11:  For RRC_CONNECTED mode UEs, the LP-WUS should be designed to support the wakeup indication with unique UE-ID.

Proposal 12: For RRC_IDLE/INACTIVE mode UEs, the LP-WUS should be designed to support the early indication of paging of a paging group/sub-group without including any paging message, such as SI change indication, ETWS/CMAS information, tracking area, and RAN PLMN area information.

Proposal 13: The LP-WUS bandwidth could be configured differently between RRC_CONNECTED and RRC_IDLE/INACTIVE modes.

Proposal 14: Same or different BW between RRC_CONNECTED and RRC_IDLE/INACTIVE mode should be supported.

Proposal 15: The frequency location of LP-WUS can be flexibly configured by the gNB.

Proposal 16: The RF envelop detector in the LP-WUR architecture does not require accurate timing information for the detection of LP-WUS. Option 3 provides the flexibility in the network configuration of synchronization signals and preambles to achieve obtain more accurate synchronisation performance with less resource overhead.

Proposal 17: For a UE support LP-WUR in IDLE/INACTIVE mode, RRM measurements on LP-WUS should not be supported.

Proposal 18: The granularity of the resource and the reference metric used for RRM measurements depend on the LP-WUS waveform and LP-WUR receiver types.

References

- RP-2304414, Summary #3 of discussions on L1 signal design and procedure for low power, Nordic, RAN1#112bis-e, April 17th – April 26th, 2023.

- R1-2304716, Low-Power WUS receiver Architectures and its performance, CATT, RAN1#113, May 22nd – May 26th, 2023.

- TR38.913, Study on Scenarios and Requirements for Next Generation Access Technologies, V15.0.0.

- TR38.840, Study on UE Power Saving, V16.0.0.

Appendix

Table 1: Simulation assumption

| Attributes | Assumption |
| --- | --- |
| Carrier Frequency | 2.6GHz |
| Waveform | OOK |
| Information bit | 8 bits |
| WUS duration | Number of OFDM symbol: OOK-1: 8 |
| SCS of LP WUS | 30KHz |
| Code scheme | Non/ Manchester code (1/2) |
| LP-WUS BW | 5MHZ |
| ADC bit width | 1 |
| Sampling rate | 15.36MHz |
| Channel model | TDL-C 300ns |
| FAR | 1% |

Table 2: Simulation assumption

| Parameter | Assumptions |
| --- | --- |
| Carrier frequency | 2.6GHz |
| Waveform | FSK |
| Information bit | 8bits |
| WUS duration | FSK-1:8 |
| Signal Bandwidth | 5MHz |
| SCS of LP WUS | 30kHz |
| Sampling Rate | 15.36MHz |
| Filter | X=3;Y=4.32 |
| ADC bit width | Ideal |
| Time/Frequency synchronization | Ideal |
| Channel Model | TDL-C 300ns |
| FAR | 1% |