3GPP TSG RAN WG1 #113                                            R1-2304755

Incheon , KR , May 22nd – May 26th, 2023

Agenda Item:	9.14.1

Source:	CATT

Title:	Evaluation methodology and performance consideration for IMT2020 Satellite RIT

Document for:	Discussion and Decision

Introduction

In 3GPP TSG RAN #99 meeting, a new study item named “Study on self-evaluation towards the 3GPP submission of an IMT-2020 satellite radio interface technology” has been approved in release 18 stage [1]. This study item will provide the description of the self-evaluation results towards IMT-2020 submission to ITU-R WP 4B against the technical performance requirements defined by Report ITU-R M.2514, using the evaluation criteria defined in the report, and complete the related compliance template and description templates [2].

At the last meeting, companies discussed the simulation conditions and parameters and reached the following agreement.

Agreement The proposals and proposed working assumption in Section 2 of R1-2304118 are endorsed.

In this document, following the proposals and proposed working assumption in Section 2 of R1-2304118, we provide partial simulation results.

Discussion of evaluation methodology

  - Peak data rate and spectral efficiency

At the last meeting, the parameters of the peak data rate are chosen based on “ideal conditions”. The SNR is a realistic parameter that has to be considered. In the last meeting, the peak spectral efficiency is considered to be calculated as

SEp=vLayers⋅Qm⋅f⋅RmaxNPRBBW,μ⋅12Tsμ1-OHBW

And the peak data rate is calculated as Rp=BW×SEp, assuming single carrier operation. Assuming  Qm and Rmax are selected according to the maximum value in Table 5.1.3.1-1 and Table 6.1.4.1-1 in TS 38.214, the SNR should be higher than 19.2dB according to our simulation results which is difficult to implement in NTN scenario. Therefore, we believe that the relevant parameters of link budget should be clarified in this meeting to determine  Qm and Rmax in NTN scenario.

- The link budget should be clarified to facilitate calculation of peak data rate with realistic parameter.

  - LLS Reliability

Table 1 Simulation parameters for LLS Reliability

| NR | Updated moderator proposal |  |
| --- | --- | --- |
| Physical channel | PUSCH | PDSCH |
| Simulation bandwidth | 1.44MHz (8PRB) | 1.44MHz (8PRB) |
| SCS | 15kHz | 15kHz |
| Waveform | DFT-S-OFDM | CP-OFDM |
| Number of users in simulation | 1 | 1 |
| Link-level Channel model | NTN TDL-C Rural | NTN TDL-C Rural |
| Antenna configuration at Satellite | 1Rx | 1Tx |
| Antenna configuration at UE | 1Tx | 1Rx |
| Transmission mode | SISO | SISO |
| Transmission rank | 1 | 1 |
| TBS | 256 | 256 |
| Modulation order | QPSK | QPSK |
| Number of repetition |  |  |
| Channel estimation | LMMSE | LMMSE |
| Channel coding scheme | LDPC | LDPC |
| Doppler spread | 5Hz | 5Hz |
| UL DMRS config | 2 DMRS per slot | Type 1, 2 symbol DMRS |

For reliability evaluations, LLS parameters for NR can be configured as Table 1. We recommend that the uplink simulation bandwidth should be set to 1.44MHz (8PRB for SCS =15kHz). Given that both the TBS of uplink and downlink are 256bit, we recommend that the downlink simulation bandwidth should be configured to be 1.44MHz which is same as the uplink. The waveform configuration also needs to be declared in the simulation condition. DFT-S-OFDM is recommended for uplink waveform, which is a more typical configuration for uplink, and the waveform for downlink is configured as CP-OFDM.

- The simulation bandwidth of uplink and downlink should be set to 1.44MHz for LLS Reliability.

- When configuring the simulation bandwidth, the SCS should be declared.

- The waveform is configured as DFT-S-OFDM for Uplink and CP-OFDM for Downlink.

  - LLS Mobility

Table 2 Simulation parameters for LLS Mobility

| NR Uplink | Updated moderator proposal |
| --- | --- |
| Physical channel | PUSCH |
| Simulation bandwidth | 1.44MHz (8PRB) |
| SCS | 15kHz |
| Waveform | DFT-S-OFDM |
| Number of users in simulation | 1 |
| Link-level Channel model | NTN TDL-C Rural |
| Antenna configuration at Satellite | 1Rx |
| Antenna configuration at UE | 1Tx |
| Transmission mode | SISO |
| Transmission rank | 1 |
| TBS |  |
| Modulation order | QPSK |
| Number of repetition |  |
| Channel estimation | LMMSE |
| Channel coding scheme | LDPC |
| Doppler spread | 463Hz |
| UL DMRS config | 2 symbol DMRS (front loaded and one additional) with configuration type 2, no FDM with data and full power utilization in DMRS symbols |

For mobility evaluations, LLS parameters for NR can be configured as Table 2. We recommend that the uplink simulation bandwidth should be set to 1.44MHz which is same as that of reliability evaluations. At this point, the only difference between the mobility evaluation and the reliability evaluation is the Doppler spread. Such a configuration is beneficial to evaluate the impact of mobility on performance.

- The simulation bandwidth for mobility evaluations should be set as same as that of reliability evaluation, that is beneficial to evaluate the impact of mobility on performance.

  - LLS mMTC

Table 3 Simulation parameters for LLS mMTC

| NR Uplink | Value |
| --- | --- |
| Physical channel | PUSCH |
| Simulation bandwidth | 1 PRB |
| SCS | 15kHz |
| Waveform | DFT-S-OFDM |
| Number of users in simulation | 1 |
| Link-level Channel model | NTN TDL-C Rural |
| Antenna configuration at Satellite | 1Rx |
| Antenna configuration at UE | 1Tx |
| Transmission mode | SISO |
| Transmission rank | 1 |
| TBS | 32 |
| Modulation order | QPSK |
| Number of repetition | 1,2,4,8 |
| Channel estimation | LMMSE |
| Channel coding scheme | LPDC |
| Doppler spread | 5Hz |
| UL DMRS config | 2 DMRS per slot |

For mMTC evaluations, LLS parameters for NR Uplink can be configured as Table 3. When the simulation bandwidth is configured to 1 PRB, it is difficult to implement TBS of 256bit with modulation order of QPSK according to Table 6.1.4.1-1 in TS 38.214. Therefore, we think that the TBS configured as 32 is more reasonable.

- The TBS configured as 32 is more reasonable for mMTC NR Uplink evaluations.

Partial simulation results for NR NTN

  - LLS Reliability

Figure 1 Simulation result of Reliability NR Uplink

The simulation result of Reliability NR Uplink is shown in Figure 1, and the simulation parameters are shown in Table 1. According to our simulation results, NR Uplink requires about SNR=-3.2dB to achieve a BLER 10-1 and SNR=1dB to achieve a BLER 10-2 for LLS Reliability.

- NR Uplink requires about SNR=-3.2dB to achieve a BLER 10-1 and SNR=1dB to achieve a BLER 10-2 for LLS Reliability.

Figure 2 Simulation result of Reliability NR Downlink

The simulation result of Reliability NR Downlink is shown in Figure 2, and the simulation parameters are shown in Table 1. According to our simulation results, NR Downlink requires about SNR=-3.5dB to achieve a BLER 10-1 and SNR=-1.3dB to achieve a BLER 10-2 for LLS Reliability.

- NR Downlink requires about SNR=-3.5dB to achieve a BLER 10-1 and SNR=-1.3dB to achieve a BLER 10-2 for LLS Reliability.

  - LLS Mobility

Figure 3 Simulation result of Mobility NR Uplink

The simulation result of Mobility NR Uplink is shown in Figure 3, and the simulation parameters are shown in Table 2. According to our simulation results, NR Uplink requires about SNR=0.5dB to achieve a BLER 10-1 and SNR=4.5dB to achieve a BLER 10-2 for LLS Mobility. The deterioration in performance due to Doppler spread caused by mobility is about 3.5dB.

- NR Uplink requires about SNR=0.5dB to achieve a BLER 10-1 and SNR=4.5dB to achieve a BLER 10-2 for LLS Mobility. The deterioration in performance due to Doppler spread caused by mobility is about 3.5dB.

  - LLS mMTC

Figure 4 Simulation result of mMTC NR Uplink

The simulation result of mMTC NR Uplink is shown in Figure 4, and the simulation parameters are shown in Table 3. According to our simulation results, NR Uplink requires about SNR=-1.2dB to achieve a BLER 10-1 and SNR=2.8dB to achieve a BLER 10-2 for LLS mMTC without repetition. NR Uplink requires about SNR=-4.8dB to achieve a BLER 10-1 and SNR=-2.4dB to achieve a BLER 10-2 for LLS mMTC with 2 repetitions. NR Uplink requires about SNR=-8.1dB to achieve a BLER 10-1 and SNR=-6.5dB to achieve a BLER 10-2 for LLS mMTC with 4 repetitions. NR Uplink requires about SNR=-11.4.1dB to achieve a BLER 10-1 and SNR=-10dB to achieve a BLER 10-2 for LLS mMTC with 8 repetitions.

- NR Uplink requires about SNR=-1.2dB to achieve a BLER 10-1 and SNR=2.8dB to achieve a BLER 10-2 for LLS mMTC without repetition. NR Uplink requires about SNR=-11.4.1dB to achieve a BLER 10-1 and SNR=-10dB to achieve a BLER 10-2 for LLS mMTC with 8 repetitions.

Conclusion

In this contribution, the evaluation methodolgy of IMT-2020 SRI technolgoy has been discussed, and some simulation results are provided. A few of observations and proposals are made as follows:

- The link budget should be clarified to facilitate calculation of peak data rate with realistic parameter.

- The simulation bandwidth of uplink and downlink should be set to 1.44MHz for LLS Reliability.

- When configuring the simulation bandwidth, the SCS should be declared.

- The waveform is configured as DFT-S-OFDM for Uplink and CP-OFDM for Downlink.

- The simulation bandwidth for mobility evaluations should be set as same as that of reliability evaluation, that is beneficial to evaluate the impact of mobility on performance.

- The TBS configured as 32 is more reasonable for mMTC NR Uplink evaluations.

- NR Uplink requires about SNR=-3.2dB to achieve a BLER 10-1 and SNR=1dB to achieve a BLER 10-2 for LLS Reliability.

- NR Downlink requires about SNR=-3.5dB to achieve a BLER 10-1 and SNR=-1.3dB to achieve a BLER 10-2 for LLS Reliability.

- NR Uplink requires about SNR=0.5dB to achieve a BLER 10-1 and SNR=4.5dB to achieve a BLER 10-2 for LLS Mobility. The deterioration in performance due to Doppler spread caused by mobility is about 3.5dB.

- NR Uplink requires about SNR=-11.4.1dB to achieve a BLER 10-1 and SNR=-10dB to achieve a BLER 10-2 for LLS mMTC with 8 repetitions.

References

- RP-230736, Study on Self-Evaluation towards the 3GPP submission of an IMT-2020 Satellite Radio Interface Technology, Ericsson, Qualcomm, Thales, MediaTek Inc.

- Report ITU-R M.2514-0, Vision, requirements and evaluation guidelines for satellite radio interface(s) of IMT-2020, ITU

- TR 38.821, Solutions for NR to support non-terrestrial networks (NTN) (Release 16), 3GPP

- TS 38.214, Physical layer procedures for data (Release 17), 3GPP

- TR 38.811, Study on New Radio (NR) to support non-terrestrial networks (Release 15), 3GPP

- R1-1802551, UE antenna assumption and beam modelling for NTN, Nokia, 3GPP

- TR 36.763, Study on Narrow-band Internet of Things (NB-IOT) / enhanced machine type communication (eMTC) support for non-terrestrial networks (NTN) (Release 17), 3GPP