3GPP TSG RAN WG1 #113                                                R1-2304757

# Incheon, Korea, May 22nd – May 26th, 2023

Title:	Draft reply LS to RAN2 on unchanged PCI

Response to:	R1-2304323(R2-2304273)

Release:	Rel-18

Work Item:	NR_NTN_enh-Core

Source:	CATT

To:	RAN2

Cc:

Attachment:	no

Contact Person:

Name:	Deshan Miao

E-mail Address:	miaodeshan@catt.cn

- Overall description

RAN1 thanks RAN2 for the LS R1-2304323(R2-2304273) on unchanged PCI issue and would like to provide the following response to the question from RAN2:

For mobility enhancement in Rel-18 NR NTN, in quasi-earth fixed cell case, RAN2 has discussed unchanged PCI scenario, including hard satellite switching (non-overlapping satellite coverage at switching time) and soft satellite switching (overlapping satellite coverage at switching time).

RAN2 has agreed that, in quasi-earth fixed cell case, for hard satellite switch in the same SSB frequency and same gNB (no key change), satellite switching without PCI changing (not requiring L3 mobility) is supported, unless major technical issues are identified by RAN1. RAN2 understands that the standard impact includes that the UE may be notified to re-acquire DL/UL synchronization with the serving cell after the satellite switching.

RAN2 understands that the feasibility of soft satellite switching without PCI change (not requiring L3 mobility) is more relevant to RAN1 aspects, and would like to check its feasibility with RAN1.

Questions:

RAN2 kindly requests RAN1 to take into account the above agreement on hard satellite switching without PCI change and provide feedback if RAN1 identifies any major technical issues, and also provide feedback on the feasibility to support soft satellite switching without PCI change.

Reply:

From RAN1’s perspective, the technical feasibility for hard satellite switching and soft switching is as follows:

For hard satellite switching, no major technique issue is identified.

For soft satellite switching, UE is assumed to connect to one satellite during soft satellite switching period. With proper resource coordination between serving satellite and upcoming satellite through network implementation, soft satellite switching without PCI change is feasible.

- Actions:

To RAN2:

RAN1 respectfully requests RAN2 to take the above information into account.

- Date of Next TSG-RAN1 Meetings:

RAN1#114		     	21th -25th Aug 2023	   			Toulouse, FR

RAN1#114bis			09th -13rd Oct 2023	   		      	Xiamen, CN