3GPP TSG RAN WG1 #113	R1-2304777

Incheon, Korea, May 22nd – May 26th, 2023

Source: 	Fujitsu

Title:	Discussion on UE features for NR NCR

Agenda item:	9.16.4

Document for:	Discussion and decision

- Introduction

This contribution will discuss UE features for NR NCR. Some relevant agreements in RAN1#112b-e meeting are listed in the Appendix.

- Discussion

2.1 FG 43-1 Basic NCR support

One remaining issue for FG 43-1 is whether to define candidate values for component 5 “Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link”.

Since RAN1 has agreed new signaling for internal delay is not needed, we don’t think candidate values are needed.

Agreement Update the agreement achieved in RAN1#109e as follows: For the signaling of the side control information of timing to align transmission / reception boundaries, new signaling may be is unnecessary. - FFS: the impact of internal delay

Observation 1: RAN1 has agreed new signaling for internal delay is not needed.

Proposal 1: For FG 43-1, do not define candidate values for component 5 and capability signalling.

- Adopt the following updates (highlighted in green).

| 43. NR_netcon_repeater | 43-1 | Basic NCR support [for C-link and backhaul link] | 1. Support of fixed beam for C-link/backhaul link 2. Support of TDMed UL transmission of C-link and backhaul link [3. Support of ON-OFF operation for NCR-Fwd based on access link beam indication] [4. Support of TDD UL/DL determination for backhaul/access link based on TDD UL/DL configuration of C-link] [5. [Support of] Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link] 6. Support of beam correspondence of the DL/UL of the access link at NCR-Fwd [7. Support of backhaul link beam determination based on predefined rule] |  | N/A | Yes | NCR is not supported | Per NCR-MT | No | No | Yes | A NCR-MT that includes ncr-NodeIndication in RRC Setup Complete must support FG 43-1 [Component 3 candidate values: FFS] [Component 4 candidate values: FFS] | Optional without capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

2.2 FG 43-3 Aperiodic beam indication for access link

For FG 43-3, one FFS is about candidate values for component 2.

| 43. NR_netcon_repeater | 43-3 | Aperiodic beam indication for access link | 1.Support aperiodic beam indication for access link [2. Supported slot-offset k values for reference slot] | 43-1 | N/A | Yes | NCR-MT cannot decode the aperiodic beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one FFS: component 2 candidate values | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Similar to timeDurationForQCL, the slot-offset k is to define the minimum time duration required by NCR-MT to perform DCI reception and apply the beams indicated by the DCI. So, the candidate values for slot-offset k can be defined with reference to timeDurationForQCL.

In the legacy, timeDurationForQCL is defined only for FR2. The value range per SCS for FR2-1 is shown in Table 1.

Table 1

| SCS (kHz) | Value range (number of symbols) of timeDurationForQCL |
| --- | --- |
| 60 | 7, 14, 28 |
| 120 | 14, 28 |

Therefore, for slot-offset k, as the starting point, the value range per SCS can be as shown in Table 2, which is simply determined by scaling based on timeDurationForQCL.

Table 2

| SCS (kHz) | Value range (number of slots) of slot-offset k |
| --- | --- |
| 15 | 1 |
| 30 | 1 |
| 60 | 1,2 |
| 120 | 1, 2 |

Observation 2: The slot-offset k is to define the minimum time duration in the number of slots required by NCR-MT to perform DCI reception and apply the beams indicated by the DCI. The candidate value for slot-offset k can be defined with reference to timeDurationForQCL.

Proposal 2: For FG 43-3, as the starting point, the candidate values for slot-offset k can be:

- 15 kHz: {1}

- 30 kHz: {1}

- 60 kHz: {1, 2}

- 120 kHz: {1, 2}

2.3 FG 43-8 Adaptive beam

As shown in the table below, the component of FG 43-8 is “Support of backhaul link beam determination based on predefined rule”. And it is FFS whether/how to support candidate values.

| 43. NR_netcon_repeater | 43-1a8 | Adaptive beam for NCR backhaul link[/C-link] |  | 1. [Support adaptive beam for NCR C-link] 2. Support [explicit] adaptive beam for NCR backhaul link Support of backhaul link beam determination based on predefined rule | 43-1, 2-2,2-4, 2-4a | N/A | Yes | The beam for backhaul link and C-link is fixed. | Per NCR-MT | No | No | Yes | FFS: Component candidate values: {Rel-15/16, Rel-17, both} FFS: relation between 43-1a and existing UE features for beam operation | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The agreements related to the “predefined rule” are as below. In the agreements, the rule highlighted in yellow is common for Rel-15/16 and Rel-17 beam indication framework for C-link, the rule highlighted in pink is dedicated for Rel-15/16 beam indication framework for C-link, and the rule highlighted in cyan is dedicated for Rel-17 beam indication framework for C-link.

Agreement The following pre-defined rules are applied to determine the beam for backhaul link: - In the time domain resource with simultaneous downlink reception or uplink transmission in C-link and backhaul link, the beam of backhaul link is the same as the beam of C-link regardless whether there is beam indicated by the dedicated signal for backhaul link. - In the time domain resource without simultaneous downlink reception or uplink transmission in C-link and backhaul link, if the NCR does not support capability with the new signalling for backhaul beam indication or if no beam is indicated for backhaul link by the dedicated signal, - When Rel-15/16 beam indication framework is used for C-link, - The beam determined by QCL assumption for CORESET with the lowest ID and spatial relationship for PUCCH with lowest PUCCH resource ID in the C-link is applied for the DL and UL of backhaul link, respectively. - When Rel-17 beam indication framework (i.e., unified TCI framework) is used for C-link, the indicated unified TCI for C-link DL and UL is applied for the DL and UL of backhaul link, respectively. [updated by the later agreement] Otherwise, the beam indicated by the dedicated signalling is applied for backhaul link. Agreement In the time domain resource without simultaneous downlink reception or uplink transmission in C-link and backhaul link, if the NCR does not support capability with the new signalling for backhaul beam indication or if no beam is indicated for backhaul link by the dedicated signal, - When Rel-17 beam indication framework is used for C-link, - If no unified TCI is applied for C-link, the beam determined by QCL assumption for CORESET with the lowest ID and spatial relationship for PUCCH with lowest PUCCH resource ID in the C-link is applied for the DL and UL of backhaul link, respectively. (i.e. same as the default beam defined for Rel-15/16 beam indication framework) - If there is unified TCI applied for C-link, the indicated unified TCI for C-link DL and UL is applied for the DL and UL of backhaul link, respectively.

It can be observed that the pre-defined rule that should be supported by the NCR can be determined by the capability of C-link. More specifically, if C-link does not support unified TCI, NCR only supports the rule for Rel-15/16 (non-unified TCI state framework), i.e., the rule highlighted in yellow and pink. If C-link supports unified TCI, NCR additionally supports the rule for Rel-17 (unified TCI state framework), i.e., the rule highlighted in cyan.

Since whether C-link supports unified TCI can be reported by legacy capability signalling and then the exact rule supported by NCR can be determined, we don’t think component candidate values for FG 43-8 are necessary. But some clarification on the relation between this FG and legacy UE feature for unified TCI should be added.

Observation 3: The pre-defined rule that should be supported by the NCR can be determined by the capability of C-link which is reported by legacy capability signaling.

- If C-link does not support unified TCI states, NCR should only support the rule for Rel-15/16 (non-unified TCI state framework). If C-link supports unified TCI states, NCR should additionally support the rule for Rel-17 (unified TCI state framework).

Proposal 3: For FG43-8, do not define component candidate values. And clarify the relation between this FG and legacy UE feature for unified TCI state.

- Adopt the following updates (highlighted in green).

| 43. NR_netcon_repeater | 43-1a8 | Adaptive beam for NCR backhaul link[/C-link] |  | 1. [Support adaptive beam for NCR C-link] 2. Support [explicit] adaptive beam for NCR backhaul link Support of backhaul link beam determination based on predefined rule | 43-1, 2-2,2-4, 2-4a | N/A | Yes | The beam for backhaul link and C-link is fixed. | Per NCR-MT | No | No | Yes | FFS: Component candidate values: {Rel-15/16, Rel-17, both} If C-link does not support unified TCI, NCR only supports the rule for non-unified TCI framework. If C-link supports unified TCI, NCR additionally supports the rule for unified TCI framework. FFS: relation between 43-1a and existing UE features for beam operation | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

2.4 FG 43-6 dedicated signaling

As shown in the table below, the component of FG 43-6 is “Support dedicated signalling for backhaul link beam indication”. And it is FFS whether/how to support candidate values.

| 43. NR_netcon_repeater | 43-6 | Dedicated signalling for backhaul link beam indication | 1. Support dedicated signalling for backhaul link beam indication | 43-1, 43-1a8 | N/A | Yes | Dedicated signalling for backhaul link beam indication is not supported | Per NCR-MT | No | No | Yes | [Component candidate values: {Rel-15/16, Rel-17, both}] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Similar to FG 43-8, the exact dedicated signaling that can be optionally supported by NCR is subject to the capability of C-link, according to the following agreement. If C-link does not support unified TCI state, only the dedicated signaling based on Rel-15/16 (i.e., the dedicated signaling for non-unified TCI state case, corresponding to the yellow highlighting) can be optionally supported by NCR. If C-link supports unified TCI states, additionally, the dedicated signaling based on Rel-17 (i.e., the dedicated signaling for unified TCI state case, corresponding to the cyan highlighting) can also be optionally supported by NCR.

Agreement The semi-static beam indication for backhaul link is supported as: - If the beam indication framework in Rel-15 is used for NCR-MT - The DL beam is indicated by MAC CE to select one of TCI state ID from the RRC-configured list of beams for C-link - The UL beam is indicated by SRI on C-link via MAC CE. - If the beam indication framework in Rel-17 is used for NCR-MT - The DL and UL beam are indicated by MAC CE to select one of TCI state ID from the RRC-configured list of beams for C-link

It can be observed that at least for the case the C-link supports unified TCI, candidate values are necessary, because NCR needs to report which type of dedicated signalling is supported. However, for the case the C-link does not support unified TCI, candidate values for FG 43-6 are unnecessary. In this case, once FG 43-6 is supported, it means the NCR supports the dedicated signaling based on Rel-15/16 (i.e., the dedicated signaling for non-unified TCI case).

Observation 4: For FG 43-6, candidate values {Rel-15/16 (non-unified TCI), Rel-17 (unified TCI), both} are necessary at least for the case when the C-link supports unified TCI.

Proposal 4: For FG 43-6, define component candidate values and clarify the relation between the candidate values and the legacy UE feature for unified TCI state.

- Adopt the following updates (highlighted in green).

| 43. NR_netcon_repeater | 43-6 | Dedicated signalling for backhaul link beam indication | 1. Support dedicated signalling for backhaul link beam indication | 43-1, 43-1a8 | N/A | Yes | Dedicated signalling for backhaul link beam indication is not supported | Per NCR-MT | No | No | Yes | Component candidate values: {Rel-15/16 (non-unified TCI), Rel-17 (unified TCI), both} If C-link does not support unified TCI, the value shall be set as “Rel-15/16 (non-unified TCI)”. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Conclusion

This contribution discusses UE features for NR NCR, and we have proposals as below.

FG 43-1

Observation 1: RAN1 has agreed new signaling for internal delay is not needed.

Proposal 1: For FG 43-1, do not define candidate values for component 5 and capability signaling.

- Adopt the following updates (highlighted in green).

| 43. NR_netcon_repeater | 43-1 | Basic NCR support [for C-link and backhaul link] | 1. Support of fixed beam for C-link/backhaul link 2. Support of TDMed UL transmission of C-link and backhaul link [3. Support of ON-OFF operation for NCR-Fwd based on access link beam indication] [4. Support of TDD UL/DL determination for backhaul/access link based on TDD UL/DL configuration of C-link] [5. [Support of] Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link] 6. Support of beam correspondence of the DL/UL of the access link at NCR-Fwd [7. Support of backhaul link beam determination based on predefined rule] |  | N/A | Yes | NCR is not supported | Per NCR-MT | No | No | Yes | A NCR-MT that includes ncr-NodeIndication in RRC Setup Complete must support FG 43-1 [Component 3 candidate values: FFS] [Component 4 candidate values: FFS] | Optional without capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 43-3

Observation 2: The slot-offset k is to define the minimum time duration in the number of slots required by NCR-MT to perform DCI reception and apply the beams indicated by the DCI. The candidate value for slot-offset k can be defined with reference to timeDurationForQCL.

Proposal 2: For FG 43-3, as the starting point, the candidate values for slot-offset k can be:

- 15 kHz: {1}

- 30 kHz: {1}

- 60 kHz: {1, 2}

- 120 kHz: {1, 2}

FG 43-8

Observation 3: The pre-defined rule that should be supported by the NCR can be determined by the capability of C-link which is reported by legacy capability signaling.

- If C-link does not support unified TCI states, NCR should only support the rule for Rel-15/16 (non-unified TCI state framework). If C-link supports unified TCI states, NCR should additionally support the rule for Rel-17 (unified TCI state framework).

Proposal 3: For FG43-8, do not define component candidate values. And clarify the relation between this FG and legacy UE feature for unified TCI state.

- Adopt the following updates (highlighted in green).

| 43. NR_netcon_repeater | 43-1a8 | Adaptive beam for NCR backhaul link[/C-link] |  | 1. [Support adaptive beam for NCR C-link] 2. Support [explicit] adaptive beam for NCR backhaul link Support of backhaul link beam determination based on predefined rule | 43-1, 2-2,2-4, 2-4a | N/A | Yes | The beam for backhaul link and C-link is fixed. | Per NCR-MT | No | No | Yes | FFS: Component candidate values: {Rel-15/16, Rel-17, both} If C-link does not support unified TCI, NCR only supports the rule for non-unified TCI framework. If C-link supports unified TCI, NCR additionally supports the rule for unified TCI framework. FFS: relation between 43-1a and existing UE features for beam operation | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 43-6

Observation 4: For FG 43-6, candidate values {Rel-15/16 (non-unified TCI), Rel-17 (unified TCI), both} are necessary at least for the case when the C-link supports unified TCI.

Proposal 4: For FG 43-6, define component candidate values and clarify the relation between the candidate values and the legacy UE feature for unified TCI state.

- Adopt the following updates (highlighted in green).

| 43. NR_netcon_repeater | 43-6 | Dedicated signalling for backhaul link beam indication | 1. Support dedicated signalling for backhaul link beam indication | 43-1, 43-1a8 | N/A | Yes | Dedicated signalling for backhaul link beam indication is not supported | Per NCR-MT | No | No | Yes | Component candidate values: {Rel-15/16 (non-unified TCI), Rel-17 (unified TCI), both} If C-link does not support unified TCI, the value shall be set as “Rel-15/16 (non-unified TCI)”. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Appendix

Agreement

Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 43. NR_netcon_repeater | 43-1 | Basic NCR support [for C-link and backhaul link] | 1. Support of fixed beam for C-link/backhaul link 2. Support of TDMed UL transmission of C-link and backhaul link [3. Support of ON-OFF operation for NCR-Fwd based on access link beam indication] [4. Support of TDD UL/DL determination for backhaul/access link based on TDD UL/DL configuration of C-link] [5. [Support of] Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link] 6. Support of beam correspondence of the DL/UL of the access link at NCR-Fwd [7. Support of backhaul link beam determination based on predefined rule] | FFS | N/A | Yes | NCR is not supported | Per NCR-MT | No | No | Yes | A NCR-MT that includes ncr-NodeIndication in RRC Setup Complete must support FG 43-1 [Component 3 candidate values: FFS] [Component 4 candidate values: FFS] [Component 5 candidate values: FFS] | Optional [with/without] capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 43. NR_netcon_repeater | 43-2 | Periodic beam indication for access link | 1.Support periodic beam indication for access link 2. Priority flag for periodic indication | 43-1 | N/A | Yes | NCR-MT cannot decode the periodic beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-3 | Aperiodic beam indication for access link | 1.Support aperiodic beam indication for access link [2. Supported slot-offset k values for reference slot] | 43-1 | N/A | Yes | NCR-MT cannot decode the aperiodic beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one FFS: component 2 candidate values | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-4 | Semi-persistent beam indication for access link | 1.Support semi-persistent beam indication for access link 2. Priority flag for semi-persistent indication | 43-1 | N/A | Yes | NCR-MT cannot decode the semi-persistent beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-7 | Priority flag | Priority flag for periodic/semi-persistent indication | 43-1, one of 43-2 or 43-4 | N/A | Yes | Priority flag is not supported | Per NCR-MT | No | No | Yes | FFS: whether to split this FG into two separate FGs for periodic and semi-persistent indication or whether to merge with FGs 43-2 and 43-4, respectively | Optional with capability signaling |

Agreement

Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 43. NR_netcon_repeater | 43-1a8 | Adaptive beam for NCR backhaul link[/C-link] | 1. [Support adaptive beam for NCR C-link] 2. Support [explicit] adaptive beam for NCR backhaul link Support of backhaul link beam determination based on predefined rule | 43-1, 2-2,2-4, 2-4a | N/A | Yes | The beam for backhaul link and C-link is fixed. | Per NCR-MT | No | No | Yes | FFS: Component candidate values: {Rel-15/16, Rel-17, both} FFS: relation between 43-1a and existing UE features for beam operation | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-6 | Dedicated signalling for backhaul link beam indication | 1. Support dedicated signalling for backhaul link beam indication | 43-1, 43-1a8 | N/A | Yes | Dedicated signalling for backhaul link beam indication is not supported | Per NCR-MT | No | No | Yes | [Component candidate values: {Rel-15/16, Rel-17, both}] | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-8a | Explicit beam indication for NCR backhaul link | Support explicit adaptive beam for NCR backhaul link | 43-8 | N/A | Yes | Explicit beam indication for NCR backhaul link is not supported | Per NCR-MT | No | No | Yes |  | Optional with capability signaling |