3GPP TSG RAN WG1 #113			     R1-2304806

Incheon, Korea, May 22nd – May 26th, 2023

Source:	Intel Corporation

Title:	On Sidelink Operation in FR2 Licensed Spectrum

Agenda item:	9.4.3

Document for:	Discussion and Decision

# Introduction

In 3GPP TSG RAN meeting #94, a new work item related to NR Sidelink (SL) evolution was approved, whose description has been recently updated during RAN#98-e [1] to study enhancements needed to support SL operation in FR-2 licensed spectrum. In this matter, the related WID currently includes the following objective:

- Study enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2] - Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1] - Study is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. [RAN1, RAN2] - Beam management in FR2 licensed spectrum considers sidelink unicast communication only.

In the context of NR SL in FR2, during the past RAN1 meeting [2] the agreements made are captured in the Annex A section. In this contribution several aspects related to the operation of SL in FR-2 licensed spectrum are discussed, while our views on other aspects of Rel.18 NR SL evolution are provided in our companion contributions [3].

Introduction

The main purpose of Sidelink Beam Management (SL BM) is to find transmit and receive beams for the communication between two devices that enable communication as well as fulfil QoS criteria, which also includes: i) the maintenance of these beams until they are no longer needed, and ii) detecting and responding to the case that communication with the selected beam fails. In the following, it is assumed that each transmission beam corresponds to a reception beam, similar to Rel-17 unified TCI framework in case of joint DL/UL TCI state, thus it is not required that the transmit and the receive beams of a device are trained separately.

The task of selecting a beam from a codebook could follow any of the beam sweeping procedures illustrated in Figure 1. SL FR2 is targeting any device category, but for illustration purpose the devices are represented by cars. In Procedure 1 both the transmit and receive beams are changed and tested in one procedure. If there are MT transmit beams and MR receive beams, this means that MT ⸱MR beam pairs need to be tested. This has the advantage that both beamforming gains of the transmit and receive antenna panels can be utilized during this procedure without prior knowledge. However, at the same time there is a disadvantage of the overhead associated with the testing of all these beam pairs. As for Procedures 2 and 3, either the transmit or the receive beam is changed, while the beam at the other side is kept the same. This can either be employed by a fixed beam or a quasi-omnidirectional beam. It is worth noting that using a quasi-omnidirectional beam reduces the coverage as the beamforming gain of the antenna panel on one of the sides cannot be utilized. Both the initial beam-pairing as well as the beam maintenance can use any of these procedures.

| Procedure 1 |  |
| --- | --- |
| Procedure 2 | Procedure 3 |

Figure 1 - Three solution directions for SL beam sweeping.

For SL beam management, the following high-level topics need to be considered for study:

- Design assumptions: There are number of design assumptions that need to be made for SL FR2. Most of them are based on Rel-16/17 assumptions for sidelink communication.

- Initial beam-pairing: This procedure is the most important to establish a connection between two UEs for communicating in SL FR2. Role assignment (initiator or responder) as well as the connection to the related reference signals need to be established. Also, the criteria for the beam reports, the beam reports’ contents, timing of the beam report and their communication interface need to be defined.

- Beam maintenance: For beam maintenance there are two important aspects: i) triggering conditions and timing, and ii) procedure to be used.

- Beam failure and recovery: It is important that devices detect when beam alignment is failing or is about to fail. Depending on the scenario as well as other system parameters, different solutions are possible.

- Reference signals for SLBM: One of the key aspects is the definition of reference signals for beam management. They need to be defined for all beam management tasks.

- Resource allocation enhancements: Both network-controlled, and UE-autonomous resource allocation modes should be supported. Rel-16/17 resource allocation design may not be suitable for SL FR2 communication aspects. It is beneficial to revisit some of the solutions to optimize the performance for SL FR2.

These topics are discussed one-by-one in the subsequent sections.

# Design Assumptions

There are a few fundamental system assumptions, which are important to be highlighted and made before RAN1 can move forward. Most of these are related to physical layer design assumptions made during Rel-16 and Rel-17 sidelink discussions. Additional assumptions beyond the ones made during Rel-16 and Rel-17 SL discussions are related to the frequency range as well as the simultaneous multi-panel transmission and reception capability for SL FR2 UEs.

In our understanding the following Rel-16/17 design assumptions should be reused for SLBM design:

- Flat hierarchy among users at the physical layer: This can be understood that all devices are equal in terms of their ability to start any procedure related to SLBM.

- Design considers both mode-1 and mode-2 operation: In our understanding both network and UE autonomous resource operation need to be supported for SLBM.

- No coexistence of mode-1 and mode-2 in the same resource pool: Like the Rel-16/17 SL design no coexistence of these two operation modes is considered at the physical layer.

    - The following Rel-16 design assumptions are reused for the SL FR2 beam management design:

      - Flat hierarchy among users at the physical layer

      - Both mode-1 and mode-2 operation support

      - No coexistence of mode-1 and mode-2 in the same resource pool

During the last RAN1 meeting [2] the assumption of beam correspondence was extensively discussed. In our understanding assuming beam correspondence reduces the complexity as well as the probability of the system failing to establish a connection. This is especially true for the case for SL FR2 standalone operation. Since the time of this study item is rather limited, it would be practical for the group to focus on systems with beam correspondence assumption, while during normative phase design without beam correspondence could be considered assuming UEs with different capabilities.

    - Prioritize UEs with beam correspondence for the study of SL FR2 BM.

Like in the legacy UL/DL case, the procedures illustrated in Figure 1 should be also defined for SL. These procedures can be used to design the different operations to align and maintain beams. It needs to be noted that the beam operations can be performed per panel for UEs operating with multiple panels simultaneously.

    - Define the following procedures for the study for SL BM:

      - P1: Both a Tx UE and a Rx UE perform beam sweeping,

      - P2: A Tx UE performs beam sweeping,

      - P3: A Rx UE performs beam sweeping.

# Initial Beam-Pairing and Directional Discover

For beam sweeping operation it is essential that each device knows which role it performs during the beam sweeping. In this context we define the roles of ‘initiator’ and ‘responder’ as the device transmitting during the initial beam sweeping and the device receiving during the initial beam sweeping, respectively. The example provided in Figure 2 illustrates the case that two devices take the role of the initiator for procedure 1. As both are transmitting, they might not know about each other presence. In a system that uses device autonomous resource selections this means that a directional discovery including initial beam-pairing can only be performed by different devices taking the role of the initiator and responder based on a randomized procedure. The procedure of initial beam-pairing would need to be performed before any communication; thus, the high energy footprint would be present even for the case where no other device with SL FR2 capability is present.

Figure 2 - Role collision during initial beam-pairing.

Based on these considerations it is worthwhile to study which information can be available at the different devices before the initial beam-pairing. These considerations are not limited to only RAN1 considerations; thus, it is worthwhile to exchange with RAN2 about this topic, especially for SL FR2 mode-2 operation. The whole process of information exchange to assist initial beam-pairing plus the initial beam-paring itself as well as the following message exchange can be considered under the concept of directional discovery.

- Study which information can be used to assist the initial beam-pairing.

After the responding UE has detected the best beam for itself as well as the best beam for the initiating UE, this information needs to be reported to the initiating UE. It is also possible that the initiating UE is detecting this information by itself. Therefore, RAN1 needs to study how it can be ensured that both UEs have the same information about the optimal beams after the initial beam pairing.

    - Study which information exchange is necessary to ensure that both UEs have the same understanding of the optimal beam at both devices.

Before initial beam alignment there is directional communication possible. To optimally utilize the beamforming gain at both UEs it is essential that the beam on both sides of the communications is trained. For the case of both UEs having beam correspondence it is possible to perform the procedure P1 once with any of the UEs being the transmitter and using the corresponding beams for the transmission of information in the other direction. In the case without beam correspondence this process needs to be performed for each communication direction.

    - Procedure P1 where both the Tx and the Rx UE perform beam sweeping is utilized for initial beam pairing.

## Initial beam pairing before sidelink unicast link establishment

An initial agreement on performing initial beam alignment before sidelink unicast link establishment was reached during the last meeting. In our understanding this would be a similar design compared to the Uu design of initial beam pairing. The following paragraph highlight different considerations that need to be taken to define a system based on the initial agreement.

In our understanding weather other reference signals can be used in this case depends on the synchronization source. If PC5 synchronization is used it is only possible to use S-SSB for the initial beam pairing as synchronization needs to be provided at the same time.

    - For the case of PC5 synchronization in the case of initial beam pairing before sidelink unicast link establishment only S-SSB can be used for initial beam pairing

Based on this observation it is clear that the design of initial beam pairing is not independent of synchronization. Therefore, we propose the following restriction of the study regarding the synchronization source.

It is also important to highlight that before the start of this procedure we assume each device does not know if any other device is present. At this stage it is even not clear at which transmit occasion a UE should transmit reference signals with different beams or receive them. Different solutions to this problem should be studied.

In our understanding no prior information about the present of other UEs is available in this case. This means the information for the reference signals used for initial beam pairing need to be (pre)-configured. The same holds true for the relationship between reference signal transmissions and beams.

In our understanding as no information exchange via another channel is possible for this case a RACH like procedure need to be used to indicate the transit beam from UE2 to UE1. This could consist of UE2 transmitting at a (pre)-configured time slot to UE1. This timeslot does correspond to the optimal detected by UE2 for the transmit beam of UE1.

    - For Initial beam pairing before sidelink unicast link establishment the following additional topics needs to be studied:

      - The choice of reference signals dependent on the used synchronization source. If synchronization also needs to be provided during initial beam pairing S-SSB must be used.

      - Study different mechanisms to assign the roles of initiating and responding device.

      - The time occasions for transmission of reference signals are (pre)-configured.

      - The relationship of reference signal transmissions to transmit beams need to be (pre)-configured.

      - The transmit beam is indicated by UE2 transmitting to UE1 in a (pre)-configured slot that is dependent on the determined transmit beam.

## Initial beam pairing during sidelink unicast link establishment

An initial agreement on performing initial beam alignment during sidelink unicast link establishment was reached during the RAN1#112b-e.

As synchronization can not be achieved during this method of initial beam pairing this method cannot rely on sidelink synchronization. As we assume the same synchronization options as in SL FR1 are applicable this means only GNSS, or network synchronization can be used.

As this method consist of repeated SL-SCH messages in our understanding it is possible to rely on PSSCH DMRS for the RSRP measurements for initial beam pairing.

Like the first case of initial beam pairing the same consideration of assigning roles and the (pre)-configuration of transmission resources need to be made. In addition, the same method of indicating the transmission beam to UE1 can be used.

    - For Initial beam pairing during sidelink unicast link establishment the following additional topics needs to be studied:

      - Sidelink synchronization is not used for this method of initial beam pairing.

      - PSSCH DMRS can be used for RSRP measurements related to initial beam pairing.

      - Study different mechanisms to assign the roles of initiating and responding device.

      - Mapping of PSCCH/PSSCH transmissions and UE1 transmit beams (pre)-configured

      - UE2 determines UE1 transmit beam(s) based by relating it to the (pre)-configurated relation to transmission slots

      - The transmit beam is indicated by UE2 transmitting to UE1 in a (pre)-configured slot that is dependent on the determined transmit beam.

## Initial beam pairing after sidelink unicast link establishment

Many, similar considerations as in the other cases can be made. However, in addition for this case we assume that the unicast link establishment can contain additional information to assist initial beam pairing like the role assignment.

In addition to the considerations made for the other options we assume that coordination information including the sidelink unicast link establishment can be exchange via a coordination interface. In our understanding this coordination interface can be for example a sidelink system operating in FR1.

    - For Initial beam pairing after sidelink unicast link establishment the following additional topics needs to be studied:

      - Sidelink synchronization is not used for this method of initial beam pairing.

      - Transmit beams and their corresponding transmit resource are aligned during the unicast link setup.

      - Any reference signal defined for the other methods of initial beam pairing can be used.

      - The beam indication between UE2 and UE1 is using the same interface that is also used for the unicast link establishment.

# Beam Maintenance

Regarding beam maintenance there are two major directions that need to be studied. First the conditions that trigger beam maintenance need to be studied. In this context it is important that after the triggering both participating UEs know the beam maintenance is active and know how to operate in this event.

    - Study which conditions may trigger beam maintenance.

When beam maintenance is triggered and after both UEs are informed, it is important to know how beam maintenance is performed. As SL FR2 beam management targets a wide set of use-cases, different system profiles and channel conditions will require different techniques for beam maintenance. For example, in a high mobility scenario it is required that beam maintenance is performed more often compared to a lower mobility scenario.

    - Study which techniques can be utilized for beam maintenance.

# Beam Failure and Recovery

In contrast to beam management for the Uu link there is no physical layer hierarchy between different UEs, this means that beam failure and recovery can be detected by either UE. Thus, RAN1 should study the conditions to detect and declare the beam failure. In our understanding the legacy definition for Radio Link Failure (RLF) can be a starting point for these conditions.

    - Study the conditions to detect and declare a sidelink beam failure.

After the beam failure is detected by one of the devices, it is necessary that the other device also knows about this state to start the beam recovery. It is possible that this information is transmitted to the other device. It is also possible that through system design the beam failure is always detected simultaneously by both UEs.

    - Study how two UEs can get a mutual agreement on the beam recovery procedure.

In our understanding the sidelink radio link failure (SL RLF) should serve as a starting point for beam failure detection. In our understanding the requirements for beam failure detection are similar compared to RLF, thus the framework can be reused. As in our understanding the timeframe for beam failures might be shorter compared to RLF. Thus, the current RLF timelines need to be adjusted to fit the expected timeline of beam failures.

    - Sidelink RLF should be used as the starting point for beam failure.

# Beam Reporting

In our understanding at different stages of the system the beam report might be reported through different interfaces and for different purposes. For example, SL RRC is only available after a unicast connection is established. In addition, it is also important to support case where the beam report for the initial beam alignment is transmitted on another interface than SL FR2 PC5 as at this stage without established beams no communication might yet be possible.

    - SL Beam reporting can utilize the following containers:

      - SL MAC CE,

      - SL RRC,

      - Signalling between the UEs through the network.

# Reference Signals for SLBM

In the Uu link the SSB as well as CSI-RS are used for beam management. During the Rel-16 SL design, it was already foreseen that the S-SSB may be used for SL FR2 beam sweeping during the initial beam-alignment, thus additional repetitions are configurable for 60 and 120 kHz SCS. However, this means that per S-SSB slot only a single beam pair can be trained. Other reference signal definitions can be studied as well. They should target enabling a beam-alignment with a lower overhead relative to utilizing S-SSB transmissions.

    - Study if other reference signals besides the ones present in Rel-16 S-SSB can be used for initial beam alignment.

There are several changes that would need to be made to the current S-SSB to make to enable S-SSB based initial beam pairing. In our understanding the following changes would need to be made.

- The mapping of beams and S-SSB transmission resource need to be (pre)-configured as otherwise it is not possible for other UEs to access the information before initial beam pairing.

- The physical structure of the S-SSB could remain the same, but in our understanding the content of the PSBCH need to be redesigned.

    - Mapping of S-SSB transmission resource and beams is (pre)-configured.

    - Physical structure of the S-SSB does stay the same.

    - Content of the PSBCH is changed to enable usage of the S-SSB for initial beam pairing.

Current SL CSI-RS accompany transmissions of PSCCH/PSSCH. This is not necessary for beam maintenance as it is even likely that either further information does not need to be exchanged or it is beneficial in terms of latency to only transmit reference signals to perform measurements related to beam training.

In our understanding current definitions of CSI-RS are not suitable for standalone transmissions. In our understanding the most suitable candidate would be low PAPR sequences, like the ones utilized for S-SSS. Therefore, we think standalone CSI-RS should utilize a similar physical structure as S-SSS.

For the mapping of CSI-RS resource can either be (per)-configured or aligned between UEs during the initial connection setup. For initial beam pairing this needs to be (pre)-configured as any UE that wants to connect to any other UE needs to have this information available. For beam maintenance UEs can align on this configuration via higher layer signaling.

    - Standalone transmission of CSI-RS is supported.

    - Standalone CSI-RS can utilize a similar structure as the S-SSS.

    - For CSI-RS for initial beam training the mapping of CSI-RS resources to transmission beams is (pre)-configured.

    - For CSI-RS for beam maintenance the mapping of CSI-RS resources to transmission beams is aligned via higher layer message exchange of connected UEs.

# Resource Allocation Enhancements

In our understanding the resource allocation designed for Rel-16 NR SL mainly targets avoiding collisions and enabling fair access to the channel in a congested radio environment. Due to utilizing directional beamformed transmissions, for NR SL FR2 the system is no longer always limited by interference at the physical layer. Furthermore, communication in FR2 can be less reliable as beams can fail at any moment. The Rel-16 resource allocation design is thus targeting a system with different consideration. We therefore think to support SL FR2 operation it is essential to also adapt the resource allocation for it, specifically the mode 2 resource allocation can benefit from incorporating the specific channel characteristics of systems operating in FR2. Thus, to improve the system performance it is desirable to study how the resource allocation can be changed to consider these changes in system design assumptions.

    - Study resource allocation enhancements for SL FR2 considering the changed system assumption and changed interference environment.

Conclusion

In this contribution, we have provided our views on the Study of Sidelink Communication in FR2. In summary, we have the following list of proposals:

    - The following Rel-16 design assumptions are reused for the SL FR2 beam management design:

      - Flat hierarchy among users at the physical layer

      - Both mode-1 and mode-2 operation support

      - No coexistence of mode-1 and mode-2 in the same resource pool

    - Prioritize UEs with beam correspondence for the study of SL FR2 BM.

    - Define the following procedures for the study for SL BM:

      - P1: Both a Tx UE and a Rx UE perform beam sweeping,

      - P2: A Tx UE performs beam sweeping,

      - P3: A Rx UE performs beam sweeping.

- Study which information can be used to assist the initial beam-pairing.

    - Study which information exchange is necessary to ensure that both UEs have the same understanding of the optimal beam at both devices.

    - Procedure P1 where both the Tx and the Rx UE perform beam sweeping is utilized for initial beam pairing.

    - For Initial beam pairing before sidelink unicast link establishment the following additional topics needs to be studied:

      - The choice of reference signals dependent on the used synchronization source. If synchronization also needs to be provided during initial beam pairing S-SSB must be used.

      - Study different mechanisms to assign the roles of initiating and responding device.

      - The time occasions for transmission of reference signals are (pre)-configured.

      - The relationship of reference signal transmissions to transmit beams need to be (pre)-configured.

      - The transmit beam is indicated by UE2 transmitting to UE1 in a (pre)-configured slot that is dependent on the determined transmit beam.

    - For Initial beam pairing during sidelink unicast link establishment the following additional topics needs to be studied:

      - Sidelink synchronization is not used for this method of initial beam pairing.

      - PSSCH DMRS can be used for RSRP measurements related to initial beam pairing.

      - Study different mechanisms to assign the roles of initiating and responding device.

      - Mapping of PSCCH/PSSCH transmissions and UE1 transmit beams (pre)-configured

      - UE2 determines UE1 transmit beam(s) based by relating it to the (pre)-configurated relation to transmission slots

      - The transmit beam is indicated by UE2 transmitting to UE1 in a (pre)-configured slot that is dependent on the determined transmit beam.

    - For Initial beam pairing after sidelink unicast link establishment the following additional topics needs to be studied:

      - Sidelink synchronization is not used for this method of initial beam pairing.

      - Transmit beams and their corresponding transmit resource are aligned during the unicast link setup.

      - Any reference signal defined for the other methods of initial beam pairing can be used.

      - The beam indication between UE2 and UE1 is using the same interface that is also used for the unicast link establishment.

    - Study which conditions may trigger beam maintenance.

    - Study which techniques can be utilized for beam maintenance.

    - Study the conditions to detect and declare a sidelink beam failure.

    - Study how two UEs can get a mutual agreement on the beam recovery procedure.

    - Sidelink RLF should be used as the starting point for beam failure.

    - SL Beam reporting can utilize the following containers:

      - SL MAC CE,

      - SL RRC,

      - Signalling between the UEs through the network.

    - Study if other reference signals besides the ones present in Rel-16 S-SSB can be used for initial beam alignment.

    - Mapping of S-SSB transmission resource and beams is (pre)-configured.

    - Physical structure of the S-SSB does stay the same.

    - Content of the PSBCH is changed to enable usage of the S-SSB for initial beam pairing.

    - Standalone transmission of CSI-RS is supported.

    - Standalone CSI-RS can utilize a similar structure as the S-SSS.

    - For CSI-RS for initial beam training the mapping of CSI-RS resources to transmission beams is (pre)-configured.

    - For CSI-RS for beam maintenance the mapping of CSI-RS resources to transmission beams is aligned via higher layer message exchange of connected UEs.

    - Study resource allocation enhancements for SL FR2 considering the changed system assumption and changed interference environment.

    - For the case of PC5 synchronization in the case of initial beam pairing before sidelink unicast link establishment only S-SSB can be used for initial beam pairing

References

- RP-222806, “WID revision: NR sidelink evolution”, OPPO, e-meeting, Dec. 2022.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #112b-e, e-meeting, April 2023.

- R1-2304805, “Remaining issues on NR-LTE sidelink co-channel co-existence”, Intel Corporation, e-meeting, April 2023.

- Annex A – List of Agreements

This section captures all the agreements made during the past RAN1 meeting [2]:

RAN1 #112: Agreement For sidelink beam management, RAN1 is to study - how transmit beam(s) training and/or receive beam(s) training is performed - whether and how spatial related information (e.g., TCI, QCL, beam ID, etc) information could be identified - the relationship between PC5 unicast link establishment and sidelink initial beam pairing (e.g., whether initial beam pairing procedure starts before, during or after sidelink unicast link establishment procedure.) Agreement RAN1 is to study sidelink beam measurement and reporting schemes (e.g., periodicity, contents, container, timing, procedure, etc.) for sidelink beam maintenance. Agreement RAN1 is to study sidelink beam indication and switching schemes (e.g., framework, general procedure, contents, signaling, timing, etc.) for sidelink beam maintenance. Agreement RAN1 is to study the information related to a sidelink beam failure instance that the PHY layer provides to the MAC layer. RAN1 #112b-e: Agreement RAN1 can study the following candidate procedure where initial beam pairing is performed before sidelink unicast link establishment, including at least the following steps and how to determine UE2: - UE1 sends reference signals via different transmit beams - Note: multiple reference signals transmissions (e.g. repetitions) from each of the beams can be studied - FFS when reference signals are sent - FFS applicable reference signal - UE2 measures the reference signals and determines a UE1 transmit beam and/or a UE2 receive beam - FFS:whether/how to determine a UE2 transmit beam - UE2 indicates to UE1 the determined UE1 transmit beam - FFS how to indicate the determined transmit beam, including its feasibility - UE1 and UE2 set up sidelink unicast link using the determined beam, following existing link establishment procedure. Agreement Consider one or more of the following items as sidelink beam reporting contents for beam maintenance: - Beam indication (e.g., CRI) - L1-RSRP - FFS: L1-SINR or other reporting contents if necessary enhancement is identified. - Note: This does not preclude performing beam maintenance without any beam reporting Agreement The container(s) of sidelink beam reporting for beam maintenance is at least selected from the following options: - Option 1: SL PHY layer signal (e.g., PSFCH, SCI) - Option 2: SL MAC CE - FFS: PC5-RRC, Signaling over Uu link (e.g., UCI) Conclusion RAN1 to prioritize the case where a UE is incapable of simultaneous transmitting or receiving PSCCH/PSSCH/PSFCH using different beams in Rel-18. Agreement For UE1 and UE2 in a unicast link, RAN1 to study the selection of both UE1’s transmit beam and UE2’s corresponding receive beam. - Note: this applies for both PSCCH/PSSCH transmission/reception and PSFCH transmission/reception. Agreement Consider using sidelink CSI-RS as a starting point for beam maintenance. - FFS: whether/how to enhance existing aperiodic and non-standalone SL CSI-RS - FFS: periodic and/or semi-persistent SL CSI-RS transmissions - FFS: standalone SL CSI-RS transmissions - Note: standalone SL CSI-RS transmission means at least no accompanying sidelink data (SL MAC SDU) transmissions in the same slot. FFS: accompanying SCI(s) or SL MAC CE transmissions or PSFCH. - FFS: one or multiple SL CSI-RS transmissions within one slot - FFS: SL CSI-RS transmissions with or without repetition on transmit beams Agreement RAN1 can study the following candidate procedure where initial beam pairing is performed during sidelink unicast link establishment - UE1 sends PSCCH/PSSCH that carries unicast link establishment message (e.g., DCR message) via different transmit beams - Note: multiple PSCCH/PSSCH transmissions (e.g., repetitions) from each of the beams can be studied. - FFS: applicable reference signals which are transmitted together with unicast link establishment message. - if UE2 successfully decodes one (or more) of the PSCCH/PSSCH(s) and UE2 determines to establish a unicast link with UE1, it indicates to UE1 one (or more) UE1 transmit beam(s) of PSCCH/PSSCH(s) which is successfully received - FFS details (e.g., implicit or explicit indication) - FFS: how to map between each PSCCH/PSSCH and UE1 transmit beam - FFS: how UE2 determines UE1 transmit beam(s) and/or UE2 transmit/receive beam(s) - UE1 uses one of the indicated beam(s) to finish the remaining sidelink unicast link establishment procedure with UE2 - FFS: how UE1 determines one of the indicated beam(s) - FFS: use of additional reference signal or additional messages or additional measurement for efficient beam pairing. Agreement RAN1 can study the following candidate procedure where initial beam pairing starts after sidelink unicast link establishment between UE1 and UE2, including studying whether and in which cases initial beam pairing after sidelink unicast link establishment is feasible. - UE1 and UE2 set up sidelink unicast link, following existing link establishment procedure - FFS the beams used for unicast link establishment. - UE1 and/or UE2 configure the resources for beam sweeping and/or beam reporting - FFS details of resources configuration - UE1 and/or UE2 use the configured resources to transmit reference signals and determine a pair of transmit beam and receive beam based on beam sweeping. - FFS applicable reference signal(s) - FFS whether/how to indicate the determined beams between UE1 and UE2 - FFS difference between initial beam pairing (after sidelink unicast link establishment) and beam maintenance Agreement To study the feasibility of adapting S-SSB for initial beam pairing between UE1 and UE2, at least the following can be considered. - Whether/how to enable UE2 to identify UE1 (e.g., source ID) from UE1’s S-SSB transmission, to enable UE1 to identify the corresponding beam measurement/reporting from UE2 - Mapping between S-SSB transmission/resource and beam related information - Allocation of beam reporting resources respectively associated with different S-SSB transmit beams - Structure and contents of S-SSB - Triggering and/or activation of S-SSB transmission, if needed - Mechanism for S-SSB monitoring and reporting/responding - Mechanism to mitigate/avoid the interference between overlapped S-SSB transmissions from different UEs, including S-SSB transmission resources - Potential impact to/from other UEs, and whether/how to avoid or mitigate this impact Agreement To study the feasibility of reusing SL CSI-RS for initial beam pairing, at least the following enhancements can be considered. - SL CSI-RS transmission with or without sidelink data transmission in the same slot - FFS: slot structure - Mapping between SL CSI-RS transmission/resource and beam related information - Periodic SL CSI-RS transmission, semi-persistent SL CSI-RS transmission, or aperiodic SL CSI-RS transmission, with or without SCI indication - Allocation of SL CSI-RS beam sweeping resources and if applicable, their associated beam reporting resources - Study the possibility to apply SL CSI-RS for initial beam pairing before, during or after unicast link establishment - FFS: How to provide SL CSI-RS resource configuration - Whether or how to mitigate/avoid the interference between overlapped SL CSI-RS transmissions from different UEs - SL CSI-RS transmission with or without repetition on transmit beams Agreement RAN1 is to study sidelink Beam Failure Recovery (BFR) mechanism at least for the scheme where SL BFI is triggered based on the measurement of reference signal for BFD (if supported), including - candidate beam(s) identification - FFS details on reference signals for candidate beam identification, including structure, procedure, timing. - sidelink BFR request (BFRQ), including resources, transmit and/or receive beams, container, timing, etc. - sidelink BFR response (BFRR), including container, procedure, timing, etc. - FFS applicability to the scheme where SL BFI is triggered based on SL HARQ feedback (if supported). Agreement Consider the following two options for determining PSFCH transmit/receive beam for a single PSFCH transmission/reception in a slot. - Option 1: PSFCH transmit beam is derived from the corresponding PSCCH/PSSCH receive beam; PSFCH receive beam is derived from the corresponding PSCCH/PSSCH transmit beam. - Note this is based on sidelink beam correspondence - Option 2: PSFCH transmit beam is derived from the PSCCH/PSSCH transmit beam for reverse data transmission; PSFCH receive beam is derived from the PSCCH/PSSCH receive beam for reverse data transmission. - Note this is based on beam training for reverse data transmission - Note: The PSFCH transmit/receive beam can be the same as PSCCH/PSSCH transmit/receive beam used for the reverse data transmission. - FFS: support both options or down select one option.