3GPP TSG RAN WG1 #113		R1-2304836

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.1.2

Source:	Google

Title:	On CSI Enhancement

Document for:	Discussion/Decision

# Introduction

In RAN1 #112b, the following agreements on CSI enhancement have been achieved.

Agreement On the Parameter Combination of Type-II codebook refinement for CJT mTRP, only the following linkages are supported (marked ‘x’), for Rel-16 eType-II based - For NTRP =1, - fully reuse seven out of the eight Parameter Combinations from Rel-16 eType-II as indicated in the table below - FFS (by RAN1#112bis-e): whether to add one more Parameter Combination for L=4 based on the legacy Rel-16 eType-II FD combo {½, ½, ¼, ¼; ½} or the agreed FD combo {½, ½, ½, ½; ½}, or not to add from the indicated seven below - For NTRP >1, only the following linkages are supported (marked ‘x’) - Note: Configured linkage(s) are associated with the configured value of NTRP, regardless whether the dynamic TRP selection (the dynamic change of N given NTRP) is configured. Also, the configured linkage(s) are valid for any dynamically selected SD basis and/or any dynamically selected CSI-RS resource (TRP). - FFS: UE feature/capability to support only a subset of linkages NTRP; SD combo; FD combo {pv},. {1/8, 1/8, 1/16, 1/16}, ¼; {1/8, 1/8, 1/16, 1/16}, ½; {1/4, ¼, 1/8, 1/8}, ¼; {1/4, ¼, 1/8, 1/8}, ½; {1/4, ¼, ¼, ¼}, ¾; {1/2, ½, ½, ½}, ½. 1; 2; x; x. 4; x; x; x. 6 w/ restriction; x; x. 2; {2,2}; x. {2,4} {4,2}; x. {4,4}; x; x; x. 3; {2,2,2}; x; x. {2,2,4} {2,4,2} {4,2,2}; x; x. {4,4,4}; x; x; x; x; x; x. 4; {2,2,2,2}; x; N/A. {2,2,2,4}; x; N/A. {2,2,4,4}; x; x; N/A. {4,4,4,4}; x; x; x; N/A. Agreement On the Type-II codebook refinement for CJT mTRP, regarding CBSR, amplitude restriction is CSI-RS-resource-specific. - FFS: Whether CBSR is always configured for each CSI-RS resource or not Conclusion On the Type-II codebook refinement for CJT mTRP, regarding CBSR for NTRP>1, there is no consensus in supporting the additional optional soft amplitude restriction. Therefore, only hard amplitude restriction (per CSI-RS resource, based on the legacy design) is supported. Agreement On the Type-II codebook refinement for CJT mTRP, for mode-1, support the use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources, i.e. (example formulation) where: - is commonly selected across N CSI-RS resources - is the layer-common FD basis selection offset for CSI-RS resource n relative to a layer-common reference CSI-RS resource with - Therefore, (N – 1) FD basis selection offset values are reported - Basic feature: - Optional feature: - FFS: UCI design details, details on Agreement On the Parameter Combination of Type-II codebook refinement for CJT mTRP, for Rel-17 FeType-II based, - For =1, the Rel-17 legacy Parameter Combination is fully reused - Regarding the combinations {M, beta}, it is proposed to reuse the legacy as below, with restriction on M=2. M; ; Condition. 1; ½. ¾. 1. 2; ½; FFS: N_trp<=3, NL=1. ¾; FFS: N_trp<=3, NL =1. - Alpha_n combinations for are derived from the Ln combinations for Rel-16 based refinement, where each entry in the combination is the nearest value of min{1, 2Ln/Pcsi-rs} to {1/2, ¾, 1}, . - Note: no other dependency of combinations is introduced, such as dependency on Pcsi-rs. - FFS: pruning on combinations Agreement For the Type-II codebook refinement for high/medium velocities, when a UE is configured with X=2 for CQI calculation and reporting, the 2nd CQI is located in UCI part 2 Agreement For the Type-II codebook refinement for high/medium velocities, when WCSI>1, if a UE supports X=2 for CQI calculation, the value of X (either 1 or 2) is gNB-configured via higher-layer (RRC) signalling Agreement For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, - When the UE is configured with Q=1: for each layer, one 2-dimensional bitmap of size-2LM reusing the legacy design is used - When the UE is configured with Q=2: for each layer, - Basic feature: two 2-dimensional bitmaps, each of size-2LM reusing the legacy design for each of the two selected DD basis vectors, are used - Optional feature, if the following down-selection succeeds: down-select from the following two alternatives in RAN#112bis-e: - Alt3A: A single 2-dimensional bitmap of size to report the selected pairs of FD basis vector and DD basis vector and a single 2-dimensional bitmap of size for indicating the location of the NZCs, where each row corresponds to a selected SD basis vector and each column corresponds to one of the selected pairs of FD basis vector and DD basis vector. - Alt4’: Q different bitmaps are supported for each layer, each of the Q bitmaps corresponds to DD basis q = 0 or 1. - For each polarization, each of the Q bitmaps contains bits included in a set of SD basis and FD basis pairs , satisfying , where - , - is the SD basis indicated by SCI - Two polarizations have same set of in the bitmap Agreement For the Type-II codebook refinement for high/medium velocities based on Rel-16 eType-II regular codebook, at least the following Parameter Combinations are supported 4; 1/4; 1/4; 1/4. 4; 1/4; 1/4; 1/2. 4 (*); 1/2; 1/4; 1/2. 4 (*); 1/4; 1/4; 3/4. 6 (*); 1/4; --; 1/2. 6 (*); 1/4; --; 3/4. (*) Note: From legacy. For L=6, the same restriction and UE optionality as legacy apply - Select at most 3 additional Parameter Combinations from the list below 2; 1/8; 1/16; 1/4. 2; 1/8; 1/16; 1/2. 2(*); ¼; 1/8; ¼. 2 (*); ¼; 1/8; ½. 4; 1/8; 1/16; 1/4. 4 (*); ¼; 1/8; 1/4. (*) Note: From legacy. - FFS: UE feature/capability to support only a subset of Parameter Combinations Agreement For the Rel-18 TRS-based TDCP reporting, regarding the quantization of wideband normalized amplitude value, - At least the following size-Q quantization alphabet is supported: where - TBD: supported value(s) of N (e.g. or a larger value), Q, s (e.g. ½, ¼, 1/8, …), whether a center threshold is also supported (and if so, higher-layer configured) - FFS: Whether different schemes can be supported for different use cases Working Assumption For the Rel-18 TRS-based TDCP reporting, for TDCP measurement and calculation, - KTRS ≥1 TRS resource set(s) can be configured in the CSI reporting setting when ReportQuantity is ‘tdcp’ - Note: the TRS resource set(s) configured for TDCP report do not impact or impose any new requirements on the UE behavior when processing TRS used as QCL type A/D source for reception of PDxCH. - No further spec enhancement on TRS is supported - [All the TRS resources in the configured resource set(s) share the same RE locations] - FFS: Whether to add further restrictions on the TRS resource set(s) on, e.g. QCL relationship, power control, RE location, slot offset between TRS resource set(s), relation with resource set used for legacy usage Conclusion On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, for NTRP>1, in addition to the supported SD combinations/permutations, there is no consensus on supporting at least one additional combination where at least one of the Ln values (n=1, …, NTRP) is 6 Agreement On the Type-II codebook refinement for high/medium velocities, regarding UCI omission, support reusing the legacy UCI omission mechanism with (Alt3) the following priority function: Prio(,l,m,q)=2L.RI.Mv.q + 2L.RI.P(m)+ RI.l +  where P(m) = m - Note: This implies that DD basis is designated the least priority - FFS: Details on the location of the new UCI parameters in G0/1/2 Agreement For the Rel-18 TRS-based TDCP reporting, regarding the value of parameter Y for Y>1, the value of Y is gNB-configured via higher-layer (RRC) signalling Agreement On the Type-II codebook refinement for CJT mTRP, for mode-1, the layer-common reference CSI-RS resource is fixed to the first of the N selected CSI-RS resource(s) - FFS: Whether more refined definition is needed for “the first”, e.g. related to the ordering of CSI-RS resources in the resource set, depending on RAN2 outcome Agreement On the Parameter Combination of Type-II codebook refinement for CJT mTRP, for NTRP =1, in addition to the already agreed seven Parameter Combinations, support the following Parameter Combination (based on legacy Parameter Combination #6): L=4, {pv;b}={ ½, ½, ¼ , ¼; ½ } Conclusion On the Parameter Combination of Type-II codebook refinement for CJT mTRP, no additional configuration signalling for indicating the linkage is needed. Per previous agreements (RAN1#111 and 112): - “The single value of {pv,b} is gNB-configured via higher-layer (RRC) signalling” - “The set of NL combinations of values for {L1, ..., LNTRP} is gNB-configured via higher-layer (RRC) signalling” Such configuration shall be according to the supported/agreed linkages. Conclusion On the Type-II codebook refinement for high/medium velocity, regarding CBSR, there is no consensus in supporting the additional optional soft amplitude restriction. Therefore, only hard amplitude restriction (based on the legacy design) is supported. Agreement For the Rel-18 TRS-based TDCP reporting, for TDCP measurement and calculation, confirm the following working assumption as an agreement with the following change - KTRS ≥1 TRS resource set(s) can be configured in the CSI reporting setting when ReportQuantity is ‘tdcp’ - Note: the TRS resource set(s) configured for TDCP report do not impact or impose any new requirements on the UE behavior when processing TRS used as QCL type A/D source for reception of PDxCH. - No further spec enhancement on TRS is supported - [All the TRS resources in the configured resource set(s) share the same RE locations] - FFS: Whether to add further restrictions on the TRS resource set(s) on, e.g. QCL relationship, power control, [RE location], slot offset between TRS resource set(s), relation with resource set used for legacy usage Agreement For the Rel-18 TRS-based TDCP reporting, regarding the value of parameter Y, in addition to Y=1, support Y=2, 3, 4 - FFS: Whether Y=7 is also supported Conclusion For the Rel-18 TRS-based TDCP reporting, there is no consensus on specifying a new priority rule. Therefore, the priority of the CSI report(s) associated with TDCP reporting is the same as CSI report(s) not carrying L1-RSRP or L1-SINR Agreement On the Type-II codebook refinement for CJT mTRP, regarding CBSR, one of the NTRP configured CSI-RS resources must be configured with CBSR, while the remaining (NTRP –1) configured CSI-RS resources can be optionally configured with CBSR - Note: if CBSR of one particular resource is absent, it means no restriction for SD basis selection for the resource. Agreement On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, support reusing the legacy UCI omission mechanism while (Alt3) replacing SD basis index l in legacy Prio calculation with , i.e., SD basis index over all resources: Prio(,l,m,n) = 2Ltot.RI.P(m)+ RI.+RI.l(n)+ - FFS: FD permutation P(.) as Rel-16-analogous, or no permutation i.e. P(m)=m Agreement For the Rel-18 TRS-based TDCP reporting, - Support the following D (delay) values: 4 symbols, 1 slot, 2 slots, 3 slots, 4 slots, 5 slots - Working assumption: Support the following D (delay) values in a separate UE Feature Group: 6 slots, 10 slots FFS: The value of Dbasic FFS: Applicability of each D value candidate for different SCS values and/or other parameters (e.g. Y, quantization) Conclusion On the Parameter Combination of Type-II codebook refinement for CJT mTRP, for Rel-17 FeType-II based, there is no consensus on introducing restriction “NTRP≤3, NL =1” for M=2. Conclusion On the Type-II codebook refinement for CJT mTRP, regarding CBSR for NTRP=1, there is no consensus in supporting the additional optional soft amplitude restriction. Therefore, only hard amplitude restriction (per CSI-RS resource, based on the legacy design) is supported. Agreement For the Type-II codebook refinement for high/medium velocities based on Rel-16 eType-II regular codebook, in addition to the already agreed six Parameter Combinations, the following three Parameter Combinations are supported: 2; 1/8; 1/16; ¼. 2 (*); ¼; 1/8; ½. 4 (*); ¼; 1/8; ¼. Agreement For the Rel-18 TRS-based TDCP reporting, regarding the quantization of wideband normalized amplitude value, down-select (by RAN1#113) from the following candidates: - Alt1: N=2Q-1 where Q=5, s={1/5, ¼, 1/3} - Alt2: N=2Q where Q=3, s={¼, 1/3, ½, 2/3, ¾} - Alt3: N=2Q where Q=4, s={¼, ½, 2/3, ¾} - Alt4: N={2Q –1, …, 2Q+1 –1} (i.e., 7-15) where Q=3, s={1/5, ¼, 1/3, 2/5, ½, 3/5, 2/3, ¾, 4/5} - Alt4A: N={2Q , 2Q+0.5,…, 2Q+1-0.5} (i.e., 8, 8.5,…,15.5) where Q=3, s={1/5, ¼, 1/3, 2/5, ½, 3/5, 2/3, ¾, 4/5} Once an alternative is selected, reducing the number of candidate values for s is not precluded. Companies can simulate each alternative with and without a configurable center threshold R1-2304065 Moderator Summary#3 on Rel-18 CSI enhancements: Round 2 Moderator (Samsung) Agreement For the Type-II codebook refinement for high/medium velocities, when a UE is configured with X=2 for CQI calculation and reporting, the 2nd CQI includes 4-bit wideband CQI and 2-bit sub-bands CQIs calculated independently from the 1st CQI Agreement On the Type-II codebook refinement for high/medium velocities, regarding UCI omission - When X=2 is configured, the 2nd TD CQI location reuses the legacy rule for the 2nd codeword CQI when RI>4, i.e. wideband CQI in G0, even-indexed sub-band CQIs in G1, odd-indexed sub-band CQIs in G2 - FFS: When the configured value of N4 is >1, whether the DD basis selection indicator is placed in G0 or G1 Agreement On the Parameter Combination of Type-II codebook refinement for CJT mTRP, for Rel-17 FeType-II based, only the following n combinations are supported (after pruning): NTRP; combination. 2; {1/2,1/2}. {1/2,1}, {1,1/2}. {3/4,3/4}. {1,1}. 3; {1/2, 1/2, 1/2}. {1/2, 1/2, 3/4}, and its permutations. {1/2, 1/2, 1}, and its permutations. {1, 1, 1}. 4; {1/2, 1/2, 1/2, 1/2}. {1/2, 1/2, 1/2, 1} and its permutations. {1/2, 1/2, 1, 1}. {1, 1, 1, 1}. Conclusion There is no consensus on the support of optional bitmap for Q=2 Agreement On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, reuse the Rel-16 eType-II (legacy) permutation function P(m) Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, - For Rel-16 eType-II-based: - where represents the indices of the N selected CSI-RS resources (out of the NTRP configured CSI-RS resources) - The payload of is determined by where the maximum is taken over the NL configured {Ln} combinations - For Rel-17 FeType-II-based: - where represents the indices of the N selected CSI-RS resources (out of the NTRP configured CSI-RS resources) - The payload of is determined by where the maximum is taken over the NL configured {an} combinations - Note: . Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding CSI calculation and measurement, - For the configured NTRP CSI-RS resources comprising the CMR, the restriction specified for Rel-17 NCJT CSI is fully reused, i.e. the configured NTRP CSI-RS resources are located either in the same slot or two consecutive slots - On PDSCH EPRE assumption for CQI calculation, down-select between the two alternatives: - Alt1. The UE can assume that the PDSCH EPRE for a given CSI-RS port follows the configured powerControlOffset value associated with its respective CSI-RS resource - Alt2. The UE can assume that the PDSCH EPRE for a given CSI-RS port follows a commonly configured powerControlOffset value for all the N selected CSI-RS resources - Alt3. The UE can assume that the PDSCH EPRE for a given CSI-RS port follows a commonly configured powerControlOffset value defined as averagePDSCH-to-averageCSIRS EPRE ratio, where averagePDSCH and averageCSIRS are average power across for all the N selected CSI-RS resources - Alt4. The UE can assume that the PDSCH EPRE divided by N for a given CSI-RS port follows a commonly configured powerControlOffset value for all the N selected CSI-RS resources - Alt 5: The UE can assume that the PDSCH EPRE for a given CSI-RS port follows the powerControlOffset value for one of the configured NTRP CSI-RS resources - Note: In legacy specification, different CSI-RS resources can be configured with different powerControlOffset values - Decide, in RAN1#113, whether an ordering of CSI-RS port indices (e.g. according to the CSI-RS resource ID in TS38.331) for CSI calculation needs to be specified or not Note: The total number of CSI-RS ports summed across N selected (out of the configured NTRP) CSI-RS resources will be used in the TS38.214 equation for CSI calculation Agreement On the Type-II codebook refinement for CJT mTRP, regarding the required number of CPUs and the values of Z/Z’, decide, in RAN1#113, at least based on the following factors: - The potential increase in the total number of CSI-RS ports due to the selection/configuration of N/ NTRP CSI-RS resources for Type-II CSI - The support for dynamic TRP selection, wherein N CSI-RS resources are selected out of the configured NTRP CSI-RS resources - Note: The fall-back of gNB configuring N=NTRP via RRC signalling is supported - The support for dynamic {Ln} selection, wherein 1 out of NL {Ln} combinations is selected - Note: The fall-back of gNB configuring NL=1 is supported Conclusion For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding the supported values of NL, there is no consensus in adding new value(s) (e.g. NL=3) to, or removing any value from the agreed NL ={1,2,4} Conclusion On the Type-II codebook refinement for CJT mTRP, regarding the codebook parameter R, there is no consensus on supporting R=4 Conclusion For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding interference measurement, beyond that supported in legacy specification, there is no consensus on supporting any additional enhancement on IMR (including the configuration for NZP CSI-RS for interference measurement or CSI-IM in relation to the configured CMR(s)). - Note: This implies that only one NZP CSI-RS resource for interference measurement or only one CSI-IM resource can be configured irrespective of the value of NTRP Agreement On the Type-II codebook refinement for high/medium velocities, regarding UCI omission, when the configured value of N4 is >12, the DD basis selection indicator is placed in G1 Agreement For the Type-II codebook refinement for high/medium velocities, - For Rel-16 eType-II-based: - For Rel-17 FeType-II-based: - Note: and . Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding CSI calculation and measurement, - The number of CSI-RS ports is the same for all the K configured CSI-RS resources comprising the CMR and the antenna ports for the same antenna port index across the K CSI-RS resources are the same. - All the K configured CSI-RS resources comprising the CMR share the same BW and RE locations - For interference measurement, legacy specification is fully reused, including the configuration for NZP CSI-RS for interference measurement or CSI-IM in relation to the configured CMR, i.e. only one NZP CSI-RS resource for interference measurement or only one CSI-IM resource can be configured irrespective of the value of K - On PDSCH EPRE assumption for CQI calculation, a same powerControlOffset value is assumed for all the K configured CSI-RS resources comprising the CMR - Alt 1: The configured powerControlOffset value is the same for all the K configured CSI-RS resources comprising the CMR - Alt 2: The assumed PDSCH EPRE of all the K CSI-RS resources follows the configured powerControlOffset value of one fixed CSI-RS resource, e.g. the first one Note: This may imply that existing section 5.2.2.2.75 of TS38.214 can apply to Rel-18 Type-II Doppler codebook in terms of Rel-18 CMR (burst of CSI-RS resources) and Rel-18 CSI reference resource Agreement For the Type-II codebook refinement for high/medium velocities, regarding the required number and/or occupation time of CPUs, the values of Z/Z’, and total number active/simultaneous CSI-RS resource/ports, decide, in RAN1#113, at least based on the following factors: - The measurement of K>1 CSI-RS resources for Type-II CSI required to perform UE-side prediction, UE-side prediction based on multiple CSI-RS occasion(s) before CSI triggering (FFS whether to support), CSI-RS occasion(s) after CSI triggering and, when the configured N4 value is >1, DD compression (when the configured N4 value is >1) Conclusion On the Type-II codebook refinement for CJT mTRP, the lists of UCI parameters (along with the description of each parameter) are given in Table 1C, 1D, and 1E. - Note: The manner in which the UCI parameters are captured is up to the spec editors Table 1C: UCI parameter list for Rel-16 based Parameter; UCI; Details/description; Status. # NZ coefficients; Part 1; RI ({1,…, RIMAX}) and KNZ,TOT (the total number of non-zero coefficients summed across all the layers and all N CSI-RS resources, where KNZ,TOT {1,2,…, 2K0} are reported in UCI part 1; Complete. Wideband CQI; Part 1; Same as R15; Complete. Subband CQI; Part 1; Same as R15; Complete. CSI-RS resource selection bitmap; Part 1; Only reported when NTRP >1: NTRP-bit bitmap to indicate the UE recommendation of N CSI-RS resources - Non-existent if the value of N is RRC-configured to NTRP; Complete. Indication of number of SD basis vectors {L1, …, LNTRP}; Part 1; UE recommendation selecting one of the NL RRC-configured value combinations (-bit indicator) - Non-existent if NL=1; Complete. N Bitmap(s) per layer; Part 2; For RI=1-4: for layer l and CSI-RS resource n, size- where n denotes the n-th CSI-RS resource; Complete. Strongest coefficient indicator (SCI); Part 2; RI=1: A -bit indicator for the strongest coefficient index RI>1: See Table 1E below; Complete. SD basis subset selection indicator for each of the N CSI-RS resources; Part 2; SD basis subset selection indicator is a -bit indicator for n=0,1,…,N–1. Details follow Rel.15; Complete. FD basis subset selection indicator; Part 2; Mode-1: See Table “SCI and FD basis subset selection indicator“ below + (N – 1) FD basis selection window offset values (basic) or (optional), n=1,2,…,N–1 Mode-2: See Table 1E “SCI and FD basis subset selection indicator“ below; Mode-1 complete Mode-2 complete. LC coefficients: phase; Part 2; Quantized independently across layers; Complete. LC coefficients: amplitude; Part 2; Alt1 (agreed): Quantized independently across layers (including a reference amplitude for weaker polarization, for each layer) Alt3 (WA): Quantized independently across layers (including 2N-1 reference amplitudes for 2N-1 (polarization, CSI-RS resource) pairs excluding the pair of (polarization, CSI-RS resource) associated with the SCI, for each layer); WA on Alt3 support needs to be confirmed or reverted. SD oversampling (rotation) factor q1, q2; Part 2; Values of q1,n, q2,n follow Rel.15, reported per CSI RS resource; Complete. Table 1D: UCI parameter list for Rel-17 based Parameter; UCI; Details/description; Status. # NZ coefficients; Part 1; RI ({1,…, RIMAX}) and KNZ,TOT (the total number of non-zero coefficients summed across all the layers and all N CSI-RS resources, where KNZ,TOT {1,2,…, 2K0} are reported in UCI part 1; Complete. Wideband CQI; Part 1; Same as R15; Complete. Subband CQI; Part 1; Same as R15; Complete. CSI-RS resource selection bitmap; Part 1; NTRP-bit bitmap to indicate the UE recommendation of N CSI-RS resources - Non-existent if the value of N is RRC-configured to NTRP; Complete. Indication of number of selected ports {L1, …, LNTRP}, where Ln=n PCSI-RS /2; Part 1; UE recommendation selecting one of the NL RRC-configured value combinations (-bit indicator) - Non-existent if NL=1; Complete. N Bitmap(s) per layer; Part 2; For layer l and CSI-RS resource n, size-, or ( where ); Complete. Strongest coefficient indicator (SCI); Part 2; For layer l: A -bit indicator for the strongest coefficient index; Complete. Port selection indicator for each of the N CSI-RS resources; Part 2; Port selection indicator is a -bit indicator for n=0,1,…,N–1, where Ln=n PCSI-RS /2. Details follow Rel.15; Complete. FD basis subset selection indicator; Part 2; Mode-1: See Mode-2+ (N – 1) FD basis selection window offset values (basic) or (optional), n=1,2,…,N–1 Mode-2: a bit indicator only if N>M=2, where is configured with the higher-layer parameter valueOfN, when .; Mode-1 complete Mode-2 complete. LC coefficients: phase; Part 2; Quantized independently across layers; Complete. LC coefficients: amplitude; Part 2; Alt1 (agreed): Quantized independently across layers (including a reference amplitude for weaker polarization, for each layer) Alt3 (WA): Quantized independently across layers (including 2N-1 reference amplitudes for 2N-1 (polarization, CSI-RS resource) pairs excluding the pair of (polarization, CSI-RS resource) associated with the SCI, for each layer); WA on Alt3 support needs to be confirmed or reverted. Table 1E: SCI and FD basis subset selection indicator for Rel-16-based Type-II CJT SCI and FD basis subset selection indicator. SCI for RI>1; Per-layer SCI defined across N CSI-RS resources, where is a –bit () indicator. The location (index) of the strongest LC coefficient for layer before index remapping is , , and is not reported. Index remapping; For layer , the index of each nonzero LC coefficient is remapped with respect to to such that . The FD basis index associated to each nonzero LC coefficient is remapped with respect to to such that . The sets and are reported. Informative note (for the purpose of reference procedure): The index of nonzero LC coefficients is remapped as . The codebook index associated with nonzero LC coefficient index is remapped as .. Combinatorial indicator for; bits. Combinatorial indicator for; bits. Reported in UCI part 2, , bits. (*) The red highlight parts are the new components in Rel-18 Conclusion For the Type-II codebook refinement for high/medium velocities, there is no consensus on supporting the following additional features when the value of N4 is 1 (or configured to 1): - X=2 TD CQIs - Additional constraint on the value of d: only d=1 is allowed Conclusion On the Type-II codebook refinement for high/medium velocities, the lists of UCI parameters (along with the description of each parameter) are given in Table 3C, 3D, and 3E. - Note: The manner in which the UCI parameters are captured is up to the spec editors Table 3C: UCI parameter list for Rel-16 based Parameter; UCI; Details/description; Status. # NZ coefficients; Part 1; RI ({1,…, RIMAX}) and KNZ,TOT (the total number of non-zero coefficients summed across all the Q selected DD basis and across all the layers, are reported in UCI part 1; Complete. Wideband CQI; Part 1; Same as R15; Complete. Subband CQI; Part 1; Same as R15; Complete. Wideband CQI for the second TD CQI; Part 2; Only applicable for X=2 (same format as CQIs for 2CW when RI>4 in R15); Complete. Subband CQI for the second TD CQI; Part 2; Only applicable for X=2 (same format as CQIs for 2CW when RI>4 in R15); Complete. Q Bitmap(s) per layer; Part 2; Q bitmaps where each bitmap has the same format/design as R16 eType-II; Complete. Strongest coefficient indicator (SCI); Part 2; RI=1: A -bit indicator for the strongest coefficient index RI>1: See Table 3E below; Complete. SD basis subset selection indicator; Part 2; SD basis subset selection indicator is a -bit indicator. Details follow Rel.15; Complete. FD basis subset selection indicator; Part 2; Details follow Rel.16 (Table 3E above); Complete. DD basis subset selection indicator (per layer); Part 2; Reported only when N4>2 and Q=2: the selection of Q out of N4 DD basis vectors is indicated by a -bit indicator; Complete. LC coefficients: phase; Part 2; Quantized independently across layers; Complete. LC coefficients: amplitude; Part 2; Quantized independently across layers (including a reference amplitude for weaker polarization, for each layer); Complete. SD oversampling (rotation) factor q1, q2; Part 2; Values of q1, q2 follow Rel.15; Complete. Table 3D: UCI parameter list for Rel-17 based Parameter; UCI; Details/description; Status. # NZ coefficients; Part 1; RI (Î{1,…, RIMAX}) and KNZ,TOT (the total number of non-zero coefficients summed across all the layers, are reported in UCI part 1; Complete. Wideband CQI; Part 1; Same as R15; Complete. Subband CQI; Part 1; Same as R15 (only X=1 TD CQI is supported); Complete. Bitmap per layer; Part 2; Same as R17 eType-II; Complete. Strongest coefficient indicator (SCI); Part 2; For layer l: A -bit indicator for the strongest coefficient index; Complete. Port selection indicator; Part 2; Port selection indicator is a -bit indicator. Where , Details follow Rel.17; Complete. FD basis subset selection indicator; Part 2; a bit indicator only if N>M=2, where is configured with the higher-layer parameter valueOfN, when .; Complete. LC coefficients: phase; Part 2; Quantized independently across layers; Complete. LC coefficients: amplitude; Part 2; Quantized independently across layers (including a reference amplitude for weaker polarization, for each layer); Complete. Table 3E: SCI and FD basis subset selection indicator for Rel-16-based Type-II Doppler SCI and FD basis subset selection indicator. SCI for RI>1; Per-layer SCI defined across Q DD basis vectors, where is a –bit () indicator. The location (index) of the strongest LC coefficient for layer before index remapping is , indicates and is not reported. Index remapping; For layer , the index of each nonzero LC coefficient is remapped with respect to to such that . The FD basis index associated to each nonzero LC coefficient is remapped with respect to to such that . The sets and are reported. Informative note (for the purpose of reference procedure): The index of nonzero LC coefficients is remapped as . The codebook index associated with nonzero LC coefficient index is remapped as .. Combinatorial indicator for; bits. Combinatorial indicator for; bits. Reported in UCI part 2, , , bits. (*) The red highlighted parts are the new components in Rel-18 Agreement For the Rel-18 TRS-based TDCP reporting, for TDCP measurement and calculation, at least the following restrictions are supported: - When all the configured KTRS resource sets are periodic, the UE can assume that all the resource sets share a same QCL-Type-A/C and, if applicable, Type-D source - If the joint use of P and AP-TRS resource sets is supported for TDCP measurement and calculation, when one of the KTRS configured resource sets is aperiodic, the UE can assume that the aperiodic resource set is configured with QCL-Type-A and, if applicable, Type-D source with the resources of the one of the (KTRS – 1) periodic TRS resource sets - Note: Following the legacy specification, no more than 1 of the KTRS resource sets is aperiodic - TBD (RAN1#113): whether the joint use of P and AP-TRS resource sets is supported for TDCP measurement and calculation or not - FFS: whether the UE shall assume the same antenna port for the CSI-RS resources in all the resource sets Agreement For the Rel-18 TRS-based TDCP reporting, regarding phase quantization, down-select (by RAN1#113) from the following candidates: - Alt1. 1-bit (early vs. late) phase indicator - Alt2. 3-bit (8-PSK) uniform quantization - Alt3. 4-bit (16-PSK) uniform quantization (full reuse of Rel-16 eType-II W2 phase quantization) - Alt4. Adaptive/gNB-configurable phase quantizer e.g. , where - : legacy (Rel.16) based - Linear: legacy -PSK - Exponential: legacy Rel.16 amplitude, or - a slope value from depending on the amplitude ) of the 1st correlation (smallest delay), e.g. the slope decreases towards 0 as increases towards 1 - Alt5. A given correlation phase value is quantized to based on the following alphabet (where denotes delay): - Alt6. A given correlation phase value is quantized to based on the following alphabet (where denotes delay and p(.) denotes amplitude quantization values used for Rel-16 e-TypeII codebook and ): - Mode 1: , - Mode 2: - The quantization mode is selected by UE and reported to gNB. - Alt7. A given correlation phase value is quantized to based on the following alphabet: , with , . TBD value(s) of The evaluation should consider the impact of delay tracking operation at the UE where the phase difference between two slots can be close to zero. Note: This proposal doesn’t preclude the UE supporting only smaller delay values (e.g. 4-symbol only) for the phase report (which is already optional) Conclusion For the Type-II codebook refinement for high/medium velocities, regarding SCI definition, there is no consensus on supporting the index remapping scheme analogous to that for FD basis for DD basis. Therefore, is a –bit indicator where and Q is the number of DD basis vectors (1 or 2)

In this contribution, we provide some discussion on CSI enhancement.

# CSI for CJT

## CMR configuration

In RAN1 #110, the following on CMR configuration was agreed, where the gNB can configure multiple CSI-RS resources as CMR and the CSI-RS resources should have the same number of CSI-RS ports.

Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP with NTRP>1 TRP/TRP-groups, the following is supported: - The CMR comprises K>1 NZP CSI-RS resources, where one resource corresponds to one TRP/TRP-group (i.e. K=NTRP) - Each of the CSI-RS resources has a same number of CSI-RS ports - Note: The terms TRP and TRP-group are used for discussion purposes only (no spec impact is implied).

With regard to the CSI measurement, the allocated RBs for the CSI-RS resources should also be the same. Otherwise, the measurement bandwidth should be clarified, e.g., whether the UE should measurement CSI based on the union or the overlapped bandwidth for the CMRs. Therefore, a simple solution is to define that the allocated RBs for the CMRs should be the same.

Proposal 1: With regard to CSI measurement, the CSI-RS resources configured as CMR for CJT should be transmitted from the same RBs.

## CQI measurement for CJT

In RAN1 #112b, the following is agreed on the EPRE ratio assumption for CQI measurement.

Agreement For the Rel-18 Type-II codebook refinement for CJT mTRP, regarding CSI calculation and measurement, - For the configured NTRP CSI-RS resources comprising the CMR, the restriction specified for Rel-17 NCJT CSI is fully reused, i.e. the configured NTRP CSI-RS resources are located either in the same slot or two consecutive slots - On PDSCH EPRE assumption for CQI calculation, down-select between the two alternatives: - Alt1. The UE can assume that the PDSCH EPRE for a given CSI-RS port follows the configured powerControlOffset value associated with its respective CSI-RS resource - Alt2. The UE can assume that the PDSCH EPRE for a given CSI-RS port follows a commonly configured powerControlOffset value for all the N selected CSI-RS resources - Alt3. The UE can assume that the PDSCH EPRE for a given CSI-RS port follows a commonly configured powerControlOffset value defined as averagePDSCH-to-averageCSIRS EPRE ratio, where averagePDSCH and averageCSIRS are average power across for all the N selected CSI-RS resources - Alt4. The UE can assume that the PDSCH EPRE divided by N for a given CSI-RS port follows a commonly configured powerControlOffset value for all the N selected CSI-RS resources - Alt 5: The UE can assume that the PDSCH EPRE for a given CSI-RS port follows the powerControlOffset value for one of the configured NTRP CSI-RS resources - Note: In legacy specification, different CSI-RS resources can be configured with different powerControlOffset values - Decide, in RAN1#113, whether an ordering of CSI-RS port indices (e.g. according to the CSI-RS resource ID in TS38.331) for CSI calculation needs to be specified or not Note: The total number of CSI-RS ports summed across N selected (out of the configured NTRP) CSI-RS resources will be used in the TS38.214 equation for CSI calculation

Based on the configured EPRE ratio, the UE can identify the combined channel by H=[α1H1, α2H2, α3H3, α4H4], where αk can be derived from the configured EPRE ratio for each CSI-RS, thus, the configured powerControlOffset should indicate the EPRE ratio between the PDSCH and CSI-RS from the same TRP, which is aligned with Alt1.

Proposal 2: For CQI measurement for CJT, the EPRE ratio (Pc) should be defined as the EPRE ratio between CSI-RS and PDSCH from the same TRP (Alt1).

# CSI for high/medium UE velocities

## CMR configuration

In RAN1 #112b, the following agreement on the CMR for CSI feedback for high/medium velocities has been achieved.

Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding CSI calculation and measurement, - The number of CSI-RS ports is the same for all the K configured CSI-RS resources comprising the CMR and the antenna ports for the same antenna port index across the K CSI-RS resources are the same. - All the K configured CSI-RS resources comprising the CMR share the same BW and RE locations - For interference measurement, legacy specification is fully reused, including the configuration for NZP CSI-RS for interference measurement or CSI-IM in relation to the configured CMR, i.e. only one NZP CSI-RS resource for interference measurement or only one CSI-IM resource can be configured irrespective of the value of K - On PDSCH EPRE assumption for CQI calculation, a same powerControlOffset value is assumed for all the K configured CSI-RS resources comprising the CMR - Alt 1: The configured powerControlOffset value is the same for all the K configured CSI-RS resources comprising the CMR - Alt 2: The assumed PDSCH EPRE of all the K CSI-RS resources follows the configured powerControlOffset value of one fixed CSI-RS resource, e.g. the first one Note: This may imply that existing section 5.2.2.2.75 of TS38.214 can apply to Rel-18 Type-II Doppler codebook in terms of Rel-18 CMR (burst of CSI-RS resources) and Rel-18 CSI reference resource

It was agreed that the CMRs should be from the same RE location and bandwidth. Now that the CMRs are repetitions, it is necessary to provide the same configuration of powerControlOffset for CQI calculation, which is aligned with Alt1.

Proposal 3: For CMRs for CSI measurement for high/medium velocities, support Alt1 (The configured powerControlOffset value is the same for all the K configured CSI-RS resources comprising the CMR).

# TDCP report

## CMR for TDCP report

In RAN1 #112b, the following on TRS configuration for TDCP report was agreed.

Agreement For the Rel-18 TRS-based TDCP reporting, for TDCP measurement and calculation, confirm the following working assumption as an agreement with the following change - KTRS ≥1 TRS resource set(s) can be configured in the CSI reporting setting when ReportQuantity is ‘tdcp’ - Note: the TRS resource set(s) configured for TDCP report do not impact or impose any new requirements on the UE behavior when processing TRS used as QCL type A/D source for reception of PDxCH. - No further spec enhancement on TRS is supported - [All the TRS resources in the configured resource set(s) share the same RE locations] - FFS: Whether to add further restrictions on the TRS resource set(s) on, e.g. QCL relationship, power control, [RE location], slot offset between TRS resource set(s), relation with resource set used for legacy usage Agreement For the Rel-18 TRS-based TDCP reporting, for TDCP measurement and calculation, at least the following restrictions are supported: - When all the configured KTRS resource sets are periodic, the UE can assume that all the resource sets share a same QCL-Type-A/C and, if applicable, Type-D source - If the joint use of P and AP-TRS resource sets is supported for TDCP measurement and calculation, when one of the KTRS configured resource sets is aperiodic, the UE can assume that the aperiodic resource set is configured with QCL-Type-A and, if applicable, Type-D source with the resources of the one of the (KTRS – 1) periodic TRS resource sets - Note: Following the legacy specification, no more than 1 of the KTRS resource sets is aperiodic - TBD (RAN1#113): whether the joint use of P and AP-TRS resource sets is supported for TDCP measurement and calculation or not - FFS: whether the UE shall assume the same antenna port for the CSI-RS resources in all the resource sets

Compared to configure two periodic TRS sets, the joint periodic+aperiodic TRS and two aperiodic TRS sets should be more useful with regard to the overhead reduction. Therefore, it is unnecessary to introduce a restriction to preclude the TDCP measurement and report based on joint periodic+aperiodic TRS and two aperiodic TRS sets.

For the two TRS sets configured for the TDCP measurement, only QCL related constraints are not sufficient, since different antenna virtualization schemes could still be possible. Therefore, the TRSs should be from the same antenna ports, which is similar to current definition of resources within a TRS set.

Proposal 4: Do not support to introduce additional constraints to preclude TDCP measurement based on periodic+aperiodic TRS sets and aperiodic+aperodic TRS sets.

Proposal 5: Support to define the TRS sets configured for TDCP measurement are from the same port.

## Quantization for TDCP report

In RAN1 #112b, the following on amplitude quantization for TDCP has been agreed.

Agreement For the Rel-18 TRS-based TDCP reporting, regarding the quantization of wideband normalized amplitude value, down-select (by RAN1#113) from the following candidates: - Alt1: N=2Q-1 where Q=5, s={1/5, ¼, 1/3} - Alt2: N=2Q where Q=3, s={¼, 1/3, ½, 2/3, ¾} - Alt3: N=2Q where Q=4, s={¼, ½, 2/3, ¾} - Alt4: N={2Q –1, …, 2Q+1 –1} (i.e., 7-15) where Q=3, s={1/5, ¼, 1/3, 2/5, ½, 3/5, 2/3, ¾, 4/5} - Alt4A: N={2Q , 2Q+0.5,…, 2Q+1-0.5} (i.e., 8, 8.5,…,15.5) where Q=3, s={1/5, ¼, 1/3, 2/5, ½, 3/5, 2/3, ¾, 4/5} Once an alternative is selected, reducing the number of candidate values for s is not precluded. Companies can simulate each alternative with and without a configurable center threshold

Table 1 illustrates the simulation results of average quantization error for each alternative. Since there are different configuration of N and s in each alternative, the table only counts the smallest error based on one configuration. It can be observed that the quantization error still depends on the number of bits. With regard to the tradeoff between quantization error and payload size, either Alt1 or Alt3 should be preferred. Further, according to Table 2 and Table 3 different value of s does not provide significant performance difference. We recommend to select s=0.2 for Alt1 and s=0.25 for Alt2 to simplify the implementation.

Table 1: Average quantization error for each alternative

| Alternative | Quantization error |
| --- | --- |
| Alt 1 | 0.0175 |
| Alt 2 | 0.0465 |
| Alt 3 | 0.0241 |
| Alt 4 | 0.0437 |
| Alt 5 | 0.0510 |

Table 2: Average quantization error for each s in Alt 1

| s | Quantization error |
| --- | --- |
| 1/5 | 0.0175 |
| 1/4 | 0.0214 |
| 1/3 | 0.0289 |

Table 1: Average quantization error for each s in Alt3

| s | Quantization error |
| --- | --- |
| 1/4 | 0.0241 |
| 1/2 | 0.0429 |
| 2/3 | 0.0561 |
| 3/4 | 0.0628 |

Proposal 6: Support Alt1 with s=0.2 or Alt3 with s=0.25 for amplitude quantization for TDCP report.

Further, in RAN1 #112b, the following on phase quantization was agreed.

Agreement For the Rel-18 TRS-based TDCP reporting, regarding phase quantization, down-select (by RAN1#113) from the following candidates: - Alt1. 1-bit (early vs. late) phase indicator - Alt2. 3-bit (8-PSK) uniform quantization - Alt3. 4-bit (16-PSK) uniform quantization (full reuse of Rel-16 eType-II W2 phase quantization) - Alt4. Adaptive/gNB-configurable phase quantizer e.g. , where - : legacy (Rel.16) based - Linear: legacy -PSK - Exponential: legacy Rel.16 amplitude, or - a slope value from depending on the amplitude ) of the 1st correlation (smallest delay), e.g. the slope decreases towards 0 as increases towards 1 - Alt5. A given correlation phase value is quantized to based on the following alphabet (where denotes delay): - Alt6. A given correlation phase value is quantized to based on the following alphabet (where denotes delay and p(.) denotes amplitude quantization values used for Rel-16 e-TypeII codebook and ): - Mode 1: , - Mode 2: - The quantization mode is selected by UE and reported to gNB. - Alt7. A given correlation phase value is quantized to based on the following alphabet: , with , . TBD value(s) of The evaluation should consider the impact of delay tracking operation at the UE where the phase difference between two slots can be close to zero. Note: This proposal doesn’t preclude the UE supporting only smaller delay values (e.g. 4-symbol only) for the phase report (which is already optional)

Since the use cases for the reported phase could be different, different quantization schemes can be considered. For example, if the phase is used for Doppler spectrum estimation, higher resolution could be considered, e.g., quantization scheme like Alt3. If the phase is only used to indicate the early or late information, 1-bit indicator could be sufficient. Therefore, the number of bits for the phase quantization can be configured by the gNB based on different use cases.

Proposal 7: Support the gNB to configure the number of bits for phase quantization based on Alt1 (1-bit) and Alt3 (4-bit with the same scheme as eType2 CSI).

## TDCP measurement behavior

Usually, a UE should have more than one Rx ports. Then the UE can measure the TDCP from the more the one Rx ports. However, it is unnecessary for the UE to report the TDCP for each Rx ports, since such Rx ports should share similar time domain channel correlation property. Therefore, similar to other metric, e.g., L1-RSRP/L1-SINR, the reported TDCP should be measured based on the average TDCP across the Rx ports.

Proposal 8: The reported TDCP should be the average TDCP measured across the Rx ports.

# Conclusion

In this contribution, we provided discussion on CSI enhancement. Based on the discussion, the following proposals have been achieved.

CSI for CJT

Proposal 1: With regard to CSI measurement, the CSI-RS resources configured as CMR for CJT should be transmitted from the same RBs.

Proposal 2: For CQI measurement for CJT, the EPRE ratio (Pc) should be defined as the EPRE ratio between CSI-RS and PDSCH from the same TRP (Alt1).

CSI for high/medium UE velocities

Proposal 3: For CMRs for CSI measurement for high/medium velocities, support Alt1 (The configured powerControlOffset value is the same for all the K configured CSI-RS resources comprising the CMR).

TDCP report

Proposal 4: Do not support to introduce additional constraints to preclude TDCP measurement based on periodic+aperiodic TRS sets and aperiodic+aperodic TRS sets.

Proposal 5: Support to define the TRS sets configured for TDCP measurement are from the same port.

Proposal 6: Support Alt1 with s=0.2 or Alt3 with s=0.25 for amplitude quantization for TDCP report.

Proposal 7: Support the gNB to configure the number of bits for phase quantization based on Alt1 (1-bit) and Alt3 (4-bit with the same scheme as eType2 CSI).

Proposal 8: The reported TDCP should be the average TDCP measured across the Rx ports.