3GPP TSG RAN WG1 #113		R1-2304844

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.2.3.1

Source:	Google

Title:	On Evaluation of AI/ML based Beam Management

Document for:	Discussion/Decision

# Introduction

In RAN1 #112b, the following agreements on evaluation of AI/ML based BM have been achieved.

Agreement - For the evaluation of the overhead for BM-Case2, adoption the following metrics: - RS overhead reduction, - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable - where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme - Companies report the assumption on additional measurements - Companies report the assumption on baseline scheme - Companies report the assumption on T1 and T2 - Other options are not precluded and can be reported by companies. Agreement - To evaluate the performance of AI/ML in beam management at least for NW side beam prediction, UCI report overhead (e.g., number of UCI reports and UCI payload size) and/or UCI overhead reduction for inference of AI/ML model can be reported by company. - UCI overhead reduction = 1- Total UCI payload size for AI/ML/Total UCI payload size of baseline. - Companies to report detailed assumption of UCI for AI/ML and baseline, e.g., including quantization mechanism Conclusion - It is optional to evaluate and compare the performance for BM Case-1 with different UE distribution assumptions: - Option 1: 80% indoor, 20% outdoor as in TR 38.901 - Option 2: 100% outdoor Agreement At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable - Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample - Companies report how to select the “best” Rx beam(s) - Option 2: Measurements of specific Rx beam(s) - Companies report how to select specific Rx beam(s) - Option 3: Measurements of random Rx beam(s) per model input sample Other options are not precluded and can be reported by companies. Observation - At least for BM-Case1 for inference of DL Tx beam with L1-RSRPs of all beams in Set B, existing quantization granularity of L1-RSRP (i.e., 1dB for the best beam, 2dB for the difference to the best beam) causes [a minor loss x%~y%, if applicable] in beam prediction accuracy compared to unquantized L1-RSRPs of beams in Set B. Agreement - For AI/ML in beam management, further study performance with different types of label, considering the following: - Option 1a: Top-1 beam(pair) in Set A - Option 1b: Top-K beam (pair)s in Set A - Option 2a: L1-RSRPs per beam of all the beams(pairs) in Set A - Option 2b: Top-K beam(pair)s in Set A and the corresponding L1-RSRPs - Option 2c: Top-1 beam(pair) in Set A and the corresponding L1-RSRP - Other options are not precluded and can be reported by companies. Observation - For BM-Case1 DL Tx beam prediction, when Set B is a subset of Set A, AI/ML can provide good beam prediction performance with less measurement/RS overhead without considering generalization aspects with the measurements from the best Rx beam without UE rotation. - (A)With measurements of fixed Set B of beams that of 1/4 of Set A of beams - evaluation results [from 4 sources] indicate that, AI/ML can achieve [about 70%~80%] beam prediction accuracy of Top-1 DL Tx beam, evaluation results [from 6 sources] indicate that, AI/ML can achieve [about 80%~90%] beam prediction accuracy of Top-1 DL Tx beam, and evaluation results [from 4 sources] show [more than 90%] beam prediction accuracy of Top-1 DL Tx beam - evaluation results [from 8 sources] indicate that, AI/ML can achieve [more than 90%] beam prediction accuracy for Top-1 DL Tx beam with 1dB margin - evaluation results [from 8 sources] indicate that, AI/ML can achieve [more than 80%] beam prediction accuracy for Top-2 DL Tx beam. The beam prediction accuracy increases with K. - evaluation results [from 9 sources] indicate that, the average L1-RSRP difference of Top-1 predicted beam can be [below or about 1dB]. - (B) With measurements of fixed Set B of beams that of 1/8 of Set A of beams - evaluation results [from 2 sources] indicate that, AI/ML can achieve [about 50%] beam prediction accuracy of Top-1 DL Tx beam, evaluation results [from 3 sources] show [about 60%~70%] beam prediction accuracy of Top-1 DL Tx beam, and evaluation results [from 2 sources] show [about 70%~80] beam prediction accuracy of Top-1 DL Tx beam. - evaluation results [from 4 sources] indicate that, AI/ML can achieve [70%-90%] beam prediction accuracy for Top-1 DL Tx beam prediction with 1dB margin - evaluation results [from 2 sources] indicate that, AI/ML can achieve [about 70%~ 80%] beam prediction accuracy for Top-2 DL Tx beam, and evaluation results [from 4 sources] indicate that, AI/ML can achieve [more than 80%] beam prediction accuracy for Top-2 DL Tx beam. The beam prediction accuracy increases with K. - Note that ideal measurements are assumed - Beams could be measured regardless of their SNR. - No measurement error. - Measured in a single-time instance (within a channel-coherence time interval). - No quantization for the L1-RSRP measurements. - No constraint on UCI payload overhead for full report of the L1-RSRP measurements of Set B for NW-side models are assumed. Agreement For performance evaluation of AI/ML based DL Tx beam prediction for BM-Case1 and BM-Case2, optionally study the performance with a quasi-optimal Rx beam (i.e., not all the measurements as inputs of AI/ML are from the “best” Rx beam) with less measurement/RS overhead compared to exhaustive Rx beam sweeping. - At least the following options can be considered: - Opt A: Identify the quasi-optimal Rx beams to be utilized for measuring Set B/Set C based on the previous measurements. - Companies can report the time information and beam type (e.g., whether the same Tx beam(s) in Set B) of the reference signal to use. - Companies report how to find the quasi-optimal Rx beam with “previous measurement” - FFS: Opt B: The Rx beams for measuring Set B/Set C consist of the X% of “best” Rx beam exhaustive Rx beam sweeping and (1-X%) of random Rx beams [or the adjacent Rx beam to the “best” Rx beam]. - X%= 80% or 90%, or other values reported by companies. - Note: X% is the percentage of measurements with “best” Rx beams out of all measurements - Other options are not precluded. - Companies report the measurement/RS overhead together with beam prediction accuracy. Conclusion To evaluate the performance of BM-Case1 for both DL Tx beam and pair prediction, aiming to analysis the following aspects: - Clarify the baseline performance in terms of beam prediction accuracy and/or average L1-RSRP difference. - Other metrics to be considered: - Measurement/RS overhead reduction - UCI overhead (reduction) potentially with different quantization - User throughput - Model size /complexity - Average predicted L1-RSRP difference, if applicable - Performance difference based on the reported results from each company - Different Set B assumption - Opt A/B, Opt C, Opt D - [(optional) with UE rotation] - (optional) with different Rx assumption for DL Tx beam prediction/DL beam pair prediction and potentially with quasi-optimal Rx beam - (optional) with quantization - [(optional) with measurement error] - [(optional) with different label, including data collection for NW side model if supported] - [(optional) Impact of different beam pair pattern for beam pair prediction, e.g., - Tx down sampling only - Tx and Rx down sampling] - Other settings: - Other percentage of Set B and Set A if reported by companies - When Set B is different from Set A (e.g., Set B is composed of wide beams and Set A is composed of narrow beams). - Other aspects are not precluded - Observation/analysis may consider UE-side and NW-side model when applicable Observation - For BM-Case1 DL Tx beam prediction, when Set B is a subset of Set A without considering other generalization aspects with the measurements from the best Rx beam without UE rotation. - (Opt 2B) For the case that Set B of beams is changed among pre-configured patterns, evaluation results [from 4 sources] show that the beam prediction accuracy degrades [no more than 5%] in terms of Top-1 beam prediction accuracy than when Set B is fixed across training and inference, where the [one source] used [24] pre-configured patterns and the rest of sources use [4 or 5] patterns; evaluation results [from 1 source] show that the beam prediction accuracy degrades [about 10%] in terms of Top-1 beam prediction accuracy than when Set B is fixed across training and inference. - Note: the above performance can also be treated as training with mixed patterns of Set B of beam, and testing with mixed patterns Set B of beams. - Note: the measurements are obtained from the best Rx beam without UE rotation - Note that ideal measurements are assumed - Beams could be measured regardless of their SNR. - No measurement error. - Measured in a single-time instance (within a channel-coherence time interval). - No quantization for the L1-RSRP measurements. - No constraint on UCI payload overhead for full report of the L1-RSRP measurements of Set B for NW-side models are assumed. - This observation is based on Set B patterns that were chosen by each company. Conclusion To evaluate the performance of BMCase-2 for both DL Tx beam and pair prediction, aiming to analysis the following aspects: - Clarify the baseline performance in terms of beam prediction accuracy and/or average L1-RSRP. - Observations based on the metrics to be considered: - Top-1/K [=2] beam prediction accuracy, Top-1 beam prediction accuracy with 1dB, average L1-RSRP difference - Measurement/RS overhead reduction - UCI overhead (reduction) potentially with different quantization - User throughput - Model size and complexity - Average predicted L1-RSRP difference, if applicable - Scenarios/assumptions/Cases for basic observations - Set A and Set B relationship - Set A= Set B - Set B /Set A =1/4, [1/6], 1/8, 1/16, [1/32] - UE speed: 30km/h - No UE rotation - FFS the following cases for results reporting. - Case 1: based on T1 and T2, where T2 is the time duration for the best beam selection, and T1 is a time duration to obtain the measurements of all the RS resource from Set B of beams. - T1 = 40ms, 80ms, 160ms, [320ms], [640ms] - T2 = 40ms, 80ms, 160ms, 320ms, [960ms] - M= [1, 2, 3, 4, 5, 8], where M is the number of time instance(s) for measurement/report in T1 as AI/ML inputs (per model) - P= [1, 2, 4, 5, …], where P is the number of time instance(s) P for prediction as AI/ML model output(s)/label(s) (per model) - Case 2: based on the number of prediction instance(s) Y for every number of measurement instance(s) X, at least consider the following values: - Minimal periodicity for time instances for measurement(s) and prediction(s) = [40ms, 80ms, 160ms] - X = [1, 2] - Y = [1, 2, 4, 5, 10] - P= [1, 2, 4, 5, …], where P is the number of time instance(s) P for prediction as AI/ML model output(s)/label(s) (per model) - The number of measurement instance(s) as AI inputs are up to implementation. - FFS whether separated observations are needed or not for the following: - UE trajectories - Performance difference based on the reported results from each company - With UE rotation - Different UE speed: e.g., 60km/h, 90km/h, 120km/h - Different observation/prediction windows or periodicity for time instances. - Different Set B assumption when Set A is a subset of Set B - Opt A/B, Opt C, Opt D - Other settings: - Other percentage of Set B and Set A if reported by companies - When Set B is different from Set A (e.g., Set B is composed of wide beams and Set A is composed of narrow beams). - Other aspects are not precluded - Observation/analysis may consider UE-side and NW-side model when applicable

In this contribution, we provide some discussion on evaluation of AI/ML based BM.

# Discussion

## Clarification of KPI

The RS overhead reduction has been agreed as a KPI for evaluation. However, it is necessary to clarify the RS overhead reduction is only nominal, which does not actually reduce the DL RS for beam measurement. Regardless of whether ML is enabled or not, the NW still needs to transmit the DL RS for all the beams in set A, which can be used for further beam tracking, pathloss measurement, L3 measurement and so on. Thus, current RS overhead reduction is only from number of measured RSs perspective, which can help for UE power saving and reduce the latency for beam measurement and report, but cannot actually reduce the actual DL RS overhead.

Observation 1: Regardless of whether ML is enabled or not, the NW still needs to transmit the DL RS for all the beam sin set A for further beam tracking, pathloss measurement, L3 measurement and so on.

Proposal 1: Clarify that the KPI “RS overhead reduction” is only from the beam measurement perspective for a UE, which does not mean the overall RS overhead reduction from the network perspective.

## Spatial domain beam index prediction

### 2.1.1 Results based on different input content

In RAN1 #109, the following alternatives on spatial domain beam prediction were agreed.

Conclusion Regarding the sub use case BM-Case1, further study the following alternatives for AI/ML input: - Alt.1: Only L1-RSRP measurement based on Set B - Alt.2: L1-RSRP measurement based on Set B and assistance information - FFS: Assistance information. The following were mentioned by companions in the discussion: Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beamwidth, etc.), expected Tx and/or Rx beam for the prediction (e.g., expected Tx and/or Rx angle, Tx and/or Rx beam ID for the prediction), UE position information, UE direction information, Tx beam usage information, UE orientation information, etc. - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side. - Alt.3: CIR based on Set B - Alt.4: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID - Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives - Note2: All the inputs are “nominal” and only for discussion purpose.

In this contribution, we evaluation the following 3 options to investigate the performance from the alterantives above.

- Option 1: L1-RSRP measured from set B beams

- Option 2: L1-RSRP measured from set B beams + UE orientation

- Option 3: CIR measured from set B beams

For Set B beams, we studied 2, 4, and 8 beams in set B as shown in Figure 1, Figure 2 and Figure 3.

Figure 1: Selection of 2 beams in Set B as input for ML based spatial domain beam prediction

Figure 2: Selection of 4 beams in Set B as input for ML based spatial domain beam prediction

Figure 3: Selection of 8 beams in Set B as input for ML based spatial domain beam prediction

Table 1-6 illustrate the beam prediction accuracy for top-x beams, where the top-x beam is counted as correct if L1-RSRP from one of the predicted top-x beam is equal to or the same as the L1-RSRP from the best beam minus a margin (0dB or 1dB). It can be observed that the CIR based beam prediction could outperform the L1-RSRP based beam prediction. Moreover, the UE orientation cannot help to increase the beam prediction accuracy for L1-RSRP based beam prediction. The neural network is a DNN with 1 hidden layer. Detailed simulation assumptions are illustrated in Table A-1 in appendix.

Table 1: Beam prediction accuracy from 2 measured beams with 1 dB margin

| Predicted beam | Option 1 (L1-RSRP) | Option 2 (L1-RSRP) | Option 3 (CIR from 2 strongest taps) | Option 3 (CIR from 4 strongest taps) | Option 3 (CIR from 6 strongest taps) |
| --- | --- | --- | --- | --- | --- |
| Top-1 | 31.35% | 30.62% | 33.00% | 33.50% | 34.34% |
| Top-2 | 47.43% | 46.85% | 49.60% | 50.63% | 51.40% |
| Top-4 | 66.41% | 65.60% | 68.44% | 69.51% | 69.70% |
| Top-8 | 83.21% | 83.18% | 85.72% | 85.91% | 86.43% |

Table 2: Beam prediction accuracy from 2 measured beams with 0 dB margin

| Predicted beam | Option 1 (L1-RSRP) | Option 2 (L1-RSRP) | Option 3 (CIR from 2 strongest taps) | Option 3 (CIR from 4 strongest taps) | Option 3 (CIR from 6 strongest taps) |
| --- | --- | --- | --- | --- | --- |
| Top-1 | 20.61% | 20.06% | 22.04% | 22.43% | 23.02% |
| Top-2 | 34.38% | 34.32% | 36.53% | 37.45% | 38.24% |
| Top-4 | 53.27% | 53.99% | 56.19% | 57.03% | 57.39% |
| Top-8 | 75.03% | 76.14% | 77.74% | 78.25% | 78.54% |

Table 3: Beam prediction accuracy from 4 measured beams with 1 dB margin

| Predicted beam | Option 1 (L1-RSRP) | Option 2 (L1-RSRP) | Option 3 (CIR from 2 strongest taps) | Option 3 (CIR from 4 strongest taps) | Option 3 (CIR from 6 strongest taps) |
| --- | --- | --- | --- | --- | --- |
| Top-1 | 47.98% | 48.18% | 49.49% | 51.32% | 52.10% |
| Top-2 | 65.49% | 65.93% | 67.49% | 69.12% | 70.13% |
| Top-4 | 82.09% | 82.30% | 83.28% | 84.32% | 85.18% |
| Top-8 | 93.62% | 93.77% | 93.87% | 94.63% | 94.46% |

Table 4: Beam prediction accuracy from 4 measured beams with 0 dB margin

| Predicted beam | Option 1 (L1-RSRP) | Option 2 (L1-RSRP) | Option 3 (CIR from 2 strongest taps) | Option 3 (CIR from 4 strongest taps) | Option 3 (CIR from 6 strongest taps) |
| --- | --- | --- | --- | --- | --- |
| Top-1 | 33.66% | 33.99% | 36.17% | 37.40% | 38.10% |
| Top-2 | 51.77% | 52.05% | 54.24% | 55.50% | 56.48% |
| Top-4 | 70.90% | 71.07% | 73.13% | 74.42% | 75.35% |
| Top-8 | 88.92% | 88.92% | 88.78% | 89.72% | 89.72% |

Table 5: Beam prediction accuracy from 8 measured beams with 1 dB margin

| Predicted beam | Option 1 (L1-RSRP) | Option 2 (L1-RSRP) | Option 3 (CIR from 2 strongest taps) | Option 3 (CIR from 4 strongest taps) | Option 3 (CIR from 6 strongest taps) |
| --- | --- | --- | --- | --- | --- |
| Top-1 | 67.96% | 69.10% | 73.53% | 75.18% | 75.23% |
| Top-2 | 81.59% | 82.23% | 86.39% | 87.99% | 87.92% |
| Top-4 | 91.25% | 92.20% | 93.87% | 94.34% | 94.38% |
| Top-8 | 96.76% | 96.96% | 97.75% | 97.78% | 97.76% |

Table 6: Beam prediction accuracy from 8 measured beams with 0 dB margin

| Predicted beam | Option 1 (L1-RSRP) | Option 2 (L1-RSRP) | Option 3 (CIR from 2 strongest taps) | Option 3 (CIR from 4 strongest taps) | Option 3 (CIR from 6 strongest taps) |
| --- | --- | --- | --- | --- | --- |
| Top-1 | 53.15% | 54.35% | 57.70% | 58.75% | 58.74% |
| Top-2 | 70.09% | 71.24% | 75.64% | 77.55% | 77.47% |
| Top-4 | 83.54% | 84.48% | 88.19% | 89.13% | 89.30% |
| Top-8 | 93.39% | 93.69% | 95.40% | 95.74% | 95.78% |

Observation 2: CIR based spatial domain beam prediction outperforms the L1-RSRP based beam prediction.

Observation 3: UE orientation could not help to increase the accuracy for L1-RSRP based beam prediction.

### 2.1.2 Measurement accuracy impact

For spatial domain beam prediction, the input could be the beam quality for some beams. However, there could be some measurement error. The measurement error could be big for some beams with lower coverage. Table 7 illustrates some results for the beam prediction accuracy based on perfect L1-RSRP as input and L1-RSRP with measurement error as input. The simulation assumption is the same as section 2.2.1. It can be observed that the measurement error could cause significant performance degradation.

Table 7: Beam prediction accuracy from 4 measured beams with 1 dB margin

| Predicted beam | Perfect L1-RSRP | L1-RSRP with up to 5dB measurement error | L1-RSRP with up to 10 dB measurement error |
| --- | --- | --- | --- |
| Top-1 | 47.98% | 30.28% | 19.82% |
| Top-2 | 65.49% | 46.92% | 32.12% |
| Top-4 | 82.09% | 65.58% | 49.01% |
| Top-8 | 93.62% | 84.82% | 70.76% |

Table 8: Beam prediction accuracy from 4 measured beams with 0 dB margin

| Predicted beam | Perfect L1-RSRP | L1-RSRP with up to 5dB measurement error | L1-RSRP with up to 10 dB measurement error |
| --- | --- | --- | --- |
| Top-1 | 33.66% | 20.04% | 12.35% |
| Top-2 | 51.77% | 33.63% | 21.41% |
| Top-4 | 70.90% | 51.63% | 36.17% |
| Top-8 | 88.92% | 74.68% | 58.35% |

Observation 4: The ML input with measurement error could cause significant performance degradation.

### 2.1.2 Beam pattern mismatch impact

For spatial domain beam prediction, it is possible that the beam pattern in the ML is different from actual beam pattern in the gNB side. Table 9 and Table 10 illustrates the case for beam prediction with and without beam pattern mismatch, where the beam pattern in the NW side is based on 8 beams in horizontal with the direction of (-52.5, -37.5, -22.5, -7.5, 7.5, 22.5, 37.5, 52.5)*7/6 degree and 4 beams in vertical with the direction of (107.5, 122.5, 137.5, 152.5)*7/6 degree. The simulation assumption is the same as section 2.2.1. It can be observed that small beam pattern mismatch could cause significant beam prediction accuracy degradation.

Table 9: Beam prediction accuracy from 4 measured beams with 1 dB margin

| Predicted beam | Beam prediction without beam pattern mismatch | Beam prediction with beam pattern mismatch |
| --- | --- | --- |
| Top-1 | 47.98% | 11.53% |
| Top-2 | 65.49% | 20.32% |
| Top-4 | 82.09% | 40.64% |
| Top-8 | 93.62% | 63.38% |

Table 10: Beam prediction accuracy from 4 measured beams with 0 dB margin

| Predicted beam | Beam prediction without beam pattern mismatch | Beam prediction with beam pattern mismatch |
| --- | --- | --- |
| Top-1 | 33.66% | 7.84% |
| Top-2 | 51.77% | 14.79% |
| Top-4 | 70.90% | 33.96% |
| Top-8 | 88.92% | 57.70% |

Observation 5: Beam pattern mismatch could cause significant performance degradation.

## Beam prediction output

In RAN1 #110, the following is agreed on the output of the spatial-domain beam prediction.

Agreement Regarding the sub use case BM-Case1 and BM-Case2, study the following alternatives for AI/ML output: - Alt.1: Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams - E.g., N predicted beams can be the top-N predicted beams - Alt.2: Tx and/or Rx Beam ID(s) of the N predicted DL Tx and/or Rx beams and other information - FFS: other information (e.g., probability for the beam to be the best beam, the associated confidence, beam application time/dwelling time, Predicted Beam failure) - E.g., N predicted beams can be the top-N predicted beams - Alt.3: Tx and/or Rx Beam angle(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams - E.g., N predicted beams can be the top-N predicted beams - FFS: details of Beam angle(s) - FFS: how to select the N DL Tx and/or Rx beams (e.g., L1-RSRP higher than a threshold, a sum probability of being the best beams higher than a threshold, RSRP corresponding to the expected Tx and/or Rx beam direction(s)) - Note1: It is up to companies to provide other alternative(s) - Note2: Beam ID is only used for discussion purpose - Note3: All the outputs are “nominal” and only for discussion purpose - Note4: Values of N is up to each company. - Note5: All of the outputs in the above alternatives may vary based on whether the AI/ML model inference is at UE side or gNB side. - Note 6: The Top-N beam IDs might have been derived via post-processing of the ML-model output

For the output of beam prediction, in general, there could be the following two options:

- Option 1 (codebook-based beamforming): Predict the beam from a beam codebook

- Option 2 (Channel-based beamforming): Predict the channel eigen vector, which is used as the beamforming weight

Figure 4 illustrates the simulation results for the RSRP distribution for the options above. It can be observed that the channel-based beamforming can provide at least 5 dB performance gain compared to the codebook-based beamforming. The simulation assumption is provided in Table A-1. Therefore, it is necessary to study the channel eigenvector (Alt3) as the output for beam prediction.

Figure 4: Simulation results for RSRP distribution for channel-based beamforming and codebook based beamforming

Observation 6: Channel-based beamforming could provide at least 5 dB RSRP gain compared to codebook-based beamforming.

## Beam prediction for MU-MIMO

Based on current study, the beam prediction is to predict the “strong” beam. However, for MU-MIMO operation, the “weak” beam information is also important for the network to determine UE pairing. Without the “weak” beam information, the network cannot identify the potential mutual interference for the co-scheduled UEs, as shown in Figure 5.

Figure 5: A potential issue for beam selection for MU-MIMO

The MU-MIMO operation is an important feature to improve the spectrum efficiency. Figure 6 illustrates a simulation result to compare the performance between MU-MIMO and SU-MIMO based on full buffer traffic, where for MU-MIMO, two cases are studied – UE pairing without “weak” beam information and UE pairing with “weak” beam information. It can be observed that MU-MIMO with UE pairing without “weak” beam information could not provide performance gain, but MU-MIMO with UE pairing based on “weak” beam information can provide significant performance gain. Therefore, it should be studied to predict the weak beam information in addition to the strong beam.

Figure 6: Average cell SE for MU-MIMO and SU-MIMO

Figure 7: Cell edge SE for MU-MIMO and SU-MIMO

Observation 7: MU-MIMO with weak beam information can provide significant performance gain compared to SU-MIMO, and MU-MIMO without weak beam information cannot provide performance gain.

Proposal 2: For spatial-domain beam prediction, study to predict the “weak” beam to facilitate the MU-MIMO UE pairing.

## UE-group based beam prediction

In the actual network, some UEs may locate closely and share the same velocity, e.g., the UEs are in a car, as shown in Figure 8. For such UEs, it is possible to perform UE-group based beam prediction. Thus, the input could be based on the beam report from one or a subset of UEs in a UE group, and the output could be the predicted beams for the group of UEs.

Figure 8: A potential scenario for UE-group based beam prediction

Proposal 3: Study the UE-group based beam prediction for the UEs with the similar location and trajectory.

## Spatial domain L1-RSRP prediction

Similar to spatial domain beam index prediction, it is possible to use ML to predict the L1-RSRP for the predicted beam index. The input for the ML could be the L1-RSRP for a subset of network beams, similar to section 2.1. We compared two schemes for L1-RSRP prediction:

- ML-based L1-RSRP prediction: The L1-RSRPs from a subset of network beams are used as the input, and the L1-RSRPs of all the network beams are the output of the ML. The input and output are normalized.

- Non-ML based L1-RSRP prediction: The highest L1-RSRP from the subset of network beams are used as the predicted L1-RSRP.

We calculated the error between the predicted L1-RSRP and actual L1-RSRP for the top-N predicted beams. Table 1 illustrates the average error for the L1-RSRP prediction schemes. Figure 9 illustrates the CDF of the L1-RSRP prediction error for each beam for both ML-based and non-ML based scheme. In the evaluation, the input is the L1-RSRPs form 8 network beams as Figure 3. It can be observed that ML-based L1-RSRP cannot provide performance gain compared to non-ML based scheme.

Table 1: Average error for L1-RSRP prediction

|  | Average error for ML based L1-RSRP prediction [dB] | Average error for non-ML based L1-RSRP prediction [dB] |
| --- | --- | --- |
| Top-1 beam | 4.4642 | 1.8889 |
| Top-2 beam | 4.3309 | 1.7120 |
| Top-4 beam | 4.5043 | 1.9163 |
| Top-8 beam | 4.5553 | 3.3471 |

Figure 9: CDF of L1-RSRP prediction error

Observation 8: ML based L1-RSRP prediction cannot provide performance gain.

# Conclusion

In this contribution, we provided discussion on evaluation of AI/ML based BM. Based on the discussion, the following proposals have been achieved.

Proposal 1: Clarify that the KPI “RS overhead reduction” is only from the beam measurement perspective for a UE, which does not mean the overall RS overhead reduction from the network perspective.

Proposal 2: For spatial-domain beam prediction, study to predict the “weak” beam to facilitate the MU-MIMO UE pairing.

Proposal 3: Study the UE-group based beam prediction for the UEs with the similar location and trajectory.

Observation 1: Regardless of whether ML is enabled or not, the NW still needs to transmit the DL RS for all the beam sin set A for further beam tracking, pathloss measurement, L3 measurement and so on.

Observation 2: CIR based spatial domain beam prediction outperforms the L1-RSRP based beam prediction.

Observation 3: UE orientation could not help to increase the accuracy for L1-RSRP based beam prediction.

Observation 4: The ML input with measurement error could cause significant performance degradation.

Observation 5: Beam pattern mismatch could cause significant performance degradation.

Observation 6: Channel-based beamforming could provide at least 5 dB RSRP gain compared to codebook-based beamforming.

Observation 7: MU-MIMO with weak beam information can provide significant performance gain compared to SU-MIMO, and MU-MIMO without weak beam information cannot provide performance gain.

Observation 8: ML based L1-RSRP prediction cannot provide performance gain.

# Appendix – Simulation Assumption

Table A-1: Simulation Assumption

| Parameter | Value |
| --- | --- |
| Scenario | Dense Urban Macro |
| Number of UEs | 100000 (80% for training and 20% for testing), 210 for throughput related simulation |
| UE dropping | Outdoor |
| gNB antenna structure | (M, N, P, Mp, Np, Mg, Ng) = (4, 8, 2, 4, 8, 1, 1) |
| UE antenna structure | (M, N, P, Mp, Np, Mg, Ng) = (1, 4, 2, 1, 4, 1, 2) |
| Carrier frequency | 30 GHz |
| SCS | 120 kHz |
| Bandwidth | 80 MHz |
| gNB beam structure for codebook-based beamforming | 8 beams in horizontal: (-52.5, -37.5, -22.5, -7.5, 7.5, 22.5, 37.5, 52.5) degree 4 beams in vertical: (107.5, 122.5, 137.5, 152.5) degree |
| UE beam structure for codebook-based beamforming | 4 beams in horizontal: (-33.75, -11.25, 11.25, 33.75) degree |
| gNB Tx power | 40 dBm |
| Minimal gNB-UE distance | 10 m |
| gNB height | 25 m |
| Scheduler | PF |
| Traffic model | Full buffer |