3GPP TSG RAN WG1 #113                                       	       R1-2304895

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.2.3.1

Source:	Xiaomi

Title:                     Evaluation on AI/ML for beam management

Document for:	Discussion and Decision

Introduction

In RAN-94 e-meeting, the SI of Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1] was approved. The objective can be seen as below:

Use cases to focus on:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project.

AI/ML model, terminology and description to identify common and specific characteristics for framework investigations:

- Characterize the defining stages of AI/ML related algorithms and associated complexity:

  - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable

  - Inference operation, e.g., input/output, pre-/post-process, as applicable

- Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g.,

  - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes]

  - Various levels of UE/gNB collaboration targeting at separate or joint ML operation.

- Characterize lifecycle management of AI/ML model: e.g.,  model training, model deployment , model inference, model monitoring, model updating

- Dataset(s) for training, validation, testing, and inference

- Identify common notation and terminology for AI/ML related functions, procedures and interfaces

- Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate

For the use cases under consideration:

- Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set:

  - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations.

    - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed.

    - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study.

    - Need for common assumptions in dataset construction for training, validation and test for the selected use cases.

    - Consider adequate model training strategy, collaboration levels and associated implications

    - Consider agreed-upon base AI model(s) for calibration

    - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes

  - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases.

    - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline

    - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered.

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

  - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference),  and management of data and AI/ML model, per RAN1 input

    - Collaboration level specific specification impact per use case

  - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2

    - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable

    - Consider the need and implications for AI/ML processing capabilities definition

Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved.

Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

This contribution provides our evaluation assumptions and results of AI/ML for beam management.

Motivation

According to 38.802 [2], the following DL L1/L2 beam management procedures are supported within one or multiple TRPs:

-	P-1: is used to enable UE measurement on different TRP Tx beams to support selection of TRP Tx beams/UE Rx beam(s)

-	For beamforming at TRP, it typically includes a intra/inter-TRP Tx beam sweep from a set of different beams. For beamforming at UE, it typically includes a UE Rx beam sweep from a set of different beams.

-	P-2: is used to enable UE measurement on different TRP Tx beams to possibly change inter/intra-TRP Tx beam(s)

-	From a possibly smaller set of beams for beam refinement than in P-1. Note that P-2 can be a special case of P-1.

-	P-3: is used to enable UE measurement on the same TRP Tx beam to change UE Rx beam in the case UE uses beamforming

During P-1, there will be a large number of TRP Tx beam sweeping and a large number of UE Rx beam sweeping. And the number of TxRx beam pair will be M*N, where M is the number of TRP Tx beam and N is the number of UE Rx beam. It means UE need to measure M*N beam pairs at most to find the best TRP Tx beam and the best UE Rx beam. It will introduce high reference signaling overhead, high UE measurement complexity and long measurement latency. In order to solve this problem, a new SI on AI/ML for NR Air Interface was approved. And in this contribution, we will discuss the AI/ML for beam management.

Spatial domain beam prediction

Use case

In the SID, the initial set of use cases for beam management includes beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement.

For beam prediction in spatial domain, the principle can be seen in Figure 3-1, there are total M*N beam pairs. Based on an AI model, only the measured L1-RSRP of a few beam pairs will be input into the AI model, and the AI model will output the L1-RSRP of the total M*N beam pairs or output the best Tx beams and/or corresponding L1-RSRP only.

Figure 3-1, Principle of AI based beam prediction in spatial domain

AI/ML model

As known that the purpose of beam measurement is to obtain the Tx beam ID and the L1-RSRP of the best N Tx beams. Thus the output of the AI model should include the Tx beam ID and the L1-RSRP of the best N Tx beams. In order to realize it, we use the AI model based on Fully Connected Neural Network with 5 layer in Figure 3-2 and the parameters can be seen in Table 3-1-a and Table 3-1-b. While for Table 3-1-a, the model was applied for beam pair prediction. The L1-RSRP or L1-RSRP+beam ID of set B will be as the input. And 256 is the number of total beam pairs with 32 TRP Tx beam and 8 UE Rx beam with 2 panels, which can be seen as Set A. The L1-RSRP of 256 beam pairs will be the output with the ascending order of beam ID.  While for Table 3-1-b, the model was applied for DL Tx beam prediction. The L1-RSRP or L1-RSRP+beam ID of set B will be considered as the input. And 32 is the number of total DL Tx beam, which can be seen as Set A. The L1-RSRP of 32 DL Tx beam with one Rx beam will be the output with the ascending order of beam ID.

As for the data set, we use about 50000 samples for model training (95%) and testing. And the data are obtained from system level simulation in urban macro. The number of input beam pairs’ RSRP is 64, the number of output beam pairs’ RSRP is 256. The model is trained based on the MSE as loss function.

Figure 3-2, AI network

Table 3-1, Parameters of AI model

Table 3-1-a, For beam pair prediction

| Layer | Output shape | Parameters |
| --- | --- | --- |
| Input layer | (B, 64) / (B, 192) | 0 |
| Dense_1(hidden unit=512) | (B, 512) | 33280 / 98816 |
| Activation(ReLU) | (B, 512) |  |
| Dense_2(hidden unit=512) | (B, 512) | 262656 |
| Activation(ReLU) | (B, 512) |  |
| Dense_3(hidden unit=256) | (B, 256) | 131328 |
| Activation(ReLU) | (B, 256) |  |
| Dense_4(hidden unit=256) | (B, 256) | 65792 |
| Activation(ReLU) | (B, 256) |  |
| Dense_5(hidden unit=256) | (B, 256) | 65792 |
| Model complexity |  |  |
| Total parameters | Model size | FLOPs |
| 558848/624384 | 6.44 MB /7.12MB | 1.116M/1.250M |

Table 3-1-b, For DL Tx beam prediction

| Layer | Output shape | Parameters |
| --- | --- | --- |
| Input layer | (B, 8) | 0 |
| Dense_1(hidden unit=64) | (B, 64) | 576 |
| Activation(ReLU) | (B, 64) |  |
| Dense_2(hidden unit=64) | (B, 64) | 4160 |
| Activation(ReLU) | (B, 64) |  |
| Dense_3(hidden unit=32) | (B, 32) | 2080 |
| Activation(ReLU) | (B, 32) |  |
| Dense_4(hidden unit=32) | (B, 32) | 1056 |
| Activation(ReLU) | (B, 32) |  |
| Dense_5(hidden unit=32) | (B, 32) | 1056 |
| Model complexity |  |  |
| Total parameters | Model size | FLOPs |
| 8928 | 0.58 MB | 0.0176M |

Evaluation assumption and parameters

According to the agreement archived in RAN1-109 e-meeting [3] and RAN1-110 meeting [4], the following options for baseline performance can be considered for spatial domain beam prediction accuracy.

Agreement

- For spatial-domain beam prediction, further study the following options as baseline performance

  - Option 1: Select the best beam within Set A of beams based on the measurement of all RS resources or all possible beams of beam Set A (exhaustive beam sweeping)

    - FFS CSI-RS/SSB as the RS resources

  - Option 2: Select the best beam within Set A of beams based on the measurement of RS resources from Set B of beams

    - FFS: Set B is a subset of Set A and/or Set A consists of narrow beams and Set B consists of wide beams

    - FFS: how conventional scheme to obtain performance KPIs

    - FFS: how to determine the subset of RS resources is reported by companies

  - Other options are not precluded.

Agreement

- Study the following options on the selection of Set B of beams (pairs)

  - Option 1: Set B is fixed across training and inference

    - FFS on the beams of Set B

  - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each report/measurement during training and/or inference)

    - FFS on fixed or variable number of beams (pairs)

    - FFS on the details

  - Other options are not precluded.

  - FFS on the number of beams (pairs) in Set B

  - Note: This does not preclude the alternative that Set B is different from Set A.

Agreement

For the sub use case BM-Case1, support the following alternatives for further study:

- Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A)

- Alt.2: Set B is a subset of Set A

- Note1: Set A is for DL beam prediction and Set B is for DL beam measurement.

- Note2: The beam patterns of Set A and Set B can be clarified by the companies.

The following agreement were archived in RAN1-110b e-meeting [5]:

Agreement

- BS antenna configuration:

  - antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ

  - Other assumptions are not precluded

- BS Tx power for evaluation:

  - 40dBm (baseline)

  - Other values (e.g. 34 dBm) are not precluded and can be reported by companies

- UE antenna configuration (Clarification of agreement in RAN 1 #110):

  - antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right)

  - Other assumptions are not precluded

Agreement

- For the evaluation of both BM-Case1 and BM-Case2, 32 or 64 downlink Tx beams (maximum number of available beams) at NW side.

  - Other values, e.g., 256, etc, are not precluded and can be reported by companies.

- For the evaluation of both BM-Case1 and BM-Case2, 4 or 8 downlink Rx beams (maximum number of available beams) per UE panel at UE side.

  - Other values, e.g., 16, etc, are not precluded and can be reported by companies.

Agreement

- Study the following options on the selection of Set B of beams (pairs)

  - Option 1: Set B is fixed across training and inference

  - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), FFS:

    - Opt A: Set B is changed following a set of pre-configured patterns

    - Opt B: Set B is randomly changed among pre-configured patterns

    - Opt C: Set B is randomly changed among Set A beams (pairs)

    - The number of beams(pairs) in Set B can be fixed or variable

    - Note: BM-Case1 and BM-Case2 may be considered for different option.

  - Other options are not precluded.

Here we take Option 2 as the baseline and consider Alt.1 that set B is a subset of set A. And the evaluation assumptions and parameters were provided in Figure 3-3 and Table 3-2. As for the pattern of set B of beam pair prediction, we consider both variable set B and fixed set B based on the mapping rule between beam pair ID and Tx beam ID& Rx beam ID in Figure 3-4.

Figure 3-3, Network deployment

Figure 3-4, Mapping rule between beam pair ID and Tx beam ID& Rx beam ID

Table 3-2, Evaluation parameters for spatial domain beam prediction

| Parameters |  | Value |
| --- | --- | --- |
| Scenarios (carrier frequency) |  | Urban Macro, 30 GHz |
| Number of cells |  | 7 cell with 3 sectors per cell |
| UE number/ per sector |  | 40 |
| UE distribution |  | Option 1: 80% indoor ,20% outdoor as in TR 38.901 |
| Bandwidth |  | 80 MHz |
| Mode |  | SU-MIMO |
| BS antenna configurations |  | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1) |
| BS Tx beam pattern |  |  |
| BS Tx beam number |  | 32 |
| UE antenna configurations |  |  |
| UE Rx beam pattern per panel |  |  |
| UE Rx beam number |  | 8 with 4 per panel |
| UE rotation speed |  | 0 |
| Beam measurement quality |  | L1-RSRP |
| baseline performance |  | Option #2 |
| Set A and Set B | For beam pair prediction | Alt.1: Set B (64 beam pairs) is a subset of Set A (256 beam pairs) |
|  | For DL Tx beam prediction | Alt.1: Set B (8 DL Tx beam) is a subset of Set A (32 DL Tx beam) |

KPI

The following agreements and working assumption on KPI were archived in RAN1-110b e-meeting.

Agreement

- The options to evaluate beam prediction accuracy (%):

  - Top-1 (%): the percentage of “the Top-1 genie-aided beam is Top-1 predicted beam”

  - Top-K/1 (%): the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams”

  - Top-1/K (%) (Optional): the percentage of “the Top-1 predicted beam is one of the Top-K genie-aided beams”

  - Where K >1 and values can be reported by companies.

Working assumption

- For the evaluation of the overhead for BM-Case1, further study the following two metrics for potential down selection:

  - Option A: RS overhead reduction, FFS for potential down selection:

    - Option 1: RS OH reduction%=1-NM

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement

      - where M is the total number of beams (pairs) to be predicted

    - Option 2: RS OH reduction%=1-NM

      - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML

      - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme

      - Companies report the assumption on beam sweeping

    - Option 3: RS OH reduction%=1-N+PM

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement

      - where M is the total number of beams (pairs) to be predicted

      - FFS the following alternatives consider different targets (e.g., beam or beam pair) for prediction:

        - Alt1: P is the number of Top-K selected beams (pairs) for beam sweeping (if applicable)

        - Alt2: P is the number of Top-K selected beams (pairs) not in Set B for beam sweeping (if applicable)

        - Alt3: P is the number of beams used for beam sweeping to get the best Rx beam (if applicable)

        - Companies report the assumption on beam sweeping

    - Other options can be reported by companies

  - Option B: RS overhead, FFS for potential down selection:

    - Option 1: RS OH = N,

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement

    - Option 2: RS OH = N + P

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement

      - FFS the following alternatives consider different targets (e.g., beam or beam pair) for prediction:

        - Alt1: P is the number of Top-K selected beams (pairs) for beam sweeping (if applicable)

        - Alt2: P is the number of Top-K selected beams (pairs) not in Set B for beam sweeping (if applicable)

        - Alt3: P is the number of beams used for beam sweeping to get the best Rx beam (if applicable)

        - Companies report the assumption on beam sweeping

    - Other options can be reported by companies

The following agreements on KPI were archived for BM-Case 1 in RAN1-111[6] meeting.

Agreement

- For the evaluation of the overhead for BM-Case1, adoption the following metrics:

  - RS overhead reduction,

    - Option 1: RS OH reduction%=1-NM

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML

      - where M is the total number of beams (pairs) to be predicted

    - Option 2: RS OH reduction%=1-NM

      - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable

      - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme, including the beams (pairs) required for additional measurements before/after the prediction if applicable

      - Companies report the assumption on additional measurements

The following agreements on KPI were archived in RAN1-112[7] meeting.

Agreement

- For AI/ML models, which provide L1-RSRP as the model output, to evaluate the accuracy of predicted L1-RSRP, companies optionally report average (absolute value)/CDF of the predicted L1-RSRP difference, where the predicted L1-RSRP difference is defined as:

  - The difference between the predicted L1-RSRP of Top-1[/K] predicted beam and the ideal L1-RSRP of the same beam.

Agreement

- For DL Tx beam prediction, the definition of Top-1 genie-aided Tx beam is defined as

  - Option A (baseline): the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams

  - Option B(optional), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s)

    - FFS on specific Rx beam(s)

    - Note: specific Rx beams are subset of all Rx beams

- For DL Tx-Rx beam pair prediction, the definition of Top-1 genie-aided Tx-Rx beam pair is defined as

  - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams

  - Other options are not precluded and can be reported by companies.

- Note: This is only for evaluation discussion

The following agreements on KPI were archived in RAN1-112b [8] e-meeting.

Agreement

- To evaluate the performance of AI/ML in beam management at least for NW side beam prediction, UCI report overhead (e.g., number of UCI reports and UCI payload size) and/or UCI overhead reduction for inference of AI/ML model can be reported by company.

  - UCI overhead reduction = 1- Total UCI payload size for AI/ML/Total UCI payload size of baseline.

  - Companies to report detailed assumption of UCI for AI/ML and baseline, e.g., including quantization mechanism

While for predicated L1-RSRP difference, we prefer to assume the best Rx beam for the idea L1-RSRP of Top-1 predication beam, which can be same as the assumption of the L1-RSRP difference.

Proposal 1: Support the best Rx beam for the ideal L1-RSRP of Top-1 predicted beam.

Evaluation results

Beam pair prediction

Random set B vs. fixed set B

In order to align with the beam measurement report, we consider three K values for predicted Top-K beams with K=1, 2 or 4.

Based on Figure 3-4, we evaluate 4 schemes listed below:

- Scheme 0: Option 2 as baseline

  - Select 64 beam pairs randomly and choose the Tx beam with the highest L1-RSRP as the best Tx beam.

- Scheme 1: Variable set B with L1-RSRP as input;

  - Select 64 beam pairs randomly and input their L1-RSRP to AI model for beam prediction.

- Scheme 2: Variable set B with L1-RSRP and beam ID as input;

  - Select 64 beam pairs randomly and input their L1-RSRP and beam ID to AI model for beam prediction.

- Scheme 3: Fixed set B with L1-RSRP as input;

  - Select 64 beam pairs with same beam pair IDs and input their L1-RSRP to AI model for beam prediction. And the beam pair IDs are beam pair ID#0, 4, 8, 12, 16……, 252.

The evaluation results can be seen in Table 3-3, 3-4, 3-5 and Figure 3-5. Table 3-3 provides the average L1-RSRP difference of Top-1 predicted beam of 4 schemes. Table 3-4 provides Beam prediction accuracy (%) for Top-1 and/or Top-K beams (K=2, 4) of 4 schemes.  Table 3-5 provides beam prediction accuracy (%) with 1dB margin for Top-1 beam of 4 schemes.  Figure 3-5 provides the CDF of L1-RSRP difference for Top-1 predicted beam of 4 schemes. From the evaluation results, we can see that AI based beam prediction scheme 1-3 provide large performance gain compared to non-AI based beam management scheme 0. In addition, we can also find that scheme 2 and scheme 3 can further improve the performance by input beam ID or by input L1-RSRP of same beam pairs. Thus we propose an observation and proposal as below.

Table 3-3, Average ideal L1-RSRP difference between Top-1 predicted beam and Top-1 genie-aided beam

| Scheme | Average L1-RSRP difference (dB) |
| --- | --- |
| Scheme 0 | 3.7 |
| Scheme 1 | 2.5 |
| Scheme 2 | 1.3 |
| Scheme 3 | 0.2 |

Table 3-4, Beam prediction accuracy (%) for Top-1 and/or Top-K beams (K=2, 4)

| Scheme | Beam prediction accuracy (%) |  |  |
| --- | --- | --- | --- |
|  | K=1 | K=2 | K=4 |
| Scheme 0 | 20.3 |  |  |
| Scheme 1 | 44.6 | 61.2 | 79.5 |
| Scheme 2 | 54.7 | 71.6 | 86.8 |
| Scheme 3 | 74.3 | 90.5 | 97.9 |

Figure 3-5, CDF of L1-RSRP difference for Top-1 predicted beam

Table 3-5, Beam prediction accuracy (%) with 1dB margin for Top-1 beam

| Scheme | Beam prediction accuracy (%) |
| --- | --- |
| Scheme 0 | 24.9 |
| Scheme 1 | 57.6 |
| Scheme 2 | 69.1 |
| Scheme 3 | 91.4 |

Observation 1: AI based beam pair prediction in spatial domain can provide good performance by random set B with corresponding beam pair ID (about 55% beam prediction accuracy of Top-1 beam) as input or by fixed set B(about 74% beam prediction accuracy of Top-1 beam) when the ratio of set B and set A is 1/4.

We also evaluate the L1-RSRP difference between the ideal L1-RSRP of Top-1 predicted beam and the predicted L1-RSRP of the Top 1 predicted beam and the evaluation result can be seen in Table 3-6. We can see that the average L1-RSRP difference is less than 0.5dB with scheme 3.

Table 3-6, Average predicted L1-RSRP difference of Top 1 predicted beam

| Scheme | Average predicted L1-RSRP difference (dB) |
| --- | --- |
| Scheme 1 | 0.76 |
| Scheme 2 | 0.63 |
| Scheme 3 | 0.17 |

Observation 2: For AI based beam pair prediction in spatial domain with fixed set B, the average predicted L1-RSRP difference between the ideal L1-RSRP of Top-1 predicted beam pair and the predicted L1-RSRP of the Top 1 predicted beam pair is less than 0.5 dB when the ratio of set B and set A is 1/4.

Different fixed set B and different set B in pre-configured set

The following agreements on set B were archived in RAN1-111 meeting [6]

Agreement

- Companies report the pattern of Set B.

- Further study the performance with different patterns of set B(s) for fixed Set B (Option 1) and different pre-configured/pre-known patterns of Set B(s) (Option 2A and 2B).

Set B with continuous beam pair ID

Based on Figure 3-4, we consider following 4 different fixed set B with 64 beam pairs with continuous beam pair IDs. In addition, Mixed set B with different set B in pre-configured set of set B are evaluated as pattern 5 and the pre-configured set of set B consists of the following 4 different fixed set B.

- Pattern 1 of fixed Set B with continuous beam pair ID: beam pair ID #0~63.

- Pattern 2 of fixed Set B with continuous beam pair ID: beam pair ID #64~127.

- Pattern 3 of fixed Set B with continuous beam pair ID: beam pair ID #128~191.

- Pattern 4 of fixed Set B with continuous beam pair ID: beam pair ID #192~255.

- Pattern 5: different Set B in pre-configured set of Set B with continuous beam pair ID.

  - mixed data samples of above Pattern#1, #2, #3 and #4 with 25% of each for training and inference。

- Pattern 6: training by dataset of Pattern 1 and inference by dataset of Pattern 2.

- Pattern 7: enhanced Pattern 5 with beam pair ID as input

The evaluation results can be seen in Table 3-7, 3-8 3-9and 3-10. From the evaluation results, we can see that the performance for pattern 1~4 are almost same as each other with different fixed set B with continuous beam pair ID. But pattern 5 with mixed fixed set B has some performance loss because the generalization capability among different fixed set B is not good enough. And pattern 7 with enhancement of pattern 5 by adding beam pair ID as input can improve the prediction accuracy.  Pattern 6 with training by dataset of pattern 1 and inference by dataset of pattern 2 provides the worst prediction accuracy.

Table 3-7, Average ideal L1-RSRP difference between Top-1 predicted beam and Top-1 genie-aided beam

| Pattern of set B | Average L1-RSRP difference (dB) |
| --- | --- |
| Pattern 1 | 0.86 |
| Pattern 2 | 0.87 |
| Pattern 3 | 0.79 |
| Pattern 4 | 0.79 |
| Pattern 5 | 2.20 |
| Pattern 6 | 3.11 |
| Pattern 7 | 1.82 |

Table 3-8, Average predicted L1-RSRP difference of Top 1 predicted beam

| Pattern of set B | Average predicted L1-RSRP difference (dB) |
| --- | --- |
| Pattern 1 | 0.30 |
| Pattern 2 | 0.30 |
| Pattern 3 | 0.29 |
| Pattern 4 | 0.29 |
| Pattern 5 | 1.68 |
| Pattern 6 | 2.12 |
| Pattern 7 | 0.63 |

Table 3-9, Beam prediction accuracy (%) for Top-1 and/or Top-K beams (K=2, 4)

| Pattern of set B | Beam prediction accuracy (%) |  |  |
| --- | --- | --- | --- |
|  | K=1 | K=2 | K=4 |
| Pattern 1 | 59 | 77 | 91 |
| Pattern 2 | 61 | 79 | 91 |
| Pattern 3 | 64 | 80 | 92 |
| Pattern 4 | 63 | 80 | 91 |
| Pattern 5 | 46 | 63 | 79 |
| Pattern 6 | 40 | 54 | 68 |
| Pattern 7 | 52 | 70 | 84 |

Table 3-10, Beam prediction accuracy (%) with 1dB margin for Top-1 beam

| Pattern of set B | Beam prediction accuracy (%) |
| --- | --- |
| Pattern 1 | 77 |
| Pattern 2 | 77 |
| Pattern 3 | 80 |
| Pattern 4 | 81 |
| Pattern 5 | 61 |
| Pattern 6 | 51 |
| Pattern 7 | 68 |

Observation 3: For AI based beam pair prediction in spatial domain with fixed set B, different fixed set B with continuous beam pair ID can provide almost same performance (about 59~64% beam prediction accuracy of Top-1 beam), but different set B in pre-configured set even with beam pair ID as input results in some performance loss (about 7~12% loss on beam prediction accuracy of Top-1 beam) when the ratio of set B and set A is 1/4.

Set B with non-continuous beam pair ID

Based on Figure 3-4, we consider following 4 different fixed set B with 64 beam pairs with non-continuous beam pair IDs. In addition, Mixed set B with different set B in pre-configured set of set B are evaluated as pattern 10 and 11, and the pre-configured set of set B consists of the following 4 or 3 different fixed set B.

- Pattern 1 of fixed Set B with non-continuous beam pair ID: beam pair ID #0, 4, 8, 12, 16…, 252.

- Pattern 2 of fixed Set B with non-continuous beam pair ID: beam pair ID #1, 5, 9, 13, 17…, 253.

- Pattern 3 of fixed Set B with non-continuous beam pair ID: beam pair ID #2, 6, 10, 14, 18…, 254.

- Pattern 4 of fixed Set B with non-continuous beam pair ID: beam pair ID #3, 7, 11, 15, 19…, 255.

- Pattern 5: different Set B in pre-configured set of Set B with non-continuous beam pair ID.

  - mixed data samples of above Pattern#6, #7, #8 and #9 with 25% of each for training and inference.

- Pattern 6: different Set B in pre-configured set of Set B with non-continuous beam pair ID.

  - mixed data samples of above Pattern#6, #7 and #8 with 33% of each for training and inference.

- Pattern 7：training by dataset of Pattern 1 and inference by dataset of Pattern 2.

- Pattern 8-1: enhanced Pattern 5 with beam pair ID as input

- Pattern 8-2: enhanced Pattern 5 with pattern ID as input

- Pattern 9-1: enhanced Pattern 6 with beam pair ID as input

- Pattern 9-1: enhanced Pattern 6 with pattern ID as input

The evaluation results can be seen in Table 3-11, 3-12, 3-13 and 3-14. From the evaluation results, we can see that the performance for pattern 6~8 are almost same as each other with different fixed set B with non-continuous beam pair ID. While pattern 9 is a little worse. Thus pattern 11 with mixture of pattern 6~9 is a little better than pattern 10 with mixture of pattern 6~10. And the generalization capability among different fixed set B is very good.

Table 3-11, Average ideal L1-RSRP difference between Top-1 predicted beam and Top-1 genie-aided beam

| Pattern of set B | Average L1-RSRP difference (dB) |
| --- | --- |
| Pattern 1 | 0.23 |
| Pattern 2 | 0.24 |
| Pattern 3 | 0.24 |
| Pattern 4 | 0.52 |
| Pattern 5 | 0.64 |
| Pattern 6 | 0.36 |
| Pattern 7 | 0.58 |
| Pattern 8-1 | 0.32 |
| Pattern 8-2 | 0.40 |
| Pattern 9-1 | 0.19 |
| Pattern 9-2 | 0.24 |

Table 3-12, Average predicted L1-RSRP difference of Top 1 predicted beam

| Pattern of set B | Average predicted L1-RSRP difference (dB) |
| --- | --- |
| Pattern 1 | 0.17 |
| Pattern 2 | 0.18 |
| Pattern 3 | 0.17 |
| Pattern 4 | 0.35 |
| Pattern 5 | 0.58 |
| Pattern 6 | 0.29 |
| Pattern 7 | 0.51 |
| Pattern 8-1 | 0.23 |
| Pattern 8-2 | 0.26 |
| Pattern 9-1 | 0.22 |
| Pattern 9-2 | 0.20 |

Table 3-13, Beam prediction accuracy (%) for Top-1 and/or Top-K beams (K=2, 4)

| Pattern of set B | Beam prediction accuracy (%) |  |  |
| --- | --- | --- | --- |
|  | K=1 | K=2 | K=4 |
| Pattern 1 | 74 | 91 | 98 |
| Pattern 2 | 71 | 90 | 98 |
| Pattern 3 | 72 | 89 | 97 |
| Pattern 4 | 66 | 82 | 93 |
| Pattern 5 | 65 | 83 | 93 |
| Pattern 6 | 67 | 85 | 96 |
| Pattern 7 | 58 | 79 | 90 |
| Pattern 8-1 | 72 | 89 | 96 |
| Pattern 8-2 | 66 | 85 | 96 |
| Pattern 9-1 | 76 | 90 | 98 |
| Pattern 9-2 | 72 | 89 | 97 |

Table 3-14, Beam prediction accuracy (%) with 1dB margin for Top-1 beam

| Pattern of set B | Beam prediction accuracy (%) |
| --- | --- |
| Pattern 1 | 91 |
| Pattern 2 | 91 |
| Pattern 3 | 91 |
| Pattern 4 | 84 |
| Pattern 5 | 83 |
| Pattern 6 | 87 |
| Pattern 7 | 79 |
| Pattern 8-1 | 90 |
| Pattern 8-2 | 87 |
| Pattern 9-1 | 93 |
| Pattern 9-2 | 91 |

Observation 4: For AI based beam pair prediction in spatial domain with fixed set B, different fixed set B with non-continuous beam pair ID can provide almost same performance (about 66~74% beam prediction accuracy of Top-1 beam), and different set B in pre-configured set of set B with beam pair ID or pattern ID as input can archive almost same performance ((about 66~72% beam prediction accuracy of Top-1 beam)) as same fixed set B when the ratio of set B and set A is 1/4.

Observation 5: For AI based beam pair prediction in spatial domain with fixed set B, fixed set B with non-continuous bam pair ID (about 66~74% beam prediction accuracy of Top-1 beam) can provide better performance than that of fixed set B with continuous beam pair ID (about 59~64% beam prediction accuracy of Top-1 beam) since non-continuous beam pair ID covers more Rx beams when the ratio of set B and set A is 1/4.

Proposal 2: Different fixed set B consists of L1-RSRP with more Rx beams for beam pair prediction should be considered with high priority.

Generalization

Generalization on different scenario- Uma and Umi

In order to evaluate the performance of generalization on different scenario, we consider scheme 3 for each generalization case.

- Scheme 3: Fixed set B with L1-RSRP as input;

  - Select 64 beam pairs with same beam pair IDs and input their L1-RSRP to AI model for beam prediction. And the beam pair IDs are beam pair ID#0, 4, 8, 12, 16……, 252.

And the performance of following 6 cases are evaluated, wherein ISD 500m and BS antenna height 25m is assumed for Uma, and ISD 200m and BS antenna height 10m is assumed for Umi. While Case 5 and Case 6 is evaluated for comparison. As for case 2 and case 3, the training data contains 50% of Uma and 50% of Umi.

- Case 1: training by data of Uma, inference by data of Umi.

- Case 2: training by data of Uma and Umi, inference by data of Umi.

- Case 3: training by data of Uma and Umi, inference by data of Uma.

- Case 4: training by data of Umi, inference by data of Uma.

- Case 5: training by data of Uma, inference by data of Uma.

- Case 6: training by data of Umi, inference by data of Umi.

The evaluation results can be seen in Table 3-15, Table 3-16, Table 3-17, Table 3-18 and Figure 3-6. Based on the evaluation results, we can see that AI model of case 2 and case 3 provide good generalization capability among different scenario. While AI model of case 1 and case 4 provide a little worse generalization capability than that of case 2 and case 3.

Table 3-15, Average L1-RSRP difference of Top-1 predicted beam for generalization

| Generalization case | Average L1-RSRP difference (dB) |
| --- | --- |
| Case1 | 1.3 |
| Case2 | 0.2 |
| Case3 | 0.4 |
| Case4 | 0.8 |
| Case5 | 0.2 |
| Case6 | 0.2 |

Table 3-16, Beam prediction accuracy (%) for Top-1 and/or Top-K beams（K=1,2,4）for generalization

| Generalization case | Beam prediction accuracy (%) |  |  |
| --- | --- | --- | --- |
|  | K=1 | K=2 | K=4 |
| Case1 | 62.7 | 82.7 | 92.5 |
| Case2 | 78.1 | 95.7 | 99.6 |
| Case3 | 65.8 | 85.7 | 96.0 |
| Case4 | 56.8 | 76.7 | 89.5 |
| Case5 | 74.3 | 91.5 | 98.3 |
| Case6 | 78.5 | 95.2 | 99.7 |

Figure 3-6, CDF of L1-RSRP difference for Top-1 predicted beam for generalization

Table 3-17, Beam prediction accuracy (%) with 1dB margin for Top-1 beam for generalization

| Generalization case | Beam prediction accuracy (%) |
| --- | --- |
| Case1 | 75.2 |
| Case2 | 92.2 |
| Case3 | 85.2 |
| Case4 | 75.0 |
| Case5 | 93.5 |
| Case6 | 92.0 |

Table 3-18, Average predicted L1-RSRP difference of Top-1 predicted beam

| Scheme | Average predicted L1-RSRP difference (dB) |
| --- | --- |
| Case1 | 0.60 |
| Case2 | 0.21 |
| Case3 | 0.23 |
| Case4 | 0.43 |
| Case5 | 0.17 |
| Case6 | 0.19 |

Observation 6: For AI based beam pair prediction in spatial domain with fixed set B, AI model trained by hybrid data of Uma and Umi for beam pair prediction in spatial domain can provide good generalization capability for Uma (about 9% performance loss on beam prediction accuracy of Top-1 beam). While AI model trained by data of Umi provide a little worse generalization capability for Uma inference (about 18% performance loss on beam prediction accuracy of Top-1 beam) when the ratio of set B and set A is 1/4.

Generalization on different UE distribution- 100% outdoor and 20% outdoor

According to the agreement archived in RAN1-110 meeting [3], the following options on UE distribution can be considered for spatial domain beam prediction.

| Parameters | Values |
| --- | --- |
| UE distribution | - For spatial domain beam prediction: - Option 1: 80% indoor ,20% outdoor as in TR 38.901 - Option 2: 100% outdoor - For time domain prediction: 100% outdoor |

In order to evaluate the performance of generalization on different UE distribution, we consider Option 1 with 80% indoor and 20% outdoor, and Option 2 with 100% outdoor. As for the input of AI model, we consider scheme 3 for each generalization case with Uma.

- Scheme 3: Fixed set B with L1-RSRP as input;

  - Select 64 beam pairs with same beam pair IDs and input their L1-RSRP to AI model for beam prediction. And the beam pair IDs are beam pair ID#0, 4, 8, 12, 16……, 252.

And the performance of following 6 cases are evaluated. While case 5 and case 6 are evaluated for comparison. As for case 2 and case 3, the training data contains 50% of Option 1 and 50% of Option 2.

- Case 1: training by data of Option 1, inference by data of Option 2.

- Case 2: training by data of Option 1 and Option 2, inference by data of Option 2.

- Case 3: training by data of Option 1 and Option 2, inference by data of Option 1.

- Case 4: training by data of Option 2, inference by data of Option 1.

- Case 5: training by data of Option 1, inference by data of Option 1.

- Case 6: training by data of Option 2, inference by data of Option 2.

The evaluation results can be seen in Table 3-19, Table 3-20, Table 3-21, Table 3-22 and Figure 3-7. Based on the evaluation results, we can see that AI model of case 2 and case 3 provide good generalization capability among different UE distribution. While AI model of case 1 and case 4 provide a little worse generalization capability than that of case 2 and case 3.

Table 3-19 Average L1-RSRP difference of Top-1 predicted beam for generalization

| Generalization case | Average L1-RSRP difference (dB) |
| --- | --- |
| Case1 | 1.3 |
| Case2 | 0.3 |
| Case3 | 0.3 |
| Case4 | 0.8 |
| Case5 | 0.2 |
| Case6 | 0.3 |

Table 3-20 Beam prediction accuracy (%) for Top-1 and/or Top-K beams（K=1,2,4）for generalization

| Generalization case | Beam prediction accuracy (%) |  |  |
| --- | --- | --- | --- |
|  | K=1 | K=2 | K=4 |
| Case1 | 47.9 | 67.3 | 82.3 |
| Case2 | 66.8 | 83.9 | 94.9 |
| Case3 | 70.8 | 86.9 | 96.8 |
| Case4 | 55.7 | 77.9 | 91.0 |
| Case5 | 74.3 | 91.5 | 98.3 |
| Case6 | 68.1 | 84.6 | 95.4 |

Figure 3-7, CDF of L1-RSRP difference for Top-1 predicted beam for generalization

Table 3-21 Beam prediction accuracy (%) with 1dB margin for Top-1 beam for generalization

| Generalization case | Beam prediction accuracy (%) |
| --- | --- |
| Case1 | 66.1 |
| Case2 | 86.7 |
| Case3 | 88.9 |
| Case4 | 73.1 |
| Case5 | 93.5 |
| Case6 | 88.1 |

Table 3-22 Average predicted L1-RSRP difference of Top-1 predicted beam

| Scheme | Average predicted L1-RSRP difference (dB) |
| --- | --- |
| Case1 | 0.64 |
| Case2 | 0.20 |
| Case3 | 0.18 |
| Case4 | 0.35 |
| Case5 | 0.17 |
| Case6 | 0.17 |

Observation 7: For AI based beam pair prediction in spatial domain with fixed set B, AI model trained by hybrid data of different UE distribution for beam pair prediction in spatial domain can provide good generalization capability for UE distribution Option B, i.e., 100% outdoor (about 1% performance loss on beam prediction accuracy of Top-1 beam). While AI model trained by data of only UE distribution Option A, i.e., 20% outdoor provides a little worse generalization capability for UE distribution Option B (about 20% performance loss on beam prediction accuracy of Top-1 beam) when the ratio of set B and set A is 1/4.

Generalization on different number of UE Rx beam

In order to evaluate the performance of generalization on different number of UE Rx beam, we consider scheme 3 for each generalization case with Uma.

- Scheme 3: Fixed set B with L1-RSRP as input;

  - Select 64 beam pairs with same beam pair IDs and input their L1-RSRP to AI model for beam prediction.

As for the number of UE Rx beam, we evaluate Option 1 and Option 2. While Option 1 means the number of Rx beam is 8, and Option 2 means the number of Rx beam is 4 with Rx beam ID#0&2 in the first and the second panel in Figure 3-4. For Option 1, the KPI is calculated considering all beam pairs and for Option 2 the KPI is calculated considering only beam pairs of the 4 Rx beams.

- Case 1: training by data of Option 2, inference by data of Option 2

    - Input for training:

      - Pattern #1: L1-RSRP of fixed 64 beam pair IDs from 0~31,64~95,128~159,192~223.

    - Input for inference:

      - Pattern #1.

- Case 2: training by data of Option 1, inference by data of Option 2

    - Input for training: mixed data of pattern#1 and pattern#2 with 50% of each pattern

      - Pattern #1: L1-RSRP of fixed 64 beam pair IDs from 0~31,64~95,128~159,192~223.

      - Pattern #2: L1-RSRP of fixed 64 beam pair IDs from 32~63,96~127,160~191,224~255.

    - Input for inference

      - Pattern #1.

The evaluation results can be seen in Table 3-23, Table 3-24, Table 3-25, and Table 3-26. Based on the evaluation results, we can see that AI model of case 1 provides good generalization capability among different number UE Rx beam.

Table 3-23 Average L1-RSRP difference of Top-1 predicted beam for generalization

| Generalization case | Average L1-RSRP difference (dB) |
| --- | --- |
| Case1 | 0.2 |
| Case2 | 0.2 |

Table 3-24 Beam prediction accuracy (%) for Top-1 and/or Top-K beams（K=1,2,4）for generalization

| Generalization case | Beam prediction accuracy (%) |  |  |
| --- | --- | --- | --- |
|  | K=1 | K=2 | K=4 |
| Case1 | 75.2 | 91.7 | 98.3 |
| Case2 | 73.1 | 89.4 | 97.1 |

Table 3-25 Beam prediction accuracy (%) with 1dB margin for Top-1 beam for generalization

| Generalization case | Beam prediction accuracy (%) |
| --- | --- |
| Case1 | 94.1 |
| Case2 | 92.4 |

Table 3-26 Average L1-RSRP difference between the ideal L1-RSRP of Top-1 predicted beam and the predicted L1-RSRP of the Top 1 predicted beam

| Scheme | Average L1-RSRP difference when Top 1 predicted beam is same as the Top 1 genie-aided beam (dB) |
| --- | --- |
| Case1 | 0.18 |
| Case2 | 0.17 |

Observation 8: For AI based beam pair prediction in spatial domain with pre-configured patterns of set B, AI model for beam pair prediction in spatial domain can provide good generalization capability among different number of UE Rx beam, e.g., AI model with more Rx beam number (8Rx) training by pre-configured patterns of set B can be applied for beam prediction of less Rx beam number (4Rx) with only about 2% performance loss on beam prediction accuracy of Top-1 beam when the ratio of set B and set A is 1/4.

DL Tx beam prediction

The following agreements on Rx beam for DL Tx beam prediction were archived in RAN1-111 meeting [6] and RAN1-112b e-meeting [8].

Agreement

At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable

- Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample

- Option 2: Measurements of specific Rx beam(s)

  - Option 2a: Measurements of specific Rx beam(s) per model input sample

  - Option 2b: Measurements of specific Rx beam(s) for all model input sample

  - FFS how to select the specific Rx beam(s)

- Option 3: Measurements of random Rx beam(s) per model input sample

- Other options are not precluded and can be reported by companies.

Agreement

At least for evaluation on the performance of DL Tx beam prediction, consider the following options for Rx beam for providing input for AI/ML model for training and/or inference if applicable

- Option 1: Measurements of the “best” Rx beam with exhaustive beam sweeping for each model input sample

  - Companies report how to select the “best” Rx beam(s)

- Option 2: Measurements of specific Rx beam(s)

  - Companies report how to select specific Rx beam(s)

- Option 3: Measurements of random Rx beam(s) per model input sample

Other options are not precluded and can be reported by companies.

DL Tx beam prediction with specific Rx beam for model input

For DL Tx beam prediction, as for the L1-RSRP for input and/ or output, Option 2 means consider only one specific Rx beam. Thus the L1-RSRP of 8 DL Tx beam (Tx beam ID #0, 4, 8, 12, 16…, 28) with one specific Rx beam are considered as the model input and the best Rx beam for each DL Tx beam is considered for the model label and output.

Since there are at most 8 Rx beams, we consider each Rx beam as the specific Rx beam for all model input sample, i.e., Option 2b. The Rx beam ID can be seen in Figure 3-4. Compared to the exhaustive beam sweeping scheme, the evaluation results can be seen in Table 3-2-1, 3-2-2, 3-2-3 and 3-2-4. The performance of DL Tx beam prediction with specific Rx beam is worse than the beam pair prediction.

- Pattern 1：Rx beam ID#0 of the first panel.

- Pattern 2：Rx beam ID#1 of the first panel.

- Pattern 3：Rx beam ID#2 of the first panel.

- Pattern 4：Rx beam ID#3 of the first panel.

- Pattern 5：Rx beam ID#0 of the second panel.

- Pattern 6：Rx beam ID#1 of the second panel.

- Pattern 7：Rx beam ID#2 of the second panel.

- Pattern 8：Rx beam ID#3 of the second panel.

Table 3-2-1, Average ideal L1-RSRP difference between Top-1 predicted beam and Top-1 genie-aided beam

| Pattern | Average L1-RSRP difference (dB) |
| --- | --- |
| Pattern 1 | 1.5 |
| Pattern 2 | 1.6 |
| Pattern 3 | 1.6 |
| Pattern 4 | 1.5 |
| Pattern 5 | 1.8 |
| Pattern 6 | 1.6 |
| Pattern 7 | 1.7 |
| Pattern 8 | 1.6 |

Table 3-2-2, Average predicted L1-RSRP difference of Top-1 predicted beam

| Pattern | Average predicted L1-RSRP difference (dB) |
| --- | --- |
| Pattern 1 | 3.90 |
| Pattern 2 | 3.62 |
| Pattern 3 | 4.21 |
| Pattern 4 | 3.42 |
| Pattern 5 | 3.67 |
| Pattern 6 | 4.13 |
| Pattern 7 | 3.68 |
| Pattern 8 | 3.22 |

Table 3-2-3, Beam prediction accuracy (%) for Top-1 and/or Top-K beams (K=2, 4)

| Pattern | Beam prediction accuracy (%) |  |  |
| --- | --- | --- | --- |
|  | K=1 | K=2 | K=4 |
| Pattern 1 | 50.9 | 68.3 | 83.9 |
| Pattern 2 | 50.0 | 70.2 | 84.2 |
| Pattern 3 | 52.5 | 68.6 | 82.6 |
| Pattern 4 | 50.8 | 67.5 | 84.3 |
| Pattern 5 | 50.8 | 69.1 | 82.7 |
| Pattern 6 | 53.1 | 71.5 | 84.2 |
| Pattern 7 | 53.8 | 70.8 | 84.1 |
| Pattern 8 | 52.9 | 69.1 | 83.8 |

Table 3-2-4, Beam prediction accuracy (%) with 1dB margin for Top-1 beam

| Pattern | Beam prediction accuracy (%) |
| --- | --- |
| Pattern 1 | 67.4 |
| Pattern 2 | 66.9 |
| Pattern 3 | 66.1 |
| Pattern 4 | 66.8 |
| Pattern 5 | 66.6 |
| Pattern 6 | 68.6 |
| Pattern 7 | 68.7 |
| Pattern 8 | 68.0 |

Observation 9: For AI based DL Tx beam prediction in spatial domain with fixed set B, considering one specific Rx beam for model input results in acceptable performance (about 50~53.8% beam prediction accuracy of Top-1 beam) compared to exhaustive beam pair sweeping when the ratio of set B and set A is 1/4.

Proposal 3: DL Tx beam prediction with specific Rx beam for model input can be supported considering the low RS overhead for model input.

DL Tx beam prediction with best Rx beam for model input

For DL Tx beam prediction, as for the L1-RSRP for input and/ or output, Option 1 means consider best Rx beam. But for how to obtain the Rx beam, there are two alternatives. Alt 1 is to obtain the best Rx beam for the best Tx beam. And that best Rx beam will be used for all Tx beam for model input. Alt 2 is to obtain the best Rx beam for each DL Tx beam. in our understanding, the RS overhead of these two alternatives are same to each other, so we consider the best Rx beam for each DL Tx beam.

Thus the L1-RSRP of 8 DL Tx beam (Tx beam ID #0, 4, 8, 12, 16…, 28) with each best Rx beam are considered as the model input and the L1-RSRP of 32 DL Tx beam with each best Rx beam are considered as the model label and model output.

Compared to the exhaustive beam sweeping scheme, the evaluation results can be seen in Table 3-2-5, 3-2-6, 3-2-7 and 3-2-8. The performance of DL Tx beam prediction with the best Rx beam is a little better than the beam pair prediction.

Table 3-2-5, Average ideal L1-RSRP difference between Top-1 predicted beam and Top-1 genie-aided beam

| Scheme | Average L1-RSRP difference (dB) |
| --- | --- |
| DL Tx beam prediction with the best Rx beam | 0.11 |

Table 3-2-6, Average predicted L1-RSRP difference of Top-1 predicted beam

| Scheme | Average predicted L1-RSRP difference (dB) |
| --- | --- |
| DL Tx beam prediction with the best Rx beam | 0.46 |

Table 3-2-7, Beam prediction accuracy (%) for Top-1 and/or Top-K beams (K=2, 4)

| Scheme | Beam prediction accuracy (%) |  |  |
| --- | --- | --- | --- |
|  | K=1 | K=2 | K=4 |
| DL Tx beam prediction with the best Rx beam | 82.8 | 96.6 | 99.4 |

Table 3-2-8, Beam prediction accuracy (%) with 1dB margin for Top-1 beam

| Scheme | Beam prediction accuracy (%) |
| --- | --- |
| DL Tx beam prediction with the best Rx beam | 96.7 |

Observation 10: For AI based DL Tx beam prediction in spatial domain with fixed set B, considering the best Rx beam for model input provides good performance (about 83% beam prediction accuracy of Top-1 beam) compared to exhaustive beam pair sweeping.

Observation 11: For AI based DL Tx beam prediction in spatial domain with fixed set B, the average predicted L1-RSRP difference of the Top 1 predicted beam is less than 0.5 dB considering the best Rx beam for model input when the ratio of set B and set A is 1/4.

Proposal 4: Support DL Tx beam prediction with the best Rx beam considering the high beam prediction accuracy.

Different set B for DL Tx beam prediction with best Rx beam

In order to evaluate the performance for different set B and pre-configured patterns of set B, we consider following 4 different fixed set B with 8 DL Tx beams with non-continuous DL Tx beam ID. In addition, Mixed set B with different set B in pre-configured pattern of set B are evaluated as pattern 5, and the pre-configured pattern of set B consists of the following 4 different fixed set B.

- Pattern 1 of fixed Set B with non-continuous Tx beam pair ID: Tx beam ID #0, 4, 8, 12, 16…, 28.

- Pattern 2 of fixed Set B with non-continuous Tx beam pair ID: Tx beam ID #1, 5, 9, 13, 17…, 29.

- Pattern 3 of fixed Set B with non-continuous Tx beam pair ID: Tx beam ID #2, 6, 10, 14, 18…, 30.

- Pattern 4 of fixed Set B with non-continuous Tx beam pair ID: Tx beam ID #3, 7, 11, 15, 19…, 31.

- Pattern 5: different Set B in pre-configured pattern of Set B with non-continuous Tx beam ID.

  - mixed data samples of above Pattern#1, #2, #3 and #4 with 25% of each for training and inference.

- training by data of Pattern 1, inference by data of pattern 2.

- training by data of Pattern 1 and Pattern 2, inference by data of Pattern 2.

- training by data of mixed data of Pattern #1, 2,3 and 4, inference by data of fixed Set B with non-continuous Tx beam ID: DL Tx beam ID #0, 2, 4, 6, 16, 18, 20, 22, which is different from pattern #1, 2, 3 and 4.

Compared to the exhaustive beam sweeping scheme, the evaluation results can be seen in Table 3-2-9, 3-2-10, 3-2-11 and 3-2-12. The performance of DL Tx beam prediction with the best Rx beam is a little better than the beam pair prediction.

Table 3-2-9, Average ideal L1-RSRP difference between Top-1 predicted beam and Top-1 genie-aided beam

| Pattern of set B | Average L1-RSRP difference (dB) |
| --- | --- |
| Pattern 1 | 0.11 |
| Pattern 2 | 0.09 |
| Pattern 3 | 0.10 |
| Pattern 4 | 0.37 |
| Pattern 5 | 0.46 |
| Pattern 6 | 0.63 |
| Pattern 7 | 0.36 |
| Pattern 8 | 0.94 |

Table 3-2-10, Beam prediction accuracy (%) for Top-1 and/or Top-K beams (K=2, 4)

| Pattern of set B | Beam prediction accuracy (%) |  |  |
| --- | --- | --- | --- |
|  | K=1 | K=2 | K=4 |
| Pattern 1 | 82.8 | 96.6 | 99.4 |
| Pattern 2 | 82.6 | 96.4 | 99.5 |
| Pattern 3 | 82.5 | 96.4 | 99.4 |
| Pattern 4 | 76.2 | 91.5 | 93.2 |
| Pattern 5 | 73.6 | 91.2 | 92.6 |
| Pattern 6 | 65.1 | 86.9 | 90.7 |
| Pattern 7 | 67.5 | 89.5 | 91.5 |
| Pattern 8 | 60.2 | 78.6 | 86.9 |

Table 3-2-11, Beam prediction accuracy (%) with 1dB margin for Top-1 beam

| Pattern of set B | Beam prediction accuracy (%) |
| --- | --- |
| Pattern 1 | 96.7 |
| Pattern 2 | 96.5 |
| Pattern 3 | 96.6 |
| Pattern 4 | 91.1 |
| Pattern 5 | 88.6 |
| Pattern 6 | 82.3 |
| Pattern 7 | 90.5 |
| Pattern 8 | 78.9 |

Table 3-2-12, Average predicted L1-RSRP difference of Top-1 predicted beam

| Pattern of set B | Average predicted L1-RSRP difference (dB) |
| --- | --- |
| Pattern 1 | 0.46 |
| Pattern 2 | 0.80 |
| Pattern 3 | 0.56 |
| Pattern 4 | 1.93 |
| Pattern 5 | 0.80 |
| Pattern 6 | 2.55 |
| Pattern 7 | 2.42 |
| Pattern 8 | 2.89 |

Observation 12: For AI based DL Tx beam prediction in spatial domain, different set B in pre-configured patterns of set B for DL Tx beam prediction results in only 3~9% performance loss on beam prediction accuracy of Top-1 beam compared to same fixed set B.

Generalization

Generalization on different set B of UE Tx beam

As for the different set B of UE Tx beam, we evaluate Option 1 and Option 2. While Option 1 means the Tx beam IDs of set B are Tx beam ID#0, 4, 8, …28, and Option 2 means the Tx beam IDs of set B are Tx beam ID#1, 5, 9, …29.

- Case 1: training by data of Option 2, inference by data of Option 2.

- Case 2: training by data of Option 1, inference by data of Option 2.

- Case 3: training by data of Option 1 and Option 2, inference by data of Option 2.

Compared to the exhaustive beam sweeping scheme, the evaluation results can be seen in Table 3-2-13, 3-2-14, 3-2-15 and 3-2-16.

Table 3-2-13, Average L1-RSRP difference of Top-1 predicted beam for generalization

| Generalization case | Average L1-RSRP difference (dB) |
| --- | --- |
| Case1 | 0.09 |
| Case2 | 0.63 |
| Case3 | 0.36 |

Table 3-2-14, Beam prediction accuracy (%) for Top-1 and/or Top-K beams（K=1,2,4）for generalization

| Generalization case | Beam prediction accuracy (%) |  |  |
| --- | --- | --- | --- |
|  | K=1 | K=2 | K=4 |
| Case1 | 82.6 | 96.4 | 99.5 |
| Case2 | 65.1 | 86.9 | 90.7 |
| Case3 | 67.5 | 89.5 | 91.5 |

Table 3-2-15, Beam prediction accuracy (%) with 1dB margin for Top-1 beam for generalization

| Generalization case | Beam prediction accuracy (%) |
| --- | --- |
| Case1 | 96.5 |
| Case2 | 82.3 |
| Case3 | 90.5 |

Table 3-2-16, Average L1-RSRP difference between the ideal L1-RSRP of Top-1 predicted beam and the predicted L1-RSRP of the Top 1 predicted beam

| Scheme | Average L1-RSRP difference when Top 1 predicted beam is same as the Top 1 genie-aided beam (dB) |
| --- | --- |
| Case1 | 0.80 |
| Case2 | 2.55 |
| Case3 | 2.42 |

Observation 13: For AI based DL Tx beam prediction in spatial domain, AI model trained by hybrid data of different set B can provide acceptable generalization capability (about 15% performance loss on beam prediction accuracy of Top-1 beam). While AI model trained by data of set B1 provide a little worse generalization capability (about 18% performance loss on beam prediction accuracy of Top-1 beam) for different set B2.

Generalization on different scenario- Uma and Umi

In order to evaluate the performance of generalization on different scenario for DL Tx beam prediction, we consider fixed set with the DL Tx beam IDs #0, 4, 8, 12, 16……, 28.

And the performance of following 3 cases are evaluated, wherein ISD 500m and BS antenna height 25m is assumed for Uma, and ISD 200m and BS antenna height 10m is assumed for Umi. As for case 3, the training data contains 50% of Uma and 50% of Umi.

- Case 1: training by data of Uma, inference by data of Uma.

- Case 2: training by data of Umi, inference by data of Uma.

- Case 3: training by data of Uma and Umi, inference by data of Uma.

Compared to the exhaustive beam sweeping scheme, the evaluation results can be seen in Table 3-2-17, 3-2-18, 3-2-19 and 3-2-20. The performance of DL Tx beam prediction with the best Rx beam is a little better than the beam pair prediction.

Table 3-2-17, Average L1-RSRP difference of Top-1 predicted beam for generalization

| Generalization case | Average L1-RSRP difference (dB) |
| --- | --- |
| Case1 | 0.11 |
| Case2 | 0.38 |
| Case3 | 0.27 |

Table 3-2-18, Beam prediction accuracy (%) for Top-1 and/or Top-K beams（K=1,2,4）for generalization

| Generalization case | Beam prediction accuracy (%) |  |  |
| --- | --- | --- | --- |
|  | K=1 | K=2 | K=4 |
| Case1 | 82.8 | 96.6 | 99.4 |
| Case2 | 66.6 | 77.3 | 96.0 |
| Case3 | 73.5 | 90.9 | 97.2 |

Table 3-2-19, Beam prediction accuracy (%) with 1dB margin for Top-1 beam for generalization

| Generalization case | Beam prediction accuracy (%) |
| --- | --- |
| Case1 | 96.7 |
| Case2 | 85.6 |
| Case3 | 90.3 |

Table 3-2-20, Average predicted L1-RSRP difference of Top-1 predicted beam

| Scheme | Average predicted L1-RSRP difference (dB) |
| --- | --- |
| Case1 | 0.44 |
| Case2 | 0.50 |
| Case3 | 0.48 |

Observation 14: For AI based DL Tx beam prediction in spatial domain, AI model trained by hybrid data of Uma and Umi can provide good generalization capability for Uma (about 9% performance loss on beam prediction accuracy of Top-1 beam). While AI model trained by data of only Umi provide a little worse generalization capability for Uma inference (about 16% performance loss on beam prediction accuracy of Top-1 beam).

Generalization on different UE distribution- 100% outdoor and 20% outdoor

In order to evaluate the performance of generalization on different UE distribution for DL Tx beam prediction, we consider Option 1 with 80% indoor and 20% outdoor, and Option 2 with 100% outdoor. As for the input of AI model, we consider fixed set with the DL Tx beam IDs #0, 4, 8, 12, 16……, 28. for each generalization case with Uma.

| Parameters | Values |
| --- | --- |
| UE distribution | - For spatial domain beam prediction: - Option 1: 80% indoor ,20% outdoor as in TR 38.901 - Option 2: 100% outdoor - For time domain prediction: 100% outdoor |

- Case 1: training by data of Option 1, inference by data of Option 2.

- Case 2: training by data of Option 1 and Option 2, inference by data of Option 2.

- Case 3: training by data of Option 1 and Option 2, inference by data of Option 1.

- Case 4: training by data of Option 2, inference by data of Option 1.

- Case 5: training by data of Option 1, inference by data of Option 1.

- Case 6: training by data of Option 2, inference by data of Option 2.

Compared to the exhaustive beam sweeping scheme, the evaluation results can be seen in Table 3-2-21, 3-2-22, 3-2-23 and 3-2-24. The performance of DL Tx beam prediction with the best Rx beam is a little better than the beam pair prediction.

Table 3-2-21 Average L1-RSRP difference of Top-1 predicted beam for generalization

| Generalization case | Average L1-RSRP difference (dB) |
| --- | --- |
| Case1 | 0.46 |
| Case2 | 0.41 |
| Case3 | 0.33 |
| Case4 | 0.40 |
| Case5 | 0.11 |
| Case6 | 0.22 |

Table 3-2-22 Beam prediction accuracy (%) for Top-1 and/or Top-K beams（K=1,2,4）for generalization

| Generalization case | Beam prediction accuracy (%) |  |  |
| --- | --- | --- | --- |
|  | K=1 | K=2 | K=4 |
| Case1 | 64.4 | 83.0 | 94.5 |
| Case2 | 66.4 | 86.3 | 94.5 |
| Case3 | 70.6 | 92.4 | 97.4 |
| Case4 | 65.7 | 91.6 | 94.6 |
| Case5 | 82.8 | 96.6 | 99.4 |
| Case6 | 81.3 | 95.1 | 99.2 |

Table 3-2-23 Beam prediction accuracy (%) with 1dB margin for Top-1 beam for generalization

| Generalization case | Beam prediction accuracy (%) |
| --- | --- |
| Case1 | 85.3 |
| Case2 | 90.0 |
| Case3 | 87.1 |
| Case4 | 83.4 |
| Case5 | 96.7 |
| Case6 | 87.5 |

Table 3-2-24 Average predicted L1-RSRP difference of Top-1 predicted beam

| Scheme | Average predicted L1-RSRP difference (dB) |
| --- | --- |
| Case1 | 0.55 |
| Case2 | 0.58 |
| Case3 | 0.48 |
| Case4 | 0.50 |
| Case5 | 0.44 |
| Case6 | 0.39 |

Observation 15: For AI based DL Tx beam prediction in spatial domain, AI model trained by hybrid data of different UE distribution can provide good generalization capability for option 1 with 20% outdoor (about 12% performance loss on beam prediction accuracy of Top-1 beam). While AI model trained by data of Option 2 with 100% outdoor provides a little worse generalization capability for option 1 inference (about 17% performance loss on beam prediction accuracy of Top-1 beam).

Temporal beam prediction

Use case

While in time domain, the principle of beam prediction can be seen in Figure 4-1. It means that the best beam at time T+m can be predicted by AI model based on the history information. And the history information may include the beam information in last N periods. Temporal beam prediction can include two schemes. The scheme 1 (Figure 4-2) is that the periodicity for history measurement instance is same as that of future time instance. For example, there are N + M short periods in a large period, and in each large period, UE perform beam measurement and report in the first N short periods and predict best beams in other M periods. The reference signal overhead and UE side measurement complexity can be reduced in the other M periods. The scheme 2 (Figure 4-3) is that the periodicity for history measurement instance is larger than that of future time instance. For example, UE only perform beam measurement and report with a long period, and based on the beam information of N long history periods, the best M beams can be predicted for the (N +1)th long period, and the  (N +1)th long period can be divided into M+1 short periods. The beam selection accuracy can be improved in the (N +1)th long period. Based on the analysis above, both scheme 1 and scheme 2 should be considered.

Figure 4-1, Principle of AI based beam prediction in time domain

Figure 4-2, Scheme 1 of AI based beam prediction in time domain

Figure 4-3, Scheme 2 of AI based beam prediction in time domain

AI/ML model

For time domain beam prediction, we check the performance of LSTM (Long-Short Term Memory), and the corresponding parameters can be seen in Table 4-1.

As for the data set, we use about 100000 samples for model training (95%) and testing. And the data are obtained from system level simulation in urban macro. The periodicity of data collection is 100ms. For scheme 1 in Figure 4-2, it means the periodicity of future time instance and the periodicity of history measurement instance are same, i.e., 100ms. While for scheme 2, the periodicity of future time instance is 100ms, and the periodicity of history measurement instance is (M+1)*100ms according to Figure 4-3. The number of beam pairs in set B is 64/256 and the number of beam pairs in Set A is 256. The model is trained based on the categorical_crossentropy as loss function.

Table 4-1, Parameters of AI model with LSTM

| Layer |  | Output shape | Parameters |
| --- | --- | --- | --- |
| Input layer |  | (B, N, 256) / (B, N, 64) | 0 |
| LSTM_1(128) |  | (B, N, 128) | 197120 / 98816 |
| Activation(ReLU) |  | (B, N, 128) |  |
| LSTM_2(64) |  | (B, 64) | 49408 |
| Activation(ReLU) |  | (B, 64) |  |
| Dense(hidden unit=32M) |  | (B, 32M) | (64+1)*32M |
| Output layer (Reshape) |  | (B, M, 32) | 0 |
| Model complexity () |  |  |  |
| The number of predicted instance(s) | Total parameters | Model size | FLOPs |
| 1 | 248608/150304 | 2.42 MB/1.36MB | 0.413M /0.260M |
| 2 | 250688 | 2.65MB | 0.421M |
| 4 | 254848 | 2.97MB | 0.428M |

Evaluation assumption and parameters

According to the agreement archived in RAN1-109 e-meeting, the following options can be considered for temporal beam prediction accuracy.

Agreement

- At least for temporal beam prediction, companies report the one of spatial consistency procedures:

  - Procedure A in TR38.901

  - Procedure B in TR38.901

Agreement

- For temporal beam prediction, the following options can be considered as a starting point for UE trajectory model for further study. Companies report further changes or modifications based on the following options for UE trajectory model. Other options are not precluded.

  - Option #2: Linear trajectory model with random direction change.

    - UE moving trajectory: UE will move straightly along the selected direction to the end of a time interval, where the length of the time interval is provided by using an exponential distribution with average interval length, e.g., 5s, with granularity of 100 ms.

      - UE moving direction change: At the end of the time interval, UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°].

      - UE move straightly within the time interval with the fixed speed.

    - FFS on UE orientation

  - Option #3: Linear trajectory model with random and smooth direction change.

    - UE moving trajectory: UE will change the moving direction by multiple steps within an time internal, where the length of the time interval is provided by using an exponential distribution with average interval length, e.g., 5s, with granularity of 100 ms.

      - UE moving direction change: At the end of the time interval, UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°].

      - The time interval is further broken into N sub-intervals, e.g. 100ms per sub-interval, and at the end of each sub-interval, UE change the direction by the angle of A_diff/N.

      - UE move straightly within the time sub-interval with the fixed speed.

    - FFS on UE orientation

  - Option #4: Random direction straight-line trajectories.

- Initial UE location, moving direction and speed: UE is randomly dropped in a cell, and an initial moving direction is randomly selected, with a fixed speed.

  - The initial UE location should be randomly drop within the following blue area

where d1 is the minimum distance that UE should be away from the BS.

    - Each sector is a cell and that the cell association is geometry based.

    - During the simulation, inter-cell handover or switching should be disabled.

For training data generation

- For each UE moving trajectory: the total length of the UE trajectory can be set as T second if it is in time, of set as D meter if it is in distance.

  - The value of T (or D) can be further discussed

  - The trajectory sampling interval granularity depends on UE speed and it can be further discussed.

- UE can move straightly along the entire trajectory, or

- UE can move straightly during the time interval, where the time interval is provided by using an exponential distribution with average interval length ΔT

  - UE may change the moving direction at the end of the time interval. UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°]

- If the UE trajectory hit the cell boundary (the red line), the trajectory should be terminated.

  - If the trajectory length (in time) is less than the length of observation window + prediction window, the trajectory should be discarded.

  - At the current stage, the length of observation window + prediction window is not fixed and the companies can report their values.

    - FFS on UE orientation

- Generalization issue is FFS

Agreement

- For temporal beam prediction, further study the following options as baseline performance

  - Option 1a: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources or all possible beams from Set A of beams at the time instants within T2

  - Option 2: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources from Set B of beams at the time instants within T1

    - Companies explain the detail on how to select the best beam for T2 from Set A based on the measurements in T1

  - Where T2 is the time duration for the best beam selection, and T1 is a time duration to obtain the measurements of all the RS resource from Set B of beams.

    - T1 and T2 are aligned with those for AI/ML based methods

  - Whether Set A and Set B are the same or different depend on the sub-use case

  - Other options are not precluded.

Agreement

Regarding the sub use case BM-Case2, the measurement results of K (K>=1) latest measurement instances are used for AI/ML model input:

- The value of K is up to companies

Agreement

Regarding the sub use case BM-Case2, AI/ML model output should be F predictions for F future time instances, where each prediction is for each time instance.

- At least F = 1

- The other value(s) of F is up to companies

Agreement

For the sub use case BM-Case2, further study the following alternatives:

- Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A)

- Alt.2: Set B is a subset of Set A (Set A and Set B are not the same)

- Alt.3: Set A and Set B are the same

- Note1: The beam pattern of Set A and Set B can be clarified by the companies.

The following agreements were archived in RAN1-110b e-meeting [5].

Agreement

- At least for BM-Case 2, consider the following assumptions for evaluation

  - Periodicity of time instance for each measurement/report in T1:

    - 20ms, 40ms, 80ms, [100ms], 160ms, [960ms]

    - Other values can be reported by companies.

  - Number of time instances for measurement/report in T1 can be reported by companies.

  - Time instance(s) for prediction can be reported by companies.

The following conclusion was archived in RAN1-112b e-meeting [8].

Conclusion

To evaluate the performance of BMCase-2 for both DL Tx beam and pair prediction, aiming to analysis the following aspects:

- Clarify the baseline performance in terms of beam prediction accuracy and/or average L1-RSRP.

- Observations based on the metrics to be considered:

  - Top-1/K [=2] beam prediction accuracy, Top-1 beam prediction accuracy with 1dB, average L1-RSRP difference

  - Measurement/RS overhead reduction

  - UCI overhead (reduction) potentially with different quantization

  - User throughput

  - Model size and complexity

  - Average predicted L1-RSRP difference, if applicable

- Scenarios/assumptions/Cases for basic observations

  - Set A and Set B relationship

    - Set A= Set B

    - Set B /Set A =1/4, [1/6], 1/8, 1/16, [1/32]

  - UE speed: 30km/h

  - No UE rotation

  - FFS the following cases for results reporting.

    - Case 1:  based on T1 and T2, where T2 is the time duration for the best beam selection, and T1 is a time duration to obtain the measurements of all the RS resource from Set B of beams.

      - T1 = 40ms, 80ms, 160ms, [320ms], [640ms]

      - T2 = 40ms, 80ms, 160ms, 320ms, [960ms]

      - M= [1, 2, 3, 4, 5, 8], where M is the number of time instance(s) for measurement/report in T1 as AI/ML inputs (per model)

      - P= [1, 2, 4, 5, …], where P is the number of time instance(s) P for prediction as AI/ML model output(s)/label(s) (per model)

    - Case 2: based on the number of prediction instance(s) Y for every number of measurement instance(s) X, at least consider the following values:

      - Minimal periodicity for time instances for measurement(s) and prediction(s) = [40ms, 80ms, 160ms]

      - X = [1, 2]

      - Y = [1, 2, 4, 5, 10]

      - P= [1, 2, 4, 5, …], where P is the number of time instance(s) P for prediction as AI/ML model output(s)/label(s) (per model)

      - The number of measurement instance(s) as AI inputs are up to implementation.

  - FFS whether separated observations are needed or not for the following:

    - UE trajectories

- Performance difference based on the reported results from each company

  - With UE rotation

  - Different UE speed: e.g., 60km/h, 90km/h, 120km/h

  - Different observation/prediction windows or periodicity for time instances.

  - Different Set B assumption when Set A is a subset of Set B

    - Opt A/B, Opt C, Opt D

- Other settings:

  - Other percentage of Set B and Set A if reported by companies

  - When Set B is different from Set A (e.g., Set B is composed of wide beams and Set A is composed of narrow beams).

- Other aspects are not precluded

- Observation/analysis may consider UE-side and NW-side model when applicable

Here we take Option 1a and Option 2 as the baseline and consider Alt.3 that set A and set B are the same. As for the baseline scheme Option 2, we use the best beam of the last measurement instance as the Top-1 beam in future time instance. In addition to Table 3-2, the additional simulation assumptions and parameters for temporal beam prediction were provided in Table 4-2. Based on these assumptions and parameters, we evaluate 2 schemes listed below:

- Scheme 0: Option 1a/option 2 as baseline

- Scheme 1 in Figure 4-2: Input L1-RSRP of beam pairs in set B of N latest measurement instances to predict Top-K beam for M future time instances, and the periodicity of latest measurement instances is same as that of future time instances.

- Scheme 2 in Figure 4-3: Input L1-RSRP of beam pairs in set B of N latest measurement instances to predict Top-K beam for M future time instances, and the periodicity of latest measurement instances is larger than that of future time instances.

Table 4-2, Simulation parameters for temporal beam prediction

| Parameters |  | Value |
| --- | --- | --- |
| UE number/ per sector |  | 5 |
| Spatial consistency procedures |  | Procedure A |
| UE distribution |  | For time domain prediction: 100% outdoor |
| UE trajectory model |  | Option #2 |
| Baseline performance |  | Option#1a/ Option #2 |
| N (N >=1) latest measurement instances |  | 4 |
| M future time instances | Scheme 1 | 1/2/4 |
|  | Scheme 2 | 1/2/4 |
| UE speed |  | 30km/h |
| Periodicity of M time instance |  | 80ms/160ms |
| Periodicity of N latest measurement instances | Scheme 1 | 80ms/160ms |
|  | Scheme 2 | (M+1)*80ms (M+1)*160ms |

KPI

The following agreements on KPI were archived for BM-case 2 in RAN1-111 meeting.

Agreement

- For the evaluation of the overhead for BM-Case2, adoption the following metrics:

  - RS overhead reduction,

    - Option 2: RS OH reduction%=1-NM

      - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable

      - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme

      - Companies report the assumption on additional measurements

    - FFS: Option 3: RS OH reduction%=1-NLM

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each time instance

      - where M is the total number of beams (pairs) to be predicted for each time instance

      - where L is ratio of periodicity of time instance for measurements to periodicity of time instance for prediction

    - Companies report the assumption on T1 and T2 patterns

    - Other options are not precluded and can be reported by companies.

The following agreements on KPI were archived for BM-case 2 in RAN1-112b e-meeting.

Agreement

- For the evaluation of the overhead for BM-Case2, adoption the following metrics:

  - RS overhead reduction,

    - RS OH reduction%=1-NM

      - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable

      - where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme

      - Companies report the assumption on additional measurements

    - Companies report the assumption on baseline scheme

    - Companies report the assumption on T1 and T2

    - Other options are not precluded and can be reported by companies.

Evaluation results

In order to align with the beam measurement report, we also consider three K values for predicted Top-K beams with K=1, 2 or 4.

Scheme 1 and scheme 2 with set B= set A

For the case of N (N >=1) latest measurement instances equals to 4, M future time instances equals to 1/2/4, and consider L1-RSRP of all beam pairs as input, the evaluation results can be seen in Table 4-3, 4-4 and 4-5. Table 4-3 provides the average L1-RSRP difference of Top-1 predicted beam. Table 4-4 provides Beam prediction accuracy (%) for Top-1 and/or Top- K beams (K=2, 4).  Table 4-5 provides beam prediction accuracy (%) with 1dB margin for Top-1 beam. From the evaluation results, both schemes provide good performance. And for the case of M=1, scheme 2 provides same performance as scheme 1 with more RS overhead reduction. While for M =4, the performance of scheme 2 is worse than that of scheme 2. It is because history information of a longer time may cause negative impact to the prediction performance.

Table 4-3, Average L1-RSRP difference of Top-1 predicted beam

| Scheme | Average L1-RSRP difference (dB) |  |  |
| --- | --- | --- | --- |
|  | M=1 | M=2 | M=4 |
| Baseline-Option 1a | 0 |  |  |
| Scheme 1 / Baseline-Option 2 (80ms) | 0.12 / 0.16 | 0.13 / 0.19 | 0.19 / 0.20 |
|  |  |  | 0.20 / 0.22 |
|  |  | 0.14 / 0.21 | 0.22 / 0.25 |
|  |  |  | 0.25 / 0.26 |
| Scheme 1/ Baseline-Option 2 (160ms) | 0.17 / 0.20 | 0.17 / 0.25 | 0.24 / 0.26 |
|  |  |  | 0.25 / 0.26 |
|  |  | 0.20 / 0.26 | 0.27 / 0.29 |
|  |  |  | 0.30 / 0.31 |
| Scheme 2/ Baseline-Option 2 (80ms) | 0.21 / 0.21 | 0.24 / 0.25 | 0.31 / 0.30 |
|  |  |  | 0.33 / 0.33 |
|  |  | 0.26 / 0. 26 | 0.36 / 0.35 |
|  |  |  | 0.39 / 0.37 |
| Scheme 2 / Baseline-Option 2 (160ms) | 0.27 / 0.29 | 0.30 / 0.32 | 0.36 / 0.39 |
|  |  |  | 0.39 / 0.42 |
|  |  | 0.34 / 0.35 | 0.41 / 0.43 |
|  |  |  | 0.44 / 0.46 |

Table 4-4, Beam prediction accuracy (%) for Top-1 and/or Top-K beams (K=2, 4)

4-4a) M=1

| Scheme | Beam prediction accuracy (%) |  |  |
| --- | --- | --- | --- |
|  | K =1 | K =2 | K =4 |
| Baseline-Option 1a | 100 |  |  |
| Scheme 1 / Baseline-Option 2 (80ms) | 89.0/ 88.5 | 92.3 / 91.0 | 94.1 / 93.6 |
| Scheme 1/ Baseline-Option 2 (160ms) | 87.3 / 85.3 | 90.4 / 89.6 | 92.3 /91.1 |
| Scheme 2/ Baseline-Option 2 (80ms) | 86.1 / 86.2 | 88.7 / 89.0 | 91.6 / 92.2 |
| Scheme 2 / Baseline-Option 2 (160ms) | 80.6 / 80.1 | 86.8 / 86.5 | 90.2 / 89.9 |

4-4b) M=2

| Scheme | Beam prediction accuracy (%) |  |  |
| --- | --- | --- | --- |
|  | K =1 | K =2 | K =4 |
| Baseline-Option 1a | 100 |  |  |
| Scheme 1 / Baseline-Option 2 (80ms) | 87.2 / 86.9 | 92.3 / 90.2 | 94.2 / 93.1 |
|  | 86.3 / 85.8 | 90.1 / 88.9 | 93.0 / 91.9 |
| Scheme 1 / Baseline-Option 2 (160ms) | 85.1 / 83.2 | 88.6 / 86.3 | 90.3 / 91.4 |
|  | 82.9 / 82.7 | 85.1 / 84.1 | 88.3 / 88.2 |
| Scheme 2 / Baseline-Option 2 (80ms) | 85.3 / 85.2 | 88.9 / 88.6 | 90.6 / 90.8 |
|  | 84.1 / 84.9 | 87.9 / 88.1 | 90.1 / 90.2 |
| Scheme 2 / Baseline-Option 2 (160ms) | 81.1 / 80.1 | 84.0 / 82.6 | 88.0 / 87.9 |
|  | 78.3 / 76.4 | 82.3 / 79.8 | 87.1 / 86.3 |

4-4c) M=4

| Scheme | Beam prediction accuracy (%) |  |  |
| --- | --- | --- | --- |
|  | K =1 | K =2 | K =4 |
| Baseline-Option 1a | 100% |  |  |
| Scheme 1 / Baseline-Option 2 (80ms) | 87.9 / 88.6 | 90.4 / 91.3 | 92.3 / 94.3 |
|  | 87.2 / 88.1 | 90.1 / 90.8 | 92.0 / 93.8 |
|  | 86.9 / 87.5 | 89.6 / 90.2 | 91.3 / 93.3 |
|  | 86.4 / 86.9 | 89.2 / 89.8 | 90.6 / 92.8 |
| Scheme 1 / Baseline-Option 2 (160ms) | 86.1 / 85.2 | 88.9 / 88.3 | 90.3 / 90.1 |
|  | 85.7 / 84.0 | 88.0 / 86.4 | 89.4 / 88.7 |
|  | 85.0 / 83.2 | 87.3 / 85.6 | 89.2 / 86.9 |
|  | 84.3 / 81.8 | 86.1 / 84.7 | 87.6 / 86.0 |
| Scheme 2 / Baseline-Option 2 (80ms) | 79.3 / 80.4 | 85.9 / 86.0 | 88.3 / 89.6 |
|  | 78.5 / 80.1 | 85.2 / 85.5 | 87.6 / 88.7 |
|  | 77.8 / 78.3 | 84.6 / 85.2 | 87.4 / 88.0 |
|  | 77.0 / 76.1 | 84.0 / 84.5 | 87.0 / 87.3 |
| Scheme 2 / Baseline-Option 2 (160ms) | 70.0 / 70.1 | 77.3 / 77.2 | 82.2 / 81.4 |
|  | 69.3 / 68.2 | 76.8 / 76.3 | 82.0 / 80.0 |
|  | 69.0 / 67.0 | 75.9 / 75.4 | 81.1 / 79.2 |
|  | 68.3 / 65.8 | 75.6 / 72.1 | 79.7 / 78.0 |

Table 4-5, Beam prediction accuracy (%) with 1dB margin for Top-1 beam

| Scheme | M=1 | M=2 | M=4 |
| --- | --- | --- | --- |
| Baseline-Option 1a | 100.0 |  |  |
| Scheme 1 / Baseline-Option 2 (80ms) | 94.8 / 94.9 | 94.9 / 93.6 | 95.0 / 92.8 |
|  |  |  | 94.3 / 91.9 |
|  |  | 93.6 / 93.4 | 93.4 / 91.0 |
|  |  |  | 92.1 / 90.2 |
| Scheme 1 / Baseline-Option 2 (160ms) | 94.0 / 93.2 | 94.2 / 93.0 | 94.6 / 92.6 |
|  |  |  | 93.2 / 92.1 |
|  |  | 93.6 / 92.5 | 92.9 / 91.6 |
|  |  |  | 91.2 / 91.1 |
| Scheme 2/ Baseline-Option 2 (80ms) | 94.2 / 94.8 | 94.3 / 92.0 | 92.2 / 91.1 |
|  |  |  | 91.0 / 90.8 |
|  |  | 92.0 / 91.1 | 90.3 / 90.2 |
|  |  |  | 89.3 / 89.4 |
| Scheme 2/ Baseline-Option 2 (160ms) | 91.8 / 90.5 | 90.2 / 89.5 | 86.5 / 81.3 |
|  |  |  | 85.4 / 80.5 |
|  |  | 89.2 / 88.2 | 84.6 / 80.1 |
|  |  |  | 84.0 / 79.1 |

Observation 16: For AI based beam pair prediction in time domain, scheme 1 and scheme 2 can provide little performance gain (less than 3% on beam prediction accuracy of Top-1 beam) compared to baseline option 2: use the best beam of the last measurement instance as the Top-1 beam in future time instances with UE speed 30km/h and 500m ISD in Uma.

- Scheme 1’s assumption:

  - Assumption 1: Periodicity in T1=80ms, the number of measurement instance in T1=4, Periodicity in T2=80ms, the number of prediction instance in T2=1, 2, 4.

  - Assumption 2: Periodicity in T1=160ms, the number of measurement instance in T1=4, Periodicity in T2=160ms, the number of prediction instance in T2=1, 2, 4.

- Scheme 2’s assumption:

  - Assumption 1: Periodicity in T1=160/240/400ms, the number of measurement instance in T1=4, the number of prediction instance(s) Y for every number of measurement instance(s) =1/2/4.

  - Assumption 2: Periodicity in T1=320/480/800ms, the number of measurement instance in T1=4, the number of prediction instance(s) Y for every number of measurement instance(s) =1/2/4.

Conclusion

In this contribution, we provide our evaluation assumptions and results of AI/ML for beam management. Based on above analysis, we provide the following observations and proposals.

Spatial domain

Observation 1: AI based beam pair prediction in spatial domain can provide good performance by random set B with corresponding beam pair ID (about 55% beam prediction accuracy of Top-1 beam) as input or by fixed set B(about 74% beam prediction accuracy of Top-1 beam) when the ratio of set B and set A is 1/4.

Observation 2: For AI based beam pair prediction in spatial domain with fixed set B, the average predicted L1-RSRP difference between the ideal L1-RSRP of Top-1 predicted beam pair and the predicted L1-RSRP of the Top 1 predicted beam pair is less than 0.5 dB when the ratio of set B and set A is 1/4.

Observation 3: For AI based beam pair prediction in spatial domain with fixed set B, different fixed set B with continuous beam pair ID can provide almost same performance (about 59~64% beam prediction accuracy of Top-1 beam), but different set B in pre-configured set even with beam pair ID as input results in some performance loss (about 7~12% loss on beam prediction accuracy of Top-1 beam) when the ratio of set B and set A is 1/4.

Observation 4: For AI based beam pair prediction in spatial domain with fixed set B, different fixed set B with non-continuous beam pair ID can provide almost same performance (about 66~74% beam prediction accuracy of Top-1 beam), and different set B in pre-configured set of set B with beam pair ID or pattern ID as input can archive almost same performance ((about 66~72% beam prediction accuracy of Top-1 beam)) as same fixed set B when the ratio of set B and set A is 1/4.

Observation 5: For AI based beam pair prediction in spatial domain with fixed set B, fixed set B with non-continuous bam pair ID (about 66~74% beam prediction accuracy of Top-1 beam) can provide better performance than that of fixed set B with continuous beam pair ID (about 59~64% beam prediction accuracy of Top-1 beam) since non-continuous beam pair ID covers more Rx beams when the ratio of set B and set A is 1/4.

Observation 6: For AI based beam pair prediction in spatial domain with fixed set B, AI model trained by hybrid data of Uma and Umi for beam pair prediction in spatial domain can provide good generalization capability for Uma (about 9% performance loss on beam prediction accuracy of Top-1 beam). While AI model trained by data of Umi provide a little worse generalization capability for Uma inference (about 18% performance loss on beam prediction accuracy of Top-1 beam) when the ratio of set B and set A is 1/4.

Observation 7: For AI based beam pair prediction in spatial domain with fixed set B, AI model trained by hybrid data of different UE distribution for beam pair prediction in spatial domain can provide good generalization capability for UE distribution Option B, i.e., 100% outdoor (about 1% performance loss on beam prediction accuracy of Top-1 beam). While AI model trained by data of only UE distribution Option A, i.e., 20% outdoor provides a little worse generalization capability for UE distribution Option B (about 20% performance loss on beam prediction accuracy of Top-1 beam) when the ratio of set B and set A is 1/4.

Observation 8: For AI based beam pair prediction in spatial domain with pre-configured patterns of set B, AI model for beam pair prediction in spatial domain can provide good generalization capability among different number of UE Rx beam, e.g., AI model with more Rx beam number (8Rx) training by pre-configured patterns of set B can be applied for beam prediction of less Rx beam number (4Rx) with only about 2% performance loss on beam prediction accuracy of Top-1 beam when the ratio of set B and set A is 1/4.

Observation 9: For AI based DL Tx beam prediction in spatial domain with fixed set B, considering one specific Rx beam for model input results in acceptable performance (about 50~53.8% beam prediction accuracy of Top-1 beam) compared to exhaustive beam pair sweeping when the ratio of set B and set A is 1/4.

Observation 10: For AI based DL Tx beam prediction in spatial domain with fixed set B, considering the best Rx beam for model input provides good performance (about 83% beam prediction accuracy of Top-1 beam) compared to exhaustive beam pair sweeping.

Observation 11: For AI based DL Tx beam prediction in spatial domain with fixed set B, the average predicted L1-RSRP difference of the Top 1 predicted beam is less than 0.5 dB considering the best Rx beam for model input when the ratio of set B and set A is 1/4.

Observation 12: For AI based DL Tx beam prediction in spatial domain, different set B in pre-configured patterns of set B for DL Tx beam prediction results in only 3~9% performance loss on beam prediction accuracy of Top-1 beam compared to same fixed set B.

Observation 13: For AI based DL Tx beam prediction in spatial domain, AI model trained by hybrid data of different set B can provide acceptable generalization capability (about 15% performance loss on beam prediction accuracy of Top-1 beam). While AI model trained by data of set B1 provide a little worse generalization capability (about 18% performance loss on beam prediction accuracy of Top-1 beam) for different set B2.

Observation 14: For AI based DL Tx beam prediction in spatial domain, AI model trained by hybrid data of Uma and Umi can provide good generalization capability for Uma (about 9% performance loss on beam prediction accuracy of Top-1 beam). While AI model trained by data of only Umi provide a little worse generalization capability for Uma inference (about 16% performance loss on beam prediction accuracy of Top-1 beam).

Observation 15: For AI based DL Tx beam prediction in spatial domain, AI model trained by hybrid data of different UE distribution can provide good generalization capability for option 1 with 20% outdoor (about 12% performance loss on beam prediction accuracy of Top-1 beam). While AI model trained by data of Option 2 with 100% outdoor provides a little worse generalization capability for option 1 inference (about 17% performance loss on beam prediction accuracy of Top-1 beam).

Proposal 1: Support the best Rx beam for the ideal L1-RSRP of Top-1 predicted beam.

Proposal 2: Different fixed set B consists of L1-RSRP with more Rx beams for beam pair prediction should be considered with high priority.

Proposal 3: DL Tx beam prediction with specific Rx beam for model input can be supported considering the low RS overhead for model input.

Proposal 4: Support DL Tx beam prediction with the best Rx beam considering the high beam prediction accuracy.

Time domain

Observation 16: For AI based beam pair prediction in time domain, scheme 1 and scheme 2 can provide little performance gain (less than 3% on beam prediction accuracy of Top-1 beam) compared to baseline option 2: use the best beam of the last measurement instance as the Top-1 beam in future time instances with UE speed 30km/h and 500m ISD in Uma.

- Scheme 1’s assumption:

  - Assumption 1: Periodicity in T1=80ms, the number of measurement instance in T1=4, Periodicity in T2=80ms, the number of prediction instance in T2=1, 2, 4.

  - Assumption 2: Periodicity in T1=160ms, the number of measurement instance in T1=4, Periodicity in T2=160ms, the number of prediction instance in T2=1, 2, 4.

- Scheme 2’s assumption:

  - Assumption 1: Periodicity in T1=160/240/400ms, the number of measurement instance in T1=4, the number of prediction instance(s) Y for every number of measurement instance(s) =1/2/4.

  - Assumption 2: Periodicity in T1=320/480/800ms, the number of measurement instance in T1=4, the number of prediction instance(s) Y for every number of measurement instance(s) =1/2/4.

References

- RP-213599 New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- TR 38.802 V14.2.0

- RAN1-109 e-meeting chairman’s notes.

- RAN1-110 meeting chairman’s notes.

- RAN1-110b e-meeting chairman’s notes.

- RAN1-111 meeting chairman’s notes.

- RAN1-112 meeting chairman’s notes.

- RAN1-112bis e-meeting chairman’s notes.