3GPP TSG RAN WG1 #113			R1-2304917

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.9.2

Source:		Xiaomi

Title:	Discussion on the network verified location for NR NTN

Document for:	Discussion

Introduction

In RAN1#112b-e meeting, the following agreements were made on the network verified location [1]:

Agreement

For RTT determination in NTN, discuss further the accuracy, and reporting details of combinations of the following UE and gNB receive-transmit time difference measurements:

- Alt-1: UE Rx-Tx time difference based on Option 3 and gNB Rx-Tx time difference as defined in TS 38.215.

  - Note 1: The signaling method of UE Rx-Tx time difference definition option 1 is not precluded if Alt1 is adopted

- Alt-2: UE Rx-Tx time difference based on Option 2 and gNB Rx-Tx time difference as defined in TS 38.215.

  - Note 2: The LMF will use the time stamp of the PRS and the time stamp of SRS to calculate the time difference between the transmission of PRS and the reception of SRS

- Alt-3: UE Rx-Tx time difference based on Option 2 and gNB Rx-Tx time difference based on Option 4

FFS: One or multiple SRS can be used in determining the arrival time

FFS: Additional enhancement including additional information to be reported, if justified

Note 3: The impact of UE autonomous adjustment of TA (when applied) should be taken into account

Note 4: The gNB Rx-Tx time difference option in the above alternatives may need updates accordingly based on the outcome of discussion on reference point for the gNB Rx – Tx time difference

In this contribution, we present our views on the normative work for supporting the network verified location.

Discussion

For the multi-RTT based solution, it makes use of the UE Rx-Tx time difference measurements of downlink signals received from the same satellite at multiple different instants (i.e. DL-PRS), measured by the UE reported to the CN and the measured gNB Rx-Tx time difference measurements, of uplink signals transmitted from UE (i.e. UL-SRS) at multiple time instants.

To calculate the UE’s position, the RTT between the UE and the serving satellite is needed. The satellite ephemeris related information needs to be reported to the CN. Meanwhile, it is possible that the multiple measurement results are derived from different serving satellites especially considering the earth moving scenarios, and thus, the satellite ID may also be needed to report to the CN.

Proposal 1: The satellite ephemeris related information and satellite IDs need to be delivered to the CN.

It was extensively discussed in previous meeting on how to define the reference point for the gNB’s Rx_Tx timing difference. Basically two kinds of reference point were discussed:

- The “physical” reference point for gNB Rx – Tx time difference measurement

This is the reference point where the actual measurement is performed to derive the gNB Rx – Tx time difference. Obviously the reference point should be located in some network entity that is capable to do the signal processing, e.g, SRS detection which is the gNB for the transparent payload.  The legacy definition can be reused.

- The reference point for gNB Rx – Tx time difference calculation

This is the reference point where the reported gNB Rx – Tx time difference to CN is derived. The definition of the reference point determines how the RTT between the UE and satellite is calculated in the CN. The RTT between the UE and the gNB can be determined as:

RTTUE-sat=actual UE Rx-Tx time difference+actual gNB Rx-Tx time difference-

2*(feeder link delay)

Where the actual UE Rx – Tx time difference is corresponding to the RTT between the UE and UL synchronization point, while the actual gNB Rx – Tx time difference is corresponding to the RTT between the gNB and UL synchronization point. The actual UE Rx – Tx time difference can be derived by the reported UE Rx_Tx  time difference from the UE. The actual gNB Rx – Tx time difference can be derived by the reported gNB Rx_Tx  time difference  from the gNB with possible offset depending on the definition of the reference point.

The following three options for the reference point of gNB RX – Tx time difference calculation were discussed:

- Option 1: Onboard the satellite

With this option, the actual gNB Rx – Tx time difference is equal to the reported gNB Rx_Tx  time difference + feeder link delay.

- Option 2: The uplink time synchronization reference point

With this option, the actual gNB Rx – Tx time difference is equal to the reported gNB Rx_Tx  time difference + offset.

- Option 3: on the gNB

With this option, the actual gNB Rx – Tx time difference is equal to the reported gNB Rx_Tx  time difference

In our understanding, either options works. The difference of the options lay on the assistance information that need to be delivered to the CN. It is preferred to align the reference point and the actual measurement point.

Proposal 2: The “physical” reference point for gNB Rx – Tx time difference measurement is on the gNB.

Proposal 3: The reference point for the reported gNB Rx – Tx time difference is on the gNB.

RTT estimation error

One issue raised in the study is the RTT estimation error that might have impact on the positioning accuracy due to the movement of the satellite.

Figure 1: Illustration of the RTT estimation error

However, the RTT change due to the movement of the satellite is negligible compared to the target positioning accuracy as shown in the figure below:

Figure 2: RTT change due to the movement of the satellite

Observation 1: The RTT change due to the satellite movement is negligible.

Definition of the Rx-Tx time difference measurement

For the multi-RTT based solution, it makes use of the UE Rx-Tx time difference measurements of downlink signals received from the same satellite at multiple different instants (i.e. DL-PRS), measured by the UE reported to the CN and the measured gNB Rx-Tx time difference measurements, of uplink signals transmitted from UE (i.e. UL-SRS) at multiple time instants. In the legacy design, the UE Rx – Tx time difference is defined as TUE-RX – TUE-TX, where TUE-RX is the UE received timing of downlink subframe #i from a Transmission Point (TP), defined by the first detected path in time. TUE-TX is the UE transmit timing of uplink subframe #j that is closest in time to the subframe #i received from the TP. The reported range of the UE Rx – Tx time difference is -985024Tc to 985024Tc which results in a range of [-0.5ms, 0.5ms].

The estimated RTT can be calculated by the following equation as illustrated in the figure 3:

RTT=(tgNB_Rx-tPRS)-(tSRS-tUERx)

=(tgNBRx-tgNBTx+m)-(tUETx-tUERx+n)

=tgNB_RxTx+tUE_RxTx+m-n

Where, tgNB_RxTx and tUE_RxTx are the gNB Rx-Tx time difference measurements and UE Rx-Tx time difference measurements defined as in the legacy design respectively. m is the nearest integer value in the unit of milliseconds by rounding the time difference of transmit timing of a DL subframe and receive timing of a UL subframe. n is the nearest integer value in the unit of milliseconds by rounding the time difference of receive timing of a DL subframe and transmit timing of a UL subframe. Note that the RTT here is the round trip time between the UE and the serving gNB on the ground.

However, the CN is not aware of the actual gap between the gNB/UE’s actual Rx – Tx time difference. It is suggested to report the actual Rx – Tx time difference at the gNB and UE side. Based on the reporting, the CN can calculate the accurate UE’s position

Figure 3: Illustration of the multi-RTT solution

Several alternatives have been identified to determine the RTT in NTN:

- Alt-1: UE Rx-Tx time difference based on Option 3 and gNB Rx-Tx time difference as defined in TS 38.215.

In this alternative, the legacy R17 definition of UE Rx-Tx time difference is adopted for NTN with an offset. While for the gNB Rx-Tx time difference, additional offset may be also needed subject to the definition of the reference point discussed in previous section. As illustrated in the figure below assuming the reference point for the reported gNB Rx_Tx time difference is on the gNB:

Figure 3: Illustration of the Alt 1

In our understanding, the essential aspect of Alt 1 is that the spirit of decoupling the PRS and SRS transmissions is still kept. However, this may have impact on the accuracy if there is no association between the PRS and SRS as the UL timing at the UE side is kept changing with the movement of the serving satellite. The maximum timing error could be up to 29*64*Tc or 944ns in FR1 with 15kHz SCS which has large impact on the positioning accuracy. In order to resolve this issue, one alternative is to have the PRS and SRS transmission confined in a window so that the timing error can be maintained at an acceptable level. However, this essentially is similar to Alt 2 and Alt 3. Alternatively, the TA drift can be reported to compensate the TA changes.

- Alt-2: UE Rx-Tx time difference based on Option 2 and gNB Rx-Tx time difference as defined in TS 38.215.

In this alternative, UE report the information that indicates the time difference between the arrival time of a DL RS for positioning and the transmit time of an SRS. While the time difference between the transmission of PRS and the reception of SRS can be derived by the gNB reported Rx_Tx time difference and the time stamp.

Figure 4: Illustration of the Alt 2

- Alt-3: UE Rx-Tx time difference based on Option 2 and gNB Rx-Tx time difference based on Option 4

In this alternative, UE report the information that indicates the time difference between the arrival time of a DL RS for positioning and the transmit time of an SRS while gNB report the information that indicates the time difference between the transmit time of a DL RS for positioning and the arrival time of an SRS. In our understanding, the only difference between the Alt 2 and Alt 3 is how to report the actual time difference the transmission of PRS and the reception of SRS.

Figure 5: Illustration of the Alt 3

Proposal 4: For RTT determination in NTN, alt 3 can be supported.

Mirror error issue

For the mirror error issue where the estimated UE location might lie at two points that are symmetric around satellite’s orbit in timing-based positioning methods. Several options have been identified as the candidate solutions. Note that this is also discussed in RAN2 and it is assumed that in general the mirror point issue can be resolved by properly configuring neighbor cell measurement to UE, for example, measurement of two neighbor cells in the opposite side of a satellite beam. It can be decided in RAN2 if any enhancement related to the measurement is needed or not.

Proposal 5: The mirror error issue can be resolved by configuring the neighboring cell measurement. It’s up to RAN2 to decide any enhancement is needed or not.

DL-TDOA based solution

UE clock error

In NR network, the frequency error at the UE side shall be within ±0.1 PPM observed over a period of 1 ms. The time required to collect the RSTD results may be up to several seconds depending on the observation times. However as defined in RAN4, the maximum error due to the clock drift is +/- 16ns which results in tens of meters’ distance error for the measurement gap no larger than 160ms. UE can rely on the DL measurement to adjust its DL timing.

Observation 2: The UE clock error issue has little impact on the performance of the DL-TDOA based solution.

Latency issue

The latency issue was widely discussed during the study phase. The LS responses from SA1 [2] and SA2 [3] basically state that there is no real time restriction on the latency of the location verification, however, location verification be capable of being completed within a period of approximately 1-minute maximum and 30 seconds preferably is requested. One notable issue is that the latency required is significantly impacted by whether 2D or 3D position is assumed in the evaluation. The latency could be up to hundreds of seconds for 3D positioning while it is only around ten seconds for 2D positioning in our evaluation. In order to analyse the impact on the positioning accuracy due to the height division error, the evaluation results for DL-TDOA and multi-RTT based positioning error are illustrated in the below table. The detailed evaluation parameters can be found in the appendix.

Table 1: Horizontal positioning error for DL-TDOA based solution

| Measurement period/s | Maximum height error/m | Horizontal positioning error/m |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 50% | 67% | 80% | 90% | 95% |
| 5 | 0 | 403.41 | 585.71 | 772.37 | 1062.37 | 1462.51 |
|  | 100 | 406.90 | 599.37 | 788.62 | 1088.76 | 1464.11 |
|  | 500 | 479.93 | 720.67 | 921.75 | 1241.09 | 1576.18 |
| 10 | 0 | 100.19 | 141.98 | 199.99 | 264.84 | 346.55 |
|  | 100 | 112.87 | 157.94 | 209.43 | 289.63 | 347.05 |
|  | 500 | 257.52 | 366.07 | 466.35 | 581.69 | 736.16 |
| 15 | 0 | 42.73 | 62.96 | 84.82 | 118.01 | 160.12 |
|  | 100 | 65.72 | 89.79 | 117.11 | 153.75 | 186.83 |
|  | 500 | 262.85 | 340.51 | 430.63 | 560.07 | 669.96 |

Table 2: Horizontal positioning error for Multi-RTT based solution

| Measurement period/s | Maximum height error/m | Horizontal positioning error/m |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 50% | 67% | 80% | 90% | 95% |
| 1 | 0 | 6.09 | 9.82 | 14.59 | 633.36 | 7618.21 |
|  | 100 | 55.28 | 76.64 | 104.47 | 647.46 | 7685.26 |
|  | 500 | 266.93 | 374.86 | 491.85 | 744.23 | 7706.93 |
| 3 | 0 | 2.32 | 3.62 | 5.38 | 696.63 | 2600.94 |
|  | 100 | 55.11 | 75.74 | 103.64 | 657.10 | 2613.47 |
|  | 500 | 269.21 | 372.69 | 492.42 | 771.74 | 2669.77 |
| 5 | 0 | 1.51 | 2.29 | 3.40 | 696.38 | 1976.01 |
|  | 100 | 55.94 | 76.37 | 104.37 | 656.70 | 1989.29 |
|  | 500 | 270.76 | 376.81 | 492.57 | 744.64 | 2002.37 |

As observed from the results, the impact on the horizontal positioning accuracy due to the height division error (e.g, from 0 to 500m) is negligible. Thus, using 2D positioning is enough to verify the UE’s location and the latency expected is around ten seconds which meets the SA’s requirements.

Observation 3: The impact on the positioning accuracy due to the height division error is negligible.

Observation 4: The latency expected for the timing-based positioning solutions can meet the SA’s recommendation.

It is stated in the WID that multiple satellite in view by the UE may be considered if time allows. The claimed benefit is mainly on the latency aspect. However, as observed, the latency expected for the timing-based positioning solutions for single satellite scenario can meet the SA’s recommendation. We don’t think additional work on the multiple satellite scenario is needed in Rel-18.

Conclusions

In this contribution, we present the discussion on the network verified location. Based on our analysis, we have the following observations and proposals:

Observation 1: The RTT change due to the satellite movement is negligible.

Observation 2: The UE clock error issue has little impact on the performance of the DL-TDOA based solution.

Observation 3: The impact on the positioning accuracy due to the height division error is negligible.

Observation 4: The latency expected for the timing-based positioning solutions can meet the SA’s recommendation.

Proposal 1: The satellite ephemeris related information and satellite IDs need to be delivered to the CN.

Proposal 2: The “physical” reference point for gNB Rx – Tx time difference measurement is on the gNB.

Proposal 3: The reference point for the reported gNB Rx – Tx time difference is on the gNB.

Proposal 4: For RTT determination in NTN, alt 3 can be supported.

Proposal 5: The mirror error issue can be resolved by configuring the neighboring cell measurement. It’s up to RAN2 to decide any enhancement is needed or not.

Appendix

| Parameter | Description/Value |
| --- | --- |
| Scenarios | Rural, LOS |
| Satellite Orbit | 600km, 1200km |
| Satellite parameters | Reuse Set-1satellite parameters as in table 6.1.1.1-1/2 of TR38.821 |
| Channel model/ Delay spread | Based on section 6.7.2 of TR 38.811 |
| FR/Carrier frequency | FR1: 2GHz, S-band (n256). |
| BW | 10MHz (52 RB) |
| Subcarrier spacing, kHz | 15 |
| Number of satellite in view | 1 |
| Orbit inclination | 90° |
| UE type | Handheld terminal |
| UE related parameters | Handheld UE characteristics as in Table 6.1.1.1-3 of TR38.821 with update of polarization, Tx/Rx antenna gain, and antenna type and configuration as agreed under AI 9.12.1 |
| Positioning signals | PRS for DL-TDOA, PRS and SRS for Multi-RTT |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | PRS: comb-2 SRS: comb-2 |
| RS type of sequence/number of ports | PRS: Gold, 1 port SRS: ZC, 1 port |
| Number of symbols used per occasion | PRS: 2 SRS: 12 |
| number of occasions used per positioning estimate | 3 |
| Time window for measurement collection | [-T, 0, T], T=1, 3, 5, 10, 15, 20, 25, 30s |
| Interference modelling (ideal muting, or other) | Ideal muting |
| Reference Signal Transmission Bandwidth | PRS: 52 RB SRS: 52 RB |
| Reference point for timing measurement | Satellite |
| Description of positioning technique / applied positioning algorithm | Taylor series |
| UE speed | 3km/h |
| UE height error | Uniform distribution within [-H, H], H=0, 100, 500m |
| Performance metrics | Horizontal accuracy (UE 2D position accuracy) |

References

- 3GPP RAN1 112b-e meeting chairman-note, 3GPP TSG RAN WG1 #112bis-e, e-Meeting, April 17th – April 26th, 2023

- 3GPP S1-223539, Reply LS on Latency impact for NTN verified UE location, SA1, 3GPP TSG SA WG 1 Meeting #100, Toulouse, France, 14 – 18 November 2022

- 3GPP S2-2211199, LS Response on Latency impact for NTN verified UE location, 3GPP TSG SA WG2 Meeting #154, Toulouse, France, November 14-18 2022