3GPP TSG-RAN WG1 Meeting #113                                         R1-2304953

Incheon, Korea, May 22nd –May 26th, 2023

Agenda Item:	9.1.3.1

Source:	Lenovo

Title:	Discussion of increased number of orthogonal DMRS ports

Document for:	Discussion

# Introduction

In RAN1 #112b meeting, the following agreement is made for increasing the number of orthogonal DMRS ports for DL/UL MU-MIMO:

Agreement

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, for S-DCI based M-TRP,

- Support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data for Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for S-TRP, in addition to row 30 for 1CW in RAN1#112 agreement.

  - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is applied to the same row(s) for S-DCI based M-TRP.

Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

- All rows for Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH for S-TRP.

  - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is applied to the same row(s) for S-DCI based M-TRP.

- For one CW, add new row 68 in Table 7.3.1.2.2-2A-X.

  - For row 68, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

Table 7.3.1.2.2-2A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  |
| --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| … | … | … | … |
| 68 | 2 | 0,2,3 | 1 |

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

- All rows for Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH for S-TRP.

  - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is applied to the same row(s) for S-DCI based M-TRP.

- For one CW, add new row 60 in Table 7.3.1.2.2-3A-X.

  - For row 60, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

Table 7.3.1.2.2-3A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| … | … | … |
| 60 | 2 | 0,2,3 |

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

- All rows for Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH for S-TRP.

  - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is also applied to the same row(s) for S-DCI based M-TRP.

- For one CW, add new row 128 in Table 7.3.1.2.2-4A-X.

  - For row 128, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

Table 7.3.1.2.2-4A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  |
| --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| … | … | … | … |
| 128 | 2 | 0,2,3 | 1 |

Agreement

Confirm the following Working Assumption in RAN1#112 at least for NCB based PUSCH:

- To support PUSCH with rank = 5-8, support the following for enhancement of DMRS port allocation tables.

  - Option 1: Separate DMRS ports tables for rank 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current UL DMRS ports table).

    - FFS: whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI9.1.4.2 [or AI9.1.3.1].

- Note: The above Working Assumption for CB based PUSCH may be confirmed later.

Agreement

For 8Tx PUSCH, specify the factor related to PUSCH to PTRS power ratio per layer per RE () based on the following principles.

- Principle 1: When the ptrs-Power configures 01, the factor () is 10log10(L), where L is the total number of PUSCH layers.

- Principle 2: When the ptrs-Power configures 00, the factor () is determined as the following

  - Principle 2.1: For fully coherent TPMIs, the factor () is 10log10(L), where L is the total number of PUSCH layers.

  - Principle 2.2: For non-coherent TPMIs, the factor () is 10log10(Qp), where Qp is the number of PTRS ports scheduled to the UE.

  - Principle 2.3: For non-codebook PUSCH, the factor () is 10log10(Qp), where Qp is the number of PTRS ports scheduled to the UE.

  - FFS: The factor () for partial coherent TPMIs

Agreement

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case,

  - For 2 CWs,

      - Alt.1: Confirm the working assumption in RAN1#112 with modification (in red).

        - Alt.3-1: Support at least row 0-3 for 2 CWs in Table 4-0.

Table 4-0: DMRS ports for 2CWs.

| Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 2 | 0,1,2,3,8 |
| 1 | 2 | 0,1,2,3,8,10 |
| 2 | 2 | 0,1,2,3,8,9,10 |
| 3 | 2 | 0,1,2,3,8,9,10,11 |
| [4] | [2] | [0,1,2,3,10] |
| [5] | [2] | [0,1,8,2,3,10] |
| [6] | [2] | [0,1,8,2,3,10,11] |
| [7] | [2] | [0,1,8,9,2,3,10,11] |
| [8] | [2] | [0,2,3,8,9] |
| [9] | [2] | [0,1,2,3,8,9] |

FFS: Additional rows (rows 4~9) if there is technical justification.

Agreement

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case,

  - For 1 CW,

    - Do not support row 21-22

    - FFS: Whether to support row 23

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 21 | [2] | [8-10] |
| 22 | [2] | [8-11] |

Working Assumption

- Adopt Table 7.3.1.1.2-12B/13B/14B/15B/16B/17B/20B/21B/22B/23B to support signalling >4 ranks PUSCH with Rel-15 DMRS ports at least for full or non-coherent UL codebook based PUSCH and non-codebook based PUSCH.

- FFS: Whether/how some of bits in the antenna ports field can be reused for other purpose for >4 ranks PUSCH.

Table 7.3.1.1.2-12B: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-4 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-13B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,6 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-14B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-16B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=1, rank=5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-4 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-17B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=1, rank=6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-5 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-20B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-4 | 1 |
| 1 | 2 | 0,1,2,3,6 | 2 |
| 12-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-21B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-5 | 1 |
| 1 | 2 | 0,1,2,3,6,8 | 2 |
| 2-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-22B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8 | 2 |
| 1-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 1-31 | Reserved | Reserved | Reserved |

Agreement

For > 4 layers PUSCH with Rel.18 eType 1/eType 2 DMRS ports, support at least the same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH with Rel.18 eType 1/eType 2 DMRS ports at least for full or non-coherent UL codebook based PUSCH and non-codebook based PUSCH.

Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-2-X.

- FFS: For row 9-11, 24-30, 55-60, and 81-83 (if agreed) in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability.

- FFS: The total number of rows for eType1 DMRS ports with maxLength =2 for PDSCH at least for S-TRP case does not exceed 64.

Table 7.3.1.2.2-2-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | [0 | 2 | 0-4 | 2] |
| 1 | 1 | 1 | 1 | [1 | 2 | 0,1,2,3,4,6 | 2] |
| 2 | 1 | 0,1 | 1 | [2 | 2 | 0,1,2,3,4,5,6 | 2] |
| 3 | 2 | 0 | 1 | [3 | 2 | 0,1,2,3,4,5,6,7 | 2] |
| 4 | 2 | 1 | 1 | 4 | 2 | 0,1,2,3,8 | 1 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0,1,2,3,8,10 | 1 |
| 6 | 2 | 3 | 1 | 6 | 2 | 0,1,2,3,8,9,10 | 1 |
| 7 | 2 | 0,1 | 1 | 7 | 2 | 0,1,2,3,8,9,10,11 | 1 |
| [8 | 2 | 2,3 | 1] | [8 | 1 | 0,1,4,5,8 | 2] |
| [9 | 2 | 0-2 | 1] | [9 | 1 | 0,1,4,5,8,12 | 2] |
| [10 | 2 | 0-3 | 1] | [10 | 1 | 0,1,4,5,8,9,12 | 2] |
| [11 | 2 | 0,2 | 1] | [11 | 1 | 0,1,4,5,8,9,12,13 | 2] |
| 12 | 2 | 0 | 2 | [12 | 2 | 0,1,4,5,8 | 2] |
| 13 | 2 | 1 | 2 | [13 | 2 | 0,1,4,5,8,12 | 2] |
| 14 | 2 | 2 | 2 | [14 | 2 | 0,1,4,5,8,9,12 | 2] |
| 15 | 2 | 3 | 2 | [15 | 2 | 0,1,4,5,8,9,12,13 | 2] |
| 16 | 2 | 4 | 2 | [16 | 2 | 2,3,6,7,10 | 2] |
| 17 | 2 | 5 | 2 | [17 | 2 | 2,3,6,7,10,14 | 2] |
| 18 | 2 | 6 | 2 | [18 | 2 | 2,3,6,7,10,11,14 | 2] |
| 19 | 2 | 7 | 2 | [19 | 2 | 2,3,6,7,10,11,14,15 | 2] |
| 20 | 2 | 0,1 | 2 | [20 | 2 | 0,1, 2,3,10 | 1] |
| 21 | 2 | 2,3 | 2 | [21 | 2 | 0,1,8,2,3,10 | 1] |
| 22 | 2 | 4,5 | 2 | [22 | 2 | 0,1,8, 2,3,10,11 | 1] |
| 23 | 2 | 6,7 | 2 | [23 | 2 | 0,1,8,9,2,3,10,11 | 1] |
| [24 | 2 | 0,4 | 2] | [24 | 1 | 0,1,4,5,12 | 2] |
| [25 | 2 | 2,6 | 2] | [25 | 1 | 0,1,8,4,5,12 | 2] |
| [26 | 2 | 0,1,4 | 2] | [26 | 1 | 0,1,8,4,5,12,13 | 2] |
| [27 | 2 | 2,3,6 | 2] | [27 | 1 | 0,1,8,9,4,5,12,13 | 2] |
| [28 | 2 | 0,1,4,5 | 2] | [28 | 2 | 0,1,4,5,12 | 2] |
| [29 | 2 | 2,3,6,7 | 2] | [29 | 2 | 0,1,8,4,5,12 | 2] |
| [30 | 2 | 0,2,4,6 | 2] | [30 | 2 | 0,1,8,4,5,12,13 | 2] |
| 31 | 1 | 8 | 1 | [31 | 2 | 0,1,8,9,4,5,12,13 | 2] |
| 32 | 1 | 9 | 1 | [32 | 2 | 2,3,6,7,14 | 2] |
| 33 | 1 | 8,9 | 1 | [33 | 2 | 2,3,10,6,7,14 | 2] |
| 34 | 2 | 8 | 1 | [34 | 2 | 2,3,10,6,7,14,15 | 2] |
| 35 | 2 | 9 | 1 | [35 | 2 | 2,3,10,11,6,7,14,15 | 2] |
| 36 | 2 | 10 | 1 | [36 | 2 | 0,2,3,8,9 | 1] |
| 37 | 2 | 11 | 1 | [37 | 2 | 0,1,2,3,8,9 | 1] |
| 38 | 2 | 8,9 | 1 |  |  |  |  |
| 39 | 2 | 10,11 | 1 |  |  |  |  |
| [40 | 2 | 8-10 | 1] |  |  |  |  |
| [41 | 2 | 8-11 | 1] |  |  |  |  |
| [42 | 2 | 8,10 | 1] |  |  |  |  |
| 43 | 2 | 8 | 2 |  |  |  |  |
| 44 | 2 | 9 | 2 |  |  |  |  |
| 45 | 2 | 10 | 2 |  |  |  |  |
| 46 | 2 | 11 | 2 |  |  |  |  |
| 47 | 2 | 12 | 2 |  |  |  |  |
| 48 | 2 | 13 | 2 |  |  |  |  |
| 49 | 2 | 14 | 2 |  |  |  |  |
| 50 | 2 | 15 | 2 |  |  |  |  |
| 51 | 2 | 8,9 | 2 |  |  |  |  |
| 52 | 2 | 10,11 | 2 |  |  |  |  |
| 53 | 2 | 12,13 | 2 |  |  |  |  |
| 54 | 2 | 14,15 | 2 |  |  |  |  |
| [55 | 2 | 8,12 | 2] |  |  |  |  |
| [56 | 2 | 10,14 | 2] |  |  |  |  |
| [57 | 2 | 8,9,12 | 2] |  |  |  |  |
| [58 | 2 | 10,11,14 | 2] |  |  |  |  |
| [59 | 2 | 8,9,12,13 | 2] |  |  |  |  |
| [60 | 2 | 10,11,14,15 | 2] |  |  |  |  |
| 61 | 2 | 8,10,12,14 | 2 |  |  |  |  |
| 62 | 1 | 0,1,8 | 1 |  |  |  |  |
| 63 | 1 | 0,1,8,9 | 1 |  |  |  |  |
| 64 | 2 | 0,1,8 | 1 |  |  |  |  |
| 65 | 2 | 0,1,8,9 | 1 |  |  |  |  |
| 66 | 2 | 2,3,10 | 1 |  |  |  |  |
| 67 | 2 | 2,3,10,11 | 1 |  |  |  |  |
| [69 | 1 | 0,1,8 | 2] |  |  |  |  |
| [70 | 1 | 0,1,8,9 | 2] |  |  |  |  |
| [71 | 1 | 4,5,12 | 2] |  |  |  |  |
| [72 | 1 | 4,5,12,13 | 2] |  |  |  |  |
| [73 | 2 | 0,1,8 | 2] |  |  |  |  |
| [74 | 2 | 0,1,8,9 | 2] |  |  |  |  |
| [75 | 2 | 4,5,12 | 2] |  |  |  |  |
| [76 | 2 | 4,5,12,13 | 2] |  |  |  |  |
| [77 | 2 | 2,3,10 | 2] |  |  |  |  |
| [78 | 2 | 2,3,10,11 | 2] |  |  |  |  |
| [79 | 2 | 6,7,14 | 2] |  |  |  |  |
| [80 | 2 | 6,7,14,15 | 2] |  |  |  |  |
| [81 | 2 | 5,8,9 | 2] |  |  |  |  |
| [82 | 2 | 7,10,11 | 2] |  |  |  |  |
| [83 | 2 | 7,12,13 | 2] |  |  |  |  |

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-3-X.

- FFS: For rows 9, 10, 20-23, 33, 34, 44-46, 60-62 (if agreed) in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability.

Table 7.3.1.2.2-3-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=1

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 3 | 0-4 |
| 1 | 1 | 1 | 1 | 3 | 0-5 |
| 2 | 1 | 0,1 | [2 | 3 | 12-16] |
| 3 | 2 | 0 | [3 | 3 | 12-17] |
| 4 | 2 | 1 | 4 | 2 | 0,1,2,3,12 |
| 5 | 2 | 2 | 5 | 2 | 0,1,2,3,12,14 |
| 6 | 2 | 3 | 6 | 2 | 0-3,12-14 |
| 7 | 2 | 0,1 | 7 | 2 | 0-3,12-15 |
| 8 | 2 | 2,3 | [8 | 3 | 0,1,2,3,12] |
| [9 | 2 | 0-2] | [9 | 3 | 0,1,2,3,12,14] |
| [10 | 2 | 0-3] | [10 | 3 | 0-3,12-14] |
| 11 | 3 | 0 | [11 | 3 | 0-3,12-15] |
| 12 | 3 | 1 | [12 | 2 | 0,2,3,12,13] |
| 13 | 3 | 2 | [13 | 2 | 0,1,2,3,14] |
| 14 | 3 | 3 | [14 | 2 | 0,1,12,2,3,14] |
| 15 | 3 | 4 | [15 | 2 | 0,1,12,2,3,14,15] |
| 16 | 3 | 5 | [16 | 2 | 0,1,12,13,2,3,14,15] |
| 17 | 3 | 0,1 | [17 | 3 | 0,1,2,3,14] |
| 18 | 3 | 2,3 | [18 | 3 | 0,1,12,2,3,14] |
| 19 | 3 | 4,5 | [19 | 3 | 0,1,12,2,3,14,15] |
| [20 | 3 | 0-2] | [20 | 3 | 0,1,12,13,2,3,14,15] |
| [21 | 3 | 3-5] |  |  |  |
| [22 | 3 | 0-3] |  |  |  |
| [23 | 2 | 0,2] |  |  |  |
| 24 | 1 | 12 |  |  |  |
| 25 | 1 | 13 |  |  |  |
| 26 | 1 | 12,13 |  |  |  |
| 27 | 2 | 12 |  |  |  |
| 28 | 2 | 13 |  |  |  |
| 29 | 2 | 14 |  |  |  |
| 30 | 2 | 15 |  |  |  |
| 31 | 2 | 12,13 |  |  |  |
| 32 | 2 | 14,15 |  |  |  |
| [33 | 2 | 12-14] |  |  |  |
| [34 | 2 | 12-15] |  |  |  |
| 35 | 3 | 12 |  |  |  |
| 36 | 3 | 13 |  |  |  |
| 37 | 3 | 14 |  |  |  |
| 38 | 3 | 15 |  |  |  |
| 39 | 3 | 16 |  |  |  |
| 40 | 3 | 17 |  |  |  |
| 41 | 3 | 12,13 |  |  |  |
| 42 | 3 | 14,15 |  |  |  |
| 43 | 3 | 16,17 |  |  |  |
| [44 | 3 | 12-14] |  |  |  |
| [45 | 3 | 15-17] |  |  |  |
| [46 | 3 | 12-15] |  |  |  |
| [47 | 2 | 12,14] |  |  |  |
| 48 | 1 | 0,1,12 |  |  |  |
| 49 | 1 | 0,1,12,13 |  |  |  |
| 50 | 2 | 0,1,12 |  |  |  |
| 51 | 2 | 0,1,12,13 |  |  |  |
| 52 | 2 | 2,3,14 |  |  |  |
| 53 | 2 | 2,3,14,15 |  |  |  |
| 54 | 3 | 0,1,12 |  |  |  |
| 55 | 3 | 0,1,12,13 |  |  |  |
| 56 | 3 | 2,3,14 |  |  |  |
| 57 | 3 | 2,3,14,15 |  |  |  |
| 58 | 3 | 4,5,16 |  |  |  |
| 59 | 3 | 4,5,16,17 |  |  |  |
| [60 | 3 | 13,15,17] |  |  |  |
| [61 | 3 | 13,15] |  |  |  |
| [62 | 2 | 13,15] |  |  |  |

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-4-X.

- FFS: For rows 9, 10, 20-23, 42-47, 67, 68, 78-80, 100-105, and 153-158 (if agreed) in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability.

Table 7.3.1.2.2-4-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=2

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | [0 | 3 | 0-4 | 1] |
| 1 | 1 | 1 | 1 | [1 | 3 | 0-5 | 1] |
| 2 | 1 | 0,1 | 1 | [2 | 2 | 0,1,2,3,6 | 2] |
| 3 | 2 | 0 | 1 | [3 | 2 | 0,1,2,3,6,8 | 2] |
| 4 | 2 | 1 | 1 | [4 | 2 | 0,1,2,3,6,7,8 | 2] |
| 5 | 2 | 2 | 1 | [5 | 2 | 0,1,2,3,6,7,8,9 | 2] |
| 6 | 2 | 3 | 1 | 6 | 2 | 0,1,2,3,12 | 1 |
| 7 | 2 | 0,1 | 1 | 7 | 2 | 0-3,12,14 | 1 |
| 8 | 2 | 2,3 | 1 | 8 | 2 | 0-3,12-14 | 1 |
| [9 | 2 | 0-2 | 1] | 9 | 2 | 0-3,12-15 | 1 |
| [10 | 2 | 0-3 | 1] | [10 | 3 | 0,1,2,3,12 | 1] |
| 11 | 3 | 0 | 1 | [11 | 3 | 0-3,12,14 | 1] |
| 12 | 3 | 1 | 1 | [12 | 3 | 0-3,12-14 | 1] |
| 13 | 3 | 2 | 1 | [13 | 3 | 0-3,12-15 | 1] |
| 14 | 3 | 3 | 1 | [14 | 1 | 0,1,6,7,12 | 2] |
| 15 | 3 | 4 | 1 | [15 | 1 | 0,1,6,7,12,18 | 2] |
| 16 | 3 | 5 | 1 | [16 | 1 | 0,1,6,7,12,13,18 | 2] |
| 17 | 3 | 0,1 | 1 | [17 | 1 | 0,1,6,7,12,13,18,19 | 2] |
| 18 | 3 | 2,3 | 1 | [18 | 2 | 0,1,6,7,12 | 2] |
| 19 | 3 | 4,5 | 1 | [19 | 2 | 0,1,6,7,12,18 | 2] |
| [20 | 3 | 0-2 | 1] | [20 | 2 | 0,1,6,7,12,13,18 | 2] |
| [21 | 3 | 3-5 | 1] | [21 | 2 | 0,1,6,7,12,13,18,19 | 2] |
| [22 | 3 | 0-3 | 1] | [22 | 2 | 2,3,8,9,14 | 2] |
| [23 | 2 | 0,2 | 1] | [23 | 2 | 2,3,8,9,14,20 | 2] |
| 24 | 3 | 0 | 2 | [24 | 2 | 2,3,8,9,14,15,20 | 2] |
| 25 | 3 | 1 | 2 | [25 | 2 | 2,3,8,9,14,15,20,21 | 2] |
| 26 | 3 | 2 | 2 | [26 | 3 | 0,1,6,7,12 | 2] |
| 27 | 3 | 3 | 2 | [27 | 3 | 0,1,6,7,12,18 | 2] |
| 28 | 3 | 4 | 2 | [28 | 3 | 0,1,6,7,12,13,18 | 2] |
| 29 | 3 | 5 | 2 | [29 | 3 | 0,1,6,7,12,13,18,19 | 2] |
| 30 | 3 | 6 | 2 | [30 | 3 | 2,3,8,9,14 | 2] |
| 31 | 3 | 7 | 2 | [31 | 3 | 2,3,8,9,14,20 | 2] |
| 32 | 3 | 8 | 2 | [32 | 3 | 2,3,8,9,14,15,20 | 2] |
| 33 | 3 | 9 | 2 | [33 | 3 | 2,3,8,9,14,15,20,21 | 2] |
| 34 | 3 | 10 | 2 | [34 | 3 | 4,5,10,11,16 | 2] |
| 35 | 3 | 11 | 2 | [35 | 3 | 4,5,10,11,16,22 | 2] |
| 36 | 3 | 0,1 | 2 | [36 | 3 | 4,5,10,11,16,17,22 | 2] |
| 37 | 3 | 2,3 | 2 | [37 | 3 | 4,5,10,11,16,17,22,23 | 2] |
| 38 | 3 | 4,5 | 2 | [38 | 2 | 0,1,2,3,14 | 1] |
| 39 | 3 | 6,7 | 2 | [39 | 2 | 0,1,12,2,3,14 | 1] |
| 40 | 3 | 8,9 | 2 | [40 | 2 | 0,1,12,2,3,14,15 | 1] |
| 41 | 3 | 10,11 | 2 | [41 | 2 | 0,1,12,13,2,3,14,15 | 1] |
| [42 | 3 | 0,1,6 | 2] | [42 | 3 | 0,1,2,3,14 | 1] |
| [43 | 3 | 2,3,8 | 2] | [43 | 3 | 0,1,12,2,3,14 | 1] |
| [44 | 3 | 4,5,10 | 2] | [44 | 3 | 0,1,12,2,3,14,15 | 1] |
| [45 | 3 | 0,1,6,7 | 2] | [45 | 3 | 0,1,12,13,2,3,14,15 | 1] |
| [46 | 3 | 2,3,8,9 | 2] | [46 | 1 | 0,1,6,7,18 | 2] |
| [47 | 3 | 4,5,10,11 | 2] | [47 | 1 | 0,1,12,6,7,18 | 2] |
| 48 | 1 | 0 | 2 | [48 | 1 | 0,1,12,6,7,18,19 | 2] |
| 49 | 1 | 1 | 2 | [49 | 1 | 0,1,12,13,6,7,18,19 | 2] |
| 50 | 1 | 6 | 2 | [50 | 2 | 0,1,6,7,18 | 2] |
| 51 | 1 | 7 | 2 | [51 | 2 | 0,1,12,6,7,18 | 2] |
| 52 | 1 | 0,1 | 2 | [52 | 2 | 0,1,12,6,7,18,19 | 2] |
| 53 | 1 | 6,7 | 2 | [53 | 2 | 0,1,12,13,6,7,18,19 | 2] |
| 54 | 2 | 0,1 | 2 | [54 | 2 | 2,3,8,9,20 | 2] |
| 55 | 2 | 2,3 | 2 | [55 | 2 | 2,3,14,8,9,20 | 2] |
| 56 | 2 | 6,7 | 2 | [56 | 2 | 2,3,14,8,9,20,21 | 2] |
| 57 | 2 | 8,9 | 2 | [57 | 2 | 2,3,14,15,8,9,20,21 | 2] |
| 58 | 1 | 12 | 1 | [58 | 3 | 0,1,6,7,18 | 2] |
| 59 | 1 | 13 | 1 | [59 | 3 | 0,1,12,6,7,18 | 2] |
| 60 | 1 | 12,13 | 1 | [60 | 3 | 0,1,12,6,7,18,19 | 2] |
| 61 | 2 | 12 | 1 | [61 | 3 | 0,1,12,13,6,7,18,19 | 2] |
| 62 | 2 | 13 | 1 | [62 | 3 | 2,3,8,9,20 | 2] |
| 63 | 2 | 14 | 1 | [63 | 3 | 2,3,14,8,9,20 | 2] |
| 64 | 2 | 15 | 1 | [64 | 3 | 2,3,14,8,9,20,21 | 2] |
| 65 | 2 | 12,13 | 1 | [65 | 3 | 2,3,14,15,8,9,20,21 | 2] |
| 66 | 2 | 14,15 | 1 | [66 | 3 | 4,5,10,11,22 | 2] |
| [67 | 2 | 12-14 | 1] | [67 | 3 | 4,5,16,10,11,22 | 2] |
| [68 | 2 | 12-15 | 1] | [68 | 3 | 4,5,16,10,11,22,23 | 2] |
| 69 | 3 | 12 | 1 | [69 | 3 | 4,5,16,17,10,11,22,23 | 2] |
| 70 | 3 | 13 | 1 |  |  |  |  |
| 71 | 3 | 14 | 1 |  |  |  |  |
| 72 | 3 | 15 | 1 |  |  |  |  |
| 73 | 3 | 16 | 1 |  |  |  |  |
| 74 | 3 | 17 | 1 |  |  |  |  |
| 75 | 3 | 12,13 | 1 |  |  |  |  |
| 76 | 3 | 14,15 | 1 |  |  |  |  |
| 77 | 3 | 16,17 | 1 |  |  |  |  |
| [78 | 3 | 12-14 | 1] |  |  |  |  |
| [79 | 3 | 15-17 | 1] |  |  |  |  |
| [80 | 3 | 12-15 | 1] |  |  |  |  |
| [81 | 2 | 12,14 | 1] |  |  |  |  |
| 82 | 3 | 12 | 2 |  |  |  |  |
| 83 | 3 | 13 | 2 |  |  |  |  |
| 84 | 3 | 14 | 2 |  |  |  |  |
| 85 | 3 | 15 | 2 |  |  |  |  |
| 86 | 3 | 16 | 2 |  |  |  |  |
| 87 | 3 | 17 | 2 |  |  |  |  |
| 88 | 3 | 18 | 2 |  |  |  |  |
| 89 | 3 | 19 | 2 |  |  |  |  |
| 90 | 3 | 20 | 2 |  |  |  |  |
| 91 | 3 | 21 | 2 |  |  |  |  |
| 92 | 3 | 22 | 2 |  |  |  |  |
| 93 | 3 | 23 | 2 |  |  |  |  |
| 94 | 3 | 12,13 | 2 |  |  |  |  |
| 95 | 3 | 14,15 | 2 |  |  |  |  |
| 96 | 3 | 16,17 | 2 |  |  |  |  |
| 97 | 3 | 18,19 | 2 |  |  |  |  |
| 98 | 3 | 20,21 | 2 |  |  |  |  |
| 99 | 3 | 22,23 | 2 |  |  |  |  |
| [100 | 3 | 12,13,18 | 2] |  |  |  |  |
| [101 | 3 | 14,15,20 | 2] |  |  |  |  |
| [102 | 3 | 16,17,22 | 2] |  |  |  |  |
| [103 | 3 | 12,13,18,19 | 2] |  |  |  |  |
| [104 | 3 | 14,15,20,21 | 2] |  |  |  |  |
| [105 | 3 | 16,17,22,23 | 2] |  |  |  |  |
| 106 | 1 | 12 | 2 |  |  |  |  |
| 107 | 1 | 13 | 2 |  |  |  |  |
| 108 | 1 | 18 | 2 |  |  |  |  |
| 109 | 1 | 19 | 2 |  |  |  |  |
| 110 | 1 | 12,13 | 2 |  |  |  |  |
| 111 | 1 | 18,19 | 2 |  |  |  |  |
| 112 | 2 | 12,13 | 2 |  |  |  |  |
| 113 | 2 | 14,15 | 2 |  |  |  |  |
| 114 | 2 | 18,19 | 2 |  |  |  |  |
| 115 | 2 | 20,21 | 2 |  |  |  |  |
| 116 | 1 | 0,1,12 | 1 |  |  |  |  |
| 117 | 1 | 0,1,12,13 | 1 |  |  |  |  |
| 118 | 2 | 0,1,12 | 1 |  |  |  |  |
| 119 | 2 | 0,1,12,13 | 1 |  |  |  |  |
| 120 | 2 | 2,3,14 | 1 |  |  |  |  |
| 121 | 2 | 2,3,14,15 | 1 |  |  |  |  |
| 122 | 3 | 0,1,12 | 1 |  |  |  |  |
| 123 | 3 | 0,1,12,13 | 1 |  |  |  |  |
| 124 | 3 | 2,3,14 | 1 |  |  |  |  |
| 125 | 3 | 2,3,14,15 | 1 |  |  |  |  |
| 126 | 3 | 4,5,16 | 1 |  |  |  |  |
| 127 | 3 | 4,5,16,17 | 1 |  |  |  |  |
| [129 | 1 | 0,1,12 | 2] |  |  |  |  |
| [130 | 1 | 0,1,12,13 | 2] |  |  |  |  |
| [131 | 1 | 6,7,18 | 2] |  |  |  |  |
| [132 | 1 | 6,7,18,19 | 2] |  |  |  |  |
| [133 | 2 | 0,1,12 | 2] |  |  |  |  |
| [134 | 2 | 0,1,12,13 | 2] |  |  |  |  |
| [135 | 2 | 6,7,18 | 2] |  |  |  |  |
| [136 | 2 | 6,7,18,19 | 2] |  |  |  |  |
| [137 | 2 | 2,3,14 | 2] |  |  |  |  |
| [138 | 2 | 2,3,14,15 | 2] |  |  |  |  |
| [139 | 2 | 8,9,20 | 2] |  |  |  |  |
| [140 | 2 | 8,9,20,21 | 2] |  |  |  |  |
| [141 | 3 | 0,1,12 | 2] |  |  |  |  |
| [142 | 3 | 0,1,12,13 | 2] |  |  |  |  |
| [143 | 3 | 6,7,18 | 2] |  |  |  |  |
| [144 | 3 | 6,7,18,19 | 2] |  |  |  |  |
| [145 | 3 | 2,3,14 | 2] |  |  |  |  |
| [146 | 3 | 2,3,14,15 | 2] |  |  |  |  |
| [147 | 3 | 8,9,20 | 2] |  |  |  |  |
| [148 | 3 | 8,9,20,21 | 2] |  |  |  |  |
| [149 | 3 | 4,5,16 | 2] |  |  |  |  |
| [150 | 3 | 4,5,16,17 | 2] |  |  |  |  |
| [151 | 3 | 10,11,22 | 2] |  |  |  |  |
| [152 | 3 | 10,11,22,23 | 2] |  |  |  |  |
| [153 | 3 | 7,12,13 | 2] |  |  |  |  |
| [154 | 3 | 9,14,15 | 2] |  |  |  |  |
| [155 | 3 | 11,16,17 | 2] |  |  |  |  |
| [156 | 3 | 9,18,19 | 2] |  |  |  |  |
| [157 | 3 | 18,19,20 | 2] |  |  |  |  |
| [158 | 3 | 21,22,23 | 2] |  |  |  |  |

Conclusion

No consensus to support MAC CE based switching between Rel.15 DMRS ports and Rel.18 DMRS ports for PDSCH

Agreement

For Rel.18 eType1/eType2 DMRS ports with maxLength=1/2 for PDSCH/PUSCH, if Rel.18 eType1/eType2 DMRS ports is configured by RRC, the DCI size of antenna ports field in DCI format 1_1/1_2/0_1/0_2 is increased by at least 1-bit from Rel.17.

- Note: it does not preclude future possibility to support more than 1-bit, if RAN1 agree the necessity.

Agreement

For RAN1#112 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH.

- Support row 7 for rank2, row1 for rank3, row 1 for rank4.

Table 7.3.1.1.2-9-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 7 | 2 | 9,11 |

Table 7.3.1.1.2-10-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 1 | 2 | 8-10 |

Table 7.3.1.1.2-11-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 1 | 2 | 8-11 |

Agreement

For two PTRS ports for partial/non-coherent PUSCH, PTRS-DMRS association for PUSCH with up to 8 layers is down selected from the following.

- Alt.1: The size of PTRS-DMRS association field is 4-bit in DCI format 0_1/0_2.

Table 1: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |
| 2 | 3rd DMRS port which shares PTRS port 0 | 2 | 3rd DMRS port which shares PTRS port 1 |
| 3 | 4th DMRS port which shares PTRS port 0 | 3 | 4th DMRS port which shares PTRS port 1 |

- Alt.2: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2.

  - The CW with the higher MCS is selected in case of two CWs.

  - If the MCS is the same for two CWs, the PTRS port is associated with the first CW.

Table 2: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |

- Alt.3: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2.

  - For PUSCH with rank 5-8, 2-bit of antenna ports field is reused in addition to 2-bit PTRS-DMRS association in DCI format 0_1/0_2, and total 4-bit is used for PTRS-DMRS association.

Table 1: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |
| 2 | 3rd DMRS port which shares PTRS port 0 | 2 | 3rd DMRS port which shares PTRS port 1 |
| 3 | 4th DMRS port which shares PTRS port 0 | 3 | 4th DMRS port which shares PTRS port 1 |

- Alt.4: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2.

Table 2: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |

Conclusion

For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports,

- For PUSCH, there is no restriction.

Agreement

For partial/non-coherent PUSCH with rank=5-8 transmission (i.e. non of the CWs is disabled) with one PTRS port, PTRS-DMRS association for PUSCH is the following.

- The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2.

  - The CW with the higher MCS is selected in case of two CWs.

    - Note: in case of PUSCH retransmission, the initial MCS is used for CW selection.

  - If the MCS is the same for two CWs, the PTRS port is associated with the first CW.

Table 7.3.1.1.2-25: PTRS-DMRS association for UL PTRS port 0

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port with the CW |
| 1 | 2nd scheduled DMRS port with the CW |
| 2 | 3rd scheduled DMRS port with the CW |
| 3 | 4th scheduled DMRS port with the CW |

Conclusion

For “The CW with the higher MCS” in RAN1#112 agreement of PTRS-DMRS association field for full-coherent PUSCH with rank=5~8 PUSCH with one port PTRS, following is clarified.

- Note: in case of PUSCH retransmission, the initial MCS is used for CW selection.

Agreement

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, for S-DCI based M-TRP,

- Support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data for Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for S-TRP, in addition to row 30 for 1CW in RAN1#112 agreement.

  - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is applied to the same row(s) for S-DCI based M-TRP.

In this contribution, we provide our views on increasing number of orthogonal DMRS ports for UL/DL MU-MIMO and 8 Tx UL SU-MIMO.

# Discussion on increasing the number of orthogonal DMRS ports

## Increasing the number of orthogonal DMRS ports for DL/UL MU-MIMO

In RAN1 #110b meeting, the working assumption is confirmed that at least Opt.1 (introduce new larger FD-OCC length (e.g. 4 or 6)) is supported to increase the number of DMRS ports for PDSCH/PUSCH. Based on the new agreements in recent 4 RAN1 meetings, we make further discussion on the design for enhanced FD-OCC DMRS.

Orphan RE/RB

In RAN1#110b meeting, the method to resolve the orphan RB issue is agreed except for two FFS. The first FFS is on scheduling restriction of different UEs in case of MU-MIMO. We think if 1) The number of consecutively scheduled PRBs for PDSCH is even, and 2)  The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even, are satisfied by each co-scheduled UE, the offset between starting RB of co-scheduled UEs is already even. Therefore, we support no additional restriction for MU-MIMO. The second FFS is related with M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme. For these two schemes, the first ⌈nPRB/2 ⌉ PRBs are transmitted from a TRP and the remaining ⌊nPRB/2 ⌋ PRBs are transmitted from a another TRP, wherein the nPRB is the total number of allocated PRBs for a UE.  Even if the number of scheduled RBs of a PDSCH/PUSCH is even, there may also be orphan RB issue for each TRP. Therefore, the restriction shall be introduced for each TRP, that is the number of consecutively scheduled PRBs for PDSCH is even for each TRP. Therefore we propose:

Proposal 1: For FD-OCC length 4 in Rel.18 eType 1 DMRS for PDSCH:

- No restriction on scheduling of different UEs is needed in case of MU-MIMO

- The number of consecutively scheduled PRBs for PDSCH transmitted from each TRP is even.

DMRS port indication

For demodulation of PDSCH/PUSCH with SU-MIMO or MU-MIMO transmission, the used DMRS port index is required to be indicated to UE. For legacy design, the number of DMRS CDM group(s) without data, DMRS port(s), number of front-load symbols are indicated by DCI signalling. With increasing the number of orthogonal DMRS ports, the enhanced signalling is required to indicate the used DMRS port(s). In RAN1 #111, it was agreed three categories of port combination can be indicated, where the port combination of Cat.1 consists of legacy port indexes; the port combination of Cat.2 consists of new port indexes; the port combination of Cat.3 consists of legacy port indexes and new port indexes.

The PDSCH DMRS port indication table for eType1 DMRS with maxLength=1 in single TRP case is agreed except that some entries in bracket are still FFS. In RAN1 #112 and RAN1 #112b, it is agree to include row 0-2, 9-11, 12-14, 24-25 and not to include row 21-22, but there is still FFS for row 23. For row 23, it is proposed to support MU-MIMO with layer combination 3+3+2. In our view, it can provide scheduling flexibility for gNb. However, there is concern on realization complexity from chip vendor. Thus, we support the compromised proposal that support row 23 in case of 1 codeword as UE capability.

Proposal 2: Introduce UE capability for supporting row 21 in case of 1 codeword of PDSCH DMRS port indication table for eType1 DMRS with maxLength=1.

For antenna ports indication table of Rel.18 eType1 PDSCH DMRS ports with maxLength = 1, it is still FFS for row 4-9 for 2 codewords in agreed Table 4-0, where more layer combination candidates are proposed for more flexibility in case of 5-8 layers. For some candidates of layer combination, the layers from one CW is designed to map into the same CDM group. However, we do not see the clear performance benefit by introducing these additional layer combination. On the other hand, it will increase the scheduling complexity on account of increased number of DMRS port combination candidates. Thus, we prefer to support only row 0-3 in agreed Table 4-0.

Proposal 3: Do not support row 4-9 for 2 codewords of PDSCH DMRS port indication table for eType1 DMRS with maxLength=1.

For PDSCH DMRS port indication table for eType1 or eType2 DMRS in case of single TRP, the port combination of Cat.3 can be further discussed in case of 2 front-load DMRS symbols. The similar DMRS overhead can be achieved by length 4 FD OCC and combination of length 2 TD OCC and length 2 FD OCC. As an example to support 4 layers MU-MIMO, similar overhead is achieved for DMRS port combination {0, 1, 8, 9} or {0, 1, 4, 5}. Therefore, we prefer not to introduce rows to indicate such kind of Cat.3 port combination in case of 2 front-load DMRS symbols. In detail, we think there is no need to support row 69-80 in agreed Table 7.3.1.2.2-2-X and row 129-152 in agreed Table 7.3.1.2.2-4-X.

Proposal 4: Do not support row 69-80 in agreed Table 7.3.1.2.2-2-X and row 129-152 in agreed Table 7.3.1.2.2-4-X.

In RAN1 #112b, it was agreed that all rows in the DMRS port indication table for Rel.18 PDSCH DMRS port for S-TRP are reused as those for M-TRP. Moreover, additional DMRS port combination {0, 2, 3} is introduced for M-TRP. To make good tradeoff between scheduling flexibility and DCI signalling overhead, we propose to make optimized design to guarantee the same DCI signalling overhead for DMRS port indication for S-TRP and M-TRP with the same DMRS type and maxLength. For example, only 1 additional DCI signalling bit can be introduced based on newly designed PDSCH DMRS port indication table for S-TRP and M-TRP relate to legacy designed PDSCH DMRS port indication table for S-TRP and M-TRP, respectively.

Proposal 5: Strive to design DMRS port indication table with the same DMRS type and maxLength for S-TRP and M-TRP with same DCI signalling overhead.

MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports

In RAN1#110 meeting, it is agreed that MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports are supported. It can be used to improve scheduling flexibility. With MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports, we need design DMRS to guarantee good channel estimation performance for both legacy Rel-15/16/17 UEs and new Rel-18 UEs. For MU-MIMO with different CDM groups, there is no MU-MIMO scheduling restriction of PUSCH/PDSCH since DMRS ports in different CDM groups are orthogonal. More discussion is further made for MU-MIMO within a CDM group. In RAN1 #112b, it is further concluded that there is no restriction for PUSCH MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports. For PDSCH MU-MIMO within a CDM group, it is still in discussion. In detail, three kind of DMRS port multiplexing for MU-MIMO are discussed separately. For Cat.1, it is multiplexed between Rel-18 legacy ports (eType1: ports 1000-1007, eType2: ports 1000-1011) from Rel.18 UE and Rel.15 DMRS ports from Rel.15/18 UE within a CDM group. On account of compatible design for Rel-18 legacy DMRS ports, there is no need for introducing additional restriction on top of legacy MU-MIMO restriction. For Cat.2, it is multiplexed between Rel-18 new ports  (eType1: ports 1008-1015, eType2: ports 1012-1023) from Rel.18 UE and Rel.15 DMRS ports within a CDM group from Rel.15 UE. If it happens, Rel.15 DMRS ports from Rel.15 UE have to suffer interference from Rel.18 new ports and this will degrade channel estimation performance of Rel.15 DMRS ports. Thus, we think gNb should avoid such scheduling by implementation and UE does not expect such MU-MIMO within a CDM group. For Cat.3, it is multiplexed between Rel-18 new ports  (eType1: ports 1008-1015, eType2: ports 1012-1023) from one Rel.18 UE and Rel.15 DMRS ports from another Rel.18 UE within a CDM group. Some companies propose to introduce additional signalling to indicated whether existed Rel.18 new ports to make length 4 FD-OCC dispreading for Rel.18 UE with Rel.15 DMRS ports. In our view, the additional signalling is not necessary and this can be realized by UE implementation if it is needed. Thus, we prefer the similar solution as Cat.2 that gNb can avoid such scheduling by implementation and UE does not expect such MU-MIMO within a CDM group.

Proposal 6: Do not support any additional restriction on top of legacy MU-MIMO restriction with multiplexing between Rel-18 legacy ports from Rel.18 UE and Rel.15 DMRS ports from Rel.15/18 UE within a CDM group.

Proposal 7: UE does not expect such MU-MIMO with multiplexing between Rel-18 new ports  from Rel.18 UE and Rel.15 DMRS ports from Rel.15/18 UE within a CDM group.

eType1 DMRS for MsgA PUSCH

For MsgA PUSCH, only type 1 DMRS is supported and maximum 4/8 orthogonal uplink DMRS ports in case of maxLength=1/maxLength=2 are supported in the current release. In detail, DMRS ports associated with a PUSCH occasion are configured by parameters msgA-PUSCH-DMRS-CDM-group, msgA-PUSCH-NrofPorts, msgA-Maxlength in MsgA-DMRS-Config-r16. For eType1 DMRS, the maximum 8/16 orthogonal uplink DMRS ports can be supported for single and double-symbol Type1 DMRS in Rel.18, respectively. If eType1 DMRS with increasing number of DMRS ports is introduced for MsgA PUSCH, it can provide benefits such as higher multiplexing efficiency (i.e. more candidate UEs in one PUSCH occasion), more than 2 Msg-A PUSCH resource configuration identified by more than 2 sub-sets of DMRS port/sequence combination, lower DMRS overhead with more DMRS ports in one CDM group. It can be useful for the scenario with higher UE load with 2 step RACH procedure.

Proposal 8: Support eType1 DMRS for MsgA PUSCH.

### PTRS Power boosting for PDSCH transmission

In RAN1 #112 meeting, PTRS power boosting for PUSCH was discussed. Power boosting is also needed for PDSCH transmission. Power boosting of PTRS is only supported for up to six-layer PDSCH transmission in Rel-17, since a UE is not expected to be scheduled with DMRS with TD-OCC and PTRS in the same slot. In Rel-18, with the increased number of DMRS ports, single-symbol DMRS can support up to 8 or 12 DMRS ports for DMRS type 1 and DMRS type 2 respectively. Therefore, PTRS power boosting should be enhanced to support seven-layer or eight-layer PDSCH transmission.

Proposal 9: Study PTRS power boosting for PDSCH transmission.

## Increasing the number of orthogonal DMRS ports for more than 4 layer UL SU-MIMO

PTRS is an essential feature to support FR2 operation for phase noise estimation, one or two PTRS ports can be configured for 4TX PUSCH transmission and a DMRS port is indicated to be associated with a PTRS for phase noise estimation. Two issues should be discussed on the PTRS for 8TX PUSCH transmission, the first one is whether more than two PTRS ports are needed for 8TX with more than two coherent antenna groups. For example, two coherent antenna group corresponding to antenna layout case 2 and four coherent antenna groups corresponding to antenna layout case 3 shall be supported for 8Tx. Similar with Rel-15 PTRS design principle, it’s necessary to assign separate PTRS port to different coherent antenna group. Therefore, up to 2 PTRS ports can be configured for 8Tx UE with two antenna groups, and up to 4 PTRS ports can be required for 8Tx UE with four antenna groups.

Proposal 10: Support the following PTRS configuration for 8Tx UE in FR2

- Up to 2 PTRS ports can be configured for a UE with two antenna groups

- Up to 4 PTRS ports can be configured for a UE with four antenna groups

The other issue is the indication for the PTRS and DMRS association when more than 4 layers are scheduled for the PUSCH transmission. For example, for CB-based PUSCH,  one of the more than 4 DMRS ports should be indicated to be associated with a PTRS port when a single PTRS port is configured for a full coherent UE. In this case, the PTRS-DMRS-association field should be extended to select one of the up to 8 DMRS port(s) for the PTRS port.

When more than one PTRS ports are transmitted by the UE, the PTRS-DMRS association field can be extended similar with the table in alt.2 for single PTRS port case. An example is provided for the UE has two antenna groups.

For the case of two coherent groups, each coherent group contains 4 antenna ports, e.g., the 1st coherent antenna group contains antenna ports {0, 1, 4, 5} and the 2nd coherent antenna group contains antenna ports {2, 3, 6, 7}. Then, PUSCH antenna port 1000, 1001, 1004, 1005 share PT-RS port 0, and PUSCH antenna port 1002, 1003, 1006, 1007 share PT-RS port 1.

UL PT-RS port 0 is associated with the UL layer 'x' of layers which are transmitted with PUSCH antenna port 1000, PUSCH antenna port 1001, PUSCH antenna port 1004 and PUSCH antenna port 1005 in indicated TPMI(s), and UL PT-RS port 1 is associated with the UL layer 'y' of layers which are transmitted with PUSCH antenna port 1002, PUSCH antenna port 1003, PUSCH antenna port 1006 and PUSCH antenna port 1007 in indicated TPMI(s), where 'x' and/or 'y' are given by DCI parameter 'PTRS-DMRS association' as shown in DCI format 0_1 and DCI format 0_2 according to Table 3:

Table 3 PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of the first 2 bits | DMRS port |  | Value of the last 2 bits | DMRS port |
| --- | --- | --- | --- | --- |
| 0 | 1st scheduled DMRS port which shares PTRS port 0 |  | 0 | 1st scheduled DMRS port which shares PTRS port 1 |
| 1 | 2nd scheduled DMRS port which shares PTRS port 0 |  | 1 | 2nd scheduled DMRS port which shares PTRS port 1 |
| 2 | 3rd scheduled DMRS port which shares PTRS port 0 |  | 2 | 3rd scheduled DMRS port which shares PTRS port 1 |
| 3 | 4th scheduled DMRS port which shares PTRS port 0 |  | 3 | 4th scheduled DMRS port which shares PTRS port 1 |

Proposal 11:  Extend the PTRS-DMRS association field to indicate the DMRS port associated with each PTRS port for CB-based PUSCH.

For NCB-based PUSCH, one or two PTRS ports can be configured. For transmission with more than 4 layers and 2 CWs, when only one PTRS port is configured, it is associated with a DMRS port used in one of the codewords. The CW with the higher MCS can be used, or the first CW (CW0) when the MCS is the same for the two CWs. Two bits will be needed to indicate which DMRS port in the chosen CW is associated with PTRS port 0. When two PTRS are configured and transmitted, PTRS port 0 can be associated with one of the DMRS ports used in CW0, and PTRS port 1 can be associated with one of the DMRS ports used in CW1. This requires 2 bits each. So when 2 PTRS ports are configured, the PTRS-DMRS association field in DCI format 0_1/0_2 need to be extended to 4 bits.

Proposal 12:  For NCB-based PUSCH, each PTRS port is associated with one of the DMRS ports used in a codewords. Extended the PTRS-DMRS field to 4 bits in DCI format 0_1/0_2.

In RAN1 #112bis meeting, PTRS power boosting for fully coherent and non-coherent PUSCH transmission was agreed, but the PTRS power boosting for partial coherent PUSCH is still FFS. For partial coherent TPMIs, it was proposed that the PTRS to PUSCH power ratio for PTRS port x is min{10log10(Lx)+10log10(Qp),10log10(L)}, where Lx is the number of PUSCH layers which are coherent with the PUSCH layer where PTRS port x is associated with. From this principle, the PTRS to PUSCH power ratio for different PTRS ports may be different. However, in the legacy system, the PTRS to PUSCH power ratio is same for each PTRS port. For example, for a three-layer PUSCH transmission, suppose the precoder is 12100100100001, even though PTRS port 0 is associated with one layer (i.e., layer 0) and PTRS port 1 is associated with two layers (i.e., layer 1 and layer 2), the PTRS to PUSCH power ratio for each PTRS port is 3*Qp-3, which is determined by assumption of one layer. Therefore, we think the Lx should be the minimum number of PUSCH layers which are precoded coherently with the PUSCH layer where different PTRS ports are associated with.

Proposal 13: Support the following principle to determine the factor related to PUSCH to PTRS power ratio per layer per RE () for partial coherent PUSCH transmission:

- For partial coherent TPMIs, the factor () is min{10log10(Lx)+10log10(Qp),10log10(L)}, where Lx is the minimum number of PUSCH layers which are precoded coherently with the PUSCH layer where different PTRS ports are associated with.

# Conclusion

Base on above discussion, our observations and proposals are summarized as follows:

Proposal 1: For FD-OCC length 4 in Rel.18 eType 1 DMRS for PDSCH:

- No restriction on scheduling of different UEs is needed in case of MU-MIMO

- The number of consecutively scheduled PRBs for PDSCH transmitted from each TRP is even.

Proposal 2: Introduce UE capability for supporting row 21 in case of 1 codeword of PDSCH DMRS port indication table for eType1 DMRS with maxLength=1.

Proposal 3: Do not support row 4-9 for 2 codewords of PDSCH DMRS port indication table for eType1 DMRS with maxLength=1.

Proposal 4: Do not support row 69-80 in agreed Table 7.3.1.2.2-2-X and row 129-152 in agreed Table 7.3.1.2.2-4-X.

Proposal 5: Strive to design DMRS port indication table with the same DMRS type and maxLength for S-TRP and M-TRP with same DCI signalling overhead.

Proposal 6: Do not support any additional restriction on top of legacy MU-MIMO restriction with multiplexing between Rel-18 legacy ports from Rel.18 UE and Rel.15 DMRS ports from Rel.15/18 UE within a CDM group.

Proposal 7: UE does not expect such MU-MIMO with multiplexing between Rel-18 new ports  from Rel.18 UE and Rel.15 DMRS ports from Rel.15/18 UE within a CDM group.

Proposal 8: Support eType1 DMRS for MsgA PUSCH.

Proposal 9: Study PTRS power boosting for PDSCH transmission.

Proposal 10: Support the following PTRS configuration for 8Tx UE in FR2

- Up to 2 PTRS ports can be configured for a UE with two antenna groups

- Up to 4 PTRS ports can be configured for a UE with four antenna groups

Proposal 11:  Extend the PTRS-DMRS association field to indicate the DMRS port associated with each PTRS port for CB-based PUSCH.

Proposal 12:  For NCB-based PUSCH, each PTRS port is associated with one of the DMRS ports used in a codewords. Extended the PTRS-DMRS field to 4 bits in DCI format 0_1/0_2.

Proposal 13: Support the following principle to determine the factor related to PUSCH to PTRS power ratio per layer per RE () for partial coherent PUSCH transmission:

- For partial coherent TPMIs, the factor () is min{10log10(Lx)+10log10(Qp),10log10(L)}, where Lx is the minimum number of PUSCH layers which are precoded coherently with the PUSCH layer where different PTRS ports are associated with.

# References

- 3GPP TSG RAN WG1 #112bis-e, "Draft Meeting Report," online, April 17th – April 26th, 2023.