3GPP TSG RAN WG1 #113									      	R1-2304985

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.4.1.1

Source: 	NEC

Title:	Channel Access of Sidelink on Unlicensed Spectrum

Document for:	Discussion/Decision

- Introduction

During the last meeting, several agreements and relevant options are concluded in [1], further analyses and views on sidelink on the unlicensed spectrum are presented in this contribution.

- Channel Access Issues

  - Channel Access for PSFCH

Agreement - Type 2A channel access procedure is applicable for S-SSB transmissions from a UE without a shared channel occupancy, when the following constraints are met: - Time duration is at most 1ms per transmission - The duty cycle of the S-SSB transmissions is at most 1/20 - FFS: details of EDT - FFS: whether/how to define observation period, including whether or not observation period would be captured in the specifications if defined - FFS: Type 2A applicability for PSFCH without a shared channel occupancy and further limitations for combined transmissions of both S-SSB and PSFCH using Type 2A channel access procedure

Besides Type 1 channel access procedure, as PSFCH resources are allocated following PSSCH symbols in a slot, for the case that a sidelink CO is initiated from PSCCH/PSSCH transmission and shared among UEs, using Type 2A channel access for PSFCH transmission may benefit the success ratio of sidelink A/N feedback information and further improve the sidelink resource efficiency. On the other hand, PSFCH resource allocation period may not meet the limitation of short control signaling according to the resource pool configuration on the unlicensed spectrum. Therefore, Type 2A for PSFCH without an SL CO should not be supported. Based on that, we prefer to use Type 2A for PSFCH transmission only within an SL CO.

- Type 2A channel access procedure can be used for PSFCH only within an SL CO.

  - Contention Window Adjustment

In the last meeting, schemes for CW adjustment for sidelink unicast and groupcast with option 2 are determined as follows, and there is still no consensus on the cases of groupcast option 1 and SL A/N disabled, further discussions and relevant proposals are presented below.

Agreement - The ACK/NACK HARQ-ACK feedback corresponding to the PSSCH for SL unicast in the reference duration for the latest SL channel occupancy for which ACK/NACK HARQ-ACK feedback is available is used as follows: - If ‘ACK’ is received, for every priority class p∈{1,2,3,4}, CWp=CWmin,p, otherwise CWp is increased to the next allowed value. - Note: this is not applied to the case that reference duration includes multiple PSSCHs with ACK/NACK HARQ-ACK enabled, if that case is supported. Agreement The ACK/NACK HARQ-ACK feedback corresponding to the PSSCH for SL groupcast option 2 in the reference duration for the latest SL channel occupancy for which ACK/NACK HARQ-ACK feedback is available is used according to Option 2 when the ratio in Option 1 is not (pre-)configured; otherwise Option 1. - Option 1: Based on a (pre-)configurable ratio of received SL HARQ-ACK feedbacks in the latest SL reference duration, CWp is reset to CWmin,p, for every priority class p∈{1,2,3,4}, otherwise increase CWp for every priority class p∈{1,2,3,4} to the next higher allowed value. - FFS: whether the ratio of the received SL HARQ-ACK feedbacks is ‘ACK’, ‘NACK’ or ‘ACK+NACK’ - FFS: how to calculate the ratio - Note: the (pre-)configuration ratio values of 100% is a valid candidate - Option 2: If at least a ‘ACK’ is received related to any transmissions within the latest SL reference duration, for every priority class p∈{1,2,3,4} CWp=CWmin,p; otherwise CWp is increased.

For the case that sidelink A/N feedback is unavailable as shown below, SL reference duration is not used for contention window determining, and some other factors should be considered to perform contention window adjustment.

- no PSFCH resource is configured in a resource pool;

- for sidelink broadcast transmissions;

- SL-HARQ is disabled by Tx UE for unicast and groupcast;

The basic principle of contention window determination is to modify the contention window based on channel status, i.e., increasing the contention window while the channel tends to have more conflicts. Accordingly, sidelink CR/CBR may be used as an alternative to sidelink A/N, as it presents the ratio of sidelink channels being occupied. As sidelink CR/CBR is already defined and used in Rel-16 sidelink, a limited effort is needed to determine a sidelink CR/CBR based contention window adjustment scheme.

- For the case that SL-HARQ feedback is unavailable, sidelink CR/CBR is used for contention window adjustment in SL-U;

  - based on a (pre-)configurable threshold of SL CR/CBR, CWp is reset to CWmin,p for every priority class p∈{1,2,3,4} when CR/CBR measurement is lower than the threshold;

  - otherwise, increase CWp for every priority class p∈{1,2,3,4}  to the next higher allowed value.

During the last meeting, it is confirmed by a majority that groupcast option 1 should be supported and no enhancement for the NACK only scheme. Meanwhile, it is not unfair to always use the latest CW for groupcast option 1. To support CW adjustment for groupcast option 1, it is beneficial to use the same CW solution for groupcast option 1 and SL-HARQ disabled. Therefore, we prefer to support option 4 here, i.e., determining CW based on SL CR/CBR.

- For contention window adjustment for groupcast option 1 (NACK only), using the same CW adjustment scheme of SL-HARQ feedback unavailable, i.e., sidelink CR/CBR is used.

According to the agreement, a (pre-)configurable ratio of received SL HARQ-ACK feedback for groupcast option 2 is used for CW adjustment. The ratio should be defined as the ratio of ‘ACK’, i.e., the number of received ‘ACK’ divided by the number of available PSFCH resources in the group.

- For the CW adjustment of SL groupcast option 2, the ratio of received SL HARQ-ACK feedback should be defined as a ratio of ‘ACK’, i.e., the number of received ‘ACK’ divided by the number of all the available PSFCH resources in the group for the PSSCH transmission.

  - CPE

The following agreements for CPE are achieved in the last meeting, and the values of CPE should be further clarified.

Agreement For the CPE agreements reached so far in this agenda, the 1 or at most 2 symbols just before the next AGC symbol for CPE transmission is/are physical symbol(s). Agreement For 15 kHz, 30kHz and 60kHz SCSs, a set of CPE starting candidate position(s) for PSCCH/PSSCH is (pre-)configured or pre-defined in the spec (to be down-selected) separately for transmission within COT and transmission outside COT. - Note: It is up to the (pre-)configuration or pre-definition in the spec (to be down-selected) whether each set of CPE starting candidate position(s) associated with Option 1 (1-symbol length) for CPE window or Option 2 (2-symbol length) for CPE window and whether each set of CPE starting candidate position(s) include one or multiple starting position(s) - FFS whether the set(s) of CPE starting positions are (pre-)configured/pre-defined per priority - FFS values for the (pre-)configured/pre-defined CPE starting candidate position(s) (including a default value) for each set, and whether the default value is the same or different for different sets Conclusion For defining the locations of CPE starting positions, RAN1 concludes that the NR-U principle for switching gaps is reused in SL-U, that is: - The TX/RX switching gap is already included in the existing channel sensing structures - The RX/TX switching gap is already included in the existing channel sensing structures

As multiple CPE lengths may be used for PSCCH/PSSCH transmission, a longer CPE length, i.e., an earlier CPE starting position, should be applied for sidelink transmission with higher priority. It may contribute to the success rate for higher priority services by using different CPE lengths for different priorities. Therefore, in the case that multiple CPE starting positions are configured for sidelink, the different CPE lengths should be used for different priorities.

According to the agreements, one or two physical symbols before the next AGC symbols can be used for CPE transmission, to meet different scenarios and requirements of sidelink on unlicensed spectrum, it is more flexible to provide the available CPE starting positions based on (pre-)configuration.

- The set(s) of CPE starting positions are (pre-)configured per priority.

The available values of CPE starting positions for SL-U should be defined by reusing the CPE (pre-)configuration scheme in NR-U, i.e., through Ci and Δi. According to the agreement, one or two symbols may be used for CPE transmission window, i.e., Ci=[1, 2], and Δi should reuse the values defined in Table 5.3.1-2 in TS 38.211.

- The available values of CPE in SL-U should be (pre-)configured through Ci and Δi:

  - Ci=1, 2;

  - Δi is defined in Table 5.3.1-2 in TS 38.211;

For S-SSB transmission, one or more UEs may transmit the same content on the same S-SSB resources, while multiple CPE starting positions may lead to an unexpected S-SSB transmission block. Therefore, only one CPE length for S-SSB is preferred.

For PSFCH transmissions, similar to the situation of S-SSB, more than one UE may try to access the channel and transmit PSFCH on the same symbol, and multiple CPE starting positions may lead to a transmission block. Therefore, only one CPE starting position should be used for PSFCH transmission.

- Support only one CPE starting position for S-SSB.

- Support only one CPE starting position for PSFCH.

- Mode 1

As described in [2] “Uu operation for mode 1 is limited to licensed spectrum only”, it means that gNB should schedule unlicensed spectrum resources for sidelink while it may not work on the unlicensed spectrum. According to the scheduling of gNB, UE should first perform a channel access procedure to obtain the scheduled sidelink resources, and then transmit sidelink signals on the assigned resources if channel access succeeds.

In the case that gNB does not work in the unlicensed spectrum, an assistant scheme may be mentioned that UE may report sensing information of the unlicensed spectrum to gNB on Uu for reference, and then gNB schedules sidelink resources in the unlicensed spectrum taking into account the assistant information. However, the sensing information reported by the UE introduces additional resource consumption on Uu, while the information may be with low timeliness by the way of Uu transmission and low accuracy with the limited receiving ability of UE. As a whole, the assistant information reporting may not be able to effectively help the scheduling of sidelink in the unlicensed spectrum. Therefore, it is unnecessary to exchange such information between UE and gNB.

- For mode 1 in SL-U, no assistant information related to the scheduling in the unlicensed spectrum needs to be exchanged between UE and gNB.

As COT sharing may benefit sidelink transmissions, it should also be supported in mode 1 resource allocation. According to the agreement that “gNB relaying/forwarding a UE initiated COT to another UE is not supported in Rel-18”, an intended SL CO may be considered to be scheduled by gNB and be shared for sidelink UEs to access channel with higher efficiency.

To execute mode 1 COT sharing on sidelink, gNB may indicate an intended SL CO for UEs which are scheduled on multiple consecutive slots, and an initial UE should also be assigned by gNB to perform the Type 1 channel access procedure to initial the intended CO. As the intended SL CO is indicated by gNB before the actual initiation, no additional indication is required to be relayed or forwarded by gNB from the initial UE to the responding UE(s). When the intended SL CO is successfully initiated, other UEs scheduled on the following slots could share the CO by using Type 2 channel access schemes. Besides, as no further indication from gNB after the intended SL CO is initiated, a common scheme for sidelink COT sharing could be designed for both mode 1 and mode 2 cases.

- The gNB schedules UEs in an intended SL CO which is to be initiated by a sidelink UE, and the SL CO may be shared by other scheduled UEs.

From the perspective of system allocation, it may be configured that mode 1 sidelink resource pools are assigned in both licensed spectrum and unlicensed spectrum. In other words, gNB may schedule sidelink resources for a UE in the licensed resources or the unlicensed resources. In this case, a specific issue to be considered is how to identify the DCI for legacy sidelink scheduling in the licensed spectrum and the ones for SL-U assignment. For example, as a dedicated RNTI is used for sidelink scheduling, another RNTI may be assigned for UE to identify DCI for SL-U scheduling.

- In the case that both licensed and unlicensed spectrum resources are configured for sidelink mode 1, it needs to be considered how to identify DCI for sidelink scheduling on the licensed spectrum or the unlicensed spectrum.

- Type 1 LBT Blocking

During the last meeting, Type 1 LBT blocking among UEs is discussed and preliminary agreements are achieved as follows.

Agreement To resolve the Type 1 LBT blocking issue, where one UE performing a Type 1 LBT procedure for using its own selected/reserved resource(s) is blocked by another UE’s SL transmission at least in a slot preceding to the selected/reserved resource and causing the LBT to fail, further study the following options in a future meeting. - Option 1: - UE avoid selection of N consecutive resource(s) before a reserved resource with high priority when the transmitting symbols of the selected resource overlap with Type 1 LBT of the reserved resource. - UE avoid selection of N consecutive resource(s) after a reserved resource when the transmitting symbols of the reserved resource overlap with LBT of the selected resource. - FFS: the avoidance should be performed by L1 exclusion or L2 MAC selection - FFS: whether / how to achieve this in RA mode 1 - FFS: How to determine value of N - Option 2: - UE prioritizes/selects resource(s) for transmission in slot(s) after a reserved resource when transmission of the selected resource is able to share the initiated COT of the reserved resource (i.e., the selected resource(s) is within the COT duration of the reserved resource and the CAPC value of the selected resource(s) is equal to or higher than that of the reserved resource). - UE prioritizes/selects resource(s) for transmission in slot(s) before a reserved resource when transmission of the selected resource is able to share its initiated COT with the reserved resource (i.e., the reserved resource is within the COT duration of the selected resource(s) and the CAPC value of the selected resource(s) is equal to or smaller than that of the reserved resource). - FFS whether / how to achieve this in RA mode 1. - Option 3: UE selects extra / more resources than required for transmitting a TB (i.e., overbooking) to accommodate potential Type 1 LBT failures. FFS how to determine/preconfigure the number of extra selected resources. - Option 4: The expected LBT duration is determined firstly, then resource selection takes into account of the expected LBT duration is performed. - Option 5: At MAC layer, selection of resource(s) among the reported set of candidate resources from L1 is up to UE implementation in mode 2 for SL-U, instead of random selection. - Option 6: UE excludes frequency resources (if any) previously reserved via SCI by other SL UEs in the corresponding slot, when estimating the detected power within a sensing slot duration in Type 1 channel access. - Option 7: SL UE deems channel busy only if the UE detects transmission other than SL transmission occupying the channel (e.g., exceeding the energy detection threshold), i.e., the energy detection for EDT checking in LBT procedure does not take into account the energy from SL transmissions. - Option X: No solution is needed. To avoid inter-UE blocking from performing Type 1 LBT can be handled based on UE implementation (e.g., as the start timing to perform LBT sensing is determined by each UE).

In general, the LBT blocking issue can be resolved through channel access procedure and relevant CW adjustment to some extent. There is no urgent need to define a specific mechanism to resolve the LBT blocking issue additionally, and we prefer to adopt option X, i.e., no solution is needed for this issue.

- Option X is preferred for the LBT blocking issue, i.e., no solution is needed.

- COT Sharing in SL-U

According to the agreement of COT sharing as follows, some further details need to be discussed.

Agreement The container for carrying the COT sharing information from a COT initiator UE includes at least the SCI. - FFS 1st and/or 2nd stage SCI. Agreement At least the following information should be used as part of COT sharing information from the COT initiator UE. - CAPC used for initiating the COT - Existing / legacy R16/17 L1 source and destination IDs - FFS additional ID(s) - Time domain information of the shared COT - FFS: starting offset, number of slots, [remaining or total] COT duration, or a combination of them - Frequency domain information of the shared COT - FFS applicable RB set(s), FRIV, and any other(s) - FFS: how each of the above is indicated. - Note, other information is not precluded.

When an SL CO is initiated and allowed to be shared by other UEs, the initial UE should indicate relevant information about the SL CO to potential responding UE(s) through SCI, including the CAPC level used for initiating the SL CO, the remaining duration of the CO, and the RB set(s) contained in the CO. Besides, based on the initial UE’s requirements, it can also disable the COT sharing, and a relevant indicator may also be included in the SCI.

- The information about an SL CO should be indicated by the initiating UE through SCI, including:

  - Whether the CO is allowed to be shared;

  - The remaining duration of the CO;

  - RB set(s) in the CO;

  - Additional ID;

Considering the information overhead, especially when additional ID(s) are supported, it is hard to involve the information along with an SCI associated with a PSSCH transmission, and a dedicated SCI to indicate the information of an SL CO should be introduced.

- The information about an SL CO should be indicated through a dedicated SCI, including the 1st and the 2nd SCI.

As each PSFCH resource is associated with relevant PSSCH transmission and there are no PSFCH resource conflicts for PSSCH transmissions without conflict. That is to say, within an SL CO, UEs can utilize relevant PSFCH resources respectively without additional conflicts. Therefore, besides the PSFCH transmission intended for the initiating UE of an SL CO, it should also be supported for the PSFCH transmission(s) to a destination UE other than the initiating UE.

- For sidelink UE-to-UE COT sharing, a responding UE can share the CO for PSFCH transmission(s) to a destination UE other than the initiating UE of the SL CO.

Considering the case that an SL CO may be successively shared by multiple responding UEs on consecutive slots, the COT sharing related SCI from the initial UE may be not detected by some UEs, and the relevant information of the SL CO should be relayed during the CO. That is to say, besides the initial UE, the responding UE may need to forward the information with an updated remaining duration of the CO.

- The information of a shared CO should be forwarded by the responding UE through SCI:

  - The remaining duration of the SL CO should be updated;

On the other hand, there are different understandings of gNB-to-UE COT sharing among companies. In our view, according to the description “Uu operation for mode 1 is limited to licensed spectrum only”, it aims to pursue a concise way to support gNB scheduling for SL-U and simplify the study of mode 1 without gNB operation in unlicensed spectrum. If gNB-to-UE COT sharing is supported, it may require gNB to operate in the unlicensed spectrum to share resources with sidelink UEs, the requirement “Uu operation for mode 1 is limited to licensed spectrum only” would instead lead to additional complexity for mode 1 discussion. Therefore, we propose to not support gNB-to-UE COT sharing at this stage.

- The gNB-to-UE COT sharing is not supported for SL-U.

- Multi-channel Operation

Regarding multi-channel operation in SL-U, the agreement is provided as follows.

Agreement For dynamic channel access mode with multi-channel case in SL-U, both NR-U DL Type A and Type B multi-channel access procedure are supported for multiple PSFCH transmissions on multiple channels. - FFS: It is up to UE implementation to perform either Type A or Type B multi-channel access procedure. - FFS: whether this can initiate a shared COT - FFS: whether there is any special handling needed for transmission in a shared COT on one or more of the channels Agreement Channel access procedures for SL multi-channel transmission(s) include the following cases. - If a UE is scheduled to transmit on a set of channels C, and if the SL transmissions are scheduled to start transmissions at the same time on all channels in the set of channels C, or - If a UE intends to perform sidelink transmissions on configured resources on the set of channels C, and if the SL transmissions are configured to start transmissions at the same time on all channels in the set of channels C, or - If a UE intends to perform sidelink transmissions on selected resources on the set of channel C, and if SL transmissions are to start at the same time on all channels in the set of channels C.

For a PSSCH transmission with multiple RB sets, the associated feedback transmission may not need to be transmitted on all the RB sets. In addition, in the case that the associated PSFCH resource is out of the SL CO used by the PSSCH transmission, Rx UE may have to perform independent channel access procedures to obtain the PSFCH transmission occasion. Hence, PSFCH transmission on a subset of RB sets for associated PSSCH transmission should be supported to facilitate the feedback transmission.

For a PSSCH transmission on multiple channels, the associated PSFCH should be transmitted on a subset of the multiple channels.

Based on the agreement, for a sidelink UE with multiple PSFCH transmissions on multiple channels, both NR-U DL Type A and Type B multi-channel access schemes are supported. As using Type A or Type B for multi-channel access is up to gNB implementation in NR-U, a similar rule should be used for SL-U, and we prefer to support that it is up to UE implementation to perform either Type A or Type B multi-channel access procedure for multiple PSFCH transmissions on multiple channels.

It is up to UE implementation to perform either Type A or Type B multi-channel access procedure for multiple PSFCH transmissions on multiple channels.

For the case that a UE uses Type A or Type B multi-channel access procedure to access channel and transmits multiple PSFCH transmissions, the SL CO starts from a PSFCH symbol, i.e., there is no PSCCH resource at the beginning of the SL CO. If the SL CO can be shared with other UEs, the SCI carrying sharing information from the initiator UE should be indicated in the next slot after the PSFCH transmissions, while there may be no PSCCH/PSSCH to be transmitted by the initiator UE. To avoid extra complexity and further workload, it should not be supported initiating a shared CO from multi-channel access procedures for PSFCH transmissions.

Do not support initiating a shared CO from multi-channel access procedures for multiple PSFCH transmissions.

- Conclusion

In this contribution, some issues related to the channel access procedure for Rel-18 SL-U are discussed, and the relevant observations and proposals are listed as follows:

Proposal 1:	Type 2A channel access procedure can be used for PSFCH only within an SL CO.

Proposal 2:	For the case that SL-HARQ feedback is unavailable, sidelink CR/CBR is used for contention window adjustment in SL-U;

−	based on a (pre-)configurable threshold of SL CR/CBR, CWp is reset to CWmin,p for every priority class p∈{1,2,3,4} when CR/CBR measurement is lower than the threshold;

−	otherwise, increase CWp for every priority class p∈{1,2,3,4}  to the next higher allowed value.

Proposal 3:	For contention window adjustment for groupcast option 1 (NACK only), using the same CW adjustment scheme of SL-HARQ feedback unavailable, i.e., sidelink CR/CBR is used.

Proposal 4:	For the CW adjustment of SL groupcast option 2, the ratio of received SL HARQ-ACK feedback should be defined as a ratio of ‘ACK’, i.e., the number of received ‘ACK’ divided by the number of all the available PSFCH resources in the group for the PSSCH transmission.

Proposal 5:	The set(s) of CPE starting positions are (pre-)configured per priority.

Proposal 6:	The available values of CPE in SL-U should be (pre-)configured through Ci and Δi:

−	Ci=1, 2;

−	Δi is defined in Table 5.3.1-2 in TS 38.211;

Proposal 7:	Support only one CPE starting position for S-SSB.

Proposal 8:	Support only one CPE starting position for PSFCH.

Proposal 9:	For mode 1 in SL-U, no assistant information related to the scheduling in the unlicensed spectrum needs to be exchanged between UE and gNB.

Proposal 10:	The gNB schedules UEs in an intended SL CO which is to be initiated by a sidelink UE, and the SL CO may be shared by other scheduled UEs.

Proposal 11:	In the case that both licensed and unlicensed spectrum resources are configured for sidelink mode 1, it needs to be considered how to identify DCI for sidelink scheduling on the licensed spectrum or the unlicensed spectrum.

Proposal 12:	Option X is preferred for the LBT blocking issue, i.e., no solution is needed.

Proposal 13:	The information about an SL CO should be indicated by the initiating UE through SCI, including:

−	Whether the CO is allowed to be shared;

−	The remaining duration of the CO;

−	RB set(s) in the CO;

−	Additional ID;

Proposal 14:	The information about an SL CO should be indicated through a dedicated SCI, including the 1st and the 2nd SCI.

Proposal 15:	For sidelink UE-to-UE COT sharing, a responding UE can share the CO for PSFCH transmission(s) to a destination UE other than the initiating UE of the SL CO.

Proposal 16:	The information of a shared CO should be forwarded by the responding UE through SCI:

−	The remaining duration of the SL CO should be updated;

Proposal 17:	The gNB-to-UE COT sharing is not supported for SL-U.

Proposal 18:	For a PSSCH transmission on multiple channels, the associated PSFCH should be transmitted on a subset of the multiple channels.

Proposal 19:	It is up to UE implementation to perform either Type A or Type B multi-channel access procedure for multiple PSFCH transmissions on multiple channels.

Proposal 20:	Do not support initiating a shared CO from multi-channel access procedures for multiple PSFCH transmissions.

- Reference

- Report of RAN1 #112bis-e

- RP-230077, WID revision: NR sidelink evolution