3GPP TSG RAN WG1 #113			R1-2305011

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.10.2

Source:	Google

Title:	Discussion on timing advance management to reduce latency

Document for: 	Discussion

- Introduction

In this contribution, we discuss some issues related to TA management in Rel-18 LTM, including TA indication, TA measurement and TA/TAG association. In Rel-18 mobility WID [1], RAN Plenary has agreed to specify a L1/L2 triggered mobility procedure, aiming to reduce latency in mobility/handover. In previous RAN1 meetings [2] [3], RAN1 has made some agreements with respect to LTM, as shown in the followings. In subsequent sections, we discuss and provide our opinions on these related issues.

RAN1 #111 Agreement On mechanism to acquire TA of the candidate cell(s) in Rel-18 LTM, at least support PDCCH ordered RACH.  The PDCCH order is only triggered by source cell  FFS: the details including content of DCI, RACH resource configuration, RAR transmission mechanism, etc.  Note: any other RACH-based solutions are for discussion separately Agreement - TA updating (i.e. re-acquisition of TA) for candidate cell can be triggered by NW. - same triggering mechanism reuse the initial TA acquisition, i.e., PDCCH order triggered RACH in a candidate cell RAN1 #112 Agreement - For PDCCH-order based RACH for TA measurement for candidate cells, legacy CBRA is not supported Agreement If reception of RAR is configured/indicated, RAR contains at least TA of candidate cell. - The maximum number of TA values memorized by UE is a UE capability - FFS: whether other parameters such as UE ID, candidate cell ID etc. is contained in RAR Agreement Whether RAR needs to be received is configured by RRC. RAN1 #112b(e) Agreement For PDCCH ordered-RACH, if reception of RAR is not configured, UE autonomous re-transmission of PRACH is not allowed, regardless of the configuration of PreambleTransMax. Agreement When reception of RAR is configured, support RAR is received from serving cell at least in intra-DU case. Agreement When reception of RAR is configured, support RAR is received from serving cell in inter-DU case. - FFS: RA response window related issues Agreement For PDCCH ordered RACH mechanism in R18 LTM, when reception of RAR is configured, - the UE stores(remembers/maintains/handles) a TA for at least one candidate cell - storing(remembering/maintaining/handling) corresponding TAs for more than one candidate cell is up to UE capability - detailed number of candidate cell is up to UE capability Agreement For PDCCH-order based PRACH for candidate cell study the following issues: - whether/how prioritizations for transmission power reduction for a PRACH transmission to a LTM candidate cell is performed - whether/how prioritizations for prioritization of a PARCH transmission to a LTM candidate cell compared to an overlapped (in time and frequency) serving cell UL transmission Agreement For PDCCH ordered-RACH, if reception of RAR is not configured - Whether power ramping is performed or not is determined from PDCCH order - If power ramping is performed, - whether PRACH is an initial transmission or retransmission is explicitly indicated in PDCCH order (FFS exact indication mechanism) - power ramping-up value is configured - else, the power should be determined by open-loop power control

- Discussion

- TA indication

In TA indication for LTM, there is one issue related to TA indication timing, i.e., when to indicate TA value for a candidate/target cell. This issue could be similar to when to indicate beam indication, which is currently discussed in other agenda. In our views, RAN1 should support TA value can be indicated before the CSC, where one use case is that TA value is indicated by RAR, if RAR is configured.

Proposal 1: Support TA value can be received before a cell switch command, at least when reception of RAR is configured.

In Rel-18 MIMO WI, it has been agreed to introduce two TAGs for M-TRP operation. However, for mobility enhancement, it is different from M-TRP operation, which does not need to mandate the UE to communicate with two cells simultaneously. Thus, it could be unnecessary for a UE to maintain more than one TAGs in a serving cell for mobility enhancement. However, the TA could be different for a UE to transmit uplink signals to different cells, as the propagation delay could be quite different. When UE starts to communicate with the target cell, it is necessary to apply a proper TA. The TA management for mobility should include two aspects:

- TA measurement for uplink signals to target cell

- TA update for uplink signals to target cell.

Proposal 2: TA management for Rel-18 mobility does not require more than two TAGs in a serving cell.

- TA measurement

For TA measurement, in RAN1 #112, the following agreement has been achieved.

RAN1 #112 Working Assumption UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported - FFS: other impacts on RAN1 spec

With the following analysis, we consider UE-based TA measurement using Rx timing difference is very useful. Therefore, we suggest confirming the WA in RAN1#112b(e).

Proposal 3: Confirm Working assumption on UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell).

UE-based TA measurement and update is a possible way for RA-less solution. The UE can measure the TA based on downlink timing for downlink signals. In this way, the UE needs to know a first TA for a first downlink signal, and the UE can measure the timing for a second downlink signal. Then, by comparing the timing difference between the two downlink signals, the UE can obtain the propagation difference between the two downlink signals so that the UE can estimate the second TA for the link with the same direction as the second downlink signal based on the first TA from the first signal and timing difference between two signals. Such measurement should require the two downlink signals are well-synchronized.

Figure 1 illustrates UE-based TA measurement. It can be observed that for well-synchronized cells, UE based TA measurement is possible. When the serving cell and target cell are well-synchronized, the UE can estimate the TA for uplink signal 2 to target cell based on the TA in serving cell and the timing difference between downlink signals from serving cell and target cell.

Figure 1: UE-based TA measurement

The potential drawback is that it could require additional PRACH procedure. Therefore, it can be considered as a complimentary, e.g., fallback operation, if any RACH-less solution is not configured/enabled. When the serving cell and target cell are not well synchronized, the UE-based TA measurement could be inaccurate, since the start timing for the DL signals for both cells could be quite different. Then, the RACH based solution can be used as a complimentary for TA measurement. For example, the gNB can use PDCCH-ordered PRACH for TA measurement.

- Association between TA/TAG and candidate cell

For mobility, the UE may not need to maintain the communication with both source cell and candidate/target cell. In such case, the UE could only need to maintain one TA and apply the TA when it starts to communicate with a cell. When the UE is communicating with the source cell, the network can indicate a TAC based on the TA measurement from a PRACH toward the source cell or the UE can autonomously update the TA. Similarly, when the UE is communicating with the candidate/target cell, the network can indicate a TAC based on the TA measurement from a PRACH toward the target cell or the UE can autonomously update the TA. Therefore, it is unnecessary to associate the TA/TAG with a candidate/target cell.

Proposal 4: Do not support associating the TA/TAG with a candidate/target cell.

- Conclusion

According to the above discussion(s) on TA management of LTM, we have the following proposal(s).

Proposal 1: Support TA value can be received before a cell switch command, at least when reception of RAR is configured.

Proposal 2: TA management for Rel-18 mobility does not require more than two TAGs in a serving cell.

Proposal 3: Confirm Working assumption on UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell).

Proposal 4: Do not support associating the TA/TAG with a candidate/target cell.

# Reference

- RP-222332 Revised WID on Further NR mobility enhancements, MediaTek

- Draft Report of 3GPP TSG RAN WG1 #111 v0.2.0 (Toulouse, France, 14th – 18th November 2022)

- Draft Report of 3GPP TSG RAN WG1 #112 v0.4.0 (Athens, Greece, February 27th – March 3rd, 2023)