3GPP TSG RAN WG1 #113	                 R1-2305064

Meeting, Incheon May 22nd – May 26th, 2023

Agenda item:	9.4.2

Source: 	Lenovo

Title: 	Discussion on co-channel coexistence for LTE sidelink and NR sidelink

Document for:	Discussion

Introduction

In RAN#99 meeting on co-channel coexistence the updated WID was agreed as following:

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible - Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A - RAN1 is tasked to support only 15 and 30 kHz SCSs for dynamic resource pool sharing. Existing RAN1 agreements for dynamic resource pool sharing apply to support of 30 kHz. - For NR PSCCH/PSSCH transmissions in 30kHz SCS, NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with an LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer - No change to the R16/17 resource allocation procedure in PHY due to this restriction - The existing SL slot structure from Rel-16 is unchanged - The starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe - For NR SL with 15/30kHz SCSs, NR SL UE avoids selecting resources for PSCCH/PSSCH transmissions where the corresponding PSFCH transmission occasions overlap with LTE SL reservations in time domain - Note, this is inline with Option 1-2 in the working assumption made in RAN1#112. No other options from the working assumption need to be considered. - Mode 2 operation only

In this contribution we provide our views on the co-channel coexistence for LTE sidelink and NR sidelink.

Discussion

In this contribution we provide our views on some remaining issues of dynamic resource poll sharing.

On the NR resource exclusion with the corresponding PSFCH transmission occasions overlap with LTE SL reserved resources following agreement was made in RAN1#112bis-e meeting:

Agreement In NR SL resource (re)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources where the corresponding PSFCH transmission occasions overlap with LTE SL reserved resources by other LTE SL UE in time domain, Down-selection one of followings: - Option 1: - When the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission - The list of the above initial SL RSRP thresholds is separately (pre)configured - FFS: whether this initial SL RSRP threshold list can be (pre)configured per subset of PSFCH resources - For determining the LTE SL periodic reserved resources by other LTE SL UE, - Reuse the same mechanism as in NR SL resource (re)selection procedure excluding NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE - The PHY layer of NR SL module applies the above procedure in Step 6 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of NR SL periodic resources when the corresponding PSFCH transmission occasions are overlapping with LTE SL reserved resources by other LTE SL UE and the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold according to condition c in Step 6 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module - Option 2: - For determining the LTE SL periodic reserved resources by other LTE SL UE, - Reuse the same mechanism as in NR SL resource (re)selection procedure excluding NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - FFS: whether to apply the above procedure based on the priority of the LTE SL reserved resources and/or the priority of the NR SL transmission - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of NR SL periodic resources when the corresponding PSFCH transmission occasions are overlapping with LTE SL reserved resources by other LTE SL UE in time domain according to Step 5 in Section 8.1.4 of TS 38.214. - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module

Legacy NR resource exclusion is to avoid the resource collision and handle the interference level of different sidelink UEs, for dynamic resource pool sharing the NR resource exclusion with the consideration of associated PSFCH resources is to avoid the AGC issue of LTE SL reception, even the measured RSRP is lower than the threshold the total reception power at LTE SL reception could also cause AGC issue which will have negative impact on the performance of LTE SL.

Proposal 1: Support Option 2 that RSRP threshold is not considered when NR SL module excludes NR SL candidate resources where the corresponding PSFCH transmission occasions overlap with LTE SL reserved resources by other LTE SL UE in time domain

On the timeline of dynamic resource pool sharing following agreement was made in RAN#112bis-e meeting:

Agreement The NR SL module uses the information from the starting LTE SL subframe to the ending LTE SL subframe in the shared information from the LTE SL module. - The starting LTE SL subframe is no later than the time (n-T_start) - n is the time where NR module triggers its NR SL resource (re)selection procedure as defined in clause 8.1.4 of TS 38.214 - Down-selection one of followings for T_start: - Option 1-1: T_start is T_0 as defined in clause 8.1.4 of TS 38.214 - Option 1-2: T_start is 1100ms - Option 1-3: T_start is up to UE implementation - The ending LTE SL subframe is no earlier than the time (n-T_valid2) - Down-selection one of followings for T_valid2: - Option 2-1: T_valid2 is T - Option 2-2: T_valid2 <= T+4ms - Option 2-3: T_valid2 is up to UE implementation

On the timeline of dynamic resource pool sharing the window of information sharing between LTE SL module and NR SL module could be both up to UE implementation since it is the internal information exchange in one SL UE.

Proposal 2: On T_start support Option 1-3 that it is up to UE implementation

Proposal 3: On T_valid2 support Option 2-3 that it is up to UE implementation

Conclusion

In this contribution, we provide out views on some remaining issues of dynamic resource sharing between LTE SL and NR SL:

Proposal 1: Support Option 2 that RSRP threshold is not considered when NR SL module excludes NR SL candidate resources where the corresponding PSFCH transmission occasions overlap with LTE SL reserved resources by other LTE SL UE in time domain

Proposal 2: On T_start support Option 1-3 that it is up to UE implementation

Proposal 3: On T_valid2 support Option 2-3 that it is up to UE implementation

References

- RP-213678, “New WID on NR Sidelink evolution”, RAN#94-e, Dec. 2021.