3GPP TSG RAN WG1 #113                                                       R1-2305074

Incheon, Korea, May 22nd – May 26th, 2023

Source: Google Inc.

Title:	On XR-specific capacity enhancements techniques

Document for: Discussion and Decision

# Introduction

A new Rel-18 Work Item for XR enhancements in NR has been approved in RAN#98-e [1]. Two main enhancements related to the system capacity are to be specified in RAN1 [1]:

-	Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration

-	Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE

In this Tdoc, we propose some techniques to specify the two enhancements above.

# Multiple CG PUSCH transmission occasions

The support of multiple PUSCH occasions in a CG period is beneficial to reduce latency for XR and improve performance.

## Time domain resource allocation of CG PUSCHs associated to a multi-PUSCHs CG

The following agreement has been made in RAN1#112bis regarding the time domain resource allocation of CG PUSCHs associated to a multi-PUSCHs CG:

Agreement:

For TDRA design for multi-CG PUSCH, prioritize Alt-A1, Alt-B, and Alt-C2 for further downscoping and/or modification from corresponding agreement in RAN1#112.

- FFS: How to address TDD configuration issue

Hence, the following three options are selected for further down-scoping

- Alt-A1: Follow the time domain resource mapping of Type A repetition

  - N configured by higher layers or indicated by activation DCI

  - Single SLIV is determined from TDRA

  - The same SLIV in N PUSCH in consecutive slots per CG period

    - FFS for non-consecutive slots

  - FFS details, including related RRC parameters

- Alt-B: TDRA determination based on NR-U framework

    - N and M configured by higher layers

    - Single SLIV is determined from TDRA.

      - The SLIV used for 1st PUSCH per CG period.

    - M consecutive PUSCH TOs with same duration in slot. The M PUSCH TOs are used in N consecutive slots per CG period

    - Note: N and M are configured independently from cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively. M and N configuration is independent from cgRetransmissionTimer configuration.

    - FFS details, including related RRC parameters

- Alt-C2: Follow Rel-17 single DCI scheduling multiple PUSCHs

  - TDRA configured by pusch-TimeDomainAllocationListForMultiPUSCH-r16 with extendedK2-r17

  - A row of TDRA with N entries determines the time domain resources allocation of N PUSCH TOs per period

    - Note: N PUSCH TOs can be non-consecutive PUSCHs and/or in non-consecutive slots.

  - FFS details, including related RRC parameters

Supporting companies need to clarify how the single DCI scheduling multiple PUSCHs in Alt-C will be adapted for CG configuration with the multiple PUSCH occasions for CG type 1. It seems Alt-C could operate with CG type 2 as the UL DCI can indicate the row of the TDRA with the N entries. However, for type-1, it seems the other alternatives are more adequate.

Rel-16 NR-U specification includes the use of CG with multiple CG PUSCH occasions within a period. The Rel-16 NR-U feature should be considered the basis for this XR enhancement in Rel-18.

This feature is already in the specification and we need to just remove the band restriction and introduce some minor modification mainly for the use of the retransmission timer.

The current parameters cg-nrofPUSCH-InSlot and cg-nrofSlots of the RRC parameter structure   ConfiguredGrantConfig defined in 3GPP TS 38.331 could be re-utilized to configure multiple PUSCH transmission occasions per CG period on licensed spectrum for XR, as these parameters are currently only supported for unlicensed spectrum operation when the RRC parameter cg-RetransmissionTimer is configured. Hence, the current restriction that the network can only configure these fields if cg-RetransmissionTimer is configured can be relaxed for the XR traffic.

As captured in TS 38.214 clause 6.1.2.3, the higher layer parameter cg-nrofSlots determines the number of consecutive slots allocated within a given CG period. The cg-nrofPUSCH-InSlot parameter configures the number of consecutive PUSCH allocations within each slot. The first allocation follows the higher layer parameter timeDomainAllocation for Type 1 PUSCH transmission or the higher layer configuration according to [10, TS 38.321], and UL grant received on the DCI for Type 2 PUSCH transmissions. Subsequent allocations have the same length, PUSCH mapping type and are appended without gaps. The same combination of start symbol, length and PUSCH mapping type repeats over the consecutively allocated slots.

The Rel-16 NR-U scheme described above can be extended to XR as is, but further improvements can be made to make it more suitable for the XR traffic.

- The TDRA determination based on NR-U framework should be extended to XR with some enhancements.

The specified NR-U scheme relies on a retransmission timer (cg-RetransmissionTimer) for autonomous retransmissions. If the UE does not receive an acknowledgment for the previous CG retransmission after this timer expires, it considers the transmission to have been unsuccessful and automatically initiates another transmission.

For XR and instead of the NR-U autonomous retransmission scheme (used mainly because the gNB can have difficulty acquiring a COT to send a DCI scheduling the retransmission), the configuredGrantTimer used with the CG legacy scheme could be applied for each HARQ process associated with each PUSCH occasion of the multiple CG PUSCH transmission occasions.  If the CG timer expires and the UE does not receive an UL DCI scheduling a retransmission then the UE can consider the corresponding HARQ transmission as successful and can consider the HARQ process released and could be used for another transmission.

- The legacy configuredGrantTimer can be used for each PUSCH occasion of the multiple CG PUSCH transmission occasions instead of the cg-RetransmissionTimer used in NR-U.

In summary to compare Alt-B and Alt-C2

|  | Pros | Cons |
| --- | --- | --- |
| Alt-B | - Simple, only required change is for TDD by clarifying that the number of slots should be contiguous UL slots - It applies to type 2 as is, no need for any specification changes. The activation DCI can carry MCS and FDRA as per legacy. Same MCS and FDRA apply to all PUSCH occasions. | - Retransmission may need some specification changes: - Option 1: Rel-17 Single DCI scheduling multi-PUSCH - Option 2: Legacy CG mechanism, CG timer, …CG timer starting after last CG PUSCH occasion or after each slot or after each N PUSCH occasion (DCI per PUSCH or DCI for multi-PUSCHs), - Option 3: NR-U retransmission mechanism. We prefer option 2 |
| Alt-C2 | - non-contiguous allocation for type 2 for TDD. - Can be applied also for retransmissions | - operation for CG type-1. Rel-17 gives the flexibility of having different K2 values for different PUSCHs, so non-contiguous allocation can be supported. But, how do we indicate the K2 for type -1 CG. K2 needs the DCI as a reference. Unless we use Rel-16 design for type-1 CG. - Spec impact if SLIVs restriction are to be relaxed |

## HARQ Process ID determination

In RAN1#112bis meeting, the following agreement has been reached:

Agreement:

From RAN1 perspective, for determination of HARQ process Ids associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH:

- The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying the following formula, whichever is applicable

  - HARQ Process ID = [floor(X*(CURRENT_symbol – offset1) / periodicity) + offset2] modulo nrofHARQ-Processes

  - HARQ Process ID = [floor(X*(CURRENT_symbol – offset1) / periodicity) + offset2] modulo nrofHARQ-Processes + harq-ProcID-Offset2

    - FFS whether in formulas above X is outside or inside floor operation, i.e.

      - HARQ Process ID = [X*floor( (CURRENT_symbol – offset1) / periodicity) + offset2] modulo nrofHARQ-Processes

      - HARQ Process ID = [X*floor((CURRENT_symbol – offset1) / periodicity) + offset2] modulo nrofHARQ-Processes + harq-ProcID-Offset2

  - (Working Assumption) The HARQ process ID of the remaining configured/valid CG PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period by Y with module operation with nrofHARQ-Processes or module operation with (nrofHARQ-Processes + harq-ProcID-Offset2), whichever applicable.

    - FFS whether X=1 or X= the number of configured PUSCHs in the CG period

    - FFS whether Y =1 or a value larger than 1, e.g. Y=2.

      - FFS: If Y>1, Y is determined based on RRC

    - FFS whether Offset 1= 0 or can be a non-zero value.

      - FFS: If offset1 is non-zero, how offset1 is determined (i.e., based on RRC)

    - FFS whether Offset 2= 0 or can be a non-zero value.

      - FFS: If offset2 is non-zero, how offset2 is determined (i.e., based on RRC or dynamically)

- Note1: The equations will be updated accordingly when FFSs are clarified, e.g., if X=1, remove X; if Y=1, remove Y; if non-zero offset1 or Offset 2 is not supported, remove offset 1 or Offset 2.

- Note2: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB.

We support the formula above with X outside the floor operation as below:

      - HARQ Process ID = [X*floor( (CURRENT_symbol – offset1) / periodicity) + offset2] modulo nrofHARQ-Processes

      - HARQ Process ID = [X*floor((CURRENT_symbol – offset1) / periodicity) + offset2] modulo nrofHARQ-Processes + harq-ProcID-Offset2

When X is inside the formula, it can create a lot of ambiguity in many cases especially if non-integer XR traffic periodicity will be agreed to be used in RAN2.

Let’s take the case of

- SCS = 30 kHz

- FPS = 120

- X = 9

- NumberHARQ = 16

The periodicity in terms of OFDM symbols is 28*1000/120 = 233.3333

|  | CURRENT_symbol |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| X inside or outside floor | 0 | 232 | 233 | 234 | 235 |
| floor(X* CURRENT_symbol /(28*(1000/120))) | 0 | 8 | 8 | 9 | 9 |
| X*floor(CURRENT_symbol /(28*(1000/120))) | 0 | 0 | 0 | 9 | 9 |

When the X is inside the floor, the transition to the new HARQ Process ID for the new periodicity can happen in stages and can be ambiguous depending on how the rounding up or down is decided for the periodicity. When X is outside the floor there is no ambiguity and the rounding-up of the periodicity is needed in this case.

Therefore, to avoid any future rounding issues, we prefer to multiply floor(CURRENT_symbol/periodicity) by X.

Also, the HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period and this can also be captured in the formula with CG occasion index I as below

HARQ Process ID = [X * floor(CURRENT_symbol/periodicity) + I ] modulo nrofHARQ-Processes

Regarding offset 1, we understand the intention of introducing it. However, we are still not sure how this will work in practice with dynamic jitter. If the jitter is changing and the UE is adjusting the HARQ ID with offset 1, how will the gNB determine the offset and differentiate between a CG occasion missed because of the jitter and a CG occasion that has not been detected. If the offset 1 is RRC configured, does that mean the jitter is semi-static? For example, if the UE has missed the first CG occasion because of the jitter in the current CG period and has missed the first two occasions in the following CG period, does the UE still apply the same offset 1?

- For the HPI determination, support the HPI formula with X outside the floor operation, i.e.

      - HARQ Process ID = [X*floor( (CURRENT_symbol – offset1) / periodicity) + offset2] modulo nrofHARQ-Processes

      - HARQ Process ID = [X*floor((CURRENT_symbol – offset1) / periodicity) + offset2] modulo nrofHARQ-Processes + harq-ProcID-Offset2

- The HPI of the remaining PUSCHs in the CG period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period and this can also be captured in the formula with CG occasion index I as below

- HARQ Process ID = [X * floor(CURRENT_symbol/periodicity) + I ] modulo nrofHARQ-Processes

# Dynamic indication of unused CG PUSCH occasion(s)

Providing assistance information to the gNB is always useful. gNB doesn’t have much awareness about the UL AR traffic and further information from the UE about the need to cancel unused CG-PUSCH occasions can help optimize the gNB scheduling. When UL AR packets arrive at the UE modem, the UE can determine the number of the CG-PUSCH occasions required to transmit the packets and can signal this information to the gNB. This allows the network to free some CG-PUSCH occasions and recycle them to schedule other UEs which results in better system capacity and spectral efficiency.

This will help mitigate the limitation of using multiple CG-PUSCH transmission occasions in the CG period for the UL AR traffic. In fact, the resources semi-statically allocated for the UL AR data transmission may become over-allocated for some UL AR frames since the gNB does not receive an accurate BSR in time. This can decrease the overall capacity of the system. To mitigate this limitation, it was agreed in the SI that the UE should be able to dynamically inform the gNB via UCI (e.g., CG-UCI or a new UCI) of any unused PUSCH occasions and the gNB can reallocate the freed resources to other UE transmissions which is expected to increase the system capacity.

## Content of the UCI indicating CG PUSCH TOs as unused

In RAN1#112bis-e, the following agreement has been reached for the content of the UCI indicating the CG PUSCH TOs as unused:

Agreement

For dynamic indication of unused CG PUSCH transmission occasion(s) based on a UCI, the indicated “unused” CG PUSCH TO(s), if any, by the UCI in a CG PUSCH for a CG configuration

- can be consecutive or non-consecutive CG PUSCH TO(s) in time domain [in one CG period]

- FFS whether/how the unused TO(s) can be associated to multiple CG configuration.

Note: FFSs and further details in corresponding agreement in RAN1#112 for the selected option are remained for further discussion

Note: Above corresponds to Option 2 (w.r.t. agreement in RAN1#112)

Agreement

The UTO-UCI provides a bitmap where a bit corresponds to a TO within a time duration/range. The bit indicates whether the TO is “unused”.

- FFS: Details including time duration/range

Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience.

We support the UCI indication of the unused TO(s) to operate for multiple CG configurations, as the XR traffic needs multiple CG configurations (CG configuration for UL AR, CG configuration for Pose/Control information, CG configuration for Audio traffic, …). The CG configuration index can be included in the new UCI and one CG configuration can be indicated at a time in one UCI.

- Support the UCI indication of the unused TO(s) to operate for multiple CG configurations

Regarding the details about the time duration/range, we prefer to have the time duration equal to one CG period for the Rel-18 multi-PUSCHs Configured Grant configuration and to have the range configurable for the legacy Configured configurations (with one single PUSCH per CG) and can be one or multiple CG periods.

- The time duration/range is:

- One CG period for the Rel-18 multi-PUSCHs CG configuration

- One or multiple CG periods for the legacy CG configuration (with single PUSCH per CG)

## UCI transmission occasions

In RAN1#112bis-e, the following agreement has been reached for location of the UCI transmission occasions:

Agreement

- Option 1: For a CG PUSCH configuration, the UTO-UCI is included in every CG PUSCH that is transmitted (that is Option 1 in corresponding agreement in RAN1#112)

  - FFS details

- Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience.

It needs to be discussed whether the exact information is transmitted each time on every CG PUSCH occasion or if the UE can overwrite the UCI content. If the information provided by UTO-UCI should be consistent, then definition of consistency should be defined.

- If the information provided by UTO-UCI should be consistent, then the exact definition of consistency should be defined.

## How the UCI is sent

In RAN1#112bis-e, the following agreements have been reached on how the UCI is sent:

Agreement

The UCI that provides information about unused CG PUSCH transmission occasions is defined as a “new UCI” (i.e. Alt. 1 of previous agreement).

Agreement

- With respect to PHY two-level priority, for a configured grant PUSCH configuration, the “UTO-UCI” has the same priority level as the configured grant PUSCH.

- Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience.

Agreement

The existing CG-UCI encoding and multiplexing procedures are reused for encoding the “UTO-UCI” in a configured grant PUSCH in absence or presence of other UCIs being multiplexed in the PUSCH, by applying the following adjustments:

- The “UTO-UCI” is used instead of CG-UCI in the corresponding procedures for encoding of CG-UCI and/or HARQ-ACK, whichever is present.

- For determining the beta-offset,

  - Beta offset is configured for the “UTO-UCI”

    - If UTO-UCI and HARQ-ACK is not jointly encoded, the beta offset for the “UTO-UCI” is used in the procedures instead of CG-UCI beta offset

    - If UTO-UCI and HARQ-ACK is jointly encoded, HARQ-ACK beta offset is used in the procedures instead of CG-UCI beta offset

- FFS on sequence generation order between UTO-UCI and HARQ-ACK

- FFS on dropping rule between UTO-UCI and HARQ-ACK when joint encoding is not configured

- Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience.

When joint encoding between UTO-UCI and HARQ-ACK is not configured, UTO-UCI should be transmitted and the HARQ-ACK can be deprioritized if associated with the eMBB traffic and transmitted by the UE on demand from the gNB. The UE can transmit the missed HARQ-ACK as a report when requested by a specific UL-DCI. Alternatively, The UE can also transmit the missed HARQ-ACK autonomously later using an originally assigned PUCCH resources.

- When joint encoding between UTO-UCI and HARQ-ACK is not configured, UTO-UCI should be transmitted and the eMBB HARQ-ACK can be deprioritized and retransmitted by the UE on demand from the gNB.

HARQ-ACK associated to the XR traffic or URLLC traffic should be treated differently compared to the HARQ-ACK of the eMBB traffic which have less stringent latency and reliability requirement.

- Support differentiated treatement of HARQ-ACK associated to the XR or URLLC traffic and the HARQ-ACK associated to the eMBB traffic.

- FFS: dynamic signalling of the joint-encoding indication

# Conclusion

In this contribution, we have presented several proposals to specify the support of multiple CG PUSCH transmission occasions in a CG period:

We made the following proposals:

- The TDRA determination based on NR-U framework should be extended to XR with some enhancements.

- The legacy configuredGrantTimer can be used for each PUSCH occasion of the multiple CG PUSCH transmission occasions instead of the cg-RetransmissionTimer used in NR-U.

- For the HPI determination, support the HPI formula with X outside the floor operation, i.e.

      - HARQ Process ID = [X*floor( (CURRENT_symbol – offset1) / periodicity) + offset2] modulo nrofHARQ-Processes

      - HARQ Process ID = [X*floor((CURRENT_symbol – offset1) / periodicity) + offset2] modulo nrofHARQ-Processes + harq-ProcID-Offset2

- The HPI of the remaining PUSCHs in the CG period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period and this can also be captured in the formula with CG occasion index I as below

- HARQ Process ID = [X * floor(CURRENT_symbol/periodicity) + I ] modulo nrofHARQ-Processes

- Support the UCI indication of the unused TO(s) to operate for multiple CG configurations

- The time duration/range is:

- One CG period for the Rel-18 multi-PUSCHs CG configuration

- One or multiple CG periods for the legacy CG configuration (with single PUSCH per CG)

- If the information provided by UTO-UCI should be consistent, then the exact definition of consistency should be defined.

- When joint encoding between UTO-UCI and HARQ-ACK is not configured, UTO-UCI should be transmitted and the eMBB HARQ-ACK can be deprioritized and retransmitted by the UE on demand from the gNB.

- Support differentiated treatement of HARQ-ACK associated to the XR or URLLC traffic and the HARQ-ACK associated to the eMBB traffic.

- FFS: dynamic signalling of the joint-encoding indication

# References

- RP-223502, “New WID on XR Enhancements for NR” Nokia, Qualcomm, 3GPP TSG RAN Meeting #98-e, Dec. 2022