3GPP TSG RAN WG1 #113		R1-2305101

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.5.2

Source: 	CMCC

Title: 	Discussion on DL/UL carrier phase positioning

Document for:	Discussion and Decision

Introduction

In RAN#98-e meeting, a new WID on expanded and improved NR positioning was approved [1], in which the application of NR positioning is further expanded to enable sidelink positioning and ranging, improved accuracy, integrity and power efficiency, and positioning for RedCap UEs. To further improve positioning accuracy, the following objectives on carrier phase positioning were identified:

- Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4]. - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements. - Specify measurements that are limited to a single carrier/PFL. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements without measurement gaps in connected and inactive mode (including PRS measurement period/reporting) and procedures [RAN4].

In this contribution, we further provide our views on carrier phase positioning based on the outcome of previous RAN1 meetings [2].

Carrier phase measurement and reporting

In RAN1#112 meeting, the following agreement on carrier phase measurement was made:

Agreement To enable UE-based and UE-assisted NR carrier phase positioning (CPP), one or both of the following new measurements should be introduced: - DL carrier phase (CP), which is obtained by a UE measuring the DL PRS signal(s) from a TRP. - FFS: The detailed definition of the DL CP - DL carrier phase difference (CPD), which is the difference of two DL CPs from two TRPs - FFS: The detailed definition of the DL CPD To enable NG-RAN node-assisted NR carrier phase positioning (CPP), the following new measurement should be introduced: - UL carrier phase (CP), which is obtained by a TRP measuring the UL SRS for positioning or MIMO SRS from a UE. - FFS: The detailed definition of the UL CP

There was a remaining open issue regarding the information that should be reported with the DL/UL carrier phase and/or DL carrier phase difference.

For the new measurements, at least timestamp and measurement quality should be reported. In addition, for the DL carrier phase difference (CPD) measurement, based on the following agreed definition, which is similar as DL RSTD, the reference TRP ID, DL PRS resource set ID and/or DL PRS resource(s) ID should be reported if UE determines a different reference than that indicated by the network.

Agreement For NR DL reference signal carrier phase difference (RSCPD) measurement for NR CPP, the RSCPD is defined as the difference of RSCPs measured from the DL PRS signals from target TRP and reference TRP. - FFS: whether/how to define per path RSCPD - Note: Whether/how to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase difference measurement for NR CPP

Therefore, we propose that:

Proposal 1: The following information should be considered in the CPP measurement reporting:

- The timestamp

- The measurement quality

- Reference TRP / DL PRS resource set / DL PRS resource(s) ID

In addition, regarding the timestamp in the measurement reporting, the following two options were agreed in the last RAN1 meeting for further down-selection:

Agreement Adopt one of the following options for a timestamp associated with a reported RSCP/RSCPD measurement (make the decision in RAN1#113): - Option 1: - NR-TimeStamp, currently defined in TS 37.355, is reused as the timestamp with the granularity of a slot. - FFS: Whether to clarify in the specification the reported RSCP/RSCPD value presents the RSCP/RSCPD of a specific OFDM symbol within the slot identified by the NR-TimeStamp. - Option 2: - NR-TimeStamp, currently defined in TS 37.355, should be enhanced to include the OFDM symbol index in a slot, as the timestamp for RSCP/RSCPD measurements.

The reporting granularity of NR-TimeStamp defined in TS 37.355 is slot level, for reporting of RSCP or RSCPD, whether finer reporting granularity on the timestamp was debated. In our views, for carrier phase measurement reporting, the carrier phase may vary across different OFDM symbols within a slot. From this perspective, we prefer Option 2, where the timestamp associated with RSCP/RSCPD can be symbol level.

Proposal 2: For a timestamp associated with a reported RSCP/RSCPD measurement, support Option 2 agreed in RAN1#112bis-e meeting:

- Option 2: NR-TimeStamp, currently defined in TS 37.355, should be enhanced to include the OFDM symbol index in a slot, as the timestamp for RSCP/RSCPD measurements.

Carrier phase multipath mitigation

In the last RAN1 meeting, the following agreement on the definition of carrier phase measurement was made:

Agreement NR DL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the DL PRS signals configured for the measurement. A RSCP is associated with a specific RF frequency. - FFS: the reference point of the RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: Whether to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase measurement for NR CPP Agreement NR UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the UL SRS signal for positioning purpose configured for the measurement. A UL RSCP is associated with a specific RF frequency. - FFS: the reference point of the UL RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: The support of MIMO SRS for positioning is transparent to UE

Based on the agreement, the DL/UL RSCP of i-th path is defined. In the note, there was a separate issue on whether only the first path or additional paths will be supported, which is related to carrier phase multipath mitigation. During the discussion in the last RAN1 meeting, companies shared different views on the necessity to consider the carrier phase of additional paths, and the latest intermediate proposal was as follows:

- (H)(Round 1) Proposal 4-1 Select one of the following options for NR carrier phase positioning: - Option 1: Report the carrier phase of the first path only - Option 2: Report the carrier phase of the first path, and optionally, the additional paths

In our views, the benefits of providing carrier phase of additional paths are not clear. The reason is that the value range of carrier phase is [0, 2pi] and is repeated for every wavelength, to obtain the carrier phase of an additional path on top of the first path, the UE should be able to abstract additional paths within a wavelength. Suppose that the carrier frequency is 3.5GHz, the wavelength of the carrier is about 0.86cm. In this sense, it would be difficult for a UE to obtain carrier phase of additional paths. Therefore, we prefer to consider only the carrier phase of the first path.

Proposal 3: For NR carrier phase positioning, support only the carrier phase of the first path.

Differential carrier phase positioning

In RAN1#109-e meeting, the following agreement was made to consider the use of PRU in the evaluation of carrier phase positioning:

Agreement The use of PRUs to facilitate NR carrier phase positioning can be evaluated in the SI by RAN1.

The use of PRU, which is similar as the reference stations deployed for carrier phase based RTK positioning, can be adopted to estimate and mitigate the carrier phase synchronization errors  between TRPs.

Assistance information for UE-based CPP

For UE-assisted DL positioning, a PRU (which is a UE) that capable of performing carrier phase measurement will measure carrier phase and reported to LMF. LMF can then obtains the errors and compensate in location calculation. For UL positioning, the carrier phase measurement of a PRU is reported from gNB to LMF and LMF performs the compensation. For UE-based DL positioning, on the other hand, assistance information provided to the target UE should be enhanced for the phase error compensation. In the last meeting, such issue has been discussed and the latest intermediate FL proposal is as follows:

- (Email)(H)(Round 4) Proposal 4.2-1 - For UE-based carrier phase positioning, consider supporting one or both of the following options (aiming to make a decision in RAN1#113): - Option 1: Enabling LMF to forward the DL carrier phase measurement reported by a PRU and other PRU information to a target UE for UE-based carrier phase positioning in the positioning assistance data. - Note: Whether DL carrier phase measurement is DL RSCP and/or DL RSCPD depends on which of them is (are) supported. - FFS: other PRU information, such as PRU location, to be included in the positioning assistance data - Option 2: Enabling LMF to provide, optionally, the differential corrections to a target UE for UE-based carrier phase positioning. - FFS: the details of the correction information, which may include the corrections for TRP phase errors, ARP/APC errors, etc.

In our views, both options are feasible, and we think that as they are essentially the same, it seems not necessary to support both options. During the discussion in the last meeting, some companies argued that PRU is not within the scope and then Option 1 should not be pursued. We think that the discussion regarding Option 1 is definitely included in the WID scope and is key to improve the positioning accuracy using CPP, to ease companies’ concern, we could replace “PRU” to “UE”, similar as what we did in sidelink positioning, where no server UE is appeared in RAN1’s agreements.

Proposal 4: For UE-based carrier phase positioning, support one of the following options:

  - Option 1: Enabling LMF to forward the DL carrier phase measurement reported by a UE and other related information (e.g., UE location) to a target UE for UE-based carrier phase positioning in the positioning assistance data.

  - Option 2: Enabling LMF to provide, optionally, the differential corrections to a target UE for UE-based carrier phase positioning.

Conclusions

In this contribution, we provide our views on the carrier phase positioning, and the following proposals are made:

Proposal 1: The following information should be considered in the CPP measurement reporting:

- The timestamp

- The measurement quality

- Reference TRP / DL PRS resource set / DL PRS resource(s) ID

Proposal 2: For a timestamp associated with a reported RSCP/RSCPD measurement, support Option 2 agreed in RAN1#112bis-e meeting:

- Option 2: NR-TimeStamp, currently defined in TS 37.355, should be enhanced to include the OFDM symbol index in a slot, as the timestamp for RSCP/RSCPD measurements.

Proposal 3: For NR carrier phase positioning, support only the carrier phase of the first path.

Proposal 4: For UE-based carrier phase positioning, support one of the following options:

  - Option 1: Enabling LMF to forward the DL carrier phase measurement reported by a UE and other related information (e.g., UE location) to a target UE for UE-based carrier phase positioning in the positioning assistance data.

  - Option 2: Enabling LMF to provide, optionally, the differential corrections to a target UE for UE-based carrier phase positioning.

References

- RP-223549, New WID on Expanded and Improved NR Positioning, Intel Corporation, CATT, Ericsson, 3GPP TSG RAN Meeting 98-e.

- R1-2304243, RAN1 agreements for Rel-18 WI on Expanded and Improved NR Positioning, Rapporteur (Intel Corporation), 3GPP TSG RAN WG1 Meeting #112bis-e