3GPP TSG-RAN WG1 Meeting #113	R1- 2305161

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.2.2.2

Source:	NVIDIA

Title:	AI and ML for CSI feedback enhancement

Document for:	Discussion

# 1	Introduction

The approval of the Rel-18 work package marks the start of 5G Advanced evolution. The package includes a study item on AI/ML for NR air interface, and the work item description can be found in [1].

The initial use cases focused in this study include:

- CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

- Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

- Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

For the use cases under consideration, the study aims to finalize representative sub use cases and assess potential specification impact.

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

  - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference),  and management of data and AI/ML model, per RAN1 input

    - Collaboration level specific specification impact per use case

  - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2

    - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable

    - Consider the need and implications for AI/ML processing capabilities definition

In the previous RAN1 meetings, initial agreements were made to provide directions for further investigation. In this contribution, we discuss relevant aspects of AI/ML for CSI feedback enhancement.

# 2	Representative sub-use cases

AI/ML techniques, such as autoencoders, can be used in CSI compression and feedback. The basic principle of an autoencoder is to take an input, perform non-linear compression by an encoder to a lower-dimensional latent representation, and decompress the latent representation by a decoder to a defined target.

In the context of CSI compression and feedback, the UE’s estimated CSI is taken as the input to the encoder. The output of the UE’s encoder (i.e., the latent representation of the channel in a quantized form) is transmitted over the air interface to the gNB. The gNB uses a matching decoder to reconstruct the CSI. Hence, the autoencoder model is divided between the UE and gNB, creating an inter-node dependency for model training and model management.

Figure 1 provides an illustration of autoencoder based CSI feedback.

Figure 1: Illustration of CSI autoencoder.

Observation 1: Autoencoder based CSI feedback is a promising AI/ML technique for CSI feedback enhancement.

Using autoencoders for CSI compression is in line with the agreement that for the evaluation of the AI/ML based CSI compression sub use cases, a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information. At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.

At RAN1#109-e, it was agreed that spatial-frequency domain CSI compression using two-sided AI model is selected as one representative sub use case. It was further noted that all pre-processing/post-processing, quantization/de-quantization are within the scope of the sub use case. The spatial-frequency domain CSI compression using two-sided AI model can compress either the raw channel matrix (i.e., full Tx * Rx MIMO channel) estimated by UE or the eigenvector(s) of the raw channel matrix estimated by UE, etc. Raw channel can be in spatial-frequency domain or angular-delay domain. The eigenvector(s) of the raw channel matrix essentially represents precoding matrix that can be in spatial-frequency domain or represented using angular-delay domain projection. Raw channel matrix-based CSI feedback provides gNB with the most flexibility in determining downlink transmission, though it deviates from the existing codebook-based CSI feedback. In contrast, the channel eigenvector-based CSI feedback is more in line with the conventional codebook-based CSI feedback methods, but it does not provide as much flexibility as the raw channel matrix-based CSI feedback. Both types of CSI feedback schemes are of interest and deserve study.

Proposal 1: Both autoencoders with raw channel matrix as input and autoencoders with eigenvector(s) of raw channel matrix as input are in scope of the spatial-frequency domain CSI compression using two-sided AI model.

In our companion contribution [2], we present the performance results of the trained autoencoders in terms of the cosine similarity between true and reconstructed eigenvectors of raw channel matrix. The evaluation results highlight the feasibility of spatial-frequency domain CSI compression using two-sided AI model.

Observation 2: Evaluation results demonstrate the feasibility of spatial-frequency domain CSI compression using two-sided AI model.

During the study on spatial-frequency domain CSI compression using two-sided AI model, different terminologies have been used. To align terminology, the following was agreed at RAN1#111:

- Output CSI assumed at UE is referred as output-CSI-UE.

- Input CSI assumed at network is referred as input-CSI-NW.

AI/ML techniques can also be used in CSI prediction. The problem of the current CSI reporting framework is that there is a delay between the time to which the reported CSI relates and the time when the BS receives the CSI report. In 5G non-terrestrial networks for satellite communications, the delay can range from a few milliseconds to hundreds of milliseconds. Such large delay can well cause the CSI to become outdated. In terrestrial networks where the delay is not large, the wireless channel can vary rapidly due to, e.g., high UE mobility, which can also cause the CSI to become outdated. CSI prediction reporting using, e.g., AI/ML algorithms, is one approach to mitigating the effect of the outdated CSI in the CSI reporting framework. In addition, CSI prediction can help reduce reference signal overhead and measurement reporting overhead. Therefore, it is of high interest to study AI/ML based algorithms for CSI prediction.

Unlike AI/ML based CSI compression where a two-sided structure (CSI encoder at UE and CSI decoder at gNB) is needed, a one-sided structure is sufficient for AI/ML based CSI prediction. The AI/ML inference of the one-sided model can be performed at either gNB or UE.

In our companion contribution [2], we present the performance results of AI/ML based CSI prediction. The evaluation results highlight the performance gains of CSI prediction using one-sided AI model.

Observation 3: Evaluation results demonstrate the performance gains of CSI prediction using one-sided AI model.

The inference of one-sided AI/ML model for CSI prediction can be performed at either gNB or UE. At RAN1#111, it was agreed that time domain CSI prediction using UE sided model is selected as a representative sub-use case for CSI enhancement. Time domain CSI prediction using gNB sided model should also be studied to assess the specification impact of performing CSI prediction at gNB side.

Proposal 2: The inference of one-sided AI/ML model for CSI prediction can be performed at either gNB or UE. Besides assessing the specification impact of performing CSI prediction at UE side, companies are encouraged to assess the specification impact of performing the CSI prediction at gNB side as well.

In addition to spatial-frequency domain CSI compression using two-sided AI model and CSI prediction using one-sided model, there are additional sub-use cases proposed by different sources. The following table summarizes the list of proposed sub-use cases and their discussion status in RAN1.

Table 1: Potential sub-use cases for CSI feedback enhancement and discussion status.

| Sub-use case | Discussion status |
| --- | --- |
| Spatial-frequency domain CSI compression using two-sided AI model | Selected |
| Temporal-spatial-frequency domain CSI compression using two-sided model | Not selected |
| Improving the CSI accuracy based on traditional codebook design using one-sided model | Not selected |
| Time-domain CSI prediction using one-sided model | Selected (with UE-sided model) |
| CSI-RS configuration and overhead reduction | Not selected |
| Resource allocation and scheduling | Not selected |
| Joint CSI prediction and compression | Not selected |

Though these potential other sub-use cases might be of interest for different considerations, it is more important to develop a thorough understanding of a limited number of sub-use cases, by carefully assessing their performance in comparison with traditional methods and the associated potential specification impacts that enable the AI/ML solution for the sub-use cases. This thorough understanding will lay the foundation for future use cases leveraging AI/ML techniques, including e.g., the other potential sub-use cases for CSI feedback enhancements.

Proposal 3: Focus on the sub-use case of spatial-frequency domain CSI compression using two-sided AI model and the sub-use case of time-domain CSI prediction using one-sided model to develop a thorough understanding of the performance of the AI models and the associated potential specification impacts.

Proposal 4: The study of potential sub-use cases other than the spatial-frequency domain CSI compression using two-sided AI model and time-domain CSI prediction using one-sided model can be postponed.

# 3	Potential specification impact

For autoencoder based CSI feedback, one important question is how to train an autoencoder model for deployment. There are different training options that have various levels of collaboration between UE and gNB and can lead to different levels of specification impact.

It was agreed that in CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied:

- Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided.

- Type 2: Joint training of the two-sided model at network side and UE side, respectively.

- Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively.

Figure 2 illustrates Type-1 training. In this case, joint training is performed at a single entity in a single training session. This entity can be at the network or UE side. For example, if the training is done at the network side, the network can transfer the encoder model to the UE, and vice versa.

Figure 2: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided.

Figure 3 illustrates Type-2 training. In this case, joint training is performed at two sides in a single training session. The encoder model and decoder model are trained at UE side and at network side, respectively. But the models are jointly trained in the same loop for forward propagation and backward propagation, by exchanging forward activation and backward gradient between UE-side entity and network-side entity.

As an example, Type-2 training procedure may go as follows. For each forward propagation and backward propagation loop (assuming that the dataset between UE side and network side is aligned):

- Step 1: UE side generates the forward propagation results (i.e., CSI feedback) based on the data samples, and sends the forward propagation results to network side.

- Step 2: Network side reconstructs the CSI based on forward propagation results, trains the CSI reconstruction part, and generates the backward propagation information (e.g., gradients), which are then sent to UE side.

- Step 3: UE side trains the CSI generation part based on the backward propagation information from network side.

Figure 3: Joint training of the two-sided model at network side and UE side, respectively.

The exchange of forward and backward propagation results during model training can be offline or over air interface. If the exchange is carried out over air interface, the signalling overhead would be significant. Therefore, it is sensible to deprioritize the study of training collaboration type 2 over the air interface for model training in CSI compression using two-sided model use case.

Figure 4 illustrates Type-3 training. In this case, separate training is performed at two sides in different training sessions. Unlike Type-2 training, there is no collaboration during training in Type-3 training. But it is necessary to have some coordination outside the training process to ensure that the UE-side CSI encoder model and the NW-side CSI decoder are compatible.

As illustrated in Figure 4, we can have separate training starting with UE side (the left part of the figure), or separate training starting with NW side (the right part of the figure).

As an example, Type-3 training procedure starting with UE side training (UE-first training) may go as follows.

- Step 1: UE side trains the UE side CSI generation part and the UE side CSI reconstruction part (which is not used for inference) jointly.

- Step 2: After UE side training is finished, UE side shares network side with a set of information (e.g., a training dataset consisting of (encoder output, target CSI)) that is used by the network side to be able to train the CSI reconstruction part.

- Step3: Network side trains the network side CSI reconstruction part based on the received set of information.

Similarly, as an example, Type-3 training procedure starting with network side training (network-first training) may go as follows.

- Step 1: Network side trains the network side CSI generation part (which is not used for inference) and the network side CSI reconstruction part jointly.

- Step 2: After network side training is finished, network side shares UE side with a set of information (e.g., a training dataset consisting of (Input of CSI encoder, encoder output)) that is used by the UE side to be able to train the UE side CSI generation part.

- Step 3: UE side trains the UE side CSI generation part based on the received set of information.

Figure 4: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively.

There are pros and cons for each training type. They are good starting points to investigate various levels of collaboration between UE and gNB, which is a key objective of the study item.

- Type-1 training offers the best possible performance as the UE-side and network-side models are trained jointly. But deployment and management of the models can be challenging. Take Type-1 training at network side for example. If UE vendors are not involved, the UE-side model designed and trained by the network side entity is not optimized for the targeted UE types or even may not be compatible with the UEs’ implementations. By involving a UE vendor, the network side entity can ensure that the trained models are suitable for inference, but a UE vendor needs to disclose its proprietary model information to the network side for joint training. In addition, if a common network-side model is used for multiple UE-side models, the network side entity needs to coordinate across UE vendors for joint training, and the release of a new UE type would trigger retraining across the vendors. Similar issues exist for Type-1 training at UE side.

- Type-2 training has the advantage of preserving proprietary information at each side, as one side does not need to disclose model information such as model structure to the other side. While it is feasible to develop compatible UE-side and network-side models, the two training entities need to coordinate to exchange forward and backward propagation results during the training iterations. The exchange can cause significant communication overhead if carried out over air interface. In addition, if a common network-side model is used for multiple UE-side models, the network side entity needs to coordinate across UE vendors for joint training, and the release of a new UE type would trigger retraining across the vendors. Similar issues exist if a common UE-side model is used for multiple network-side models.

- Type-3 training has the advantage of preserving proprietary information at each side, as one side does not need to disclose model information such as model structure to the other side. Furthermore, unlike Type-2 training, Type-3 training does not require collaboration during the training iterations, as coordination can occur outside the training process. When a common network-side model is used for multiple UE-side models (or vice versa), the retraining due to the release of a new UE type can be confined to involve only the associated UE vendor and the network vendor (i.e., it does not trigger retraining of the models of the other UE vendors), because the UE-side model and network-side model are trained separately.

Proposal 5: In CSI compression using two-sided model use case, study the pros and cons of the following AI/ML model training collaborations:

- Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided.

- Type 2: Joint training of the two-sided model at network side and UE side, respectively.

- Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively.

In CSI compression using two-sided model use case, it is necessary to study CQI determination in CSI report, if CQI in CSI report is configured. In a first option, CQI is not calculated based on the output of CSI reconstruction part from the realistic channel estimation, including a) CQI is calculated based on target CSI with realistic channel measurement, b) CQI is calculated based on target CSI with realistic channel measurement and potential adjustment, c) CQI is calculated based on legacy codebook. In a second option, CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including a) CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment and b) CQI is calculated using two stage approach where UE derives CQI using precoded CSI-RS transmitted with a reconstructed precoder.

Besides CQI determination, there are several other design aspects that require further study in CSI compression using two-sided model use case. One issue is related to network configuration for determining CSI payload size, e.g., possible CSI payload size, possible rank restriction and/or other related configuration. A related question is how UE determines/reports the actual CSI payload size and/or other CSI related information within constraints configured by the network. Another aspect is how to support the legacy CSI reporting principles including the priority rule regarding CSI collision handling and CSI omission, codebook subset restriction, CSI processing unit, etc.

As pointed out above, AI/ML model training can occur at gNB or UE side or both sides. In either case, the training entity can benefit from assistance from the other entity for training data collection. Relevant areas for discussion include training data type/size, training data source determination, and assistance signalling and procedure for training data collection. Specifically, to facilitate UE side data collection, it is beneficial to study enhancement of CSI-RS configuration to enable higher accuracy measurement, assistance information for UE data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site, etc., and signaling for triggering the data collection. To facilitate network side data collection, it is beneficial to study enhancement of SRS and/or CSI-RS measurement and/or CSI reporting to enable higher accuracy measurement, latency requirement for data collection, signaling for triggering the data collection, as well as contents of the ground-truth CSI including data sample type (e.g., precoding matrix, channel matrix etc.), data sample format (scaler quantization and/or codebook-based quantization) and assistance information (e.g., time stamps, and/or cell ID, assistance information for network data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site, etc., and data quality indicator).

Proposal 6: For AI/ML model training for CSI feedback enhancement, study potential specification impact related to training data type, training data source determination, and assistance signalling and procedure for training data collection.

Similar to many other functionalities in 3GPP systems, the usage of AI/ML model for a certain functionality should be under network control, if the functionality at one side cannot be made transparent to the other side. Therefore, assistance signalling and procedure for model configuration, model activation/deactivation, model recovery/termination, model selection, etc. should be investigated.

Proposal 7: For AI/ML based CSI feedback, study potential specification impact related to assistance signalling and procedure for model configuration, model activation/deactivation, model recovery/termination, and model selection.

AI/ML models are data driving. They are trained to learn patterns from data. But the environment of a mobile communication system is dynamic and changes over time, and thus the data also keeps changing. As the data from the environment changes, the AI/ML model performance may be degraded. Therefore, it is essential to monitor the AI/ML model performance and regularly update the model to maintain satisfactory model performance. To this end, assistance signalling and procedure for model performance monitoring and model update/tuning should be investigated.

In the use case of CSI compression using two-sided model, performance monitoring can be conducted at the network side and/or at the UE side. For network-side performance monitoring, network monitors the performance and make decisions of model activation/ deactivation/updating/switching. For UE-side performance monitoring, UE monitors the performance and reports to network, and network makes decisions of model activation/deactivation/updating/switching. The performance monitoring metrics/methods may include intermediate KPIs as monitoring metrics (e.g., SGCS), eventual KPIs (e.g., Throughput, hypothetical BLER, BLER, NACK/ACK), legacy CSI based monitoring (i.e., schemes using additional legacy CSI reporting), and other monitoring solutions (e.g.,	input or output data based monitoring such as data drift between training dataset and observed dataset and out-of-distribution detection).

In the use case of CSI compression using two-sided model, for intermediate KPIs based monitoring, network-side monitoring can be based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side. UE-side monitoring can be based on the output of the CSI reconstruction model, subject to the aligned format, associated to the CSI report, indicated by the NW or obtained from the network side. Alternatively, UE-side monitoring can be based on the output of the CSI reconstruction model at the UE-side.

In the use case of CSI compression using two-sided model, the fallback mechanism is the legacy non-AI/ML-based CSI feedback mode. It is necessary to investigate the co-existence between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode.

Proposal 8: For AI/ML based CSI feedback, study potential specification impact related to assistance signalling and procedure for model performance monitoring and model update/tuning.

When it comes to AI/ML model inference input, report/feedback of model input for inference (e.g., UE feedback as input for network side model inference) may be needed. In general, the type of model input, and model input acquisition and pre-processing may have potential specification impact. Similarly, when it comes to AI/ML model inference output, outputs generated by an AI/ML model may need to be delivered from gNB to UE or from UE to gNB. The post-processing of AI/ML model inference output may have potential specification impact as well.

In particular, in the use case of CSI compression using two-sided model, quantization method alignment between CSI generation part at UE and CSI reconstruction part at gNB is a critical issue. It is necessary to study the alignment of the quantization/dequantization method and the feedback message size between network and UE.

Proposal 9: For AI/ML based CSI feedback, study potential specification impact related to report/feedback of model input for inference (e.g., quantization and feedback message size), type of model input, and model input acquisition and pre-processing.

Proposal 10: For AI/ML based CSI feedback, study potential specification impact related to report/feedback of model inference output (e.g., quantization and feedback message size) and post-processing.

Different UEs may have different capabilities when it comes to the support of AI/ML algorithms for CSI feedback. Therefore, UE capability for AI/ML based CSI feedback including model training, model inference and model monitoring needs to be investigated and defined.

Proposal 11: For AI/ML based CSI feedback, study potential specification impact related to UE capability for AI/ML based beam prediction including model training, model inference and model monitoring.

At RAN1#112bis, it was agreed that functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. Which aspects should be specified as conditions of a Feature/FG available for functionality will need to be discussed specifically for AI/ML-based CSI feedback.

Proposal 12: For AI/ML based CSI feedback, study the aspects that should be specified as conditions of a Feature/FG available for functionality.

Also, at RAN1#112bis, it was agreed that model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will need to be discussed specifically for AI/ML-based CSI feedback.

Proposal 13: For AI/ML based CSI feedback, study the aspects that should be considered as additional conditions and how to include them into model description information during model identification.

# Conclusion

In the previous sections, we discuss general aspects of AI/ML framework for NR air interface and make the following observations:

Observation 1: Autoencoder based CSI feedback is a promising AI/ML technique for CSI feedback enhancement.

Observation 2: Evaluation results demonstrate the feasibility of spatial-frequency domain CSI compression using two-sided AI model.

Observation 3: Evaluation results demonstrate the performance gains of CSI prediction using one-sided AI model.

Based on the discussion in the previous sections we propose the following:

Proposal 1: Both autoencoders with raw channel matrix as input and autoencoders with eigenvector(s) of raw channel matrix as input are in scope of the spatial-frequency domain CSI compression using two-sided AI model.

Proposal 2: The inference of one-sided AI/ML model for CSI prediction can be performed at either gNB or UE. Besides assessing the specification impact of performing CSI prediction at UE side, companies are encouraged to assess the specification impact of performing the CSI prediction at gNB side as well.

Proposal 3: Focus on the sub-use case of spatial-frequency domain CSI compression using two-sided AI model and the sub-use case of CSI prediction using one-sided model to develop a thorough understanding of the performance of the AI models and the associated potential specification impacts.

Proposal 4: The study of potential sub-use cases other than the spatial-frequency domain CSI compression using two-sided AI model and CSI prediction using one-sided model can be postponed.

Proposal 5: In CSI compression using two-sided model use case, study the pros and cons of the following AI/ML model training collaborations:

- Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided.

- Type 2: Joint training of the two-sided model at network side and UE side, respectively.

- Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively.

Proposal 6: For AI/ML model training for CSI feedback enhancement, study potential specification impact related to training data type, training data source determination, and assistance signalling and procedure for training data collection.

Proposal 7: For AI/ML based CSI feedback, study potential specification impact related to assistance signalling and procedure for model configuration, model activation/deactivation, model recovery/termination, and model selection.

Proposal 8: For AI/ML based CSI feedback, study potential specification impact related to assistance signalling and procedure for model performance monitoring and model update/tuning.

Proposal 9: For AI/ML based CSI feedback, study potential specification impact related to report/feedback of model input for inference (e.g., quantization and feedback message size), type of model input, and model input acquisition and pre-processing.

Proposal 10: For AI/ML based CSI feedback, study potential specification impact related to report/feedback of model inference output (e.g., quantization and feedback message size) and post-processing.

Proposal 11: For AI/ML based CSI feedback, study potential specification impact related to UE capability for AI/ML based beam prediction including model training, model inference and model monitoring.

Proposal 12: For AI/ML based CSI feedback, study the aspects that should be specified as conditions of a Feature/FG available for functionality.

Proposal 13: For AI/ML based CSI feedback, study the aspects that should be considered as additional conditions and how to include them into model description information during model identification.

# Reference

RP-221348, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” 3GPP TSG RAN #96, June 2022.

R1-2305160, “Evaluation of AI and ML for CSI feedback enhancement,” RAN1#113, NVIDIA.