3GPP TSG-RAN WG1 #113	R1-2305166

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.17

Source:	Ericsson, Qualcomm

Title:	Initial higher layer parameters for endorsed TEI-18 on single DCI scheduling multiple PUSCHs

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, we discuss the higher layer parameters for the agreed TEI-18 for single DCI scheduling multiple PUSCHs with non-consecutive allocations in FR1.

# 2	Discussion

In Rel-16, the feature multi-PUSCH scheduling was introduced whereby a single DCI 0_1 can schedule up to 8 PUSCHs. The feature is agnostic to subcarrier spacing, frequency range, and whether or not shared spectrum access is required for the frequency band. The only restriction in Rel-16 specifications is that the multiple scheduled PUSCHs occur in contiguous slots. The corresponding Rel-16 UE feature parameter is as follows (see [1])

| multiPUSCH-UL-grant-r16 Indicates whether the UE supports scheduling up to 8 PUSCH with a single DCI 0_1. This capability is also applicable to a frequency band that does not require shared spectrum access. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

In Rel-17, multi-PUSCH scheduling was extended to support non-contiguous slots in addition to contiguous slots. In contrast this feature is not agnostic to frequency range; it is limited to FR2 which includes sub-ranges FR2-1 (up to 52.6 GHz) and the newly introduced FR2-2 (up to 71 GHz). For FR2-1 the UE feature is limited to 120 kHz only. The corresponding Rel-17 UE feature parameter for FR2-1 is given is as follows (see [2]):

| multiPUSCH-SingleDCI-FR2-1-SCS-120kHz-r17 Indicates whether the UE supports multi-PUSCH scheduling by single DCI for the operation with 120kHz SCS in FR2-1 with non-contiguous allocation. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

During last meeting the following was agreed that extend the Rel-17 feature to FR1.

Agreement

Introduce UE feature(s) for multi-PUSCH scheduling with single DCI 0_1 for non-contiguous slots in FR1 for all defined SCSs

- Note: there is no RAN1 impact

The extension in Rel-17 to non-contiguous slots was enabled by RRC parameter extendedK2 as shown below (see[3])

-- ASN1START -- TAG-PUSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-START PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation PUSCH-TimeDomainResourceAllocation ::= SEQUENCE { k2 INTEGER(0..32) OPTIONAL, -- Need S mappingType ENUMERATED {typeA, typeB}, startSymbolAndLength INTEGER (0..127) } PUSCH-TimeDomainResourceAllocationList-r16 ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations-r16)) OF PUSCH-TimeDomainResourceAllocation-r16 PUSCH-TimeDomainResourceAllocation-r16 ::= SEQUENCE { k2-r16 INTEGER(0..32) OPTIONAL, -- Need S puschAllocationList-r16 SEQUENCE (SIZE(1..maxNrofMultiplePUSCHs-r16)) OF PUSCH-Allocation-r16, ... } PUSCH-Allocation-r16 ::= SEQUENCE { mappingType-r16 ENUMERATED {typeA, typeB} OPTIONAL, -- Cond NotFormat01-02-Or-TypeA startSymbolAndLength-r16 INTEGER (0..127) OPTIONAL, -- Cond NotFormat01-02-Or-TypeA startSymbol-r16 INTEGER (0..13) OPTIONAL, -- Cond RepTypeB length-r16 INTEGER (1..14) OPTIONAL, -- Cond RepTypeB numberOfRepetitions-r16 ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16} OPTIONAL, -- Cond Format01-02 ..., [[ numberOfRepetitionsExt-r17 ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16, n20, n24, n28, n32, spare4, spare3, spare2, spare1} OPTIONAL, -- Cond Format01-02-For-TypeA numberOfSlotsTBoMS-r17 ENUMERATED {n1, n2, n4, n8, spare4, spare3, spare2, spare1} OPTIONAL, -- Need R extendedK2-r17 INTEGER (0..128) OPTIONAL -- Cond MultiPUSCH ]] } -- TAG-PUSCH-TIMEDOMAINRESOURCEALLOCATIONLIST-STOP -- ASN1STOP

extendedK2 Corresponds to L1 parameter 'K2' (see TS 38.214 [19], clause 6.1.2.1) configurable per PUSCH allocation. Only values {0..32} are applicable for PUSCH SCS of 120 kHz. When the field is absent for the first PUSCH if multiple PUSCH are configured per PDCCH, or when the field is absent and only one PUSCH is configured per PDCCH, the UE applies the value 1 when PUSCH SCS is 15/30 kHz; the value 2 when PUSCH SCS is 60 kHz, the value 3 when PUSCH SCS is 120 kHz, the value 11 when PUSCH SCS is 480 kHz, and the value 21 when PUSCH SCS is 960 kHz.

We provide the information to RAN2 to make necessary changes such that this parameter can be applied for Rel-18 for operation in FR1. Please note that the maximum range value is suggested to be reduced for FR1.

Note that the attached Excel Sheet is provided for convenience, as well.

Therefore, we propose the following:

Endorse the higher layer parameter in Table 1 (or equivalently in the attached Excel sheet) for enabling in FR1 scheduling multiple PUSCHs on non-consecutive slots by a single DCI format 0_1.

Table 1: Proposed RRC parameter for single DCI scheduling multiple PUSCHs on non-consecutive slots in FR1

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parent IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | Required for initial access or IDL/INACTIVE | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TEI18 | Multiple PUSCHs scheduling by single DCI for non-consecutive slots in FR1 | 38.214 |  | PUSCH-TimeDomainResourceAllocationList |  | extendedK2 | Up to RAN2 |  | The field description of | {0, .., 32} |  | Per UE |  | 38.331 | Multiple PUSCHs scheduled by a single DCI format 0_1 in Rel-17 is enabled on non-consecutive slots by extendedK2-r17. However, the applicability to non-consecutive slots is permitted in Rel-17 only in FR2-1/FR2-2. Based on the Rel-18 TEI agreement below , this feature is addiitonally enabled this for FR1 in Rel-18. It is up to RAN2 how to implement the agreement below. Please note that this row suggests a smaller range value, i.e. {0, ... . 32} for extendedK2 for FR1 as compared to Rel-17. Agreement Introduce UE feature(s) for multi-PUSCH scheduling with single DCI 0_1 for non-contiguous slots in FR1 for all defined SCSs - Note: there is no RAN1 impact |

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Endorse the higher layer parameter in Table 1 (or equivalently in the attached Excel sheet) for enabling in FR1 scheduling multiple PUSCHs on non-consecutive slots by a single DCI format 0_1.

# References

3GPP TS 38.306, “User Equipment (UE) radio access capabilities (Release 16),” v.16.12.0.

3GPP TS 38.306, “User Equipment (UE) radio access capabilities (Release 17),” v.17.4.0.

3GPP TS 38.331, “Radio Resource Control (RRC) protocol specification (Release 17),” v.17.4.0.