3GPP TSG RAN WG1 Meeting #113                    	                                             R1-2305187

Incheon, Korea, May 22nd – 26th, 2023

Agenda Item:                9.3.1

Source:                         MediaTek Inc.

Title: 	                           Discussion on evaluation of NR duplex evolution

Document for:              Discussion and Decision

# Introduction

A study item on evolution of NR duplex operation was approved in RAN#94e with the following objective [1]:

- Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In this contribution, we examine the Subband Full Duplexing (SBFD) evaluation results in comparison to the legacy Time Division Duplexing (TDD) and make some important observations from these evaluations.

# SBFD Evaluation Results

An analysis of the performance of SBFD compared to TDD is conducted based on Deployment Case 1 of SBFD. The FR1 UMa scenario is used for this comparison. Full buffer and FTP3 traffic are taken into consideration to make key observations.

## Full Buffer Traffic

### SLS validation with TDD and SBFD

Some subbands of TDD DL slots are allocated as UL resources to enable SBFD. This forms the SBFD format as ‘XXXXU’, where 20% of the RBs are allocated to UL, with the subband configuration < ND, NU, NG > = <104, 54, 5>. Thus, some DL resources are sacrificed to provide UL resources, leading to a theoretical calculation of resource gain for UL and resource loss for DL. This can be compared with the expected gain/loss of spectral efficiency of SBFD compared to TDD. This evaluation can be considered as a good validation method for the system level simulator (SLS).

From Table 1, the DL and UL resource gain of SBFD (XXXXU) compared to TDD (DDDSU) are examined in relation to their corresponding spectral efficiency gain. As expected, a clear correlation is seen between resource gain and spectral efficiency.

Table 1: Comparison of resource gain and spectral efficiency gain

Observation 1: The resource gain/loss in UL and DL, as well as the spectral efficiency gain/loss in UL and DL for SBFD over the legacy TDD, are closely aligned

### Impact of inter-UE CLI in Downlink

In the legacy TDD, UL and DL transmissions take place in distinct time slots, meaning there is no risk of interference between UL and DL signals from neighbouring UEs. However, with SBFD, UL and DL transmissions occur during the same time slot at the gNB, leading to Inter-UE CLI, as the UL transmission of one UE negatively affects the reception of DL signal at another nearby UE.

In traditional simulations, UEs are uniformly and randomly distributed, leading to a scenario where the distances between UEs are relatively large. As a result, the effect of Inter-UE CLI is not captured, as the separation distance is too big to account for it. This uniform random distribution also does not represent a real-world scenario, where users with multiple mobile devices typically group together in places like meeting rooms, restaurants, and public transport, with much smaller separation distances than those depicted in the simulation. Thereby, clustered UE dropping is agreed [2] to study the inter-UE CLI.

The UE clustering distribution for the baseline setup is M=20 and X=2, while the optional setup is M=10 and X=1. R is the cluster radius, which is 25m for the evaluation and the results for other values of R may be submitted. Dmacro-to-cluster is the minimum distance between a macro gNB and the cluster, which is 35m+ R, and Dinter-cluster is the minimum distance between two clusters, which is 2R m. UEs within the clusters are indoors, with a mobility of 3km/h, while UEs outside the clusters are outdoors in a car with a mobility of 30km/h. The UE height is 1.5m for all UEs.

In this section, the DL interferences such as co-channel interference and inter-UE cross link interference are analysed.

Figure 1: CCI over CLI ratio CDFs for UEs’ deployment with and without clustering

Figure 1 illustrates the ratio of CCI to Inter-UE CLI for clustered (with radius of 25m and 10m) and uniformly distributed deployments. The CDF plots demonstrate that the addition of the clusters leads to an increase in CLI over the CCI.

Observation 2: Inter-UE CLI has significant impact to the DL performance in clustered UE scenario

### Impact of intra/inter-gNB CLI and SI in UL

In the uplink interference analysis, co-channel interference (CCI) and total leakage powers with two capabilities (RCIC and RSIC) are compared. RCIC is the ratio of co-site interference cancellation, while RSIC is the ratio of self-interference cancellation. Furthermore, the frequency isolation in all cases is modelled by the adjacent channel interference model (ACIR) with an ACLR of 45dB and ACS of 43dB. The RSIC capability is assumed to be 102 dB, which is considered the best value when accounting for a 1dB receiver sensitivity loss.

Figure 2: Comparison of CCI and total leakage power in UL

From Figure 2, it is evident that the UL reception is significantly impacted by the RCIC capability. In this analysis, the RSIC capability is assumed as 102 dB which is considered to be the best value. It is derived by assuming 1dB receiver sensitivity loss.

Observation 3: Intra-site gNB CLI has significant impact to the UL performance

### Spectral Efficiency

The spectral efficiency of both DL and UL are compared in the following cases.

- Baseline case: without modelling of any leakage power (ideal SBFD)

- With modelling of only inter-site gNB CLI in UL and inter-UE CLI in DL

- With modelling of inter/intra-site gNB CLI and gNB SI in UL, and inter-UE CLI in DL

  - RSIC = 0dB and RSIC = 0dB

  - RSIC = 102dB and RSIC = 75dB

  - RSIC = 102dB and RSIC = 100dB

Table 2 shows the absolute values of spectral efficiency (SE) and Figure 3 shows the SE loss of SBFD due to the different cross-link interferences and gNB self-interference.

Table 2: Spectral Efficiency (absolute values)

|  | w/o CLI and SI (Baseline) | with inter-site gNB CLI in UL and inter-UE CLI in DL | with inter-site-CLI + co-site CLI + gNB-SI in UL and inter-UE CLI in DL |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | RSIC = 0 dB, RCIC = 0 dB | RSIC = 102 dB, RCIC = 75 dB | RSIC = 102 dB, RCIC = 100 dB |
| DL | 4.14 | 3.27 | 3.35 | 3.35 | 3.31 |
| UL | 2.70 | 2.26 | 1.17 | 1.63 | 2.17 |

Figure 3: Spectral Efficiency loss comparison for different cases

Figure 3 reveals that the DL SE is degraded by 20% due to the high inter-UE CLI, and UL SE is degraded by 16% due to the inter-site gNB CLI. Additionally, the UL spectral efficiency is heavily influenced by the RSIC and RCIC capabilities. Since DL reception at the UE is unaffected by these gNB capabilities, the DL SE remains unchanged for the other cases.

Observation 4: Intra-site isolation has a major influence on UL performance, and it is observed that 75 dB of isolation is insufficient, and 100 dB is preferred in addition to the frequency isolation

Observation 5: RSIC of 102 dB in addition to the frequency isolation is required to mitigate the gNB self-interference

## FTP3 Traffic

### UPT and Latency Performances of SBFD over TDD

The user perceived throughput (UPT) performances of the SBFD and the legacy TDD are analysed in all 3 RU conditions namely low (~10%), Medium (~27%) and High (~50%). This is done using FTP model 3 with 0.5Mbyte and 0.125Mbyte as DL and UL packet size. Here, the best gNB capabilities (RSIC=102dB, RCIC=100dB) are assumed. Table 3 shows the absolute values of UPT and Figure 4 shows the UPT gain of SBFD over TDD.

When comparing DL UPT and packet latency of SBFD and TDD, SBFD suffers due to the DL resource loss and inter-UE CLI. On the other hand, UL UPT and packet latency of SBFD has a better performance than TDD, due to the UL resource gain with the best RSIC and RCIC capabilities of gNB. To further improve the UPT performance, a good scope of studying the solutions for the inter-site gNB CLI management with minimal or no impact to the UE can be done.

Figure 4: UPT gain of SBFD over TDD

Table 3: UPT (absolute values)

Observation 6: DL UPT of SBFD suffers (~43% loss in mean UPT of medium RU) when compared to TDD due to DL resource loss and inter-UE CLI

Observation 7: UL UPT of SBFD is gained (~69% gain in mean UPT of medium RU) when compared to TDD due to UL resource gain and with the best RSIC and RCIC capabilities at gNB

Observation 8: Good scope of studying the solutions for the inter-site gNB CLI management with minimal or no impact to the UE.

Table 4 shows the absolute values of packet latency and Figure 5 shows the latency reduction of SBFD over TDD.

Figure 5: Latency reduction of SBFD over TDD

Table 4: Packet Latency (absolute values)

From Figure 5, when comparing DL latency of SBFD and TDD, SBFD is worse off due to the DL resource loss and inter-UE CLI. In contrast, UL latency of SBFD has an advantage compared to TDD, due to the UL resource gain with the best RSIC and RCIC capabilities of gNB.

Observation 9: DL latency of SBFD suffers (~124% loss in mean packet latency of medium RU) when compared to TDD due to DL resource loss and inter-UE CLI

Observation 10: UL latency of SBFD is gained (~43% loss in mean packet latency of medium RU) when compared to TDD due to UL resource gain and with the best RSIC and RCIC capabilities at gNB.

# CSI Reporting Evaluation Results

In RAN1#112, two CSI reporting options ware agreed to be studied for CSI reports associated with periodic/semi-persistent CSI-RS across SBFD and non-SBFD slots/symbols.

Agreement: For SBFD-aware UEs, study the following options for CSI report associated with periodic/semi-persistent CSI-RS, at least, across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD or all non-SBFD symbols): - Option 1: separate CSI reporting for SBFD symbols and non-SBFD symbols - Option 2: same CSI reporting for SBFD symbols and non-SBFD symbols

To evaluate the performance of separate CSI reporting versus same CSI reporting, an analysis is conducted based on Deployment Case 1 of SBFD, using the FR1 UMa scenario for comparison. Both full buffer and FTP3 traffic are considered in the analysis.

## Full Buffer Traffic

### SINR and BLER Analysis

The SBFD slot configuration ‘DXXXU’ is used for this evaluation. In this SBFD slot configuration, DL resources are distributed across two types of slots: DL-only slots and SBFD slots. Note that DL resources on SBFD slots/symbols are affected by both CCI and inter-UE CLI, whereas DL resources on non-SBFD slots/symbols are affected by only CCI. This motivates the study of slot-specific CSI reporting. The need for separate CSI reporting will be especially needed in clustered UE scenarios where CLI tends to dominate over CCI.

In Figure 6 (a), effective SINR is collected separately for SBFD and non-SBFD slots and compared. As expected, DL-SINR on SBFD slots is lower than that on non-SBFD slots. This underscores the importance of separate CSI reporting and link adaptation for each slot type.

- (b)

Figure 6: (a) Effective SINR distribution on DL-only slots and SBFD slots (b) DL-BLER performance in SBFD slots

In Figure 6 (b), BLER is compared across SBFD slots with same CSI reporting and separate CSI reporting. This simulation was conducted without outer-loop link adaptation to analyse the impact of separate CSI reporting on the BLER of fresh transmissions. As seen in the figure, separate CSI reporting outperforms same CSI reporting in terms of BLER performance.

Observation 11: The high level of interference on SBFD slots have a noticeable impact on both effective SINR and BLER performance, underscoring the need for separate CSI reporting for SBFD versus non-SBFD slots/symbols.

## FTP3 Traffic

This section compares the DL user-perceived throughput (UPT) performance for same CSI reporting and separate CSI reporting schemes. Figure 7 shows that for FTP3 traffic at high loads (around 70%) the separate CSI reporting scheme performs better in both 5% and 50% average-UPT scenarios. However, same CSI reporting outperforms separate CSI reporting in the 95% average-UPT scenario. This is because for the UEs achieving 95% average UPT, the SINR difference in DL-only slot and SBFD slot is small. As a result, the conservative MCS adopted in SBFD slot does not benefit BLER reduction, but instead lengthens packet latency and causes UPT loss. Furthermore, the interference power estimated on the DL resources of SBFD slot may not be so accurate because of the somewhat random nature of inter-UE CLI caused by the dynamic scheduling of UEs in the UL direction. This issue could be addressed if the scheduler had information on aggressor-victim UE pairs.

Figure 7: UPT and Latency

Observation 12: Using separate CSI reporting for SBFD and non-SBFD slots/symbols leads to a 5-10% improvement in performance compared to using the same CSI reporting.

# Conclusion

In this contribution, we discussed the SBFD evaluation results in comparison to the legacy TDD. We have the following observations:

- The resource gain/loss in UL and DL, as well as the spectral efficiency gain/loss in UL and DL for SBFD over the legacy TDD, are closely aligned

- Inter-UE CLI has significant impact to the DL performance in clustered UE scenario.

- Intra-site gNB CLI has significant impact to the UL performance.

- Intra-site isolation has a major influence on UL performance, and it is observed that 75 dB of isolation is insufficient, and 100 dB is preferred in addition to the frequency isolation.

- RSIC of 102 dB in addition to the frequency isolation is required to mitigate the gNB self-interference.

- DL UPT of SBFD suffers (~43% loss in mean UPT of medium RU) when compared to TDD due to DL resource loss and inter-UE CLI.

- UL UPT of SBFD is gained (~69% gain in mean UPT of medium RU) when compared to TDD due to UL resource gain and with the best RSIC and RCIC capabilities at gNB.

- Good scope of studying the solutions for the inter-site gNB CLI management with minimal or no impact to the UE.

- DL latency of SBFD suffers (~124% loss in mean packet latency of medium RU) when compared to TDD due to DL resource loss and inter-UE CLI.

- UL latency of SBFD is gained (~43% loss in mean packet latency of medium RU) when compared to TDD due to UL resource gain and with the best RSIC and RCIC capabilities at gNB.

- The high level of interference on SBFD slots have a noticeable impact on both effective SINR and BLER performance, underscoring the need for separate CSI reporting for SBFD versus non-SBFD slots/symbols.

- Using separate CSI reporting for SBFD and non-SBFD slots/symbols leads to a 5-10% improvement in performance compared to using the same CSI reporting.

# References

- RP-213591, “New SI: Study on evolution of NR duplex operation”, CMCC, RAN#94e, December 2021.

- Chairman notes, RAN1#110

# Appendix: Simulation assumptions in SLS evaluation

Table 5: Simulation assumptions in SLS for SBFD evaluation.

| Parameter set |  | Urban Macro (FR1) |
| --- | --- | --- |
| A. General | Carrier frequency | 4 GHz |
|  | System bandwidth | 100 MHz |
|  | Numerology | 14 OFDM symbol slot, SCS = 30kHz |
| B. Layout and UE distribution | BS Layout | Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around |
|  | Wrap-round | distance-based |
|  | Inter-BS (2D) distance | 500m |
|  | BS antenna height | 25m |
|  | UE distribution | UE clustering distribution - (Baseline) M=20, X=2 - R' = 25m - 8 UE per cluster per direction - Dmacro-to-cluster = 35m+R' - Dinter-cluster = 2R' m (Optional) Uniform UE distribution -10 users per macro TRP per direction |
|  | Minimum BS-UE (2D) distance | 35m |
|  | Minimum UE-UE (2D) distance | 1m |
|  | UE outdoor/indoor proportion | 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h |
|  | UE height (m) | 1.5m |
| C. Interference modelling | gNB self-interference - αSI | based on 1 dB UL desense |
|  | Co-site inter-sector co-channel inter-subband CLI | Spatial isolation: 75dB (RAN4 typical value) / 93dB (RAN4 best value) / 100dB Digital cancelation: companies to report, e.g., 10dB |
|  | BS ACLR | 45 dBc |
|  | BS ACS | 46 dBc |
|  | UE ACLR | 30 dBc |
|  | UE ACS | 33 dBc |
|  | UE IBE | Refer to Annex A.2.X in TR 38.858 |
|  | UE ICS | Not Modelled |
| D. SBFD subband and slot configurations | SBFD slot configuration | Legacy TDD: {DDDSU}, where S=[10D:2G:2U] SBFD: XXXXU and DXXXU |
|  | SBFD Subband configuration | {DUD}: < ND, NU, NG > = <104, 54, 5> |
|  | Guard symbols |  |
| E. Tranmit power | BS transmit power for legacy TDD | 53 dBm for 100MHz |
|  | BS transmit power for SBFD | Power boosting is not assumed for SBFD symbols compared to DL-only symbols (as in legacy systems) |
|  | UE Tx power | 23dBm |
| F. Antenna configurations | BS antenna configuration for legacy TDD | (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8) , (dH,dV) = (0.5, 0.8)λ, +45°/-45° polarization |
|  | BS antenna configuration for SBFD | Twice area & same TxRUs (higher priority): SBFD antenna configuration Option 2 |
|  | BS antenna radiation pattern | Table 9 in Report ITU-R M.2412 |
|  | UE antenna configuration | 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization |
| G. Traffic model | DL/UL traffic assignment for the same UE | Option 2: Each UE is assigned both UL traffic and DL traffic |
|  | DL/UL FTP packet size | 0.5Mbyte for DL and 0.125 Mbytes for UL |
|  | DL/UL traffic load for legacy TDD | Low load (Type-2 RU: < 10%) Medium load (Type-2 RU: 20%-40%) High load (Type-2 RU: >= 50%) |
| H. Channel model | gNB-UE | Refer to Annex A.3 in TR 38.858 |
|  | gNB-gNB | Option2: Large scale fading only |
|  | UE-UE | Refer to Annex A.3 in TR 38.858 Option2: Large scale fading only |
|  | UE-UE details | TR 38.901 |
| I. Others | BS receiver noise figure | 5dB |
|  | UE receiver noise figure | 9 dB |
|  | Open loop power control parameters | P0= -80 dBm, alpha = 0.8 |
|  | UE receiver | MMSE-IRC |
|  | Feedback assumption | Realistic |
|  | Channel estimation | Ideal |
|  | UE processing capability | UE processing capability 1 as baseline |
|  | Handover margin (dB) | 3 dB |
|  | UE attachment | Based on RSRP from port 0 |
|  | Transmission scheme | SU-MIMO, maximum layers for SU-MIMO = 2 |
|  | Polarized antenna model | Model-1 in clause 7.3.2 in TR 38.901 |
|  | DL/UL Modulation | Up to 256QAM |
|  | Scheduling | PF |
|  | Overhead | DL: 27.38% UL: 14.9% |