3GPP TSG RAN WG1 Meeting #113                    	                                             R1-2305193

Incheon, Korea, May 22nd – 26th, 2023

Agenda Item:           5

Source:                    MediaTek Inc.

Title: 	                     Discussion on RAN4 LS on Rel-18 Tx switching across 3/4 bands

Document for:         Discussion and Decision

# Introduction

In this contribution, we discuss the LS from RAN4 on Rel-18 Tx switching across 3/4 bands [1], and we provide our views on the response that RAN1 should provide to RAN4.

# Discussion

RAN4 sent an LS to RAN1 requesting clarifications on the following issue.

RAN4 would like to ask RAN1 one question: In sub-clause 6.1.6 of TS 38.214, it is specified that: If an uplink switching is triggered for an uplink transmission starting at T0, after T0-Toffset, the UE is not expected to cancel the uplink switching, or to trigger any other new uplink switching occurring before T0 for any other uplink transmission that is scheduled after T0-Toffset, where Toffset is the UE processing procedure time defined for the uplink transmission triggering the switch given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213]. On the definition and understanding of T0: - When gNB provides sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier, in RAN4 understanding, the T0 is the starting time for an uplink transmission on the switched-to carrier from network scheduling perspective and the starting time of actual uplink transmission from UE perspective. - When gNB does not provide sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier, and the switching period is located at the switch-to carrier according to the RRC signalling uplinkTxSwitchingPeriodLocation, UE may omit uplink transmission on certain symbol(s) on the switch-to carrier. In this case, is T0 the starting time of uplink transmission from network scheduling perspective, or the starting time of actual uplink transmission from UE perspective?

The part of RAN1 specs that is mentioned above defines a duration of time, before the UL transmission, where the UE doesn’t expect the gNB to send a scheduling instruction that cancels the (already triggered) UL Tx switching (illustrated in Figure 1). At T0-Toffset, the UE starts preparing the UL transmission, and any change in the scheduling instructions will require the UE to cancel the ongoing preparation of the corresponding UL transmission and trigger another process of preparation based on updated scheduling instructions, which causes pipelining issues at the UE.

Figure 1: Illustration of time duration for UL transmission preparation (without interruption to the UL transmission).

The reference point (T0-Toffset) where the UE start the perpetration for the UL transmission is not impacted by if the UL transmission is fully or partially transmitted (e.g., omitting the few symbols due to the switching gap). This case illustrated in Figure 2.

Figure 2: Illustration of time duration for UL transmission preparation (with interruption to the UL transmission).

Observation 1: The reference point (T0-Toffset) where the UE starts the perpetration for the UL transmission is not impacted by if the UL transmission fully or partially transmitted.

Proposal 1: RAN1 replay to RAN4 LS confirming the following:

“When gNB does not provide sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier, and the switching period is located at the switch-to carrier according to the RRC signalling uplinkTxSwitchingPeriodLocation, UE may omit uplink transmission on certain symbol(s) on the switch-to carrier. In this case, T0 is the starting time of uplink transmission from network scheduling perspective.

# Conclusion

In this contribution, we discussed the LS from RAN4 on Rel-18 Tx switching across 3/4 bands and we have the following observation and proposal:

Observation 2: The reference point (T0-Toffset) where the UE start the perpetration for the UL transmission is not impacted by of the UL transmission fully or partially transmitted.

Proposal 2: RAN1 replay to RAN4 LS confirming the following:

“When gNB does not provide sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier, and the switching period is located at the switch-to carrier according to the RRC signalling uplinkTxSwitchingPeriodLocation, UE may omit uplink transmission on certain symbol(s) on the switch-to carrier. In this case, T0 the starting time of uplink transmission from network scheduling perspective.

# References

- R4-2306623, “LS on Rel-18 Tx switching across 3/4 bands”, RAN4#106-e-bis, April 2023.