3GPP TSG- RAN WG1 Meeting #113                                   R1-2305237

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.2.3.2

Source:	Apple Inc.

Title:                     Discussion on other aspects of AI/ML based beam management enhancements

Document for:	Discussion/Decision

# Introduction

At RAN1 #109-e, the following agreements were reached:

Agreement

For AI/ML-based beam management, support BM-Case1 and BM-Case2 for characterization and baseline performance evaluations

- BM-Case1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams

- BM-Case2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams

- FFS: details of BM-Case1 and BM-Case2

- FFS: other sub use cases

Note: For BM-Case1 and BM-Case2, Beams in Set A and Set B can be in the same Frequency Range

Agreement

For the sub use case BM-Case1, consider both Alt.1 and Alt.2 for further study:

- Alt.1: AI/ML inference at NW side

- Alt.2: AI/ML inference at UE side

Agreement

For the sub use case BM-Case2, consider both Alt.1 and Alt.2 for further study:

- Alt.1: AI/ML inference at NW side

- Alt.2: AI/ML inference at UE side

Agreement

Regarding the sub use case BM-Case2, the measurement results of K (K>=1) latest measurement instances are used for AI/ML model input:

- The value of K is up to companies

Agreement

Regarding the sub use case BM-Case2, AI/ML model output should be F predictions for F future time instances, where each prediction is for each time instance.

- At least F = 1

- The other value(s) of F is up to companies

Also the conclusions were drawn at RAN1 #109-e for use case 1:

Conclusion

For the sub use case BM-Case1, consider the following alternatives for further study:

- Alt.1: Set B is a subset of Set A

  - FFS: the number of beams in Set A and B

  - FFS: how to determine Set B out of the beams in Set A (e.g., fixed pattern, random pattern, …)

- Alt.2: Set A and Set B are different (e.g. Set A consists of narrow beams and Set B consists of wide beams)

  - FFS: the number of beams in Set A and B

  - FFS: QCL relation between beams in Set A and beams in Set B

  - FFS: construction of Set B (e.g., regular pre-defined codebook, codebook other than regular pre-defined one)

- Note1: Set A is for DL beam prediction and Set B is for DL beam measurement.

- Note2: The narrow and wide beam terminology is for SI discussion only and have no specification impact

- Note3: The codebook constructions of Set A and Set B can be clarified by the companies.

Conclusion

Regarding the sub use case BM-Case1, further study the following alternatives for AI/ML input:

- Alt.1: Only L1-RSRP measurement based on Set B

- Alt.2: L1-RSRP measurement based on Set B and assistance information

  - FFS: Assistance information. The following were mentioned by companions in the discussion:  Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beamwidth, etc.), expected Tx and/or Rx beam for the prediction (e.g., expected Tx and/or Rx angle, Tx and/or Rx beam ID for the prediction), UE position information, UE direction information, Tx beam usage information, UE orientation information, etc.

    - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side.

- Alt.3: CIR based on Set B

- Alt.4: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID

- Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives

- Note2: All the inputs are “nominal” and only for discussion purpose.

And conclusions for use case 2 were drawn at RAN1 #109-e:

Conclusion

For the sub use case BM-Case2, further study the following alternatives with potential down-selection:

- Alt.1: Set A and Set B are different (e.g. Set A consists of narrow beams and Set B consists of wide beams)

  - FFS: QCL relation between beams in Set A and beams in Set B

- Alt.2: Set B is a subset of Set A (Set A and Set B are not the same)

  - FFS: how to determine Set B out of the beams in Set A (e.g., fixed pattern, random pattern, …)

- Alt.3: Set A and Set B are the same

- Note1: Predicted beam(s) are selected from Set A and measured beams used as input are selected from Set B.

- Note2: It is up to companies to provide other alternative(s)

- Note3: The narrow and wide beam terminology is for SI discussion only and have no specification impact

Conclusion

Regarding the sub use case BM-Case2, further study the following alternatives of measurement results for AI/ML input (for each past measurement instance):

- Alt.1: Only L1-RSRP measurement based on Set B

- Alt 2: L1-RSRP measurement based on Set B and assistance information

  - FFS: Assistance information. The following were mentioned by companies in the discussion:, Tx and/or Rx beam angle, position information, UE direction information, positioning-related measurement (such as Multi-RTT), expected Tx and/or Rx beam/occasion for the prediction (e.g., expected Tx and/or Rx beam angle for the prediction, expected occasions of the prediction), Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight directions (azimuth and elevation), 3dB beamwidth, etc.) , increase ratio of L1-RSRP for best N beams, UE orientation information

    - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side.

- Alt.3: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID

- Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives

- Note2: All the inputs are “nominal” and only for discussion purpose.

At RAN1 #110, the following were reached:

Agreement

At least for the sub use case BM-Case1 and BM-Case2, support both Alt.1 and Alt.2 for the study of AI/ML model training:

- Alt.1: AI/ML model training at NW side;

- Alt.2: AI/ML model training at UE side.

Note: Whether it is online or offline training is a separate discussion.

Agreement

For the sub use case BM-Case1 and BM-Case2, further study the following alternatives for the predicted beams:

- Alt.1: DL Tx beam prediction

- Alt.2: DL Rx beam prediction

- Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam)

- Note1: DL Rx beam prediction may or may not have spec impact

Agreement

- Study the following options on the selection of Set B of beams (pairs)

  - Option 1: Set B is fixed across training and inference

    - FFS on the beams of Set B

  - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each report/measurement during training and/or inference)

    - FFS on fixed or variable number of beams (pairs)

    - FFS on the details

  - Other options are not precluded.

  - FFS on the number of beams (pairs) in Set B

  - Note: This does not preclude the alternative that Set B is different from Set A.

Agreement

- To evaluate the performance of AI/ML in beam management at least for NW side beam prediction, UCI report overhead can be further studied as one of KPI options.

  - FFS: number of UCI reports and UCI payload size

Agreement

For the sub use case BM-Case1, support the following alternatives for further study:

- Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A)

- Alt.2: Set B is a subset of Set A

- Note1: Set A is for DL beam prediction and Set B is for DL beam measurement.

- Note2: The beam patterns of Set A and Set B can be clarified by the companies.

Agreement

For the sub use case BM-Case2, further study the following alternatives:

- Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A)

- Alt.2: Set B is a subset of Set A (Set A and Set B are not the same)

- Alt.3: Set A and Set B are the same

- Note1: The beam pattern of Set A and Set B can be clarified by the companies.

Agreement

Regarding the sub use case BM-Case1 and BM-Case2, study the following alternatives for AI/ML output:

- Alt.1: Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams

  - E.g., N predicted beams can be the top-N predicted beams

- Alt.2: Tx and/or Rx Beam ID(s) of the N predicted DL Tx and/or Rx beams and  other information

  - FFS: other information (e.g., probability for the beam to be the best beam, the associated confidence, beam application time/dwelling time, Predicted Beam failure)

  - E.g., N predicted beams can be the top-N predicted beams

- Alt.3: Tx and/or Rx Beam angle(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams

  - E.g., N predicted beams can be the top-N predicted beams

  - FFS: details of Beam angle(s)

At RAN1 #111, the following were agreed:

Agreement

For the sub use case BM-Case1 and BM-Case2, at least support Alt.1 and Alt.2 for AI/ML model training and inference for further study:

- Alt.1. AI/ML model training and inference at NW side

- Alt.2. AI/ML model training and inference at UE side

- The discussion on Alt.3 for BM-Case1 and BM-Case2 is dependent on the conclusion/agreement of Agenda item 9.2.1 of RAN1 and/or RAN2 on whether to support model transfer for UE-side AI/ML model or not

Alt.3. AI/ML model training at NW side, AI/ML model inference at UE side

In this contribution, we provide our views on enhancement use cases and potential spec impact for AI based beam management.

# Discussion

## Discussion on model generalization

The AI model’s generalization performance plays a central role from dataset collection, inference, and performance monitoring. Broadly there can be two fundamental approaches to construct AI/ML models for beam management:

- 1) With at least beam measurement at UE as input, the entity performing AI/ML inference (UE or network) estimates key parameters in the wireless channel and identifies the best Tx beam. With that, there is not much difference in principle between the estimator implicitly or explicitly embedded in the AI/ML model and an estimator developed by a systems engineer. Just by the universal approximation theorem, the AI/ML model performing the estimation does not need to exhibit how it is actually done, and perhaps even the estimation problem does not need explicitly formulation. With that, anything particular to a specific cell or terrain type may be secondary inputs or does not constitute as inputs to the AI/ML model at all.

- 2) Looking from another angle, if the radio condition between network and a UE does not change or changes slowly, the best Tx beam at a location for one UE remains the best Tx beam at the same location for another UE (we ignore UE orientation here to simplify discussion).  In this case, there is no need to build a universal beam estimator or channel parameter estimator, rather a neural network with good memory about a particular geographical area serves the beam management purpose well. Beam measurement is used as a key to make an inquiry to the Tx beam database embodied by the neural network. With that, ideally for each cell a neural network model is customized. As the AI model embodies the terrain information implicitly and each cell can be different from terrain perspective, intuitively suitable models for cells will be different. If model generalizes at all, that may just be a coincidence.

We have

Observation 1: AI/ML models can be crafted as a universal channel parameter estimator with good generalization or as a beam management database and associated query mechanism customized for a specific cell which is not expected to generalize well.

Four generalization cases (Generalization Case 1/2/2A/3) were agreed at RAN1 #111:

Agreement

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

      - Note: Companies to report the ratio for dataset mixing

      - Note: number of the multiple scenarios/configurations can be larger than two

- FFS the detailed set of scenarios/configurations

- The following case for generalization verification, can be optionally considered by companies:

  - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B.

    - Company to report the fine-tuning dataset setting (e.g., size of dataset) and the improvement of performance

    - FFS: Investigate of the feasibility the fine-tuning on the UE/Network side

As for details of scenarios and configurations in the generalization study, details are also agreed at RAN1 #111:

Agreement

- For generalization performance verification, consider the following

  - Scenarios

    - Various deployment scenarios,

      - e.g., UMa, UMi and others,

      - e.g., 200m ISD or 500m ISD and others

      - e.g., same deployment, different cells with different configuration/assumption

      - e.g., gNB height and UE height

      - FFS: e.g., Carrier frequencies

    - Various outdoor/indoor UE distributions, e.g., 100%/0%, 20%/80%, and others

    - Various UE mobility,

      - e.g., 3km/h, 30km/h, 60km/h and others

  - Configurations (parameters and settings)

    - Various UE parameters, e.g., number of UE Rx beams (including number of panels and UE antenna array dimensions)

    - Various gNB settings, e.g., DL Tx beam codebook (including various Set A of beam(pairs) and gNB antenna array dimensions)

    - Various Set B of beam (pairs)

    - T1 for measurement /T2 for prediction for BM-Case2

  - Other scenarios/configurations (parameters and settings) are not precluded and can be reported by companies.

Regarding the evaluation of Set B in generalization study, also at RAN1 #111, we have

Agreement

For BM Case-1 and BM Case 2, to verify the generalization performance of an AI/ML model over various scenarios/configurations, additionally considering

- Various Set B of beam(pairs)

Based on evaluation provided in our companion paper,  it noted for Case 1 (matched training/test datasets), the AI based method’s performance tends to be much better than the conventional method’s given the same number of beams for measurement, which can motivates further work on AI enabled beam management at least for Case 1. For Case 2 (mismatched training/test datasets), the AI based method’s performance tends to be much worse than that for Case 1 matching test dataset or matching the training dataset. Case 3 provides a remedy wherein mixture of datasets is used to train an AI model.

## Discussion on assistance information

From the conclusions reached at RAN1 #112:

Conclusion

Regarding the explicit assistance information from UE to network for NW-side AI/ML model, RAN1 has no consensus to support the following information

- UE location

- UE moving direction

- UE Rx beam shape/direction

Conclusion

Regarding the explicit assistance information from network to UE for UE-side AI/ML model, RAN1 has no consensus to support the following information

- NW-side beam shape information

  - E.g., 3dB beamwidth, beam boresight directions, beam shape, Tx beam angle, etc.

- Note: Other information (e.g., relative information) of Tx beam(s) preserving sensitive proprietary information is a separate discussion

  - e.g., some information following the same principle of Rel-17 positioning agreement

Then for RAN1 discussion, it should be assumed that assistance information for the NW-side AI/ML model such as UE location, UE moving direction or UE Rx beam shape/direction is not available, and for UE-side AI/ML model, NW-side beam shaping information is not available.

If NW-side beam information is available at the UE-side, it may be possible for the UE-side model trainer to build a generic Tx beam predictor by taking the beam angle information for set A beams and set B beams implicitly or explicitly in the AI/ML model. From the RAN1 #112 conclusion, such a possibility is excluded.

Then related to Alt. 2 (UE side training/UE side inference), to come up with an UE side AI/ML model with good generalization performance to unknown scenarios is difficult. Deploying multiple AI/ML models on the UE side and requesting the network side to validate the model may be difficult. Consequently the feasibility of Alt. 2 should be carefully examined.

For the NW-side model, since assistance information concerning UE Rx beam shape/direction is not available, it is not feasible for the network side to perform Rx beam prediction for the UE. As for the UE-side model, We note conventionally the Rx beam operation is purely up to UE implementation. In the P3 operation, with repetition on for CSI-RS, UE can test different RX beams but there is no CSI reporting on RSRP out of that operation. In our view, departing from the conventional practice for RX beam management is not desirable or warranted. Thus we don’t support the study of beam pair prediction in Rel-18 AI/ML. Also in the current NR design, P2 and P3 procedures are clearly defined, performing beam pair prediction rather than separate prediction for Tx beam and Rx beam complicate the design without benefit.

We have

Proposal 1:   Deprioritize beam pair prediction in the study.

## Discussion on model training and model inference

From the agreements reached at RAN1 #109-e and RAN1 #110:

Agreement

For the sub use case BM-Case1, consider both Alt.1 and Alt.2 for further study:

- Alt.1: AI/ML inference at NW side

- Alt.2: AI/ML inference at UE side

Agreement

For the sub use case BM-Case2, consider both Alt.1 and Alt.2 for further study:

- Alt.1: AI/ML inference at NW side

- Alt.2: AI/ML inference at UE side

Agreement

At least for the sub use case BM-Case1 and BM-Case2, support both Alt.1 and Alt.2 for the study of AI/ML model training:

- Alt.1: AI/ML model training at NW side;

- Alt.2: AI/ML model training at UE side.

Note: Whether it is online or offline training is a separate discussion.

At RAN1 #111, the following was agreed:

Agreement

For the sub use case BM-Case1 and BM-Case2, at least support Alt.1 and Alt.2 for AI/ML model training and inference for further study:

- Alt.1. AI/ML model training and inference at NW side

- Alt.2. AI/ML model training and inference at UE side

- The discussion on Alt.3 for BM-Case1 and BM-Case2 is dependent on the conclusion/agreement of Agenda item 9.2.1 of RAN1 and/or RAN2 on whether to support model transfer for UE-side AI/ML model or not

Alt.3. AI/ML model training at NW side, AI/ML model inference at UE side

Regarding these alternatives, we observe

- Alt. 1. Model training at the NW side & inference at the NW side

  - Data collection:

    - As network is aware of the RRC configuration of CSI-RS and CSI/BM reporting configuration, even though CSI-RS and CSI/BM reporting are configured on a UE-specific way, that does not prevent network from aggregating data from CSI/BM reporting and utilize them for model training.

  - Model generalization problem can be circumvented as whether a single AI model or multiple AI models are needed for inference is transparent to the UE.

  - Specifical impact for this combination can include enhanced beam reporting, e.g., more efficient beam reporting. It can be expected the feedback overhead to support this combination will be more than those for other combinations, and it is likely it will be more than that for the conventional beam management related feedback overhead. As shown in Figure 1, the current NR beam reporting targets a small number of beams, inheriting the same design for beam reporting for AI/ML may not be resource efficient. There are at least 3 alternatives for set B design:

- Set-B-alt-1: Set B is fixed in both number of beams and the beam constellations (analog beam design)

- Set-B-alt-2: The number of beams in Set B is fixed, but the beam constellations can be changed with time.

- Set-B-alt-3: Both the number of beams and beam constellations in Set B can change.

For both alternatives Set-B-alt-2 and Set-B-alt-3, whether set B indication or beam indication should be used is not so clear yet. In general beam indication provides a finer granularity and full flexibility, yet that also comes with the accompanied feedback overhead. When RSRPs and/or CIRs (Channel Impulse Responses) are fed back to network, a careful re-examination on the Rel-15 design is needed.

For Alt. 1, if the number of measurements is large, e.g., 16 set B beams, using the same CRI/SSBRI indication design in the legacy design is inefficient and un-necessary. Since the maximum number of beams reported by the UE is limited by the set B size, and no selection among set B beam is needed. However, some set B beams can be rather weak, it does not bring benefit to report them back to the network. In that case, efficient signalling scheme can be considered to reduce feedback overhead.

- Alt. 3. Model training at the NW side & inference at the UE side

  - Specific impact for this combination can include model transfer or model delivery if superior beam management performance with cell-specific AI models is the target as discussed in the generalization section.

  - Even with non-cell specific AI models, whether a single AI model or multiple AI models are needed should be discussed. In our companion paper[3], we have investigated model mismatch with training data set and test data set as follows:

- Set A design (Column first vs Row first)

- Set B design (Column shift = 0 and 1 for Set A at 16 beams, and Column shift = 0, 1, 2 and 3 for Set A at 4/8 beams)

- Antenna element spacing ((dV=0.5,dH=0.5) vs (dV=0.8,dH=0.4))

And AI model performance degrades compared with the case without model mismatch.

  - Model generalization performance is key for this combination: if model generalizes well across different scenarios/configurations, it can be expected model delivery takes place infrequently. On the other hand, if model generalization performance for beam management is not so good, then frequent model delivery may be needed, e.g., model 1 is used by network in Cell 1 while model 2 is used by network in Cell 2. Further depending on the amount of memory and the number of neural network engines available at the UE which moves from one cell to another cell, the time when a new AI/ML model is activated may vary.

- Alt. 2 Model training at the UE side, and inference at the UE side

  - For this combination, inference and performance monitoring are left to UE implementation. However, data collection still needs discussion, e.g., whether cell-specific signals are supported so the data for model training can be readily collected; otherwise each UE has a siloed view on the Tx beams from the network as CSI-RS is currently configured in a UE-specific way, data collection then will take a prolonged time. in an idealized setup, UE or UE side server has all the freedom to optimize its model(s), it offers opportunities for UE performance differentiation. However, if network does not provide any help to the UE, realistically it is not clear how many AI models the UE needs to store, and by what criteria model switching/selection can be done by the UE itself, especially for cell-specific or site specific models. In the “performance monitoring” section, we explore whether it is possible to use test data to facilitate selection/switching of an AI/ML model.

In summary, we have

Proposal 2:

- For Model training at the NW side & inference at the NW side (Alt. 1), study efficient signalling of set B selection or beam selection and RSRP representation.

- For Model training at the NW side & inference at the UE side (Alt. 3), study model generalization performance, study model transfer/model delivery for cell-specific AI models and non cell-specific AI models.

Observation 2:

- Alt. 1 (NW side training/NW side inference) does not require disclosure of network implementation information.

- Alt. 3 (NW side training/UE side inference) can ensure AI/ML performance when assistance information from network is not available at the UE/UE server.

## Discussion on quantization error and measurement error

Figure 1 Feedback overhead for beam reporting in NR design

First the discussion for RSRP quantization should be only for the NW side model. In legacy NR design, the quantization step for a reference beam is 1 dB, and the quantization step for the differential RSRP is 2 dB, and the lowest representable differential RSRP is -30 dB. The feedback overhead to support the NW side model is

{overhead for the reference beam} + {overhead for the differential RSRPs}.

From the discussion on Set C and Set B at RAN1 #112, it may happen not all the measured beams need to be reported, e.g., a weak beam’s RSRP does not need reporting. The study on quantization error can establish the tradeoff between performance and the feedback overhead for AI enabled beam management, hence it is properly in the domain of RAN1. On the other hand, as the RSRP measurement accuracy requirements are specified by RAN4, RAN4 understands the error mechanism and the context of those requirements, it is more suitable for RAN4 to perform study if it is indeed necessary. We have

Proposal 3: RAN1 should prioritize the study on quantization error’s impact to AI/ML model inference performance over measurement error.

## Discussion on data collection, user privacy and user consent

The term “data collection” has been used for measurements by UE which are reported for model training, inference, and performance monitoring. It is clear that for inference, UE’s reporting on its measurements is beneficial to both end user’s experience and network’s operation. For performance monitoring, under certain conditions, e.g., the performance monitoring leads to actions on the network side (if NW performance monitoring is used) to ensure UE radio performance, it can be also beneficial to the end user’s experience.

However, for data collection for model training, as offline training is agreed as the mode where an AI/ML model is trained, there is no direct benefit to the UE’s radio performance and consequently no immediate benefit to end user experience. It may also happen a UE which provides data collection intended by network for model training may not benefit from the effort at all: e.g., the UE has UE-side inference model while the data collection is for model training for NW-side inference model. Hence in this case, data collection by UE as instructed by the network does not bring any tangible benefits to the UE’s radio operation, it may not be a fair use of the UE, e.g., draining battery with no apparent reason to the UE.

Also another point deserves consideration is training data is key to the success of AI/ML.  As such, training data is valuable, and which should be properly accounted for. In a hypothetical system design, if an instruction sent by network for UE reporting, which can be masqueraded as for inference, and is actually intended for model training, that can be considered a misuse of UE reporting’s functionality. Such a misuse should be avoided; hence the purpose of data collection should be clearly stated in network operation. lumping UE reporting for all purposes under “data collection” may blur such important differences among them. We have

Proposal 4: for discussion on AI/ML BM, the purpose for “data collection” should always be clear, e.g., data collection for training data, UE reporting for inference, UE reporting for performance monitoring.

Also, for data collection for training data, there is no urgency in processing the measurements and also providing too much data to network raises user privacy concern. In contrast to data collection for inference, data collection for training data may contain much more information, e.g., many RSRPs at high resolution, which may facilitate tracking UE’s location by its correlation to RF signal measurements. From both radio technology development and user privacy consideration, there should always be a clear indication from the network, whether the data collection is for training data, or for inference, etc.

We have

Proposal 5: the difference between data collection for various purposes is reflected in the air interface design if AI/ML BM is eventually specified.

Since data collection for training data does not bring direct benefit to a UE’s radio performance and may raise user privacy concern, a consent from the UE/end user should be obtained in data collection signaling flow. Only the UE/end user agrees the training data collection request, the UE will honor the request. It can be appreciated if network sends frequent requests for training data collection, and the UE’s response is almost instantaneous, the tracking of UE’s location can be real-time. Since the training data collection is not for online training anyway, a mechanism needs to be introduced so the risk of using training data collection for real-time tracking is mitigated.

We have

Proposal 6: UE consent is required for training data collection and mechanism needs to be introduced to mitigate the risk of using training data collection for real-time tracking.

## Discussion on QCL assumption

For discussions from companies and preliminary conclusions drawn from the evaluation agenda item, it can be seen the valuation proposal for AI-BM is on the reduction on measurement overhead while achieving the same or better performance as the conventional approach; or achieving better performance at the same measurement overhead. In the evaluation assumptions, 32 Tx beams or 64 Tx beams are assumed, and 256 Tx beams are also mentioned as an option.

We note conventionally  QCL chains and TCI state(s) are used to propagate the QCL assumptions from a source (e.g., SSB) down to a reference signal/channel. Enumeration of beams through TCI state index, CSI-RS resource index or SSB index is possible. With finer Tx beam reporting, as by beam angles, one possible consequence is that enumeration of beams through TCI state index, CSI-RS resource index or SSB index may not be feasible or desirable anymore.  Then beam indication for the case with beam angle reporting needs to be clarified. We have

Observation 3: corresponding beam indication’s enhancement needs to be clarified when beam angle reporting is proposed.

## Discussion on beam reporting

Agreement

For the sub use case BM-Case2, further study the following alternatives:

·       Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A)

·       Alt.2: Set B is a subset of Set A (Set A and Set B are not the same)

·       Alt.3: Set A and Set B are the same

·       Note1: The beam pattern of Set A and Set B can be clarified by the companies.

With the above agreement, for BM Case-2, when Set B equals Set A,  by using a simple AI/ML model (e.g., a LSTM model), the time domain prediction at each Tx beam over time can be conducted. In this case, the correlation among different Tx beams is not considered critical information for AI/ML inference, and inference at the UE side is feasible.

When Set B does not equal Set A, network’s analog beam information is needed for the UE-side inference to make that deduction from observed beams in Set B to beams in Set A. Inference at UE side may encounter more challenges.  One straightforward solution is to support inference at NW side instead. However, the feedback overhead will be substantial.

## For both BM case-1 and BM case-2 (time domain beam prediction), when NW-side inference is used, the UE feedback is key to the AI/ML inference performance on network side. It is intuitive that when RSRP measurements are reported with high resolution and feedback overhead is not a limiting factor, the AI/ML model on the NW-side will not see much performance degradation due to quantization error. And that intuitive understanding is supported by the conclusion drawn at RAN1 #112bis-e:

Observation (RAN1 #112bis-e):

- At least for BM-Case1 for inference of DL Tx beam with L1-RSRPs of all beams in Set B, existing quantization granularity of L1-RSRP (i.e., 1dB for the best beam, 2dB for the difference to the best beam) causes [a minor loss x%~y%, if applicable] in beam prediction accuracy compared to unquantized L1-RSRPs of beams in Set B.

However, feedback overhead is always a key design consideration. The discussion on beam reporting for NW-side inference is intimately linked to quantization scheme design for RSRPs. From Rel-15 NR, quantization of the RSRP for the reference beam (the strongest beam) is with 1 dB from -144 dBm to -44 dBm, and the weaker beams are quantized with 2 dB with 30 dB range with respect to the reference beam’s RSRP.  By the using of CRIs, up to 4 beams are selected among the configured/measured beams.

It can be seen that for beams weaker than the reference beam by more than 30 dB, purely considering the quantization error aspect, a RSRP report on those beams is un-necessary, as setting its RSRP value to -30 dB with respect to the reference beam does not degrade AI/ML performance much. For BM Case-2, assuming there are M RSRP reports for N occasions, as there can be correlation among different occasions, e.g., a weak beam in Occasion 1 tends to remain weak in Occasion 2, exploiting the correlation can be reduced the number of reported RSRPs from M×N to a smaller number.

We have

Proposal 7: study the correlation among RSRPs for the same Tx beams at Occasions for BM Case-2, and achieve reduced feedback overhead by exploiting the temporal correlation in their RSRPs.

## Discussion on performance monitoring

Agreement (RAN1 #111):

For the sub use case BM-Case1 and BM-Case2, at least support Alt.1 and Alt.2 for AI/ML model training and inference for further study:

- Alt.1. AI/ML model training and inference at NW side

- Alt.2. AI/ML model training and inference at UE side

- The discussion on Alt.3 for BM-Case1 and BM-Case2 is dependent on the conclusion/agreement of Agenda item 9.2.1 of RAN1 and/or RAN2 on whether to support model transfer for UE-side AI/ML model or not

Alt.3. AI/ML model training at NW side, AI/ML model inference at UE side

Assume a number of AI/ML models are trained and stored at a UE, it can be envisioned in its meta data, some information regarding NW beam information may be stored, if the UE is allowed to make inquiry to the network to establish the validity of the AI/ML model’s  use in the current cell, any response from network can be used to deduce the network’s beam shape information.

For UE-side training/UE-side inference, concepts such as dataset ID/zone ID were proposed to allow a UE to determine whether the deployed model is a suitable choice for the serving cell or not. We note however since those IDs may be global, RAN proprietary information may be revealed by a systematic check on those global IDs. However, such risks may not invalidate the feasibility study on Alt. 2 UE-side training/UE-side inference. By the universal approximation theorem, a deep network can approximate a quite arbitrary function, which means there can be abrupt changes in output in a small vicinity of an input.  With that, it will be difficult to deduce the AI/ML model’s output if the input is perturbated in a small way. In practice, perhaps the issue can be somehow mitigated: firstly as observed by multiple companies, AI/ML inference models for BM can be supported by multiple fully connected layers, which is much simpler than the transformer models often used for AI/ML CSI feedback, thus less prone to such abrupt changes; secondly, inevitably there can be imperfections in the input such as quantization error and an AI/ML inference model for BM has to be robust against such small perturbation, then a smoothness assumption on the AI/ML model may be valid. With such considerations, it may be possible to check whether an AI/ML model is suitable for inference at a serving by checking its response to test data. The test data can consist of a set of inputs, and a corresponding set of outputs, if the AI/ML model can perform inference with the test data with adequate accuracy, then the AI/ML model may be suitable for use at a given serving cell. To check the AI/ML inference model thoroughly, the size of test data cannot be too small, whether that is logistically feasible should be studied. Of course, this is also tied to efficient representation of test data.

Proposal 8: Study using test data to verify whether an AI/ML model is suitable for inference at a given serving cell, including the required test data size and efficient representation of test data.

# Conclusion

In this contribution, we provided some discussion on enhancement for AI based beam management. Based on the discussion, we have:

Observation 1: AI/ML models can be crafted as a universal channel parameter estimator with good generalization or as a beam management database and associated query mechanism customized for a specific cell which is not expected to generalize well.

Observation 2:

- Alt. 1 (NW side training/NW side inference) does not require disclosure of network implementation information.

- Alt. 3 (NW side training/UE side inference) can ensure AI/ML performance when assistance information from network is not available at the UE/UE server.

Observation 3: corresponding beam indication’s enhancement needs to be clarified when beam angle reporting is proposed.

Proposal 1:   Deprioritize beam pair prediction in the study.

Proposal 2:

- For Model training at the NW side & inference at the NW side (Alt. 1), study efficient signalling of set B selection or beam selection and RSRP representation.

- For Model training at the NW side & inference at the UE side (Alt. 3), study model generalization performance, study model transfer/model delivery for cell-specific AI models and non cell-specific AI models.

Proposal 3: RAN1 should prioritize the study on quantization error’s impact to AI/ML model inference performance over measurement error.

Proposal 4: for discussion on AI/ML BM, the purpose for “data collection” should always be clear, e.g., data collection for training data, UE reporting for inference, UE reporting for performance monitoring.

Proposal 5: the difference between data collection for various purposes is reflected in the air interface design if AI/ML BM is eventually specified.

Proposal 6: UE consent is required for training data collection and mechanism needs to be introduced to mitigate the risk of using training data collection for real-time tracking.

Proposal 7: study the correlation among RSRPs for the same Tx beams at Occasions for BM Case-2, and achieve reduced feedback overhead by exploiting the temporal correlation in their RSRPs.

Proposal 8: Study using test data to verify whether an AI/ML model is suitable for inference at a given serving cell, including the required test data size and efficient representation of test data.

# References

- R1-2206032, “Evaluation on AI/ML for CSI feedback enhancement”, vivo, RAN1 #110, August 2022.

- R1-2211807, “Evaluation on AI/ML for beam management”, Apple, RAN1 #111, November 2022

- R1-2301339, “Evaluation for AI/ML based beam management enhancements”, Apple, RAN1 #112, February  2023.