3GPP TSG RAN WG1 #113			      R1-2305255

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.7.1

Source:	Apple

Title:	Discussion on spatial and power domain enhancements to support network energy saving

Document for:	Discussion/Decision

Introduction

In RAN1 #112b-e meeting, several agreements on spatial and power domain enhancements to support network energy saving was made, including resource configurations, report configurations, and a CSI reporting framework including both single-CSI and multi-CSI report, etc. In this contribution, we provide our considerations correspondingly.

Resource configuration enhancement

In RAN1 #112b-e meeting [1], the agreements regarding resource configuration enhancements are listed:

Agreement Support configurability of NZP CSI-RS resource(s) for channel measurement within one resource setting corresponding to more than one spatial adaptation patterns with at least one of the following - A1-1-revised: a resource set with multiple resources is configured within a resource setting, where each resource is associated with only one spatial adaptation pattern - A1-2-revised: For a resource configured in a resource set within a resource setting, the resource can be associated with more than one spatial adaptation patterns - One or more resources can be configured in the resource set for channel measurement. Agreement For power domain adaptation, support the following configuration(s) for CSI-RS resource configuration, - A1-2-power: one or more resources can be configured in a resource set within a resource setting and each resource can be associated with one or more power offset values - FFS: A1-1-power: a resource set with multiple resources is configured within a resource setting, where resources can have different power offset values - FFS: Details of how the different power offset values(s) are configured/indicated. Working Assumption Al-1-revised and A1-2-revised are supported - FFS: Which Type of SD adaptation A1-1-revised and A1-2-revised are applicable for Agreement For R18 NES, only legacy port configuration values (N1, N2) or (Ng, N1, N2) are supported. - FFS: Whether/what restriction for A1-1-revised and A-1-2-revised w.r.t number of ports

There were discussions in last meeting that whether A1-1-revised and A1-2-revised is applied for both Types of SD adaptation or not. It is noted that to support Type 2 SD adaptation can be configured using A1-1-revised only, and it was agreed as working assumption that both A1-1-revised and A1-2-revised will both be supported. If A1-1-revised is also used for Type 2 SD adaptation, it would be a redundant spec work to support the same function using two different configurations. Therefore, it is preferred to support the two resource configuration separately for the to avoid redundant work.

If A1-1-revised is used for Type 2 SD adaptation only, then the multiple CSI-RS resources within the resource set are associated with different TCI-State and/or powerControlOffset and/or powerControlOffsetSS values. This can be actually achieved by current spec. From resource configuration perspective, no spec change is needed to support Type 2 SD adaptation, therefore, it is proposed to maintain the restriction that the multiple CSI-RS resources within a resource set is configured with the same nrofPorts value.

Proposal 1: A1-1-revised is used for Type 2 SD adaptation only, where the multiple CSI-RS resources within the resource set are associated with different TCI-State and/or powerControlOffset and/or powerControlOffsetSS values. The restriction that the multiple CSI-RS resources within a resource set is configured with the same nrofPorts is maintained. No spec change is needed for resource configuration for Type 2 SD adaptation.

A1-2-revised can only be used for Type 1 SD adaptation and power domain(PD) adaptation. In this case, each CSI-RS resource will be associated with one or more hypothetical nrofPorts values or one or more hypothetical powerControlOffset values for CSI computation. It was agreed that one or multiple resources can be configured within one resource set. However, whether there are more restrictions on the related parameters are still under discussion.

According to our understanding, the motivation for configuring multiple resources is for UE to measure and report a good CRI. It then makes sense to compare multiple resources under the same assumptions. Thus, it is resonable to configure a same set of hypothetical parameters, otherwise, it will be too complicated for both NW and UE to measure and report.

If for the multiple resources in the set, there are no further restrictions, e.g. a resource set can be configured with two resources as following:

{Resource #0, with nrofPorts 64 and 32; Resource #1, with nrofPorts 32 and 16},

there will be three codebookConfigs needed for this resource set, for codebook size = 64 and 16, UE will measure on only one CSI-RS resource, and for codebook size = 32, UE will measure on both of the CSI-RS resources, which will complicate UE measurement, CPU occupation calculation and active CSI-RS resources calculation. Therefore, it is proposed that the multiple resources are configured with a same set of nrofPorts values.

Proposal 2: A1-2-revised is used for Type 1 SD adaptation and PD adaptation only, where one CSI-RS resource can be associated with one or more hypothetical nrofPorts values or one or more hypothetical powerControlOffset values for CSI computation. If multiple CSI-RS resources are configured within one resource set, the same set of hypothetical parameters is configured.

Report configuration enhancements

In RAN1 #112b-e meeting[1], the agreements regarding report configuration enhancements are listed:

Agreement At least support A2-2, i.e. one CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern. - FFS: impact on CSI processing requirement Agreement For a CSI report config with L sub-configuration(s), support a framework that enables a UE to report N CSI(s) in one reporting instance where the N CSI(s) are associated with N sub-configuration(s) from L (where ) and each CSI corresponds to one sub-configuration. - For discussion purpose, N=1 refers to single-CSI while N>1 refers to multi-CSI. - For Semi-persistent/Aperiodic CSI reporting, support gNB trigger/indicate/activate report of N≤L CSIs where N>=1 - The maximum value of N and L are subject to UE capability - Further study how to address/minimize additional UE complexity The following bullet was not agreed due to objection from Apple and vivo - For Periodic CSI reporting, at least the case of N=L is supported where N>=1 Agreement For power domain adaptation, for CSI(s) reporting, support configuration of more than one power offset values for PDSCH relative to CSI-RS - FFS: impact on CSI processing requirement - FFS: details on configuration/indication of the power offset values - FFS: whether/how to additionally consider the case where CSI-RS power is changed Agreement For Semi-persistent/Aperiodic CSI reporting with , study what enhancements to the current DCI and MAC-CE mechanisms are needed for gNB triggering/indication/activation of the N CSI(s) in a reporting instance, where the N CSI(s) are associated with N sub-configuration(s) from L in a report config.

Periodic CSI report

In last RAN1 meeting, the CSI reporting framework was agreed to support reporting N CSI(s) in one reporting instance from the L reporting sub-configurations. However, there were not consensus on whether to support N<=L for periodic CSI report.

For periodic CSI report, since it is transmitted on PUCCH, the payload is a major concern for reporting all the L CSIs if N=L is supported. It is then proposed by some companies to consider the CSI overhead reduction schemes to support N=L for P CSI. However, according to our understanding and logic, it is more natural to at least support the scheme with N<L where N is small enough so that no advanced CSI overhead reduction schemes are need.

Observation 1: Supporting N<L where N is small enough, requires no advanced CSI overhead reduction schemes and reduces UE CSI computation complexity

This can be achieved by the following scheme:

For a CSI report Config with L>1 sub-configurations, to report a total number of L CSIs with N CSI(s) in each reporting instance, UE can report in an total number of L/N reporting instances where in each instance, the N CSI(s) corresponds to N different sub-configuration from the L sub-configuration.

Proposal 3: Support for N<L for periodic CSI reporting, where for a CSI report Config with L>1 sub-configurations,  a total number of L CSIs can be reported to the NW in a total number of L/N reporting instances with N CSI(s) in each instance, where the N CSI(s) corresponds to N different sub-configuration from the L sub-configuration.

There were discussions in last meeting that how to enable or indicate different N CSI(s) in the periodic CSI report. It is our understanding that there could be multiple ways for either with or without additional signaling.

- Opt 1: Pre-determine in spec a cycling pattern of N sub-configuration(s).

In this option, for example, it can be pre-determined that UE will report the first N sub-configurations in the first instance, the second N sub-configurations in the second instance, and so on. Or the N sub-configurations corresponding to the N largest values of nrofPorts/codebook sizes in the first instance, the N sub-configurations corresponding to the second N largest values of nrofPorts/codebook sizes in the second instance, and so on. These rules can be pre-determined in spec, so that no additional signaling is needed.

- Opt 2: Use RRC to configure a cycling pattern of N sub-configuration(s).

In this option, it can be configured in the RRC together with the L sub-configurations a cycling pattern for UE to report. For example, RRC configures a sequence of the L=4 sub-configurations, as {2,3,1,4}, with N=2, UE will report the 2nd an 3rd sub-configurations in the first reporting instance and the 1st and 4th sub-configurations in the second reporting instance. This option can provide more flexibility than opt 1.

- Opt 3: Use MAC CE to indicate a cycling pattern or to activate a different N sub-configuration(s)

In this option, considering that this is periodic report, UE starts CSI reporting after the RRC configuration. A default N sub-configuration(s) can be determined in the RRC configuration, where when there is no MAC CE signaling, UE will report the N CSI(s) corresponding to the N default sub-configuration(s). When receiving the MAC CE, UE will start to report based on either the indicated cycling pattern in the MAC CE or report based on the one-time activation of a different set of N sub-configuration(s).

- Opt 4: Use DCI to indicate a different set of N sub-configuration(s) for UE

In this option, similar to the MAC CE case, DCI can be used to indicate a different set of N sub-configuration(s) for UE to report in each reporting instance.

Proposal 4: The following four options can be considered to configure UE which N CSI(s) to be reported in each of the L/N reporting instances:

- Opt 1: Pre-determine in spec a cycling pattern of N sub-configuration(s).

- Opt 2: Use RRC to configure a cycling pattern of N sub-configuration(s).

- Opt 3: Use MAC CE to indicate a cycling pattern or to activate a different N sub-configuration(s).

- Opt 4: Use DCI to indicate a different set of N sub-configuration(s).

Semi-Persitent/Aperiodic CSI report

For Semi-persistent/Aperiodic CSI report, it has already been agreed that N<=L is supported. It will be furthere studied what enhancements to the current DCI and MAC-CE mechanisms are needed for gNB triggering/indication/activation of the N CSI(s) in a reporting instance, where the N CSI(s) are associated with N sub-configuration(s) from L in a report config.

For N=L case, there activation/triggering mechanisms remain the same as current spec, where a MAC CE/DCI activates/triggers a CSI-ReportConfig, means the all the L sub-configurations are activated/triggered together.

For N<L case, the activation/triggering signaling should be able to indicate which N of the L sub-configurations are activated/triggered.

Proposal 5: For N<L case, the activation/triggering signaling should be enhanced to indicate which N of the L sub-configurations are activated/triggered.

For semi-persistent CSI report with N<L, similar to the periodic CSI report, we consider it necessary to support UE reporting a total of L CSIs for network in multiple reporting instances. There are two schemes to support this functionality:

- Opt 1: The activation MAC CE indicates a cycling pattern

In this option, MAC CE configures  a sequence of the L sub-configurationsa patters, e.g {2,3,1,4} when L=4, UE will start reporting upon activating sub-config #2 and #3 in the first reporting instance and #1 and #4 in the second instance and so on.

- Opt 2: Each activation MAC CE indicates a set of N sub-configurations

In this optios, the MAC CE activates only N sub-configuration, if no more MAC CE is received, UE will continue with the current N sub-configurations until it receives a new MAC CE to activate another set of N sub-configurations.

Proposal 6: The following enhancements can be considered for activating which N CSI(s) will be reported for semi-persistent CSI report:

- Opt 1: The activation MAC CE indicates a cycling pattern

- Opt 2: Each activation MAC CE indicates a set of N sub-configurations, new MAC CE activates a new set of N sub-configurations and deactivates the previous one

CPU occupation

For the CPU occupation, there were discussions in last meeting that how it should be scaled with N. It has been acknowledged by most companies that the scaling depends on N, however, the exact cases may be more complicated. According to the current CSI processing criteria described in TS38.214, the counting for a CPU occupation is based on per CSI-ReportConfig and mainly depending on how many CSI-RS resources a UE needs measure and calculates corresponding CSI (OCPU=KsOCPU=KS, where Ks Ks is the number of CSI-RS resources in the CSI-RS resource set for channel measurement.). Under the NES CSI reporting framework,  It is more reasonable to count CPU occupations per effective sub-configuration to achieve a fair requirement for UE.

To be more specific, the spec will expressed in the following way for the CPU occupation counting:

-	for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', or 'cri-RI-LI-PMI-CQI', for each sub-configuration indicated for reporting

-	if max{ µPDCCH, µCSI-RS, µUL} ≤ 3, and if a CSI report is aperiodically triggered without transmitting a PUSCH with either transport block or HARQ-ACK or both when L = 0 CPUs are occupied, where the CSI corresponds to a single CSI with wideband frequency-granularity and to at most 4 CSI-RS ports in a single resource without CRI report and where codebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', OCPU=NCPUOCPU=NCPU,

-	If a sub-config in the CSI-ReportConfig is configured with codebookType set to 'typeI-SinglePanel' and the corresponding CSI-RS Resource Set for channel measurement is configured with two Resource Groups and N Resource Pairs, OCPU=X⋅N+M, where X is the number of CPUs occupied by a pair of CMRs subject to [UE capability] and M is defined in clause 5.2.1.4.2.

-	otherwise, OCPU=KsOCPU=KS, where Ks Ks is the number of CSI-RS resources in the CSI-RS resource set for channel measurement.

Proposal 7: The CPU counting, is based on per effective sub-configuration (indicated by RRC/MAC CE/DCI to be reported on a certain reporting instance), and the total number of supported CPUs for CSI measurement remains the same as current UE capability.

Single CSI report

While we propose to support N<L, for N>1 case, there is still more spec enhancement or clarification needed on what is the maximum number of N or how to redefine the CPU occupation(will be discussed in the following), etc. It is noted that N=1 is a special case of N<L.

In the evaluation result in the TR [2], for spatial adaptation, the evaluation results are mainly categorized into the following three cases and the corresponding gains are summarized:

Case 1: Semi-static adaptation without multi-CSI:  4.8%~48.2% NES gain with UPT loss of 0.02%~87.08% (9 sources)

Case 2: Dynamic adaption without multi-CSI: 6.7%~26.5% NES gain with UPT loss of 0.3%~18.5% (2 sources)

Case 3: Dynamic adaption with multi-CSI: 6.8%~31.3% NES gain with UPT loss of 0.02%~7% (3 sources)

For power adaptation, the evaluation results are mainly categorized into the following three cases and the corresponding gains are summarized:

Case 1: Semi-static PDSCH power reduction of 3~18dB: 2.3%~51.5% NES gain with UPT loss of 2.03%~19.49% (8 sources)

Case 2: Dynamic PDSCH power reduction with multi-CSI: 12.1%~23.8% NES gain over no adaptation with UPT loss of less than 1.17% (1 source) and 5.3%~11.5%, compared to dynamic power reduction without multi-CSI report (1 source)

It can be observed that Semi-static and dynamic adaptation without multi-CSI can already provide remarkable NES gain. Therefore, we believe it is s reasonable to start with supporting a basic NES working mode, i.e. single CSI feedback is supported for Periodic CSI and Semi-persistent

Observation 2: Single CSI as a special case of N<L, can provide L CSIs for gNB without increasing UE complexity and achieve remarkable NES gain.

Proposal 8:  Support single CSI (N=1) with L sub-configurations for periodic and Semi-persistent CSI report for NES. A total number of L CSIs can be reported to the NW in a total number of L reporting instances.

Content in the report sub-configuration

Agreement For CSI report configuration, if L>1 in a CSI report configuration, at least the following can be included for each sub-configuration for Type 1 SD adaptation - N1, N2 for single-panel and N1, N2, Ng for multi-panel - FFS: details on explicit indication or implicit derivation - Port subset indication when A1-2 is used (if A1-2 is supported) - FFS: details on explicit indication or implicit derivation - FFS: rank restriction - FFS: codebook subset restriction - FFS: supported codebook types for PMI, e.g., Type-I or Type-II - FFS: report quantity - FFS: reportFreqConfiguration - FFS: Group identity of NZP CSI-RS resource(s) in a resource set for channel measurement when A1-1 is used For CSI report configuration for type 2 SD adaptation, further study under which cases sub-configurations may or may not be needed including sub-configuration content

It has been agreed that N1,N2,Ng and port subset indication (if A1-2 is supported) will be included in each sub-configuration for Type 1 SD adaptation.

For the port subset indication, both explicit indication based on bitmap and implicit derivation can be considered to leverage from configuration flexibility and configuration overhead.

For N1,N2,Ng and codebook subset restriction, they could be explicit indicated using current codebookConfig.

For rank restriction, codebook type, report quantity, and reportFreqConfiguration, we do not see much need to configure them differently, thus they can be excluded in the sub-configurations.

For the Group identity for NZP CSI-RS resource(s), our understanding of the motivation is to link the sub-configuration to a CSI-RS resource of the mulitple resources corresponding to different SD patterns in the resource set. We think an associated CSI-RS resource ID would be enough for this case.

For other contents not configured in the sub-configuration, use the value configured in the CSI report config.

Proposal 9: The following are to be included  for each sub-configuration for Type 1 SD adaptation,

- CodebookConfig

- Port subset indication (if A1-2 is supported), both explicit indication based on bitmap and implicit derivation can be considered

For Type 2 SD adaptation, an associated CSI-RS resource ID is to be included in each sub-configurations.

Proposal 10: For other contents not configured in the sub-configuration, use the value configured in the CSI report config.

CSI overhead/report payload reduction

In RAN1 #112b-e meeting[2], the agreement regarding CSI overhead/report payload reduction study points is as follows:

Agreement - For CSI feedback with CSI overhead/report payload reduction, further study whether/how to report a common value and/or a differential and/or joint coded value across same CSI quantity of different sub-configurations/adaptation patterns, at least for the following - CRI - RI - PMI - CQI - FFS: L1-RSRP - Other (new) report quantity, if any - Further study whether/how it is feasible/possible for the UE to skip the evaluations of some sub-configurations/adaptation patterns to reduce the burden at the UE

For this study point, we think the motivation of the proposal needs to be clarified that whether/how this intends to impact UE complexity and CPU occupation. According to the current proposals, we thinks the existing solutions have different intentions. For the differential or joint coded values across same CSI quantity of different sub-configurations/adaptation patterns, the intention is to purely reduce CSI overhead, where UE needs to calculated N CSI(s) independently, where the CPU occupation scaling does not change. For the common value reporting, the intention seems to change UE CSI measurement behavior and will have further impact on CPU occupation rule, which will be also dependent on which of the parameters could be common and needs further clarification.

Observation 3: Differential and joint coded value reporting across same CSI quantity of different sub-configurations/adaptation patterns does not save UE CSI measurement complexity to report N CSI(s). For the common value reporting, it will have further impact on CPU occupation rule, which will be dependent on which of the parameters could be common and needs further clarification.

Proposal 11: If differential and/or joint coded value across same CSI quantity of different sub-configurations/adaptation patterns is supported, the CPU occupation counting is still based on per sub-configuration.

Conclusion

- In this contribution, we discussed our considerations on spatial and power domain enhancement to support network energy saving, the following proposals are made:

Observation 1: Supporting N<L where N is small enough, requires no advanced CSI overhead reduction schemes and reduces UE CSI computation complexity

Observation 2: Single CSI as a special case of N<L, can provide L CSIs for gNB without increasing UE complexity and achieve remarkable NES gain.

Observation 3: Differential and joint coded value reporting across same CSI quantity of different sub-configurations/adaptation patterns does not save UE CSI measurement complexity to report N CSI(s). For the common value reporting, it will have further impact on CPU occupation rule, which will be dependent on which of the parameters could be common and needs further clarification.

- Resource Configuration:

Proposal 1: A1-1-revised is used for Type 2 SD adaptation only, where the multiple CSI-RS resources within the resource set are associated with different TCI-State and/or powerControlOffset and/or powerControlOffsetSS values. The restriction that the multiple CSI-RS resources within a resource set is configured with the same nrofPorts is maintained. No spec change is needed for resource configuration for Type 2 SD adaptation.

Proposal 2: A1-2-revised is used for Type 1 SD adaptation and PD adaptation only, where one CSI-RS resource can be associated with one or more hypothetical nrofPorts values or one or more hypothetical powerControlOffset values for CSI computation. If multiple CSI-RS resources are configured within one resource set, the same set of hypothetical parameters is configured.

Periodic CSI report:

Proposal 3: Support for N<L for periodic CSI reporting, where for a CSI report Config with L>1 sub-configurations,  a total number of L CSIs can be reported to the NW in a total number of L/N reporting instances with N CSI(s) in each instance, where the N CSI(s) corresponds to N different sub-configuration from the L sub-configuration.

Proposal 4: The following four options can be considered to configure UE which N CSI(s) to be reported in each of the L/N reporting instances:

- Opt 1: Pre-determine in spec a cycling pattern of N sub-configuration(s).

- Opt 2: Use RRC to configure a cycling pattern of N sub-configuration(s).

- Opt 3: Use MAC CE to indicate a cycling pattern or to activate a different N sub-configuration(s).

- Opt 4: Use DCI to indicate a different set of N sub-configuration(s).

Semi-Persistent/Aperiodic CSI report

Proposal 5: For N<L case, the activation/triggering signaling should be enhanced to indicate which N of the L sub-configurations are activated/triggered.

Proposal 6: The following enhancements can be considered for activating which N CSI(s) will be reported for semi-persistent CSI report:

- Opt 1: The activation MAC CE indicates a cycling pattern

- Opt 2: Each activation MAC CE indicates a set of N sub-configurations, new MAC CE activates a new set of N sub-configurations and deactivates the previous one

CPU occupation

Proposal 7: The CPU counting, is based on per effective sub-configuration (indicated by RRC/MAC CE/DCI to be reported on a certain reporting instance), and the total number of supported CPUs for CSI measurement remains the same as current UE capability.

Single CSI report

Proposal 8:  Support single CSI (N=1) with L sub-configurations for periodic and Semi-persistent CSI report for NES. A total number of L CSIs can be reported to the NW in a total number of L reporting instances.

Content in the report sub-configuration

Proposal 9: The following are to be included  for each sub-configuration for Type 1 SD adaptation,

- CodebookConfig

- Port subset indication (if A1-2 is supported), both explicit indication based on bitmap and implicit derivation can be considered

For Type 2 SD adaptation, an associated CSI-RS resource ID is to be included in each sub-configurations.

Proposal 10: For other contents not configured in the sub-configuration, use the value configured in the CSI report config.

Proposal 11: If differential and/or joint coded value across same CSI quantity of different sub-configurations/adaptation patterns is supported, the CPU occupation counting is still based on per sub-configuration.

Reference

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #112bis-e, e-Meeting, April 17th – April 26th, 2023

- TR38.864, Study on network energy savings for NR, Dec.2022.