3GPP TSG RAN WG1 #113	                                         	           R1-2305270

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.11.3

Source:	Apple Inc.

Title:	On the L1 signal design and procedures for low power wake-up signal

Document for:	Discussion/Decision

# Introduction

In Rel-18, a study item was approved for low-power wake-up signal and receiver for NR (WID in RP-222644 [1]), and it includes the following objectives.

- Identify evaluation methodology (including the use cases) & KPIs [RAN1] - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables - Other use cases are not precluded - Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4] - Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4] - Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1] - Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1] - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

In this contribution, we focus on wake-up signal (WUS) design and the related procedures to support WUS.

# L1 signal design

First of all, we would like to discuss some high-level functions that are desirable to be supported by LP WUS/WUR.

## DRX operation

The LP WUR can either continuously monitor for WUS, or periodically monitor with a DRX cycle. The following was agreed in RAN1#112:

Agreement

Study further pros and cons of the following monitoring behaviors of LP-WUR

- Option1: Duty cycle, corresponds to LP-WUR switches between ON/OFF states

- Option2: Continuous monitoring, corresponds to LP-WUR is ON all the time

We think DRX operation or duty cycle operation should be prioritized in the study for the following reasons:

- For the purpose of power saving, continuous monitoring could only make sense if the power consumption of WUR is so low that continuous monitoring does not result in too much additional power consumption. If the power consumption of WUR is ~1 mW (or higher), the power consumption of continuous monitoring is on similar order as the deep sleep state of MR, which may not provide attractive power saving. Even if much lower power consumption for WUR is possible to provide significant power saving, it comes with cost on other aspects such as sensitivity and overhead.

- Continuous monitoring creates challenge for false alarm. As the UE constantly detects for WUS (or preamble), the probability for false alarm within a fixed duration can be much higher. Alternatively, if the overall probability for false alarm is reduced to a satisfactory level, it leads to performance degradation in missed detection rate.

- DRX operation allows UEs to be multiplexed in time for WUS monitoring. If WUS cannot carry a unique UE ID (48 bits as in paging record), DRX operation provides an additional dimension in time domain to multiplex UEs, while continuous monitoring would require the UE ID or group ID to be carried as part of the WUS. This provides flexibility for network configuration. It could also allow few UEs to be allocated to the same group (if UE-specific WUS is not used), which reduces the probability of MR being waken up due to other UEs.

- Latency is considered as one of the potentially advantages for continuous monitoring. However, the wake-up time needed for MR is already 400 ms, and additional enhancements would not bring too much gain. If latency is a concern, a smaller duty cycle can be configured, instead of continuous monitoring. Note that the MR takes a significant amount of time to ramp up (400 ms assumed in the model), this ramp-up time can be the dominating factor in latency, and there is not much difference in latency between a small duty cycle and continuous monitoring.

- One potential disadvantage of DRX operation is that the network may need to transmit additional signals (e.g. periodic sync signal) to assist the UE in time/frequency synchronization. The corresponding overhead needs to be considered. The overhead depends on how frequent such signals need to be transmitted. If we take time offset as an example, assuming a very conservative 200ppm max frequency error, it translates into a max time drift of 0.256ms after 1.28 second. Assuming the UE could handle 0.256 ms uncertainty by doing a search within this time window, it means the network can transmit sync signals every 1.28 seconds. The overall overhead can be very low (e.g. <0.1%). Of course this aspect still needs further study.

Proposal 1: Duty cycle operation of WUR should be prioritized in the LP WUS design.

## Synchronization

For synchronization, the following was agreed in RAN1#112:

Agreement

Study synchronisation signal used by LP-WUR, if needed, based on

- Option 1: aperiodic signal transmitted as part of LP-WUS

  - FFS: Whether the signal can additionally be transmitted separately from LP-WUS

- Option 2: periodic signal transmitted separately from LP-WUS

- Option 3: Option1 + Option2

If an OFDM-based receiver is considered, we can potentially reuse the existing PSS/SSS for synchronization. Otherwise, for any WUS design, synchronization signal would be needed to keep the time offset and frequency offset at the WUR within a reasonable range.

The issue with Option 1 is that a UE may not receive WUS for a very long time, especially for some IoT applications with very low-frequency traffic. If no other signal is provided, the UE clock may drift away too far, which would require the UE to monitor a very large time window for WUS. This can become continuous monitoring once the clock drifts aways for sufficiently long.

For Option 2, the question is the accuracy required by the LP WUR, which determines how frequent the synchronization signal is transmitted. If there is no preamble transmitted with WUS, the accuracy requirement would be relatively high.

Option 3, on the other hand, can use the periodic signals providing the UE a reference for coarse synchronization, while the preamble allows finer synchronization to facilitate WUS detection. This can reduce the frequency of periodic signals. However, whether Option 3 is better than Option 2 requires some quantitative analysis based on the synchronization requirements.

Observation 1: If synchronization signal is needed, Option 1 (aperiodic signal transmitted as part of LP-WUS) is not sufficient as a standalone solution, while the comparison between Option 2 (periodic signal) and Option 3 (Option 1 + Option 2) requires quantitative analysis based on the synchronization requirements.

## Information to be carried for WUR

For the content of LP-WUS, the following was agreed:

Agreement

- For IDLE/INACTIVE mode study at least following candidates for content of LP-WUS

  - information on which user(s) is/are targeted by the LP-WUS

    - e.g. UE-group, -subgroup or -ID

  - FFS: cell information

  - FFS: SI change and ETWS/CMAS information, tracking area information, and RAN area information

- For CONNECTED mode, study at least following candidates for content of LP-WUS

  - information on which user(s) is/are targeted by the LP-WUS

    - e.g UE-group, -subgroup or -ID

  - indication to wake-up to PDCCH monitoring.

- Other information candidates are not precluded

- Study pros and cons of including above information to LP-WUS.

- Note: the information may be explicitly or implicitly indicated.

The bare minimum information that needs to be received by the LP WUR is the wake-up indication that tells a UE whether the main radio should wake up or not. Depending on the design and configuration, WUS can carry a single-bit information (0 vs 1), or the full UE ID, or a group ID. UEs or UE groups may be additionally separated by using different time/frequency resources when DRX is enabled.

UE-specific WUS would consume more resources and generate more overhead from system perspective, but it allows maximum UE power saving as it avoids the unnecessary wake-up due to other UEs in the same group. From design point of view, it is good to allow some flexibility for gNB configuration, so that the desired tradeoff between overhead and UE power saving can be achieved. In this sense, it is desirable to support both UE-specific and group-common WUS, which can be supported using the same framework and achieved by proper configuration.

Proposal 2: For idle/inactive UEs, a group ID may be carried/embedded in the LP-WUS. A special case is that the group ID is a UE ID, which becomes UE-specific indication.

It is useful to carry cell information in LP-WUS (e.g. cell ID) via e.g. payload or scrambling. This allows the UE to differentiate the signals transmitted from different cells.

Proposal 3: For idle/inactive UEs, a cell ID may be carried/embedded in the LP-WUS.

Other than the wake-up indication itself, we think the information carried for WUR should be minimized. Due to the low power consumption requirement, it is generally very inefficient to carry information here compared to MR reception. On the other hand, the overhead to wake up main radio is large, while performing some additional function using MR once it wakes up typically does not lead to much additional power consumption. Unless the benefit can be clearly justified, we should avoid adding information here.

Proposal 4: The additional information carried for WUR should be avoided or minimized.

## WUS Design

In the previous meetings, we agreed to study WUS candidates including MC-OOK, MC-FSK and OFDM-based signals/channels. For OFDM-based signals/channels, the proponents are generally considering sequence-based design, such as SSS-like sequence or ZC sequence. It is generally understood that there is different tradeoff for different kind of WUS design, in terms of power consumption, performance and overhead etc. There are also a diverse range of use cases and devices that have different requirements in terms of power consumption, latency, and mobility, etc. Another consideration is the difference between RRC idle/inactive mode and RRC connected mode, where the main radio may go into ultra-deep sleep state in idle/inactive mode, but may have to stay in deep/light/micro sleep state in RRC connected mode.

Due to these different considerations, it can be useful to consider a signal design that can accommodate different types of UE receivers. For the WUS candidates under study, the MC-OOK and sequence-based design can be easily harmonized to support either OOK-based receiver or a sequence-based receiver. Consider Figure 1 which shows a diagram that is mostly common for the basic MC-OOK design (OOK-1) and sequence-based design. For OOK-1, there is no specific requirements on what symbols are carried on the M SCs. As long as some signal is transmitted, the UE with OOK-based receiver can detect the signal. On the other hand, for sequence-based design, a specific sequence needs to be transmitted for the UE to detect. The harmonized WUS design would be that a specific sequence is carried on the M SCs, so that both the UE with OOK-based receiver and the UE with sequence-based receiver can detect the WUS. This can leave flexibility to UE implementation in terms of what type of receiver to implement, considering the use cases and requirements. A UE with OOK-based receiver can achiever lower power consumption with certain sacrifice in performance, while a UE with sequence-based receiver can potentially achieve better performance but with higher power consumption.

Figure 1 Harmonized WUS design for OOK-based receiver and sequence-based receiver

Proposal 5: Further consider a harmonized WUS design that can be detected by both OOK-based receiver and sequence-based receiver, by defining the sequence used to generate the WUS signal.

## WUS link level performance

In this section, we provide some initial link level simulation results for OOK-3.

- Option OOK-3: Multi-tone single-bit OOK

  - N SCs of LP-WUS is separated into L segments (L=2 on Figure) without guard-bands in-between segment, but possibly around

  - OOK=1 means 1 sub-carrier (known by UE) of each segment is modulated, rest of SC is zero power (from base-band point of view)

  - OOK=0 means all SCs in all segments are zero power (from base-band point of view)

  - FFS architecture

In the simulations, we assume a total LP-WUS BW of 5MHz. To keep the signaling as simple as possible and to maximize the tone separation for improved resilience to channel and receiver impairments, we choose the LP-WUS configuration as shown in Table 1, which is further illustrated in Figure 2.

Table 1 Parameters for OOK-3 (MT-OOK) example

| MT-WUS Parameter | Value |
| --- | --- |
| LP-WUS symbol length | 66.67us |
| M, number of tone groups | 2 |
| N, number of tones per tone group | 3 |
| Df, tone spacing | 720 kHz |
| Total used bandwidth by WUS | 4.32 MHz |
| Total allocated bandwith for WUS | 5 MHz |

Figure 2 MT-WUS example in 5MHz bandwidth

The simulations are performed based on the following assumptions using a non-coherent detector using Goertzel processing (refer to [2] for details).

- Channel models: AWGN, TDL-A 30ns 11Hz and TDL-C 300ns 11Hz.

- The probability of missed detection and false alarm is set as: PMD = 1% , PFA = 0.1%. A sampling rate of Fs=10MHz is assumed, which helps reducing complexity and power consumption at ADC block.

- Frequency offset: 4kHz

- Integrated phase noise: -3 dBc

  - The phase noise is modeled using the same profile proposed in [4] and the integrated phase noise value is further increased by 3dB.

In Table 2, the required SNR to achieve the required MDR and FAR is provided. The SNR value is given in relation to the allocated WU-BW, in the example 5MHz, and in the processed band, M times the Geortzel filter band (M*30kHz=60kHz in our example). The SNR conversion formula is as follows:

Required SNR in allocated BW [dB] = Required SNR in processed BW + 10*log10(60kHz/5MHz)

Table 2 Required SNR for the example MT-WUS

| Num of Served UEs | Channel Model | Required SNR in processed BW [dB] | Required SNR in allocated BW[dB] |
| --- | --- | --- | --- |
| 1 | AWGN | 9.02 | -10.19 |
|  | TDL-A 30ns 11Hz | 20.02 | 0.81 |
|  | TDL-C 300ns 11Hz | 17.42 | -1.79 |
| 2 | AWGN | 9.02 | -10.19 |
|  | TDL-A 30ns 11Hz | 20.22 | 1.01 |
|  | TDL-C 300ns 11Hz | 18.02 | -1.19 |

## Coverage enhancement

It has been agreed to study techniques/mechanisms to enhance the coverage LP-WUS, with the potential target to match the regular NR coverage.

Agreement

- Study techniques/mechanisms to enhance coverage performance of LP-WUS

The typical time domain enhancements to improve the coverage includes e.g. lower data rate/coding/longer time duration/repetition in time.

In addition to time domain enhancements, coverage can also be enhanced by exploiting frequency diversity, which can be achieved by either distributed transmission in frequency or frequency hopping, as illustrated in Fig. 3. This is applicable to any of the candidate waveforms. The WUS signal is transmitted in different frequency segments within and/or across multiple symbols in time domain. Distributed transmission in frequency requires multiple parallel receivers for the multiple frequency segments, which lead to higher complexity/cost at the UE. Frequency hopping, on the other hand, would require longer transmission duration for the same data rate. Both can be considered for LP-WUS design. Frequency diversity can be especially beneficial for LP-WUS because LP-WUS is not expected to occupy large bandwidth, which can be vulnerable to deep fade within the bandwidth.

- Distributed transmission in frequency              (b) frequency hopping

Figure 3 Illustration of methods to achieve frequency diversity

Another potential technique for coverage enhancement is to deploy SFN (single-frequency network) for LP-WUS, to have the same LP-WUS signal transmitted from multiple cells simultaneously. LP WUS can be particularly suitable for SFN operation. The purpose of LP WUS is to wake up the UE. Such a message, similar to the Page message for a UE, typically needs to be transmitted in multiple cells simultaneously, as the network may not know exactly where the UE is. With some coordination, these multiple cells can transmit the same signal in the same time/frequency resource. In this case, the UE would see the combined signal from these multiple cells and greatly reduced interference.

However, using SFN transmission would lead to larger delay spread, which can impact the signal detection performance. Typical way to mitigate such impact is to have longer CP or larger guard time between WUS symbols to reduce the inter-symbol interference.

Proposal 6: For coverage enhancements, at least the following can be considered:

- Distributed transmission in frequency or frequency hopping to achieve frequency diversity

- SFN transmission for LP-WUS

# WUS related procedures

## Enabling and disabling of LP-WUS

The first level of enabling and disabling of LP WUS is on the cell level, where the gNB may notify the UEs whether LP WUS is enabled in the cell via broadcast signaling such as SIB.

If LP-WUS coverage is insufficient, mechanisms are necessary so that WUS can be disabled when the UE moves out of the WUS coverage and then enabled again when the UE moves back into the WUS coverage. This is captured in the agreement below.

Agreement

- Study techniques/mechanisms to enhance coverage performance of LP-WUS

- Study potential gains available as well as drawback(s) of the technique(s)/mechanisms(s), e.g. system overhead, increased complexity network energy consumption etc…

- Study potential issues and corresponding solutions for the case when LP-WUS coverage is insufficient

  - At least study fallback mechanisms where the Main Radio switches to legacy operation in case the channel condition of LP-WUS is not sufficient, e.g. below threshold.

Such enabling and disabling would be done in a UE-specific manner. The following two options can be studied:

- Option 1: the UE determines whether to enable or disable WUS

  - E.g. the UE may make decision based on the mobility, serving cell signal strength/quality, the traffic pattern, and/or some other information available at the UE.

  - The network may provide some configuration parameters for the UE to make the decision, e.g. the threshold for RSRP and/or speed.

  - The UE may or may not notify the gNB about the decision change.

    - If the UE notifies the gNB, the gNB has the up-to-date information on the need to transmit WUS for the UE, and may stop transmitting WUS if it is not enabled at the UE. But this requires additional communications between the gNB and the UE, which consumes UE power and system overhead. It is also additional complexity for the gNB to track the status of each individual UE.

    - If the UE does not notify the gNB, the gNB would need to transmit both WUS and the legacy paging, as it does not know whether WUS/WUR is enabled at the UE or not.

- Option 2: gNB determines whether to enable or disable WUS for a UE based on UE report.

  - The gNB may configure what the UE needs to measure and report and when to report.

  - Compared to Option 1, it requires more signaling exchange for the report, which consumes additional UE power.

Proposal 7: The enabling and disabling LP WUS/WUR for a UE can be done via the following:

- The gNB may broadcast a cell-specific message to enable LP-WUS (and provide configurations)

- If LP-WUS coverage is insufficient, the following options can be considered for UE-specific enabling and disabling of LP-WUS

  - Option 1: the UE determines whether to enable or disable LP-WUS/WUR

    - The gNB may provide configurable parameters for the UE to make such decision

    - The UE may notify the gNB on whether LP-WUS is enabled or disabled

  - Option 2: gNB determines whether to enable or disable LP-WUS/WUR for a UE based on UE report

    - gNB may configure the rules/parameters for UE measurement/report

## Procedures for idle/inactive UEs

As discussed in Section 2.1, we think DRX operation should be supported for LP WUS. Periodicity and offset can be configured for WUS monitoring. One issue that needs to be considered is how to define the WUS monitoring occasions, considering the possibility of multiple beams (even in case of FR1). There can be multiple WUS monitoring occasions defined, each corresponding to a beam. But a simpler approach would be to define a single monitoring time window, and the gNB can transmit WUS from different beams within this window.

Figure 4 Illustration for a WUS monitoring window

Proposal 8: For WUS monitoring with duty cycle operation, consider defining periodicity, offset and ON duration for the monitoring window.

After receiving WUS, the UE can wake up to monitor PEI or paging DCI (in case of a group-common WUS) or transmit PRACH directly (in case of a UE-specific WUS). For the latter (transmitting PRACH directly), false alarm needs to be very carefully considered, because the overhead for a false wake-up is very large in terms of both UE power consumption and system resource overhead. Due to this reason, waking up to monitor PEI or paging DCI may be preferable. For PEI or paging DCI monitoring, the most straightforward way is to follow the legacy monitoring occasions. However, this may create unnecessary delay. To reduce the delay, the UE could start PDCCH monitoring after an offset from WUS monitoring/detection (which allows the main radio to turn on and prepare), without having to wait for the legacy PO. Procedures can be defined accordingly.

Figure 5 Illustration for a WUS monitoring window

Observation 2: Transmitting PRACH directly after receiving WUS may not be desirable due to the overhead associated with the false alarm.

Proposal 9: For PEI or paging DCI monitoring after WUS reception, consider starting the monitoring after an offset from WUS monitoring/detection, without waiting for the legacy PO.

## Procedures for connected UEs

For connected UEs without C-DRX, LP-WUS can replace PDCCH monitoring when there is no data, to allow the main radio to stay in sleep state longer.

For connected UEs with C-DRX, LP-WUS can be used to replace R16 WUS, and at the same time, it can also replace PDCCH monitoring during C-DRX ON duration.

For either case, the UE procedures for transitioning into and out of LP-WUS monitoring need to be defined. For example, the UE may start LP-WUS monitoring and stop PDCCH monitoring if it receivers an explicit indication in a DCI, or it may do so after it does not receiver any DCI for a time duration. If it receivers the LP-WUS, it should then start PDCCH monitoring.

Proposal 10: For connected UEs, LP-WUS can be used with or without C-DRX. It can be used to replace PDCCH monitoring under certain conditions. It can also replace R16 WUS.

# Conclusion

In contribution, we have discussed WUS design and related procedures, and proposed the following:

Proposal 1: Duty cycle operation of WUR should be prioritized in the LP WUS design.

Observation 1: If synchronization signal is needed, Option 1 (aperiodic signal transmitted as part of LP-WUS) is not sufficient as a standalone solution, while the comparison between Option 2 (periodic signal) and Option 3 (Option 1 + Option 2) requires quantitative analysis based on the synchronization requirements.

Proposal 2: For idle/inactive UEs, a group ID may be carried/embedded in the LP-WUS. A special case is that the group ID is a UE ID, which becomes UE-specific indication.

Proposal 3: For idle/inactive UEs, a cell ID may be carried/embedded in the LP-WUS.

Proposal 4: The additional information carried for WUR should be avoided or minimized.

Proposal 5: Further consider a harmonized WUS design that can be detected by both OOK-based receiver and sequence-based receiver, by defining the sequence used to generate the WUS signal.

Proposal 6: For coverage enhancements, at least the following can be considered:

- Distributed transmission in frequency or frequency hopping to achieve frequency diversity

- SFN transmission for LP-WUS

Proposal 7: The enabling and disabling LP WUS/WUR for a UE can be done via the following:

- The gNB may broadcast a cell-specific message to enable LP-WUS (and provide configurations)

- If LP-WUS coverage is insufficient, the following options can be considered for UE-specific enabling and disabling of LP-WUS

  - Option 1: the UE determines whether to enable or disable LP-WUS/WUR

    - The gNB may provide configurable parameters for the UE to make such decision

    - The UE may notify the gNB on whether LP-WUS is enabled or disabled

  - Option 2: gNB determines whether to enable or disable LP-WUS/WUR for a UE based on UE report

    - gNB may configure the rules/parameters for UE measurement/report

Proposal 8: For WUS monitoring with duty cycle operation, consider defining periodicity, offset and ON duration for the monitoring window.

Observation 2: Transmitting PRACH directly after receiving WUS may not be desirable due to the overhead associated with the false alarm.

Proposal 9: For PEI or paging DCI monitoring after WUS reception, consider starting the monitoring after an offset from WUS monitoring/detection, without waiting for the legacy PO.

Proposal 10: For connected UEs, LP-WUS can be used with or without C-DRX. It can be used to replace PDCCH monitoring under certain conditions. It can also replace R16 WUS.

# Reference

- RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, RAN#97e, September 2021.

- R1-2303506, On low power wake-up receiver architectures, Apple, RAN1#112bis-e, April 2023.