3GPP TSG RAN WG1 Meeting #113	                                                       R1-2305277

Incheon, Korea, May 22nd – 26th, 2023

Agenda Item:	9.16.4

Source:	Apple

Title:	On UE features for Rel-18 NR network-controlled repeaters

Document for:	Discussion/Decision

# Introduction

In this contribution we share our views on the initially agreed UE features for NR network-controlled repeaters in Rel-18 in RAN1#112bis-e [1]. In the appendix section, we have listed the initial set of agreed FGs for NCR.

# Discussion on UE features for NR network-controlled repeaters

For FG 43-1, one of the remaining aspects is on the component of Tx/Rx timing determination for the backhaul/access link based on Tx/Rx timing of C-link. Based on the agreement in RAN1, there is a possible delay between receiving and forwarding. For the gNB to correctly determine the time difference and accordingly schedule, it is necessary for the gNB to be aware of the timing difference. Therefore, component 5 should be supported that allows NCR to report the timing difference as a mandatory capability whenever basic NCR support is reported. The exact values that can be reported by NCR can be further discussed. On the aspects of mandatory/optional, in our view, it is sufficient that FG 43-1 is optional without the capability signaling. We don’t see any specific need/benefit to have optional with capability signaling.

Proposal 1: For FG 43-1, agree the support of Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link (component 5)

- Candidate values can be further discussed

Proposal 2: FG 43-1 can be agreed as optional without capability signaling

On the aspect of which one among the three capabilities 43-2, 43-3 and 43-3 should be mandatory, if basic NCR support is reported, in our view, 43-3 should be the mandatory one and it should be merged with 43-1. With the functionality of aperiodic beam indication for access link, the NCR should be able to provide all the basic support in terms for forwarding for different physical channels/signals. FG 43-2 and FG 43-4 can be separate optional capabilities. Furthermore, for FG 43-3, it needs to be discussed whether to support slot-offset k values for reference slot. In our view, similar to other timeline related capabilities for beam application, this should also be a reported as  a capability. Candidate values can be further discussed.

Proposal 3: For beam indication for access link, merge FG 43-3 on aperiodic beam indication for access link with FG 43-1 to make this as mandatory feature for basic NCR support

Proposal 4: For aperiodic beam indication for access link, support slot-offset k values for reference slot as a reported capability

- Candidate values can be further discussed

For semi-persistent beam indication, the aspect of overriding the already configured beam index(es) by RRC configuration should be considered as a baseline feature with semi-persistent beam indication. Therefore, this FG can be merged with the UE capability signaling for semi-persistent beam indication for access link. Furthermore, it is still under discussion whether we add the word “temporary” to the override or not. Although, there might not be any impact to the functionality as such but adding temporary to override will make it clear that the override can be turned off with deactivation of semi-persistent beam index(es).

Proposal 5: For semi-persistent beam indication for access link, merge FG 43-4a on beam index updates for semi-persistent beam indication with FG 43-4

Proposal 6: For beam index updates for semi-persistent beam indication, keep temporary override in the component description

For FG 43-6 and 43-8, candidate values for the beam management framework needs to be discussed. In our view, if NCR reports these FGs, then there should be at least one beam management framework that is default, if no specific value is reported. The default should be Rel-15/16 framework. For the candidate values, Rel-17 can be additionally reported and consequently both Rel-15/16 and Rel-17 framework are supported. Alternatively, if there is no default value, then candidate values can be {Rel-15/16, both Rel-15/16 and Rel-17}.

Proposal 7: For FG 43-6 and FG 43-8, support one of the two options for the candidate values for beam indication

- Option 1: Rel-15/16 is default and doesn’t need to be explicitly reported. Rel-17 can be optional value that can be reported in which case of both Rel-15/16 and Rel-17 are assumed to be supported

- Option 2: Two candidate values are agreed (no default value assumed) including {Rel-15/16, both Rel-15/16 and Rel-17}

# Conclusion

In this contribution, we have discussed our views on the initial list of UE features for network-controlled repeaters in NR Rel-18 and provided following proposals:

Proposal 1: For FG 43-1, agree the support of Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link (component 5)

- Candidate values can be further discussed

Proposal 2: FG 43-1 can be agreed as optional without capability signaling

Proposal 3: For beam indication for access link, merge FG 43-3 on aperiodic beam indication for access link with FG 43-1 to make this as mandatory feature for basic NCR support

Proposal 4: For aperiodic beam indication for access link, support slot-offset k values for reference slot as a reported capability

- Candidate values can be further discussed

Proposal 5: For semi-persistent beam indication for access link, merge FG 43-4a on beam index updates for semi-persistent beam indication with FG 43-4

Proposal 6: For beam index updates for semi-persistent beam indication, keep temporary override in the component description

Proposal 7: For FG 43-6 and FG 43-8, support one of the two options for the candidate values for beam indication

- Option 1: Rel-15/16 is default and doesn’t need to be explicitly reported. Rel-17 can be optional value that can be reported in which case of both Rel-15/16 and Rel-17 are assumed to be supported

- Option 2: Two candidate values are agreed (no default value assumed) including {Rel-15/16, both Rel-15/16 and Rel-17}

# Reference

- 3GPP RAN1#112bis-e, “Chairman Notes”

# Appendix - Initial agreed FGs for NR network-controlled repeaters in Rel-18

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-1 | Basic NCR support | 1. Support of fixed beam for C-link/backhaul link 2. Support of TDMed UL transmission of C-link and backhaul link 3. Support of ON-OFF operation for NCR-Fwd based on access link beam indication 4. Support of TDD UL/DL determination for backhaul/access link based on TDD UL/DL configuration of C-link 5. [Support of] Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link 6. Support of beam correspondence of the DL/UL of the access link at NCR-Fwd | FFS | N/A | Yes | NCR is not supported | Per NCR-MT | No | No | Yes | A NCR-MT that includes ncr-NodeIndication in RRC Setup Complete must support FG 43-1 [Component 5 candidate values: FFS] | Optional [with/without] capability signaling |
| 43. NR_netcon_repeater | 43-2 | Periodic beam indication for access link | 1.Support periodic beam indication for access link 2. Priority flag for periodic indication | 43-1 | N/A | Yes | NCR-MT cannot decode the periodic beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-3 | Aperiodic beam indication for access link | 1.Support aperiodic beam indication for access link [2. Supported slot-offset k values for reference slot] | 43-1 | N/A | Yes | NCR-MT cannot decode the aperiodic beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one FFS: component 2 candidate values | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-4 | Semi-persistent beam indication for access link | 1.Support semi-persistent beam indication for access link 2. Priority flag for semi-persistent indication | 43-1 | N/A | Yes | NCR-MT cannot decode the semi-persistent beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-4a | Beam index updates for semi-persistent beam indication | 1.Support of [temporary] override of the RRC configuration of the beam index(es) in semi-persistent beam indication | 43-1, 43-4 | N/A | Yes | The semi-persistent beam indication cannot provide updates for the beam index(es). | Per NCR-MT | No | No | Yes | FFS: whether to merge this FG with 43-4 | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-5 | Simultaneous UL transmission of backhaul link and C-link | 1. Simultaneous UL transmission of backhaul link and C-link | 43-1 | N/A | Yes | NCR only supports TDMed UL transmission of C-link and backhaul link | Per NCR-MT | No | No | Yes |  | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-6 | Dedicated signalling for backhaul link beam indication | 1. Support dedicated signalling for backhaul link beam indication | 43-1, 43-8 | N/A | Yes | Dedicated signalling for backhaul link beam indication is not supported | Per NCR-MT | No | No | Yes | [Component candidate values: {Rel-15/16, Rel-17, both}] | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-8 | Adaptive beam for NCR backhaul link/C-link | Support of backhaul link beam determination based on predefined rule | 43-1, 2-2,2-4, 2-4a | N/A | Yes | The beam for backhaul link and C-link is fixed. | Per NCR-MT | No | No | Yes | FFS: Component candidate values: {Rel-15/16, Rel-17, both} | Optional with capability signaling |