3GPP TSG RAN WG1 #113	R1-2305317

Incheon, Korea, May 22nd – May 26th, 2023

Title:	Draft Reply to the LS on 1-symbol PRS

Response to:	R2-2304510

Release:	Rel-18

Work Item:	TEI18

Source:	Qualcomm [RAN1]

To:	RAN2

Cc:	RAN3

Contact person:

Name:		Alexandros Manolakos

E-mail Address:	amanolak@qti.qualcomm.com

# Overall Description

RAN1 thanks RAN2 for their LS on the 1-symbol PRS. In the following question addressed to RAN1:

Question: Do we need changes to DL PRS configuration used for RTT-based Propagation Delay Compensation?

RAN1 would like to provide the following response:

Reply:

In RAN1#112 meeting TEI agenda, RAN1 has made the following agreement for NR positioning

- Introduce 1-symbol PRS with legacy comb sizes. - UE expects the suitable expected RSTD windows provided by LMF such that peak ambiguity is addressed. Otherwise no measurement accuracy requirements are expected to be met. - Not to define RAN4 RRM requirement, including core/performance in Rel-18 Send an LS to RAN2 and RAN3 to ask necessary signalling enhancements

Following the above, in RAN1#112-bis meeting TEI agenda, RAN1 captured in R1-2304238 the higher layers parameters for the 1-symboL PRS (TEI18) and introduced related UE capabilities for 1-symbol PRS for PDC.

In addition to the already provided RRC parameter changes, for the 1-symbol PRS PDC, since there is no expected RSTD window provided to the UE for peak ambiguity purposes, RAN1 proposes the following note to be added in the numSymbols field description:

- The UE does not expect to be configured for PDC with a PRS with numSymbols equals to n1 unless an SSB index is provided as a Type-C or Type-C & Type-D QCL source, or another PRS resource with numSymbols more than 1 is provided as QCL source.

# Actions

To RAN2:

Action: RAN1 kindly asks RAN2 to take the above reply into account for further work.

# Dates of Next TSG-RAN1 Meeting:

TSG-RAN WG1 Meeting #114	August 2023      	     Toulouse, FR

TSG-RAN WG1 Meeting #114-bis	October 2023      	     Xiamen, CN