3GPP TSG-RAN WG1 #113	 R1-2305338

Meeting, May 22th – 26th, 2023

Agenda item:	9.4.1.2

Source: 	Qualcomm Incorporated

Title: 	Physical Channel Design for Sidelink on Unlicensed Spectrum

Document for:	Discussion/Decision

# Introduction

In RAN1#112-bis, we have reached 14 agreements and one working assumption which includes

- Additional S-SSB occasion exclusion from resource pool, contiguous RB subchannel handling when overlapping with intra-cell guard band, same subchannel index(s) across multiple RB-sets, and interlaced subchannel #0 alignment with NR-U interlace #0

- Reference symbol duration for TBS calculation with two starting symbols, Tx UE behavior on choosing the starting symbol

- Reference number of RB for TBS calculation for interlaced subchannel, PSCCH resource mapping to lowest subchannel index in the allocation, two-part FRIV

- S-SSB with configurable frequency repetition if necessary, S-SSB repetition in more than one RB-set if necessary, allowing UE to attempt to transmit on additional S-SSB occasion(s)

- PSFCH interlaced waveform options addressing the UE multiplexing capacity issue

- Preconfigured or/and dynamically indicated multiple PSFCH occasion(s) to alleviate LBT failure for PSFCH transmission

In this paper, we share our views on the design options, continue to tackle FFS items in the agreements and provide some insights on the potential issues within the scope. For half slot access (allow two candidate starting symbols in a slot), we elaborate designs on AGC symbols in half slot structure, reducing the Tx/Rx UE complexity and protecting the AGC of the full-slot receiver. Techniques for reducing the PAPR of the frequency repeated S-SSB are studied. S-SSB slot padding signal is also introduced to maintain COT transmission across S-SSB gaps. To accommodate eMBB traffic, there may also be a need to introduce new PSFCH which can carry HARQ codebook (reusing existing PUCCH formats) and type-1/2/3 HARQ codebook designs could be considered.

Below is the summary of further design aspects that SL-U needs to consider

- AGC symbol location in half/full slot structure (with two candidate starting symbols in a slot), complexity reduction and AGC problem remediation for half-slot channel access

- Resource allocation and reservation: two-part FRIV based on legacy FRIV encoding for both interlace and RB-set

- PUCCH format 0 (Rel’16 SL only supported format), capacity enhanced PSFCH and PUCCH format 2 based PSFCH enhancements

- HARQ codebook design

- Multiple PSFCH opportunities and PSFCH resource mapping

- Multiple S-SSB candidate location config, S-SSB padding for wideband COT and S-SSB candidate occasions to maintain COT transmission burst across S-SSB slot gap(s)

# Discussion

## Resource Pool

### Interlaced Subchannel

In RAN1 #110bis and #112, we made the following agreements regarding subchannels and RB-sets

Agreement (#110bis) Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets: - Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets - Such PRBs are not used for PSCCH transmission FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission Agreement (#112) For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and interlace, 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool Agreement (#112-bis) For interlace RB-based PSCCH/PSSCH transmission in SL-U, regarding details of mapping between sub-channel and interlace: - In a resource pool with multiple RB sets, sub-channel with the same index is mapped to K interlace(s) with the same index(s) in different RB sets. - In a resource pool, support the following - At least for the agreed case where one SL resource pool can be (pre-)configured to include integer number of RB sets - Option 2: sub-channel#0 is mapped to K interlace(s) starting from interlace#0 - sub-channel#1 is mapped to K interlace(s) starting from interlace#K, and so on - At least support that the above K interlace(s) are contiguous - FFS: whether/how to support the above K interlace(s) are non-contiguous - FFS: if RAN1 agrees to support that one SL resource pool can be (pre-)configured to include sub-set of PRBs of one RB set, the mapping between sub-channel and interlace for this case will be further discussed - Interlace is indexed as per NR-U For interlace RB-based PSCCH/PSSCH transmission in SL-U, considering 1 sub-channel equals K interlace(s), support the followings: - Option A: - TBS is determined based on a reference number of PRBs of one interlace within 1 RB set (denoted as N_ref), down-select one of the followings in RAN1#113: - Option A1: N_ref is a fixed value, e.g., 10, 11 - Option A2: N_ref is (pre-)defined - e.g., N_ref is the average number of PRBs per interlace, which is determined by total number of PRBs of the RP divided by the number of interlaces. - Option A3: N_ref is (pre-)configured - Option A4: N_ref is dynamically indicated by Tx UE - Note: The number of PRBs within a sub-channel can be different among sub-channels in a single resource pool subject to (pre-)configuration. - FFS: for TBS determination, whether/how to handle the impact of additional available PRB(s) in intra-cell guard band(s) for PSCCH/PSSCH transmission across multiple RB sets

In RAN1 112-bis agreement, the same subchannel index(ices) are chosen in different RB-sets for the wideband transmission. Based on the RAN1 110-bis agreements, the RBs in the intra-cell guard bands can be used for the PSSCH transmission if and only if a UE can transmit on the respective RB-set(s) after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission. Whether the PSSCH transmission in the intra-cell guardband should take the same interlace index as the PSSCH in the RB-sets is still FFS. In our view, choosing interlaced RBs in the guard bands with different interlace index(es) from the allocated interlaces in the RB-sets may have potential PAPR problem. If one or more interlaces are allocated in two (contiguous) RB-sets, the corresponding interlace RBs (same interlace index as the PSSCH allocation in the RB-sets) in the intra-cell guard band between the two RB-sets are preferred to be included for resource allocation as shown in Figure 1.

Proposal 1: For interlace resource pool, if one or more interlaced subchannels are allocated in two (contiguous) RB-sets, the interlaced RBs (with the same interlace index(es) as the RBs for PSSCH transmission in these two RB sets) in the intra-cell guard band between the two RB-sets are included for resource allocation

Figure 1: Two interlaced subchannels allocated across two RB-sets and in the intra-cell guard band

The interlaced subchannels within an RB-set or in different RB-sets may contain different number of interlaced RBs as the result of intra-cell guard band as shown in Figure 1. With potentially different subchannel assignments for the initial transmission and re-transmission (i.e., different interlace indices or RB-sets), the number of PSSCH resources could be different among re-transmissions of the same TB. As TB size is expected to be the same for the initial transmission and re-transmission, the transmitter may want to choose TB size based on some nominal number of RBs for the allocated subchannel(s) and rate match to actual number of REs for transmission. In the RAN1 112-bis agreement, option A4 with dynamic indication Nref requires additional bit in SCI, and option A2/A3 may have inconsistent assumption/configuration among different UEs, e.g., different assumption on RB-set config in initial message exchanges. We prefer the nominal number of RBs for a subchannel being a fixed value, e.g. Nref=10 RB, since most interlaced subchannels contains 10RBs among different possible RB-set configs (option A1). And the nominal number of RBs could be determined by

NRBnominal=Nref×Nsubch×NRBset

where Nsubch is the number of allocated interlaced subchannels per allocated RB-set, and NRBset is the number of the allocated RB-sets.

Proposal 2: For potentially different sizes of interlaces, calculate TBS size based on the fixed reference number of PRBs per interlace, e.g. Nref=10RBs (option A1)

### RB-set configuration considerations

In NR-U, the DL/UL RB-sets are RRC configured by gNB per UE. The RB-set config specifies the guard band in between the 20MHz LBT subbands. For SL mode 2, we cannot always rely on gNB for the SL RB-set configurations. Two options could be considered

Option 1: Common RB-set configuration for all SL nodes in the network via UE profile or gNB configuration.

Option 2: Allow RB-set configuration to be set up per link via RRC messages

Option 1 seems to be the simpler solution for deployment. However, different UEs from different vendors could have different implementations and guard band requirements could be different. Having a (worst case) common RB-set that all UEs can support could be a solution, but sacrifices the spectral efficiency as the interlace RBs in the guard band(s) cannot potentially be used for transmission by high-end SL nodes.

For option 2, distributed SL UE to UE communication needs to have common assumptions among UEs with respect to the initial RB-set configuration over which a new RB-set configuration, used for the following transmissions, can be delivered/agreed upon via RRC message exchanges. Hence, some minimum RB-set configuration as shown in Figure 2 needs to be defined via network config or UE profile so two nodes can communicate before the per link RB-set configuration is available. The minimum RB-set configuration could be the intersection of all the supported RB-set configs in the specification. This minimum RB-set could also be used for broadcast and groupcast option 1 transmissions.

Proposal 3: The RB-set config for 20MHz subbands could be either common config for all SL nodes/links in the NW or configurable per link via RRC message exchange

Proposal 4: If RB-set config per link is configurable, one may need to define the minimum RB set for a 20MHz subband for the initial RRC message exchange in order to set up the RB-set config of a link

Figure 2: Minimum RB-set config

## Slot Structure

In this section, we discuss the optimization in slot structure to create two transmission starting positions within a single slot, which allows UE to transmit as early as possible after clearing the LBT, and control signaling optimization for scheduling multiple slots transmission.

### Half-slot structure for half-slot channel access

In RAN1 #112, we made the following three agreement regarding the half-slot structure which allows 2 candidate starting symbols for PSCCH/PSSCH in a slot. And in RAN1 #112-bis, there is a nearly stable FL proposal regarding the Tx UE behavior on have one or two starting symbols for the COT MCSt transmission.

Agreement (#112) For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - The location of 1st starting symbol can be (pre)configured from {#0,#1,#2,#3,#4,#5,#6} per BWP - By default (if no (pre)configuration), the location of the 1st starting symbol is symbol#0 - The location of 2nd starting symbol is (pre-)configured from {#3,#4,#5,#6,#7} per BWP - It shall be configured such that within a slot, the number of symbols used for PSCCH/PSSCH transmission from 2nd starting symbol is not smaller than 6 - It shall be configured such that within a slot, the 2nd starting symbol is later than the 1st starting symbol - PSCCH/PSSCH transmission starting from 1st or 2nd starting symbol shall have the same ending symbol within a slot - Note: assume symbol index in a slot starts from #0 Agreement (#112-bis) Regarding Tx UE behavior, at least when it initiates a COT: - For the 1st slot of a COT, the Tx UE chooses the earliest starting symbol for PSCCH/PSSCH transmission after clearing LBT. - Note: in the same slot, Tx UE can use the 2nd starting symbol only if LBT fails at the 1st starting symbol - FFS: whether/how to support that for the remaining slots of a COT, the Tx UE only chooses the 1st starting symbol for PSCCH/PSSCH transmission. - FFS applicable scenarios - e.g., at least for MCSt with no greater than 16us gap - e.g., at least for transmission with no greater than 16us gap from the previous transmission by any UE - FFS: Rx UE behavior FFS: COT sharing case

With the 2 preconfigured candidate starting symbols for PSCCH/PSSCH transmission as in the agreement, we can define a half slot starting from the 2nd candidate starting symbol (e.g. symbol #7) and a full slot starting from the 1st candidate starting symbol (e.g. symbol #0) as shown in Figure 3. One can completely reuse the legacy DMRS pattern based on the number of PSSCH and PSCCH symbols.

To support COT MCSt transmission, the Tx UE may choose half slot or full slot with the earliest transmission starting point after clearing the LBT and, if it starts its transmission with the half-slot, switch to full slot transmission after transmission of the 1st half-slot for better spectrum efficiency and COT continuation, i.e., avoiding leaving greater than 16us gaps in between COT transmission burst (see Figure 4).

From the receiver point of view, as the PSCCH/PSSCH transmission could start at either 1st or 2nd starting symbol, based on the transmitter LBT outcome, it needs to monitor two PSCCH/PSSCH occasions in each (full) slot.

Proposal 5: For the PSSCH in the half/full slot starting from the 2nd/1st starting symbol, reuse the legacy DMRS pattern which is based on the number of PSCCH/PSSCH symbols

Proposal 6: For the 1st slot of the COT transmission, the transmitter chooses the earliest starting symbol after clearing the LBT and, for the remaining slots of a COT (at least for MCSt), the Tx UE may only choose the 1st starting symbol for PSCCH/PSSCH (supporting the RAN1#112-bis FFS)

Proposal 7: The half-slot access could be configured per link. If configured, the receiver monitors two PSCCH occasions in each (full) slot by default

Figure 3: a) half-slot structure, b) full-slot structure

Figure 4: Tx UE starts the COT with earliest starting position, resulting in a first half-slot transmission, and switches to full-slot transmission

In RAN1#112 agreement, it is agreed that the 2nd starting symbol needs to be later than the 1st starting symbol. However, the PSCCH/PSSCH waveform starting from the 2nd starting symbol may be different from the 1st starting symbol, e.g., the DMRS scrambling sequence depends on symbol index and the DMRS pattern may be different. For some Tx UE implementations which generate per symbol waveform on the fly, a few symbols after the 1st starting symbol may be needed to accommodate Tx UE timeline for generating the waveform from the 2nd starting symbol if Tx UE fails the LBT at the 1st starting symbol. Hence, we need to determine the number of symbols needed for Tx UE  to prepare the Tx waveform starting from the 2nd starting symbol when the LBT fails at the 1st starting symbol.

Proposal 8: The (pre-)configured 1st and 2nd starting symbol positions are separated by X symbols (FFS: the value of X)

#### AGC

Agreement (#111) For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - Regarding Tx UE behaviour: - If PSCCH/PSSCH transmission starts from 1st starting symbol, down-select one of the followings - Option 1: The PSCCH/PSSCH transmission has 2 symbols for AGC purpose - Option 2: The PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Option 3: The PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, FFS details - If PSCCH/PSSCH transmission starts from 2nd starting symbol, the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Regarding Rx UE behaviour, down-select one of the followings: - Option A: The Rx UE always monitors two AGC symbols in such slot - Option B: The Rx UE monitors two AGC symbols in such slot by default, but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol - FFS details - Option C: The Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot FL proposal (#112-bis) [M] Proposal 2-2-1e (AGC, Tx) For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, support the following: - Regarding Tx UE behaviour: - If PSCCH/PSSCH transmission starts from 1st starting symbol, - The PSCCH/PSSCH transmission has 1 or 2 AGC symbols subject to (pre-)configuration - FFS: whether this (pre-)configuration is explicit or implicit - Note: AGC symbol is duplication of next symbol [M] Proposal 2-2-2e (AGC, Rx) For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, support the following: - Regarding Rx UE behaviour: - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot - Note: AGC symbol is duplication of next symbol

There are some concerns that a UE transmission not starting at the (full) slot boundary (i.e., half-slot transmission) might cause AGC problem to UEs trying to receive a full-slot at the same time. As an example shown in Figure 5: a) AGC margin b) Protecting the full-slot reception via SCI-1 detection the half-slot transmission from transmitter #1 may impact the full-slot receiver #2’s AGC in case that transmitter #1 can clear the LBT in the middle of the slot transmission. The AGC of receiver #2 is trained based on the first AGC symbol in the slot. If transmitter #1 is close to receiver #2, the AGC setting of receiver #2 may not be appropriate for the second half of the full slot and clipping may occur in A/D if total signal power of transmitter #1 and transmitter #2 exceeds the interference margin.

It is worthy to note that the AGC issue may occur across intra-RAT and inter-RAT nodes in unlicensed band. WiFi will anyway start its transmission anywhere in the middle of an ongoing full-slot SL transmission if it clears LBT and does nothing to protect SL-U, i.e., AGC issue will nevertheless occasionally occur by WiFi access alone. Introducing additional AGC symbols for SL-U transmission (option 1) would not solve the Rx UE AGC issues of AGC training not capturing interference starting from non-AGC symbols. Hence, the SL-U Rx UE needs to select a conservative AGC setting which has enough interference margin for A/D converter.

Observation 1: WiFi interference may start anywhere in the middle of an ongoing SL slot reception and introducing additional AGC symbols within a slot cannot solve the Rx UE AGC issue of getting strong interference starting from non-AGC symbols. Instead, SL-U Rx UE needs to choose a conservative AGC setting allowing more interference margin

But still, there are also some RAN1 discussions on whether to have an additional AGC symbol in the middle of the full-slot transmission so that the full-slot receiver can retune the AGC if there is a half slot transmission starting in the middle of the slot. Retuning AGC in the middle of the slot may hurt the phase continuity for PSSCH reception and additional AGC symbol overhead is undesirable. Additional AGC symbols also requires redesigning DMRS patterns to avoid getting punctured/replaced by AGC symbol (which happens at least for some combinations of second starting symbol location and DMRS pattern, e.g., #7 and 4-symbols DMRS).

Rather, if we want to completely avoid the case described above among SL-U devices, one can further limit the half-slot transmission (beyond the limits already imposed by LBT operation) if the transmitter detects full slot SCI-1 or the PSCCH’s DMRS RSRP is above some preconfigured threshold based on priority in the same slot and the RB-sets close to the intended half-slot transmission.

The Rx UE would monitor both AGC candidate symbols in the half-slot access slots since it does not know the transmission starting symbol a priori. But when a transmission starting from the 1st starting symbol is detected (either by PSCCH’s DMRS or SCI-1), the Rx UE may skip the monitoring the 2nd AGC candidate symbol since the 2nd AGC training could be disruptive to channel estimation for full-slot PSCCH/PSSCH reception.

Hence, we support option 2 for Tx UE behavior and option B/D for Rx UE behavior.

Observation 2: Additional AGC symbol in the middle of full slot PSSCH is undesirable since AGC retuning in the middle of the slot would hurt phase continuity for PSSCH reception, reduces PSSCH resources and may require redesigning some of DMRS patterns

Proposal 9: For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, we support option 2, i.e., the Tx UE transmits only 1 AGC symbol no matter the PSCCH/PSSCH transmission starts from 1st or 2nd starting symbol

Proposal 10: For a slot with 2 candidate starting symbols for a PSCCH/PSSCH reception, the Rx UE monitors two AGC symbols in such slot by default, but may only adjust its AGC setting once based on the 1st starting symbol if a PSCCH or DMRS transmission starting from the 1st starting symbol is detected (option B/D)

Proposal 11: To protect the full-slot receiver, a transmitter may initiate transmission from the 2nd starting symbol, only if it has attempted to decode PSCCHs from the 1st starting symbol and has not identified/detected presence of SCI-1 with a priority higher than a (pre-) configured threshold and/or PSCCH DMRS with RSRP higher than a (pre-) configured threshold in the same slot and the RB-sets close to the intended half-slot transmission

Figure 5: a) AGC margin b) Protecting the full-slot reception via SCI-1 detection

Another approach is to limit half-slot channel access only under operational conditions that really call for it, which are under significant WiFi interference. In the extreme case of no WiFi interference, half-slot access offers no benefit (and can potentially harm concurrent full-slot transmissions, as discussed above). A UE can use metrics such as LBT failure rate and CBR to determine whether it operates under strong WiFi interference conditions and, if so, proceed to transmit over half slot.

Proposal 12: A transmitter may initiate transmission with 2 starting symbols in a slot as long as it has detected strong WiFi interference conditions indicated by metrics such as LBT failure rate and CBR, possibly accounting for priority of transmission

#### TBS Calculation

Agreement If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - TBS is determined based on a reference number of symbols (denoted as L_ref) - Support the followings - Alt 1: Support Option 4 only - Note: the options are as below - Option 4: The reference number of symbols is determined by (pre-)configuration - FFS details, e.g., in TS 38.214 Clause 8.1.3.2, whether L_ref replaces Nsymbsh or sl-LengthSymbols or other usage of L_ref, whether NsymbPSFCH is determined in the same way as in legacy NR SL, etc.

Based on the agreement from RAN1#112-bis, if the resource pool is configured with slots with 2 starting symbols, the TBS calculation for all the slots is determined by preconfigured number of symbols (denoted as Lref).  However, the choice of Lref is critical to resource utilization. If the Lref is not chosen based on the 1st starting symbol, e.g., if based on the 2nd starting symbol and the 2nd starting symbol is preconfigured to be symbol #7, the code rate or the highest throughput rate is effectively half. Moreover, not all the SL slots have two starting symbols, e.g., the slot containing PSFCH has one starting symbol only, it may not be wise to under-utilize the resources.

Hence, we believe that Lref needs to be chosen based on the 1st starting symbol for maximizing the resource utilization and the Tx UE, by implementation, may choose lower MCS for the slots with two starting symbols if the chance to clear LBT before the 1st starting symbol is not high.

Observation 3: For the resource pool configured with two starting symbols, if the reference number of symbol (Lref) for TBS calculation is chosen based on the 2nd staring symbol, e.g. Lref=7 , the highest throughput rate is greatly reduced.

Proposal 13: For the resource pool configured with two starting symbols, the transmitter determines the reference number of symbol (Lref) for TBS calculation based on the 1st starting symbol, and Tx UE (by implementation) may choose lower MCS for the slots with two starting symbols when the chance to clear LBT before the 1st symbol is not high

#### Receiver complexity reduction

If a link is configured to support half-slot channel access, the receiver needs to monitor PSCCH at multiple starting symbols (occasions) within a (full) slot. The UE implementation may be feasible if the number of PSCCH candidates per slot is less than or equal to 44 or 32 for 15KHz and 30 KHz SCS, respectively, referring to the UE capability of up to 44/32 blind PDCCH decodes per (full) slot in Uu for 15KHz/30KHz SCS [4]. The number of PSCCH candidates per (full) slot, when half-slots are supported, is twice the number of subchannels. As shown in the table below, both 15 and 30 KHz SCS, with 10 and 5 subchannels included per 20MHz, respectively, can support up to 40 MHz bandwidth.

|  | BW=20MHz | 40MHz | 80MHz | 100MHz |
| --- | --- | --- | --- | --- |
| SCS 15kHz | 20 | 40 | 80 | 100 |
| SCS 30kHz | 10 | 20 | 40 | 50 |

Table 1: Number of PSCCH candidates per slot for SL RP with half-slot support

To support wider transmission bandwidths than those indicated above, the following options may be considered:

Option 1: Reduce SCI search space in the half slot (starting at 2nd starting symbol) as illustrated as option 1a in Figure 6, or in both starting symbols (e.g. symbol#0 and symbol#7) in half-slot access slots as illustrated as option 1b in Figure 6.

- PSCCH and the starting subchannel of PSSCH only occur within a subset of the total subchannels available in resource pool , e.g., PSCCH are only present in even-indexed subchannels for each 20MHz RB-set at symbol #7 (as shown in Figure 6 option 1a) could reduce the number PSCCH candidates in 80MHz and SCS=30KHz case from 40 to 32.

Option 2: Reduce PSCCH candidate searches at receiver, based on some skipping rule per slot and per RB-set

- For a given RB-set, if there is a first transmitter which clears the LBT before symbol #0 and starts a full-slot transmission, it is less likely for a second transmitter operating in the same RB-set to clear the LBT in the middle of the full-slot transmission and start a half-slot transmission from symbol #7, unless two transmitters are outside the ED range. The receiver may want to skip the PSCCH monitoring at later potential PSCCH candidate locations within a slot if the SCI-1 or PSCCH DMRS is detected in the earlier candidate location within the same slot and RB-set. More specifically, if the SCI-1 is decoded and/or the RSRP of PSCCH DMRS is greater than a preconfigured threshold at a PSCCH candidate location in a slot and RB-set, the receiver can skip the PSCCH monitoring in the remainder of the slot (see Figure 7 for an example).

Figure 6: Option 1a) Reduced PSCCH search space in half slot. Option 1b) Reduced PSCCH search space at #0 and #7

Proposal 14: Limit PSCCH search locations to a subset of subchannels in half-slot access slots to meet the blind SCI decoding limit per slot

Proposal 15: When half-slot access is configured, the receiver may skip the PSCCH monitoring at later potential PSCCH candidate locations in a (full) slot within an RB-set, if, in an earlier candidate location in that slot and the same RB-set, SCI-1 is decoded and/or PSCCH DMRS RSRP is above some preconfigured threshold

Figure 7: PSCCH candidates skipping

#### UE capability and applicability of traffic type

With half-slot access being a feature that potentially increases UE complexity, it may be argued that not all UEs in the network are capable of supporting it. Therefore, a system-wide application of half-slot access may not be possible. In that case, one alternative would be to selectively enable half-slot access only for links among high-end UEs that are actually capable of transmitting and receiving on a half-slot granularity. The UE capabilities would be reported during the initial RRC setup stage and half slot access could be RRC configured per link. For broadcast or groupcast option 1 transmissions, where the capability of receiver UEs is unknown, no half-slots transmissions would be expected.

Proposal 16: Half slot access could be RRC configured per link based on UE capability for unicast and groupcast option 2

### Multiple contiguous slots transmission and signalling

Towards making the most out of a cleared LBT attempt, a transmitter may utilize multiple contiguous slots for transmission of large size data (i.e., multiple TBs). This avoids multiple LBTs for multiple standalone slots/TBs and improves channel access efficiency significantly. In addition, with multiple contiguous slots per transmission (burst of slots), the following aspects can contribute to transmission overhead reduction.

- Instead of having each slot transmitting PSCCH as per Rel’16, a single PSCCH can appear only in the first slot of the burst (COT) with the corresponding SCI-1 scheduling/reserving all slots in the burst (multi-TTI SCI grant), as shown in the example of Figure 9a. This may require a change from the PSCCH-to-PSFCH resource mapping in Rel’ 16 to a PSSCH-to-PSFCH resource mapping

- In mode 1, instead of having multiple DCI3-0 scheduling multiple contiguous slots/TBs, an enhanced, single multi-TTI “DCI3-x” grant could be used to schedule multiple contiguous slots/TBs (see Figure 8).

- Further overhead reduction can be achieved by avoiding repetition of common SCI-2 parameters (if any) across TBs/slots such as destination ID, cast type indicator, zone ID, etc. Only a single SCI-2 is transmitted in the first slot with the understanding that its parameters/fields are applicable to all subsequent slots/TBs in the transmission (Figure 9a). In case an SCI-2 parameter/field is not common to all TBs, TB-specific SCI-2 can appear along with the PSSCH of a TB that indicates the “correct” parameter for that TB (see Figure 9b). One example case is that of TBs with the same SCI-2 parameters but directed to different destination IDs. In that case, each PSSCH is transmitted with TB-specific SCI-2 that is used to indicate the corresponding destination ID.

- DMRS overhead reduction: In one example, with all slots/TBs directed towards the same UE(s), reduction of DMRS symbols in some or all slot(s) may be possible under low mobility conditions with no performance cost, leveraging DMRS symbols available in adjacent slots

- Utilization of gap and AGC symbols for PSSCH: last gap symbol between consecutive slots can be utilized as additional PSSCH symbols (with the exception of the gap symbol of the last slot of the burst). Furthermore, the AGC symbol of all slots following the first in the burst could also be used for PSSCH transmission. One example is shown in Figure 9. These two features may be activated/indicated in first slot SCI

Proposal 17: Support enhanced DCI3_x to schedule multiple contiguous slots in mode 1 and further study the minimum signaling required

Proposal 18: Support enhanced SCI-1/2 to schedule multiple contiguous slots for one or multiple SL receivers and further study the minimum signaling required

Figure 8: Multi-TTI DCI grant for 4 slots

(a)

(b)

Figure 9: a) Multi-TTI SCI-1 grant for a 3 slots/TBs burst with common SCI-2 (arrows indicate which channel(s) control signaling applies to); b) same as a) but with TB-specific SCI-2 indicating TB-specific SCI-2 (and, possibly, SCI-1) parameters. AGC and gap symbols within the burst are used as additional PSSCH symbols

## Physical Waveform

### PSCCH and PSSCH

#### PSCCH resource mapping

Agreement (#110-bis) For PSCCH and PSSCH in SL-U: - PSCCH is transmitted within 1 sub-channel - At least support Option 1 below - Option 1: PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH - Note: the lowest sub-channel may not be entirely contained in the lowest RB set - FFS whether/how to handle the case where UEs supporting different bandwidths can use the same resource pool to communicate with each other, e.g., whether/how to additionally support Option 2 below - Option 2: PSCCH locates in every RB set of corresponding PSSCH - Note: the above options do not imply any restriction on the mapping of sub-channels to PRBs. - FFS other details Agreement(#112-bis) For interlace RB-based PSCCH/PSSCH transmission in SL-U, support the following: - Option 1: lowest sub-channel is the sub-channel with smallest sub-channel index

##### interlaced waveform

In Rel’16 SL, the first subchannel that PSSCH occupies is the leading (starting) subchannel and the scheduling PSCCH is contained in the leading subchannel. The FRIV in SCI-1 uses the PSCCH subchannel as the reference point for resource indication. For SL-U, the PSCCH could allocate multiple interlaces and RB-sets for PSSCH transmission in which the intra-cell guard bands in the middle of scheduled RB-sets could be occupied by PSSCH as shown in Figure 10. Since the basic resource allocation unit for SL-U is one subchannel in one RB-set, we may want to put PSCCH into the first (leading) subchannel and the first (leading) RB-set that PSSCH occupies by reusing the legacy SL design and the PSSCH in SL-U could TDM and/or FDM with the PSCCH. Repetition of PSCCH in multiple RB-sets (option 2 in the RAN1 #110-bis agreement) may cause FRIV interpretation confusion on the receiver end since the reference point the FRIV uses is unknown when the receiver does not monitor all RB-sets. In Rel’16 SL, the PSCCH occupies a (pre-) configurable number of RBs, i.e., {10, 12, 15, 20, 25} RBs.  To allow configurability of PSCCH within one subchannel as in Rel’16 while keeping PSCCH resource mapping simple, we may allow PSCCH to occupy a (pre-) configurable number of leading interlaces in the leading subchannel/RB-set. For example, if one interlace within one RB-set consists of 10 RBs, then PSCCH could occupy 10, 20,…,  or 10⋅K-1  RBs in which  K is the number of interlaces within one subchannel.

Moreover, within a 20MHz LBT subband, the PSCCH is rate matched to the common or minimum RB-set. Even if we allow RB-set to be configurable per link via RRC messages, confining the rate matching to the minimum RB-set is necessary since PSCCH contains reservation information that needs to be understood by other SL nodes with different RB-set configs.

Observation 4: Repeating PSCCH in every RB-set of the corresponding PSSCH transmission as in option 2 may cause receiver confusion on FRIV reference point if the receiver does not monitor all RB-sets configured at the transmitter

Proposal 19:  Support option 1 for PSCCH resource mapping in which PSCCH is located in the lowest sub-channel of lowest RB set of corresponding PSSCH

Proposal 20: For interlaced waveforms, PSCCH occupies (pre-)configurable number of leading interlaces in the leading subchannel within the leading RB-set of the allocated resources and PSSCH is multiplexed with PSCCH via TDM and/or FDM

Proposal 21: For interlaced waveforms, PSCCH is rate matched to the common or minimum RB-set within a 20MHz subband to allow nodes with different RB-set configurations to communicate to each other

Figure 10: PSCCH resource mapping

##### Contiguous RB waveform

Agreement (#112-bis) For contiguous RB-based PSCCH/PSSCH transmission in SL-U, regarding sub-channel(s) which include intra-cell guardband PRBs, down-select one or more of the followings in RAN1#113: - Option 2: Such sub-channel(s) can be used for PSCCH/PSSCH transmission - Note: PRBs within intra-cell guard band are not used for PSCCH transmission as per previous agreement - Option 3: Such sub-channel(s) cannot be used for PSCCH transmission, and can be used for PSSCH transmission - FFS details, e.g., conditions to apply the above Option(s) - FFS impacts on definition of candidate resource, and resource selection

Based on the contiguous RB subchannels agreement in RAN1#112, some subchannels may overlap with intra-cell guard bands (subchannel #6 and #11 in Figure 11). It was also agreed in RAN1#110-bis that the intra-cell guard band RBs cannot be used for PSCCH transmission. If one wants to allow PSCCH transmission in subchannel #6, PSCCH REs need to rate match around the guard band PRBs which leads to the undesirable irregular PSCCH rate matching pattern and complicates Rx UE processing. Instead, one may consider skipping PSCCH in the subchannel overlapping with intra-cell guard band, i.e., those subchannels overlapping with guard band cannot be the leading or the 1st subchannel in the resource allocation.

Proposal 22: For contiguous RB waveforms, a subchannel overlapping with intra-cell guard band cannot be used as a starting/leading subchannel of a transmission which contains PSSCH, i.e., it can only contain PSSCH, (supporting option 3 in RAN1#112-bis agreement)

Figure 11: Handling for subchannel overlapping with guard band RBs

#### PSSCH resource mapping

##### Interlaced waveform

From a receiver point of view, different transmitters could apply different RB-set config and the receiver needs to identify the transmitter’s ID in SCI-2 to adjust the RB-set assumption accordingly. In case the RB-set is configurable per link, the SCI-2 should be rate matched to (a subset of) the minimum RB-set(s) (instead of the configured RB-set(s)) and the number of RB-sets used by PSSCH is indicated by SCI-1. These would allow any receiver to identify the transmission SCI-2, irrespective of the mismatches in RB-set configuration between nodes. If two or more (contiguous) RB-sets are allocated for SCI-2, SCI-2 may be rate matched to the interlaced RBs in the guard band as shown in Figure 12. If the NW only allows a global (common) RB-set config per 20MHz subband as in option 1 in Sec. 2.1.1, then the SCI-2 is rate matched to the common RB-set(s).

For the data PSSCH REs, data could be rate matched to the configured RB-set of the link for better efficiency if the link configures the RB-set via initial RRC message exchanges. Otherwise, data PSSCH is rate matched to RB-set(s) belonging to the common RB-set(s) or minimum RB-set(s), as indicated by SCI-1. If two or more contiguous RB-sets are allocated, data PSSCH may rate matched to the interlaced RBs in the guard band as shown in Figure 12.

Proposal 23: For interlaced waveforms, SCI-2 is rate matched to RB-sets belonging to the common or minimum RB-set(s) and the RB-set indices are indicated by SCI-1

Proposal 24: For interlaced waveforms, PSSCH could be rate matched to the per-link configured RB-set(s) (if supported) after the RRC config. If not supported/configured, PSSCH is rate matched to the common RB-set(s) or minimum RB-set(s) and the RB-set indices are given by SCI-1

Figure 12: PSSCH rate matching

##### Contiguous RB waveform

In RAN1 #112, we made the following agreements regarding subchannels and RB-sets

Agreement (RAN1#112) For contiguous RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and PRBs, down-select one of the followings during RAN1#112: - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size - FFS: how to deal with the remaining PRBs, e.g. for meeting OCB requirements

In the contiguous RB-based subchannel agreement, it is agreed to further study how to handle subchannel PRBs that fall outside of the RB-set (i.e. in the intra-cell guard band). For the example shown in Figure 11 (SCS=30KHz), subchannel #11 has some PRBs residing in the RB-set and some residual PRBs falling outside of the RB-set. To improve the frequency resource utilization, the last subchannel (e.g. subchannel #11) of the RB-set which overlaps with the guard band can still be utilized via PSSCH puncturing when Tx UE schedules a number of contiguous subchannels which is greater than a threshold. Only when both RB-sets (e.g. RB-set #1 and #2) are allocated ( i.e., subchannel #11 is not the last subchannel of the resource allocation), the entire subchannel #11 can be used for PSSCH transmission.

Proposal 25: To improve the resource utilization when the number of allocated subchannels in an RB-set is greater than a threshold, the last subchannel in the RB-set overlapping with intra-cell guard band can be allocated while the PSSCH REs outside of the RB-set are punctured

#### PSSCH resource allocation

In RAN1 #110-bis, #112 and #112-bis, we made the following agreements regarding the PSCCH/PSSCH

Agreement (#110-bis) Regarding frequency domain resource indication for interlace RB-based PSSCH transmission: - Down-select one of the followings - Option 1: Support explicitly indicating the used sub-channel index(es) and RB set index(s) - Option 2: Support explicitly indicating at least the used sub-channel index(s) - At least RB set index(es) is not explicitly indicated - FFS details Agreement (#112) For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and interlace, 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool Agreement (#112-bis) Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings: - Option A: Support that for one PSSCH transmission, the used interlace index(s) in different used RB sets are always the same - Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s) - Frequency domain resource of PSSCH transmission is determined by an intersection of the resource blocks of the indicated sub-channel(s) and the union of the indicated set of RB sets and intra-cell guard bands between the indicated RB sets, if any - For a TB, the initial transmission and reservation of the resource(s) for retransmission(s) use the same number of sub-channel(s) and same number of RB set(s) - FFS: whether additionally support different number of RB set(s) in such case while keeping total number of sub-channels unchanged between initial transmission and retransmission(s) for a TB - Use X bits for indicating sub-channel index(s), and use Y bits for indicating contiguous RB set index(s) - R16 NR SL FRIV is reused as baseline - FFS details, e.g., signaling design, bit size, whether to consider bitmap design, whether/how the used interlace(s) can be non-contiguous, etc. - FFS others E.g., considering one PSSCH transmission may occupy one or multiple RB sets, whether or not to re-define single-slot candidate resource, and update resource selection and/or signaling from MAC to PHY, etc.

In RAN1#112-bis, we have agreed to have two-part FRIV (X, Y) for resource allocation and reuse R16 NR SL FRIV as the baseline. In Rel.16 SL, the FRIV can signal up to 2 future reservations with the same number of subchannels and contiguous subchannel allocation. One can reuse the same signalling framework for the interlace subchannels with an RB-set and the RB-sets independently. FRIV-X signals up to two future reservations among the (interlaced) subchannels within an RB-set. Similar to Rel’16 FRIV, assuming that only contiguous subchannel allocation within Msubchannel available interlace subchannels is allowed within an RB-set, the FRIV-X contains ⌈log2 Msubchannel( Msubchannel+1)2⌉ bits for resource pools with a maximum of 1 future reservation or ⌈log2 Msubchannel( Msubchannel+1)(2 Msubchannel+1)6⌉ for resource pools with a maximum of 2 future reservations. Similarly, assuming that only contiguous RB-set allocation within MRB-set RB-sets is allowed, the FRIV-Y contains ⌈log2MRB-set(MRB-set+1)2⌉ for 1 future reservation or ⌈log2MRB-set(MRB-set+1)(2MRB-set+1)6⌉ for 2 future reservations.

It is worthy to point out that the Rel.16 SL FRIV is in the form of relative allocation, using the current SCI-1 reception subchannel as a reference (starting subchannel), and indicates how many subchannels are allocated, while (X, Y) resource allocation in NR-U indicates absolute interlace/RB-set index. The relative allocation could be useful when two SL nodes have different assumptions on the absolute interlace or RB-set index, e.g., two nodes (UE0 and UE1 in Figure 13) are configured with resource pools with partially overlapping RB-sets. If one defines Y allocation relative to the SCI reception RB-set, two nodes could still communicate via overlapping RB-sets.

Figure 13: UEs with different RB-set configs

Proposal 26: Use two-part FRIV-(X,Y)  to handle the interlace and RB-set allocation and reservations independently in which X/Y is encoded in legacy fashion based on pre-determined total number of interlaced subchannels/RB-sets, and the FRIV is defined relative to the SCI-1 reception interlace subchannel and RB-set

### PSFCH

In RAN1 #112-bis, the following agreements are made for PSFCH waveforms

Agreement Regarding PSFCH transmission with 15 kHz and 30 kHz SCS: - RAN1 down-select one of followings in RAN1#113: - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s) - K3 is (pre-)configured, FFS value range - On the K3 dedicated PRB(s), multiple CS pairs can be used as in legacy NR SL PSFCH transmission - When a PRB of common interlace and a dedicated PRB locate within the same 1 MHz bandwidth, UE only transmits on the dedicated PRB - FFS: whether any impact on meeting OCB requirement - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace - Alt 3-2b: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set - K2=2 - K4 is (pre-)configured, FFS value range - FFS: how to meet PSD limitation - FFS: whether to introduce any restrictions on the locations of K4 dedicated PRB(s) and/or K2 common PRBs, e.g., whether/how they are on the same interlace - R16 NR SL PSSCH-PSFCH mapping is reused as baseline, FFS details - Note: companies are encouraged to give more details and analyze the specification impact - E.g., whether PSSCH transmissions on non-overlapped resources are mapped to non-orthogonal PSFCH resources, i.e., whether PSFCH collision may happen and whether/how to address it, etc. - E.g., whether introducing more than 6 CS pairs is needed - E.g., for group cast option 2, what’s the maximum group size that can be supported - E.g., how to support “more than 1 PSFCH occasion(s) per PSCCH/PSSCH” - FFS: regardless of which Alt above is selected, whether or not to support PRB-level cyclic shift hopping as in NR-U to reduce PAPR - FFS: whether IBE issue exists and whether/how to address it - E.g., whether to introduce guardband PRB/RE between common PRB and dedicated PRB

#### Interlaced waveforms and PSFCH formats

To address PSD limit and minimum OCB requirement, we have agreed to introduce interlaced waveform for PUCCH format 0 (PF0) based PSFCH as in Figure 14. One Ack/Nak bit is carried via one interlace and one cyclic shift pair. The interlaced PF0-based PSFCH is inefficient for eMBB use cases as it can only carry one-bit Ack/Nak information with one interlace (9/10 interlaced RBs) and one CS pair. Suppose the PSSCH receiver has multiple Ack/Nak’s to report to the same transmitter. It would be more efficient to arrange Ack/Nak’s in a HARQ codebook, e.g., type-1/2/3, and transmit with some multiple-bit PSFCH format with one PSFCH transmission instead of multiple PF0 PSFCH transmissions. Also note that the number of simultaneous PF0 PSFCH could depend on UE capability so that the number of Ack/Nak’s that can be delivered via PF0 PSFCH per time instance could be limited. This will have a performance impact in high data rate use cases with large number of TBs transmitted per unit of time and a correspondingly large number of feedbacks required.

In NR-U, PUCCH format 2 (PF2) with interlaced waveform is introduced to carry multiple Ack/Nak bits. One may reuse the interlaced PUCCH format 2 (PF2) design for SL-U. The new PF2-based PSFCH could occupy one symbol and one interlace with one AGC symbol ahead just as the PF0 based PSFCH (see Figure 14). To improve user multiplexing, frequency domain OCC-2/4 on PF2 PSFCH data REs and DMRS REs could be applied, just as in NR-U PF2.

Proposal 27: For multi-bit A/N’s carrying PSFCH, reuse 1-symbol interlaced NR-U PF2 with additional front AGC symbol

Figure 14: PSFCH interlaced waveform and PF2 based PSFCH

If the network is configured to support groupcast option 2 (GC2) with a large number of groupcast Rx UEs, there could be a concern on the UE multiplexing capacity of the interlaced PF0 PSFCH since one PSFCH resource is mapped to at least 9/10 interlaced RBs. The UE multiplexing capacity for PF0 PSFCH is, therefore, about 10 times less than using the legacy 1-RB PSFCH waveform. For the groupcast, the PSFCH capacity could be an issue as multiple receivers need to be hashed to different CS pairs/interlaces within the PSFCH resource pool.

In RAN1#112-bis, we agree to study three main alternatives for increasing the UE multiplexing capacity, i.e., Alt 1-1b, Alt 2-3a and Alt 3-2b.

Figure 15: Unified PSFCH Waveform

In alt 1-1b, one may consider configuring more than one A/N carrying IRBs (in which case a single A/N is repeated over multiple IRBs) to solve the low transmit power issue due to PSD limit. How to configure the number of A/N carrying IRBs in the network could be based on the trade-off between multiplexing capacity and PSFCH coverage and the use cases. In the use case where groupcast option 2 is disabled or there is no PSFCH capacity issue when groupcast option 2 is enabled, e.g. limited number of groupcast receivers, interlaced PSFCH waveform (Alt 2-3a) as in NR-U would be good enough and does not have irregular waveform and PSD problem.

In our view, a unified waveform based on Alt 1-1b and Alt 2-3a can be designed in which K3 configurable A/N carrying RBs in the one of the non-common interlaces are transmitted along with a common interlace. In the common interlace, N-K3 dummy IRBs are transmitted where N is the number of IRBs within one interlace. As shown in Figure 15, the K3 A/N carrying PRBs are placed in contiguous IRBs within a single non-common interlace (in K3 IRB clusters) and N-K3 dummy IRBs are placed in N-K3 IRB clusters different from the K3 IRB clusters, in which an IRB cluster consists of 5/10 contiguous RBs (one RB from each interlaces) for SCS=30/15KHz. In this way, A/N carrying IRBs could avoid sharing 10dBm/Hz PSD with dummy IRBs in the common interlace. If there is still some edge dummy IRBs that are too close to the A/N carrying IRBs, they could be dropped from dummy signal transmission so that the full 10dBm/Hz PSD is allocated to the A/N carrying IRBs. It is worthy to note that, if K3=N (full interlace), the PSFCH waveform becomes Alt 2-3a. Additionally, to reduce PAPR, common cyclic shift ramping can be introduced to both A/N carrying and dummy IRBs. As in NR-U, 5 cyclic shift offsets could be introduced across two IRBs in the same interlace as shown in  Figure 15.

Observation 5: In the use case with no groupcast option 2 or limited number of GC2 receivers, interlaced PSFCH waveform (Alt 2-3a) as in NR-U would be good enough and does not have irregular waveform and PSD problem.

Proposal 28: (Unified framework between Alt 1-1a and Alt 2-3a/2-4a) Configurable number (K3) of contiguous A/N carrying PRBs (K3) on a non-common interlace and N-K3 dummy PSFCH IRBs on the common interlace with reserved cyclic shift resources to fulfill the OCB requirement, where N is the number of IRBs in one interlace. The waveform becomes a full interlaced PSFCH waveform as in Alt 2-3a/2-4a when K3=N

Proposal 29: Place the N-K3 dummy IRBs in the different IRB clusters of the common interlace from the K3 A/N carrying IRBs, and drop the edge dummy IRBs which are within X-RBs of the A/N carrying IRBs to boost the A/N transmit power

Proposal 30: Apply common cyclic shift ramping to both A/N carrying and dummy IRBs of PSFCH to reduce the PAPR

#### PF0 PSFCH Resource Mapping and Hashing

For eMBB-like traffic, there is a good chance that the PF0 PSFCH occasion could share the same COT with the associated PSSCH. We may want to define a PF0 PSFCH (including capacity enhanced PF0 PSFCH) resource mapping rule such that the COT initiator could resume its own COT after sharing to PSFCH transmission. Treating the case of PSSCH contained within one RB-set first, it would be reasonable to map PSSCH to a PSFCH resource in the same RB-set. Within the PSSCH RB-set, one can define a one-to-one mapping from PSSCH leading interlace (or subchannel) index and slot index to the PSFCH interlace index/partial interlace index and a set of cyclic shift` (CS) pairs.

In the example shown in Figure 16a) for alt 2-3a, i.e., K3=10, PSSCH interlace #X is mapped to PSFCH interlace #X and PSSCH in slot #i is mapped to cyclic shift pair set #i.  In another example shown in Figure 16b) for alt 1-1b (K3=2 and interlace #0 is the common interlace), K3-IRB partial interlaces in the interlaces other than interlace #0 are evenly distributed across slots and subchannels, and each slot/subchannel is mapped to one set of K3-IRB partial interlace(s), which is consist of 2 partial interlaces, in time first manner.

Within the mapped cyclic shift pair set (PSFCH resource pool) which corresponds to one subchannel/slot, hashing is performed based on

PID+MIDmod NCS-setPSFCH

where PID is transmitter’s L1 ID,  MID is receiver’s groupcast ID (MID=0 for unicast and groupcast option 1 transmissions) and NCS-setPSFCH is the number of CS pairs within the mapped PSFCH resource pool (set of CS pairs).

For wideband PSCCH/PSSCH transmission across multiple RB-sets, if there is a PFSCH instance in the middle of the COT and PSFCH transmission is contained in only one of the RB-sets, the COT transmission is terminated in the other RB-sets in which PSFCH is not transmitted. In the example shown in Figure 17, the transmitter starts a COT in two RB-sets. However, the PSFCH transmission in the duration of the COT only occupies RB-set#0 via COT sharing. So, the transmitter cannot resume the wideband COT transmission in RB-set #1 after the PSFCH instance and an additional type-1 LBT is required for that RB-set. A wideband PSFCH waveform is therefore needed to maintain a wideband COT transmission. One may consider mapping one PSFCH resource to a wideband PSFCH waveform across all RB-sets. The wideband PSFCH waveform could consist of multiple copies of 10/11-RB interlaced PSFCH waveform occupying the same interlace in all RB-sets that the associated PSSCH occupies as shown in Figure 18.  Furthermore, to reduce PAPR, cyclic shift ramping could be applied to the wideband interlaced PSFCH waveforms (spanning multiple RB-sets) across different RB-sets.

Proposal 31: To determine PF0 based PSFCH resource within the RB-set(s) occupied by PSSCH, perform one to one mapping from PSSCH leading interlace and slot index to PSFCH interlace (for alt 2-3a) or PSFCH partial interlace set (for alt 1-1b and each partial interlace contains K3 IRBs as in the discussion above) and CS pair set therein, and hashing within the associated CS pair set

Proposal 32: To support wideband PSSCH transmission, map one PF0-based PSFCH resource to an PSFCH interlace spanning all the RB-sets in the data resource pool and only transmit the truncated PF0 PSFCH in the PSSCH transmitting RB-set(s), i.e., PSFCH occupies the same RB-set(s) as the associated PSSCH

Figure 16: PSFCH resource mapping for a) K3=10, alt 2-3a (PSFCH occupies one interlace) b) K3=2, alt 1-1b (PSFCH occupies 2-IRB partial interlace)

Figure 17: Wideband COT transmission from COT initiator cannot be resumed after PSFCH gap

Figure 18: Wideband PSFCH transmission

Figure 19: RB-set Padding Signal for PSFCH Transmission

Additionally, when a UE performs PSFCH transmission as part of its ongoing 2nd COT transmission, this COT may occupy a different number of RB-sets than the PSSCH transmission the PSFCH transmission corresponds to (as shown in Figure 19).

The 2nd COT needs to cover at least all the RB-sets that wideband PSFCH occupies (but could be wider) because the PSFCH Rx UE assumes the PSFCH occupies the same RB-sets as in the associated PSSCH. If the 2nd COT includes an additional RB-set, some RB-set padding signals can be transmitted along with the wideband PSFCH to hold the wideband COT (if necessary). Similar to the COT padding signal in the unused PSFCH symbols at the beginning of a COT as described in our companion paper [5], the PSFCH Tx UE can send PSFCH-like RB-set padding signals on its own or reserved PSFCH resources in the additional RB-set(s) along with the wideband PSFCH as shown in Figure 19. Furthermore, the PSFCH-like RB-set padding signal may occupy the same interlace as the wideband PSFCH to reduce the PAPR.

Proposal 33: A PSFCH transmission in the context of a wideband COT occupying more RB-sets than the associated PSSCH transmission, is allowed to transmit PSFCH-like RB-set padding signal in the additional RB-set(s) to hold the wideband COT. The resources used by the padding signal could be PSFCH resources or system-wide configured (i.e., common) resources

### S-SSB

In RAN1 #112-bis, we made the following agreements regarding the S-SSB

Agreement For S-SSB transmission within 1 RB set, for 15 kHz and 30 kHz SCS, Alt6 is supported: - Alt 6: Support both Option 3-1(revised) and Option B, and enable one of them by (pre-)configuration Note: the Options are as below - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB - Option 3-1(revised): Transmit legacy S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement - FFS the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), whether/how to reduce PAPR. - FFS gap of 0 - Option A: Legacy S-SSB - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS for OCB exemption. - Option B: Legacy S-SSB - RAN1 does not pursue further study on how to meet the minimum 2 MHz requirements under 15 kHz SCS for OCB exemption. Note: Option A and B are applicable in region with no OCB requirement, or with OCB exemption.

#### S-SSB Waveform

From Rx UE point of view, mandating UE to process with wideband SPSS/SSSS in option 3-1 may drastically increase the S-SSB searcher complexity and buffer size due to increased S-SSB bandwidth and correspondingly higher sampling rate (compared to narrowband S-SSB). In Table 2, we compare the complexity (in terms of multiplications/additions) for initial sync and buffer size of the S-SSB searcher between the wideband SPSS/S-SSS (option 3-1) and the narrowband SPSS/S-SSS, in which M is the number of time samples in search window and N is the bandwidth of the S-PSS. For the wideband S-SSB searcher, the complexity increases by a factor of 16 and the searcher buffer size increases by a factor of 4 for SCS=30KHz due to 4 times the sampling rate for wideband S-PSS/S-SSS. The wideband S-SSB bandwidth is assumed to be 44/88RB for SCS=30/15KHz to fulfill the 80% OCB while the narrowband S-SSB occupies 11RB. The searcher complexity (in terms of multiplications/additions) is dominated by S-PSS detection in which it includes multiple frequency and timing hypotheses over the S-SSB period. For S-PSS detection, time domain correlation method is assumed, and we are showing the complexity for one frequency hypothesis.

Observation 6: Mandating Rx UE to process the wideband S-SSB waveform, e.g., options 3-1, results in a multifold searcher complexity and buffer size increase compared to legacy narrowband SSSS/SPSS

| Complexity increase factor from narrowband to wideband S-SSB | SCS=30 KHz | SCS=15 KHz |
| --- | --- | --- |
| S-PSS correlation: O(M⋅N) | 16 | 64 |
| Searcher buffer size: O(M) | 4 | 8 |

Table 2: Searcher complexity increase factor from narrowband to wideband S-SSB

Figure 20: a) Legacy S-SSB only (option B). b) Legacy S-SSB+frequency repetitions (option 3-1)

For S-SSB Rx UE, reusing the legacy S-SSB searcher implementation is preferrable since designing new S-SSB searcher to process the new interlaced S-SSB waveform or the legacy S-SSB + S-SSB frequency repetitions would have huge impact on Rx UE complexity and may require extensive performance studies. Hence, it would be the best that the S-SSB Rx UE is only required by spec to monitor the legacy 11 RB S-SSB, while it could be up to UE implementation on exploiting the S-SSB repetitions (if present). Some companies may have concern on the S-SSB coverage due to PSD limitation. In our view, SL-U is not targeting for more coverage compared with NR-U. As NR-U uses legacy SSB waveform with no further enhancement, we don’t think SL-U S-SSB Rx UE need to combine S-SSB in receiver processing to improve S-SSB coverage.

Proposal 34: The Rx UE is only required to monitor the legacy 11RB S-SSB and whether to monitor S-SSB frequency repetitions (if present) is up to UE implementation

Regarding the number of the repetitions (N) and gap between S-SSB repetitions in the FFS, the repetition number N=2 is good enough to fulfil the OCB regulation since the Rx UE is not expected to combine S-SSB repetitions to harvest power gain, and the choice of gap(s) between S-SSB repetitions needs to ensure minimum OCB is satisfied, e.g., gap greater than 22 /66RBs is expected in SCS=30/15KHz.

Proposal 35: For S-SSB repetition waveform (option 3-1), two repetitions (N=2) and a gap >22/66 RBs for SCS=30/15 KHz suffices to fulfill the OCB requirement

#### PAPR Reduction

For S-SSB waveform option 3-1, the RAN1 agrees to further the PAPR impact with S-SSB frequency repetitions. To understand the impact of S-SSB frequency repetitions on PAPR, Figure 21 shows the CDF of PAPRs of all the SPSS/SSSS sequences for legacy SPSS/SSSS, 2 frequency copies of SPSS/SSSS’s on the edge of an RB-set with 26 RB gap and 4 frequency copies of SPSS/SSSS with 1RB gap. We also try to approximate the PAPRs of legacy PSBCH, 2 frequency repetitions of PSBCH with 26 RB gap and 4 frequency repetitions of PSBCH with 1 RB gap by treating PSBCH as an 11 RB OFDM waveform with random QPSK symbols and show the CDF of PAPRs in Figure 22. The average PAPR values of SPSS, SSSS and PSBCH are shown in Table 3. Comparing with legacy S-SSB, 2 repeated S-SSBs increase the average PAPR by 2.7~3dB, and 4 repeated S-SSBs increase the average PAPR by 5.2~5.6dB. With one S-SSB, the PAPRs seem to be dominated by SSSS and PSBCH, and one may want to consider PAPR reduction techniques for SSSS and PSBCH in particular.

Figure 21: SPSS/SSSS PAPR analysis for option 3-1 with and without enhancements

Figure 22: PSBCH PAPR analysis for option 3-1

| Ave PAPR (dB) | SPSS | SSSS | PSBCH |
| --- | --- | --- | --- |
| 1x | 4.83 | 7.64 | 7.97 |
| 2x (with 26 RB gap) | 7.59 | 10.51 | 10.91 |
| 4x (with 1 RB gap) | 10.36 | 12.92 | 13.37 |

Table 3: Average PAPR values for S-SSB frequency domain repetition

| Average PAPR reduction (dB) | SSSS | PSBCH |
| --- | --- | --- |
| 2x (with 26 RB gap) | 10.51 8.5 | 10.918.92 |
| 4x (with 1 RB gap) | 12.92 9.3 | 13.378.92 |

Table 4: Average PAPR reduction with SSSS and PSBCH PAPR reduction techniques

Figure 23: PAPR reduction on SSSS and PSBCH

In  Figure 22, we also try to estimate the PAPR reduction when modulation symbols are randomized across 2/4 frequency repetitions of PSBCH by assuming that QPSK symbols across 2/4 S-SSB repetitions are independent. From Table 4, one can see that the average PAPR of 2/4 frequency copies of PSBCH may potentially be reduced from 10.91/13.37dB to 8.92dB. The PAPR reduction can be achieved by applying different scrambling sequences on DMRS and SCH REs of PSBCH for different frequency repetitions while keeping the legacy scrambling sequence for PSBCH transmitted over the legacy S-SSB location as illustrated in Figure 23. One may consider using the legacy scrambling sequence generator across PSBCH repetitions while choosing different initial seeds for different frequency copies. The legacy scrambling initial seed is determined by the sidelink UE ID, i.e., Cinit=NIDSL. If different NIDSL’s for different non legacy S-SSB repetitions are chosen, we can get different scrambling sequences for different frequency copies of PSBCH.

Proposal 36: To reduce the PAPR of frequency repeated PSBCH, study applying different scrambling sequences for different frequency copies of PSBCH

The S-SSS waveform is a product of two pseudo random sequences x0(n) and x1(n), i.e.,

dS-SSSn=1-2x0n+m0 mod 1271-2x1n+m1 mod 127

where m0=15NID,1SL112+5NID,2SL and m1=NID,1SL mod 112. To reduce the PAPR of frequency repeated SSSS, one may consider applying different sequence shifts in x0n or x1(n) for different frequency copies of SSSS. For example, one may consider choosing a different NIDSL for the SSSS in the S-SSB repetitions other than legacy S-SSB, while the SSSS in legacy S-SSB still carries the actual NIDSL. In the simulation shown in Figure 21, we choose NIDSLi=NIDSL+i for the ith frequency repetition of S-SSB and the average PAPR values for 2/4 SSS frequency are reduced from 10.51/12.92dB to 8.5/9.3dB ( 2/3.6dB PAPR reduction). Choosing different NIDSLi for different S-SSB frequency repetitions while the legacy S-SSB uses the actual sidelink UE ID seems to be a simple solution to reduce the PAPR of frequency repeated SSSS and PSBCH. As the initial seed of PSBCH scrambling sequence aligns with the detected NIDSLi from the SPSS/SSSS in the ith S-SSB frequency repetition, a narrowband S-SSB searcher could follows the same S-SSB detection procedures while decoding different S-SSB frequency copies. Note that NR-U has only one sync raster per 20MHz channel in unlicensed band. Here, as long as the frequency repeated S-SSBs are not on the sync raster (the legacy S-SSB is on one of the sync rasters), the Rx UE should not have problem detecting the correct NIDSL.

Proposal 37: To reduce the PAPR of frequency repeated SSSS, study applying different sequence shift(s) to pseudorandom sequence(s) for different frequency copies of SSSS

Observation 7: Choosing different NIDSLi for different S-SSB frequency repetitions except for the legacy S-SSB seems to be a simple solution to reduce the PAPR of frequency repeated SSSS and PSBCH

## Physical channel procedures

### Multiple PSFCH opportunities for PSFCH

In RAN1 #112-bis, we made the following agreements regarding how to address LBT uncertainty for PSFCH transmission.

Agreement Regarding more than 1 PSFCH occasion per PSCCH/PSSCH transmission, support the followings: - One PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration - FFS value range of N - FFS detailed design of such N associated candidate PSFCH occasion(s) - E.g., they are in different slots of the same RB set, or in different RB sets of the same slot, or combination thereof, etc. - E.g., whether PSSCH transmission and its related PSFCH occasion(s) are in the same RB set(s) - FFS: whether to support that COT initiating UE can dynamically indicate which subset of the (pre-)configured PSFCH occasions within its COT are available for PSFCH transmissions. - FFS: whether other associated candidate PSFCH occasion(s) within its COT are used for PSSCH transmissions, and applicable scenarios. - FFS: whether AGC issue and PSFCH/PSSCH collision issue exist, and whether/how to address them - FFS other details - E.g., how to meet the HARQ RTT restriction - E.g., UE behavior on transmitting PSFCH - E.g., how to avoid the risk of losing the COT if the COT is interrupted by periodic PSFCH occasions

Legacy PSFCH HARQ timeline has one opportunity (occasion) for PSFCH transmission. In a unicast link, if receiver PSFCH LBT fails, the transmitter treats the missing PSFCH signal as a Nack and introduces PSSCH retransmission overhead, even if the receiver was actually able to decode in the first place. For groupcast option 1, if LBT for PSFCH NACK fails, the transmitter will treat it as an ACK and do not retransmit.

RAN1#112-bis agreement supports to configure multiple PSFCH opportunities (occasions) associated with a PSSCH so as to provide more opportunities to pass LBT. An example is shown in in Figure 24. The PSSCH receiver may continue to try LBT on different PSFCH opportunities until it clears the LBT. If the interference at different slots is uncorrelated, the PSFCH will have more chances to clear the LBT.

Multiple PSFCH opportunities can be pre-configured (each with different HARQ timeline) and could be mapped to different PSFCH resource sets, i.e., resources used for first PSFCH opportunities are restricted to a pre-configured resource set of the resource pool, second PSFCH opportunities are mapped to a different pre-configured resource set in the resource pool, etc. These different PSFCH resource sets could be TDM+FDM’ed (see Figure 24) or TDM’ed (see Figure 25).

Another option with less spec impact is to have different PSFCH opportunities share the same set of periodic PSFCH instances. Different PSFCH opportunities associated with the same PSSCH could be mapped to different PSFCH instances of that set, based on different HARQ timeline. One example is to map X PSFCH opportunities to X PSFCH instances sequentially in time as shown in Figure 26. Within one PSFCH instance, we need to multiplex different PSFCH opportunities coming from different PSSCHs. Instead of FDM’ing and/or TDM’ing PSFCH resources for different PSFCH opportunities from the different PSSCH’s from different slots, PSFCH opportunities could be hashed to different PSFCH resources in the mapped PSFCH resource pool as follows

PID+MID+mmod NCS-setPSFCH,

where m is the opportunity index taking the values of {0,1,…, Mopportunities-1}, where Mopportunities is the number of PSFCH opportunities , PID is transmitter’s L1 ID,  MID is receiver’s groupcast ID (MID=0 for unicast and groupcast option 1 transmissions) and NCS-setPSFCH is the number of CS pairs within the mapped PSFCH resource pool (set of CS pairs). Hashing collision, e.g., among different PSFCH opportunities corresponding to PSSCHs transmitted from different slots/subchannels, is possible and becomes more frequent with the increased number of PSFCH opportunities. However, this option avoids the waste of PSFCH resources as in TDM’ing or TDM+FDM’ing when PSFCH transmitter can clear the LBT at the first few PSFCH opportunities and/or the sidelink congestion is sufficiently small.

Proposal 38: To support multiple PSFCH opportunities associated with a PSSCH with different HARQ timelines, configure multiple (FDM/TDM) PSFCH resource sets (one set per HARQ timeline) or use differently hashed resources over a common/shared PSFCH resource set for each HARQ timeline

Figure 24: Multiple PSFCH opportunities illustration for FDM+TDM (interlace structure not shown)

Figure 25: Multiple PSFCH opportunities illustration for TDM (interlace structure not shown)

Figure 26: Two PSFCH opportunities illustrations with different hashing in PSFCH resource pool

#### Close the gap in the PSFCH symbol

In the RAN1#112-bis agreement, we have agreed to further study how to avoid the risk of losing the COT if the COT is interrupted by periodic PSFCH occasions. For the eMBB case, the Tx UE schedules TBs in a burst and may not expect Ack/Nak at the beginning of the bursts. However, the periodic PSFCH occasions are common across all links in the network, there could be some un-used PSFCH instances at the beginning of the data burst and could cause COT termination. These PSFCH occasions could potentially be used by other links, but the COT initiating transmitter may or may not transmit/receive PSFCH to/from other SL nodes. If the COT initiator cannot guarantee or assume there is a transmission in the PSFCH occasion with ≤16us gap, either from the COT initiating transmitter or other SL nodes, the COT will be terminated and additional LBT overhead would be undesirable. In Figure 27 (upper figure), the COT-initiating UE (UE#0) is transmitting 4 TBs to its receiver. TB #0 and #1 are associated with the 2nd PSFCH instance and the 1st PSFCH instance is unused by the UE#0 and its Rx UE. Then, the 1st COT of UE#0 is terminated at the 1st PSFCH instance and additional LBT or another COT may be required for UE #0 to finish the data burst transmission. To address the COT termination problem, the following two options are possible:

Option 1: The Tx UE (COT initiator) triggers it Rx UE to send padding PSFCH signal over the unused PSFCH instances for the link

Option 2: The Tx UE (COT initiator) transmits padding PSFCH signal over the unused PSFCH instances

The padding PSFCH signal could be PSFCH waveform with reserved PSFCH resource or unused PSFCH resource for the link.

Figure 27: Unused PSFCH instances causes termination of COT (upper figure). PSFCH-like signals can be used to retain continue the COT (lower figure)

Observation 8:  If there exists an unused PSFCH instance in the middle of data burst, additional type-1 LBT may be required by SL transmitter to continue the remaining transmission.

Observation 9: If the COT-initiating transmitter could transmit or its receiver could be scheduled to transmit some signals at the unused PSFCH instances, we can reduce the LBT overhead.

Proposal 39: Support PSFCH-like padding signal to fill the PSFCH symbol when the COT initiating UE is neither sending nor expecting to receive a PSFCH

- FFS: the case where the COT initiator UE transmits the PSFCH-like padding signal

- FFS: the case where the COT initiating UE can trigger a receiving UE to send a PSFCH-like padding signal

#### Impact of multiple PSFCH candidates on mode 1 HARQ reporting timeline and HARQ RTT timer

In Rel’16 SL mode 1 operation, the UE provides the HARQ-ACK information from sidelink in a PUCCH transmission within slot n+k  (subject to the overlapping conditions) with reference to slots for PUCCH transmissions and for a number of PSFCH reception occasions ending in slot n, where k is a number of slots indicated by a PSFCH-to-HARQ_feedback timing indicator field in DCI 3. With the introduction of multiple PSFCH candidates associated with each PSSCH, it is natural to set the reference slot for PUCCH transmission, i.e., slot n, to be the slot after the last PSFCH candidate since the Ack/Nak over PC-5 may come back at the last PSFCH.

Proposal 40: For mode 1 operation, if multiple PSFCH candidates are configured, the reference slot n for PUCCH transmission to report HARQ (or for PSFCH-to-HARQ_feedback indicated by DCI 3) is the slot after the last PSFCH candidate

For Rel’17 sidelink DRX, the sl-drx-HARQ-RTT-Timer is started in the 1st slot after the corresponding PSFCH instance which carries A/N or A/N is dropped due to priority rule as shown in Figure 28. With the introduction of multiple PSFCH candidates, the sl-drx-HARQ-RTT-Timer can be started after the 1st PSFCH transmission which clears the LBT (Figure 29) or the last PSFCH candidates to maximize the power saving.

Proposal 41: If multiple PSFCH candidates are configured, for DRX operation, the sl-drx-HARQ-RTT-Timer can be started after the 1st PSFCH transmission which clears the LBT or the last PSFCH candidates for the maximal power saving

Figure 28: sl-drx-HARQ-RTT-Timer starting point for Rel’17 SL

Figure 29: sl-drx-HARQ-RTT-Timer starting point with multiple PSFCH candidates

### HARQ codebook

For eMBB like traffic, transmitting multiple Ack/Nak’s from one PSSCH receiver to one PSSCH transmitter with interlaced PF0 PSFCH could be inefficient or not even possible as the number of simultaneous PF0-based PSFCH transmissions is based on UE capability. HARQ codebook is needed so the Ack/Nak’s for multiple TBs can be carried via PF2 PSFCH in one PSFCH transmission. Moreover, HARQ codebook is beneficial for Ack/Nak re-transmission (potentially due to LBT failure). In NR-U, type-3 HARQ codebook is introduced which allows gNB to pull the Ack/Nak’s of the configured HARQ IDs when gNB believes that some Ack/Nak’s are not transmitted in the regular PUCCH occasions due to LBT.

As in Rel’16 SL, all the SL nodes in the network may share common PSFCH occasions with configurable periodicity of 0/1/2/4 slots. The HARQ timeline may follow the legacy timeline with MinTimeGapPSFCH constraint. Instead of sending multiple Ack/Nak’s via different PF0 PSFCH resources in the same PSFCH instance, the Ack/Nak’s (with the same fixed HARQ timeline) targeting for the same UE in the same PSFCH instance are collected to form a type-1 HARQ codebook and transmitted via PF2 PSFCH with one PSFCH transmission at the legacy PSFCH instances as shown in Figure 30. Also, type-3 like HARQ could also be introduced to enable triggered Ack/Nak retransmission.

Figure 30: Type-1 HARQ codebook with fixed timeline

### Multiple S-SSB candidate locations

In RAN1 #111, we made the following agreements regarding how to address LBT uncertainty for S-SSB transmission.

Agreement: (#111) Regarding the number and location(s) of additional candidate S-SSB occasions, RAN1 further study the followings: - Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS - Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured - FFS details, e.g., value of K, details on gap length, etc. - Option 3: The number and location(s) of additional candidate S-SSB occasions are separately (pre-)configured - Option 4: Introduce M contiguous candidate S-SSB occasions in one S-SSB period - Option 5: the number of candidate S-SSB occasions is (pre-)configured, and locations are determined based on the (pre-)configured number Conclusion (#112-bis) Regarding additional candidate S-SSB occasions, in the same S-SSB period, UE can attempt to transmit on additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s). Agreement: (#112-bis) When the SL-BWP contains multiple RB sets, study the followings: - When UE attempts to transmit S-SSB in a S-SSB occasion (e.g., R16/17 S-SSB occasion, R18 additional candidate S-SSB occasion) - Alt 1: UE may transmit S-SSB repetition in more than one RB set - FFS details, e.g., location of such S-SSB repetition(s) (e.g., (pre-)configured and/or pre-defined), whether/how to address potential power reduction and/or potential fluctuation of PSBCH-RSRP - FFS: the relationship with UE’s COT - FFS: the scenario that UE may or may not transmit S-SSB repetition in more than one RB set - Note: whether UE can transmit S-SSBs over non-contiguous RB sets is subject to RAN4’s reply, details can be found in RAN1’s LS to RAN4 in R1-2304218 Working assumption (#112-bis) Additional candidate S-SSB occasions are excluded from resource pool

In Rel’16 SL, the number of S-SSB occasions (K) within one S-SSB period is as follows

  - FR1, SCS = 15 kHz: K=1

  - FR1, SCS = 30 kHz: K=1, 2

  - FR1, SCS = 60 kHz: K=1, 2, 4

Due to LBT uncertainty, the SL S-SSB transmitting node may need more occasions to transmit S-SSB. We may also want to limit the multiple occasions to be contained within a configured window in order to limit the S-SSB searcher’s complexity instead of letting the searcher to be active throughout the entire S-SSB period.  One can reuse NR-U DRS window design with multiple candidate occasions clustered in the DRS window. The receiver may only search for the S-SSB within the configured DRS window. In NR-U, the gNB is allowed to transmit up to 4/8 S-SSBs in 10/20 S-SSB candidate locations for SCS=15/30KHz.

With the PSD limit of 10dBm/MHz, the total transmit power of S-SSB is limited especially for the SCS=15KHz, we may want to allow multiple (L) S-SSB transmissions (S-SSB beam sweeping as in Uu or repetition) to improve the coverage. Hence, we may reuse the similar design from NR-U in which the S-SSB transmitter may transmit up to L S-SSB slots (as shown in Figure 31) among the M S-SSB candidate occasions within one DRS window.

Option 1 in RAN1#111 agreement which increases the number of legacy S-SSB occasions (sl-NumSSB-WithinPeriod) within one S-SSB period could be a waste of Tx power as a S-SSB Tx UE is supposed to transmit in every legacy S-SSB occasion (subject to LBT). Hence, we support option 2 in RAN1#111 agreement with the minimum/no gap to minimizing the search complexity and allow multiple (L) S-SSB transmission in the S-SSB candidates following each (legacy) nominal S-SSB instance(s)

Figure 31: S-SSB Transmission within S-SSB candidate locations (L=2 and M=4)

Observation 10: Option 1 in RAN1#111 agreement increases the number of S-SSB transmissions within one S-SSB period and may waste the Tx power

Proposal 42: Introduce multiple (M) S-SSB candidate slots starting from each nominal S-SSB instance(s) to allow multiple LBT tries for S-SSB transmission and allow multiple S-SSB transmission slots (L), and further study the number M and L

#### Padding the S-SSB gap for COT transmission

Based on the RAN1#112-bis working assumption, the additional S-SSB occasions are excluded from the resource pool, and additional S-SSB gaps are created in time domain which may cause unnecessary COT transmission interruption. One S-SSB slot excluded from data resource pool per 160ms implies 3.75% of the 6ms COT transmissions would be impacted. When multiple consecutive S-SSB candidate slots (X) per S-SSB instance are configured to counter the LBT uncertainty, the impact on performance will be even more prominent as shown in Figure 32 (detailed simulation assumptions and more simulation results are available in [7]). For X=4/10, 10%/20% of throughput loss is observed at 5Mbps offered load in low density deployment (6 pairs of SL-U UEs and 4 STAs per AP and the SL-U link association RSRP=-72dBm).

Figure 32: Performance degradation by excluding S-SSB candidate slots from RP (X consecutive S-SSB candidate slots per S-SSB instance and two S-SSB instances within 160ms)

To alleviate the COT interruption problem at least for the S-SSB Tx UE, we may want to allow S-SSB time repetitions all S-SSB occasions within the COT transmission to pad the COT as shown in Figure 33. Additionally, if the Tx UE initiates a wideband COT across multiple RB-sets, for all the S-SSB occasions within the COT transmission burst, we may want to allow UE to repeat the S-SSB in the non-anchor RB-set(s) for S-SSB transmission as shown in Figure 33 so that the wideband COT transmission can continue after those S-SSB gaps. There could be some worries about the potential fluctuation of PSBCH-RSRP if UE is allowed to dynamically repeating frequency padding S-SSB in multiple RB-set.  Considering that the 11RB S-SSB Tx power in the anchor RB-set is capped to 16/13dBm under PSD limit for SCS=30/15kHz, if we can limit the S-SSB repetition in non-anchor RB-set(s) subject to UE’s maximum Tx power or start with less Tx power for a single 11-RB SSB transmission, the PSBCH-RSRP fluctuation could be avoided.

Proposal 43: At least for the S-SSB Tx UE, if wideband COT transmission is initiated across multiple RB-set, for all the S-SSB occasions within COT transmission burst, allow UE to repeat the S-SSB in the non-anchor RB-sets for S-SSB transmission

Proposal 44: To avoid PSBCH-RSRP fluctuation with dynamic S-SSB repetition in multiple RB-sets, one can limit the number of the S-SSB repetition in different RB-set(s) subject to UE’s maximum Tx power or start with lower Tx power for legacy 11-RB S-SSB transmission in anchor RB-set

Figure 33: S-SSB gap padding in additional S-SSB slot(s) or/and non-anchor S-SSB RB-set for COT transmission

# Summary

In this section, we summarize the observations and proposals on different design aspects on SL-U

Observation 1: WiFi interference may start anywhere in the middle of an ongoing SL slot reception and introducing additional AGC symbols within a slot cannot solve the Rx UE AGC issue of getting strong interference starting from non-AGC symbols. Instead, SL-U Rx UE needs to choose a conservative AGC setting allowing more interference margin

Observation 2: Additional AGC symbol in the middle of full slot PSSCH is undesirable since AGC retuning in the middle of the slot would hurt phase continuity for PSSCH reception

Observation 3: For the resource pool configured with two starting symbols, if the reference number of symbol (Lref) for TBS calculation is chosen based on the 2nd staring symbol, e.g. Lref=7 , the highest throughput rate is greatly reduced

Observation 4: Repeating PSCCH in every RB-set of the corresponding PSSCH transmission as in option 2 may cause receiver confusion on FRIV reference point

Observation 5: In the use case with no groupcast option 2 or limited number of GC2 receivers, interlaced PSFCH waveform (Alt 2-3a) as in NR-U would be good enough and does not have irregular waveform and PSD problem.

Observation 6: Mandating Rx UE to process the wideband S-SSB waveform, e.g., options 3-1, results in a multifold searcher complexity and buffer size increase compared to legacy narrowband SSSS/SPSS

Observation 7: Choosing different NIDSLi for different S-SSB frequency repetitions except for the legacy S-SSB seems to be a simple solution to reduce the PAPR of frequency repeated SSSS and PSBCH

Observation 8:  If there exists an unused PSFCH instance in the middle of data burst, additional type-1 LBT may be required by SL transmitter to continue the remaining transmission.

Observation 9: If the COT-initiating transmitter could transmit or its receiver could be scheduled to transmit some signals at the unused PSFCH instances, we can reduce the LBT overhead.

Observation 10: Option 1 in RAN1#111 agreement increases the number of S-SSB transmissions within one S-SSB period and may waste the Tx power

Proposal 1: For interlace resource pool, if one or more interlaced subchannels are allocated in two (contiguous) RB-sets, the interlaced RBs (with the same interlace index(es) as the RBs for PSSCH transmission in these two RB sets) in the intra-cell guard band between the two RB-sets are included for resource allocation

Proposal 2: For potentially different sizes of interlaces, calculate TBS size based on the fixed

Proposal 3: The RB-set config for 20MHz subbands could be either common config for all SL nodes/links in the NW or configurable per link via RRC message exchange

Proposal 4: If RB-set config per link is configurable, one may need to define the minimum RB set for a 20MHz subband for the initial RRC message exchange in order to set up the RB-set config of a link

Proposal 5: For the PSSCH in the half/full slot starting from the 2nd/1st starting symbol, reuse the legacy DMRS pattern which is based on the number of PSCCH/PSSCH symbols

Proposal 6: For the 1st slot of the COT transmission, the transmitter chooses the earliest starting symbol after clearing the LBT and, for the remaining slots of a COT (at least for MCSt), the Tx UE may only choose the 1st starting symbol for PSCCH/PSSCH (supporting the RAN1#112-bis FFS)

Proposal 7: The half-slot access could be configured per link. If configured, the receiver monitors two PSCCH occasions in each (full) slot by default

Proposal 8: The (pre-)configured 1st and 2nd starting symbol positions are separated by X symbols (FFS: the value of X)

Proposal 9: For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, we support option 2, i.e., the Tx UE transmits only 1 AGC symbol no matter the PSCCH/PSSCH transmission starts from 1st or 2nd starting symbol

Proposal 10: For a slot with 2 candidate starting symbols for a PSCCH/PSSCH reception, the Rx UE monitors two AGC symbols in such slot by default, but may only adjust its AGC setting once based on the 1st starting symbol if a PSCCH or DMRS transmission starting from the 1st starting symbol is detected (option B/D)

Proposal 11: To protect the full-slot receiver, a transmitter may initiate transmission from the 2nd starting symbol, only if it has attempted to decode PSCCHs from the 1st starting symbol and has not identified/detected presence of SCI-1 with a priority higher than a (pre-) configured threshold and/or PSCCH DMRS with RSRP higher than a (pre-) configured threshold in the same slot and the RB-sets close to the intended half-slot transmission

Proposal 12: A transmitter may initiate transmission with 2 starting symbols in a slot as long as it has detected strong WiFi interference conditions indicated by metrics such as LBT failure rate and CBR, possibly accounting for priority of transmission

Proposal 13: For the resource pool configured with two starting symbols, the transmitter determines the reference number of symbol (Lref) for TBS calculation based on the 1st starting symbol, and Tx UE (by implementation) may choose lower MCS for the slots with two starting symbols when the chance to clear LBT before the 1st symbol is not high

Proposal 14: Limit PSCCH search locations to a subset of subchannels in half-slot access slots to meet the blind SCI decoding limit per slot

Proposal 15: When half-slot access is configured, the receiver may skip the PSCCH monitoring at later potential PSCCH candidate locations in a (full) slot within an RB-set, if, in an earlier candidate location in that slot and the same RB-set, SCI-1 is decoded and/or PSCCH DMRS RSRP is above some preconfigured threshold

Proposal 16: Half slot access could be RRC configured per link based on UE capability for unicast and groupcast option 2

Proposal 17: Support enhanced DCI3_x to schedule multiple contiguous slots in mode 1 and further study the minimum signaling required

Proposal 18: Support enhanced SCI-1/2 to schedule multiple contiguous slots for one or multiple SL receivers and further study the minimum signaling required

Proposal 19:  Support option 1 for PSCCH resource mapping in which PSCCH is located in the lowest sub-channel of lowest RB set of corresponding PSSCH

Proposal 20: For interlaced waveforms, PSCCH occupies (pre-)configurable number of leading interlaces in the leading subchannel within the leading RB-set of the allocated resources and PSSCH is multiplexed with PSCCH via TDM and/or FDM

Proposal 21: For interlaced waveforms, PSCCH is rate matched to the common or minimum RB-set within a 20MHz subband to allow nodes with different RB-set configurations to communicate to each other

Proposal 22: For contiguous RB waveforms, a subchannel overlapping with intra-cell guard band cannot be used as a starting/leading subchannel of a transmission which contains PSSCH, i.e., it can only contain PSSCH

Proposal 23: For interlaced waveforms, SCI-2 is rate matched to RB-sets belonging to the common or minimum RB-set(s) and the RB-set indices are indicated by SCI-1

Proposal 24: For interlaced waveforms, PSSCH could be rate matched to the per-link configured RB-set(s) (if supported) after the RRC config. If not supported/configured, PSSCH is rate matched to the common RB-set(s) or minimum RB-set(s) and the RB-set indices are given by SCI-1

Proposal 25: To improve the resource utilization when the number of allocated subchannels in an RB-set is greater than a threshold, the last subchannel in the RB-set overlapping with intra-cell guard band can be allocated while the PSSCH REs outside of the RB-set

Proposal 26: Use two-part FRIV-(X,Y)  to handle the interlace and RB-set allocation and reservations independently in which X/Y is encoded in legacy fashion based on pre-determined total number of interlaced subchannels/RB-sets, and the FRIV is defined relative to the SCI-1 reception interlace subchannel and RB-set

Proposal 27: For multi-bit A/N’s carrying PSFCH, reuse 1-symbol interlaced NR-U PF2 with additional front AGC symbol

Proposal 28: (Unified framework between Alt 1-1a and Alt 2-3a/2-4a) Configurable number (K3) of contiguous A/N carrying PRBs (K3) on a non-common interlace and N-K3 dummy PSFCH IRBs on the common interlace with reserved cyclic shift resources to fulfill the OCB requirement, where N is the number of IRBs in one interlace. The waveform becomes a full interlaced PSFCH waveform as in Alt 2-3a/2-4a when K3=N

Proposal 29: Place the N-K3 dummy IRBs in the different IRB clusters of the common interlace from the K3 A/N carrying IRBs, and drop the edge dummy IRBs which are within X-RBs of the A/N carrying IRBs to boost the A/N transmit power

Proposal 30: Apply common cyclic shift ramping to both A/N carrying and dummy IRBs of PSFCH to reduce the PAPR

Proposal 31: To determine PF0 based PSFCH resource within the RB-set(s) occupied by PSSCH, perform one to one mapping from PSSCH leading interlace and slot index to PSFCH interlace (for alt 2-3a) or PSFCH partial interlace set (for alt 1-1b and each partial interlace contains K3 IRBs as in the discussion above) and CS pair set therein, and hashing within the associated CS pair set

Proposal 32: To support wideband PSSCH transmission, map one PF0-based PSFCH resource to an PSFCH interlace spanning all the RB-sets in the data resource pool and only transmit the truncated PF0 PSFCH in the PSSCH transmitting RB-set(s), i.e., PSFCH occupies the same RB-set(s) as the associated PSSCH

Proposal 33: A PSFCH transmission in the context of a wideband COT occupying more RB-sets than the associated PSSCH transmission, is allowed to transmit PSFCH-like RB-set padding signal in the additional RB-set(s) to hold the wideband COT. The resources used by the padding signal could be PSFCH resources or system-wide configured (i.e., common) resources

Proposal 34: The Rx UE is only required to monitor the legacy 11RB S-SSB and whether to monitor S-SSB frequency repetitions (if present) is up to UE implementation

Proposal 35: For S-SSB repetition waveform (option 3-1), two repetitions (N=2) and a gap >22/66 RBs for SCS=30/15 KHz suffices to fulfill the OCB requirement

Proposal 36: To reduce the PAPR of frequency repeated PSBCH, study applying different scrambling sequences for different frequency copies of PSBCH

Proposal 37: To reduce the PAPR of frequency repeated SSSS, study applying different sequence shift(s) to pseudorandom sequence(s) for different frequency copies of SSSS

Proposal 38: To support multiple PSFCH opportunities associated with a PSSCH with different HARQ timelines, configure multiple (FDM/TDM) PSFCH resource sets (one set per HARQ timeline) or use differently hashed resources over a common/shared PSFCH resource set for each HARQ timeline

Proposal 39: Support PSFCH-like padding signal to fill the PSFCH symbol when the COT initiating UE is neither sending nor expecting to receive a PSFCH

Proposal 40: For mode 1 operation, if multiple PSFCH candidates are configured, the reference slot n for PUCCH transmission to report HARQ (or for PSFCH-to-HARQ_feedback indicated by DCI 3) is the slot after the last PSFCH candidate

Proposal 41: If multiple PSFCH candidates are configured, for DRX operation, the sl-drx-HARQ-RTT-Timer can be started after the 1st PSFCH transmission which clears the LBT or the last PSFCH candidates for the maximal power saving

Proposal 42: Support type-1 HARQ codebook with the legacy fixed HARQ timeline and further study other SL HARQ codebooks to improve Ack/Nak feedback efficiency and help with Ack/Nak retransmission due to LBT failure

Proposal 43: Introduce multiple (M) S-SSB candidate slots starting from each nominal S-SSB instance(s) to allow multiple LBT tries for S-SSB transmission and allow multiple S-SSB transmission slots (L), and further study the number M and L

Proposal 44: At least for the S-SSB Tx UE, if wideband COT transmission is initiated across multiple RB-set, for all the S-SSB occasions within COT transmission burst, allow UE to repeat the S-SSB in the non-anchor RB-sets for S-SSB transmission

Proposal 45: To avoid PSBCH-RSRP fluctuation with dynamic S-SSB repetition in multiple RB-sets, one can limit the number of the S-SSB repetition in different RB-set(s) subject to UE’s maximum Tx power or start with lower Tx power for legacy 11-RB S-SSB transmission in anchor RB-set

# Reference

- RP-213678, “New WID on NR sidelink evolution”, RAN #94, OPPO, LG Electronics

- ETSI EN 301 893 V2.1.1

- RP- 2205033, “Channel Access Mechanism for SL-U”, RAN1 #109e, Qualcomm

- 3GPP TS 38 306, “User Equipment (UE) radio access capabilities” V17.1.0 (2022-06)

- R1-2305337, “Channel Access Mechanism for SL-U”, RAN1 #113, Qualcomm

- IEEE Std 802.11ax-2021, "IEEE Standard for Information Technology--Telecommunications and Information Exchange between Systems Local and Metropolitan Area Networks--Specific Requirements Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications Amendment 1: Enhancements for High-Efficiency WLAN”

- R1-2209985, “Channel Access Mechanism for SL-U”, RAN1 #110-bis-e, Qualcomm