3GPP TSG RAN WG1 Meeting #113	R1-2305339

Incheon, Korea, May 22nd – 26th, 2023

Agenda item:	9.4.2

Source: 	Qualcomm Incorporated

Title: 	Co-Channel Coexistence between LTE SL and NR SL

Document for:	Discussion/Decision

# Introduction

The work item for Release-18 NR sidelink enhancement includes multiple objectives [1]:

- Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN4]

- Support only LTE sidelink CA features for NR (i.e., SL carrier (re-)selection, synchronization of aggregated carriers, power control for simultaneous sidelink TX, packet duplication)

- The work is limited to intra-band CA for the ITS band in FR1 (Band n47).

- No specific enhancements of Rel-17 sidelink features with sidelink CA support.

- This feature is backwards compatible in the following regards

  - A Rel-16/Rel-17 UE can receive Rel-18 sidelink broadcast/groupcast transmissions with CA for the carrier on which it receives PSCCH/PSSCH and transmits the corresponding sidelink HARQ feedback (when SL-HARQ is enabled in SCI)

- Only Mode 2 operation

- Same subcarrier spacing (SCS) among CA carriers to avoid resource selection enhancements and AGC issues

  - Time resources for PSFCH are aligned among the carriers for CA

- No enhancement related to SCI transmissions on PSCCH/PSSCH, PSFCH transmission, RSRP feedback, CSI feedback and congestion control compared to Rel-16 (i.e., per-carrier operation)

  - SL resource indication remains to be per-resource pool and per-carrier basis (no cross-carrier scheduling in SCI)

  - UE transmits SL HARQ feedback on the same carrier on which it receives the associated PSSCH

- No consideration for limited transmission and reception capability

- No primary/secondary carrier differentiation

- Reuse the LTE sidelink CA design for the following aspects:

  - Sidelink carrier (re-)selection, synchronization of aggregated carriers, Tx power split for simultaneous sidelink transmissions, packet duplication

- The CA band combination work in RAN4 is limited to intra-band contiguous CA in Rel-18.

- Note: The SL CA work in Rel-18 mainly targets some V2X use cases

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4]

- Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation

  - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation

    - No specific enhancements for Rel-17 resource allocation mechanisms

    - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval.

- Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum

  - The existing NR sidelink and NR-U channel structure shall be reused as the baseline.

- No specific enhancements for existing NR SL feature

- Focus on FR1 unlicensed bands (n46 and n96/n102).

- Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel.

- Study enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2] (Determine in RAN#98-e whether to continue the study or study + specification work for FR2 until the end of R18)

- Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1]

- Study is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. [RAN1, RAN2]

  - Beam management in FR2 licensed spectrum considers sidelink unicast communication only.

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4]

- Reuse the in-device coexistence framework defined in Rel-16 as much as possible

- Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A

- RAN1 is tasked to support only 15 and 30 kHz SCSs for dynamic resource pool sharing. Existing RAN1 agreements for dynamic resource pool sharing apply to support of 30 kHz.

  - For NR PSCCH/PSSCH transmissions in 30kHz SCS, NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with an LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer

    - No change to the R16/17 resource allocation procedure in PHY due to this restriction

    - The existing SL slot structure from Rel-16 is unchanged

    - The starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe

  - For NR SL with 15/30kHz SCSs, NR SL UE avoids selecting resources for PSCCH/PSSCH transmissions where the corresponding PSFCH transmission occasions overlap with LTE SL reservations in time domain

    - Note, this is inline with Option 1-2 in the working assumption made in RAN1#112. No other options from the working assumption need to be considered.

  - Mode 2 operation only

- UE Tx and Rx RF requirement for supporting new features introduced in this WI, sidelink frequency bands for single-carrier operation and frequency band combinations for carrier aggregation operation [RAN4]

- The exact frequency bands for both licensed and ITS-dedicated spectrum in FR1 and FR2 are to be determined based on company input during the WI.

- The frequency band combinations is limited to the ITS-dedicated spectrum only (Band n47) for contiguous carrier aggregation.

- Frequency bands for the unlicensed spectrum in FR1 are [n46 and n96/n102] (i.e., 5GHz and 6GHz) in accordance with corresponding national regulatory requirements.

- Support of new sidelink frequency bands and band combinations should ensure coexistence between sidelink and Uu interface in the same and adjacent channels in licensed spectrum.

- UE RRM core requirement for the new features introduced in this WI [RAN4]

Rel-18 sidelink should be able to coexist with Rel-16/17 sidelink in the same resource pool. This does not preclude the possibility of operating Rel-18 sidelink in a dedicated resource pool.

In this contribution we propose a design to achieve co-channel coexistence between NR Sidelink (SL) and LTE Sidelink based on dynamically sharing a common resource pool. The proposed design focuses on the coexistence of dual-RAT (Type A) devices with collocated NR SL and LTE SL modules and legacy (Type C) devices with only LTE SL modules in a shared resource pool operating outside network coverage (Mode 2 NR SL + Mode 4 LTE SL).

The rest of the paper is organized as follows. In Section 2, we discuss the proposed scheme for the cochannel coexistence of NR SL and LTE SL UE-s based on dynamically sharing the shared resource pool (RP). Next, in Section 3, we discuss the issue and propose a solution to mitigate inter-RAT conflicts in a Type A (dual-RAT) device. In Section 4, we discuss NR SL and LTE SL operating with different numerologies over the shared channel using dynamic RP sharing. Evaluation results are presented in Section 5 and the simulation assumptions are included in the appendices.

# Dynamic Resource Pool Sharing by NR SL and LTE SL

Assigning a set of transmission resources for the use of NR SL is necessary to enable cochannel coexistence. Rel. 16 (and 17) NR SL UE-s can share a common resource pool (RP) with legacy LTE SL UE-s using a (semi-)static partition of the resource pool through (pre-)configuration. The following conclusion was reached on this issue in RAN 110.

Conclusion

For co-channel coexistence in Rel-18, RAN1 concludes that the TDM-based semi-static resource pool partitioning based on Rel-16/17 specifications is one possible solution to ensure co-channel coexistence between LTE-V UEs and NR-V UEs.

- Note: The LTE and NR resource pools do not overlap in time with each other in the TDM-based semi-static resource pool partitioning.

- Note 2: Rel-16 in-device coexistence framework can ensure alignment between the slot boundary of the NR SL time slot and the subframe boundary of the LTE SL subframe

- FFS: potential enhancements for synchronization can be further investigated

The (semi-)static resource pool partition can be determined, for instance, based on some estimate of the ratio of the traffic load over LTE SL to that of NR SL. As more, newer, NR UE-s are introduced in the system the ratio of the traffic load over NR SL to the traffic load over LTE SL will change. These changes may also occur under other scenarios, such as when a vehicle moves from one geographic location to another.  Thus, a technique to update the resource partition with the change in traffic over the shared channel is beneficial for cochannel deployments of NR SL and LTE SL.

The dynamic resource pool sharing (DRPS) between NR SL and LTE SL needs to be achieved in a manner such that (a) there are no changes required for existing (legacy) LTE devices, and (b) the resource set can adapt to the current traffic load over the sidelink system. For the rest of this paper, we will consider Rel. 18 NR SL UE-s to have a collocated LTE SL UE (i.e., Type A device) as per the following working assumption.

Working assumption: 
Co-channel coexistence between LTE SL and NR SL is supported for device type A. Device type A contains both LTE SL and NR SL modules. For device type A, the NR SL module may use the sensing and resource reservation information shared by the LTE SL module.

In this section, we discuss how a NR SL module of a Type A UE, based on the input received from the collocated LTE SL module, dynamically determines a set of transmission resources for the NR SL transmission over the shared channel.

## Information shared between collocated NR SL and LTE SL modules

For DRPS, the physical (PHY) layer LTE SL module of a Type A UE shares LTE SL resource reservation/usage information with the PHY layer of the collocated NR SL module. The NR SL module needs to make an accurate estimate of the resources that are required by LTE SL transmissions to guarantee reliable transmission for the traffic over LTE SL which includes legacy services such as basic safety messages (BSM), coordinated autonomy messages (CAM), etc. For this, the LTE SL module of a Type A UE should share information for SL resources, represented by the time and frequency location of the resources, over which the LTE SL module transmits or receives data, or may transmit or receive data in a future occasion. Moreover, for resources over which a data packet has been received or is scheduled to be received, an indication of the received signal power is useful to determine the validity of the reservation.

Observation 1: The LTE SL module of a Type A UE, based on the Rel. 16/17 in-device coexistence framework, shares the time and frequency location of the resources over which it has transmitted or received a SL packet or is scheduled to transmit or receive a SL packet.

Proposal 1: The LTE SL module of a Type A UE shares the received signal reference power (RSRP) measured over the resources over which an LTE SL transmission was received.

Further, the LTE SL module of a Type A UE may indicate the priority of the traffic associated with the indicated time-frequency resources. Note that the in-device coexistence mechanism is based on traffic priorities and the priority information is already shared between the NR SL and the LTE SL modules. For dynamic coexistence, the priority indicated for the LTE SL resource information may be further used to determine the potential relevance of the reservation.

Observation 2: The LTE SL module of a Type A UE, based on the Rel. 16/17 in-device coexistence framework, shares the priorities associated with the indicated time-frequency resources.

## NR SL PHY layer resource selection for DRPS

The following agreement was made in RAN#112bis-e regarding NR SL resource exclusion by the NR SL module of a Type A UE based on the resources reserved by other LTE SL UE-s for transmissions (as shared by the collocated LTE SL module) [5].

Agreement

In NR SL resource (re)selection procedure, for dynamic resource pool sharing, the list of initial SL RSRP thresholds is separately (pre)configured (i.e., Alt 3 in the agreement of RAN1#112 meeting) for the PHY layer of NR SL module to exclude NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE.

- FFS: whether a different initial SL RSRP threshold list may be (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH.

The NR SL module of a Type A UE will also need to factor in the resources that are selected or reserved by the collocated LTE SL module for its own transmissions. Without this consideration the Type A UE will incur half-duplex losses. For this, the flowing agreement was reached in RAN 112-bis [5].

Agreement

In NR SL resource (re)selection procedure for dynamic resource pool sharing, the PHY layer of NR SL module excludes NR SL candidate resources in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission

- For the LTE SL periodic resources selected to be used for LTE SL module’s own LTE SL transmission,

  - For determining the above LTE SL selected resources, the LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission are repeated according to the LTE SL resource reservation period and LTE SL resource reselection count

- The PHY layer of NR SL module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214

  - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes all NR SL candidate resources in a NR SL slot where NR SL periodic resources are in the NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission according to Step 5 in Section 8.1.4 of TS 38.214

  - Note: When the PHY layer of NR SL module cancels the above procedure according to Step 5a in Section 8.1.4 of TS 38.214, UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule

- Alt 1: The above procedure is applied at least when the priority of LTE SL transmission is higher than the priority of NR SL transmission

  - It is up to UE implementation whether or not to apply the above procedure when the priority of LTE SL transmission is not higher than the priority of NR SL transmission

- Note: It is assumed that the information relevant to LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission used in the above procedure is shared from LTE SL module to NR SL module

Further, a key concern for NR SL and LTE SL cochannel coexistence is the degradation to LTE SL reception caused by the transmission of NR SL feedback over the NR SL PSFCH channel. LTE SL does not use HARQ-based feedback transmissions. One the other hand, for NR SL, HARQ-based re-transmissions is a basic feature that enables ultra-reliable communications. Based on the update to the work item description from RAN #99 [1], only transmitter-side exclusion of resources is considered to minimize the collisions between NR PSFCH and LTE sidelink for DRPS. More specifically, the NR sidelink resource (re)selection procedure excludes resources from the set of transmission resources for which the associated feedback slot collides with LTE SL reservations from other UE-s in the time domain. For this, RAN 1 made the following agreement in RAN 112bis-e meeting [5]:

Agreement

In NR SL resource (re)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources where the corresponding PSFCH transmission occasions overlap with LTE SL reserved resources by other LTE SL UE in time domain,

- Down-selection one of followings:

  - Option 1:

    - When the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission

      - The list of the above initial SL RSRP thresholds is separately (pre)configured

- FFS: whether this initial SL RSRP threshold list can be (pre)configured per subset of PSFCH resources

    - For determining the LTE SL periodic reserved resources by other LTE SL UE,

      - Reuse the same mechanism as in NR SL resource (re)selection procedure excluding NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE

    - The PHY layer of NR SL module applies the above procedure in Step 6 in Section 8.1.4 of TS 38.214

      - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of NR SL periodic resources when the corresponding PSFCH transmission occasions are overlapping with LTE SL reserved resources by other LTE SL UE and the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold according to condition c in Step 6 in Section 8.1.4 of TS 38.214

    - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module

  - Option 2:

    - For determining the LTE SL periodic reserved resources by other LTE SL UE,

      - Reuse the same mechanism as in NR SL resource (re)selection procedure excluding NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE

    - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214

      - FFS: whether to apply the above procedure based on the priority of the LTE SL reserved resources and/or the priority of the NR SL transmission

      - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of NR SL periodic resources when the corresponding PSFCH transmission occasions are overlapping with LTE SL reserved resources by other LTE SL UE in time domain according to Step 5 in Section 8.1.4 of TS 38.214.

    - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module

The NR SL resource (re)selection procedure for DRPS between NR SL and LTE SL should, then on one hand reduce the number of collisions between NR SL PSCCH/PSSCH and LTE SL PSCCH/PSSCH transmissions, and on the other hand minimize the collisions between LTE SL PSCCH/PSSCH transmissions with NR SL feedback transmissions over PSFCH.

Observation 3: NR SL transmit-side resource exclusion procedure in Clause 8.1.4 of TS 38.214 is enhanced to reduce collisions between LTE SL PSSCH/PSCCH transmissions with NR SL feedback transmissions over PSFCH for DRPS.

Observation 4: NR SL resource (re)selection procedure for DRPS with LTE SL needs to minimize both NR SL PSCCH/PSSCH and LTE SL PSCCH/PSSCH collisions and NR SL PSFCH and LTE SL PSCCH/PSSCH collisions.

Based on the updated WID [1] and the previous RAN 1 agreements, a NR SL module of a Type A UE transmitting over the PSCCH/PSSCH in the shared RP needs to avoid collisions in time-domain between associated NR SL feedback transmissions over PSFCH and LTE SL transmissions. For this, the NR SL UE excludes a resource for the PSSCH transmission from the set of single-slot resources when the NR SL PSFCH slot over which the feedback for the PSSCH transmission will be sent collides with an LTE SL reservation received with an RSRP of γfeedbackLTE or greater. Here we emphasize that having no RSRP threshold for

Figure 1: Comparing NR SL performance of dynamic RP sharing with NR SL transmitter avoiding feedback overlap in time domain with LTE SL reservations based on a RSRP threshold versus NR SL transmitter avoiding feedback overlap in time domain with all decoded LTE SL reservations.

checking the PSFCH collision with LTE SL reservations at the NR SL transmitter inevitably leads to an over-exclusion of NR SL transmission resources degrading of NR SL performance. The effect of this over exclusion is demonstrated in Figure 1. We observe that, excluding a resource for NR SL transmissions when the associated feedback slot has a LTE SL reservation, irrespective of the RSRP associated with the reservation, leads to a severe over-exclusion of NR SL resources.

Observation 5: Excluding a resource from the NR SL set of available resources based on the corresponding PSFCH overlap with LTE SL reservations in time domain irrespective of the RSRP associated with the LTE SL reservation leads to over exclusion of NR SL resources.

On the other hand, based on the agreement in RAN 112-bis meeting, excluding resources where the corresponding PSFCH occasion overlap with LTE SL reservation in time domain in Step 5 (option 2) can lead to a high number of collisions between NR SL feedback and LTE SL transmissions. This is because, in most cases, a large number of resources will be excluded in Step 5 leading the available resource in set SA to be less X*Mtotal. This in turn will lead to the cancellation of Step 5 in Step 5a of Clause 8.1.4 of [4]. Note that, deactivation of Step 5a to force the NR SL module to force the exclusion of resources with corresponding PSFCH overlapping with LTE SL transmissions will lead to undefined behaviour. For many cases, there will be no protection available for LTE SL transmissions from NR SL PSFCH collisions due to the cancellation of Step 5. Performing the resource exclusion for collision between LTE SL transmissions and NR SL feedback in Step 6 based on adapting a (pre)configured RSRP thresholds provides a way to balance the performance of NR SL and LTE SL in the shared pool (as shown in Section 5.2).

Observation 6: Excluding NR SL transmit resources based on the corresponding time-domain PSFCH overlap with LTE SL reservations in Step 5 will lead to a large number of exclusions and subsequent cancellation of Step 5 based exclusion.

Observation 7: Deactivation of Step 5a to force NR SL to exclude resource leads to undefined behaviour for NR SL resource (re)selection.

Observation 8: Resource exclusion based on PSFCH overlap with LTE SL in time domain in Step 5 leads to no protection available for LTE SL collision with NR PSFCH.

Proposal 2: NR SL module of a Type A UE excludes in Step 6 a resource from the set of NR SL transmit resources if an LTE SL reservation received with an SL RSRP greater than a threshold overlaps with the corresponding PSFCH occasion in time domain (Option 1).

As the mechanism to avoid NR PSFCH collision with LTE SL transmissions is performed at the NR SL transmitter it is possible to have some NR feedback collide with LTE SL transmissions. For example, the LTE SL UE-s near the NR SL UE receiving on a PSSCH and transmitting feedback may not be sufficiently protected by transmit-side resource exclusion. For a considerable reduction in LTE SL collisions with NR feedback, it is necessary for the NR Tx resource (re)selection procedure to prioritize transmissions on NR SL slots with PSFCH resources. The prioritizing of NR SL transmissions on slots with PSFCH resources will lead LTE SL UE-s recording a high received signal strength on the NR PSFCH slots. This enables the LTE UE-s in the shared channel to de-prioritize transmissions on the LTE SL subframes colliding with NR PSFCH slots based on the RSSI-based resource ranking.

Observation 9: Prioritizing NR SL PSSCH transmissions over NR SL slots with PSFCH aids LTE SL UE-s in the shared pool to avoid transmissions over NR SL PSFCH slots.

Proposal 3: NR SL resource (re)selection algorithm prioritizes NR SL PSSCH transmissions on slots with PSFCH resources.

To prioritize NR SL transmissions over the slots with PSFCH resources, the NR SL module of a Type A UE may be configured with two separate RSRP thresholds:

- An initial RSRP threshold  γ1LTE to exclude NR SL single-slot resources colliding with LTE SL reservations on slots without NR SL PSFCH, and

- An initial RSRP threshold γ2LTE to exclude NR SL single-slot resources colliding with LTE SL reservations on slots with NR SL PSFCH resources.

As an example, consider the case when the NR SL module is configured to exclude (in step 6 of clause 8.1.4 of TS 38.214 [4]) single-slot resources that collide with LTE SL reservations with γ1LTE< γ2LTE. In this case, an LTE SL reservation that maps to a NR SL slot with PSFCH resources will need to have an RSRP higher than reservations that maps to non-PSFCH slots. Thus, the NR SL resource selection mechanism prioritizes NR PSSCH transmissions on slots with NR PSFCH channel. This “crowding” of the NR PSFCH slots ensures that the LTE UE-s in the systems record a high RSSI over NR PSFCH slots and avoid them based on Step 8 and 9 of clause 14.1.1.6 of TS 36.213 [3].

Proposal 4: The NR SL module of a Type A UE performs resource exclusion in step 6 of clause 8.1.4 of TS 38.214 based on LTE SL reservations using different RSRP thresholds for resources on slots without NR PSFCH and for resources on slots with NR PSFCH.

Proposal 5: SL RSRP threshold list are separately (pre)configured for dynamic resource pool sharing with a different SL RSRP threshold list (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH.

A simple way then to determine the value of γfeedbackLTE is to use the same RSRP exclusion threshold used for excluding the PSSCH resource for which this feedback occasion is used. In this case, if a single-slot resource for NR PSSCH transmission is in a NR SL slot without PSFCH, the NR SL UE will include the resource in the NR Tx resource set if this resource is not colliding with an LTE SL reservation received with a RSRP greater than or equal to γ1LTE and the PSFCH slot over which the feedback for this PSSCH transmission is received does not collide with an LTE SL reservation received with a RSRP greater than or equal to γfeedbackLTE=γ1LTE. Similarly, if the single slot resource is in a NR SL slot with PSFCH, the NR SL UE determines to exclude the resource using the RSRP thresholds γ2LTE and γfeedbackLTE=γ2LTE.

Proposal 6: A resource is excluded from the set of available NR SL transmission resource if either the resource or the slot with the corresponding feedback collides with an LTE reservation received with an RSRP above a threshold.

Figure 2: Example of NR SL resource exclusion based on LTE SL reservations.

In Figure 2 we show an example of the resource exclusion procedure based on the proposed differentiated NR SL RSRP thresholds. In this example, γ1LTE< γ2LTE. As shown in Figure 2, the resource in slot a (a NR PSFCH slot) is available for NR SL transmission as both the resource in this slot and the associated feedback slot a+4 does not collide with an LTE SL reservation received with a RSRP ≥γ2LTE. Similarly, the resource on slot a+9 (a NR slot without PSFCH) is available for transmission as both the resource and the corresponding PSFCH occasion do not collide (in the time domain) with LTE reservations received with a RSRP ≥γ1LTE. On the other hand, the resource in slot a+2 (a NR slot without PSFCH) is excluded from the set of available resources as even though it itself does not collide with an LTE SL reservation received with a RSRP ≥γ1LTE, the corresponding feedback occasion collides (in the time domain) with a LTE SL reservation received with a RSRP >γ1LTE.

Proposal 7: For excluding NR SL candidate resources where the corresponding PSFCH transmission occasion overlap with LTE SL reserved resources by other LTE SL UE in time domain a list of SL RSRP is separately (pre)configured for resources on slots without NR PSFCH and for resources on slots with NR PSFCH.

Another aspect for NR SL resource exclusion based on LTE SL reservation is to determine the value of the number of times an LTE SL reserved resource is repeated based on the indicated LTE SL resource reservation periodicity Prsvp_RX. As the resource exclusion is performed by the NR SL module, it was agreed in RAN 1 that the NR SL module should use the formula for the determination of Q based on Section 8.1.4 of TS 38.214 [4].

Agreement

In NR SL resource (re)selection procedure, for determining the LTE SL periodic reserved resources by other LTE SL UE, the formula of Q in Section 8.1.4 in TS 38.214 is used.

- FFS: whether the formula of Q in Section 14.1.1.6 in TS 36.213 is used additionally to derive the largest value between the two formulas

Note: The reservation period and the location of the LTE SL periodic reserved resources are determined based on the set of LTE SL logical subframe that may belong to LTE SL resource pool as specified in TS 36.213.

Here we note that calculating the value of Q based on the formula for NR SL resource exclusion is sufficient to make sure that all candidate resources in the selection window of size Tscal colliding with a periodic LTE SL reservation is excluded from the Tx resource set SA (based on the associated RSRP). There is then no need to introduce additional complexity in the resource exclusion procedure by deriving the value of Q from the maximum of the values from the NR SL formula and the LTE SL formula.

Proposal 8: The NR SL module excludes LTE SL reservation based on the indicated P’rsvp_RX and using only the formula for Q defined in TS 38.214.

## Timing constraints on the information shared between NR SL and LTE SL module of a Type A UE

The following agreements were reached during previous RAN 1 meetings on information sharing between LTE SL module and NR SL module of a Type A device [5].

Agreement

For dynamic resource pool sharing, the NR SL module uses the information shared by the LTE SL module to the NR SL module to determine the set of resources for its own transmission.

- FFS: which layer carries out the resource determination: PHY layer or MAC layer.

Agreement

For dynamic resource pool sharing, the NR SL module is expected to use the information shared by the LTE SL module to the NR SL module which is known by NR SL module at the latest T ms prior to slot n (as defined in clause 8.1.4 of TS 38.214), to determine a set of resources for its own (re)transmission.

- T is defined using

  - T≤Tmax ms, and is based on UE implementation, according to the Rel-16 NR SL timeline for in-device coexistence.

    - FFS: Value of Tmax

- FFS: any discussion on the earliest information, if needed

Agreement

Based on the agreement in RAN1#110bis-e, the value of Tmax = 4 ms.

Agreement

The NR SL module uses the information from the starting LTE SL subframe to the ending LTE SL subframe in the shared information from the LTE SL module.

- The starting LTE SL subframe is no later than the time (n-T_start)

  - n is the time where NR module triggers its NR SL resource (re)selection procedure as defined in clause 8.1.4 of TS 38.214

  - Down-selection one of followings for T_start:

    - Option 1-1: T_start is T_0 as defined in clause 8.1.4 of TS 38.214

    - Option 1-2: T_start is 1100ms

    - Option 1-3: T_start is up to UE implementation

- The ending LTE SL subframe is no earlier than the time (n-T_valid2)

  - Down-selection one of followings for T_valid2:

    - Option 2-1: T_valid2 is T

    - Option 2-2: T_valid2 <= T+4ms

    - Option 2-3: T_valid2 is up to UE implementation

Figure 3: Timeline for resource sharing by LTE SL module to NR SL module for a Type A UE.

For the NR SL resource selection to be consistent across multiple NR devices in the system, RAN 1 should define the time window over which an LTE SL resource reservation information should be considered for NR SL resource selection in a shared resource pool. If this is left up to UE implementation, two NR SL modules of tow Type A devices receiving the same set of LTE SL reservations will have a different understanding of the LTE SL traffic over the network. This will have negative consequence on the mode 2 resource (re)selection procedure for DRPS. Especially, the NR SL transmit side exclusion of resources based on NR PSFCH and LTE SL collision avoidance due to this mismatch may not be able to exclude some resources as based on the PSFCH collision criteria and lead to LTE SL performance degradation.

Observation 10: Mismatch in the timeline for LTE SL resource exclusion across UE-s in the system leads to a degradation in the performance of the DRPS scheme.

Proposal 9: RAN 1 to specify the timeline for NR SL module of a Type A UE to consider LTE SL resource reservations in its resource (re)selection procedure.

As shown in Figure 3, for a resource (re)selection process triggered at a slot n, the NR SL UE considers all LTE SL reservation information received in the time window [n-Tstart, n-T], where based on previous agreements T≤Tmax= 4 ms. Thus, the ending LTE subframe for which the resource information shared with NR SL module of a Type A UE is used for resource (re)selection triggered at slot n is Tvalid2=T ms prior to the slot n. Here, the exact value of T≤Tmax is up to UE implementation and follows the in-device coexistence timeline defined in [3].

Proposal 10: The NR SL module uses the information up to the ending LTE SL subframe at n- T ms in the shared information from the LTE SL module, where T ≤ Tmax and is up to UE implementation following the in-device coexistence mechanism.

For proper exclusion of resources, the value of Tstart should ensure that an LTE SL SCI received on LTE SF tmSL with a reservation periodicity P'rsvp_RX (in logical slots/subframes) is considered for NR SL resource selection trigger at slot n when n-m<P'rsvp_RX. This can be ensured by setting Tstart≥max⁡(Prsvp_RX) ms. This can be satisfied by setting the NR SL window to consider LTE SL resource information to 1100 ms, which would include the largest LTE reservation period.

Observation 11: The value of Tstart should be selected such that the NR SL UE can exclude both the current reservation as well as a periodic repetition of the reservation at least P’rsvp_RX logical subframes in the future.

Proposal 11: RAN 1 defines the value of Tstart = 1100 ms.

# Type A UE behaviour

For the study of NR SL and LTE SL cochannel coexistence, RAN 1 focuses on UE-s that support both NR SL and LTE SL modules referred to as a Type A UE. Type A UE-s will need to transmit and receive both NR SL and LTE SL packets, hence RAN 1 will need to specify a mechanism to avoid or resolve conflicts between NR SL Tx and LTE SL Tx, between NR SL Rx and LTE SL Rx, and between NR SL Tx and LTE SL Rx. A Type A UE may be able to simultaneously receive NR SL and LTE SL packets over the same channel but will need to transmit in a time-division multiplexed fashion, i.e., either LTE SL or NR SL at a given time [6].

Observation 12: A Type A UE transmits NR SL and LTE SL in a TDM manner but can received simultaneously in the same channel.

One potential solution is to re-use the in-device coexistence mechanism as detailed in [2]. This may be adequate when one RAT has a higher priority than the other. On the other hand, when both the RAT-s (NR and LTE) have the same priority or when for both the applications over NR and over LTE require sufficiently high reliability, re-using the in-device coexistence mechanism for cochannel transmissions is not a good design choice as the de-prioritized RAT may have a very high packet drop rate leading to loss in reliability.

Observation 13: Type A devices, which contains both NR SL and LTE SL modules, need to implement a mechanism to resolve inter-RAT Tx-Tx and Tx-Rx conflicts for co-channel coexistence.

Observation 14: Dropping of transmission and reception based on a priority associated with a RAT will severely degrade the reliability of the transmission over the lower priority RAT.

A proper balance between the two RATs is ensured when a Type A UE does the following: (a) prioritizes transmissions over reception, i.e., a transmission is not dropped when in conflict with a known packet/feedback reception; and (b) in the case of a Tx-Tx conflict between the collocated NR SL and LTE SL, the Type A UE drops the NR transmission. Here, we note that as the NR SL module includes the resources selected or reserved for transmission by the collocated LTE SL module, such Tx-Tx conflicts have a low probability for PSSCH transmissions.

Proposal 12: Type A UE prioritizes transmissions over reception for a Tx-Rx conflict and prioritizes LTE SL transmissions over NR SL transmissions for a Tx-Tx conflict.

# NR SL and LTE SL operations with different numerologies

Based on the updated WID from RAN #99, the DRPS mechanism for NR SL and LTE SL cochannel coexistence will support NR SL operations with 15 kHz SCS or 30 kHz SCS (defined on a per resource pool basis) in the same band as LTE SL operations with 15 kHz SCS.

a. NR overlapped slots m and m+1 corresponding to one LTE SL subframe k.

b. Transmission only on the second NR SL overlapping slot.

Figure 4: One LTE SL subframe (SCS = 15 kHz) corresponds to two overlapping NR SL slots (SCS = 30 kHz). NR SL transmission only over the second overlapping slot leads to incorrect AGC setting for an LTE SL receiver.

As shown in Figure 4a, when NR SL operates with 30 kHz SCS in a RP shared with LTE SL (operating with 15 kHz SCS), one LTE SL subframe will be twice as long as a NR SL slot. In this case, one LTE SL subframe k  will correspond to two NR SL slots m and m+1. Alignment in time between the first symbol of LTE SL subframe k and the first symbol of the first NR SL slot m corresponding to this LTE SL SF (k) is ensured by resource pool configuration. These two NR SL slots, m and m+1, are overlapped with one LTE SL subframe k and NR transmissions over these slots will impact the reception of LTE SL transmissions over the subframe k.

Observation 15: When NR SL operates with 30 kHz SCS in a channel shared with LTE SL, two successive NR slots overlap with one LTE SL subframe.

The following agreements were made in RAN 112-bis [5] to support dynamic resource pool sharing between NR SL operating with 30 kHz SCS and LTE SL operating with 15 kHz SCS.

Agreement

For NR SL transmissions of 30kHz SCS with dynamic resource pool sharing, the power level of the NR PSCCH/PSSCH transmission in the first of NR SL slots overlapping with an LTE SL subframe is always larger than or equal to the power level(s) of the NR PSCCH/PSSCH transmission in the subsequent NR SL slot overlapping with the LTE SL subframe.

- Note: How to ensure the above condition is up to UE implementation

- FFS on whether same or different frequency allocation may be used in the second overlapping slot

Agreement

When the same TB is transmitted on the NR SL slots overlapping with the LTE SL subframe, it is up to UE implementation how to avoid transmitting NR PSCCH/PSSCH only in the subsequent NR SL slot overlapping with an LTE SL subframe according to RAN#99’s agreement for NR PSCCH/PSSCH transmissions of 30kHz SCS with dynamic resource pool sharing

- FFS: whether/how to differently handle the case when different TBs are transmitted on the NR SL slots overlapping with the LTE SL subframe and the NR SL transmission in the first overlapping NR SL slot is dropped or reselected.

If the NR SL MAC selects to transmit over both the first and the second NR SL slots overlapping with a LTE SL subframe, the transmissions over the second slot may be the same transport block (TB) that was transmitted in the first of the NR slots overlapping with an LTE SL SF or may be a different TB. Transmission of the same TB over consecutive slots may be possible when feedback is disabled for the TB. In fact, from RAN 1 perspective, the determination of the TB-s to be transmitted on pair of slots overlapping with an LTE SL SF is up to UE implementation.

Proposal 13: NR SL MAC may determine to transmit two separate TB or the same TB over both the first and the second slots overlapping with an LTE SL subframe.

Proposal 14: From RAN 1 perspective, it is up to UE implementation to re-transmit the same TB or transmit a different TB in the first and the second slots overlapping with an LTE SL subframe.

As shown in Figure 4b, NR SL UE transmitting over only the second of the NR SL slot overlapping with a LTE SL subframe will lead to an incorrect gain setting on the second half of the LTE SL subframe. Moreover, the total received power in the second half of the subframe in this case can be considerably greater than the power received during AGC training in the first symbol at an LTE SL received. This will saturate the LTE SL receiver and lead to decode failures due to signal clipping.  In the case when the SL MAC determines to transmit two separate TBs on the first and the second consecutive slots overlapping with a LTE SF, and the transmission of the first TB is dropped or the resources are re-selected (either due to resource evaluation or pre-emption), the NR SL UE will not transmit on the second slot and re-select the resources for the second TB.

Proposal 15: A NR SL UE always avoids transmitting NR PSCCH/PSSCH only in the second NR SL slot overlapping with an LTE SL subframe.

Proposal 16: When different TBs are transmitted on the NR SL slots overlapping with the LTE SL subframe and the NR SL transmission in the first of the overlapping slots is dropped or reselected (due to re-evaluation or pre-emption), the resources for the NR SL transmission on the second of the overlapping slot is reselected.

Next, we note that, depending on the resource pool configuration, a NR SL slot with PSFCH resources may map to either the first or the second slot of a pair of slots overlapping with a LTE SL subframe, when NPSSCHPSFCH=2, or 4 [2]. If the NR SL PSFCH slots are scheduled on the first slot of the slots overlapping with the LTE SL SF, it will collide with two symbols in the middle of the LTE SL. This can lead to a decode failure at the LTE SL receiver as the gain setting will be incorrect over multiple data symbols. On the other hand, if the RP configuration ensures that the PSFCH resources are always in the second of the NR slots overlapping with an LTE SL SF, this will ensure that only a part of the last data symbol and the LTE SL guard symbol are impacted due to PSFCH transmissions. This can considerably reduce the impact of LTE SL collision with NR SL PSFCH transmissions when NR SL UE-s operate using 30 kHz SCS.

Observation 16: NR PSFCH resources, if present in the second of the NR SL slots overlapping with a LTE SL subframe, will collide with only the last PSSCH symbol of the LTE SL and the guard symbol.

Observation 17: NR SL PSFCH if present on the second NR SL slots overlapping with LTE SL subframes will minimize the impact of PSFCH collision with LTE SL.

Proposal 17: NR SL PSFCH resources are always in the second of the NR SL slots overlapping with LTE SL subframe.

To further minimize the effect of NR SL PSFCH collision with LTE SL transmissions in a dynamically shared RP with NR operation with 30 kHz SCS, only NPSSCHPSFCH = 4 is supported for NR SL. This will ensure that at most one in every two LTE SF is affected by NR SL PSFCH collisions. A higher value of PSFCH periodicity, for example 8 or 10, can be investigated to minimize the NR SL PSFCH collisions with LTE SL transmissions when NR SL operates with 30 kHz SCS.

Proposal 18: NR SL operating with 30 kHz SCS over a RP shared with LTE SL is configured with NPSSCHPSFCH = 4.

# Evaluation Results

In this section we provide evaluation results for the techniques for NR SL and LTE SL cochannel coexistence discussed in Sections 2 (where both NR SL and LTE SL operate with 15 kHz SCS). We consider the highway scenario in our analysis where the inter-vehicle distance is determined by UE dropping model A [5]. In this set up, a UE may be a Type A UE with both NR SL and LTE SL capabilities or a Type C UE which supports only LTE SL. Detailed simulation assumptions are provided in Appendix A.

## Comparison of DRPS schemes

To quantify and compare the performance of the proposed dynamic RP sharing scheme, the study presented in this section considers the following scenarios:

- Alternative 3: The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a separately (pre-)configured SL RSRP threshold. Here we consider two possible SL RSRP threshold -106 dBm and -126 dBm. γ1LTE=γ2LTE= -106 dBm, and γ1LTE=γ2LTE= -126 dBm

- Alternative 4: The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold of γ1LTE=-126 dBm if the candidate resource is on a slot without PSFCH, or higher than a SL RSRP threshold of γ2LTE=-106 dBm if the candidate resource is on a slot with PSFCH

For all the three alternatives studied in this section, the simulations consider the following:

- Step 5 based resource exclusion for the resources selected or reserved for transmission by the collocated LTE SL module,

- PHY layer of NR excludes a NR SL candidate resource for which the corresponding PSFCH occasion overlaps in time domain with LTE SL reserved resource with RSRP greater than a threshold γ1LTE when the resource is in a slot without a PSFCH channel or with RSRP greater than a threshold γ2LTE when the resource is in a slot with a PSFCH channel (Proposal 6), and

- Based on the current NR SL specifications the SL RSRP thresholds γ1LTE and γ2LTE are increased by 3 dB when X.Mtotal resources are not available after resource exclusion as per Step 7 of Clause 8.1.4 of TS 38.214.

Note that for dynamic RP sharing, our simulations consider that the Type A UE avoids inter-RAT conflicts based on the mechanism detailed in Section 3.

In Figure 5 we show the performance of DRPS with the proposed Alternative 4 and DRPS with two configurations for Alternative 3. From Figure 5, we observe that DRPS with the proposed Alternative 4 outperforms Alternative 3. In fact, it is critical to note here that the proposed Alt. 4 provides a greater protection to LTE SL transmissions in the system. For Alt. 3, we observe that setting a higher or a lower RSRP threshold for LTE SL resource exclusion at the NR SL module can trade-off NR SL performance for LTE SL performance and vice-versa, but it does not reduce NR SL PSFCH and LTE SL collisions. This is because Alt. 4 reduces the probability that an LTE SL UE will select resources on slots with NR PSFCH.

To better illustrate this, in Figure 6 we show the distribution of LTE SL transmissions over time. The LTE SL subframes and NR SL slots are aligned, and NR PSFCH is configured every 4th slot (i.e., slot 3, 7, 11, 15 and 19). We observe that compared to the two configurations for Alt. 3 based resource exclusion, Alt. 4 based exclusion reduces that LTE SL transmissions on NR PSFCH slots by over 40─50%. Conversely, the two configurations of Alt. 3 will lead to 1.5─2x increase in the number of NR PSFCH collision with LTE SL transmissions compared to Alt. 4.

Observation 18: NR SL exclusion of LTE SL reservations based on Alternative 4 provides greater protection to LTE SL transmissions compared to Alternative 3.

Observation 19: The proposed NR SL resource exclusion mechanism leads to a 40% to 50% reduction in the probability of LTE SL colliding with NR PSFCH.

Figure 5: Performance of NR dynamic resource pool sharing mechanism with the comparing Alternative 3 and Alternative 4 for Step 6 resource exclusion of LTE SL reservation with step 5 based exclusion of transmit resource of the collocated LTE module.

Figure 6: Histogram of subframes (aligned with NR slots) for LTE SL transmissions over the shared channel for the three alternatives studied. The scenario under consideration is for a network with 300 Type A and 300 Type C UE-s (corresponding to the bottom panel of Figure 5).

## Effect of SL RSRP threshold for exclusion based on PSFCH collision.

Next, we evaluate the performance of the Alt. 3 based resource exclusion when the SL RSRP threshold γfeedbackLTE for exclusion of resources where the corresponding PSFCH occasion collides with LTE SL reservation. We compare the performance with the proposed Alt. 4. The handling of the PSFCH collision is given below.

- Alt. 4:  PHY layer of NR excludes a NR SL candidate resource for which the corresponding PSFCH occasion overlaps in time domain with LTE SL reserved resource with RSRP greater than a threshold γ1LTE when the resource is in a slot without a PSFCH channel or with RSRP greater than a threshold γ2LTE when the resource is in a slot with a PSFCH channel (Proposal 6),

- Alt 3: PHY layer of NR excludes a NR SL candidate resource for which the corresponding PSFCH occasion overlaps in time domain with LTE SL reserved resource with RSRP greater than a threshold γfeedbackLTE=-106, -136 dBm.

The results are shown in  Figure 7. We observed that for Alt. 3 based resource (re)selection, using a lower SL RSRP threshold for exclusion of resources based on the associated PSFCH collision with LTE reservations offers better protection to LTE SL at the expense of NR SL performance. As observed in Figure 7, a stricter γfeedbackLTE improves the 95% PRR range of LTE SL by 30 m, while reducing the NR SL 95% PRR range by 100 m. Moreover, what is interesting to note is that optimizations on Alt. 3 based on changing the value of γfeedbackLTE does not outperform the proposed Alt. 4. In fact, from Figure 7, we see that Alt. 4 provides the best performance balance between NR SL and LTE SL in a shared RP.

Observation 20: A SL RSRP threshold associated with the exclusion of a NR SL candidate resource based on the collision of the associated PSFCH occasion with LTE SL reservation provides mechanism to balance NR SL and LTE SL performance in a shared RP.

Observation 21: Alt. 4 outperforms Alt. 3 even when the value of the SL RSRP threshold for exclusion of resources based on PSFCH collision is optimized.

Observation 22: (Pre)configuring different initial SL RSRP threshold list for selecting single slot resources in NR SL slots with NR PSFCH (Alt. 4) provides better protection to LTE SL compared to using a single RSRP threshold of all resources (Alt. 3).

Figure 7: Performance of NR dynamic resource pool sharing mechanism comparing Alternative 3 with different thresholds for resource exclusion for PSFCH collision γfeedbackLTE and Alternative 4 for Step 6 resource exclusion of LTE SL reservation with step 5 based exclusion of transmit resource of the collocated LTE module.

Proposal 19: RAN 1 defines DRPS for NR SL and LTE SL based on (a) exclusion of resources for which transmission resource or corresponding PSFCH occasion overlaps with resource selected for transmission by collocated LTE SL module in step 5, and (b) exclusion of resources in step 6 for which transmission resource or corresponding PSFCH occasion overlaps with resource reserved by other LTE SL UE-s based on RSRP thresholds determined by Alt. 4.

# Conclusions

Observation 1: The LTE SL module of a Type A UE, based on the Rel. 16/17 in-device coexistence framework, shares the time and frequency location of the resources over which it has transmitted or received a SL packet or is scheduled to transmit or receive a SL packet.

Observation 2: The LTE SL module of a Type A UE, based on the Rel. 16/17 in-device coexistence framework, shares the priorities associated with the indicated time-frequency resources.

Observation 3: NR SL transmit-side resource exclusion procedure in Clause 8.1.4 of TS 38.214 is enhanced to reduce collisions between LTE SL PSSCH/PSCCH transmissions with NR SL feedback transmissions over PSFCH for DRPS.

Observation 4: NR SL resource (re)selection procedure for DRPS with LTE SL needs to minimize both NR SL PSCCH/PSSCH and LTE SL PSCCH/PSSCH collisions and NR SL PSFCH and LTE SL PSCCH/PSSCH collisions.

Observation 5: Excluding a resource from the NR SL set of available resources based on the corresponding PSFCH overlap with LTE SL reservations in time domain irrespective of the RSRP associated with the LTE SL reservation leads to over exclusion of NR SL resources.

Observation 6: Excluding NR SL transmit resources based on the corresponding time-domain PSFCH overlap with LTE SL reservations in Step 5 will lead to a large number of exclusions and subsequent cancellation of Step 5 based exclusion.

Observation 7: Deactivation of Step 5a to force NR SL to exclude resource leads to undefined behaviour for NR SL resource (re)selection.

Observation 8: Resource exclusion based on PSFCH overlap with LTE SL in time domain in Step 5 leads to no protection available for LTE SL collision with NR PSFCH.

Observation 9: Prioritizing NR SL PSSCH transmissions over NR SL slots with PSFCH aids LTE SL UE-s in the shared pool to avoid transmissions over NR SL PSFCH slots.

Observation 10: Mismatch in the timeline for LTE SL resource exclusion across UE-s in the system leads to a degradation in the performance of the DRPS scheme.

Observation 11: The value of Tstart should be selected such that the NR SL UE can exclude both the current reservation as well as a periodic repetition of the reservation at least P’rsvp_RX logical subframes in the future.

Observation 12: A Type A UE transmits NR SL and LTE SL in a TDM manner but can received simultaneously in the same channel.

Observation 13: Type A devices, which contains both NR SL and LTE SL modules, need to implement a mechanism to resolve inter-RAT Tx-Tx and Tx-Rx conflicts for co-channel coexistence.

Observation 14: Dropping of transmission and reception based on a priority associated with a RAT will severely degrade the reliability of the transmission over the lower priority RAT.

Observation 15: When NR SL operates with 30 kHz SCS in a channel shared with LTE SL, two successive NR slots overlap with one LTE SL subframe.

Observation 16: NR PSFCH resources, if present in the second of the NR SL slots overlapping with a LTE SL subframe, will collide with only the last PSSCH symbol of the LTE SL and the guard symbol.

Observation 17: NR SL PSFCH if present on the second NR SL slots overlapping with LTE SL subframes will minimize the impact of PSFCH collision with LTE SL.

Observation 18: NR SL exclusion of LTE SL reservations based on Alternative 4 provides greater protection to LTE SL transmissions compared to Alternative 3.

Observation 19: The proposed NR SL resource exclusion mechanism leads to a 40% to 50% reduction in the probability of LTE SL colliding with NR PSFCH.

Observation 20: A SL RSRP threshold associated with the exclusion of a NR SL candidate resource based on the collision of the associated PSFCH occasion with LTE SL reservation provides mechanism to balance NR SL and LTE SL performance in a shared RP.

Observation 21: Alt. 4 outperforms Alt. 3 even when the value of the SL RSRP threshold for exclusion of resources based on PSFCH collision is optimized.

Observation 22: (Pre)configuring different initial SL RSRP threshold list for selecting single slot resources in NR SL slots with NR PSFCH (Alt. 4) provides better protection to LTE SL compared to using a single RSRP threshold of all resources (Alt. 3).

Proposal 1: The LTE SL module of a Type A UE shares the received signal reference power (RSRP) measured over the resources over which an LTE SL transmission was received.

Proposal 2: NR SL module of a Type A UE excludes in Step 6 a resource from the set of NR SL transmit resources if an LTE SL reservation received with an SL RSRP greater than a threshold overlaps with the corresponding PSFCH occasion in time domain (Option 1).

Proposal 3: NR SL resource (re)selection algorithm prioritizes NR SL PSSCH transmissions on slots with PSFCH resources.

Proposal 4: The NR SL module of a Type A UE performs resource exclusion in step 6 of clause 8.1.4 of TS 38.214 based on LTE SL reservations using different RSRP thresholds for resources on slots without NR PSFCH and for resources on slots with NR PSFCH.

Proposal 5: SL RSRP threshold list are separately (pre)configured for dynamic resource pool sharing with a different SL RSRP threshold list (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH.

Proposal 6: A resource is excluded from the set of available NR SL transmission resource if either the resource or the slot with the corresponding feedback collides with an LTE reservation received with an RSRP above a threshold.

Proposal 7: For excluding NR SL candidate resources where the corresponding PSFCH transmission occasion overlap with LTE SL reserved resources by other LTE SL UE in time domain a list of SL RSRP is separately (pre)configured for resources on slots without NR PSFCH and for resources on slots with NR PSFCH.

Proposal 8: The NR SL module excludes LTE SL reservation based on the indicated P’rsvp_RX and using only the formula for Q defined in TS 38.214.

Proposal 9: RAN 1 to specify the timeline for NR SL module of a Type A UE to consider LTE SL resource reservations in its resource (re)selection procedure.

Proposal 10: The NR SL module uses the information up to the ending LTE SL subframe at n- T ms in the shared information from the LTE SL module, where T ≤ Tmax and is up to UE implementation following the in-device coexistence mechanism.

Proposal 11: RAN 1 defines the value of Tstart = 1100 ms.

Proposal 12: Type A UE prioritizes transmissions over reception for a Tx-Rx conflict and prioritizes LTE SL transmissions over NR SL transmissions for a Tx-Tx conflict.

Proposal 13: NR SL MAC may determine to transmit two separate TB or the same TB over both the first and the second slots overlapping with an LTE SL subframe.

Proposal 14: From RAN 1 perspective, it is up to UE implementation to re-transmit the same TB or transmit a different TB in the first and the second slots overlapping with an LTE SL subframe.

Proposal 15: A NR SL UE always avoids transmitting NR PSCCH/PSSCH only in the second NR SL slot overlapping with an LTE SL subframe.

Proposal 16: When different TBs are transmitted on the NR SL slots overlapping with the LTE SL subframe and the NR SL transmission in the first of the overlapping slots is dropped or reselected (due to re-evaluation or pre-emption), the resources for the NR SL transmission on the second of the overlapping slot is reselected.

Proposal 17: NR SL PSFCH resources are always in the second of the NR SL slots overlapping with LTE SL subframe.

Proposal 18: NR SL operating with 30 kHz SCS over a RP shared with LTE SL is configured with NPSSCHPSFCH = 4.

Proposal 19: RAN 1 defines DRPS for NR SL and LTE SL based on (a) exclusion of resources for which transmission resource or corresponding PSFCH occasion overlaps with resource selected for transmission by collocated LTE SL module in step 5, and (b) exclusion of resources in step 6 for which transmission resource or corresponding PSFCH occasion overlaps with resource reserved by other LTE SL UE-s based on RSRP thresholds determined by Alt. 4.

# References

- RP-230077, “WID revision: NR sidelink evolution,” OPPO, RAN #99, Rotterdam, Netherlands, 2023.

- TS 38.213, “5G NR: Physical layer procedures for control,” v17.2.0, 2022.

- TS 36.213, “Evolved Universal Terrestrial Radio Access (E-UTRA): Physical layer procedures,” v17.3.0, 2022.

- TS 38.214, “5G NR Physical layer procedures for date,” v17.2.0, 2022.

- R1-2304178, “FL Summary #5 for AI 9.4.2: Co-channel coexistence for LTE sidelink and NR sidelink,” Moderator (LG Electronics), RAN 112bis-e, e-Meeting, Apr. 17—26, 2023.

- R1-2302271, “LS on co-channel coexistence,” 3GPP TSG RAN WG1 #112bis-e, Apr. 2023.

# Appendix A

The simulation assumptions for the results presented in Section 5 are detailed in Table 1.

Table 1: Simulation Assumptions

| Sidelink Frequency | 6 GHz |
| --- | --- |
| Traffic model for NR SL | Aperiodic traffic: Inter-packet arrival time: 50 ms + Exponentially random variable with mean of 50 ms. Packet size: Uniformly distributed in the set {200, 400, 600, 800, 1000, 1200, 1400, 1600, 1800, 2000} bytes. Latency requirement: 50 ms. |
| Traffic model for LTE SL | Periodic Traffic: One 300-byte packet followed by four 190-byte packets. Packet generation periodicity: 100 ms |
| Simulation environment | Highway |
| UE drop and mobility | Highway: Option A (140Kmph) (as per TR 37.885) with 600 vehicles, corresponding to 800 and 900 devices total, over 20 Km. UE Dropping Model A [5] Inter-vehicle distance between adjacent devices in the same lane given as: max{2 meters, 4 sec * Exponential random variable with average of speed} |
| Number of Tx/Rx antenna elements | 1Tx/2Rx |
| Antenna model | Option 1 |
| Simulation bandwidth | 20 MHz |
| SCS | 15 kHz for both NR SL and LTE SL |
| Pathloss, shadowing, blocking and dual mobility models | Enabled (as per TR 37.885) |
| Number of retransmissions (NR) | 1 Initial Transmission + up to 3 HARQ retransmissions |
| Number of retransmissions (LTE) | 1 Initial + 1 Retransmission |
| Communication Mode (NR) | Groupcast Option 1 |
| Required Communication Range (NR) | 240 m for Groupcast Option 1 |