3GPP TSG RAN WG1 #113			R1-2305367

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:		9.16.1

Source: 	Qualcomm Incorporated

Title: 	UE features for NR MIMO evolution

Document for:	Discussion/Decision

- Introduction

This contribution includes the initial RAN1 UE features list for Rel-18 NR after RAN1 #112bis-e meeting.

- Unified TCI framework extension for multi-TRP

First of all, we have the following general views on the feature dependency

- UE supporting R18 mTRP unifed TCI does not need to support R17 sTRP unified TCI, since R18 mTRP also supports sTRP operation.

- R18 sDCI mTRP unified TCI and R18 multi-DCI mTRP unified TCI are two independent features

Based on above principles, at least the following basic FGs are required:

- Support of unified TCI for single DCI mTRP

  - Support joint DL/UL TCI for single DCI mTRP

    - Support two MAC-CE activated joint TCI states per CC

    - Support more than two MAC-CE activated joint TCI states per CC

  - Support separate DL/UL TCI for single DCI mTRP

    - Support two MAC-CE activated DL and/or UL TCI states per CC

    - Support more than two MAC-CE activated DL and/or UL TCI states per CC

  - Support per-set or per-resource based TCI association for AP CSI-RS

  - Support two default PDSCH/AP CSI-RS beams

  - Support [TCI selection field] in DCI format 1_1 and 1_2

  - Support PDSCH-CJT

  - Support common multi-CC TCI state ID update and activation

  - Indication/configuration of R18 TCI states for aperiodic CSI-RS, PDCCH, PDSCH, SRS by reusing the Rel-15/16 signaling/configuration design(s)

- Support of unified TCI for multi-DCI mTRP

  - Support joint DL/UL TCI for multi-DCI mTRP

    - Support two MAC-CE activated joint TCI states per CC

    - Support more than two MAC-CE activated joint TCI states per CC

  - Support separate DL/UL TCI for multi-DCI mTRP

    - Support two MAC-CE activated DL and/or UL TCI states per CC

    - Support more than two MAC-CE activated DL and/or UL TCI states per CC

  - Support two default PDSCH/AP CSI-RS beams

  - Support per-TRP BFR with unified TCI framework

  - Support common multi-CC TCI state ID update and activation

  - Indication/configuration of R18 TCI states for aperiodic CSI-RS, PDCCH, PDSCH, SRS by reusing the Rel-15/16 signaling/configuration design(s)

Based on the discussions above, we propose:

Proposal 1: R18 mTRP unifed TCI and R17 sTRP unified TCI are two independent features

Proposal 2: R18 sDCI mTRP unified TCI and R18 multi-DCI mTRP unified TCI are two independent features

Proposal 3: For R18 sDCI mTRP unified TCI, adopt the following FGs.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-1-1 | Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC | 1. Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) 2. The maximum number of configured joint TCI states per BWP per CC in a band 3. Maximum of two MAC-CE activated joint TCI states per CC in a band 4. MAC CE based TCI state indication for a single activated TCI codepoint 5. The maximum number of MAC-CE activated joint TCI states across all CC(s) in a band | At least one of 16-2b-1, 16-2b-2, 16-2b-3, 16-2b-4, 16-2b-5, 23-2-1, 23-3-1, 23-3-2, 23-3-3, 23-6-1, 23-6-2, [FGs for STxMP] | Yes |  |  | Per band | N/A | N/A | N/A | Component 2 candidate value {8, 12, 16, 24, 32, 48, 64, 128} Component 5 candidate value {2, 4, 6, 8, 16} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-1a | Unified TCI with joint DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC | 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) 2. The minimum beam application time in Y symbols per SCS 3. The maximum number of MAC-CE activated joint TCI states per CC in a band | 40-1-1 | Yes |  |  | Per band | N/A | N/A | N/A | Component 2 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336 } only can be indicated in FR2 Component 3 candidate values: {3, 4, 5, 6, 7, 8} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-2 | Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with single activated TCI codepoint per CC | 1. Separate DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) 2. The maximum number of configured DL TCI states per BWP per CC 3. The maximum number of configured UL TCI states per BWP per CC 4. The maximum number of MAC-CE activated DL TCI states per CC in a band 5. The maximum number of MAC-CE activated UL TCI states per CC in a band 6. MAC CE based TCI state indication for a single TCI codepoint 7. The maximum number of MAC-CE activated DL TCI states across all CC(s) in a band 8. The maximum number of MAC-CE activated UL TCI states across all CC(s) in a band | 40-1-1 | Yes |  |  | Per band | N/A | N/A | N/A | Component 2 candidate value {4, 8, 12, 16, 24, 32, 48, 64, 128} Component 3 candidate value {4, 8, 12, 16, 24, 32, 48, 64} Component 4 candidate value {1, 2} Component 5 candidate value {1, 2} Component 7 candidate value {1, 2, 4, 8, 16} Component 8 candidate value {1, 2, 4, 8, 16} Note: UE should support candidate value of 2 for at least one of Component 4 and 5 | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-2a | Unified TCI with separate DL/UL TCI update for single-DCI based multi-TRP with multiple activated TCI codepoints per CC | 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) 2. The minimum beam application time in Y symbols per SCS 3. The maximum number of MAC-CE activated DL TCI states per CC in a band 4. The maximum number of MAC-CE activated UL TCI states per CC in a band | 40-1-2 | Yes |  |  | Per band | N/A | N/A | N/A | Component 2 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336 } only can be indicated in FR2 Component 3 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} Component 4 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH Note: UE should support candidate value of at least 3 for at least one of Component 3 and 4 | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-3 | Two default beams for single-DCI based multi-TRP | Support of default QCL assumption with two TCI states | 40-1-1, 40-1-2 | Yes |  |  | Per band | N/A | N/A | FR2 only |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-4 | Association between joint/DL TCI state and AP CSI-RS based on per CSI-RS resource set for single-DCI based multi-TRP | Support of RRC configuration per CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the CSI-RS resource | 40-1-1, 40-1-2 | Yes |  |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-5 | Association between joint/DL TCI state and AP CSI-RS based on per CSI-RS resource for single-DCI based multi-TRP | Support of RRC configuration per CSI-RS resource to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the CSI-RS resource | 40-1-1, 40-1-2 | Yes |  |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-6 | [TCI selection field] for single DCI based multi-TRP | Support of [TCI selection] field in DCI format 1_1 and 1_2 | 40-1-1, 40-1-2 | Yes |  |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-7 | PDSCH-CJT for single DCI based multi-TRP | 1. Support of PDSCH-CJT for single DCI based multi-TRP 2. The maximum number of indicated joint TCI states for PDSCH-CJT per BWP per CC in a band | 40-1-1 | Yes |  |  | Per band | N/A | N/A | FR1 only | Component 2 candidate values: {1, 2} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-8 | Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP | Common multi-CC TCI state ID update and activation | 40-1-1, 40-1-2 | Yes |  |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-9 | Indication/configuration of R18 joint/DL TCI states for aperiodic CSI-RS, PDCCH, PDSCH for single-DCI based multi-TRP | Support of indication/configuration of R18 joint/DL TCI states for aperiodic CSI-RS, PDCCH, PDSCH (except for TRS) reusing the Rel-15/16 signaling/configuration design(s) | 40-1-1, 40-1-2 | Yes |  |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-10 | Indication/configuration of R18 joint/UL TCI states for SRS for single-DCI based multi-TRP | Support of indication/configuration of R17 TCI states for SRS (except for periodic/semi-persistent SRS for BM) reusing the Rel-15/16 signaling/configuration design(s) | 40-1-1, 40-1-2 | Yes |  |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

Proposal 4: For R18 mDCI mTRP unified TCI, adopt the following FGs.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-1-13 | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC | 1. Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) 2. The maximum number of configured joint TCI states per BWP per CC in a band 3. Maximum of two MAC-CE activated joint TCI states per CC in a band 4. MAC CE based TCI state indication for a single activated TCI codepoint per CORESETPoolIndex 5. The maximum number of MAC-CE activated joint TCI states across all CC(s) in a band | 16-2a, 23-4, [FGs for STxMP] | Yes |  |  | Per band | N/A | N/A | N/A | Component 2 candidate value {8, 12, 16, 24, 32, 48, 64, 128} Component 5 candidate value {2, 4, 8, 16} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-13a | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC | 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) 2. The minimum beam application time in Y symbols per SCS 3. The maximum number of MAC-CE activated joint TCI states per CC in a band | 40-1-13 | Yes |  |  | Per band | N/A | N/A | N/A | Component 2 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336 } only can be indicated in FR2 Component 3 candidate values: {3, 4, 5, 6, 7, 8} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-14 | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC | 1. Separate DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) 2. The maximum number of configured DL TCI states per BWP per CC 3. The maximum number of configured UL TCI states per BWP per CC 4. Maximum of two MAC-CE activated DL TCI states per CC in a band 5. The maximum number of MAC-CE activated UL TCI states per CC in a band 6. MAC CE based TCI state indication for a single activated TCI codepoint per CORESETPoolIndex 7. The maximum number of MAC-CE activated DL TCI states across all CC(s) in a band 8. The maximum number of MAC-CE activated UL TCI states across all CC(s) in a band | 40-1-13 | Yes |  |  | Per band | N/A | N/A | N/A | Component 2 candidate value {4, 8, 12, 16, 24, 32, 48, 64, 128} Component 3 candidate value {4, 8, 12, 16, 24, 32, 48, 64} Component 5 candidate value {1, 2} Component 7 candidate value {2, 4, 6, 8, 16} Component 8 candidate value {1, 2, 4, 6, 8, 16} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-14a | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC | 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) 2. The minimum beam application time in Y symbols per SCS 3. The maximum number of MAC-CE activated DL TCI states per CC in a band 4. The maximum number of MAC-CE activated UL TCI states per CC in a band | 40-1-14 | Yes |  |  | Per band | N/A | N/A | N/A | Component 2 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336 } only can be indicated in FR2 Component 3 candidate values: {2, 3, 4, 5, 6, 7, 8} Component 4 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH Note: UE should support candidate value of at least 3 for at least one of Component 3 and 4 | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-15 | Two default beams for multi-DCI based multi-TRP | Support of default QCL assumption with two TCI states | 40-1-13, 40-1-14 | Yes |  |  | Per band | N/A | N/A | FR2 only |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-16 | Per-TRP BFR with unified TCI framework for multi-DCI based multi-TRP | Support of per-TRP BFR with unified TCI framework | 40-1-13, 40-1-14 | Yes |  |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-17 | Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP | Common multi-CC TCI state ID update and activation | 40-1-13, 40-1-14 | Yes |  |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-18 | Indication/configuration of R18 joint/DL TCI states for aperiodic CSI-RS, PDCCH, PDSCH for multi-DCI based multi-TRP | Support of indication/configuration of R18 joint/DL TCI states for aperiodic CSI-RS, PDCCH, PDSCH (except for TRS) reusing the Rel-15/16 signaling/configuration design(s) | 40-1-13, 40-1-14 | Yes |  |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-19 | Indication/configuration of R18 joint/UL TCI states for SRS for multi-DCI based multi-TRP | Support of indication/configuration of R17 TCI states for SRS (except for periodic/semi-persistent SRS for BM) reusing the Rel-15/16 signaling/configuration design(s) | 40-1-13, 40-1-14 | Yes |  |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

- Two TAs for multi-DCI

Based on the existing agreements, at least the following basic FGs are required:

- Support of two TAGs in the same CC for intra-cell multi-DCI based multi-TRP

- Support of two TAGs in the same CC for inter-cell multi-DCI based multi-TRP

- Support of two TAGs in the same CC for Rel-18 multi-DCI based STxMP PUSCH+PUSCH

Furthermore, it is already agreed that “as an optional UE capability, Rx timing difference between the two DL reference timings can be assumed to be larger than CP length”. Hence, a separate FG is needed for that. Also, the max number of additional PRACH configurations should be a component for inter-cell case.

Based on the discussions above, we propose:

Proposal 5: For Rel-18 two TAs for multi-DCI, adopt the following FGs.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-2-1 | Two TAs for multi-DCI based mTRP – intra-cell | Support of two TAGs in a same CC for intra-cel multi-DCI based multi-TRP | 16-2a | Yes | N/A |  | FS | No | No | N/A | Note: Rx timing difference between the two DL reference timings associated with the two TAGs is within a CP | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-2-2 | Two TAs for multi-DCI based mTRP – inter-cell | 1. Support of two TAGs in a same CC inter-cel multi-DCI based multi-TRP 2. The maximum number of additional PRACH configurations corresponding to different additional PCIs | 23-4 | Yes | N/A |  | FS | No | No | N/A | Note: Rx timing difference between the two DL reference timings associated with the two TAGs is within a CP Component 2 candidate values: {1,2,…,7} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-2-3 | Two TAs for multi-DCI based STxMP PUSCH+PUSCH | Support of two TAGs in a same CC for Rel-18 multi-DCI based STxMP PUSCH+PUSCH | FFS | Yes | N/A |  | FS | No | No | N/A | Note: Rx timing difference between the two DL reference timings associated with the two TAGs is within a CP | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-2-4 | Rx timing larger than CP length for multi-DCI based mTRP | Maximum receive timing difference between the DL transmissions from two TRPs can exceed a CP for multi-DCI based mTRP operation | 16-2a | Yes | N/A |  | FSPC | No | No | N/A |  | Optional with capability signaling |

- CSI enhancement

Based on CSI agreements on Type-II-CJT, we propose:

Proposal 6: For Rel-18 Type-II-CJT CSI, adopt the following FGs.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-3-1a | Regular eType-II-CJT | Basic components: 1. {Max # of Tx ports in one resource set, Max # of resource sets, and total # of Tx ports} to support regular eType-II-CJT for R=1 2. Support of parameter combinations satisfying - N_TRP = 2, and - L_total <= 8, and - p_v != {1/8, 1/8, 1/16, 1/16} 3. Support of rank 1,2 | 2-35 | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource set: {8,...,128} - Max # resource sets: {1 to 16} - Max # total ports: {4 to 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1a-1 | Support of PMI sub-bands with R=2 | 1. {Max # of Tx ports in one resource set, Max # of resource sets, and total # of Tx ports} to support regular eType-II-CJT for R=1 | 40-3-1a | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource set: {8,...,128} - Max # resource sets: {1 to 16} - Max # total ports: {4 to 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1a-2a | Support of parameter combinations other than basic components | Support of parameter combinations satisfying N_TRP = {3,4} | 40-3-1a | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1a-2b | Support of parameter combinations other than basic components | Support of parameter combinations satisfying L_total > 8 | 40-3-1a | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1a-2c | Support of parameter combinations other than basic components | Support of parameter combinations satisfying p_v != {1/8,1/8, 1/16, 1/16} | 40-3-1a | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1a-3 | Support of rank 3,4 | Support of rank 3,4 | 40-3-1a | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1a-4 | Support of mode-1 FD-separate CB | Support of mode-2 FD-separate CB | 40-3-1a | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1a-4-1 | Support of fractional FD selection index offset | Support of fractional FD selection index offset | 40-3-1a-4 | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1a-5 | Number of CPUs | Counting of number of CPUs for regular eType-II-CJT CSI | 40-3-1a | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | FFS | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1a-6 | Active CSI-RS resources and ports in the presence of regular eType-II-CJT CSI | Counting of active CSI-RS resources and ports in the presence of regular eType-II-CJT CSI | 40-3-1a | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | FFS | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1a-7 | Timeline for regular eType-II-CJT CSI | Timeline for regular eType-II-CJT CSI | 40-3-1a | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | FFS | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1b | Port selection FeType-II-CJT | Basic components: 1. {Max # of Tx ports in one resource set, Max # of resource sets, and total # of Tx ports} to support port selection FeType-II-CJT for M=1 and R=1 2. Support rank 1,2 3. Support of parameter combinations satisfying - M=1, and - …(FFS) | 2-35 | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource set: {8,...,128} - Max # resources: {1 to 64} - Max # resource sets: {1 to 16} - Max # total ports: {4 to 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1b-1 | Support of M=2 and R=1 for port selection FeType-II-CJT | {Max # of Tx ports in one resource set, Max # of resource sets, and total # of Tx ports} to support regular eType-II-CJT for M=2 and R=1 | 40-3-1b | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource set: {8,...,128} - Max # resources: {1 to 64} - Max # resource sets: {1 to 16} - Max # total ports: {4 to 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1b-2 | Support of M=2 and R=2 for port selection FeType-II-CJT | {Max # of Tx ports in one resource set, Max # of resource sets, and total # of Tx ports} to support port selection FeType-II-CJT for M=2 and R=2 | 40-3-1b | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource set: {8,...,128} - Max # resources: {1 to 64} - Max # resource sets: {1 to 16} - Max # total ports: {4 to 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1b-3 | Support of rank 3,4 | Support of rank 3,4 | 40-3-1b | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1b-4 | Number of CPUs | Counting of number of CPUs for port selection FeType-II-CJT CSI | 40-3-1b | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | FFS | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1b-5 | Active CSI-RS resources and ports in the presence of port selection FeType-II-CJT CSI | Counting of active CSI-RS resources and ports in the presence of port selection FeType-II-CJT CSI | 40-3-1b | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | FFS | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1b-6 | Timeline for port selection FeType-II-CJT CSI | Timeline for port selection FeType-II-CJT CSI | 40-3-1b | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | FFS | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-x | Active CSI-RS resources and ports for mixed Type-II-CJT codebook types in any slot | 1. List of codebook combinations 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | 40-3-1b | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | Codebook 1 = {Type I SP, Type I MP} (Codebook 2, Codebook 3) = {(eType-II-CJT R=1, NULL), (eType-II-CJT R=2, NULL), (FeType-II-CJT PS R=1 M=1, NULL), (FeType-II-CJT PS R=1 M=2, NULL), (FeType-II-CJT PS R=2 M=2, NULL), (FFS CB combos…)} | Optional with capability signaling |

Based on CSI agreements on Type-II-Doppler, we propose:

Proposal 7: For Rel-18 Type-II-Doppler CSI, adopt the following FGs.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-3-2a | Regular eType-II-Doppler | Basic components: 1. {Max # of Tx ports in one resource, Max # of resources, and total # of Tx ports} to support regular eType-II-Doppler for R=1 2. Support of parameter combinations satisfying - L = {2,4} 3. Support of rank 1,2 4. Supported number of aperiodic CSI-RS occasions K=4 | 2-35 | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,...,64} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2a-1 | Support of PMI sub-bands with R=2 | 1. {Max # of Tx ports in one resource, Max # of resources, and total # of Tx ports} to support regular eType-II-Doppler for R=1 | 40-3-2a | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,...,64} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2a-2 | Support of parameter combinations other than basic components | Support of parameter combinations satisfying L = 6 | 40-3-2a | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2a-3 | Support of rank 3,4 | Support of rank 3,4 | 40-3-2a | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A |  | Optional with capability signaling |
| 41. NR_MIMO_evo_DL_UL | 40-3-2a-4 | Number of aperiodic CSI-RS occasions K>4 | Supported number of aperiodic CSI-RS occasions K>4 | 40-3-2a | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | Candidate values for K: {8,12} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2a-5 | Number of CPUs | Counting of number of CPUs for regular eType-II-Doppler CSI | 40-3-2a | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | FFS | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2a-6 | Active CSI-RS resources and ports in the presence of regular eType-II-Doppler CSI | Counting of active CSI-RS resources and ports in the presence of regular eType-II-Doppler CSI | 40-3-2a | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | FFS | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2a-7 | Timeline for regular eType-II-Doppler CSI | Timeline for regular eType-II-Doppler CSI | 40-3-2a | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | FFS | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1b | Port selection FeType-II-Doppler | Basic components: 1. {Max # of Tx ports in one resource set, Max # of resource sets, and total # of Tx ports} to support Port selection FeType-II-Doppler for M=1 and R=1 2. Support rank 1,2 3. Support of parameter combinations satisfying - M=1, and - …(FFS) 4. Support aperiodic CSI-RS with number of occasions K=4, and support periodic CSI-RS 5. Supported N4=1 | 2-35 | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,...,64} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1b-1 | Support of M=2 and R=1 for port selection FeType-II-Doppler | {Max # of Tx ports in one resource set, Max # of resource sets, and total # of Tx ports} to support Port selection FeType-II-Doppler for M=2 and R=1 | 40-3-1b | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,...,64} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1b-2 | Support of M=2 and R=2 for port selection FeType-II-Doppler | {Max # of Tx ports in one resource set, Max # of resource sets, and total # of Tx ports} to support Port selection FeType-II-Doppler for M=2 and R=2 | 40-3-1b | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,...,64} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1b-3 | Support of rank 3,4 | Support of rank 3,4 | 40-3-1b | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A |  | Optional with capability signaling |
| 41. NR_MIMO_evo_DL_UL | 40-3-2a-4 | Number of aperiodic CSI-RS occasions K>4 | Supported number of aperiodic CSI-RS occasions K>4 | 40-3-2a | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | Candidate values for K: {8,12} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1b-5 | Number of CPUs | Counting of number of CPUs for port selection FeType-II-Doppler CSI | 40-3-1b | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | FFS | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1b-6 | Active CSI-RS resources and ports in the presence of port selection FeType-II-Doppler CSI | Counting of active CSI-RS resources and ports in the presence of port selection FeType-II-Doppler CSI | 40-3-1b | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | FFS | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1b-7 | Timeline for port selection FeType-II-Doppler CSI | Timeline for port selection FeType-II-Doppler CSI | 40-3-1b | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | FFS | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-x | Active CSI-RS resources and ports for mixed Type-II-Doppler codebook types in any slot | 1. List of codebook combinations 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | 40-3-1b | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | Codebook 1 = {Type I SP, Type I MP} (Codebook 2, Codebook 3) = {(eType-II-Doppler R=1, NULL), (eType-II-Doppler R=2, NULL), (FeType-II-Doppler PS R=1 M=1, NULL), (FeType-II-Doppler PS R=1 M=2, NULL), (FeType-II-Doppler PS R=2 M=2, NULL), (FFS CB combos…)} | Optional with capability signaling |

Based on CSI agreements on TDCP, we propose:

Proposal 8: For Rel-18 TDCP CSI, adopt the following FGs.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-3-3 | TDCP | Basic components: 1. Support of Y=1 delay value for TDCP report 2. Support of delay<=D_basic symbols (FFS D_basic=[1] slot) 3. Support of amplitude report | 2-35 | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-1 | Number of delay values | Supported number (Y>1) of delay values | 40-3-3 | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | Candidate values: {2,3,4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-2 | Delay value | Supported delay value (>D_basic) | 40-3-3 | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | Candidate values: {[2],3,4,5,6,10} slots | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-3 | Phase report | Support of phase report | 40-3-3 | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-4 | Aperiodic CSI-RS resource | DCI triggered aperiodic CSI-RS resource /resource set | 40-3-3 | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-5 | Number of CSI-RS resources for TDCP | 1. Supported max # of configured CSI-RS resources for TDCP per CC 2. Supported max # of configured CSI-RS resources for TDCP across all CCs 3. Supported max # of CSI-RS resources for simultaneous TDCP measurement across all CCs | 40-3-3 | Yes | N/A |  | Per-band Per-BC | No | N/A | N/A | Candidate values for component 1: {1,...,Y+1} Candidate values for component 2: {2,4,8} Candidate values for component 3 <= value configured for componenet 2 | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-x | Resources for TDCP, beam management, pathloss measurement, BFD, RLM and new beam identification | 1. The maximum total number of SSB/CSI-RS/CSI-IM resources configured to measure simultaneously across all CCs in FR1 for any of TDCP measurement, L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification 2. The maximum total number of SSB/CSI-RS/CSI-IM resources configured across all CCs in FR1 for any of TDCP measurement, L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification | 16-1g, 40-3-3 | Yes | N/A |  | Per-band Per-BC | No | Yes | Yes | FR1-only Component-1: candidate value set is {2, 4, 8, 12, 16, 32, 64, 128} Component-2: candidate value set is {2, 4, 8, 12, 16, 32, 40, 48, 64, 72, 80, 96, 128, 256} | Optional with capability signaling |

- Increased number of orthogonal DMRS ports

For Rel-18 DMRS FGs, the structure of Rel-15 DMRS FGs should be reused as baseline. With this principle, the following proposal is made.

Proposal 9: For Rel-18 DMRS enhancement, adopt the following FGs.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-4-1 | Downlink DMRS for scheduling type A | 1. Support 1 symbol FL DMRS without additional symbol(s) 2. Support 1 symbol FL DMRS and 1 additional DMRS symbol 3. Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port. |  | YES | N/A |  | Per FS | NO | NO | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-2 | Downlink DMRS for scheduling type B | 1. Support 1 symbol FL DMRS without additional symbol(s) 2. Support 1 symbol FL DMRS and 1 additional DMRS symbol |  | YES | N/A |  | Per FS | NO | NO | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-3 | Support 1+2 DMRS (downlink) | 1. Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port |  | YES | N/A |  | Per FS | NO | NO | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-4 | Supported 2 symbols front-loaded DMRS(downlink) | 1. Support 2 symbols FL-DMRS |  | YES | N/A |  | Per FS | NO | NO | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-5 | Supported 2 symbols front-loaded +2 symbols additional DMRS(downlink) | 1. Support 2-symbol FL DMRS + one additional 2-symbols DMRS |  | YES | N/A |  | Per FS | NO | NO | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6 | Support 1+3 DMRS symbols(downlink) | 1. Support 1 symbol FL DMRS and 3 additional DMRS symbols |  | YES | N/A |  | Per FS | NO | NO | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-7 | Support DMRS type (downlink) | 1. Supported DMRS types |  | YES | N/A |  | Per FS | NO | NO | N/A | Candidate values for component 1 = {type 1, type 2, both type 1 and type 2} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-8 | Layers to CDM group mapping for 2 CWs PDSCH | 1. For 2CW PDSCH, support map layers of a CW to more than 1 DMRS CDM groups. |  | YES | N/A |  | Per FS | NO | NO | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-9 | Even RB restriction for type 1 DMRS | 1. Support type 1 DMRS with or without even RB restriction |  | YES | N/A |  | Per FS | NO | NO | N/A | Candidate values for component 1 = {Type 1 DMRS with even RB restrictions, Type 1 DMRS without even RB restrictions} Note: Even RB restrictions are the following. 1. The number of consecutively scheduled PRBs for PDSCH is even. 2. The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even. | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-10 | Even RB restriction for type 2 DMRS | 1. Support type 2 DMRS with or without even RB restriction |  | YES | N/A |  | Per FS | NO | NO | N/A | Candidate values for component 1 = {Type 2 DMRS with even RB restrictions, Type 2 DMRS without even RB restrictions} 1. The number of consecutively scheduled PRBs for PDSCH is even. 2. The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even. | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-11 | Support Rel-18 DL DMRS with S-DCI based M-TRP | 1. Support Rel-18 DL DMRS with S-DCI based M-TRP PDSCH |  | YES | N/A |  | Per FS | NO | NO | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-12 | Support new DMRS port entry {0,2,3} for S-DCI based SDM for M-TRP PDSCH | 1.Support DMRS port entry {0,2,3} for S-DCI based SDM for M-TRP PDSCH | 40-4-11 | YES | N/A |  | Per FS | NO | NO | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-13 | Support Rel-18 DL DMRS with M-DCI based M-TRP | 1. Support Rel-18 DL DMRS with M-DCI based M-TRP |  | YES | N/A |  | Per FS | NO | NO | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-14 | Rank >4 PUSCH for a Rel-18 UE configured with Rel-15 DMRS ports by RRC | 1. Support Rank >4 PUSCH for a Rel-18 UE configured with Rel-15 DMRS ports by RRC |  | YES | N/A |  | Per FS | NO | NO | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-15 | New DMRS port entry for SDM scheme for PUSCH | Support of new DMRS port entry {0,2,3} for single-DCI based SDM STxMP PUSCH with Rel-18 DMRS |  | YES | N/A |  | Per FS | NO | NO | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-16 | Uplink DMRS for scheduling type A | 1. Support 1 symbol FL DMRS without additional symbol(s) 2. Support 1 symbol FL DMRS and 1 additional DMRS symbols 3. Support 1 symbol FL DMRS and 2 additional DMRS symbols |  | YES | N/A |  | Per FS | NO | NO | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-17 | Uplink DMRS for scheduling type B | 1. Support 1 symbol FL DMRS without additional symbol(s) 2. Support 1 symbol FL DMRS and 1 additional DMRS symbol |  | YES | N/A |  | Per FS | NO | NO | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-18 | Support 1+2 DMRS (uplink) | 1. Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port |  | YES | N/A |  | Per FS | NO | NO | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-19 | Supported 2 symbols front-loaded DMRS (uplink) | 1. Support 2 symbols FL-DMRS |  | YES | N/A |  | Per FS | NO | NO | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-20 | Supported 2 symbols front-loaded +2 symbols additional DMRS (uplink) | 1. Support 2-symbol FL DMRS + one additional 2-symbols DMRS |  | YES | N/A |  | Per FS | NO | NO | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-21 | Support 1+3 uplink DMRS symbols(uplink) | 1. Support 1 symbol FL DMRS and 3 additional DMRS symbols |  | YES | N/A |  | Per FS | NO | NO | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-22 | Support DMRS type (uplink) | 1. Supported DMRS types |  | YES | N/A |  | Per FS | NO | NO | N/A | Candidate values for component 1 = {type 1, type 2, both type 1 and type 2} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-23 | PTRS enhancement for S-TRP based PUSCH with more than 4 layers | 1. Support 1 port PTRS for full coherent PUSCH with more than 4 layers 2. Support 2 port PTRS for partial coherent PUSCH with more than 4 layers 3. Support 2 port PTRS for non coherent PUSCH with more than 4 layers 4. Supported PTRS power boost configurations |  | YES | N/A |  | Per FS | NO | NO | N/A | Candidate values for component 4 = {ptrs-Power configures 00, ptrs-Power configures 01, both ptrs-Power configures 00 and 01} | Optional with capability signaling |

- SRS enhancement targeting TDD CJT and 8 TX operation

For TDD CJT, based on the existing agreements, at least the following basic FGs are required:

- Support of cyclic shift hopping for SRS

- Support of comb offset hopping for SRS

Furthermore, combination of the above schemes with legacy group/sequence hopping should be a dependent FG. For com offset hopping, as agreed during the WI, UE should be able to indicate the support of the granularity of hopping. Component 2 is added for this purpose.

Based on the discussions above, we propose:

Proposal 10: For Rel-18 SRS targeting TDD CJT, adopt the following FGs.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-5-1 | Cyclic shift hopping for SRS | Support of cyclic shift hopping for SRS |  | Yes | N/A |  | band | No | No | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-1a | Combination of cyclic shift hopping and group / sequence hopping | Support of cyclic shift hopping combined with one of group / sequence hopping on a SRS resource | 40-5-1 | Yes | N/A |  | band | No | No | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-2 | Comb offset hopping for SRS | 1. Support of comb offset hopping for SRS 2. Hopping granularily |  | Yes | N/A |  | band | No | No | N/A | Component 2 candidate values: {‘per SRS symbol’,’per R SRS symbols’, ‘both’} Note: In case of ’per R SRS symbols’, R is the repetitionFactor, and the comb offset is unchaged within the R symbols. | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-2a | Combination of comb offset hopping and group / sequence hopping | Support of comb offset hopping combined with one of group / sequence hopping on a SRS resource | 40-5-2 | Yes | N/A |  | band | No | No | N/A |  | Optional with capability signaling |

Given 8Tx is a very high-cost UE feature (i.e., a lot of Tx), it is likely a UE will implement this feature on one CC (or at most a few CCs) in UL CA. Therefore, per FSPC granularity is needed for the SRS sounding for 8 Tx PUSCH.

Proposal 11: For Rel-18 SRS targeting 8 Tx PUSCH, adopt the following FGs.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-5-3 | 8-ports SRS without TDM for antenna switch | 1. Support 8-ports SRS without TDM for antenna switch, without SRS repetition and without SRS frequency hopping 2. Support 8-ports SRS without TDM for antenna switch, with SRS repetition and without SRS frequency hopping 3. Support 8-ports SRS without TDM for antenna switch, without SRS repetition and with SRS frequency hopping 4. Support 8-ports SRS without TDM for antenna switch, with SRS repetition and with SRS frequency hopping 5. Supported repetition factor R 6. Supported number of SRS symbols m in one slot |  | Yes | N/A |  | Per FSPC | No | No | N/A | Candidate value for component 5 = a subset of {1, 2, 4, 5, 6, 7, 8, 10, 12, 14} Candidate value for component 6 = a subset {1, 2, 4, 8, 10, 12, 14} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-4 | 8-ports SRS without TDM for codebook PUSCH | 1. Support 8-ports SRS without TDM for codebook PUSCH, without SRS repetition and without SRS frequency hopping 2. Support 8-ports SRS without TDM for codebook PUSCH, with SRS repetition and without SRS frequency hopping 3. Support 8-ports SRS without TDM for codebook PUSCH, without SRS repetition and with SRS frequency hopping 4. Support 8-ports SRS without TDM for codebook PUSCH, with SRS repetition and with SRS frequency hopping 5. Supported repetition factor R 6. Supported number of SRS symbols m in one slot |  |  | N/A |  | Per FSPC |  |  |  | Candidate value for component 5 = a subset of {1, 2, 4, 5, 6, 7, 8, 10, 12, 14} Candidate value for component 6 = a subset {1, 2, 4, 8, 10, 12, 14} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-5 | 8-ports SRS with TDM for antenna switch | 1. Support 8-ports SRS with TDM for antenna switch, without SRS repetition and without SRS frequency hopping 2. Support 8-ports SRS with TDM for antenna switch, with SRS repetition and without SRS frequency hopping 3. Support 8-ports SRS with TDM for antenna switch, without SRS repetition and with SRS frequency hopping 4. Support 8-ports SRS with TDM for antenna switch, with SRS repetition and with SRS frequency hopping 5. Supported repetition factor R 6. Support number of SRS symbols m 7. Supported TDM factor s |  | Yes | N/A |  | Per FSPC | No | No | N/A | Candidate value for component 5 = a subset of {1, 2, 4, 5, 6, 7, 8, 10, 12, 14} Candidate value for component 6 = a subset {1, 2, 4, 8, 10, 12, 14} Candidate value for component 7 = {2,[4]} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-6 | 8-ports SRS with TDM for codebook PUSCH | 1. Support 8-ports SRS with TDM for codebook PUSCH, without SRS repetition and without SRS frequency hopping 2. Support 8-ports SRS with TDM for codebook PUSCH, with SRS repetition and without SRS frequency hopping 3. Support 8-ports SRS with TDM for codebook PUSCH, without SRS repetition and with SRS frequency hopping 4. Support 8-ports SRS with TDM for codebook PUSCH, with SRS repetition and with SRS frequency hopping 5. Supported repetition factor R 6. Support number of SRS symbols m 7. Supported TDM factor s |  | Yes | N/A |  | Per FSPC | No | No | N/A | Candidate value for component 5 = a subset of {1, 2, 4, 5, 6, 7, 8, 10, 12, 14} Candidate value for component 6 = a subset {1, 2, 4, 8, 10, 12, 14} Candidate value for component 7 = {2,[4]} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-7 | 8-ports SRS for noncodebook PUSCH | 1. Support 8-ports SRS for noncodebook PUSCH, without SRS repetition and without SRS frequency hopping 2. Support 8-ports SRS for noncodebook PUSCH, with SRS repetition and without SRS frequency hopping 3. Support 8-ports SRS for noncodebook PUSCH, without SRS repetition and with SRS frequency hopping 4. Support 8-ports SRS for noncodebook PUSCH, with SRS repetition and with SRS frequency hopping 5. Supported repetition factor R 6. Supported number of SRS symbols m in one slot 7. Supported max number of SRS resource per set (SRS set use is configured as for non-codebook transmission). 8. Maximum number of simultaneous transmitted SRS resources at one OFDM symbol |  | Yes | N/A |  | Per FSPC | No | No | N/A | Candidate value for component 5 = a subset of {1, 2, 4, 5, 6, 7, 8, 10, 12, 14} Candidate value for component 6 = a subset {1, 2, 4, 8, 10, 12, 14} Candidate value for component 7 = {1,2,3,4,5,6,7,8} Candidate value for component 8 = {1,2,4,8} | Optional with capability signaling |

- Simultaneous multi-panel transmission

For this Rel-18 enhacement (STxMP), at least the following basic FGs are required:

- Single-DCI based SDM scheme – codebook-based PUSCH

- Single-DCI based SDM scheme – non-codebook-based PUSCH

- Single-DCI based SFN scheme – codebook-based PUSCH

- Single-DCI based SFN scheme – non-codebook-based PUSCH

- Single-DCI based SFN scheme for PUCCH

- Multi-DCI based PUSCH+PUSCH – codebook-based PUSCH

- Multi-DCI based PUSCH+PUSCH – non-codebook-based PUSCH

For each of the basic FGs above, the details including components/type/other dependent FGs should be similar to the related Rel-15 FGs 2-14 / 2-15 / 2-15a / 2-15b as well as the related Rel-17 FGs 23-3-1 / 23-3-1-2 / 23-3-1-2a / 23-3-1-2b / 23-3-2 / 23-3-3. Furthermore, for multi-DCI based STxMP PUSCH, similar structure of Rel-16 FG 16-2a can be considered wrt overlap in time/freq and max number of PUSCHs per slot. Given these, we propose the following:

Proposal 12: For Rel-18 STxMP, adopt the following FGs.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-6-1 | Single-DCI based SDM STxMP -codebook based PUSCH | 1. Support of single-DCI based SDM STxMP for codebook-based PUSCH 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set 4. Supported number of layers associated with one SRS resource set for SDM scheme 5. Assumption on PUSCH ports | 2-14 | Yes | N/A |  | Per FSPC | No | No | N/A | Component 3 candidate values: {1,2,4} Note: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c. Component 4 candidate values: {1,2} Component 5 candidate values: {‘separate’, ‘shared’, ‘both’} Note: ‘separate’ means the max number of PUSCH ports for SDM scheme is twice the max number of PUSCH ports for sTRP. ‘shared’ means the max number of PUSCH ports for SDM scheme is the same as the max number of PUSCH ports for sTRP. | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-2 | Single-DCI based SDM STxMP -non-codebook based PUSCH | 1. Support of single-DCI based SDM STxMP for non-codebook-based PUSCH 2. Support of two SRS resource sets with usage set to 'nonCodebook' 3. Supported number of SRS resources in one SRS resource set 4. Supported number of layers associated with one SRS resource set for SDM scheme | 2-15 | Yes | N/A |  | Per FSPC | No | No | N/A | Component 3 candidate values: {1,2,4} Component 4 candidate values: {1,2} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-2a | Associated CSI-RS resources for SDM scheme | 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook-based SDM scheme 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously | 2-15a, 40-6-2 | Yes | N/A |  | Per band | No | No | N/A | Component 2: {1 to 8} Component 3: {1 to 8} Component 4: {0 to 8} Component 5: {1 to 16} Component 6: {1,2} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3 | New DMRS port entry for SDM scheme | Support of new DMRS port entry {0,2,3} for single-DCI based SDM STxMP PUSCH | 40-6-1 or 40-6-2 | Yes | N/A |  | Per band | No | No | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-4 | 2 PTRS ports for SDM scheme | Support of 2 PTRS ports for single-DCI based SDM STxMP PUSCH | 40-6-1 or 40-6-2 | Yes | N/A |  | Per band | No | No | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-5 | Single-DCI based SFN STxMP -codebook based PUSCH | 1. Support of single-DCI based SFN STxMP for codebook-based PUSCH 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set 4. Supported number of layers for SFN scheme 5. Assumption on PUSCH ports | 2-14 | Yes | N/A |  | Per FSPC | No | No | N/A | Component 3 candidate values: {1,2,4} Note: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c. Component 4 candidate values: {1,2,4} Component 5 candidate values: {‘separate’, ‘shared’, ‘both’} Note: ‘separate’ means the max number of PUSCH ports for SFN scheme is twice the max number of PUSCH ports for sTRP. ‘shared’ means the max number of PUSCH ports for SFN scheme is the same as the max number of PUSCH ports for sTRP. | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-6 | Single-DCI based SFN STxMP -non-codebook based PUSCH | 1. Support of single-DCI based SFN STxMP for non-codebook-based PUSCH 2. Support of two SRS resource sets with usage set to 'nonCodebook' 3. Supported number of SRS resources in one SRS resource set 4. Supported number of layers for SFN scheme | 2-15 | Yes | N/A |  | Per FSPC | No | No | N/A | Component 3 candidate values: {1,2,4} Component 4 candidate values: {1,2,4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-6a | Associated CSI-RS resources for SFN scheme | 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook-based SFN scheme 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously | 2-15a, 40-6-6 | Yes | N/A |  | Per band | No | No | N/A | Component 2: {1 to 8} Component 3: {1 to 8} Component 4: {0 to 8} Component 5: {1 to 16} Component 6: {1,2} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-7 | Single-DCI based SFN scheme for PUCCH | 1. Support of Single-DCI based SFN STxMP scheme for PUCCH 2. Supported PUCCH formats for this scheme |  | Yes | N/A |  | Per FS | No | No | N/A | Component 2 candidate values: {PF0/2, PF1/3/4, PF0-4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-8 | Multi-DCI based STxMP - codebook based PUSCH | 1. Support of multi-DCI based STxMP PUSCH+PUSCH for codebook-based PUSCH: Support fully-overlapping PUSCHs in time and non-overlapping in frequency 2. Support of two SRS resource sets with usage set to 'codebook' associated with two coresetPoolInde values 3. Supported number of SRS resources in one SRS resource set 4. Supported number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain 5. Supported number of used PUSCH ports for each PUSCH of PUSCH+PUSCH overlapping in time domain 6. Maximum number of PUSCHs per CORESETPoolIndex per slot | 2-14 | Yes | N/A |  | Per FSPC | No | No | N/A | Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,4} Note: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c. Component 4 candidate values: {1,2} Component 5 candidate values: {1,2,4} Note: If a row of the TPMI consists of all 0’s, the corresponding PUSCH port is not used. Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-9 | Multi-DCI based STxMP -non-codebook based PUSCH | 1. Support of multi-DCI based STxMP PUSCH+PUSCH for non-codebook-based PUSCH: Support fully overlapping PUSCHs in time and non-overlapping in frequency 2. Support of two SRS resource sets with usage set to 'nonCodebook' associated with two coresetPoolInde values 3. Supported number of SRS resources in one SRS resource set 4. Supported number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain 5. Maximum number of PUSCHs per CORESETPoolIndex per slot | 2-15 | Yes | N/A |  | Per FSPC | No | No | N/A | Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,4} Component 4 candidate values: {1,2} Component 5 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-9a | Associated CSI-RS resources for multi-DCI based STxMP | 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook-based multi-DCI based STxMP 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously | 2-15a, 40-6-9 | Yes | N/A |  | Per band | No | No | N/A | Component 2: {1 to 8} Component 3: {1 to 8} Component 4: {0 to 8} Component 5: {1 to 16} Component 6: {1,2} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-10 | Partial-overlapping PUSCHs in time and non-overlapping in frequency | Support partial-overlapping PUSCHs in time and non-overlapping in frequency | 40-6-8 or 40-6-9 | Yes | N/A |  | Per band | No | No | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-11 | Overlapping PUSCHs in time and fully overlapping in frequency and time | Support PUSCHs with fully overlapping REs, i.e. the allocated REs for PUSCH associated with CORESETPoolIndex = 0 and PUSCH associated with CORESETPoolIndex = 1 are exactly the same REs | 40-6-8 or 40-6-9 | Yes | N/A |  | Per band | No | No | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-12 | Overlapping PUSCHs in time and partially overlapping in frequency | Support PUSCHs with partially overlapping REs, i.e. the allocated REs for PUSCH associated with CORESETPoolIndex = 0 and PUSCH associated with CORESETPoolIndex = 1 are partially overlapped, with at least one RE | 40-6-8 or 40-6-9 | Yes | N/A |  | Per band | No | No | N/A |  | Optional with capability signaling |

- SRI/TPMI for 8 TX UL transmission

Given 8Tx is a very high-cost UE feature (i.e., a lot of Tx), it is likely a UE will implement this feature on one CC (or at most a few CCs) in UL CA. Therefore, per FSPC granularity is needed for 8 Tx PUSCH.

Proposal 13: For Rel-18 8 TX PUSCH, adopt the following FGs.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-7-1 | Codebook based PUSCH | 1.Supported maximal PUSCH MIMO layers for codebook based PUSCH |  | YES | N/A |  | Per FSPC | NO | NO | NO | Component 1 candidate values: {1,2,3,4,5,6,7,8} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-2 | Noncodebook based PUSCH | 1.Supported maximal PUSCH MIMO layers for noncodebook based PUSCH |  | YES | N/A |  | Per FSPC | NO | NO | NO | Component 1 candidate values: {1,2,3,4,5,6,7,8} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-3 | CBG based 2 CWs PUSCH | 1.Support CBG based transmission for 2 CWs PUSCH |  | YES | N/A |  | Per FSPC | NO | NO | NO |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-4 | s | 1.Support 2 CWs type 1 CG-PUSCH |  | YES | N/A |  | Per FSPC | NO | NO | NO |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-5 | 2 CWs CG-PUSCH type 2 | 1. Support 2 CWs type 2 CG-PUSCH |  | YES | N/A |  | Per FSPC | NO | NO | NO |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-6 | Full power mode 0 for 8 Tx PUSCH | 1. Support full power mode 0 for 8 Tx PUSCH |  | YES | N/A |  | Per FSPC | NO | NO | NO |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-7 | Full power mode 1 for 8 Tx PUSCH | 1. Support full power mode 1 for 8 Tx PUSCH |  | YES | N/A |  | Per FSPC | NO | NO | NO |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-8 | Full power mode 2 for 8 Tx PUSCH | 1. Support full power mode 2 for 8 Tx PUSCH FFS how to indicate full power precoders under full power mode 2. |  | YES | N/A |  | Per FSPC | NO | NO | NO |  | Optional with capability signaling |