3GPP TSG RAN WG1 #113			R1-2305368

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.16.2

Source: 	Qualcomm Incorporated

Title: 	UE features on Expanded and improved NR Positioning

Document for:		Discussion and Decision

# Introduction

This contribution includes our first set of UE features and components for the WI Expanded and improved NR Positioning.

Proposal 1: Use the UE features shown in Appendix  as the starting point for  the UE feature discussion on  Expanded and Improved Positioning.

# 3 Conclusions

We make the following proposal:

Proposal 1: Use the UE features shown in Appendix  as the starting point for  the UE feature discussion on  Expanded and Improved Positioning.

# 4 Appendix: NR_pos_enh2

#### 4.1 SL Positioning

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | - Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_pos_enh2 | X-SLPRS-1 | Support of SL-PRS reception & processing in shared resource pool | 1. Maximum SL PRS bandwidth in MHz in a shared resource pool for positioning, which is supported and reported by UE for SL-PRS measurement 2. SL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 3. Duration of SL PRS symbols N in units of ms a UE can process every T ms assuming maximum SL PRS bandwidth in MHz, which is supported and reported by UE 4. Max number of SL PRS resources that UE can process in a slot |  | [TBD] | Yes |  | per FS | n/a | n/a | n/a | Component 1 candidate values: FR1 bands: {5, 10, 20, 40, 50, 80, 100} FR2 bands: {50, 100, 200, 400} Component 2 candidate values: {Type 1, Type 2} Component 3 candidate values: a) T: {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 4 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Notes for component 3: a. UE is not expected to support SL PRS bandwidth that exceeds the reported SL PRS bandwidth value b.UE SL PRS processing capability is agnostic to SL PRS comb factor configuration c.The reporting of (N, T) values for maximum BW in MHz is not dependent on SCS Need for location server / server UE to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-SLPRS-2 | Support of SL-PRS reception & processing in dedicated resource pool | 1. Maximum SL PRS bandwidth in MHz in a dedicated resource pool for positioning, which is supported and reported by UE for SL-PRS measurement 2. SL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 3. Duration of SL PRS symbols N in units of ms a UE can process every T ms assuming maximum SL PRS bandwidth in MHz, which is supported and reported by UE 4. Max number of SL PRS resources that UE can process in a slot 5. Support receiving a single SL-PRS resource in a slot in the allocated BW |  | [TBD] | Yes |  | per FS | n/a | n/a | n/a | Component 1 candidate values: FR1 bands: {5, 10, 20, 40, 50, 80, 100} FR2 bands: {50, 100, 200, 400} Component 2 candidate values: {Type 1, Type 2} Component 3 candidate values: a) T: {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 4 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Notes for component 3: a. UE is not expected to support SL PRS bandwidth that exceeds the reported SL PRS bandwidth value b.UE SL PRS processing capability is agnostic to SL PRS comb factor configuration c.The reporting of (N, T) values for maximum BW in MHz is not dependent on SCS Need for location server / server UE to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-SLPRS-3 | TDM-based multiplexing of SL PRS from different UEs in a dedicated resource pools. | Support of Transmit and receive TDM-based multiplexing of multiple SL PRS resources in a slot in a dedicated resource pools. |  | Yes | Yes |  | per FS | n/a | n/a | n/a | Need for location server / server UE to know if the feature is supported | Optional with capability signaling |
|  | X-SLPRS-4 | Comb-based multiplexing of SL-PRS from different UEs in a dedicated resource pool | Support of Transmit and receive comb-based multiplexing of SL PRS from different UEs in a slot in a dedicated resource pools. |  | Yes | Yes |  | Per FS |  |  |  | Need for location server / server UE to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-SLPRS-5 | Transmission of SL-PRS in shared resource pool using Scheme 1 RA | - Support of SL-PRS transmission in a shared resource pool - Max number of SL-PRS Resources - Max number of SL-PRS Resources per slot - Support downlink pathloss based open loop power control |  | Yes | Yes |  | Per band | n/a | n/a | n/a | Need for location server / server UE to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-SLPRS-6 | Transmission of SL-PRS in shared resource pool using Scheme 2 RA with full sensing | - Support of SL-PRS transmission in a shared resource pool - Max number of SL-PRS Resources - Max number of SL-PRS Resources per slot - Support DL pathloss based open loop power control when configured by NR Uu |  | Yes | Yes |  | Per FS | n/a | n/a | n/a | Need for location server / server UE to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-SLPRS-7 | Transmission of SL-PRS in shared resource pool using Scheme 2 RA with partial sensing | - Support of SL-PRS transmission in a shared resource pool - Max number of SL-PRS Resources - Max number of SL-PRS Resources per slot - Support DL pathloss based open loop power control when configured by NR Uu |  | Yes | Yes |  | Per band | n/a | n/a | n/a | Need for location server / server UE to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-SLPRS-8 | Transmission of SL-PRS in shared resource pool using Scheme 2 RA with random selection | - Support of SL-PRS transmission in a shared resource pool - Max number of SL-PRS Resources - Max number of SL-PRS Resources per slot - Support DL pathloss based open loop power control when configured by NR Uu |  | Yes | Yes |  | Per band | n/a | n/a | n/a | Need for location server / server UE to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-SLPRS-9 | Transmission of SL-PRS in dedicated resource pool using Scheme 1 RA | - Support of SL-PRS transmission in a shared resource pool - Max number of SL-PRS Resources - Max number of SL-PRS Resources per slot - Support downlink pathloss based open loop power control for SL-PRS resource allocation scheme 1 - Support DL pathloss based open loop power control when SL-PRS resource allocation scheme 2 is configured by NR Uu |  | Yes | Yes |  | Per band | n/a | n/a | n/a | Need for location server / server UE to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-SLPRS-10 | Transmission of SL-PRS in dedicated resource pool using Scheme 2 RA with full sensing | - Support of SL-PRS transmission in a shared resource pool - Max number of SL-PRS Resources - Max number of SL-PRS Resources per slot - Support DL pathloss based open loop power control when SL-PRS resource allocation scheme 2 is configured by NR Uu |  | Yes | Yes |  | Per FS | n/a | n/a | n/a | Need for location server / server UE to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-SLPRS-11 | Transmission of SL-PRS in dedicated resource pool using Scheme 2 RA with random selection | - Support of SL-PRS transmission in a shared resource pool - Max number of SL-PRS Resources - Max number of SL-PRS Resources per slot - Support DL pathloss based open loop power control when SL-PRS resource allocation scheme 2 is configured by NR Uu |  | Yes | Yes |  | Per band | n/a | n/a | n/a | Need for location server / server UE to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-SLPRS-12 | SL UE Rx-Tx measurement Report for SL-RTT | 1.Support measuring and reporting SL UE Rx-Tx measurements for a UE |  | No | Yes | SL UE Rx-Tx measurement and report is not supported | Per band | n/a | n/a | n/a | Need for location server / server UE to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-SLPRS-13 | SL UE RSTD measurement Report for SL-TDOA | 1.Support measuring and reporting SL RSTD measurements for a pair of UEs |  | No | Yes |  | Per band | n/a | n/a | n/a | Need for location server / server UE to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-SLPRS-14 | SL UE Azimuth of arrival measurement Report for SL-AoA | 1.Support measuring and reporting SL UE Azimuth of arrival (AoA) measurements for a UE |  | No | Yes |  | Per band | n/a | n/a | n/a | Need for location server / server UE to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-SLPRS-15 | SL UE Zenith of arrival measurement Report for SL-AoA | 1.Support measuring and reporting SL UE Zenoth of arrival (ZoA) measurements for a UE |  | No | Yes |  | Per band | n/a | n/a | n/a | Need for location server / server UE to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-SLPRS-16 | Additional path reporting for SL-TDOA measurement report | 1. Support of additional detected path timing reporting for K additional paths for SL-TDOA 2. Support of RSRPP reporting for additional paths |  | No | Yes | Additional path reporting for SL-TDOA is not supported | Per band | No | No | No | Component 1 candidate values: {1, 2, 4, 6, 8} Need for location server / server UE to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-SLPRS-17 | Additional path reporting for SL-RTT | 1. Support of additional detected path timing reporting for K additional paths for SL-RTT 2. Support of RSRPP reporting for additional paths |  | No | Yes | Additional path reporting for SL-RTT is not supported | Per band | No | No | No | Component 1 candidate values: {1, 2, 4, 6, 8} Need for location server / server UE to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-SLPRS-18 | LOS/NLOS Indicator reporting | Support reporting LoS/NLoS indicator type |  | No | Yes | LOS/NLOS Indicator reporting is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {hard value, hard+soft value} Need for location server / server UE to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-SLPRS-19 | First path RSRPP reporting in SL for SL-TDOA measurement report | Support of SL-RSRPP reporting for first path for SL-TDOA |  | No | Yes |  | per band | n/a | n/a | n/a | Need for location server / server UE to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-SLPRS-20 | First path RSRPP reporting in SL for SL-RTT measurement report | Support of SL-RSRPP reporting for first path for SL-RTT |  | No | Yes |  | per band | n/a | n/a | n/a | Need for location server / server UE to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-SLPRS-21 | SL-PRS congestion control in dedicated RP | 1) UE can report CBR measurement to gNB when operating in Mode 1 and mode 2 2) UE can adjust its radio parameters based on CBR measurement and CRlimit. 3) UE can process CBR and CR within the time it indicates |  | Yes | No |  | Per band | n/a | n/a | n/a | Component-3 candidate value set {Congestion process time 1, Congestion process time 2} where Congestion process time 1: 2, 2, 4, 8 slots for 15, 30, 60, 120 kHz subcarrier spacing. Congestion process time 2: 2, 4, 8, 16 slots for 15, 30, 60, 120 kHz subcarrier spacing | Optional with capability signalling . |
| NR_pos_enh2 | X-SLPRS-22 | SL-PRS congestion control in shared RP | 1) UE can report CBR measurement to gNB when operating in Mode 1 and mode 2 2) UE can adjust its radio parameters based on CBR measurement and CRlimit. 3) UE can process CBR and CR within the time it indicates |  | Yes | Yes |  | Per band | n/a | n/a | n/a | Component-3 candidate value set {Congestion process time 1, Congestion process time 2} where Congestion process time 1: 2, 2, 4, 8 slots for 15, 30, 60, 120 kHz subcarrier spacing. Congestion process time 2: 2, 4, 8, 16 slots for 15, 30, 60, 120 kHz subcarrier spacing | Optional with capability signalling . |
| NR_pos_enh2 | X-SLPRS-23 | Support of open loop SL power control and RSRP report in shared RP | - Support sidelink pathloss based open loop power control for SL-PRS and RSRP reporting |  | Yes | Yes |  | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| NR_pos_enh2 | X-SLPRS-24 | Support of open loop SL power control and RSRP report in dedicated RP | - Support sidelink pathloss based open loop power control for SL-PRS and RSRP reporting |  | Yes | Yes |  | Per band | n/a | n/a | n/a |  | Optional with capability signaling |

#### 4.2 Carrier Phase Positioning

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | - Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_pos_enh2 | X-CPP-1a | DL PRS Carrier Phase measurement in RRC_CONNECTED within a MG | 1. Support of carrier phase measurements in RRC_CONNECTED state within a MG | [13-1] | No |  |  | - Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported Note: UE supporting this feature shall support the measurement of at least one RSCP or RSCPD | Optional with capability signaling |
| NR_pos_enh2 | X-CPP-1b | DL PRS Carrier Phase measurement in RRC_INACTIVE | 1. Support of carrier phase measurements in RRC_ INACTIVE state | [13-1] | No |  |  | - Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported Note: UE supporting this feature shall support the measurement of at least one RSCP or RSCPD | Optional with capability signaling |
| NR_pos_enh2 | X-CPP-2 | DL PRS RSCP measurement report of the first path for RTT | 1. Support of reporting the DL PRS RSCP of the first path for RTT positioning method 2. The maximum number of first path PRS RSCP measurements per TRP | [13-1,13-4] | No |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported Component 2 candidate values: {1, 2, 3, 4} | Optional with capability signaling |
| NR_pos_enh2 | X-CPP-3 | DL PRS RSCPD measurement report of the first path for UE-assisted DL-TDOA | 1. Support of measuring and reporting the DL PRS RSCPD of the first path for UE-assisted DL-TDOA positioning method 2. The maximum number of first path PRS RSCPD measurements per pair of TRPs | [13-1, 13-3] | No |  |  | Per band | No | Yes | N/A | Need for location server to know if the feature is supported Component 2 candidate values: {1, 2, 3, 4} | Optional with capability signaling |
| NR_pos_enh2 | X-CPP-4 | Assistance data for UE-based Carrier Phase Positioning | Support the assistance data enhancements for UE-based Carrier Phase Positioning | [13-1] | No |  |  | Per UE | N/A | N/A | N/A | Need for location server to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-CPP-5a | Support of Rx Phase Error Group reporting for carrier phase measurements | 1. Support of Rx PEG IDs for UE-assisted DL TDOA and/or Multi-RTT positioning 2. The maximum number of UE-RxPEG, which is supported and reported by UE for carrier phase measurements for UE assisted DL TDOA and/or Multi-RTT positioning | [13-1, X-CPP-2] | No |  |  | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported Component 2 candidate values: {1, 2, 3, 4, 6, 8, 16, 32} | Optional with capability signaling |
| NR_pos_enh2 | X-CPP-5b | Support of Tx Phase Error Group reporting for carrier phase measurements for UL TDOA | 1. Support of UE TxPEG IDs for UL-TDOA 2. The maximum number of UE TxPEG, which is supported and reported by UE for carrier phase measurements for UL-TDOA | [13-1, X-CPP-2] | Yes |  |  | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported Component 2 candidate values: {1, 2, 3, 4, 6, 8, 16, 32} | Optional with capability signaling |
| NR_pos_enh2 | X-CPP-5b | Support of Tx Phase Error Group reporting for carrier phase measurements for Multi-RTT | 1. Support of UE TxPEG IDs for Multi-RTT positioning 2. The maximum number of UE TxPEG, which is supported and reported by UE for carrier phase measurements for Multi-RTT positioning | [13-1, X-CPP-2] | No |  |  | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported Component 2 candidate values: {1, 2, 3, 4, 6, 8, 16, 32} | Optional with capability signaling |
| NR_pos_enh2 | X-CPP-5c | Support of PRS PEG association information for UE-based DL-TDOA | Support of reception of association between PRS and TRP Tx PEG for UE-based positioning |  | No |  |  | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-5 | Support of Time Window(s) indication for performing measurement | Support performing measurements on [indicated] DL PRS resources occurring within indicated time window(s). | [13-1] | No |  |  | Per UE | N/A | N/A | N/A | Need for location server to know if the feature is supported | Optional with capability signaling |

#### 4.3 LPHAP Positioning

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | - Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_pos_enh2 | X-LPHAP-1 | Area-specific positioning SRS transmission in RRC_INACTIVE state for initial UL BWP | Support of area-specific positioning SRS transmission in RRC_INACTIVE state for initial UL BWP |  |  |  | Area-specific Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-LPHAP-2 | Area-specific Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP | Support of Area-specific Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP | 27-15 | Yes |  | Area-specific Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-LPHAP-3 | PathLoss estimate maintenance across all cells for area-specific SRS transmission in RRC_INACTIVE | - Max number of pathloss estimates that the UE can simultaneously maintain for all the area-specific SRS resource sets for positioning - Candidate values are {1, 4, 8, 16} |  | Yes | N/A |  | Per UE | No | No | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| NR_pos_enh2 | X-LPHAP-4 | Spatial relation maintenance across all cells for area-specific SRS transmission in RRC_INACTIVE | - Max Number of maintained spatial relations for all the area-specific SRS resource sets for positioning across all serving cells. Values = {0,1,2,4,8,16} |  | Yes | N/A |  | Per UE | No | No (FR2 only) | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| NR_pos_enh2 | X-LPHAP-5 | UE autonomous UL timing determinaton for area-specific SRS for Positioning in RRC_INACTIVE | Support the UE to autonomously adjust the TA such that the UL timing remains constant with respect to the UL timing before reselecting to the new camping cell, when the DL reference timing difference between the last camping cell and the new camping cell exceeds a threshold. |  | Yes | N/A |  | Per UE | No | No | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| NR_pos_enh2 | X-LPHAP-6 | Preconfiguration of multiple SRS for positioning | Maximum number of SRS for Positioning pre-configurations for RRC_INACTIVE state |  | Yes | N/A |  | Per UE | No | No | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| NR_pos_enh2 | X-LPHAP-7 | PRS processing in RRC_IDLE | Support of PRS processing in RRC_IDLE | 13-1 | Yes |  | PRS processing in RRC_INACTIVE is not supported | per band | n/a | n/a | n/a | Note: UE supporting this feature shall support at least one from DL RSTD, DL PRS-RSRP, or UE Rx – Tx time difference measurement | Optional with capability signaling. |
| NR_pos_enh2 | X-LPHAP-8 | DL PRS processing capabilities in RRC_IDLE state | 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE 3. Max number of DL PRS resources that UE can process in a slot |  | No |  | DL PRS processing in RRC inactive state is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {Type 1, Type 2} Component 2 candidate values: T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Need for location server to know if the feature is supported Note: Having the PRS processing capabilities in RRC_Idle state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling |
| NR_pos_enh2 | X-LPHAP-9 | Support of PRS measurement in RRC_IDLE state for DL-TDOA | Support of PRS measurement in RRC_IDLE state for DL-TDOA - location server | 13-3, 27-6 | No |  | PRS measurement in RRC_IDLE state for DL-TDOA is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: Applicable for both UE-assisted and UE-based DL-TDOA Note: PRS capabilities for DL-TDOA measurement and reporting described in FGs in 13-3, 13-3a, 13-3b, 13-6, 13-13 are the same for RRC Inactive. Support of PRS processing measurement in RRC_IDLE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling |
| NR_pos_enh2 | X-LPHAP-10 | Support of PRS measurement in RRC_IDLE state for DL-AoD | Support of PRS measurement in RRC_IDLE state for DL-AoD - location server | 13-2, 27-6 | No |  | PRS measurement in RRC_IDLE state for DL-AoD is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: Applicable for both UE-assisted and UE-based DL-AoD Note: PRS capabilities for DL-AOD measurement and reporting described in FGs 13-2, 13-2a, 13-2b, 13-5, 13-13 are the same for RRC Inactive. Support of PRS processing measurement in RRC_IDLE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling |
| NR_pos_enh2 | X-LPHAP-11 | Support of PRS measurement in RRC_IDLE state for Multi-RTT | 1. Support of PRS measurement in RRC_IDLE state for Multi-RTT - location server | 13-4, 13-11, 27-6 | No |  | PRS measurement in RRC_IDLE state for Multi-RTT is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: PRS capabilities for Multi-RTT measurement and reporting described in FGs in 13-4, 13-4a, 13-4b, 13-11, 13-11a, 13-14 are the same for RRC Inactive Support of PRS processing measurement in RRC_IDLE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling |

#### 4.4 PRS/SRS BW Aggregation

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | - Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| NR_pos_enh2 | X-Agg-1 | DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous within a MG | - Maximum DL PRS bandwidth in MHz for each PFL, which is supported and reported by UE. a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} - Maximum DL PRS bandwidth in MHz summed across both PFLs, which is supported and reported by UE. a) FR1 bands: {10, 20, 40, 50, 80, 100, 160, 200} b) FR2 bands: {100, 200, 300, 400, 600, 800} - DL PRS buffering capability for each PFL: Type 1 or Type 2 - Type 1 – sub-slot/symbol level buffering - Type 2 – slot level buffering - Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz for each PFL, which is supported and reported by UE. - T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms - N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms - Max number of DL PRS resources that UE can process in a slot for each PFL: - FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz - FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) of no more than 30%. | [13-1] | No |  | Aggregated PRS processing of 2 PFLs in this band is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Notes for component 4: a.UE reports one combination of (N, T) values per band, where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz in each of the PFLs supported by UE b. UE is not expected to support DL PRS bandwidth that exceeds the reported DL PRS bandwidth value in each PFL d.UE DL PRS processing capability is agnostic to DL PRS comb factor configuration e.The reporting of (N, T) values for maximum BW in MHz is not dependent on SCS Note: if the UE does not indicate this capability for a band, the UE does not support this PRS aggregation in this band. Note for component 5: This is not applicable for 1-symbol PRS | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_pos_enh2 | X-Agg-2 | DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous within a MG | - Maximum DL PRS bandwidth in MHz for each PFL, which is supported and reported by UE. a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} - Maximum DL PRS bandwidth in MHz summed across both PFLs, which is supported and reported by UE. a) FR1 bands: { 10, 15, 20, 30, 40, 50, 60, 80, 100, 120, 150, 180, 200, 240, 300} b) FR2 bands: { 150, 200, 300, 400, 600, 800, 1000, 1200} - DL PRS buffering capability for a PFL: Type 1 or Type 2 - Type 1 – sub-slot/symbol level buffering - Type 2 – slot level buffering - Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz for each PFL, which is supported and reported by UE. - T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms - N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms - Max number of DL PRS resources that UE can process in a slot for each PFL: - FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz - FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) of no more than 30%. | [13-1, X-Agg-1] | No |  | Aggregated PRS processing of 3 PFLs in this band is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Notes for component 4: a.UE reports one combination of (N, T) values per band, where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz in each of the PFLs supported by UE b. UE is not expected to support DL PRS bandwidth that exceeds the reported DL PRS bandwidth value in each PFL d.UE DL PRS processing capability is agnostic to DL PRS comb factor configuration e.The reporting of (N, T) values for maximum BW in MHz is not dependent on SCS Note: if the UE does not indicate this capability for a band, the UE does not support this PRS aggregation in this band. Note for component 5: This is not applicable for 1-symbol PRS | Optional with capability signaling |
| NR_pos_enh2 | X-Agg-3 | Support of UE Rx TEGs for measuring the same aggregated DL PRS resources | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS aggregated PRS resources |  | No |  | Up to 1 RxTEG is used to measure the same aggregated DL PRS resources of a TRP | per band | n/a | n/a | n/a | The candidate values are {2, 3, 4, 6, 8} Need for location server to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-Agg-4 | Support of UE Rx TEGs for measuring the same aggregated DL PRS resources simultaneously | The maximum number of UE Rx TEGs for measuring the same aggregated DL PRS resources simultaneously | 27-1-4 | No |  | No assumption can be made regarding multiple Rx TEGs measuring the same aggregated DL PRS resources simultaneously | Per band | n/a | n/a | n/a | The candidate values are {1,2,3,4,6,8} Need for location server to know if the feature is supported. | Optional with capability signaling |
| NR_pos_enh2 | X-Agg-5 | M-sample aggregated measurements in RRC_CONNECTED | The capability to support reporting an aggregated measurement based on measuring M=1 or 2 samples (instances) of aggregated DL PRS resources | 13-1 | No |  | If the UE does not provide the capability, the UE is assumed to support M=4 only | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported Note: this feature is supported for both UE-assisted and UE based positioning | Optional with capability signaling |
| NR_pos_enh2 | X-Agg-6 | Aggregated SRS Transmissions of 2 SRS for Positioning in intra-band contiguous CCs in RRC_Connected state | Support of Aggregated SRS Transmissions of 2 SRS for Positioning in intra-band contiguous CCs. - The UE supports the simultaneous transmission in a coherent manner of 2 SRS resources in 2 intra-band contiguous CCs. Note: No other signal is expected to be simultaneously transmitted with the 2 aggregated SRS resources in the band | [13-8]c | Yes |  | Aggregated SRS Transmissions of 2 SRS for Positioning in intra-band contiguous CCs in this band is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| NR_pos_enh2 | X-Agg-7 | Aggregated SRS Transmissions of 3 SRS for Positioning in intra-band contiguous CCs in RRC_Connected state | Support of Aggregated SRS Transmissions of 3 SRS for Positioning in intra-band contiguous CCs. - The UE supports the simultaneous transmission in a coherent manner of 3 SRS resources in 3 intra-band contiguous CCs, each SRS resource on one CC. Note: No other channel is expected to be simultaneously transmitted with the 3 aggregated SRS resources in the band | [13-8] | Yes |  | Aggregated SRS Transmissions of 3 SRS for Positioning in intra-band contiguous CCs in this band is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| NR_pos_enh2 | X-Agg-8 | Decoupled SRS for Positioning for aggregation from the UL communication CA in RRC_Connected state - 1 CC without PUSCH/PUCCH | - the maximum SRS bandwidth supported for each SCS that UE supports within a single CC | [13-8] | Yes |  | If the UE doesn’t report this feature, it doesn’t support the configuration of decoupled SRS for Positioning from the UL Communication CA | Per band | n/a | n/a | n/a | Component 1 candidate values: - FR1 bands: {5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100} - FR2 bands: {50, 100, 200, 400} Note: Support the configuration and transmission of periodic SRS within a CC without PUSCH/PUCCH which can be linked for aggregation with an SRS configured within an UL active BWP of a regular UL communication CC | Optional with capability signaling |
| NR_pos_enh2 | X-Agg-9 | Decoupled SRS for Positioning for aggregation from the UL communication CA in RRC_Connected state - 2 CC without PUSCH/PUCCH | - Maximum SRS bandwidth supported for each SCS that UE supports within a single CC | [X-Agg-8] | Yes |  |  | Per band | n/a | n/a | n/a | Component 1 candidate values: - FR1 bands: {5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100} - FR2 bands: {50, 100, 200, 400} Note: Support the configuration and transmission of periodic SRS within 2 CCs without PUSCH/PUCCH which can be linked for aggregation with an SRS configured within an UL active BWP of a regular UL communication CC | Optional with capability signaling |
| NR_pos_enh2 | X-Agg-10 | Guard time between and after for the decoupled SRS for Positioning | - Guard time needed before and after the aggregated SRS transmissions for the decoupled SRS for Positioning scenario | [X-Agg-8] | Yes |  |  | Per band | n/a | n/a | n/a | Values Up to RAN4 | Optional with capability signaling |
| NR_pos_enh2 | X-Agg-11 | Aggregated SRS transmissions for 2 SRS for positioning in a band in RRC_INACTIVE state configured outside initial UL BWP | - Support of aggregated SRS transmissions for 2 SRS for Positioning in a band in RRC_INACTIVE state configured outside initial UL BWP - TBD additional components based on further agreements | [27-15b] | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| NR_pos_enh2 | X-Agg-12 | Aggregated SRS transmissions for 3 SRS for positioning in a band in RRC_INACTIVE state configured outside initial UL BWP | - Support of aggregated SRS transmissions for 3 SRS for Positioning in a band in RRC_INACTIVE state configured outside initial UL BWP - TBD additional components based on further agreements | [27-15b] | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |

#### 4.5 Redcap Frequency Hopping

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | - Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_pos_enh2 | X-Redcap-1 | Support of DL PRS Hopping and processing within a MG | - Maximum PRS BW per hop which is supported and reported by UE a) FR1 bands: {5, 10, 20} b) FR2 bands: {50, 100} - One or more Frequency domain overlap(s) between hops which are supported and reported by UE a) {1 PRB, 2 PRBs, 4 PRBs} - RF Rx retune times between consecutive hops which is supported and reported by the UE a) FR1 bands: {70 usec, 140 usec} b) FR2 bands: {35 usec, 70 usec} Note: The UE is expected to support both intra-slot and inter-slot PRS hopping | [13-1] | No |  | DL PRS frequency hopping is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported.. | Optional with capability signaling |
| NR_pos_enh2 | X-Redcap-2 | Support of Tx Frequency Hopping SRS for Positioning resources | - Maximum number of SRS BW per hop which is supported and reported by UE - FR1 bands: {5, 10, 20} - FR2 bands: {50, 100} - One or more Frequency domain overlap(s) between hops which are supported and reported by UE - {0 RPB, 1 PRB, 2 PRBs, 4 PRBs} - RF Tx retune times between consecutive hops which is supported and reported by the UE - FR1 bands: {70 usec, 140 usec} - FR2 bands: {35 usec, 70 usec, 140 usec} - Max number of Tx Frequency Hopping SRS Resource Sets for positioning supported by UE per CC. Values = {1, 2, 4, 8, 12, 16}. - Max number of P/SP/AP Tx Frequency Hopping SRS Resources for positioning per CC. Values = {1,2,4,8,16,32,64} - Max number of P/SP/AP SRS Resources including the SRS resources for positioning per CC per slot. Values = {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} - Max number of periodic SRS Resources for positioning per CC. Values = {1,2,4,8,16,32,64} - Max number of periodic SRS Resources for positioning per CC per slot. Values = {1,2,3,4,5,6,8,10,12,14} Note: The UE is expected to support intra-slot SRS Tx frequency hopping |  | Yes |  | Tx Frequency Hopping SRS for Positioning is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: No additional UE requirements shall be specified for the case of Tx hopping with non-overlapping hops compared to the case of Tx hopping with overlapping hops, e.g., a UE is not responsible for keeping phase continuity across the hops in either case of overlapping or non-overlapping hops. | Optional with capability signaling |
| NR_pos_enh2 | X-Redcap-3 | Support of SRS Tx inter-slot Frequency Hopping for SRS for Positioning | Support of inter-slot repetition configuration for an SRS resource for positioning | [X-Redcap-2] | Yes |  | Inter-slot SRS hopping is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |