3GPP TSG RAN WG1 #113				R1-2305374

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.16.11

Source: 	Qualcomm Incorporated

Title: 	UE features for IOT NTN

Document for:	Discussion/Decision

- Introduction

In this contribution, we discuss our initial views on UE features for Rel-18 IOT NTN.

- UE features for IOT NTN

In the table below, we show our initial views for the necessary UE features in IOT NTN. Features for NB-IoT NTN are highlighted in orange, and features for eMTC NTN are highlighted in blue.

Proposal: The following table is taken as the starting point for UE features for Rel-18 IOT NTN.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IoT_NTN_enh | X-1 | HARQ feedback disabling for eMTC in CE mode A | Support of HARQ disabling in CE mode A. | 2-1 | Yes |  |  | Per band | FDD only |  | GSO / NGSO differentiation is supported for this feature group. | Optional with capability signalling |
| IoT_NTN_enh | X-2 | HARQ feedback disabling for eMTC in CE mode B - semistatic | Support of HARQ disabling with Option 1 in CE mode B. | 2-1 | Yes |  |  | Per band | FDD only |  | GSO / NGSO differentiation is supported for this feature group. | Optional with capability signalling |
| IoT_NTN_enh | X-3 | HARQ feedback disabling for eMTC in CE mode B – dynamic | Support of HARQ disabling with Option 3 in CE mode B | 2-1 | Yes |  |  | Per band | FDD only |  | GSO / NGSO differentiation is supported for this feature group. | Optional with capability signalling |
| IoT_NTN_enh | X-4 | HARQ feedback disabling for eMTC in CE mode B – dynamic + semistatic | Support of HARQ disabling with Option 1 + Option 3 in CE mode B. | 2-1 | Yes |  |  | Per band | FDD only |  | GSO / NGSO differentiation is supported for this feature group. | Optional with capability signalling |
| IoT_NTN_enh | X-5 | HARQ feedback disabling for eMTC in CE mode A – multi-TB | Support of HARQ disabling with multi-TB in CE mode A | [Multi-TB], 2-1 | Yes |  |  | Per band | FDD only |  | GSO / NGSO differentiation is supported for this feature group. | Optional with capability signalling |
| IoT_NTN_enh | X-6 | HARQ feedback disabling for eMTC in CE mode B – multi-TB | Support of HARQ disabling with multi-TB in CE mode B | [Multi-TB], 2-1 | Yes |  |  | Per band | FDD only |  | GSO / NGSO differentiation is supported for this feature group. | Optional with capability signalling |
| IoT_NTN_enh | X-7 | HARQ feedback disabling with SPS | Support of simultaneous configuration of HARQ disabling and SPS. | TBD | Yes |  |  | Per band | FDD only |  | GSO / NGSO differentiation is supported for this feature group. | Optional with capability signalling |
| IoT_NTN_enh | X-1a | HARQ feedback feedback disabling for NB-IoT – semistatic | Support of HARQ disabling with Option 1 | 2-1b | Yes |  |  | Per band | FDD only |  | GSO / NGSO differentiation is supported for this feature group. | Optional with capability signalling |
| IoT_NTN_enh | X-2a | HARQ feedback disabling for NB-IoT – dynamic | Support of HARQ disabling with Option 3 | 2-1b | Yes |  |  | Per band | FDD only |  | GSO / NGSO differentiation is supported for this feature group. | Optional with capability signalling |
| IoT_NTN_enh | X-3a | HARQ feedback disabling for NB-IoT – dynamic + semistatic | Support of HARQ disabling with Option 1 + Option 3 | 2-1b, [X-2a and X-1a] | Yes |  |  | Per band | FDD only |  | GSO / NGSO differentiation is supported for this feature group. | Optional with capability signalling |
| IoT_NTN_enh | X-4a | HARQ feedback disabling for NB-IoT – multi-TB | Support of HARQ disabling with multi-TB | 2-1b | Yes |  |  | Per band | FDD only |  | GSO / NGSO differentiation is supported for this feature group. | Optional with capability signalling |
| IoT_NTN_enh | X-8 | Triggered GNSS reacquisition for eMTC | Support of GNSS reacquisition by MAC-CE trigger. | 2-1 | Yes |  |  | Per band | FDD only |  | GSO / NGSO differentiation is supported for this feature group. | Optional with capability signalling |
| IoT_NTN_enh | X-9 | Autonomous GNSS reacquisition for eMTC | Support of autonomous GNSS reacquisition. | 2-1 | Yes |  |  | Per band | FDD only |  | GSO / NGSO differentiation is supported for this feature group. | Optional with capability signalling |
| IoT_NTN_enh | X-10 | [RRC Connection extension via closed loop for eMTC] | Support of extension of RRC Connection via closed loop commands for eMTC. | 2-1 | Yes |  |  | Per band | FDD only |  | GSO / NGSO differentiation is supported for this feature group. | Optional with capability signalling |
| IoT_NTN_enh | X-8a | Triggered GNSS reacquisition for NB-IoT | Support of GNSS reacquisition by MAC-CE trigger. | 2-1b | Yes |  |  | Per band | FDD only |  | GSO / NGSO differentiation is supported for this feature group. | Optional with capability signalling |
| IoT_NTN_enh | X-9a | Autonomous GNSS reacquisition for NB-IoT | Support of autonomous GNSS reacquisition. | 2-1b | Yes |  |  | Per band | FDD only |  | GSO / NGSO differentiation is supported for this feature group. | Optional with capability signalling |
| IoT_NTN_enh | X-10a | [RRC Connection extension via closed loop for NB-IoT] | Support of extension of RRC Connection via closed loop commands for NB-IoT. | 2-1b | Yes |  |  | Per band | FDD only |  | GSO / NGSO differentiation is supported for this feature group. | Optional with capability signalling |