3GPP TSG RAN WG1 #113		R1-2305410

Incheon, Korea, May 22nd – May 26th, 2023

Source:	OPPO

Title:	Discussion on SRS insertion loss issue

Agenda Item:	5

Document for:	Discussion and Decision

Introduction

In RAN4#106-e meeting, LS R4-2303519 [1] was sent to RAN1 with following action:

ACTION: RAN4 respectfully ask RAN1 to consider above issue with, but not limited to the three resolutions listed in the Annex for their future study. ANNEX: 1. Enable UE report on the actual IL imbalance for each diversity branch used for SRS so network can use this information for AS-SRS based channel estimation accuracy improvement where the granularity of such report can be per SRS resource and either static or dynamic. 2. Utilize PCMAX, f, c via PHR type 3, where power imbalances across antenna ports are derived by comparing the maximum configured power of main branch to those of diversity branches, where such report is per transmission occasion per SRS (only a report of a single SRS resource is transmitted per transmission occasion). 3. Define UE measurements of downlink channels which are reported in order to assist the network in determining the difference between the UE insertion losses for two given antenna ports, where the network also does its own measurements of SRS channels. 4. RAN4 does not preclude other options. The above alternatives are considered from Release 18 onwards and for 8Rx capable UE’s, and possible applicability UE’s supporting 2RX or 4RX is FFS.

In this contribution, we discuss the SRS IL imbalance issue with some evaluation results.

Discussion

  - Performance loss due to SRS IL imbalance

In this section, we provide some evaluation results for downlink non-codebook based transmission with/without SRS IL imbalance. The evaluation assumption can be found in appendix with 1T8R. The SRS IL imbalance among the 8 antennae is assumed to be [0, -3, -3, -3, -3, -4, -4, -4] dB. The performance of both low rank (e.g. Rank 1-2 with rank adaptation) and high rank (rank=4) is evaluated.

To further evaluate the impact of SRS IL imbalance to downlink transmission with MU-MIMO, SLS simulation result is also provided, with 1-2 layers rank adaptation per UE.

| DL THP | cell average (bps/Hz) ILI OFF | cell average (bps/Hz) ILI ON | cell edge (bps/Hz) ILI OFF | cell edge (bps/Hz) ILI ON | Average loss | Edge loss |
| --- | --- | --- | --- | --- | --- | --- |
| 1T8R | 17.87 | 17.65 | 7.23 | 7.08 | -1.2% | -2.0% |

From the results, it can be observed that SRS IL imbalance has no impact to downlink transmission with low rank, and the loss is also negligible with high rank.

  - On potential resolutions

On the 3 potential resolutions provided by RAN4 for further study, we have the following observations:

- For resolution 1, we think semi-static reporting cannot work at all. The mapping between physical Tx and antenna port can be changed by UE, and the IL loss may also vary with the environment (e.g. temperature). For dynamic reporting, UE needs to persistently measure the IL in each antenna port, which would introduce significant UE complexity and power consumption.

- For resolution 2, the standardization impact is significant considering current PHR reporting is per SRS resource set instead of per resource/port. The performance loss is not worth the effort.

- For resolution 3, the standardization impact is also great and the corresponding measurement requires additional UE complexity and power consumption.

These resolutions can be considered when the impact of SRS IL imbalance is significant. But based on our evaluation, we haven’t observed the use case by now.

Proposal: From RAN1 perspective, for downlink transmission without PMI feedback, no enhancement is needed to address the SRS IL imbalance issue.

References

- R4-2303519, LS on the UE SRS IL imbalance issue, Huawei, HiSilicon, RAN4#106-e meeting

Appendix

Table 1: Simulation assumption

| Parameter | Assumption |
| --- | --- |
| Carrier frequency | 4 GHz |
| SCS | 30 kHz |
| Channel model | CDL-C in TR 38.901 with 100ns delay spread |
| UE velocity | 3km/h |
| Allocation bandwidth | 20MHz |
| BS antenna configuration | 32T: (M, N, P, Mg, Ng, Mp, Np) = (2,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| UE antenna configuration | 1T8R: (M, N, P, Mg, Ng, Mp, Np) = (2,2,2,1,1,2,2), (dH,dV) = (0.5, 0.5)λ |
| SU-MIMO layers | Rank=4 Rank=1-2 with adaptation |
| Link adaptation | MCS adaptation |
| Precoding | SVD based precoding with PRG=4 |
| SRS periodicity | 20ms |
| SRS channel estimation | Realistic channel estimation |
| Receiver type | MMSE |
| IL values | SRS IL = [0, -3, -3, -3, -3, -4, -4, -4] dB |